| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0013-V.3.4.1 |
| Document Name: | Interoperability Testing |
| Date: | 2025-08-26 |
| Abstract: | The specification addresses the testing of the primitives on the oneM2M interfaces as specified in TS-0001 [1] and TS-0004 [2]. The purpose of the interoperability testing is to prove end-to-end functionality between Application Entities and Common Service Entities over the Mca and Mcc reference points. |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1. Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners’ Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at: http//www.oneM2M.org

Copyright Notification

No part of this document may be reproduced, in an electronic retrieval system or otherwise, except as authorized by written permission.

The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

1 Scope
2 References
    2.1 Normative references
    2.2 Informative references
3 Definitions of terms, symbols and abbreviations
    3.1 Terms
    3.2 Symbols
    3.3 Abbreviations
4 Conventions
5 Testing conventions
    5.1 The Test Description proforma
    5.2 Test Description naming convention
    5.3 Test Settings
    5.4 Pre-conditions
        5.4.1 Registration
        5.4.2 Security
        5.4.3 Service Subscription
        5.4.4 ID allocation
        5.4.5 Existence of resource
        5.4.6 Management Session between Management Server and Management Client
    5.5 Binding message convention
6 Test Description Summary
    6.1 Tests list
7 Configuration
    7.1 Test Configuration
        7.1.1 No hop
            7.1.1.1 M2M_CFG_01
            7.1.1.2 M2M_CFG_02
            7.1.1.3 M2M_CFG_10
        7.1.2 Single hop
            7.1.2.1 M2M_CFG_03
            7.1.2.2 M2M_CFG_04
            7.1.2.3 M2M_CFG_05
            7.1.2.4 M2M_CFG_08
            7.1.2.5 M2M_CFG_09
            7.1.2.6 M2M_CFG_11
            7.1.2.7 M2M_CFG_12
        7.1.3 Multi hops
            7.1.3.1 M2M_CFG_06
            7.1.3.2 M2M_CFG_07
8 Test Descriptions
    8.1 No Hop configuration testing
        8.1.1 CSEBase Management
            8.1.1.1 CSEBase Retrieve on Mca
        8.1.2 RemoteCSE Management
            8.1.2.1 RemoteCSE Create
            8.1.2.2 remoteCSE Retrieve
            8.1.2.3 remoteCSE Update
            8.1.2.4 remoteCSE Delete
        8.1.3 Application Entity Registration
            8.1.3.1 AE Create
            8.1.3.2 AE Retrieve
            8.1.3.3 AE Update
            8.1.3.4 AE Delete
        8.1.4 Container Management
            8.1.4.1 Container Create
            8.1.4.2 Container Retrieve
            8.1.4.3 Container Update
            8.1.4.4 Container Delete
        8.1.5 ContentInstance Management
            8.1.5.1 ContentInstance Create
            8.1.5.2 ContentInstance Retrieve
            8.1.5.3 ContentInstance Delete
            8.1.5.4 <latest> ContentInstance Delete
            8.1.5.5 <oldest> ContentInstance Delete
            8.1.5.6 ContentInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <container> resource
            8.1.5.7 <latest> ContentInstance Retrieve
                8.1.5.7.1 Attribute locationID of the <container> resource configured
                8.1.5.7.2 Attribute locationID of the <container> resource not configured
            8.1.5.8 <oldest> ContentInstance Retrieve
        8.1.6 Discovery
            8.1.6.1 Discovery of all resources
            8.1.6.2 Discovery with label filter criteria
            8.1.6.3 Discovery with limit filter criteria
            8.1.6.4 Discovery with multiple filter criteria
            8.1.6.5 Discovery with level filter criteria
            8.1.6.6 Discovery with offset filter criteria
        8.1.7 Subscription Management
            8.1.7.1 Subscription Create
            8.1.7.2 Subscription Retrieve
            8.1.7.3 Subscription Update
            8.1.7.4 Subscription Delete
        8.1.8 accessControlPolicy Management
            8.1.8.1 accessControlPolicy Create
            8.1.8.2 accessControlPolicy Retrieve
            8.1.8.3 accessControlPolicy Update
            8.1.8.4 accessControlPolicy Delete
            8.1.8.5 Unauthorized operation (Insufficient Access Rights, operations)
            8.1.8.6 Unauthorized operation (Insufficient Access Rights, originators)
            8.1.8.7 Authorized operation
        8.1.9 Group Management
            8.1.9.1 Group Retrieve
            8.1.9.2 Group Create
            8.1.9.3 Group Update
            8.1.9.4 Group Delete
        8.1.10 Node Management
            8.1.10.1 Node Create
            8.1.10.2 Node Retrieve
            8.1.10.3 Node Update
            8.1.10.4 Node Delete
        8.1.11 PollingChannel Management
            8.1.11.1 PollingChannel Create
            8.1.11.2 PollingChannel Retrieve
            8.1.11.3 pollingChannel Update
            8.1.11.3 pollingChannel Update
            8.1.11.4 pollingChannel Delete
            8.1.11.5 Long Polling on a PollingChannel Retrieve
        8.1.12 FanoutPoint Management
            8.1.12.1 FanoutPoint Create
            8.1.12.2 FanoutPoint Retrieve
            8.1.12.3 FanoutPoint Update
            8.1.12.4 FanoutPoint Delete
        8.1.13 Notification Management
            8.1.13.1 Notification
            8.1.13.2 Update Notification
            8.1.13.3 Delete Notification
            8.1.13.4 Creation of Direct Child Resource Notification
            8.1.13.5 Deletion of Direct Child Resource Notification
            8.1.13.6 Notifcation Aggregation
        8.1.14 FlexContainer Management
            8.1.14.1 FlexContainer Create
            8.1.14.2 FlexContainer Retrieve
            8.1.14.3 FlexContainer Update
            8.1.14.4 FlexContainer Delete
            8.1.14.5 Notification Create
            8.1.14.6 Discovery with attribute filter criteria over customAttributes
        8.1.15 External Management Operations Management
            8.1.15.1 mgmtCmd Create
            8.1.15.2 mgmtCmd Retrieve
            8.1.15.3 mgmtCmd Update (Normal)
            8.1.15.4 mgmtCmd Update (Execute)
            8.1.15.5 mgmtCmd Delete
            8.1.15.6 execInstance Retrieve
            8.1.15.7 execInstance Update (Cancel)
            8.1.15.8 execInstance Delete
        8.1.16 SemanticDescriptor Management
            8.1.16.1 SemanticDescriptor Create
            8.1.16.2 SemanticDescriptor Retrieve
            8.1.16.3 SemanticDescriptor Update
            8.1.16.4 SemanticDescriptor Delete
        8.1.17 Semantic Resource Discovery
            8.1.17.1 Discovery with semanticFilter filter criteria
        8.1.18 ResultContent
            8.1.18.1 ResultContent=0
            8.1.18.2 ResultContent=1
            8.1.18.3 ResultContent=2
            8.1.18.4 ResultContent=3
            8.1.18.5 ResultContent=4
        8.1.19 timeSeries Management
            8.1.19.1 timeSeries Create
            8.1.19.2 timeSeries Retrieve
            8.1.19.3 timeSeries Update
            8.1.19.4 timeSeries Delete
        8.1.20 timeSeriesInstance Management
            8.1.20.1 timeSeriesInstance Create
            8.1.20.2 timeSeriesInstance Retrieve
            8.1.20.3 timeSeriesInstance Delete
            8.1.20.4 timeSeriesInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <timeSeries> resource
        8.1.21 Location Management
            8.1.21.1 LocationPolicy Create
            8.1.21.2 LocationPolicy Retrieve
            8.1.21.3 LocationPolicy Update
            8.1.21.4 LocationPolicy Delete
        8.1.22 Schedule management
            8.1.22.1 Schedule Create
            8.1.22.2 Schedule Retrieve
            8.1.22.3 Schedule Update
            8.1.22.4 Schedule Delete
            8.1.22.5 Notification to AE with configured Schedule resource
    8.2 Non-blocking configuration testing
        8.2.1 Synchronous request
            8.2.1.1 Container management
                8.2.1.1.1 Container Create
                8.2.1.1.2 Container Retrieve
                8.2.1.1.3 Container Update
                8.2.1.1.4 Container Delete
        8.2.2 Asynchronous request
            8.2.2.1 Container management
                8.2.2.1.1 Container Create
                8.2.2.1.2 Container Retrieve
                8.2.2.1.3 Container Update
                8.2.2.1.4 Container Delete
    8.3 Single hop configuration testing
        8.3.1 Retargeting
            8.3.1.1 RetargetingResource Create (Generic Test Description)
            8.3.1.2 <Resource> Create
            8.3.1.3 Resource Retrieve (Generic Test Description)
            8.3.1.4 <Resource> retrieve
            8.3.1.5 Resource Update (Generic Test Description)
            8.3.1.6 <Resource> update
            8.3.1.7 Resource Delete (Generic Test Description)
            8.3.1.8 <Resource> delete
            8.3.1.9 Discovery with multiple filter criteria
            8.3.1.10 Unauthorized operation (Insufficient Access Rights)
            8.3.1.11 Notification
        8.3.2 <mgmtObj> Test Description
            8.3.2.1 <mgmtObj> Create
            8.3.2.2 <mgmtObj> Update
            8.3.2.3 <mgmtObj> Retrieve
            8.3.2.4 <mgmtObj> Delete
        8.3.3 Announcement Management
            8.3.3.1 AEAnnc Create
            8.3.3.2 ContainerAnnc Create
            8.3.3.3 ContainerAnnc Update
            8.3.3.4 ContainerAnnc Retrieve
            8.3.3.5 ContainerAnnc Retrieve Original
            8.3.3.6 ContainerAnnc Delete by updating announceTo attribute
        8.3.4 Single Hop <fanOutPoint> operations
            8.3.4.1 Create <fanOutPoint>
            8.3.4.2 Retrieve <fanOutPoint>
            8.3.4.3 Update <fanOutPoint>
            8.3.4.4 Delete <fanOutPoint>
    8.4 Security management
        8.4.1 Secure AE Registration
            8.4.1.1 PSK Security Association Establishment Framework
        8.4.2 Authentication
            8.4.2.1 Authentication using the Provisioned Symmetric Key Security Association Establishment Framework with TLS
            8.4.2.2 Authentication using the Certificate-Based Security Association Establishment Framework with TLS
        8.4.3 Authorization
            8.4.3.1 Authorization using selfPrivileges
            8.4.3.2 Authorization using accessControlPolicy privileges
            8.4.3.3 Authorization using default access privileges (owner is configured)
            8.4.3.4 Authorization using default access privileges (owner is not configured)
            8.4.3.5 Direct Dynamic Authorization
            8.4.3.6 Indirect Dynamic Authorization
        8.4.4 Key provisioning management
            8.4.4.1 MEF Handshake Procedure using certificates
            8.4.4.2 MEF Handshake Procedure using Master Credentials
            8.4.4.3 MEF Client Registration Procedure
            8.4.4.4 MEF Client Configuration Retrieval Procedure
            8.4.4.5 MEF Client Configuration Update Procedure
            8.4.4.6 MEF Client De-Registration Procedure
            8.4.4.7 MEF Key Registration Procedure
            8.4.4.8 MEF Key Retrieval Procedure
            8.4.4.9 MEF Key Registration Update Procedure
            8.4.4.10 MEF Key De-Registration Procedure
        8.4.5 End-to-End security management
            8.4.5.1 End-to-End Security of Primitives (ESPrim) Architecture
            8.4.5.2 End-to-End Certificate-based Key Establishment (ESCertKE)
    8.5 HAIM Device Model
        8.5.1 HAIM Light Device Creation
        8.5.2 HAIM Light Device Status Read
        8.5.3 HAIM Light Device Update
        8.5.4 HAIM Light Device Toggle Action
        8.5.5 HAIM Power Outlet SubDevice Create
        8.5.6 HAIM Toggle Action Create
        8.5.7 HAIM Device Properties Create
    8.6 Semantics management
        8.6.1 Semantic Access Control Policy management
            8.6.1.1 Procedure for creating ACP triples when a new <accessControlPolicy> resource is created
            8.6.1.2 Procedure for updating ACP triples when a new <accessControlPolicy> resource is updated
            8.6.1.3 Procedure for deleting ACP triples when an existing <accessControlPolicy> resource is deleted
            8.6.1.4 Procedure for creating ACP-SD binding triples and SD relationship in SGS
            8.6.1.5 Procedure for updating ACP-SD binding triples in SGS
            8.6.1.6 Procedure for updating SD relationship triples in SGS
            8.6.1.7 Procedure for deleting SD relationship triples and ACP-SD binding triples in SGS
        8.6.2 Semantic Filtering and discovery
            8.6.2.1 Semantic Filtering and Discovery using <semanticFanOutPoint> resource
            8.6.2.2 Resource link-based Semantic Discovery
            8.6.2.3 Semantic query
        8.6.3 Semantic Mashup management
            8.6.3.1 SemanticMashupJobProfile Create
            8.6.3.2 SemanticMashupJobProfile Retrieve
            8.6.3.3 SemanticMashupJobProfile Update
            8.6.3.4 SemanticMashupJobProfile Delete
            8.6.3.5 SemanticMashupInstance Create
            8.6.3.6 SemanticMashupInstance Retrieve
            8.6.3.7 SemanticMashupInstance Update
            8.6.3.8 SemanticMashupInstance Delete
            8.6.3.9 SemanticMashupResult Retrieve
            8.6.3.10 SemanticMashupResult Delete
            8.6.3.11 Mashup Retrieve
        8.6.4 Ontology Repository management
            8.6.5.1 OntologyRepository Create
            8.6.4.2 OntologyRepository Retrieve
            8.6.4.3 OntologyRepository Update
            8.6.4.4 OntologyRepository Delete
        8.6.5 Semantic validation management
            8.6.5.1 Semantic validation independent of <semanticDescriptor> resource operation
            8.6.5.2 Semantic validation triggered when Create a semanticDescriptor resource
        8.6.6 Ontology Mapping management
            8.6.6.1 OntologyMapping Create
            8.6.6.2 OntologyMapping Retrieve
            8.6.6.3 OntologyMapping Update
            8.6.6.4 OntologyMapping Delete
    8.7 3GPP Interworking
        8.7.1 Cellular IoT non-IP data delivery (NIDD)
            8.7.1.1 SCEF Configuration for NIDD
            8.7.1.2 SCEF-based Mobile Terminated NIDD
            8.7.1.3 SCEF-based Mobile Originated NIDD
        8.7.2 Monitoring events
            8.7.2.1 UE Reachability monitoring
            8.7.2.2 UE Availability after DDN Failure
            8.7.2.3 UE Communication Failure
            8.7.2.4 Roaming Status
            8.7.2.5 Location updating triggered by retrieval
        8.7.3 3GPP Based Device triggering
            8.7.3.1 General Procedure for 3GPP Based Device Triggering
            8.7.3.2 3GPP Based Device Trigger Recall/Replace Procedure
        8.7.4 Configuration of traffic patterns
        8.7.5 Group message delivery using MBMS
            8.7.5.1 Create MBMS Group
            8.7.5.2 Group message delivery using MBMS
    8.8 Advanced Subscriptions & Notifications management
        8.8.1 Notification Target removal procedure
        8.8.2 NotificationTargetMgmtPolicyRef management
            8.8.2.1 NotificationTargetMgmtPolicyRef Create
            8.8.2.2 NotificationTargetMgmtPolicyRef Retrieve
            8.8.2.3 NotificationTargetMgmtPolicyRef Update
            8.8.2.4 NotificationTargetMgmtPolicyRef Delete
        8.8.3 NotificationTargetPolicy management
            8.8.3.1 NotificationTargetPolicy Create
            8.8.3.2 NotificationTargetPolicy Retrieve
            8.8.3.3 NotificationTargetPolicy Update
            8.8.3.4 NotificationTargetPolicy Delete
        8.8.4 PolicyDeletionRules management
            8.8.4.1 PolicyDeletionRules Create
            8.8.4.2 PolicyDeletionRules Retrieve
            8.8.4.3 PolicyDeletionRules Update
            8.8.4.4 PolicyDeletionRules Delete
        8.8.5 CrossResourceSubscription management
            8.8.5.1 CrossResourceSubscription Create
            8.8.5.2 CrossResourceSubscription Retrieve
            8.8.5.3 CrossResourceSubscription Update
            8.8.5.4 CrossResourceSubscription Delete
            8.8.5.5 Cross-Resource Notification
    8.9 Modbus Interworking
        8.9.1 Modbus Thermometer Device Create
        8.9.2 Retrieve data from a Modbus Thermometer device
        8.9.3 Write data to a Modbus Thermometer device
    8.10 NoDN Interworking
        8.10.1 Retrieve data from a NoDN device
        8.10.2 Write data to a NoDN device
History

# 1 Scope

The present document specifies Interoperability Test Descriptions (TDs) for the oneM2M Primitives as specified in oneM2M TS-0001 [1], oneM2M TS-0004 [2], the bindings oneM2M TS-0008 [3], oneM2M TS-0009 [4] and oneM2M TS-0010 [5].

# 2 References

## 2.1 Normative references

References are either specific (identified by date of publication and/or edition number or version number) or nonspecific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

- [1] oneM2M TS-0001: “Functional Architecture- Release 3”.

- [2] oneM2M TS-0004: “Service Layer Core protocol Specification - Release 3”.

- [3] oneM2M TS-0008: “CoAP Protocol Binding Release 3”.

- [4] oneM2M TS-0009: “HTTP Protocol Binding - Release 3”.

- [5] oneM2M TS-0010: “MQTT Protocol Binding - Release 3”.

- [6] oneM2M TS-0015: “Testing Framework”.

- [7] oneM2M TS-0011: “Common Terminology”.

- [8] IETF RFC 3986: “Uniform Resource Identifier (URI): Generic Syntax”.

- [9] IETF RFC 7230: “Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing”.

- [10] oneM2M TS-0005: “Management Enablement (OMA) - Release 3”.

- [11] oneM2M TS-0006: “Management Enablement (BBF) - Release 3”.

- [12] oneM2M TS-0003: “Security Solutions - Release 3”.

- [13] oneM2M TS-0034: “Semantics Support - Release 3”.

- [14] oneM2M TS-0023: “Home Appliances Information Model and Mapping - Release 3”.

- [15] oneM2M TS-0026: “3GPP interworking - Release 4”.

- [16] oneM2M TS-0040: “Modbus Interworking - Release 4”.

## 2.2 Informative references

References are either specific (identified by date of publication and/or edition number or version number) or nonspecific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- [i.1] oneM2M Drafting Rules.

NOTE: Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

- [i.2] BBF TR-069: “CPE WAN Management Protocol”.

# 3 Definitions of terms, symbols and abbreviations

## 3.1 Terms

For the purposes of the present document, the terms given in oneM2M TS-0011 [7] and the following apply:

NOTE: A term defined in the present document takes precedence over the definition of the same term, if any, in oneM2M TS-0011 [7].

hosting CSE : CSE where the addressed resource is hosted

M2M service provider domain: part of the M2M System that is associated with a specific M2M Service Provider

mc : interface between the management server and the management client

NOTE: This interface can be realized by the existing device management technologies such as BBF TR-069 [i.2], OMA DM [10], etc.

receiver CSE : any CSE that receives a request

registrar CSE : CSE where an Application or another CSE has registered

registree: AE or CSE that registers with another CSE

resource: uniquely addressable entity in oneM2M architecture

transit CSE : any receiver CSE that is not a Hosting CSE

## 3.2 Symbols

Void.

## 3.3 Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACP       Access Control Policy
ACP-SD    Access Control Policy-Semantic Descriptor
ADN-AE    AE which resides in the Application Dedicated Node
AE        Application Entity
AE-ID     Application Entity Identifier
APP-ID    Application Identifier
BBF       BroadBand Forum
CFG       Configuration
CoAP      Constrained Application Protocol
CP        Communication Patterns
CSE       Common Services Entity
CSE-ID    Common Service Entity Identifier
DAS       Dynamic Authorization System
DDN       Downlink Data Notification
DM        Device Management
DTLS      Datagram Transport Layer Security
DUT       Device Under Test
ECDHE     Elliptic-curve Diffie-Hellman
FQDN      Fully Qualified Domain Name
HAIM      Home Appliances Information Model
HSS       Home Subscriber Serve
HTTP      HyperText Transfer Protocol
IN        Infrastructure Node
IN-AE     Application Entity that is registered with the CSE in the Infrastructure Node
IN-CSE    CSE which resides in the Infrastructure Node
IOP       Interoperability
IP        Internet Protocol
IPE       Interworking Proxy Entity
JSON      JavaScript Object Notation
LWM2M     Lightweight M2M
M2M       Machine to Machine
MA        Mandatory Announced
MBMS      Multimedia Broadcast Multicast Service
Mca       Reference Point for M2M Communication with AE
Mcc       Reference Point for M2M Communication with CSE
MEF       M2M Enrolment Function
MH        Multi Hop
MNO       Mobile Network Operator
MO        Management Object
MQTT      Message Queuing Telemetry Transport
MT        Mobile Terminated
NB        Non-Blocking
NH        No Hop
NIDD      non-IP Data Delivery
NODN      Non oneM2M Device Node
OMA       Open Mobile Alliance
PDN       Packet Data Network
PRO       Protocol
PSK       Pre-Shared Key
PSM       Power Savings Mode
RDS       Reliable Data Service
RFC       Request for Comments
RP        Reference Point
RPC       Remote Procedure Calls
RQI       Request-ID
SCEF      Service Capability Exposure Function
SD        Semantic Descriptor
SE        Security
SGS       Semantic Graph Store
SH        Single Hop
SP        Service Provider
SPARQL    SPARQL Protocol and RDF Query Language
SUID      Security Usage Identifier
SUT       System Under Test
TCP       Transmission Control Protocol
TD        Test Description
TLS       Transport Layer Security
TMGI      Temporary Mobile Group Identity
TP        Traffic Pattern
UDP       User Datagram Protocol
URI       Uniform Resource Identifier
XML       eXtensible Markup Language

# 4 Conventions

The key words “Shall”, “Shall not”, “May”, “Need not”, “Should”, “Should not” in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

# 5 Testing conventions

## 5.1 The Test Description proforma

The testing methodology used in the present document is specified in the oneM2M TS-0015 [6].

A Test Description (TD) is a well detailed description of a process that aims to test one or more functionalities of an implementation. Applying to interoperability testing, these testing objectives address the interoperable functionalities between two or more vendor implementations.

In order to ensure the correct execution of an interoperability test, the following information should be provided by the test description:

- The proper configuration of the vendor implementations.

- The availability of additional equipment (protocol monitors, functional equipment, …) required to achieve the correct behaviour of the vendor implementations.

- The correct initial conditions.

- The correct sequence of the test events and test results.

In order to facilitate the specification of test cases an interoperability test description should include, at a minimum, the following fields as indicated table 5.1-1.

Table 5.1-1: Interoperability test description

| Identifier | A unique test description ID. |
| --- | --- |
| Objective | A concise summary of the test which should reflect the purpose of the test and enable readers to easily distinguish this test from any other test in the document. |
| References | A list of references to the base specification section(s), use case(s), requirement(s) and Test Purposes which are either used in the test or define the functionality being tested. |
| Applicability | A list of features and capabilities which are required to be supported by the SUT in order to execute this test (e.g. if this list contains an optional feature to be supported, then the test is optional). |
| Configuration or Architecture | A list of all required equipment for testing and possibly also including a reference to an illustration of a test architecture or test configuration. |
| Pre-Test Conditions | A list of test specific pre-conditions that need to be met by the SUT including information about equipment configuration, i.e. precise description of the initial state of the SUT required to start executing the test sequence. |
| Test Sequence | An ordered list of equipment operation and observations. The test sequence may also contain the conformance checks as part of the observations. |

The test descriptions are provided in proforma tables. In order to ensure the correct execution of an interoperability test, the following information is provided in the test description:

- The configuration applied for the test.

- The need of additional equipment (protocol monitors, functional equipment, etc.) required to achieve the correct behaviour of the implementations.

- The initial conditions.

- The sequence of the test events and test results.

The following different types of test operator actions are considered during the test execution:

- Astimulus corresponds to an event that enforces a DUT to proceed with a specific protocol action, such as sending a message.

- A configure corresponds to an action to modify the DUT configuration.

- An IOP check consists of observing that one DUT behaves as described in the standard: i.e. resource creation, update, deletion, etc. For each IOP check in the Test Sequence, a result can be recorded. The overall IOP Verdict will be considered OK if all the IOP checks in the sequence are OK.

- In the context of Interoperability Testing with Conformance Checks, an additional step type, PRO checks can be used to verify the appropriate sequence and contents of protocol messages, this is helpful for debugging purposes. PRO Verdict will be PASS if all the PRO checks are PASS.

## 5.2 Test Description naming convention

| TD/<root>/<gr>/<nn> |  |  |
| --- | --- | --- |
| <root> = root | M2M | oneM2M |
| <gr> = group | NH | No Hop: Testing on Mca reference point |
|  | NB | Non-Blocking scenario |
|  | SH | Single Hop: management of remote resources on Mca + Mcc |
|  | MH | Multi Hop |
|  | SE | Security |
| <nn> = sequential number |  | 01 to 99 |

## 5.3 Test Settings

This clause contains some test requirements applied to the testing, some constraints, restrictions for executions or some recommendations.

In order to ease test setup and execution, the CSE and AE are requested to support the following settings:

- Security shall be disable as it is out of scope of this interoperability testing.

- Resource names are pre-provisioned, except for content instance resources that are automatically assigned by the hosting CSE.

- After each “Delete” primitive on a resource, the user shall check the resource is effectively deleted.

- Unless it is indicated in the test cases prerequisites, by default, all the applications shall have the required access rights to manage resources on the CSE.

In order to address the TBDs in the oneM2M CoAP binding specification (oneM2M TS-0008 [3]), basic XML and JSON media-type numbers shall be used in the contentFormat option.

In the test descriptions specified below, the following definitions of terms used for short-hand notation apply:

Serialized Representation: refers to either an XML or a JSON representation of data in text-string format as defined in clauses 8.3 and 8.4 of oneM2M TS-0004 [2].

Host Address: refers to the authority part of a target URI as defined in IETF RFC 3986 [8] and IETF RFC 7230 [9] which can be represented as an IP literal encapsulated within square brackets, an IPv4 address in dotted decimal form, or a registered name, and optionally extended by a port identifier.

## 5.4 Pre-conditions

### 5.4.1 Registration

The AE or CSE that originates the request has been successfully registered to its corresponding CSE. The registration of the AE includes the creation of <AE> resource under the <CSEBase> of its registrar CSE. The registration of the CSE includes the creation of <remoteCSE> resource representing itself under the <CSEBase> of its registrar CSE as well as the creation of <remoteCSE> resource representing the registrar CSE under its own <CSEBase> resource. The creation of <remoteCSE> resource representing the registrar CSE can be achieved by remotely retrieving the <CSEBase> resource of the registrar CSE.

### 5.4.2 Security

The Originator and the receiver have successfully established security association between each other. This may involve the exchange of key and the establishment of a security connection.

The security pre-condition also assumes that the originator has the appropriate access control privilege towards the requested resource.

### 5.4.3 Service Subscription

Service subscription means that the originator is allowed to be connected with the oneM2M system by contract between the owner of the application and the service provider of the oneM2M system. This may require a corresponding information record in the <m2mServiceSubscriptionProfile> resource.

### 5.4.4 ID allocation

ID allocation means that the Originator has already acquired usable identity, either from its registrar CSE or the IN-CSE of the oneM2M system. The ID may be CSE relative or SP relative. The ID is then further used as the identity of the Originator to perform access control, charging, etc.

### 5.4.5 Existence of resource

Existence of resource means the resource been addressed and has already been created.

### 5.4.6 Management Session between Management Server and Management Client

Before the device management using external technologies is executed, it is required that a management session has already been established between the Management Server and Management Client. If there is no existing management session, the IN-CSE shall request the establishment of a management session between the Management Server and Management Client.

## 5.5 Binding message convention

In HTTP/CoAP/MQTT binding messages, the present document defines the convention for <variable>:

- <resourceType> represents a resource name (i.e. resourceName attribute) of a resource instance in that resourceType. For example, <CSEBase>/<AE> can represent “CSE1base/AE1” in structured resource ID format.

- <parameter> represents a value of a oneM2M request/response parameter. For example, <Request ID> can represent “0001” value of the Request ID parameter. Parameter names are case sensitive and in long names as specified in oneM2M TS-0004 [2].

- <ID> represents an AE-ID or CSE-ID in MQTT Topic names.

The value will be given at an interoperability test event.

In oneM2M TS-0010 [5], all oneM2M request/response parameters are carried in the MQTT message payload since it has no message header concept. Therefore, the MQTT message payload needs to be described more than HTTP and CoAP messages to describe those parameters in clause 8. In HTTP and CoAP binding messages, payloads are described as “empty” or “<container> resource to be created” in a very abstract way.

Since the representation can be XML or JSON, payload should be abstract to support XML and JSON. The following example is an XML representation and its abstraction for creating a <container> resource.

| XML payload example for MQTT binding | 1 CSE1Base /CSE1/C_AE1 2001 3 cont1 3 SmartMeter 20141003T112033 |
| --- | --- |
| Abstracted payload example for MQTT binding | op = 1 to = CSE1Base fr = /CSE1/C_AE01 rqi = 3001 ty = 3 name = cont1 rti.rt = 3 pc.cnt.lbl = SmartMeter pc.cnt.et = 20141003T112033 |
| Abstracted payload example for MQTT binding adopting the payload convention | op = 1 to = fr = rqi = ty = 3 name = rti.rt = 3 pc = |

# 6 Test Description Summary

## 6.1 Tests list

Category: Resource management & Single Hop

| Nb | TD ID | TD Description | Procedure/ Resource |
| --- | --- | --- | --- |
| 1 | TD_M2M_NH_01 | AE retrieves the CSEBase resource | CSEBase Management |
| 2 | TD_M2M_NH_02 | Registree CSE registers to Registrar CSE | RemoteCSE |
| 3 | TD_M2M_NH_03 | Registree CSE retrieves RemoteCSE from Registrar CSE | RemoteCSE |
| 4 | TD_M2M_NH_04 | Registree CSE updates RemoteCSE from Registrar CSE | RemoteCSE |
| 5 | TD_M2M_NH_05 | Registree CSE deletes RemoteCSE from Registrar CSE | RemoteCSE |
| 6 | TD_M2M_NH_06 | AE registers to its registrar CSE via an AE Create Request | Application Entity |
| 7 | TD_M2M_NH_07 | AE retrieves <AE> resource via an AE Retrieve Request | Application Entity |
| 8 | TD_M2M_NH_08 | AE updates attribute in <AE> resource via an AE Update Request | Application Entity |
| 9 | TD_M2M_NH_09 | AE de-registers by deleting <AE> resource via an AE Delete Request | Application Entity |
| 10 | TD_M2M_NH_10 | AE creates a container resource in registrar CSE via a container Create Request | Container |
| 11 | TD_M2M_NH_11 | AE retrieves information of a container resource via a container Retrieve Request | Container |
| 12 | TD_M2M_NH_12 | AE updates attribute in application resource via a container Update Request | Container |
| 13 | TD_M2M_NH_13 | AE deletes a specific container resource via a container Delete Request | Container |
| 14 | TD_M2M_NH_14 | AE adds a contentInstance resource <contentInstance> to a specific container in Registrar CSE via a contentInstance Create Request and the registrar CSE updates the parent <container> resource with stateTag, and currentNrOfInstances, CurrentByteSize attributes correspondingly | ContentInstance |
| 15 | TD_M2M_NH_15 | AE retrieves information of a contentInstance resource via a contentInstance Retrieve Request | ContentInstance |
| 16 | TD_M2M_NH_17 | AE deletes contentInstance resource via a Delete Request and the registrar CSE updates the parent <container> resource with currentNrOfInstances, and CurrentByteSize attribute correspondingly | ContentInstance |
| 17 | TD_M2M_NH_49 | AE deletes a <latest> resource in a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> | ContentInstance |
| 18 | TD_M2M_NH_50 | AE deletes a <oldest> resource in a <container> resource and the Registrar CSE points an oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> | ContentInstance |
| 19 | TD_M2M_NH_51 | AE sends a <contentInstance> CREATE request to a <container> which contains attribute currentNrOfInstances whose value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <contentInstance> from the parent <container> and then creates the requested <contentInstance> resource | ContentInstance |
| 20 | TD_M2M_NH_102 | AE retrieves a <latest> resource of a <container> for which attribute locationID is configured, value of locationUpdatePeriod is marked ‘0’ or not defined and locationSource attribute is ‘Network Based’ | ContentInstance |
| 21 | TD_M2M_NH_71 | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> | ContentInstance |
| 22 | TD_M2M_NH_72 | AE retrieves a <oldest> resource of a <container> and the Registrar CSE points a oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> | ContentInstance |
| 23 | TD_M2M_NH_18 | AE discovers resources residing in Registrar CSE | Discovery |
| 24 | TD_M2M_NH_19 | AE discovers accessible resources residing in Registrar CSE using the label filter criteria | Discovery |
| 25 | TD_M2M_NH_20 | AE discovers accessible resources residing in Registrar CSE limiting the number of matching resources to the specified value | Discovery |
| 26 | TD_M2M_NH_21 | AE discovers accessible resources residing in Registrar CSE using multiple Filter Criteria | Discovery |
| 27 | TD_M2M_NH_58 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 1 | Discovery |
| 28 | TD_M2M_NH_59 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 2 | Discovery |
| 29 | TD_M2M_NH_60 | AE1 discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 3 | Discovery |
| 30 | TD_M2M_NH_61 | AE discovers accessible resources residing in Registrar CSE using the offset filter criteria value set to 3 | Discovery |
| 31 | TD_M2M_NH_62 | AE discovers all the accessible resources residing in Registrar CSE using the offset filter criteria | Discovery |
| 32 | TD_M2M_NH_22 | AE creates a subscription to Application Entity resource via subscription Create Request | Subscription |
| 33 | TD_M2M_NH_23 | AE retrieves information about a subscription via subscription Retrieve Request such as expirationTime, labels, etc. | Subscription |
| 34 | TD_M2M_NH_24 | AE updates information about a subscription via subscription Retrieve Request | Subscription |
| 35 | TD_M2M_NH_25 | AE cancels subscription via an subscription Delete Request | Subscription |
| 36 | TD_M2M_NH_26 | AE creates an accessControlPolicy resource | AccessControlPolicy |
| 37 | TD_M2M_NH_27 | AE retrieves accessControlPolicy resource | AccessControlPolicy |
| 38 | TD_M2M_NH_28 | AE updates attribute in accessControlPolicy resource | AccessControlPolicy |
| 39 | TD_M2M_NH_29 | AE deletes accessControlPolicy resource | AccessControlPolicy |
| 40 | TD_M2M_NH_30 | AE delete request is rejected due to accessControlPolicy | AccessControlPolicy |
| 41 | TD_M2M_NH_73 | AE delete request is rejected due to accessControlPolicy (accessControlOriginators) | AccessControlPolicy |
| 42 | TD_M2M_NH_74 | AE delete request is allowed due to accessControlPolicy | AccessControlPolicy |
| 43 | TD_M2M_NH_31 | AE creates a group resource | Group |
| 44 | TD_M2M_NH_32 | AE retrieves group resource | Group |
| 45 | TD_M2M_NH_33 | AE updates attribute in group resource | Group |
| 46 | TD_M2M_NH_34 | AE deletes group resource | Group |
| 47 | TD_M2M_NH_35 | AE creates a node resource | Node |
| 48 | TD_M2M_NH_36 | AE retrieves node resource | Node |
| 49 | TD_M2M_NH_37 | AE updates attribute in node resource | Node |
| 50 | TD_M2M_NH_38 | AE deletes node resource | Node |
| 51 | TD_M2M_NH_39 | AE creates a <pollingChannel> resource in registrar CSE via a Create Request | PollingChannel |
| 52 | TD_M2M_NH_40 | AE retrieves information of a pollingChannel resource via a Retrieve Request | PollingChannel |
| 53 | TD_M2M_NH_41 | AE updates attribute in pollingChannel resource via a Update Request | PollingChannel |
| 54 | TD_M2M_NH_42 | AE deletes a pollingChannel resource via a Delete Request | PollingChannel |
| 55 | TD_M2M_NH_43 | AE retrieves information of a pollingChannel resource via a Retrieve Request | PollingChannel |
| 56 | TD_M2M_NH_44 | AE creates a <contentInstance> resource in each group member | FanoutPoint |
| 57 | TD_M2M_NH_45 | AE retrieves the <container> resource from in each group member | FanoutPoint |
| 58 | TD_M2M_NH_46 | AE updates an <container> resource of each member resource | FanoutPoint |
| 59 | TD_M2M_NH_47 | AE deletes a <container> of each member | FanoutPoint |
| 60 | TD_M2M_NH_48 | AE receives a notification request from the HOST CSE | Notification |
| 61 | TD_M2M_NH_80 | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 | Notification |
| 62 | TD_M2M_NH_81 | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Resource’, Hosting CSE send notification to AE1 | Notification |
| 63 | TD_M2M_NH_82 | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘’Create of Direct Child ’Resource’, Hosting CSE send notification to AE1 | Notification |
| 64 | TD_M2M_NH_83 | AE2 sends DELETE request to the <contentInstance> which located under the subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Direct Child Resource’, Hosting CSE send notification to AE1 | Notification |
| 65 | TD_M2M_NH_89 | AE creates <subscription> resources by sending Create Request to the fanOutPoint. Since AE has set notifyAggregation to 2, Hosting CSE aggregate notification and send aggregated notification to AE | Notification |
| 66 | TD_M2M_NH_52 | AE creates a flexcontainer resource in Registrar CSE via a flexcontainer Create Request | FlexContainer |
| 67 | TD_M2M_NH_53 | AE retrieves information of a flexContainer resource via a flexContainer Retrieve Request | FlexContainer |
| 68 | TD_M2M_NH_54 | AE updates attribute in application resource via a flexContainer Update Request | FlexContainer |
| 69 | TD_M2M_NH_55 | AE deletes a specific container resource via a container Delete Request | FlexContainer |
| 70 | TD_M2M_NH_56 | AE receives a notification request on flexContainer update from the HOST CSE | FlexContainer |
| 71 | TD_M2M_NH_57 | AE discovers accessible resources residing in Registrar CSE using attribute filter criteria which has a customAttribute name and value assigned to it | FlexContainer |
| 72 | TD_M2M_NH_63 | AE creates a mgmtCmd resource | External Management Operations |
| 73 | TD_M2M_NH_64 | AE retrieves mgmtCmd resource | External Management Operations |
| 74 | TD_M2M_NH_65 | AE updates attribute (not with ‘true’ in execEnable attribute) in mgmtCmd resource | External Management Operations |
| 75 | TD_M2M_NH_66 | AE updates attribute (with ‘true’ in execEnable attribute) in mgmtCmd resource | External Management Operations |
| 76 | TD_M2M_NH_67 | AE deletes mgmtCmd resource | External Management Operations |
| 77 | TD_M2M_NH_68 | AE retrieves execInstance resource | External Management Operations |
| 78 | TD_M2M_NH_69 | AE updates attribute ‘execDisable’ to true in execInstance resource to cancel pending management command | External Management Operations |
| 79 | TD_M2M_NH_70 | AE deletes execInstance resource | External Management Operations |
| 80 | TD_M2M_NH_75 | AE creates a SemanticDescriptor resource in Registrar CSE via a SemanticDescriptor Create Request | SemanticDescriptor Management |
| 81 | TD_M2M_NH_76 | AE retrieves information of a semanticDescriptor resource via a semanticDescriptor Retrieve Request | SemanticDescriptor Management |
| 82 | TD_M2M_NH_77 | AE updates attribute in <semanticDescriptor> resource via a semanticDescriptor Update Request | SemanticDescriptor Management |
| 83 | TD_M2M_NH_78 | AE deletes SemanticDescriptor resource via a SemanticDescriptor Delete Request | SemanticDescriptor Management |
| 84 | TD_M2M_NH_79 | AE discovers accessible resources residing in Registrar CSE using the semanticFilter filter criteria | Semantic Resource Discovery |
| 85 | TD_M2M_NH_84 | Check creation of <container> resource with result content set to 0(nothing) | ResultContent |
| 86 | TD_M2M_NH_85 | Check creation of <container> resource with result content set to 1(attributes) | ResultContent |
| 87 | TD_M2M_NH_86 | Check creation of <container> resource with result content set to 2(hierarchical address) | ResultContent |
| 88 | TD_M2M_NH_87 | Check creation of <container> resource with result content set to 3(hierarchical address and attributes) | ResultContent |
| 89 | TD_M2M_NH_88 | Check retrievability of <container> resource with result content set to 4(attributes and child resources) | ResultContent |
| 90 | TD_M2M_NH_90 | AE creates a <timeSeries> resource in registrar CSE via a Create Request | timeSeries |
| 91 | TD_M2M_NH_91 | AE retrieves information of a <timeSeries> resource via a Retrieve Request | timeSeries |
| 92 | TD_M2M_NH_92 | AE updates attribute in <timeSeries> resource via a Update Request | timeSeries |
| 93 | TD_M2M_NH_93 | AE deletes a <timeSeries> resource via a Delete Request | timeSeries |
| 94 | TD_M2M_NH_94 | AE sends Create Request of a <timeSeriesInstance> resource to a <timeSeries> resource in Registrar CSE. Registrar CSE creates the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly | timeSeriesInstance |
| 95 | TD_M2M_NH_95 | AE retrieves information of a <timeSeriesInstance> resource via a Retrieve Request | timeSeriesInstance |
| 96 | TD_M2M_NH_96 | AE sends Delete Request of a <timeSeriesInstance> resource in Registrar CSE. Registrar CSE delete the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly | timeSeriesInstance |
| 97 | TD_M2M_NH_97 | AE sends a <timeSeriesInstance> resource Create Request to a <timeSeries> resource which contains currentNrOfInstances value equals to that of maxNrOfInstances and Registrar CSE delets the oldest <timeSeriesInstance> resource from the <timeSeries> resource and then creates the requested <timeSeriesInstance> resource | timeSeriesInstance |
| 98 | TD_M2M_NH_98 | AE creates a <locationPolicy> resource in registrar CSE via a locationPolicy Create Request | LocationPolicy |
| 99 | TD_M2M_NH_99 | AE retrieves information of a <locationPolicy> resource via a locationPolicy Retrieve Request | LocationPolicy |
| 100 | TD_M2M_NH_100 | AE updates attribute in <locationPolicy> resource via a locationPolicy Update Request | LocationPolicy |
| 101 | TD_M2M_NH_101 | AE deletes a specific <locationPolicy> resource via a locationPolicy Delete Request | LocationPolicy |
| 102 | TD_M2M_NH_155 | AE creates a <schedule> resource in Registrar CSE via a Schedule Create Request | Schedule |
| 103 | TD_M2M_NH_156 | AE retrieves information of a <schedule> resource via a schedule Retrieve Request | Schedule |
| 104 | TD_M2M_NH_157 | AE updates attribute in <schedule> resource via a schedule Update Request | Schedule |
| 105 | TD_M2M_NH_158 | AE deletes Schedule resource via a Schedule Delete Request | Schedule |
| 106 | TD_M2M_NH_159 | CSE sends a notification request to the AE when Schedule resource is configured | Schedule |

Category: Non-Blocking

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 107 | TD_M2M_NB_01 | AE creates a container resource using non-blocking synchronous request in registrar CSE | Synchronous request |
| 108 | TD_M2M_NB_02 | AE retrieves a Container resource using non-blocking synchronous request in registrar CSE | Synchronous request |
| 109 | TD_M2M_NB_03 | AE updates a Container resource using non-blocking synchronous request in registrar CSE | Synchronous request |
| 110 | TD_M2M_NB_04 | AE deletes a Container resource using non-blocking synchronous request | Synchronous request |
| 111 | TD_M2M_NB_05 | AE creates a container resource using non-blocking asynchronous request | Asynchronous request |
| 112 | TD_M2M_NB_06 | AE retrieves a Container resource using non-blocking asynchronous request | Asynchronous request |
| 113 | TD_M2M_NB_07 | AE updates a Container resource using non-blocking asynchronous request | Asynchronous request |
| 114 | TD_M2M_NB_08 | AE deletes a Container resource using non-blocking asynchronous request | Asynchronous request |

Category: Single Hop

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 115 | TD_M2M_SH_01 | AE creates a remote <Resource> resource | Retargeting |
| 116 | TD_M2M_SH_02 | AE retrieves a remote <Resource> resource | Retargeting |
| 117 | TD_M2M_SH_03 | AE updates a remote <Resource> resource | Retargeting |
| 118 | TD_M2M_SH_04 | AE delete a remote <Resource> resource | Retargeting |
| 119 | TD_M2M_SH_09 | AE discovers accessible resources residing in the remote Hosting CSE using multiple Filter Criteria | Discovery |
| 120 | TD_M2M_SH_10 | AE delete request is rejected after access rights verification using retargeting | Unauthorized operation |
| 121 | TD_M2M_SH_11 | AE receives a notification request from the remote hosting CSE | Notification |
| 122 | TD_M2M_SH_05 | AE creates a <mgmtObj> resource | mgmtObj |
| 123 | TD_M2M_SH_06 | AE updates a <mgmtObj> resource | mgmtObj |
| 124 | TD_M2M_SH_07 | AE retrieves a <mgmtObj> resource | mgmtObj |
| 125 | TD_M2M_SH_08 | AE deletes a <mgmtObj> resource | mgmtObj |
| 126 | TD_M2M_SH_12 | AE1 announces itself to CSE2 | Announcement |
| 127 | TD_M2M_SH_13 | AE1 announces a child container to CSE2 | Announcement |
| 128 | TD_M2M_SH_14 | AE1 announces an Optional Announce attribute to CSE2 | Announcement |
| 129 | TD_M2M_SH_15 | AE2 retrieves an Announced Resource | Announcement |
| 130 | TD_M2M_SH_16 | AE2 retrieves the original resource representation of an announced resource | Announcement |
| 131 | TD_M2M_SH_21 | ContainerAnnc Delete by updating announceTo attribute. AE1 deletes its announced child container from CSE2 | Announcement |
| 132 | TD_M2M_SH_22 | ContainerAnnc Delete by deleting original resource. AE1 deletes its announced child container from CSE2 | Announcement |
| 133 | TD_M2M_SH_23 | Announced attribute Create by addition to announcedAttribute attribute AE1 announces an announcable attribute of its child container to CSE2 | Announcement |
| 134 | TD_M2M_SH_24 | Announced attribute Create by creation of a MA attribute at the original resource. AE1 announces an MA attribute of its child container to CSE2 | Announcement |
| 135 | TD_M2M_SH_25 | Announced attribute Delete by deletion from announcedAttribute attribute. AE1 de-announces an announcable attribute of its child container to CSE2 | Announcement |
| 136 | TD_M2M_SH_26 | Announced attribute Delete by deletion of a MA attribute at the original resource. AE1 de-announces an MA attribute (conditionally mandatory) of its child container to CSE2 | Announcement |
| 137 | TD_M2M_SH_17 | AE creates a <contentInstance> resource in each group member, where some memberIDs are on a remoteCSE | fanOut |
| 138 | TD_M2M_SH_18 | AE retrieves a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE | fanOut |
| 139 | TD_M2M_SH_19 | AE updates a <container> resource in each group member, where some memberIDs are on a remoteCSE | fanOut |
| 140 | TD_M2M_SH_20 | AE deletes a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE | fanOut |

Category: Security

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 141 | TD_M2M_SE_01 | AE uses Provisioned Symmetric Key Security Association Establishment Framework to enable mutual authentication with the Registrar CSE. Registrar CSE performs AE authorization check on incoming AE registration request | Secure AE Registratiopn |
| 142 | TD_M2M_SE_02 | AE establishes mutual authentication with the Registrar CSE using Provisioned Symmetric Key Security Association Establishment Framework | Authentication |
| 143 | TD_M2M_SE_03 | AE establishes mutual authentication with the Registrar CSE using Certificate-Based Security Association Establishment Framework | Authentication |
| 144 | TD_M2M_SE_04 | AE accesses <accessControlPolicy> resource using its selfPrivileges credentials | Authorization |
| 145 | TD_M2M_SE_05 | AE accesses <AE> resource using its accessControlPolicyIDs attribute | Authorization |
| 146 | TD_M2M_SE_06 | AE accesses <AE> resource using default access privileges | Authorization |
| 147 | TD_M2M_SE_07 | AE accesses <AE> resource using default access privileges | Authorization |
| 148 | TD_M2M_SE_08 | AE accesses <AE> resource using Direct Dynamic Authorization | Authorization |
| 149 | TD_M2M_SE_09 | AE accesses <AE> resource using Indirect Dynamic Authorization | Authorization |
| 150 | TD_M2M_SE_10 | A MEF Handshake procedure establishes a mutually authenticated TLS session for protecting the communication between an MEF Client and MEF using pre-provisioned certificates | Key Provisioning management |
| 151 | TD_M2M_SE_11 | A MEF Handshake procedure establishes a mutually authenticated TLS or DTLS session for protecting the communication between an MEF Client and MEF using pre-provisioned Master Credentials | Key Provisioning management |
| 152 | TD_M2M_SE_12 | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder | Key Provisioning management |
| 153 | TD_M2M_SE_13 | The MEF Client retrieves MEF Client Configurations provided by the administrating stakeholder to the MEF | Key Provisioning management |
| 154 | TD_M2M_SE_14 | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels | Key Provisioning management |
| 155 | TD_M2M_SE_15 | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder | Key Provisioning management |
| 156 | TD_M2M_SE_16 | Source MEF Client establishes a symmetric key with the MEF which can be retrieved for use by one or more Target MEF Clients | Key Provisioning management |
| 157 | TD_M2M_SE_17 | The Target MEF Client to retrieve the Key Value from a MEF corresponding to a RelativeKeyID received by the Target MEF Client | Key Provisioning management |
| 158 | TD_M2M_SE_18 | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels | Key Provisioning management |
| 159 | TD_M2M_SE_19 | Source MEF Client requests the MEF to stop distributing the registered key | Key Provisioning management |
| 160 | TD_M2M_SE_20 | AE sends an arbitrary request primitive inside of ESPrim Object to CSE | End-to-End security management |
| 161 | TD_M2M_SE_21 | AE establishes a connection with the Registrar CSE using pairwiseE2EKey | End-to-End security management |

Category: HAIM

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 163 | TD_M2M_NH_102 | AE1 creates a HAIM Light Device Model | HAIM Device Model |
| 164 | TD_M2M_NH_103 | AE2 reads the status of a HAIM Light Device Model | HAIM Device Model |
| 165 | TD_M2M_NH_104 | AE2 turns the binarySwitch of a HAIM Light Device Model “ON” or “OFF” | HAIM Device Model |
| 166 | TD_M2M_NH_105 | AE2 toggles the state of a HAIM Light Device Model | HAIM Device Model |
| 167 | TD_M2M_NH_160 | AE1 creates a Power Outlet SubDevice Model | HAIM Device Model |
| 168 | TD_M2M_NH_161 | AE1 creates a Toggle Action Model | HAIM Device Model |
| 169 | TD_M2M_NH_162 | AE1 creates Device Properties Model | HAIM Device Model |

Category: Semantics

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 170 | TD_M2M_NH_106 | ACP triples are created when a new <accessControlPolicy> resource is created | Semantic Access Control Policy |
| 171 | TD_M2M_NH_107 | ACP triples are updated when an existing <accessControlPolicy> resource is updated | Semantic Access Control Policy |
| 172 | TD_M2M_NH_108 | ACP triples are deleted when an existing <accessControlPolicy> resource is deleted | Semantic Access Control Policy |
| 173 | TD_M2M_NH_109 | ACP-SD Binding Triples and SD relationship in SGS are created when AE creates a <semanticDescriptor> resource in Registrar CSE | Semantic Access Control Policy |
| 174 | TD_M2M_NH_110 | ACP-SD Binding Triples are updated when the accessControlPolicyIDs attribute of a <semanticDescriptor> resource is updated | Semantic Access Control Policy |
| 175 | TD_M2M_NH_111 | SD Relationship Triples are updated when the descriptor attribute of a <semanticDescriptor> resource is changed | Semantic Access Control Policy |
| 176 | TD_M2M_NH_112 | SD Relationship Triples are deleted when the descriptor attribute of a <semanticDescriptor> resource is deleted | Semantic Access Control Policy |
| 177 | TD_M2M_NH_113 | AE discovers accessible resources residing in Registrar CSE using the <semanticFanOutPoint> | Semantic Filtering and discovery |
| 178 | TD_M2M_NH_114 | AE discovers accessible resources residing in Registrar CSE using the resource link-based Semantic Discovery | Semantic Filtering and discovery |
| 179 | TD_M2M_NH_115 | AE performs a Semantic Query request in Registrar CSE using the semanticFilter filter criteria | Semantic Filtering and discovery |
| 180 | TD_M2M_NH_116 | AE creates a SemanticMashupJobProfile resource in Registrar CSE via a SemanticMashupJobProfile Create Request | Semantic Mashup |
| 181 | TD_M2M_NH_117 | AE retrieves information of a semanticMashupJobProfile resource via a semanticMashupJobProfile Retrieve Request | Semantic Mashup |
| 182 | TD_M2M_NH_118 | AE updates attribute in <semanticMashupJobProfile> resource via a semanticMashupJobProfile Update Request | Semantic Mashup |
| 183 | TD_M2M_NH_119 | AE deletes semanticMashupJobProfile resource via a semanticMashupJobProfile Delete Request | Semantic Mashup |
| 184 | TD_M2M_NH_120 | AE creates a semanticMashupInstance resource in Registrar CSE via a semanticMashupInstance Create Request | Semantic Mashup |
| 185 | TD_M2M_NH_121 | AE retrieves information of a semanticMashupInstance resource via a semanticMashupInstance Retrieve Request | Semantic Mashup |
| 186 | TD_M2M_NH_122 | AE updates attribute in <semanticMashupInstance> resource via a semanticMashupInstance Update Request | Semantic Mashup |
| 187 | TD_M2M_NH_123 | AE deletes semanticMashupInstance resource via a semanticMashupInstance Delete Request | Semantic Mashup |
| 188 | TD_M2M_NH_124 | AE retrieves information of a semanticMashupResult resource via a semanticMashupResult Retrieve Request | Semantic Mashup |
| 189 | TD_M2M_NH_125 | AE deletes semanticMashupResult resource via a semanticMashupResult Delete Request | Semantic Mashup |
| 190 | TD_M2M_NH_126 | AE triggers a calculation and generation of the mashup result by sending a <semanticMashupInstance>/<mashup> Retrieve Request | Semantic Mashup |
| 191 | TD_M2M_NH_127 | AE creates a OntologyRepository resource in Registrar CSE via a OntologyRepository Create Request | Ontology Repository |
| 192 | TD_M2M_NH_128 | AE retrieves information of a ontologyRepository resource via a ontologyRepository Retrieve Request | Ontology Repository |
| 193 | TD_M2M_NH_129 | AE updates attribute in <ontologyRepository> resource via a ontologyRepository Update Request | Ontology Repository |
| 194 | TD_M2M_NH_130 | AE deletes OntologyRepository resource via a OntologyRepository Delete Request | Ontology Repository |
| 195 | TD_M2M_NH_131 | AE checks the validity of the <semanticDescriptor> resource via a <semanticValidation> Update Request | Semantic Validation |
| 196 | TD_M2M_NH_132 | AE creates a <semanticDescriptor> resource visa SemanticDescriptor Create Request and Registrar CSE checks the validity of the created <semanticDescriptor> resource | Semantic Validation |
| 197 | TD_M2M_NH_133 | AE creates an OntologyMapping resource in Registrar CSE via an OntologyMapping Create Request | Ontology Mapping |
| 198 | TD_M2M_NH_134 | AE retrieves information of an ontology mapping result via a ontologyMapping Retrieve Request | Ontology Mapping |
| 199 | TD_M2M_NH_135 | AE updates attribute in <ontologyMapping> resource via a ontologyMapping Update Request | Ontology Mapping |
| 200 | TD_M2M_NH_136 | AE deletes OntologyMapping resource via a OntologyMapping Delete Request | Ontology Mapping |

Category: 3GPP Interworking

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 201 | TD_M2M_SH_27 | IN-CSE establishes SCEF Configuration for NIDD | Cellular IoT non-IP data delivery (NIDD) |
| 202 | TD_M2M_SH_28 | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE | Cellular IoT non-IP data delivery (NIDD) |
| 203 | TD_M2M_SH_29 | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE | Cellular IoT non-IP data delivery (NIDD) |
| 204 | TD_M2M_SH_30 | IN-AE monitors UE Reachability status | Monitoring events |
| 205 | TD_M2M_SH_31 | UE Availability after DDN Failure scenario | Monitoring events |
| 206 | TD_M2M_SH_32 | UE Communication Failure scenario | Monitoring events |
| 207 | TD_M2M_SH_33 | Roaming status scenario | Monitoring events |
| 208 | TD_M2M_SH_34 | Location Reporting scenario | Monitoring events |
| 209 | TD_M2M_SH_35 | IN-AE triggers ADN-AE hosted on UE | 3GPP Based Device triggering |
| 210 | TD_M2M_SH_36 | IN-AE recalls/replaces a trigger request targeting ADN-AE hosted on UE that has been already created in IN-CSE | 3GPP Based Device triggering |
| 211 | TD_M2M_SH_37 | IN-CSE translates the oneM2M Node Traffic Pattern (TP) into a 3GPP Device Communication Pattern | Configuration of traffic patterns |
| 212 | TD_M2M_SH_38 | IN-AE creates a MBMS Group for handling group related requests | Group message delivery using MBMS |
| 213 | TD_M2M_SH_39 | IN-AE sends a request for accessing member resources to the Group Hosting CSE | Notification Target removal |

Category: Advanced Subscriptions & Notifications

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 214 | TD_M2M_NH_137 | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request | Notification TargetMgmtPolicyRef |
| 215 | TD_M2M_NH_138 | AE creates a notificationTargetMgmtPolicyRef resource in registrar CSE via a notificationTargetMgmtPolicyRef Create Request | Notification TargetMgmtPolicyRef |
| 216 | TD_M2M_NH_139 | AE retrieves notificationTargetMgmtPolicyRef resource from Registrar CSE | Notification TargetMgmtPolicyRef |
| 217 | TD_M2M_NH_140 | AE updates information about a notificationTargetMgmtPolicyRef via notificationTargetMgmtPolicyRef>Update Request | Notification TargetMgmtPolicyRef |
| 218 | TD_M2M_NH_141 | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request | Notification TargetMgmtPolicyRef |
| 219 | TD_M2M_NH_142 | AE creates a notificationTargetPolicy resource in registrar CSE via a notificationTargetPolicy Create Request | NotificationTargetPolicy |
| 220 | TD_M2M_NH_143 | AE retrieves notificationTargetPolicy resource from Registrar CSE | NotificationTargetPolicy |

Category: Modbus Interworking

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 232 | TD_M2M_NH_163 | AE1 creates Device Model for Modbus device | Modbus Thermometer device |
| 233 | TD_M2M_NH_164 | Modbus IPE reads data from Modbus device and updates Registrar CSE with the read data | Modbus Thermometer device |
| 234 | TD_M2M_NH_165 | AE writes data into a Modbus device by updating <flexContainer> resource in Registrar CSE | Modbus Thermometer device |

Category: NoDN Interworking

| Nb | TD ID | TD Description | Procedure/Resource |
| --- | --- | --- | --- |
| 235 | TD_M2M_NH_166 | NoDN IPE reads data from a NoDN device and updates Registrar CSE with the read data | Generic IPE |
| 236 | TD_M2M_NH_167 | AE writes data into a NoDN device by updating <flexContainer> resource in Registrar CSE | Generic IPE |

# 7 Configuration

## 7.1 Test Configuration

### 7.1.1 No hop

#### 7.1.1.1 M2M_CFG_01

The AE manages resources on the registrar CSE (Hosting CSE).

oneM2M entities model

Figure 7.1.1.1-1

#### 7.1.1.2 M2M_CFG_02

oneM2M entities model

Figure 7.1.1.2-1

#### 7.1.1.3 M2M_CFG_10

oneM2M entities model

Figure 7.1.1.3-1

NOTE: For HAIM Model tests, clause 8.5, AE1 represents a native oneM2M device that implements the HAIM models or a combined IPE(AE) plus a non-oneM2M device node (nodn).

### 7.1.2 Single hop

#### 7.1.2.1 M2M_CFG_03

The AE manages resources on the remote CSE.

oneM2M entities model

Figure 7.1.2.1-1

#### 7.1.2.2 M2M_CFG_04

oneM2M entities model

Figure 7.1.2.2-1

#### 7.1.2.3 M2M_CFG_05

oneM2M entities model

Figure 7.1.2.3-1

#### 7.1.2.4 M2M_CFG_08

This configuration concerns group management when the AE is using a group to fan out requests to multiple members. The connection between the AE and the Group Hosting CSE, the Group Hosting CSE and the Member Hosting CSE may be a multi hop connection following the definition in clause 7.1.3.

This configuration is mapped to cases including:

- AE sends a request addressing <group>/fanOutPoint in the Group Hosting CSE, the Group Hosting CSE then further fans out the request to each Member Hosting CSE.

- The Member Hosting CSE sends a notification to the Group Hosting CSE pertaining to the subscription made through the Group Hosting CSE. The Group Hosting CSE then further aggregates the notification and sends it back to the AE.

Figure 7.1.2.4-1

#### 7.1.2.5 M2M_CFG_09

This configuration concerns device management using external technologies.

This configuration is mapped to cases including:

- The AE sends a request addressing <mgmtObj> to IN-CSE. IN-CSE then further acts as a Management Server to send management commands to Managed Entity over the mc interface. The management command is defined in OMA DM, BBF TR069 or LWM2M.

Figure 7.1.2.5-1

#### 7.1.2.6 M2M_CFG_11

This configuration concerns device management using 3GPP network.

#### 7.1.2.7 M2M_CFG_12

This configuration concerns group management when the IN-AE is using a group to fan out requests to multiple members in 3GPP interworking scenraios. The connection between the IN-AE and the Group Hosting CSE, the Group Hosting CSE and the Member Hosting CSE may be a multi hop connection following the definition in clause 7.1.3.

This configuration is mapped to cases including:

- IN-AE sends a request addressing <group>/fanOutPoint in the Group Hosting CSE, the Group Hosting CSE then further fans out the request to each Member Hosting CSE through 3GPP network.

### 7.1.3 Multi hops

#### 7.1.3.1 M2M_CFG_06

oneM2M entities model

Figure 7.1.3.1-1

#### 7.1.3.2 M2M_CFG_07

oneM2M entities model

Figure 7.1.3.2-1

# 8 Test Descriptions

## 8.1 No Hop configuration testing

### 8.1.1 CSEBase Management

#### 8.1.1.1 CSEBase Retrieve on Mca

Interoperability Test Description

| Identifier: | TD_M2M_NH_01 |
| --- | --- |
| Objective: | AE retrieves the CSEBase resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.2 | +——————–+————————————————————————————————————————-+

Pre-test conditions:

- CSEBase resource has been automatically created in CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a retrieve Request to CSE with name {CSEBaseName} |
| 2 | Mca | PRO Check Primitive | • Operation (op) = 2 (Retrieve) • To (to) = Resource-ID of requested <CSEBase> resource, assumed CSE-relative here • From (from) = AE-ID of request originator • Request Identifier (rqi) = (token-string) |
| 3 | Mca | PRO Check Primitive | • Response Status Code (rsc) = 2000 (OK) • Request Identifier (rqi) = same string as received in request message • Content (pc) = Serialized Representation of <CSEBase> resource |
| 4 |  | IOP Check | AE indicates successful operation |

### 8.1.2 RemoteCSE Management

#### 8.1.2.1 RemoteCSE Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_02 |
| --- | --- |
| Objective: | Registree CSE registers to Registrar CSE |
| Configuration: | M2M_CFG_02 |

oneM2M TS-0004 [2], clause 7.3.3.2.1 | +——————–+————————————————————————————————————————–+

Pre-test conditions:

- CSEBase resource has been created in registrar CSE with name {CSEBaseName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE Create request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = Registree CSE-ID • rqi = (token-string) • ty = 16 (RemoteCSE) • pc = Serialized representation of <RemoteCSE> resource |
| 3 | Mcc | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <RemoteCSE> resource |
| 4 |  | IOP Check | Check if possible that the <remoteCSE> resource has been created in registrar CSE |
| 5 |  | IOP Check | Check if possible that the corresponding <remoteCSE> resource has been also created in registree CSE |
| 6 |  | IOP Check | Registree CSE indicates successful operation |

#### 8.1.2.2 remoteCSE Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_03 |
| --- | --- |
| Objective: | Registree CSE retrieves RemoteCSE from Registrar CSE |
| Configuration: | M2M_CFG_02 |

oneM2M TS-0004 [2], clause 7.3.3.2.2 | +——————–+————————————————————————————————————————–+

Pre-test conditions:

- CSEBase resource has been created in registrar CSE with name {CSEBaseName}

- Registree CSE has created a remoteCSE resource on registrar CSE with name {RemoteCSEName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE retrieve request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{remoteCSEName} • fr = Registree CSE-ID • rqi = (token-string) • pc = empty |
| 3 | Mcc | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <RemoteCSE> resource |
| 4 |  | IOP Check | Registree CSE indicates successful operation |

#### 8.1.2.3 remoteCSE Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_04 |
| --- | --- |
| Objective: | Registree CSE updates RemoteCSE from Registrar CSE |
| Configuration: | M2M_CFG_02 |

oneM2M TS-0004 [2], clause 7.3.3.2.3 | +——————–+————————————————————————————————————————–+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

Registree CSE has created a remoteCSE resource on registrar CSE with name {RemoteCSEName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE update request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{remoteCSEName} • fr = Registree CSE-ID • rqi = (token-string) • pc = Serialized representation of updated <RemoteCSE> resource |
| 3 |  | IOP Check | Check if possible that the <remoteCSE> resource has been updated in registrar CSE |
| 4 | Mcc | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <RemoteCSE> resource |
| 5 |  | IOP Check | Registree CSE indicates successful operation |

#### 8.1.2.4 remoteCSE Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_05 |
| --- | --- |
| Objective: | Registree CSE deletes RemoteCSE from Registrar CSE |
| Configuration: | M2M_CFG_02 |

oneM2M TS-0004 [2], clause 7.3.3.2.4 | +——————–+————————————————————————————————————————–+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

Registree CSE has created a remoteCSE resource on registrar CSE with name {RemoteCSEName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE delete request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{remoteCSEName} • fr = Registree CSE-ID • rqi = (token-string) • pc = empty |
| 3 | Mcc | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <remoteCSE> resource has been removed from registrar CSE |
| 5 |  | IOP Check | Check if possible that the <remoteCSE> resource is also removed from registree CSE |
| 6 |  | IOP Check | Registree CSE indicates successful operation |

### 8.1.3 Application Entity Registration

#### 8.1.3.1 AE Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_06 |
| --- | --- |
| Objective: | AE registers to its registrar CSE via an AE Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.5.2.1 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in CSE with name {CSEBaseName}

AE does not have an AE-ID, i.e. it registers from scratch

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a AE Create request to register to the Registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 2 (AE) • pc = Serialized representation of <AE> resource |
| 3 |  | IOP Check | Check if possible that the <AE> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <AE> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.3.2 AE Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_07 |
| --- | --- |
| Objective: | AE retrieves <AE> resource via an AE Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.5.2.2 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}bgf

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a AE retrieve request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE} • fr = AE-ID of request originator • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <AE> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.3.3 AE Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_08 |
| --- | --- |
| Objective: | AE updates attribute in <AE> resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.5.2.3 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an AE Update Request |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{AE} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <AE> resource |
| 3 |  | IOP Check | Check if possible that the <AE> resource has been updated in registrar CSE |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <AE> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.3.4 AE Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_09 |
| --- | --- |
| Objective: | AE de-registers by deleting <AE> resource via an AE Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.5.2.4 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an AE Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{AE} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <AE> resource has been removed from registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.1.4 Container Management

#### 8.1.4.1 Container Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_10 |
| --- | --- |
| Objective: | AE creates a container resource in registrar CSE via a container Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <container> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE> resource • fr = AE-ID • rqi = (token-string) • ty = 3 (Container) • pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Check if possible that the <container> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.4.2 Container Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_11 |
| --- | --- |
| Objective: | AE retrieves information of a container resource via a container Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.2 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <container> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <container> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.4.3 Container Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_12 |
| --- | --- |
| Objective: | AE updates attribute in application resource via a container Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.3 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a container Update Request to update the lifetime of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <container> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the < container > resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.4.4 Container Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_13 |
| --- | --- |
| Objective: | AE deletes a specific container resource via a container Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.4 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a container Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <container> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <container> resource is deleted in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <container> resource has been removed in registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

### 8.1.5 ContentInstance Management

#### 8.1.5.1 ContentInstance Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_14 |
| --- | --- |
| Objective: | AE adds a contentInstance resource <contentInstance> to a specific container in Registrar CSE via a contentInstance Create Request and the Registrar CSE updates the parent <container> resource with stateTag, currentNrOfInstances, and CurrentByteSize attributes correspondingly |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.7.1.1 | +——————–+————————————————————————————————————————————————————————————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

AE has created a container resource <container> on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a RETRIEVE request with resultContent set to 1 (default value) to retrieve the <container> resource and AE sends a request to create a < contentInstance > resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of < container > resource • fr = AE-ID • rqi = (token-string) • ty = 4 (contentInstance) • pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the <contentInstance> resource is created in Registrar CSE and AE sends a RETRIEVE request to the <container> resource to check that if the Registrar CSE has updated stateTag, currentNrOfInstances, and CurrentByteSize attribute correspondingly which is resulted from the successful creation of child <contentInstance> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | AE indicates successful CREATE operation of <contentInstance> and indicates Registrar CSE has updated stateTag, currentNrOfInstances, and CurrentByteSize attribute correspondingly by checking the response of a <container> request to the <container> resource |

IOP Verdict:

Set verdict to pass if IOP check goal is achieved exactly, otherwise verdict fail is set with corresponding error message

#### 8.1.5.2 ContentInstance Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_15 |
| --- | --- |
| Objective: | AE retrieves information of a contentInstance resource via a contentInstance Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.7.2.2 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created a contentInstance resource <contentInstance> as child resource of <container> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <contentInstance> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <contentInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <contentInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.5.3 ContentInstance Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_17 |
| --- | --- |
| Objective: | AE deletes contentInstance resource via a contentInstance Delete Request and the Registrar CSE updates the parent <container> resource with currentNrOfInstances, and CurrentByteSize attributes correspondingly |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.7.2.4 | +——————–+————————————————————————————————————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created a contentInstance resource <contentInstance> as child resource of <container> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a RETRIEVE request with resultContent set to 1 (default value) to retrieve the <container> resource and AE is requested to send a contentInstance Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <contentInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <contentInstance> resource is deleted in Registrar CSE and AE sends a RETRIEVE request to the parent <container> resource to check that if the Registrar CSE has updated currentNrOfInstances, and CurrentByteSize attribute correspondingly which is resulted from the successful deletion of child <contentInstance> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <contentInstance> resource has been removed in registrar CSE. |
| 6 |  | IOP Check | AE indicates successful DELETE operation of <contentInstance> and indicates Registrar CSE has updated currentNrOfInstances, and CurrentByteSize attribute correspondingly |

IOP Verdict:

Set the verdict to pass if both the <contentInstance> is deleted and the Registrar CSE updatedcurrentNrOfInstances, and CurrentByteSize attribute. Otherwise, set the verdict to fail with corresponding error message

#### 8.1.5.4 <latest> ContentInstance Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_49 |
| --- | --- |
| Objective: | AE deletes a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.27.2.4 | +——————–+——————————————————————————————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created more than one contentInstances <contentInstance> as child of <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE retrieves a <latest> resource in a <container> and then sends a DELETE request to the <latest> resource of the <container> |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <latest> resource of a <container> • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = representation of deleted <latest> resource of a <container> |
| 4 |  | IOP Check | AE indicates successful DELETE operation of a <latest> resource and AE sends a RETRIEVE request to <latest> resource of a <container> to check if the retrieved <latest> resource in the <container> is different with that one that was retrieved before DELETE request of the <latest> resource in terms of resourceID and resourceName attribute value |

IOP Verdict:

Set the verdict to pass if IOP check goal is achieved, otherwise set the verdict to fail with corresponding error message

#### 8.1.5.5 <oldest> ContentInstance Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_50 |
| --- | --- |
| Objective: | AE deletes a <oldest> resource of a <container> and the Registrar CSE points an oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.28.2.4 | +——————–+——————————————————————————————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created more than one contentInstances <contentInstance> as child of <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE retrieves a <oldest> resource of a <container> and AE sends a DELETE Request to the <oldest> resource of the <container> |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <oldest> resource of a <container> • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = representation of deleted <oldest> resource of a <container> |
| 4 |  | IOP Check | AE indicates successful DELETE operation of a <oldest> resource and AE sends a RETRIEVE request to <oldest> resource of a <container> to check if the retrieved <oldest> resource in the <container> is different with that one that was retrieved before DELETE request of the <oldest> resource in terms of resourceID and resourceName attribute values |

IOP Verdict:

Set the verdict to pass if IOP check goal is achieved, otherwise set the verdict to fail with corresponding error message

#### 8.1.5.6 ContentInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <container> resource

Interoperability Test Description

| Identifier: | TD_M2M_NH_51 |
| --- | --- |
| Objective: | AE sends a <contentInstance> CREATE request to a <container> which contains attribute currentNrOfInstances whose value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <contentInstance> from the parent <container> and then creates the requested <contentInstance> resource for the originator AE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1 | +——————–+—————————————————————————————————————————————————————————————————————————————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

AE has created a container resource <container> (where the number of contentInstances equals to the value set in maxNrOfInstance) on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a RETRIEVE request with resultContent set to 1 (default value) to retrieve the <oldest> contentInstance resource and AE sends a request to create a <contentInstance> resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <container> resource • fr = AE-ID • rqi = (token-string) • ty = 4 (contentInstance) • pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the <oldest> resource of a <container> is deleted |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | AE indicates successful CREATE operation of <contentInstance> and indicates the representation of the recent <oldest> resource in the <container> is different with that of <oldest> resource retrieved at the beginning of test in terms of resourceID and resourceName attribute value |

IOP Verdict:

Set the verdict to pass if IOP check goal is achieved, otherwise set the verdict to fail with corresponding error message

#### 8.1.5.7 <latest> ContentInstance Retrieve

##### 8.1.5.7.1 Attribute locationID of the <container> resource configured

Interoperability Test Description

| Identifier: | TD_M2M_NH_102 |
| --- | --- |
| Objective: | AE retrieves a <latest> resource of a <container> for which attribute locationID is configured, value of locationUpdatePeriod is marked ‘0’ or not defined and locationSource attribute is ‘Network Based’ |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.27.2.2 | +——————–+————————————————————————————————————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created a <locationPolicy> resource on Registrar CSE having its resourceID and locationContainerID attribute set to locationID and resourceID attribute of the <container> resource respectively

In resource <locationPolicy>, value of locationUpdatePeriod is marked ‘0’ or not defined and locationSource attribute is ‘Network Based’

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <latest> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <container> resource/la • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of latest <contentInstance> resource, created after acquiring location info from Location Server, in response to Location Request made by the CSE, using the attributes stored in the <locationPolicy > resource |
| 4 |  | IOP Check | AE indicates successful operation |

##### 8.1.5.7.2 Attribute locationID of the <container> resource not configured

Interoperability Test Description

| Identifier: | TD_M2M_NH_71 |
| --- | --- |
| Objective: | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.27.2.2 | +——————–+———————————————————————————————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created multiple contentInstance resources <contentInstance> as child resource of <container> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <latest> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <container> resource/la • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of latest <contentInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.5.8 <oldest> ContentInstance Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_72 |
| --- | --- |
| Objective: | AE retrieves a <oldest> resource of a <container> and the Registrar CSE points a oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.28.2.2 | +——————–+———————————————————————————————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created multiple contentInstance resources <contentInstance> as child resource of <container> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <oldest> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <container> resource/ol • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of oldest <contentInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |

### 8.1.6 Discovery

#### 8.1.6.1 Discovery of all resources

Interoperability Test Description

| Identifier: | TD_M2M_NH_18 |
| --- | --- |
| Objective: | AE discovers all accessible resources from registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a discovery request to registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • fu=1 • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing addresses of all discovered resources |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.6.2 Discovery with label filter criteria

Interoperability Test Description

| Identifier: | TD_M2M_NH_19 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using the label filter criteria |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

A <Container> resource with label “key1” is created on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover the <Container> resource using the label filter criteria |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • fu=1 • lbl=key1 • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the address of the <Container> address |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.6.3 Discovery with limit filter criteria

Interoperability Test Description

| Identifier: | TD_M2M_NH_20 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE limiting the number of matching resources to the specified value |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+——————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover at most 2 resources in registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • fu = 1 • lim = 2 • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • cnst = 1 • cno t= 2 • pc = Serialized representation of data object containing the address of the <Container> address |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.6.4 Discovery with multiple filter criteria

Interoperability Test Description

| Identifier: | TD_M2M_NH_21 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using multiple Filter Criteria |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

Two <Container> resources with labels “key1” and “key2” are created in Registrar CSE

A <Group> resources with labels “key1” and “key2” is created in Registrar CSE

Test Sequence

#### 8.1.6.5 Discovery with level filter criteria

Interoperability Test Description

| Identifier: | TD_M2M_NH_58 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 1 |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

<AE1> and <AE2> resources are created in Registrar CSE

A <Container> resource is created under both <AE> resources in Registrar CSE

A <ContentInstance> resource is created under both <Container> resources in Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using level filter criteria value set to 1 |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE1-ID • rqi = (token-string) • fu=1 • lvl=1 • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the address of both <AE> resources |
| 4 |  | IOP Check | AE1 indicates successful operation |

Interoperability Test Description

| Identifier: | TD_M2M_NH_59 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 2 |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

<AE1> and <AE2> resources are created in Registrar CSE. A <Container> resource is created under both <AE> resources in Registrar CSE

A <ContentInstance> resource is created under both <Container> resources in Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using level filter criteria value set to 2 |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE1-ID • rqi = (token-string) • fu = 1 • lvl = 2 • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the address of all <AE> and <Container> resources |
| 4 |  | IOP Check | AE1 indicates successful operation |

Interoperability Test Description

| Identifier: | TD_M2M_NH_60 |
| --- | --- |
| Objective: | AE1 discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 3 |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

<AE1> and <AE2> resources are created in Registrar CSE

A <Container> resource is created under both <AE> resources in Registrar CSE

A <ContentInstance> resource is created under both <Container> resources in Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using level filter criteria value set to 3 |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE1-ID • rqi = (token-string) • fu=1 • lvl=3 • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the address of all <AE>, <Container> and <ContentInstance>resources |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.6.6 Discovery with offset filter criteria

Interoperability Test Description

| Identifier: | TD_M2M_NH_61 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using the offset filter criteria value set to 3 |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

<AE1> and <AE2> resources are created in Registrar CSE. A <Container> resource is created under both <AE> resources in Registrar CSE

A <ContentInstance> resource is created under both <Container> resources in Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using offset filter criteria value set to 3 |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE1-ID • rqi = (token-string) • fu=1 • ofst=3 • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing only 3 of the 6 <AE>, <container> and <contentInstance> resources hosted by the Registrar CSE |
| 4 |  | IOP Check | AE1 indicates successful operation |

Interoperability Test Description

| Identifier: | TD_M2M_NH_62 |
| --- | --- |
| Objective: | AE discovers all the accessible resources residing in Registrar CSE using the offset filter criteria |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+————————————————————————————————————————+

Pre-test conditions:

<AE1> and <AE2> resources are created in Registrar CSE

A <Container> resource is created under both <AE> resources in Registrar CSE

A <ContentInstance> resource is created under both <Container> resources in Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using offset filter criteria attribute value set to 0 (Default value) and limit filter Criteria attribute value set to 2 |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE1-ID • rqi = (token-string) • fu = 1 • lim = 2 • pc = empty |
| 3 |  | IOP Check | Registrar CSE sends success response to AE1 |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • cnst = 1 • cnot = 2 • pc = Serialized representation of data object containing the address of first 2 resources hosted by Registrar CSE |
| 5 |  | IOP Check | AE1 sends discovery request to Registrar CSE with offset filtercriteria value set to 2 and limit filter criteria attribute value set to 2 |
| 6 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE1-ID • rqi = (token-string) • fu=1 • ofst = 2 • lim = 2 • pc = empty |
| 7 |  | IOP Check | Registrar CSE sends success response to AE1 |
| 8 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • cnst=1 • cnot=4 • pc = Serialized representation of data object containing the address of next 2 resources hosted by Registrar CSE |
| 9 |  | IOP Check | AE1 sends discovery request to Registrar CSE with offset filtercriteria value set to 4 and limit filtercriteria attribute value set to 2 |
| 10 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE1-ID • rqi = (token-string) • fu = 1 • ofst = 4 • lim = 2 • pc = empty |
| 11 |  | IOP Check | Registrar CSE sends success response to AE1 |
| 12 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • cnst = 2 • pc = Serialized representation of data object containing the address of last 2 resources hosted by Registrar CSE |
| 13 |  | IOP Check | AE1 indicates successful operation |

### 8.1.7 Subscription Management

#### 8.1.7.1 Subscription Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_22 |
| --- | --- |
| Objective: | AE creates a subscription to Application Entity resource via subscription Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.8.2.1 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

AE has created a container resource <container> on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a subscription Create request to the Registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <Container> resource • fr = AE-ID • rqi = (token-string) • ty = 23 (Subscription) • pc = Serialized representation of <Subscription> resource |
| 3 |  | IOP Check | Check if possible that the <Subscription> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <Subscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.7.2 Subscription Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_23 |
| --- | --- |
| Objective: | AE retrieves subscription resource from Registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.8.2.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created a subscription resource <subscription> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <subscription> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <Subscription> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <Subscription> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.7.3 Subscription Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_24 |
| --- | --- |
| Objective: | AE updates information about a subscription via subscription Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.8.2.3 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created a subscription resource <subscription> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a subscription Update Request to update the lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <Subscription> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <Subscription> resource |
| 3 |  | IOP Check | Check if possible that the <subscription> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <Subscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.7.4 Subscription Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_25 |
| --- | --- |
| Objective: | AE cancels subscription via an subscription Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.8.2.4 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

AE has created a subscription resource <subscription> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a subscription Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <Subscription> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <Subscription> resource is deleted in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <subscription> resource has been removed in registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

### 8.1.8 accessControlPolicy Management

#### 8.1.8.1 accessControlPolicy Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_26 |
| --- | --- |
| Objective: | AE creates an accessControlPolicy resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.2.2.1 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy Create Request |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/{AE} • fr = AE-ID • rqi = (token-string) • ty = 1 (accessControlPolicy) • pc = Serialized representation of <accessControlPolicy > resource |
| 3 |  | IOP Check | Check if possible that the <container> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.8.2 accessControlPolicy Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_27 |
| --- | --- |
| Objective: | AE retrieves accessControlPolicy resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.2.2.2 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a accessControlPolicy retrieve request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{accessControlPolicyName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.8.3 accessControlPolicy Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_28 |
| --- | --- |
| Objective: | AE updates attribute in accessControlPolicy resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.2.2.3 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy update request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{AE}/{accessControlPolicyName} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <accessControlPolicy > resource |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been updated in registrar CSE |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.8.4 accessControlPolicy Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_29 |
| --- | --- |
| Objective: | AE deletes accessControlPolicy resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.2.2.4 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy delete request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{AE}/{accessControlPolicyName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been removed from registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.8.5 Unauthorized operation (Insufficient Access Rights, operations)

Interoperability Test Description

| Identifier: | TD_M2M_NH_30 |
| --- | --- |
| Objective: | AE delete request is rejected due to accessControlPolicy (accessControlOperations) |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0004 [2], clause 7.3.3.15 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}, and accessControlOperations with no delete privilege

AE has created a <container> resource on registrar CSE under <AE>, with name {containerName} and accessControlPolicyIDs including proper identifier of accessControlPolicy resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a container Delete Request for resource <container> |
| 2 | Mca | PRO Check Primitive | •op = 4 (Delete) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <container> resource has not been removed in registrar CSE |
| 5 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |

#### 8.1.8.6 Unauthorized operation (Insufficient Access Rights, originators)

Interoperability Test Description

| Identifier: | TD_M2M_NH_73 |
| --- | --- |
| Objective: | AE delete request is rejected due to accessControlPolicy (accessControlOriginators) |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0004 [2], clause 7.3.3.15 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}, and accessControlOriginators with no privilege for AE

AE has created a <container> resource on registrar CSE under <AE>, with name {containerName} and accessControlPolicyIDs including proper identifier of accessControlPolicy resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a container Delete Request for resource <container> |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <container> resource has not been removed in registrar CSE |
| 5 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |

#### 8.1.8.7 Authorized operation

Interoperability Test Description

| Identifier: | TD_M2M_NH_74 |
| --- | --- |
| Objective: | AE delete request is allowed due to accessControlPolicy |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0004 [2], clause 7.3.3.15 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created a <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}, and accessControlOperations with delete privilege and accessControlOriginators with privilege for AE

AE has created a <container> resource on registrar CSE under <AE>, with name {containerName} and accessControlPolicyIDs including proper identifier of accessControlPolicy resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a container Delete Request for resource <container> |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <container> resource has been removed in registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.1.9 Group Management

#### 8.1.9.1 Group Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_32 |
| --- | --- |
| Objective: | AE retrieves group resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.2.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <group> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a group Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (RETRIEVE) • to = {CSEBaseName}/{group} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <group> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.9.2 Group Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_31 |
| --- | --- |
| Objective: | AE creates a group resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.2.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

void

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a group Create Request |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 9 (group) • pc = Serialized representation of <group> resource |
| 3 |  | IOP Check | Check if possible that the <group> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <group> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.9.3 Group Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_33 |
| --- | --- |
| Objective: | AE updates attribute in group resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.2.4 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <group> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a group Update Request |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{group} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <group> resource |
| 3 |  | IOP Check | Check if possible that the <group> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <group> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.9.4 Group Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_34 |
| --- | --- |
| Objective: | AE deletes group resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.2.5 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <group> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a group Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (DELETE) • to = {CSEBaseName}/{group} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check if possible that the <group> resource is deleted in Registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.1.10 Node Management

#### 8.1.10.1 Node Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_35 |
| --- | --- |
| Objective: | AE creates a node resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.18.2.1 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

void

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a node Create Request |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 14 (node) • pc = Serialized representation of <node> resource |
| 3 |  | IOP Check | Check if possible that the <node> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <node> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.10.2 Node Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_36 |
| --- | --- |
| Objective: | AE retrieves node resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.18.2.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <node> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a node Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (RETRIEVE) • to = {CSEBaseName}/{node} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <node> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.10.3 Node Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_37 |
| --- | --- |
| Objective: | AE updates attribute in node resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.18.2.3 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <node> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a node Update Request |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{node} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <node> resource |
| 3 |  | IOP Check | Check if possible that the <node> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <node> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.10.4 Node Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_38 |
| --- | --- |
| Objective: | AE deletes node resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.18.2.4 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <node> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a node Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (DELETE) • to = {CSEBaseName}/{node} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check if possible that the <node> resource is deleted in Registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.1.11 PollingChannel Management

#### 8.1.11.1 PollingChannel Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_39 |
| --- | --- |
| Objective: | AE creates a <pollingChannel> resource in registrar CSE via a Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.21.2.1 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a < pollingChannel > |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE> resource • fr = AE-ID • rqi = (token-string) • ty = 15 (pollingChannel) • pc = Serialized representation of < pollingChannel > resource |
| 3 |  | IOP Check | Check if possible that the < pollingChannel > resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < pollingChannel > resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.11.2 PollingChannel Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_40 |
| --- | --- |
| Objective: | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.21.2.2 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource < pollingChannel > on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a < pollingChannel > |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of < pollingChannel > resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of < pollingChannel > resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.11.3 pollingChannel Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_41 |
| --- | --- |
| Objective: | AE updates attribute in pollingChannel resource via a Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.21.2.3 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a pollingChannel Update Request to update the lifetime of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of < pollingChannel > resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated < pollingChannel > resource |
| 3 |  | IOP Check | Check if possible that the < pollingChannel > resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of < pollingChannel > resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.11.3 pollingChannel Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_41 |
| --- | --- |
| Objective: | AE updates attribute in pollingChannel resource via a Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.21.2.3 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a pollingChannel Update Request to update the lifetime of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of < pollingChannel > resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated < pollingChannel > resource |
| 3 |  | IOP Check | Check if possible that the < pollingChannel > resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of < pollingChannel > resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.11.4 pollingChannel Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_42 |
| --- | --- |
| Objective: | AE deletes a pollingChannel resource via a Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.21.2.4 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a subscription Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of < pollingChannel > resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the < pollingChannel > resource is deleted in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the < pollingChannel > resource has been removed in registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

#### 8.1.11.5 Long Polling on a PollingChannel Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_43 |
| --- | --- |
| Objective: | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.22.2.2 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

A pollingChannel resource < pollingChannel > has been created in application <AE> on the Registrar CSE

A subscription to a <container> resource has been created using the <pollingChannel> as a notificationURI in the subscription

A single <contentInstance> resource is created in the subscribed to resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a pollingChannelURI Retrieve Request for <pollingChannelURI> |
| 2 | Mca | PRO Check Primitive | Sent RETRIEVE request contains: • To : <CSEBase>/<AE>/<pollingChannel>/pollingChannelURI • Fr : AE-ID |
| 3 | Mca | PRO Check Primitive | Sent RETRIEVE response contains: • To : AE-ID • Fr : CSE-ID • Response Status Code : OK • Cn: pending Notification request |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  |  | Repeat steps 1-2. There is no pending request. When the Request Expiration Timestamp expires Registrar sends response indicating “REQUEST_TIMEOUT” |
| 6 | Mca | PRO Check Primitive | Sent RETRIEVE response contains: • To : AE-ID • Fr : CSE-ID • Response Status Code : REQUEST_TIMEOUT |

### 8.1.12 FanoutPoint Management

#### 8.1.12.1 FanoutPoint Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_44 |
| --- | --- |
| Objective: | AE creates a <contentInstance> resource in each group member |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.3.1 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

A group is created containing 2 members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Create Request to create <contentInstance> in each group member |
| 2 | Check Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) • ty = 4 (contentInstance) • pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the <contentInstance> resource is created in each member hosting CSE |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = aggregated response |
| 5 |  | IOP Check | AE indicates successful operation |

IOP Verdict:

Verify that the aggregate response includes responses from each member of the group

#### 8.1.12.2 FanoutPoint Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_45 |
| --- | --- |
| Objective: | AE retrieves the <container> resource from in each group member |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.3.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

A group is created containing 2 members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request to the fanoutPoint of <group> resource |
| 2 | Check Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) |
| 3 |  | IOP Check |  |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = aggregated response |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  | Verify that the aggregate response includes responses from each member of the group |  |
| PRO Verdict |  |  |  |

IOP Verdict:

Verify that the aggregate response includes responses from each member of the group

#### 8.1.12.3 FanoutPoint Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_46 |
| --- | --- |
| Objective: | AE updates an <container> resource of each member resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.3.3| +——————–+—————————————————————————————————————————+

Pre-test conditions:

A group is created containing 2 members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Update Request to the fanoutPoint of <group> resource to lifetime of the resource |
| 2 | Check Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Check if possible that both of the <container> resources have been updated in registrar CSE |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • pc = aggregated response |
| 5 |  | IOP Check | AE indicates successful operation |

IOP Verdict:

Verify that the aggregate response includes responses from each member of the group

#### 8.1.12.4 FanoutPoint Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_47 |
| --- | --- |
| Objective: | AE deletes a <container> of each member |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.14.3.4| +——————–+—————————————————————————————————————————+

Pre-test conditions:

A group is created containing 2 members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Delete ‘oldest’ Request to the fanoutPoint of <group> resource |
| 2 | Check Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) |
| 3 | Check Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = aggregated response |
| 4 |  | Verify | Check if possible that the oldest <contentInstance> resource has been removed in registrar CSE |
| 5 |  | Verify | AE indicates successful operation |

IOP Verdict:

Verify that the aggregate response includes responses from each member of the group

### 8.1.13 Notification Management

#### 8.1.13.1 Notification

Interoperability Test Description

| Identifier: | TD_M2M_NH_48 |
| --- | --- |
| Objective: | AE receives a notification request from the HOST CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.1 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a container resource <container> on registrar CSE

AE1 has created a <subscription> as a child resource of a <container>

AE2 has created an application resource <AE> on registrar CSE

AE2 has permissions to UPDATE the container created by AE1

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Update request to the <container> created by AE1. This triggers or causes the HOST CSE to send a notification to AE1 |
| 2 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = notificationURI of subscription resource • from = Registrar CSE-ID<br /• >rqi = (token-string) • pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if the notification representation |
| 4 | Check Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE1 indicates notification received |

#### 8.1.13.2 Update Notification

Interoperability Test Description

| Identifier: | TD_M2M_NH_80 |
| --- | --- |
| Objective: | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 |
| Configuration: | M2M_CFG_10 |

oneM2M TS-0004 [2], clause 6.3.4.2.19 | +——————–+———————————————————————————————————————————————————————————————————————————–+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE1 has created a <container> resource on registrar CSE

AE1 has created <subscription> resources under the <container> resource. AE1 has set attribute of eventNotificationCriteria to a specific condition (e.g. maxNrOfInstances in this scenario) and notificationEventType set to 1(Update of Resource)

AE2 has permissions to UPDATE the <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Update Request to the <container>. This triggers or causes the Hosting CSE to send a notification to AE1 |
| 2 | Check Mca | PRO Check Primitive | • op = 3 (Update) • to = ID of <container> resource • from = AE2-ID • rqi = (token-string) • pc = Serialized representation of maxNrOfInstances update |
| 3 |  | IOP Check | Hosting CSE successfully updated mxNrOfInsatnce of the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | AE2 successfully received response of Update request |
| 6 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = ID of <subscription> resource • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notify request which contain notificationEventType equal to 1(Update of Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| 10 | Check Mca | PRO Check Primitive | • op = 3 (Update) • to = ID of <container> resource • from = AE2-ID • rqi = (token-string) • pc = Serialized representation of labels update(can be any other attribute) |
| 11 |  | IOP Check | Hosting CSE successfully updated labels of the <container> resource |
| 12 | Check Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 13 |  | IOP Check | AE2 successfully received response of Update request, notify request will not be sent |

IOP Verdict:

Check that the notification is only sent when there is update of maxNrOfInstances due to setting of the eventNotificationCriteria

#### 8.1.13.3 Delete Notification

Interoperability Test Description

| Identifier: | TD_M2M_NH_81 |
| --- | --- |
| Objective: | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Resource’, Hosting CSE send notification to AE1 |
| Configuration: | M2M_CFG_10 |

oneM2M TS-0004 [2], clause 6.3.4.2.19 | +——————–+—————————————————————————————————————————————————————————————————————————-+

Pre-test conditions:

AE2 has permissions to UPDATE the <container>

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE1 has created a <container> resource on registrar CSE

AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 2(Delete of Resource)

AE2 has permissions to DELETE the <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Delete Request to the <container>. This triggers or causes the Hosting CSE to send a notification to AE1 |
| 2 | Check Mca | PRO Check Primitive | • op = 4 (Delete) • to = ID of <container> resource • fr = AE2-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Hosting CSE successfully deleted the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | AE2 successfully received response of Delete request |
| 6 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = ID of <subscription> resource • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notify request which contain notificationEventType equal to 2(Delete of Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |

#### 8.1.13.4 Creation of Direct Child Resource Notification

Interoperability Test Description

| Identifier: | TD_M2M_NH_82 |
| --- | --- |
| Objective: | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Create of Direct Child Resource’, Hosting CSE send notification to AE1 |
| Configuration: | M2M_CFG_10 |

oneM2M TS-0004 [2], clause 6.3.4.2.19 | +——————–+——————————————————————————————————————————————————————————————————————————————————-+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE1 has created a <container> resource on registrar CSE

AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 3(Create of Direct Child Resource)

AE2 has permissions to CREATE child resource under the <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Create Request of <contentInstance> resource to the <container> resource. This triggers or causes the Hosting CSE to send a notification to AE1 |
| 2 | Check Mca | PRO Check Primitive | • op = 1 (Create) • to = ID of <container> resource • fr = AE2-ID • rqi = (token-string) • ty = 4 (contentInstance) • pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <contentInstance> resource under the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | AE2 successfully received response of Create request |
| 6 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = ID of <subscription> resource • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notify request which contain notificationEventType equal to 3(Create of Direct Child Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |

#### 8.1.13.5 Deletion of Direct Child Resource Notification

Interoperability Test Description

| Identifier: | TD_M2M_NH_83 |
| --- | --- |
| Objective: | AE2 sends DELETE request to the <contentInstance> which is located under the subscribed-to resource. Since <subscription> resource has notificationEventType with ‘Delete of Direct Child Resource’, Hosting CSE send notification to AE1 |
| Configuration: | M2M_CFG_10 |

oneM2M TS-0004 [2], clause 6.3.4.2.19 | +——————–+————————————————————————————————————————————————————————————————————————————————-+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE1 has created a <container> resource on registrar CSE

AE1 has created a <contentInstance> as a child resource of <container> created by AE1

AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 4(Delete of Direct Child Resource)

AE2 has permissions to DELETE the <contentInstance>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Delete Request to the <contentinstance>. This triggers or causes the Hosting CSE to send a notification to AE1 |
| 2 | Check Mca | PRO Check Primitive | • op = 4 (Delete) • to = ID of <contentInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Hosting CSE successfully deleted the <contentInstance> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | AE2 successfully received response of Delete request |
| 6 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = ID of <subscription> resource • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notify request which contain notificationEventType equal to 4(Delete of Direct Child Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Check Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |

#### 8.1.13.6 Notifcation Aggregation

Interoperability Test Description

| Identifier: | TD_M2M_NH_89 |
| --- | --- |

Since AE has set notifyAggregation to 2, Hosting CSE aggregate notification and send aggregated notification to AE | +——————–+————————————————————————————————————————————————————————————————————–+ | Configuration: | M2M_CFG_01 | +——————–+————————————————————————————————————————————————————————————————————–+ | References: | oneM2M TS-0001 [1], clause 10.2.7.11
oneM2M TS-0004 [2], clause 7.4.14.2.4 | +——————–+————————————————————————————————————————————————————————————————————–+

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created two <container> resources on registrar CSE

AE has created a <group> resource with memberIDs set to two <container> resources. AE has set number in notifyAggregation to 2

AE has created <subscription> resources under the members of <group> resources by sending Create Request to the fanOutPoint. AE has set notificationForwardingURI and notificationEventType set to 3(Create of Direct Child Resource)

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Create Request of <contentInstance> resource to the fanOutPoint |
| 2 | Check Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) • ty = 4 (contentInstance) • pc = Serialized representation of <subscription> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <contentInstance> resources in each member |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = aggregated response |
| 5 |  | IOP Check | AE successfully received converged response |
| 6 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = ID of <subscription> resource • from = Registrar CSE-ID • rqi = (token-string) • pc = aggregated Notify request which contain the occurrence of child resource creation |
| 7 |  | IOP Check | AE successfully received aggregated Notify request |
| 8 | Check Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Group Hosting CSE successfully received response of Notify request |

IOP Verdict:

Check that the response is aggregated by the group Hosting CSE and successfully parse to AE

### 8.1.14 FlexContainer Management

#### 8.1.14.1 FlexContainer Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_52 |
| --- | --- |
| Objective: | AE creates a flexContainer resource in Registrar CSE via a flexContainer Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.37.2.1 | +——————–+————————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <flexContainer> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.14.2 FlexContainer Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_53 |
| --- | --- |
| Objective: | AE retrieves information of a flexContainer resource via a flexContainer Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.37.2.2 | +——————–+————————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a flexContainer resource <flexContainer> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <flexContainer> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <flexContainer> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.14.3 FlexContainer Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_54 |
| --- | --- |
| Objective: | AE updates attribute in application resource via a flexContainer Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.37.2.3 | +——————–+————————————————————————————————————————————–+

| Pre-testconditions: |  |  | - AE has created an Application Entity resource on Registrar CSE - AE has created a flexContainer resource on Registrar CSE |  |
| --- | --- | --- | --- | --- |
| Test Sequence |  |  |  |  |
| Step | RP | Type |  | Description |
| 1 |  | Stimulus |  | AE is requested to send a flexContainer Update request to update any customAttribute of the resource |
| 2 | Mca | PRO Check Primitive |  | - op = 3 (Update) - to = {CSEBaseName}/URI of resource - fr = AE-ID - rqi = (token-string) - pc = Serialized representation of updated resource |
| 3 |  | IOP Check |  | Check that the resource is updated in the Registrar CSE |
| 4 | Mca | PRO Check Primitive |  | - rsc = 2004 (Updated) - rqi = (token-string) same as received in request message - pc = Serialized representation of resource |
| 5 |  | IOP Check |  | AE indicates successful operation |

#### 8.1.14.4 FlexContainer Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_55 |
| --- | --- |
| Objective: | AE deletes a specific container resource via a container Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.37.2.4 | +——————–+————————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a flexContainer resource <flexContainer> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a flexContainer Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <flexContainer> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <flexContainer> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

#### 8.1.14.5 Notification Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_56 |
| --- | --- |
| Objective: | AE receives a notification request on flexContainer update from the HOST CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.1 | +——————–+———————————————————————————————————————–+

Pre-test conditions:

AE1 has created an application resource <AE> on Registrar CSE

AE1 has created a flexContainer resource <flexContainer> on Registrar CSE

AE1 has created a <subscription> as a child resource of a <flexContainer>

AE2 has created an application resource <AE> on Registrar CSE

AE2 has permissions to UPDATE customAttributes of flexContainer

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a update request to <flexContainer> for updating customAttribute. This triggers or causes the HOST CSE to send a notification to AE1 |
| 2 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = notificationURI of subscription resource • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if the notification representation |
| 4 | Check Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE1 indicates notification received |

#### 8.1.14.6 Discovery with attribute filter criteria over customAttributes

Interoperability Test Description

| Identifier: | TD_M2M_NH_57 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using attribute filter criteria which has a customAttribute name and value assigned to it. |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.14 | +——————–+——————————————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a flexContainer resource <flexContainer> on Registrar CSE with customAttribute set to a specific value “x”, created on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover the <Container> resource using attribute filter criteria |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • fu=1 • atr=<nm>,<val> • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the address of the <flexContainer> address |
| 4 |  | IOP Check | AE indicates successful operation |

### 8.1.15 External Management Operations Management

#### 8.1.15.1 mgmtCmd Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_63 |
| --- | --- |
| Objective: | AE creates a mgmtCmd resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.16.2.1 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Create Request |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 12 (mgmtCmd) • pc = Serialized representation of <mgmtCmd> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtCmd> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <mgmtCmd> resource |
| 5 |  | IOP Check | AE indicates successful operation |

IOP Verdict:

Set verdict to pass if IOP check goal is achieved exactly, otherwise verdict fail is set with corresponding error message.

#### 8.1.15.2 mgmtCmd Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_64 |
| --- | --- |
| Objective: | AE retrieves mgmtCmd resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.16.2.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (RETRIEVE) • to = {CSEBaseName}/{mgmtCmd} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <mgmtCmd> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.15.3 mgmtCmd Update (Normal)

Interoperability Test Description

| Identifier: | TD_M2M_NH_65 |
| --- | --- |
| Objective: | AE updates attribute (not with ‘true’ in execEnable attribute) in mgmtCmd resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.16.2.3.1 | +——————–+————————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Update Request |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{mgmtCmd} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <mgmtCmd> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtCmd> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <mgmtCmd> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.15.4 mgmtCmd Update (Execute)

Interoperability Test Description

| Identifier: | TD_M2M_NH_66 |
| --- | --- |
| Objective: | AE updates attribute (with ‘true’ in execEnable attribute) in mgmtCmd resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.16.2.3.2 | +——————–+———————————————————————————————————————————–+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Update Request |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{mgmtCmd} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <mgmtCmd> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtCmd> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <mgmtCmd> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.15.5 mgmtCmd Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_67 |
| --- | --- |
| Objective: | AE deletes mgmtCmd resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.16.2.4 | +——————–+——————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (DELETE) • to = {CSEBaseName}/{mgmtCmd} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check if possible that the <mgmtCmd> resource is deleted in Registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.15.6 execInstance Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_68 |
| --- | --- |
| Objective: | AE retrieves execInstance resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.17.2.2| +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE

AE has executed the mgmtCmd resource <mgmtCmd> on Registrar CSE (update execEnable attribute with ‘true’)

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a execInstance Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (RETRIEVE) • to = {CSEBaseName}/{mgmtCmd}/{execInstance} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <execInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.15.7 execInstance Update (Cancel)

Interoperability Test Description

| Identifier: | TD_M2M_NH_69 |
| --- | --- |
| Objective: | AE updates attribute ‘execDisable’ to true in execInstance resource to cancel pending management command. |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.17.2.1 | +——————–+——————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE

AE has executed the mgmtCmd resource <mgmtCmd> on Registrar CSE (update execEnable attribute with ‘true’)

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a execInstance Update Request |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{mgmtCmd}/{execInstance} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <execInstance> resource |
| 3 |  | IOP Check | Check if possible that the <execInstance> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <execInstance> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.15.8 execInstance Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_70 |
| --- | --- |
| Objective: | AE deletes execInstance resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.17.2.3| +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a node resource <node> on Registrar CSE

AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE

AE has executed the mgmtCmd resource <mgmtCmd> on Registrar CSE (update execEnable attribute with ‘true’)

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an execInstance Delete request |
| 2 | Mca | PRO Check Primitive | • op = 4 (DELETE) • to = {CSEBaseName}/{mgmtCmd}/{execInstance} • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check that the <execInstance> resource is deleted in the Registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.1.16 SemanticDescriptor Management

#### 8.1.16.1 SemanticDescriptor Create

Interoperability Test Description +——————–+—————————————————————————————————————————+ | Identifier: | TD_M2M_NH_75 | +——————–+—————————————————————————————————————————+ | Objective: | AE creates a SemanticDescriptor resource in Registrar CSE via a SemanticDescriptor Create Request | +——————–+—————————————————————————————————————————+ | Configuration: | M2M_CFG_01 | +——————–+—————————————————————————————————————————+ | References: | oneM2M TS-0034 [13], clause 6.1.2
oneM2M TS-0004 [2], clause 7.4.34.2.1 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a container resource <container> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <semanticDescriptor> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of < container > resource • fr = AE-ID • rqi = (token-string) • ty = 24 (semanticDescriptor) • pc = Serialized representation of <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.16.2 SemanticDescriptor Retrieve

Interoperabili1ty Test Description +——————–+—————————————————————————————————————————+ | Identifier: | TD_M2M_NH_76 | +——————–+—————————————————————————————————————————+ | Objective: | AE retrieves information of a semanticDescriptor resource via a semanticDescriptor Retrieve Request | +——————–+—————————————————————————————————————————+ | Configuration: | M2M_CFG_01 | +——————–+—————————————————————————————————————————+ | References: | oneM2M TS-0034 [13], clause 6.1.3
oneM2M TS-0004 [2], clause 7.4.34.2.2 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticDescriptor> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <semanticDescriptor> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticDescriptor> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.16.3 SemanticDescriptor Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_77 |
| --- | --- |
| Objective: | AE updates attribute in <semanticDescriptor> resource via a semanticDescriptor Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.34.2.3 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Update Request to update the descriptor attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <semanticDescriptor> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.16.4 SemanticDescriptor Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_78 |
| --- | --- |
| Objective: | AE deletes SemanticDescriptor resource via a SemanticDescriptor Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.34.2.4 | +——————–+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a semanticDescriptor resource <semanticDescriptor> as child of <AE> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <semanticDescriptor> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticDescriptor> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation. |

### 8.1.17 Semantic Resource Discovery

#### 8.1.17.1 Discovery with semanticFilter filter criteria

Interoperability Test Description

| Identifier: | TD_M2M_NH_79 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using the semanticFilter filter criteria |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.18 | +——————–+———————————————————————————————————————–+

Pre-test conditions:

AE1 has created an application resource <AE> on Registrar CSE

AE1 has created a container resource <container> on Registrar CSE

AE1 has created a <semanticDescriptor> as a child resource of a <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request to discover the <container> resource using the semanticFilter filterCriteria |
| 2 | Check Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • from = AE-ID • rqi = (token-string) • fu=1 • smf=sparqlQuery1 • pc = empty |
| 3 | Check Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the <Container> address |
| 4 |  | IOP Check | AE1 indicates notification received |

### 8.1.18 ResultContent

#### 8.1.18.1 ResultContent=0

Interoperability Test Description

| Identifier: | TD_M2M_NH_84 |
| --- | --- |
| Objective: | Check creation of <container> resource with result content set to 0(nothing) |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 6.3.4.2.7 | +——————–+————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in CSE with name {CSEBaseName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a <container> resource Create Request to the Registrar CSE with rcn=0 |
| 2 | Check Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 3 (container) • rcn = 0 • pc = Serialized representation of <container> resource which contain {containerName} as a rn |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE successfully received response of Create request |

IOP Verdict:

Check that the response body does not include any message

#### 8.1.18.2 ResultContent=1

Interoperability Test Description

| Identifier: | TD_M2M_NH_85 |
| --- | --- |
| Objective: | Check creation of <container> resource with result content set to 1(attributes) |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 6.3.4.2.7 | +——————–+————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in CSE with name {CSEBaseName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a <container> resource Create Request to the Registrar CSE with rcn=1 |
| 2 | Check Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 3 (container) • rcn = 1 • pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource which contain attributes |
| 5 |  | IOP Check | AE successfully received response of Create request |

IOP Verdict:

Check that the response body include attributes

#### 8.1.18.3 ResultContent=2

Interoperability Test Description

| Identifier: | TD_M2M_NH_86 |
| --- | --- |
| Objective: | Check creation of <container> resource with result content set to 2(hierarchical address) |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 6.3.4.2.7 | +——————–+————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in CSE with name {CSEBaseName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a <container> resource Create Request to the Registrar CSE with rcn = 2 |
| 2 | Check Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 3 (container) • rcn = 2 • pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Hierarchical address of <container> resource |
| 5 |  | IOP Check | AE successfully received response of Create request |

IOP Verdict:

Check that the response body include hierarchical address

#### 8.1.18.4 ResultContent=3

Interoperability Test Description

| Identifier: | TD_M2M_NH_87 |
| --- | --- |
| Objective: | Check creation of <container> resource with result content set to 3(hierarchical address and attributes) |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 6.3.4.2.7 | +——————–+————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in CSE with name {CSEBaseName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a <container> Create Request to the Registrar CSE with rcn=3 |
| 2 | Check Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 3 (container) • rcn = 3 • pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource with hierarchical address |
| 5 |  | IOP Check | AE successfully received response of Create request |

IOP Verdict:

Check that the response body include hierarchical address and attributes

#### 8.1.18.5 ResultContent=4

Interoperability Test Description

| Identifier: | TD_M2M_NH_88 |
| --- | --- |
| Objective: | Check retrievability of <container> resource with result content set to 4 (attributes and child resources) |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 6.3.4.2.7 | +——————–+————————————————————————————————————————+

Pre-test conditions:

CSEBase resource has been created in CSE with name {CSEBaseName}

AE has created a <container> resource on registrar CSE

AE has created two <contentInstance> resources under the <container> resources

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a <container> Retrieve Request to the Registrar CSE with rcn=4 |
| 2 | Check Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = ID of <container> resource • fr = AE-ID • rqi = (token-string) • rcn = 4 |
| 3 |  | IOP Check | Hosting CSE successfully received Retrieve request of the <container> resource |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> and child resources |
| 5 |  | IOP Check | AE successfully received response of Retrieve request |

IOP Verdict:

Check that the response body include attributes and child resources

### 8.1.19 timeSeries Management

#### 8.1.19.1 timeSeries Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_90 |
| --- | --- |
| Objective: | AE creates a <timeSeries> resource in registrar CSE via a Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.38.2.1| +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE> resource • fr = AE-ID • qi = (token-string) • ty = 29 (timeSeries) • pc = Serialized representation of <timeSeries> resource |
| 3 |  | IOP Check | Registrar CSE successfully created the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |

#### 8.1.19.2 timeSeries Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_91 |
| --- | --- |
| Objective: | AE retrieves information of a <timeSeries> resource via a Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.38.2.2| +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

AE has created a <timeSereis> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <timeSeries> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully received Retrieve Request of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Retrieve Request |

#### 8.1.19.3 timeSeries Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_92 |
| --- | --- |
| Objective: | AE updates attribute in <timeSeries> resource via a Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.38.2.3| +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

AE has created a <timeSereis> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Update Request to the <timeSeries> resource to update the maxNrOfInstances |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <timeSeries> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of maxNrOfInstances update(can be any other attribute) |
| 3 |  | IOP Check | Registrar CSE successfully updated the maxNrOfInstances of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Update Request |

#### 8.1.19.4 timeSeries Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_93 |
| --- | --- |
| Objective: | AE deletes a <timeSeries> resource via a Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.38.2.4| +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

AE has created a <timeSeries> resource on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Delete Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <timeSeries> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully deleted the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | AE successfully received response of Delete Request |

### 8.1.20 timeSeriesInstance Management

#### 8.1.20.1 timeSeriesInstance Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_94 |
| --- | --- |
| Objective: | AE sends Create Request of a <timeSeriesInstance> resource to a <timeSeries> resource in Registrar CSE. Registrar CSE creates the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.39.2.1 | +——————–+——————————————————————————————————————————————————————————————————————————————————————————————————+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

AE has created a <timeSeries> resource on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <timeSeries> resource • fr = AE-ID • rqi = (token-string) • ty = 30 (timeSeriesInstance) • pc = Serialized representation of <timeSeriesInstance> resource |
| 3 |  | IOP Check | Registrar CSE successfully created <timeSeriesInstance> resource and updated currentNrOfInstances and currentByteSize of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |

#### 8.1.20.2 timeSeriesInstance Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_95 |
| --- | --- |
| Objective: | AE retrieves information of a <timeSeriesInstance> resource via a Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.39.2.2 | +——————–+——————————————————————————————————————————+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

AE has created a <timeSeries> resource on registrar CSE

AE has created a <timeSeriesInstance> resource as child resource of the <timeSeries> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <timeSeriesInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully received Retrieve Request of the <timeSeriesInstance> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Retrieve Request |

#### 8.1.20.3 timeSeriesInstance Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_96 |
| --- | --- |
| Objective: | AE sends Delete Request of a <timeSeriesInstance> resource in Registrar CSE. Registrar CSE delete the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.39.2.4 | +——————–+—————————————————————————————————————————————————————————————————————————————————————————–+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

AE has created a <timeSeries> resource on registrar CSE

AE has created a <timeSeriesInstance> resource as child resource of the <timeSeries> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Delete Requeest a <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <timeSeriesInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully deleted <timeSeriesInstance> resource and updated currentNrOfInstances and currentByteSize of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | AE successfully received response of Delete Request |

#### 8.1.20.4 timeSeriesInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <timeSeries> resource

Interoperability Test Description

| Identifier: | TD_M2M_NH_97 |
| --- | --- |
| Objective: | AE sends a <timeSeriesInstance> resource Create Request to a <timeSeries> resource which contains currentNrOfInstances value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <timeSeriesInstance> resource from the <timeSeries> resource and then creates the requested <timeSeriesInstance> resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.38.2.1 | +——————–+—————————————————————————————————————————————————————————————————————————————————————————————————————————————————–+

Pre-test conditions:

AE has created a <AE> resource on Registrar CSE

AE has created a <timeSeries> resource on registrar CSE

AE has created several <timeSeriesInstance> resources and currentNrOfInstances of the <timeSeries> resource reach the maxNrOfInstances

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <timeSeries> resource • fr = AE-ID • rqi = (token-string) • ty = 30 (timeSeriesInstance) • pc = Serialized representation of <timeSeriesInstance> resource |
| 3 |  | IOP Check | Registrar CSE successfully deleted <timeSeriesInstance> resource with the oldest dataGenerationTime and created <timeSeriesInstance> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |

### 8.1.21 Location Management

#### 8.1.21.1 LocationPolicy Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_98 |
| --- | --- |
| Objective: | AE creates a <locationPolicy> resource in registrar CSE via a locationPolicy Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.10.2.1 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <locationPolicy> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 10 (LocationPolicy) • pc = Serialized representation of <locationPolicy> resource |
| 3 |  | IOP Check | Check if possible that the <locationPolicy> resource is created in registrar CSE Check if possible that the <container> resource is created on registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <locationPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.21.2 LocationPolicy Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_99 |
| --- | --- |
| Objective: | AE retrieves information of a <locationPolicy> resource via a locationPolicy Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.10.2.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a locationPolicy resource <locationPolicy> on Registrar CSE

AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <locationPolicy> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <locationPolicy> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <locationPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.21.3 LocationPolicy Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_100 |
| --- | --- |
| Objective: | AE updates attribute in <locationPolicy> resource via a locationPolicy Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.10.2.3 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a locationPolicy resource <locationPolicy> on Registrar CSE

AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a locationPolicy Update Request to update the lifetime of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <locationPolicy> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <locationPolicy> resource |
| 3 |  | IOP Check | Check if possible that the < locationPolicy > resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <locationPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.21.4 LocationPolicy Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_101 |
| --- | --- |
| Objective: | AE deletes a specific <locationPolicy> resource via a locationPolicy Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.10.2.4 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a locationPolicy resource <locationPolicy> on Registrar CSE

AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a locationPolicy Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <locationPolicy> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <locationPolicy> resource is deleted in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <locationPolicy> resource has been removed in registrar CSE |
| 6 |  | IOP Check | Check if possible that the associated resources (e.g. <container>, <contentInstance> resources) are removed from the registrar CSE |
| 7 |  | IOP Check | Check if possible that if the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource were set with appropriate value, the session with underlying network are torn down |
| 8 |  | IOP Check | AE indicates successful operation |

### 8.1.22 Schedule management

#### 8.1.22.1 Schedule Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_155 |
| --- | --- |
| Objective: | AE creates a <schedule> resource in Registrar CSE via a Schedule Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.9.2.1 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <schedule> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 18 (schedule) • pc = Serialized representation of <schedule> resource |
| 3 |  | IOP Check | Check if possible that the <schedule> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <schedule> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.22.2 Schedule Retrieve

Interoperability Test Description +——————–+—————————————————————————————————————————-+ | Identifier: | TD_M2M_NH_156 | +——————–+—————————————————————————————————————————-+ | Objective: | AE retrieves information of a <schedule> resource via a schedule Retrieve Request | +——————–+—————————————————————————————————————————-+ | Configuration: | M2M_CFG_01 | +——————–+—————————————————————————————————————————-+ | References: | oneM2M TS-0001 [1], clause 10.2.17.3
oneM2M TS-0004 [2], clause 7.4.9.2.2 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a schedule resource <schedule> as child resource of <CSEBase> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <schedule> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <schedule> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <schedule> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.1.22.3 Schedule Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_157 |
| --- | --- |
| Objective: | AE updates attribute in <schedule> resource via a schedule Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.9.2.3 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a schedule resource <schedule> as child resource of <AE> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a schedule Update Request to update an attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <schedule> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <schedule> resource |
| 3 |  | IOP Check | Check if possible that the <schedule> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <schedule> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.1.22.4 Schedule Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_158 |
| --- | --- |
| Objective: | AE deletes <schedule> resource via a Schedule Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.9.2.4 | +——————–+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <schedule> resource as child of <AE> resource

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a schedule Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <schedule> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <schedule> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <schedule> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

#### 8.1.22.5 Notification to AE with configured Schedule resource

Interoperability Test Description +——————–+—————————————————————————————————————————–+ | Identifier: | TD_M2M_NH_159 | +——————–+—————————————————————————————————————————–+ | Objective: | CSE sends a notification request to the AE when <schedule> resource is configured | +——————–+—————————————————————————————————————————–+ | Configuration: | M2M_CFG_10 | +——————–+—————————————————————————————————————————–+ | References: | oneM2M TS-0001 [1], clause 9.6.9, 10.2.12
oneM2M TS-0004 [2], clause 7.4.1 | +——————–+—————————————————————————————————————————–+

Pre-test conditions:

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a container resource <container> on registrar CSE

AE1 has created a <subscription> as a child resource of a <container>

AE1 has created a <schedule> as a child resource of a <subscription>

AE2 has created an application resource <AE> on registrar CSE

AE2 has permissions to UPDATE the container created by AE1

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Update request to the <container> created by AE1 |
| 2 | Check Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <container> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | The HOST CSE prepares a notification to AE1 and sends it within the time window indicated in scheduleElement attribute of <schedule> resource |
| 4 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = notificationURI of subscription resource • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 5 | Check Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 6 |  | IOP Check | Check that AE1 has received the notification within the time window indicated in scheduleElement attribute of <schedule> resource |

## 8.2 Non-blocking configuration testing

### 8.2.1 Synchronous request

#### 8.2.1.1 Container management

##### 8.2.1.1.1 Container Create

Interoperability Test Description

| Identifier: | TD_M2M_NB_01 |
| --- | --- |
| Objective: | AE creates a <Container> resource using non-blocking synchronous request in registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a non-blocking synchronous request to create a <Container> resource in registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 1 (Create) • to = {CSEBaseName} • fr= AE-ID • rqi = (token-string) • rt = 1 (non-blocking synchronous) • ty = 3 (container) • pc = Serialized Representation of the <Container> resource |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: • rsc = 1000 (Accepted) • rqi = token-string) same as received in request message • pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to wait then send a retrieve request to <Request> reference |
| 6 | Mca | PRO Check Primitive | Sent Retrieve request contains: • op = 2 (Retrieve) • to = <Request> reference • fr = AE-ID • rqi = (token-string) • pc = empty |
| 7 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = <Request> resource with the parameter “requestStatus” set to 1 (COMPLETED) and the “operationResult” parameter containing the <Container> resource |
| 8 |  | IOP Check | AE indicates successful operation |

##### 8.2.1.1.2 Container Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NB_02 |
| --- | --- |
| Objective: | AE retrieves a <container> resource using non-blocking synchronous request from registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

AE has created a <Container> resource in registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a non-blocking synchronous request to retrieve the <Container> resource from registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <container> resource • fr= AE-ID • rqi = (token-string) • rt = 1 (non-blocking synchronous) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: • rsc = 1000 (Accepted) • rqi = token-string) same as received in request message • pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to send a retrieve request to <Request> reference |
| 6 | Mca | PRO Check Primitive | Sent Retrieve request contains: • op = 2 (Retrieve) • to = <Request> reference • fr = AE-ID • rqi = (token-string) • pc = empty |
| 7 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = <Request> resource with the parameter “requestStatus” set to 1 (COMPLETED) and the “operationResult” parameter containing the <Container> resource |
| 8 |  | IOP Check | AE indicates successful operation |

##### 8.2.1.1.3 Container Update

Interoperability Test Description

| Identifier: | TD_M2M_NB_03 |
| --- | --- |
| Objective: | AE updates a <Container> resource using non-blocking synchronous request in registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

AE has created a <Container> resource in registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a non-blocking synchronous request to update the <Container> resource |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 3 (Update) • to = {CSEBaseName}/URI of <Container> resource • fr= AE-ID • rqi = (token-string) • rt = 1 (non-blocking synchronous) • pc = Serialized Representation of the updated <Container> resource |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: • rsc = 1000 (Accepted) • rqi = token-string) same as received in request message • pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to wait then send a retrieve request to <Request> reference |
| 6 | Mca | PRO Check Primitive | Sent Retrieve request contains: • op = 2 (Retrieve) • to = <Request> reference • fr = AE-ID • rqi = (token-string) • pc = empty |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = <Request> resource with the parameter “requestStatus” set to 1 (COMPLETED) and the “operationResult” parameter containing the <Container> resource |
| 8 |  | IOP Check | AE indicates successful operation |

##### 8.2.1.1.4 Container Delete

Interoperability Test Description

| Identifier: | TD_M2M_NB_04 |
| --- | --- |
| Objective: | AE deletes a Container resource using non-blocking synchronous request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

AE has created a <Container> resource in registrar CSE

Test Sequence

Test Sequence |

+==========+========+======================+==========================================================================================================================================================================================================+ | Step | RP | Type | Description | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 1 | | Stimulus | AE is requested to send a non-blocking synchronous request to delete the  resource | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 2 | Mca | PRO Check Primitive | Sent request contains: | | | | | - op = 4 (Delete) | | | | | - to = {CSEBaseName}/URI of  resource | | | | | - fr = AE-ID | | | | | - rqi = (token-string) | | | | | - rt = 1 (non-blocking synchronous) | | | | | - pc = empty | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal  resource and sends acknowledgement response containing: | | | | | - rsc = 1000 (Accepted) | | | | | - rqi = (token-string) same as received in request message | | | | | - pc = Reference to the created  resource | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 4 | | IOP Check | AE indicates successful operation | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 5 | | Stimulus | AE is requested to send a retrieve request to  reference | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 6 | Mca | PRO Check Primitive | Sent Retrieve request contains: | | | | | - op = 2 (Retrieve) | | | | | - to =  reference | | | | | - fr = AE-ID | | | | | - rqi = (token-string) | | | | | - pc = empty | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 7 | Mca | PRO Check Primitive | Response contains: | | | | | - rsc = 2000 (OK) | | | | | - rqi = (token-string) same as received in request message | | | | | - pc =  resource with the parameter “requestStatus” set to 1 (COMPLETED) | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+ | 8 | | IOP Check | AE indicates successful operation | +———-+——–+———————-+———————————————————————————————————————————————————————————————————-+

### 8.2.2 Asynchronous request

#### 8.2.2.1 Container management

##### 8.2.2.1.1 Container Create

Interoperability Test Description +——————–+————————————————————————————————————————–+ | Identifier: | TD_M2M_NB_05 | +——————–+————————————————————————————————————————–+ | Objective: | AE creates a <Container> resource using non-blocking asynchronous request | +——————–+————————————————————————————————————————–+ | Configuration: | M2M_CFG_01 | +——————–+————————————————————————————————————————–+ | References: | oneM2M TS-0001 [1], clause 10.2.4.1
oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

AE is reachable on the URI: “AE-Notification-URI”

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to create the <Container> resource in registrar CSE |
| 2 | Mca | PRO Check Primitive | • Sent request contains: • op = 1 (Create) • to = {CSEBaseName} • fr= AE-ID • rqi = (token-string) • rt = 2 (non-blocking asynchronous) • ty = 3 (container) • nu= AE-Notification-URI • oneM2M-RQI: Request-ID •pc = Serialized Representation of the <Container> resource |
| 3 | Mca | PRO Check Primitive | • Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: • rsc = 1000 (Accepted) • rqi = token-string) same as received in request message • pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains: • op = 5 (Notify) • to = AE-Notification-URI • fr = registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |

##### 8.2.2.1.2 Container Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NB_06 |
| --- | --- |
| Objective: | AE retrieves a <container> resource using non-blocking asynchronous request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

AE has created a <Container> resource on registrar CSE

AE is reachable on the URI: “AE-Notification-URI”

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to retrieve the <Container> resource from registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <container> resource • fr = AE-ID • rqi = (token-string) • rt = 2 (non-blocking asynchronous) • nu = AE-Notification-URI • pc = empty |
|  | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: • rsc = 1000 (Accepted) • rqi = token-string) same as received in request message • pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains: • op = 5 (Notify) • to = AE-Notification-URI • fr = registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |

##### 8.2.2.1.3 Container Update

Interoperability Test Description

| Identifier: | TD_M2M_NB_07 |
| --- | --- |
| Objective: | AE updates a <Container> resource using non-blocking asynchronous request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

AE has created a <Container> resource on registrar CSE

AE is reachable on the URI: “AE-Notification-URI”

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to update the <Container> resource in registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 3 (Update) • to = {CSEBaseName}/URI of <Container> resource • fr = AE-ID • rqi = (token-string) • rt = 2 (non-blocking asynchronous) • nu = AE-Notification-URI • pc = Serialized Representation of the updated <Container> resource |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: • rsc = 1000 (Accepted) • rqi = token-string) same as received in request message • pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains: • op = 5 (Notify) • to = AE-Notification-URI • fr = registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |

##### 8.2.2.1.4 Container Delete

Interoperability Test Description

| Identifier: | TD_M2M_NB_08 |
| --- | --- |
| Objective: | AE deletes a Container resource using non-blocking asynchronous request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.6.2.1| +——————–+————————————————————————————————————————–+

Pre-test conditions:

AE has created a <Container> resource on registrar CSE

AE is reachable on the URI: “AE-Notification-URI”

Test Sequence

| r Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to delete the <Container> resource in registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 4 (Delete) • to = {CSEBaseName}/URI of <container> resource • fr = AE-ID • rqi = (token-string) • rt = 2 (non-blocking asynchronous) • nu = AE-Notification-URI • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: • rsc = 1000 (Accepted) • rqi = token-string) same as received in request message • pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains: • op = 5 (Notify) • to = AE-Notification-URI • fr = registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |

## 8.3 Single hop configuration testing

### 8.3.1 Retargeting

#### 8.3.1.1 RetargetingResource Create (Generic Test Description)

Interoperability Test Description

| Identifier: | TD_M2M_SH_01 |
| --- | --- |
| Objective: | AE creates a remote <Resource> resource |
| Configuration: | M2M_CFG_03 |
| References: |  |

Pre-test conditions:

Parents resources need to be created on the hosting CSE

Test Sequence +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | Test Sequence | +==========+========+======================+=====================================================================================================================================================================================+ | Step | RP | Type | Description | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 1 | | Stimulus | AE is requested to send a Create Request to create on the Hosting CSE | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 2 | Mca | PRO Check Primitive | - op = 1 (Create) | | | | | - to = URI of the parent resource | | | | | - fr = AE-ID | | | | | - rqi = (token-string) | | | | | - ty = type number | | | | | - pc = Serialized representation of resource | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 3 | | IOP Check | Check if possible that the request is forwarded by the Registrar CSE to the Hosting CSE | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 4 | Mcc | PRO Check Primitive | - op = 1 (Create) | | | | | - to = URI of the parent resource | | | | | - fr = AE-ID | | | | | - rqi = (token-string) | | | | | - ty = m2m:resourceType | | | | | - pc = Serialized representation of resource | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 5 | | IOP Check | Check if possible that the resource is created in the Hosting CSE | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 6 | Mcc | PRO Check Primitive | - rsc = 2001 (CREATED) | | | | | - rqi = (token-string) same as received in request message | | | | | - pc = Serialized representation of resource | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 7 | | IOP Check | Check if possible that the response is forwarded by the Registrar CSE to the AE | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 8 | Mca | PRO Check Primitive | - rsc = 2001 (CREATED) | | | | | - rqi = (token-string) same as received in request message | | | | | - pc = Serialized representation of resource | +———-+——–+———————-+————————————————————————————————————————————————————————————-+ | 9 | | IOP Check | AE indicates successful operation | +———-+——–+———————-+————————————————————————————————————————————————————————————-+

#### 8.3.1.2 <Resource> Create

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_01#01 | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.5.2.1 |  |  |
| <contentInstance> | TD_M2M_SH_01#02 | oneM2M TS-0001 [1], clause 10.2.19.2 oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <subscription> | TD_M2M_SH_01#03 | oneM2M TS-0001 [1], clause 10.2.11.2 oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_01#04 | oneM2M TS-0001 [1], clause 10.2.21.1 oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_01#05 | oneM2M TS-0001 [1], clause 10.2.7.2 oneM2M TS-0004 [2], clause 7.3.12.2.1 |  |  |
| <pollingChannel> | TD_M2M_SH_01#06 | oneM2M TS-0001 [1], clause 10.2.13.2 oneM2M TS-0004 [2], clause 7.3.21.2.1 |  |  |
| <fanOutPoint> | TD_M2M_SH_01#07 | oneM2M TS-0001 [1], clause 10.2.7.6 oneM2M TS-0004 [2], clause 7.3.14.3.1 |  |  |
| <node> | TD_M2M_SH_01#08 | oneM2M TS-0001 [1], clause 10.2.14.1 oneM2M TS-0004 [2], clause 7.3.18.2.1 |  |  |

#### 8.3.1.3 Resource Retrieve (Generic Test Description)

Interoperability Test Description

| Identifier: | TD_M2M_SH_02 |
| --- | --- |
| Objective: | AE retrieves a remote <Resource> resource |
| Configuration: | M2M_CFG_03 |
| References: |  |

Pre-test conditions:

Parents resources need to be created on the hosting CSE

Resource <Resource> has been created in Hosting CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request to retrieve <Resource> on the remote Hosting CSE |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = URI of the <Resource> resource U • fr = AE-ID • rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE |
| 4 | Mcc | PRO Check Primitive | • op = 2 (Retrieve) • to URI of the <Resource> resource • fr = AE-ID • rqi = (token-string) |
| 5 | Mcc | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <Resource> resource |
| 6 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE |
| 7 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <Resource> resource |
| 8 |  | IOP Check | AE indicates successful operation |

#### 8.3.1.4 <Resource> retrieve

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_02#01 | oneM2M TS-0001 [1], clause 10.2.4.2 oneM2M TS-0004 [2], clause 7.3.5.2.2 |  |  |
| <contentInstance> | TD_M2M_SH_02#02 | oneM2M TS-0001 [1], clause 10.2.19.3 oneM2M TS-0004 [2], clause 7.3.6.2.2 |  |  |
| <subscription> | TD_M2M_SH_02#03 | oneM2M TS-0001 [1], clause 10.2.11.3 oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_02#04 | oneM2M TS-0001 [1], clause 10.2.21.2 oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_02#05 | oneM2M TS-0001 [1], clause 10.2.7.3 oneM2M TS-0004 [2], clause 7.3.12.2.2 |  |  |
| <pollingChannel> | TD_M2M_SH_02#06 | oneM2M TS-0001 [1], clause 10.2.13.3 oneM2M TS-0004 [2], clause 7.3.21.2.2 |  |  |
| <fanOutPoint> | TD_M2M_SH_02#07 | oneM2M TS-0001 [1], clause 10.2.7.8 oneM2M TS-0004 [2], clause 7.3.14.3.2 |  |  |
| <node> | TD_M2M_SH_02#08 | oneM2M TS-0001 [1], clause 10.2.14.2 oneM2M TS-0004 [2], clause 7.3.18.2.2 |  |  |
| <remoteCSE> | TD_M2M_SH_02#09 | oneM2M TS-0001 [1], clause 10.2.2.3 oneM2M TS-0004 [2], clause 7.3.3.2.3 |  |  |
| <ae> | TD_M2M_SH_02#10 | oneM2M TS-0001 [1], clause 10.2.1.2 oneM2M TS-0004 [2], clause 7.3.5.2.2 |  |  |
| <CSEBase> | TD_M2M_SH_02#11 | oneM2M TS-0001 [1], clause 10.2.3.2 oneM2M TS-0004 [2], clause 7.3.2 |  |  |

#### 8.3.1.5 Resource Update (Generic Test Description)

Interoperability Test Description

| Identifier: | TD_M2M_SH_03 |
| --- | --- |
| Objective: | AE updates a remote <Resource> resource |
| Configuration: | M2M_CFG_03 |
| References: |  |

Pre-test conditions:

Parents resources need to be created on the hosting CSE

Resource <Resource> has been created in Hosting CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an Update Request to update the <Resource> on the Hosting CSE |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = URI of the resource <Resource> • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <Resource> resource |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE |
| 4 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = URI of the resource <Resource> • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <Resource> resource |
| 5 |  | IOP Check | Check if possible that the <Resource> resource is updated in the Hosting CSE |
| 6 | Mcc | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <Resource> resource |
| 7 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <Resource> resource |
| 9 |  | IOP Check | AE indicates successful operation |

#### 8.3.1.6 <Resource> update

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_03#01 | oneM2M TS-0001 [1], clause 10.2.4.3 oneM2M TS-0004 [2], clause 7.3.5.2.3 |  |  |
| <subscription> | TD_M2M_SH_03#02 | oneM2M TS-0001 [1], clause 10.2.11.4 oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_03#03 | oneM2M TS-0001 [1], clause 10.2.21.3 oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_03#04 | oneM2M TS-0001 [1], clause 10.2.7.4 oneM2M TS-0004 [2], clause 7.3.12.2.3 |  |  |
| <pollingChannel> | TD_M2M_SH_03#05 | oneM2M TS-0001 [1], clause 10.2.13.4 oneM2M TS-0004 [2], clause 7.3.21.2.3 |  |  |
| <fanOutPoint> | TD_M2M_SH_03#06 | oneM2M TS-0001 [1], clause 10.2.7.9 oneM2M TS-0004 [2], clause 7.3.14.3.3 |  |  |
| <node> | TD_M2M_SH_03#07 | oneM2M TS-0001 [1], clause 10.2.14.3 oneM2M TS-0004 [2], clause 7.3.18.2.3 |  |  |
| <remoteCSE> | TD_M2M_SH_03#08 | oneM2M TS-0001 [1], clause 10.2.2.3 oneM2M TS-0004 [2], clause 7.3.3.2.3 |  |  |
| <ae> | TD_M2M_SH_03#09 | oneM2M TS-0001 [1], clause 10.2.1.3 oneM2M TS-0004 [2], clause 7.3.5.2.3 |  |  |

#### 8.3.1.7 Resource Delete (Generic Test Description)

Interoperability Test Description

| Identifier: | TD_M2M_SH_04 |
| --- | --- |
| Objective: | AE delete a remote <Resource> resource |
| Configuration: | M2M_CFG_03 |
| References: |  |

Pre-test conditions:

Parents resources need to be created on the hosting CSE

Resource <Resource> has been created in Hosting CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Delete Request to delete <Resource> on the Hosting CSE |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = URI of the resource <Resource> • fr = AE-ID • rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE |
| 4 | Mcc | PRO Check Primitive | • op = 4 (Delete) • to = URI of the resource <Resource> • fr = AE-ID • rqi = (token-string) |
| 5 |  | IOP Check | Check if possible that the <Resource> resource is deleted in the Hosting CSE |
| 6 | Mcc | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 7 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE |
| 8 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | AE indicates successful operation |

#### 8.3.1.8 <Resource> delete

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_04#01 | oneM2M TS-0001 [1], clause 10.2.4.4 oneM2M TS-0004 [2], clause 7.3.5.2.4 |  |  |
| <contentInstance> | TD_M2M_SH_04#02 | oneM2M TS-0001 [1], clause 10.2.19.5 oneM2M TS-0004 [2], clause 7.3.6.2.4 |  |  |
| <subscription> | TD_M2M_SH_04#03 | oneM2M TS-0001 [1], clause 10.2.11.5 oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_04#04 | oneM2M TS-0001 [1], clause 10.2.21.4 oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_04#05 | oneM2M TS-0001 [1], clause 10.2.7.5 oneM2M TS-0004 [2], clause 7.3.12.2.4 |  |  |
| <pollingChannel> | TD_M2M_SH_04#06 | oneM2M TS-0001 [1], clause 10.2.13.5 oneM2M TS-0004 [2], clause 7.3.21.2.4 |  |  |
| <fanOutPoint> | TD_M2M_SH_04#07 | oneM2M TS-0001 [1], clause 10.2.7.10 oneM2M TS-0004 [2], clause 7.3.14.3.4 |  |  |
| <node> | TD_M2M_SH_04#08 | oneM2M TS-0001 [1], clause 10.2.14.4 oneM2M TS-0004 [2], clause 7.3.18.2.4 |  |  |

#### 8.3.1.9 Discovery with multiple filter criteria

Interoperability Test Description

| Identifier: | TD_M2M_SH_09 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in the remote Hosting CSE using multiple Filter Criteria |
| Configuration: | M2M_CFG_03 |

oneM2M TS-0004 [2], clause 7.2.3.13 | +——————–+————————————————————————————————————————+

Pre-test conditions:

Two <Container> resources with labels “key1” and “key2” are created in Hosting CSE

A <Group> resources with labels “key1” and “key2” is created in Hosting CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a discovery request to discover specific resources located in hosting CSE using multiple filter criteria (label, resource type and limit) |
| 2 | Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = URI of hosting CSEBase • fr = AE-ID • rqi = (token-string) • fu=1 • lbl=key1 • lbl=key2 • rty=3 • lim=1 • pc = empty |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE |
| 4 | Mcc | PRO Check Primitive | Forwarded request contains: • op = 2 (Retrieve) • to = hosting CSEBase • fr = AE-ID • rqi = (token-string) • fu=1 • lbl=key1 • lbl=key2 • rty=3 • lim=1 • pc = empty |
| 5 |  | IOP Check | Check if possible that the response is sent by the hosting CSE to the registrar CSE |
| 6 | Mcc | PRO Check Primitive | Hosting CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the address of one of the <Container> resources |
| 7 |  | IOP Check | Check if possible that the response is forwarded from the registrar CSE to AE |
| 6 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the address of one of the <Container> resources |
| 7 |  | IOP Check | AE indicates successful operation |

#### 8.3.1.10 Unauthorized operation (Insufficient Access Rights)

Interoperability Test Description

| Identifier: | TD_M2M_SH_10 |
| --- | --- |
| Objective: | AE delete request is rejected after access rights verification using retargeting |
| Configuration: | M2M_CFG_03 |
| References: | oneM2M TS-0004 [2], clause 7.3.1.2 |

Pre-test conditions:

An <accessControlPolicy> resource with name {ACPName} has been created in remote hosting CSE, not allowing delete operation

AE has created an <AE> resource on registrar CSE with name {AEName}

AE has created a <container> sub-resource in the <AE> resource with name {containerName} and having as accessControlPolicy-ID the ID of the remote <accessControlPolicy>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Request to delete the <container> resource from the registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = URI of addressed resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that a request is sent by the registrar CSE to the Hosting CSE to retrieve the corresponding remote <accessControlPolicy> resource |
| 4 | Mcc | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = URI of addressed resource • fr = Registrar CSE-ID • rqi = (token-string) • pc = empty |
| 5 |  | IOP Check | Check if possible that the response is sent by the hosting CSE to the registrar CSE |
| 6 | Mcc | PRO Check Primitive | Hosting CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 7 |  | IOP Check | Check if possible that an access denied error response is sent by registrar CSE to AE |
| 8 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • pc = empty |
| 9 |  | IOP Check | Check if possible that the <container> resource has not been deleted |
| 10 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |

#### 8.3.1.11 Notification

Interoperability Test Description

| Identifier: | TD_M2M_SH_11 |
| --- | --- |
| Objective: | AE receives a notification request from the remote hosting CSE |
| Configuration: | M2M_CFG_03 |

oneM2M TS-0004 [2], clause 7.4.1 | +——————–+———————————————————————————————————————-+

Pre-test conditions:

A <container> resource has been created on hosting CSE

AE has created an <AE> resource on registrar CSE

AE has created a <subscription> resource for the <container> resource on the remote hosting CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | A <contentInstance> sub-resource is created on the <container> resource. This triggers or causes the hosting CSE to send a notification to AE |
| 2 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = URI of AE resource • from = Hosting CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if possible that the Notify request is forwarded by the registrar CSE to the AE-ID |
| 4 | Mcc | PRO Check Primitive | • op = 5 (Notify) • to = AE • from = Hosting CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 5 |  | IOP Check | Check if possible that the response is sent by the AE to the registrar CSE |
| 6 | Mcc | PRO Check Primitive | AE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message pc = empty |
| 7 |  | IOP Check | Check if possible that the response is forwarded by registrar CSE to Hosting CSE |
| 8 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = empty |
| 9 |  | IOP Check | Check if possible that the <container> resource has not been deleted |
| 10 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |

### 8.3.2 <mgmtObj> Test Description

#### 8.3.2.1 <mgmtObj> Create

Interoperability Test Description

| Identifier: | TD_M2M_SH_05 |
| --- | --- |
| Objective: | AE creates a <mgmtObj> resource |
| Configuration: | M2M_CFG_03 |
| References: | oneM2M TS-0001 [1], clause 10.2.8.8 |

Pre-test conditions:

Management Session between Management Server and Management Client

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Create Request to create an <mgmtObj> on IN-CSE |
| 2 | Mca | PRO Check Primitive | • op: 1 (CREATE) • fr: AE-ID • to: {CSEBaseName}/{node} • rqi = (token-string) • ty = 13 (mgmtObj) • pc: Serialized representation of the <mgmtObj> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is created in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to create the corresponding MO using Add DM command. The mapping of <mgmtObj> and MO can be referenced from clause 5.3 of oneM2M TS-0005 [10] |
|  |  | PRO Check BBF TR069 | Requests to create the corresponding information model using AddObject RPC. The mapping of <mgmtObj> and information model or RPC can be referenced from clause 7 of oneM2M TS-0006 [11] |
|  |  | PRO Check OMA LWM2M | Requests to create the corresponding Objects using Create LWM2M Create operations. The mapping of <mgmtObj> and Object can be referenced from clause 6.3 of oneM2M TS-0005 [10] |
| 5 |  | IOP Check | Check if possible that the corresponding MO for OMA DM, information model for BBF TR069 or Object for OMA LWM2M is created on the Managed Entity. |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK. Details can be found in clause 5.4 oneM2M TS-0005 [10] |
|  |  | PRO Check BBF TR069 | Successful response of the RPC. Details can be found in clause 8.1 oneM2M TS-0006 [11] |
|  |  | PRO Check OMA LWM2M | Response with status code 2.01 Created. Details can be found in clause 6.4 oneM2M TS-0005 [10] |
| 7 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <mgmtObj> resource |
| 8 |  | IOP Check | AE indicates successful operation |

#### 8.3.2.2 <mgmtObj> Update

Interoperability Test Description

| Identifier: | TD_M2M_SH_06 |
| --- | --- |
| Objective: | AE updates a <mgmtObj> resource |
| Configuration: | M2M_CFG_03 |
| References: | oneM2M TS-0001 [1], clause 10.2.8.10 |

Pre-test conditions:

Management Session between Management Server and Management Client

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Update Request to update an <mgmtObj> on IN-CSE |
| 2 | Mca | PRO Check Primitive | • op: 3 (UPDATE) • fr: AE-ID • to: {CSEBaseName}/{node}/{mgmtObj} • rqi = (token-string) • pc: Serialized representation of the <mgmtObj> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is updated in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to update the corresponding MO using Replace DM command. The mapping of <mgmtObj> and MO can be referenced from clause 5.3 of oneM2M TS-0005 [10] |
|  |  | PRO Check BBF TR069 | Requests to Update the corresponding information model using SetParameterValues RPC. The mapping of <mgmtObj> and information model or RPC can be referenced from clause 7 of oneM2M TS-0006 [11] |
|  |  | PRO Check OMA LWM2M | Requests to Update the corresponding Objects using LWM2M Write operations. The mapping of <mgmtObj> and Object can be referenced from clause 6.3 of oneM2M TS-0005 [10] |
| 5 |  | IOP Check | Check if possible that the corresponding MO for OMA DM, information model for BBF TR069 or Object for OMA LWM2M is Updated on the Managed Entity |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK. Details can be found in clause 5.4 oneM2M TS-0005 [10] |
|  |  | PRO Check BBF TR069 | Successful response of the RPC. Details can be found in clause 8.1 oneM2M TS-0006 [11] |
|  |  | PRO Check OMA LWM2M | Response with status code 2.04 Changed. Details can be found in clause 6.4 oneM2M TS-0005 [10] |
| 7 | Mca | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <mgmtObj> resource |
| 8 |  | IOP Check | AE indicates successful operation |

#### 8.3.2.3 <mgmtObj> Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_SH_07 |
| --- | --- |
| Objective: | AE retrieves a <mgmtObj> resource |
| Configuration: | M2M_CFG_03 |
| References: | oneM2M TS-0001 [1], clause 10.2.8.9 |

Pre-test conditions:

Management Session between Management Server and Management Client

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Retrieve Request to retrieve an <mgmtObj> on IN-CSE |
| 2 | Mca | PRO Check Primitive | • op = 2 (RETRIEVE) • to = {CSEBaseName}/{node}/{mgmtObj} • fr = AE-ID • rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is retrieved in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to retrieve the corresponding MO using Get DM command |
|  |  | PRO Check BBF TR069 | Requests to retrieve the corresponding information model using GetParametersValue RPC |
|  |  | PRO Check OMA LWM2M | Requests to retrieve the corresponding Objects using Retrieve LWM2M Read operation |
| 5 |  | IOP Check |  |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK with the information of the MO. Details can be found in clause 5.4 oneM2M TS-0005 [10] |
|  |  | PRO Check BBF TR069 | Successful response of the RPC with the information about the management related information. Details can be found in clause 8.1 oneM2M TS-0006 [11] |
|  |  | PRO Check OMA LWM2M | Response with status code 2.05 Content with the information of the Object. Details can be found in clause 6.4 oneM2M TS-0005 [10] |
| 7 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <mgmtObj> resource |
| 8 |  | IOP Check | AE indicates successful operation |

#### 8.3.2.4 <mgmtObj> Delete

Interoperability Test Description +——————–+———————————————————–+ | Identifier: | TD_MM_SH_08 | +——————–+———————————————————–+ | Objective: | AE deletes a <mgmtObj> resource | +——————–+———————————————————–+ | Configuration: | M2M_CFG_03 | +——————–+———————————————————–+ | References: | oneM2M TS-0001 [1], clause 10.2.8.11| +——————–+———————————————————–+

Pre-test conditions:

Management Session between Management Server and Management Client

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Delete Request to delete an <mgmtObj> on IN-CSE |
| 2 | Mca | PRO Check Primitive | • op = 4 (DELETE) • to = {CSEBaseName}/{node}/{mgmtObj} • fr = AE-ID • rqi = (token string) |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is deleted in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to delete the corresponding MO using Delete DM command |
|  |  | PRO Check BBF TR069 | Requests to delete the corresponding information model using DeleteObject RPC |
|  |  | PRO Check OMA LWM2M | Requests to delete the corresponding Objects using LWM2M Delete operation |
| 5 |  | IOP Check | Check if possible that the corresponding MO for OMA DM, information model for BBF TR069 or Object for OMA LWM2M is deleted on the Managed Entity |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK. Details can be found in clause 5.4 oneM2M TS-0005 [10] |
|  |  | PRO Check BBF TR069 | Successful response of the RPC. Details can be found in clause 8.1 oneM2M TS-0006 [11] |
|  |  | PRO Check OMA LWM2M | Response with status code 2.02 Deleted. Details can be found in clause 6.4 oneM2M TS-0005 [10] |
| 7 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | AE indicates successful operation |

### 8.3.3 Announcement Management

#### 8.3.3.1 AEAnnc Create

Interoperability Test Description

| Identifier: | TD_M2M_SH_12 |
| --- | --- |
| Objective: | AE1 announces itself to CSE2 |
| Configuration: | M2M_CFG_04 |
| References: |  |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

CSE1 is registered to CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send an AE Update Request with announceTo attribute set to CSE2 CSE-ID |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{AE} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <AE> resource |
| 3 |  | IOP Check | Check if possible that the CREATE <AEannc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName2}/{CSEBaseName1} • fr = CSE1-ID • rqi = (token-string) • ty = 10002 (AEAnnc) • pc = Serialized representation of <AEAnnc> resource |
| 5 |  | IOP Check | Check if possible that the <AEAnnc> resource is created in CSE2 with only MA attributes |
| 6 | Mcc | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <AEAnnc> resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <AE> resource |
| 9 |  | IOP Check | AE indicates successful operation |

#### 8.3.3.2 ContainerAnnc Create

Interoperability Test Description

| Identifier: | TD_M2M_SH_13 |
| --- | --- |
| Objective: | AE1 announces a child container to CSE2 |
| Configuration: | M2M_CFG_04 |
| References: |  |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

AE2 has created a <AE> resource on registrar CSE with name {AE2}

CSE1 is registered to CSE2

<container> resource is created as a child of AE1

AE1 is announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with announceTo attribute set to CSE2 CSE-ID |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{container} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the CREATE <ContainerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName2}/{AE1Annc} • fr = CSE1-ID • rqi = (token-string) • ty = 10003 (containerAnnc) • pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is created in CSE2 with only MA attributes |
| 6 | Mcc | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |

#### 8.3.3.3 ContainerAnnc Update

Interoperability Test Description

| Identifier: | TD_M2M_SH_14 |
| --- | --- |
| Objective: | AE1 announces an Optional Announce attribute to CSE2 |
| Configuration: | M2M_CFG_04 |
| References: |  |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

AE2 has created a <AE> resource on registrar CSE with name {AE2}

CSE1 is registered to CSE2

<container> resource is created as a child of AE1

AE1 is announced on CSE2

<container> is announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with announcedAttribute = maxNrOfInstances |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{container} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName2}/{ ContainerAnnc } • fr = CSE1-ID • rqi = (token-string) • pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is update in CSE2 with maxNrOfInstances attributes |
| 6 | Mcc | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca |  | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE1 indicates successful operation |

#### 8.3.3.4 ContainerAnnc Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_SH_15 |
| --- | --- |
| Objective: | AE2 retrieves an Announced Resource |
| Configuration: | M2M_CFG_04 |
| References: |  |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

AE2 has created a <AE> resource on registrar CSE with name {AE2}

CSE1 is registered to CSE2

<container> resource is created as a child of AE1

AE1 is announced on CSE2

<container> is announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Retrieve Request for a < containerAnnc > |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName2}/URI of < containerAnnc > resource • fr = AE2-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <containerAnnc> resource |
| 4 |  | IOP Check | AE indicates successful operation |

IOP Verdict

Verify that this is a containAnnc resource

#### 8.3.3.5 ContainerAnnc Retrieve Original

Interoperability Test Description

| Identifier: | TD_M2M_SH_16 |
| --- | --- |
| Objective: | AE2 retrieves the original resource representation of an announced resource |
| Configuration: | M2M_CFG_04 |
| References: |  |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

AE2 has created a <AE> resource on registrar CSE with name {AE2}

CSE1 is registered to CSE2

<container> resource is created as a child of AE1

AE1 is announced on CSE2

<container> is announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a Retrieve Request to a < containerAnnc > with rcn = 7 |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName2}/URI of < containerAnnc > resource • fr = AE2-ID • rqi = (token-string) • rcn = 7 (original) • pc = empty |
| 3 |  | IOP Check | Check if possible that the GET <container> is sent from CSE2 to CSE1 |
| 4 | Mcc | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName1}/{ Container} • fr = AE2-ID • rqi = (token-string) • pc = empty |
| 5 |  | IOP Check |  |
| 6 | Mcc | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 7 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE |
| 8 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |

#### 8.3.3.6 ContainerAnnc Delete by updating announceTo attribute

Interoperability Test Description

| Identifier: | TD_M2M_SH_21 |
| --- | --- |
| Objective: | AE1 deletes its announced child container from CSE2 |
| Configuration: | M2M_CFG_04 |
| References: | oneM2M TS-0001 [1], clause 10.2.13.4 |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

CSE1 is registered to CSE2

AE1 is announced on CSE2

<container> resource is created as a child of AE1 and announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a an container Update Request with announceTo attribute set to NULL |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName1}/{AE1}/{container} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the DELETE <containerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} • fr = CSE1-ID • rqi = (token-string) |
| 5 |  | IOP Check | Check if possible that the <containerAnnc> resource is deleted in CSE2 |
| 6 | Mcc | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |

8.3.3.7 ContainerAnnc Delete by deleting original resource

Interoperability Test Description

| Identifier: | TD_M2M_SH_22 |
| --- | --- |
| Objective: | AE1 deletes its announced child container from CSE2 |
| Configuration: | M2M_CFG_04 |
| References: | oneM2M TS-0001 [1], clause 10.2.13.4 |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

CSE1 is registered to CSE2

AE1 is announced on CSE2

<container> resource is created as a child of AE1 and announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a an container Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName1}/{AE1}/{container} • fr = AE-ID • rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the DELETE <containerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} • fr = CSE1-ID • rqi = (token-string) |
| 5 |  | IOP Check | Check if possible that the <containerAnnc> resource is deleted in CSE2 |
| 6 | Mcc | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 7 |  | IOP Check | CSE1 sends a DELETE response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | AE indicates successful operation |

8.3.3.8 Announced attribute Create by addition to announcedAttribute attribute

Interoperability Test Description

| Identifier: | TD_M2M_SH_23 |
| --- | --- |
| Objective: | AE1 announces an announcable attribute of its child container to CSE2 |
| Configuration: | M2M_CFG_04 |
| References: | oneM2M TS-0001 [1], clause 10.2.13.7 |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

CSE1 is registered to CSE2

AE1 is announced on CSE2

<container> resource is created as a child of AE1 and announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with announcedAttribute attribute containing currentNrOfInstances |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName1}/{AE1}/{container} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} • fr = CSE1-ID • rqi = (token-string) • pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with currentNrOfInstances attribute |
| 6 | Mcc | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |

8.3.3.9 Announced attribute Create by creation of a MA attribute at the original resource

Interoperability Test Description

| Identifier: | TD_M2M_SH_24 |
| --- | --- |
| Objective: | AE1 announces an MA attribute of its child container to CSE2 |
| Configuration: | M2M_CFG_04 |
| References: | oneM2M TS-0001 [1], clause 10.2.13.7 |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

CSE1 is registered to CSE2

AE1 is announced on CSE2

<container> resource is created as a child of AE1 and announced on CSE2

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with labels attribute |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName1}/{AE1}/{container} • fr = AE-ID |

Interoperability Test Description

|  |  |  | rqi = (token-string) pc = Serialized representation of updated <container> resource |
| --- | --- | --- | --- |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} • fr = CSE1-ID • rqi = (token-string) • pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with labels attribute |
| 6 | Mcc | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |

8.3.3.10 Announced attribute Delete by deletion from announcedAttribute attribute

Interoperability Test Description

| Identifier: | TD_M2M_SH_25 |
| --- | --- |
| Objective: | AE1 de-announces an announcable attribute of its child container to CSE2 |
| Configuration: | M2M_CFG_04 |
| References: | oneM2M TS-0001 [1], clause 10.2.13.8 |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

CSE1 is registered to CSE2

AE1 is announced on CSE2

<container> resource is created as a child of AE1 and announced on CSE2 with currentNrOfInstances attribute announced

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a <container> Update Request with announcedAttribute attribute set to NULL |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName1}/{AE1}/{container} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} • fr = CSE1-ID • rqi = (token-string) • pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with no currentNrOfInstances attribute |
| 6 | Mcc | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |

8.3.3.11 Announced attribute Delete by deletion of a MA attribute at the original resource

Interoperability Test Description

| Identifier: | TD_M2M_SH_26 |
| --- | --- |
| Objective: | AE1 de-announces an MA attribute (conditionally mandatory) of its child container to CSE2 |
| Configuration: | M2M_CFG_04 |
| References: |  |

Pre-test conditions:

<CSEBase> resource has been created in CSE1 with name {CSEBaseName1}

AE1 has created a <AE> resource on registrar CSE with name {AE1}

<CSEBase> resource has been created in CSE2 with name {CSEBaseName2}

CSE1 is registered to CSE2

AE1 is announced on CSE2

<container> resource is created as a child of AE1 and announced on CSE2 with labels attribute announced

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a <container> Update Request with labels attribute set to NULL |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName1}/{AE1}/{container} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2 |
| 4 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} • fr = CSE1-ID • rqi = (token-string) • pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with no labels attribute |
| 6 | Mcc | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1 |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |

### 8.3.4 Single Hop <fanOutPoint> operations

#### 8.3.4.1 Create <fanOutPoint>

| Interoperability Test Description :======================================+:======================================================================================================= Identifier ¦ TD_M2M_SH_17 |  |  |
| --- | --- | --- |
| Objective | AE creates a resource in each group member, where some memberIDs are on a remoteCSE |  |
| Configuration | M2M_CFG_08 |  |
| References | oneM2M TS-0001 1, clause 10.2.7.6 oneM2M TS-0004 2, clauses 7.4.14.2 and 7.4.14.3 |  |

Pre-test conditions:

Two or more resources of type <container> exist on the member hosting CSE

A group exists containing these two members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Create Request to create <contentInstance> in each group member |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) • ty = 4 (contentInstance) • pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar/Group Hosting CSE to the Member Hosting CSE |
| 4 | Mcc | PRO Check Primitive | • op = 1 (Create) • to = {MemberCSEBaseName}/{subgroupId}/fopt • or {MemberCSEBaseName}/{memberId} • fr = AE-ID • rqi = (token-string) • gid = (grpId-token-string) • ty = 4 (contentInstance) • pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | Check if possible that the <contentInstance> resource is created in the Member Hosting CSE |
| 6 | Mcc | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • gid = (grpId-token-string) same as received in request message • pc = Serialized representation of <contentInstance> resource or <aggregated response> |
| 7 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE |
| 8 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <aggregated response> |
| 9 |  | IOP Check | AE indicates successful operation |

#### 8.3.4.2 Retrieve <fanOutPoint>

Interoperability Test Description

| Identifier: | TD_M2M_SH_18 |
| --- | --- |
| Objective: | AE retrieves a <container> resource from each group member, where some memberIDs are on a remoteCSE |
| Configuration: | M2M_CFG_08 |
| References: | oneM2M TS-0001 [1], clause 10.2.7.7 |

Pre-test conditions:

Two or more resources of type <container> exist on the member hosting CSE

A group exists containing these two members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request to the fanoutPoint of <group> resource |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar/group hosting CSE to the Member Hosting CSE |
| 4 | Mcc | PRO Check Primitive | • op = 2 (Retrieve) • to = {MemberCSEBaseName}/{subgroupId}/fopt • or {MemberCSEBaseName}/{memberId} • fr = AE-ID • rqi = (token-string) • gid = (grpId-token-string) |
| 5 | Mcc | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • gid = (grpId-token-string) same as received in request message • pc = Serialized representation of <container> resource |
| 6 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE |
| 7 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <aggregated_response> |
| 8 |  | IOP Check | AE indicates successful operation |

#### 8.3.4.3 Update <fanOutPoint>

Interoperability Test Description

| Identifier: | TD_M2M_SH_19 |
| --- | --- |
| Objective: | AE updates a <container> resource in each group member, where some memberIDs are on a remoteCSE |
| Configuration: | M2M_CFG_08 |
| References: | oneM2M TS-0001 [1], clause 10.2.7.8 |

Pre-test conditions:

Two or more resources of type <container> exist on the member hosting CSE

A <group> exists containing these two members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Update Request to the fanoutPoint of <group> resource to lifetime of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/{group}/fopt • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar/group hosting CSE to the Member Hosting CSE |
| 4 | Mcc | PRO Check Primitive | • op = 3 (Update) • to = {MemberCSEBaseName}/{subgroupId}/fopt • or {MemberCSEBaseName}/{memberId} • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | Check if possible that the <Resource> resource is updated in the Hosting CSE |
| 6 | Mcc | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • gid = (grpId-token-string) same as received in request message • pc = Serialized representation of <container> resource or <aggregated response> |
| 7 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE |
| 8 | Mca | PRO Check Primitive | • rsc = 2004 (CHANGED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <aggregated response> |
| 9 |  | IOP Check | AE indicates successful operation |

#### 8.3.4.4 Delete <fanOutPoint>

Interoperability Test Description

| Identifier: | TD_M2M_SH_20 |
| --- | --- |
| Objective: | AE deletes a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| Configuration: | M2M_CFG_08 |
| References: | oneM2M TS-0001 [1], clause 10.2.7.9 |

Pre-test conditions:

Two or more resources of type <container> exist on the member hosting CSE

Each <container> has at least 1 <contentInstance>

A group exists containing these two members of type <container>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Delete ‘oldest’ Request to the fanoutPoint of <group> resource |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{group}/fopt/ol • fr = AE-ID • rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE |
| 4 | Mcc | PRO Check Primitive | • op = 4 (Delete) • to = {MemberCSEBaseName}/{subgroupId}/fopt/ol • or {MemberCSEBaseName}/{memberId}/ol • fr = AE-ID • rqi = (token-string) • gid = (grpId-token-string) |
| 5 |  | IOP Check | Check if possible that the <Resource> resource is deleted in the Hosting CSE |
| 6 | Mcc | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • gid = (grpId-token-string) same as received in request message |
| 7 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE |
| 8 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <aggregated_response> |
| 9 |  | IOP Check | AE indicates successful operation |

## 8.4 Security management

### 8.4.1 Secure AE Registration

#### 8.4.1.1 PSK Security Association Establishment Framework

Interoperability Test Description

| Identifier: | TD_M2M_SE_01 |
| --- | --- |
| Objective: | AE uses Provisioned Symmetric Key Security Association Establishment Framework to enable mutual authentication with the Registrar CSE. Registrar CSE performs AE authorization check on incoming AE registration request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0001 [1], clauses 9.6.29, 9.6.19, 9.16.20 | +——————-+————————————————————————————————————————————————————————————————————————–+

Pre-test conditions:

AE and Registrar CSE are pre-Provisioned with Kpsa = 123456,KpsaId = test@onem2m.com and Cipher Suites = TLS_PSK_WITH_AES_128_CBC_SHA256, TLS_PSK_WITH_AES_128_CCM_8

Registrar CSE is provisioned with Service Subscribed Profile and Service Subscribed Node Resources

Service Subscribed Node contains csi <Registrar CSE-ID> and rlk < URI of serviceSubscribedAppRule > attributes

Registrar CSE is configured with <serviceSubscribedAppRule> resource having a CredentialD, APP-ID and AE-ID with the following values:

<m2m:asar rn=“asar”>

<aci>00-test@onem2m.com</aci>

<aai>APP01</aai>

<aae>AE-ID</aae>

</m2m:asar>

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a primitive to the Registrar CSE |
| 2 | Mca | PRO Check Primitive | Security Association Establishment |
| ROWSPAN | ROWSPAN | PRO Check TCP | TLS Handshake Cipher Suite:TLS_PSK_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 KpsaId = test@onem2m.com |
| ROWSPAN | ROWSPAN | PRO Check UDP | DTLS Handshake Cipher Suite:TLS_PSK_WITH_AES_128_CCM_8 Version: DTLS v1.2 KpsaId = test@onem2m.com |
| 3 |  | IOP Check | Check if possible that Handshake was successful |
| 4 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 2 (AE) • pc = Serialized representation of <AE> resource |
| 5 |  | IOP Check | Check that APP-ID, AE-ID, Credential ID are in <serviceSubscribedAppRule> Check if possible that the <AE> resource is created in registrar CSE |
| 6 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <AE> resource |
| 7 |  | IOP Check | AE indicates successful operation |

### 8.4.2 Authentication

#### 8.4.2.1 Authentication using the Provisioned Symmetric Key Security Association Establishment Framework with TLS

Interoperability Test Description

| Identifier: | TD_M2M_SE_02 |
| --- | --- |
| Objective: | AE establishes mutual authentication with the Registrar CSE using Provisioned Symmetric Key Security Association Establishment Framework |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.2.2.1 |

Pre-test conditions:

AE and Registrar CSE are pre-Provisioned with Kpsa = 123456,KpsaId = test@onem2m.com and Cipher Suites = TLS_PSK_WITH_AES_128_CBC_SHA256

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The TLS client on AE sends a Client Hello Handshake message |
| 2 | Mca | PRO Check TCP | Client Hello handshake message • Handshake Type = 0x01 (Client Hello) • Cipher Suite:TLS_PSK_WITH_AES_128_CBC_SHA256 • Version: TLS v1.2 |
| 3 | Mca | PRO Check TCP | Server Hello handshake message • Handshake Type = 0x02 (Server Hello) • Cipher Suite:TLS_PSK_WITH_AES_128_CBC_SHA256 • Version: TLS v1.2 • Server Hello Done handshake message • Handshake Type = 0x0e (Server Hello Done) |
| 4 |  | Stimulus | The TLS client on AE sends Client Key Exchange, Change Cipher Spec, Finished messages |
| 5 | Mca | PRO Check TCP | The TLS client Key Exchange handshake message • Handshake Type = 0x10 (Client Key Exchange) • psk_identity = test@onem2m.com • Version: TLS v1.2 • Client Change Cipher Spec message • Content type = 0x14 (Change Cipher Spec) • Client Finished handshake message • Handshake Type = 0x14 (Client Finished) • Version: TLS v1.2 |
| 6 |  | IOP Check | Check that The TLS server authenticated the Client by validating Verify Data Check that AE associated the established TLS session with the CSE-ID |
| 7 | Mca | PRO Check TCP | Server New Session Ticket handshake message • Handshake Type = 0x04 (New Session Ticket) • psk_identity = test@onem2m.com • Version: TLS v1.2 • Server Change Cipher Spec message • Content type = 0x14 (Change Cipher Spec) • Server Finished handshake message • Handshake Type = 0x14 (Client Finished) • Version: TLS v1.2 |
| 8 |  | IOP Check | Check that The TLS client authenticated the Server by validating Verify Data |

#### 8.4.2.2 Authentication using the Certificate-Based Security Association Establishment Framework with TLS

Interoperability Test Description

| Identifier: | TD_M2M_SE_03 |
| --- | --- |
| Objective: | AE establishes mutual authentication with the Registrar CSE using Certificate-Based Security Association Establishment Framework |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.2.2.2 |

Pre-test conditions:

The Registrar CSE uses the CSE-ID certificate signed by a root CA certificate

AE uses the AE-ID certificate signed by a root CA certificate

Cipher Suite = TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The TLS client on AE sends a client Hello Handshake message |
| 2 | Mca | PRO Check TCP | The TLS client sends a Hello handshake message to the TLS server • Handshake Type = 0x01 (Client Hello) • Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • Version: TLS v1.2 |
| 3 | Mca | PRO Check TCP | The TLS server sends Hello, Certificate, Key Exchange, Certificate Request, Hello Done messages to the TLS client • Server Hello handshake message • Handshake Type = 0x02 (Server Hello) • Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • Version: TLS v1.2 • Server Certificate handshake message • Handshake Type = 0x0b (Server Certificate) • Certificate: the Registrar CSE certificate • Server Key Exchange handshake message • Handshake Type = 0x0c (Server Key Exchange) • Public key: ECDHE generated key • Server Certificate Request handshake message • Handshake Type = 0x0d (Certificate Request) • Server Hello Done handshake message • Handshake Type = 0x0e (Server Hello Done) |
| 4 |  | IOP Check | The TLS client on AE checks if the certificate of the Server is valid |
| 5 |  | Stimulus | The TLS client on AE sends Certificate, Client Key exchange, Certificate Verify, Change Cipher Spec, Finished messages |
| 6 | Mca | PRO Check TCP | Client Certificate handshake message • Handshake Type = 0x0b (Client Certificate) • Certificate: AE certificate • Client Key Exchange message • Handshake Type = 0x10 (Client Key Exchange) • Public key: ECDHE generated key • Client Certificate Verify message • Handshake Type = 0x0f (Certificate Verify) • Client Change Cipher Spec message • Content type = 0x14 (Change Cipher Spec) • Client Finished handshake message • Handshake Type = 0x14 (Client Finished) |
| 7 |  | IOP Check | The TLS server on CSE checks if the certificate of the Client is valid |
| 8 | Mca | PRO Check TCP | The TLS server sends New Session Ticket, Change Cipher Spec, and Finished messages to the TLS client • Server New Session Ticket message • Handshake Type = 0x04 (New Session Ticket) • Server Change Cipher Spec message • Content type = 0x14 (Change Cipher Spec) • Server Finished message • Handshake Type = 0x14 (Client Finished) • Version: TLS v1.2 |
| 9 |  | IOP Check | Check that The TLS client authenticated the Server by validating Verify Data |

### 8.4.3 Authorization

#### 8.4.3.1 Authorization using selfPrivileges

Interoperability Test Description

| Identifier: | TD_M2M_SE_04 |
| --- | --- |
| Objective: | AE accesses <accessControlPolicy> resource using its selfPrivileges credentials |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0001 [1], clause 9.6.2.0 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created an <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

selfPrivileges attribute of {accessControlPolicyName} contains the following access control tuple:

acor = AE-ID

acop = 63

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{accessControlPolicyName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.4.3.2 Authorization using accessControlPolicy privileges

Interoperability Test Description

| Identifier: | TD_M2M_SE_05 |
| --- | --- |
| Objective: | AE accesses <AE> resource using its accessControlPolicyIDs attribute |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0001 [1], clause 9.6.2.0 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created an <AE> resource on registrar CSE with name {AE}

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

accessControlPolicyIDs attribute of {AE} is set to resource id of {accessControlPolicyName}

privileges attribute of {accessControlPolicyName} contains the following access control tuple:

acor = AE-ID

acop = 34

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an AE Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to send an AE Delete Request |
| 6 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{AE} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 7 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • pc = empty |
| 8 |  | IOP Check | Check if possible that the <AE> resource has not been removed in registrar CSE. |
| 9 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |

#### 8.4.3.3 Authorization using default access privileges (owner is configured)

Interoperability Test Description

| Identifier: | TD_M2M_SE_06 |
| --- | --- |
| Objective: | AE accesses <AE> resource using default access privileges |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0001 [1], clause 9.6.2.0 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created an <AE> resource on registrar CSE with name {AE}

<container> resource has been created in registrar CSE under <AE> resource with name {containerName}

accessControlPolicyIDs attribute of {containerName} is NULL

owner attribute of {containerName} = AE-ID

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a container Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE2 is requested to send a container Retrieve Request |
| 6 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE2-ID • rqi = (token-string) • pc = empty |
| 7 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • pc = empty |
| 8 |  | IOP Check | AE indicates unsuccessful operation (Retrieve error - no privilege) |

#### 8.4.3.4 Authorization using default access privileges (owner is not configured)

Interoperability Test Description

| Identifier: | TD_M2M_SE_07 |
| --- | --- |
| Objective: | AE accesses <AE> resource using default access privileges |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0001 [1], clause 9.6.2.0 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created an <AE> resource on registrar CSE with name {AE}

<container> resource has been created in registrar CSE under <AE> resource with name {containerName}

accessControlPolicyIDs attribute of {containerName} is NULL

owner attribute of {containerName} is not set

creator attribute of {containerName} = AE-ID

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a ContainerRetrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE2 is requested to send a Container Retrieve Request |
| 6 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE2-ID • rqi = (token-string) • pc = empty |
| 7 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • pc = empty |
| 8 |  | IOP Check | AE indicates unsuccessful operation (Retrieve error - no privilege) |

#### 8.4.3.5 Direct Dynamic Authorization

Interoperability Test Description

| Identifier: | TD_M2M_SE_08 |
| --- | --- |
| Objective: | AE accesses <AE> resource using Direct Dynamic Authorization |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12] , clause 7.3.2.2 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created an <AE> resource on registrar CSE with name {AE}

<container> resource has been created in registrar CSE under <AE> resource with name {containerName}

Arbitrary set of <accessControlPolicy> resources are linked to the {containerName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Container Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that Tokens or Token-Ids have not been included in the request |
| 4 |  | IOP Check | Check if possible that CSE selected a DAS Server based on accessControlRules linked to the requested resource |
| 5 | Mca | PRO Check Primitive | Registrar CSE sends a Notify request to the DAS server: • op = 6 (Notify) • pc: securityInfo: Direct Dynamic Authorization Originator = AE-ID Originator Resource Type = 3 (Container) Operation = 2 (Retrieve) |
| 6 |  | IOP Check | Check that if the DAS Server issued token(s), they conform to the Token structure (oneM2M TS-0003 [12], clause 7.3.2.4) |
| 7 | Mca | PRO Check Primitive | The DAS server responds to the Registrar CSE: • op = 6 (Notify response) • pc: securityInfo: Direct Dynamic Authorization (optional) token(s): authorization token(s) (optional) dynamicACPInfo: information for creating accessControlPolicy dynamicaly |
| 8 |  | IOP Check | Check that if token(s) present in response content, the token is validated in the Registrar CSE successfully (oneM2M TS-0003 [12], clause 7.3.2.5) |
| 9 |  | IOP Check | Check that if dynamicACPInfo present in response content, the Registrar CSE created <accessControlPolicy> resource matching the dynamicACPInfo. |
| 10 | Mca | PRO Check Primitive | If access is granted, the Registrar CSE responds to the AE: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <container> resource If access is not granted, the Registrar CSE responds to the AE: • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • pc = empty |
| 11 |  | IOP Check | If access is granted, AE indicates successful operation, otherwise AE indicates unsuccessful operation (Retrieve error - no privilege) |

#### 8.4.3.6 Indirect Dynamic Authorization

Interoperability Test Description

| Identifier: | TD_M2M_SE_09 |
| --- | --- |
| Objective: | AE accesses <AE> resource using Indirect Dynamic Authorization |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12] , clause 7.3.2.3 |

Pre-test conditions:

CSEBase resource has been created in registrar CSE with name {CSEBaseName}

AE has created an <AE> resource on registrar CSE with name {AE}

<container> resource has been created in registrar CSE under <AE> resource with name {containerName}

Arbitrary set of <accessControlPolicy> resources are linked to the {containerName}

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Container Retrieve Request |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 4103 (ACCESS_DENIED) • rqi = (token-string) same as received in request message • tqf: DAS Server PoA • pc = empty |
| 4 |  | IOP Check | AE indicates unsuccessful operation (Retrieve error - no privilege) |
| 5 |  | Stimulus | AE is requested to send a token request to the DAS using original request data. A uthorSignIndicator parameter is optional. |
| 6 |  | IOP Check | Check that if the DAS Server issued token(s), they conform to the Token structure (oneM2M TS-0003 [12], clause 7.3.2.4) |
| 7 |  | Stimulus | AE is requested to send a Container Retrieve Request with additional token(s) information |
| 8 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/{AE}/{containerName} • fr = AE-ID • rqi = (token-string) • (optional) tkns: token(s) if ESData-protected Token(s) are provided • (optional) tids: token Id(s) if ESData-protected Token(s) are not provided • pc = empty |
| 9 | Mca | PRO Check Primitive | If the request in step 7 includes token Id(s), the Registrar CSE sends a Notify request to the DAS Server: • op = 6 (Notify) securityInfo Type: Indirect Dynamic Authorization • pc: • tids: token Id(s) |
| 10 | Mca | PRO Check Primitive | The DAS server responds to the Registrar CSE: • op = 6 (Notify response) • pc: securityInfo: Indirect Dynamic Authorization token(s): authorization token(s) corresponding token Id(s) |
| 12 |  | IOP Check | Check that the token(s) are validated in the Registrar CSE successfully (oneM2M TS-0003 [12], clause 7.3.2.5) |
| 13 |  | IOP Check | If access is granted, AE indicates successful operation, otherwise AE indicates unsuccessful operation (Retrieve error - no privilege) |
| 14 | Mca | PRO Check Primitive | If access is granted, the Registrar CSE responds to the AE: • rsc = 2000 (OK) • ltids: Local-Token-ID(s) • tkns: Token(s) • rqi = (token-string) same as received in request message •pc = Serialized representation of <container> resource If access is not granted, the Registrar CSE responds to the AE: • rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 15 |  | IOP Check | If access is granted, AE indicates successful operation, otherwise AE indicates unsuccessful operation (Retrieve error - no privilege) |

### 8.4.4 Key provisioning management

#### 8.4.4.1 MEF Handshake Procedure using certificates

Interoperability Test Description

| Identifier: | TD_M2M_SE_10 |
| --- | --- |
| Objective: | A MEF Handshake procedure establishes a mutually authenticated TLS session for protecting the communication between an MEF Client and MEF using pre-provisioned certificates. |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.2 |

Pre-test conditions:

The MEF Client and MEF have been provisioned with certificates and Cipher Suite = TLS_PSK_WITH_AES_128_CBC_SHA256

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | MEF Client and MEF establish the TLS or DTLS session using the certificate-based TLS handshake |
| 2 |  | IOP Check | Check that MEF Handshake is successful Check that the MEF’s certificate is verified against the set of provisioned MEF certificate trust anchors (as described in oneM2M TS-0003 [12]) |

#### 8.4.4.2 MEF Handshake Procedure using Master Credentials

Interoperability Test Description

| Identifier: | TD_M2M_SE_ 11 |
| --- | --- |
| Objective: | A MEF Handshake procedure establishes a mutually authenticated TLS or DTLS session for protecting the communication between an MEF Client and MEF using pre-provisioned Master Credentials. |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.2 |

Pre-test conditions:

The MEF Client and MEF have been provisioned with Kpm = 123456, KpmID = psk_identity, and Cipher Suites = TLS_PSK_WITH_AES_128_CBC_SHA256, TLS_PSK_WITH_AES_128_CCM_8

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | MEF Client and MEF establish the TLS or DTLS session using the certificate-based TLS handshake |
| 2 | Mca | PRO Check TCP/UDP | • psk_identity = test@onem2m.com • psk = 123456\ |
| 3 |  | IOP Check | Check that MEF Handshake is successful |

#### 8.4.4.3 MEF Client Registration Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_ 12 |
| --- | --- |
| Objective: | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.3 |

Pre-test conditions:

The MEF Client, and MEF have been provisioned with the parameters described in oneM2M TS-0003 [12], clause 8.3.7

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Client Registration request |
| 3 | Mca | PRO Check TCP/UDP | • MEF-FQDN = FQDN of the MEF • adminFQDN = FQDN of the administrating stakeholder • expirationTime = time when the registration shall expire |
| 4 |  | IOP Check | Check if possible that MEF has created a MEF Client Registration record |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Client Registration response • MEFClientRegID = Identifier for the new MEF Client Registration • expirationTime = time when the MEF Client Registration record shall expire • MEF Client ID = Identifier of the MEF Client • adminFQDN = FQDN of the administrating stakeholder |
| 6 |  | IOP Check | Check if possible that MEF Client has stored parameters provided by the MEF |

#### 8.4.4.4 MEF Client Configuration Retrieval Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_13 |
| --- | --- |
| Objective: | The MEF Client retrieves MEF Client Configurations provided by the administrating stakeholder to the MEF |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.4 |

Pre-test conditions:

The MEF Client has previously performed the MEF Client Registration procedure to create the MEF Client Registration record

The MEF Client Registration record is not expired

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Client Configuration Retrieval request |
| 3 | Mca | PRO Check TCP/UDP | • MEF-FQDN = FQDN of the MEF, from MEF Instruction Configuration • MEFClientRegID = Identifier for the MEF Client registration record being updated |
| 4 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Client Configuration Retrieval response • MEFClientCfg = MEF Client Configuration currently associated with the identified MEF Client registration record |

#### 8.4.4.5 MEF Client Configuration Update Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_14 |
| --- | --- |
| Objective: | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels. |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.5 |

Pre-test conditions:

The MEF Client has previously performed the MEF Client Registration procedure to create the MEF Client Registration record

The MEF Client Registration record is not expired

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client shall send a MEF Client Registration Update request |
| 3 | Mca | PRO Check TCP/UDP | • MEF-FQDN = FQDN of the MEF • MEFClientRegID = Identifier for the MEF Client registration record being updated • (optional) expirationTime = time when the MEF Client registration record shall expire • (optional) labels = labels to aid discovery of the MEF Client registration record • NOTE: At least one of expirationTime and labels shall be included. |
| 4 |  | IOP Check | Check if possible that MEF has updated the MEF Client Registration record with the proposed values |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Client Registration Update response • (optional) expirationTime = time when the MEF Client registration record shall expire • (optional) labels = labels to aid discovery of the MEF Client registration record NOTE: The response only includes expirationTime and/or labels if those parameters were present in the corresponding request. |
| 6 |  | IOP Check | Check if possible that MEF Client has stored parameters provided by the MEF |

#### 8.4.4.6 MEF Client De-Registration Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_15 |
| --- | --- |
| Objective: | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.6 |

Pre-test conditions:

The MEF Client has previously performed the MEF Client Registration procedure to create the MEF Client Registration record

The MEF Client Registration record is not expired

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Client De-Registration request |
| 3 | Mca | PRO Check TCP/UDP | • MEF-FQDN = FQDN of the MEF • MEFClientRegID = Identifier for the MEF Client Registration record being ended |
| 4 |  | IOP Check | Check if possible that MEF has deleted the information associated with the identified MEF Client Registration record |
| 5 |  | IOP Check | The MEF sends a MEF Client Registration Update response. The MEF Client indicates the success of the operation |

#### 8.4.4.7 MEF Key Registration Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_16 |
| --- | --- |
| Objective: | Source MEF Client establishes a symmetric key with the MEF which can be retrieved for use by one or more Target MEF Clients |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.7 |

Pre-test conditions:

The Source MEF Client is provided with (or has otherwise determined) the information in the MEF Key Registration Configuration (oneM2M TS 0003 [12], clause 8.3.7.3)

The Source MEF Client has performed the MEF Client Registration procedure (oneM2M TS-0003 [12], clause 8.3.5.2.3) with the MEF for the administrating stakeholder identified in the MEF Key Registration Configuration

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The Source MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Key Registration request |
| 3 | Mca | PRO Check TCP/UDP | • MEF-FQDN = FQDN of the MEF • expirationTime = time when the MEF Client Registration shall expire • adminFQDN = Identifier for the administrating stakeholder • SUID = The Security Usage Identifier limiting the security feature in which the symmetric key may be used • (optional) targetIDs = list of identifiers for the initial set of Target MEF Clients authorized to retrieve the symmetric key • (optional) Key Value = output symmetric key value which is self-generated by the Source MEF Client |
| 4 |  | IOP Check | If the MEF Key Registration request included Key Value, check that MEF has stored the value. Otherwise, MEF generates Key Value from the (D)TLS session using TLS Key Export |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Key Registration response: • RelativeKeyID = the relative part of the Key Identifier associated with the Key Registration • expirationTime = time when the MEF Client Registration record shall expire • Source MEF Client ID = Identifier of the Source MEF Client • adminFQDN = FQDN of the administrating stakeholder • SUID = the Security Usage Identifier limiting the security feature in which the symmetric key may be used • targetIDs =list of identifiers for the initial set of Target MEF Clients authorized to retrieve the symmetric key |
| 6 |  | IOP Check | Check if possible that the Source MEF Client and MEF has stored the output symmetric key value and corresponding Key Identifier |

#### 8.4.4.8 MEF Key Retrieval Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_17 |
| --- | --- |
| Objective: | The Target MEF Client to retrieve the Key Value from a MEF corresponding to a RelativeKeyID received by the Target MEF Client |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.8 |

Pre-test conditions:

The Target MEF Client has performed the MEF Client Credential Configuration with the MEF, including configuration of the MEF Key Retrieval URI

The Source MEF Client has performed the MEF Key Registration procedure with the MEF, resulting in a registered Key Value and assigned RelativeKeyID for a specific administrating stakeholder and Security Usage Identifier

The Target MEF Client received a Key Identifier from the Initiating-MEF Client in a security feature with the SUID which the Source MEF Client provided to the MEF during the MEF Key Registration procedure

The Target MEF Client may expect that it is authorized to obtain the corresponding output symmetric key value

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Key Retrieval request |
| 3 | Mca | PRO Check Primitive | • RelativeKeyID = The relative part of the Key Identifier received from the Source MEF Client in a security feature |
| 4 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Key Retrieval response: • expirationTime = time when the Key Registration shall expire • Source MEF Client ID = Identifier of the Source MEF Client • adminFQDN = Identifier for the administrating stakeholder • SUID = the Security Usage Identifier limiting the security feature in which the symmetric key may be used • Key Value = The registered value of the output symmetric key |

#### 8.4.4.9 MEF Key Registration Update Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_18 |
| --- | --- |
| Objective: | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels. |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.9 |

Pre-test conditions:

The MEF Client has previously performed the MEF Key Registration procedure to create the key registration

The key registration is not expired

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client shall send a MEF Key Registration Update request |
| 3 | Mca | PRO Check TCP/UDP | • MEF-FQDN = FQDN of the MEF • RelativeKeyID = the relative part of the Key Identifier associated with the Key Registration • (optional) expirationTime = time when the Key Registration shall expire • (optional) labels = labels to aid discovery of the registered key • (optional) targetIDs = proposed list of identifiers for the set of Target MEF Clients authorized to retrieve the symmetric key NOTE: At least one of expirationTime, labels or targetIDs shall be included. |
| 4 |  | IOP Check | Check if possible that MEF has updated the metadata with the proposed values |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Key Registration Update response • (optional) expirationTime = current time when the key registration shall expire • (optional) labels = Updated list of labels to aid discovery of the Key Registration, if any • (optional) targetIDs = current list of identifiers for the initial set of Target MEF Clients authorized to retrieve the symmetric key NOTE: The response includes only those parameters that were present in the corresponding request. |

#### 8.4.4.10 MEF Key De-Registration Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SE_19 |
| --- | --- |
| Objective: | Source MEF Client requests the MEF to stop distributing the registered key |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.3.5.2.10 |

Pre-test conditions:

The MEF Client has previously performed the MEF Key Registration procedure to create the key registration

The key registration is not expired

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Key De-Registration request |
| 3 | Mca | PRO Check TCP/UDP | • MEF-FQDN = FQDN of the MEF • RelativeKeyID = the relative part of the Key Identifier associated with the Key Registration |
| 4 |  | IOP Check | Check if possible that MEF has deleted the information associated with the identified key registration |
| 5 |  | IOP Check | The MEF sends a MEF Client De-Registration response. The MEF client indicates success of the operation |

### 8.4.5 End-to-End security management

#### 8.4.5.1 End-to-End Security of Primitives (ESPrim) Architecture

Interoperability Test Description

| Identifier: | TD_M2M_SE_20 |
| --- | --- |
| Objective: | AE sends an arbitrary request primitive inside of ESPrim Object to CSE |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.4.2 |

Pre-test conditions:

AE and CSE has established a secure ESPrim connection, so that both are able to extract ESPrim Objects sent from each other

AE has produced an ESPrim Object from the serialization of the arbitrary request primitive

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a NOTIFY Request Message with ESPrim Object |
| 2 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = {CSEBaseName} • from = AE-ID • rqi = (token-string) • pc: {seci: {sit = “esprimObject”, epo: serialized ESPrim Object }} |
| 3 |  | IOP Check | Check if possible that the CSE successfully extracted the inner request primitive Check if possible that the CSE successfully processed the inner request primitive |
| 4 | Mca | PRO Check Primitive | The CSE sends a NOTIFY response to the AE: • op = 5 (Notify) • to = AE-ID • from = CSE-ID • rqi = (token-string) • pc: {seci: {sit = “esprimObject”, epo: serialized ESPrim Object }} |
| 5 |  | IOP Check | Check that the AE successfully extracted the inner response primitive Check that the AE successfully processed the inner response primitive |

#### 8.4.5.2 End-to-End Certificate-based Key Establishment (ESCertKE)

Interoperability Test Description

| Identifier: | TD_M2M_SE_21 |
| --- | --- |
| Objective: | AE establishes a connection with the Registrar CSE using pairwiseE2EKey |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0003 [12], clause 8.7.2 |

Pre-test conditions:

Both the Registrar CSE and AE support ESCertKE and are provisioned with private key and certificates. Both entities are configured with the information needed for the authentication and identification

Cipher Suite = TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends an ESCertKE Message 1 in Notify request |
| 2 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = {CSEBaseName} • from = AE-ID • rqi = (token-string) • pc: {seci: {sit = “escertkeMessage”,eckm: ESCertKE Message 1 }} • ESCertKE Message 1 includes TLS a Client Hello handshake message: • Handshake Type = 0x01 (Client Hello) • Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • Version: TLS v1.2 |
| 3 | Mca | PRO Check Primitive | The Registrar CSE sends an ESCertKE Message 2 in Notify response: • op = 5 (Notify) • to = AE-ID • from = CSE-ID • rqi = (token-string) • pc: {seci: {sit = “escertkeMessage”,eckm: ESCertKE Message 2 }} ESCertKE Message 2 includes Server Hello, Certificate, Server Key Exchange, Certificate Request, Server Hello Done messages Server Hello handshake message: • Handshake Type = 0x02 (Server Hello) • Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 • Version: TLS v1.2 Certificate handshake message: • Handshake Type = 0x0b (Server Certificate) • Certificate: the Registrar CSE certificate Server Key Exchange handshake message: • Handshake Type = 0x0c (Server Key Exchange) • Public key: ECDHE generated key Certificate Request handshake message • Handshake Type = 0x0d (Certificate Request) Server Hello Done handshake message • Handshake Type = 0x0e (Server Hello Done) |
| 4 |  | IOP Check | The TLS client on AE checks if the certificate of the Server is valid |
| 5 |  | Stimulus | AE sends an ESCertKE Message 3 in Notify request |
| 6 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = {CSEBaseName} • from = AE-ID • rqi = (token-string) • pc: {seci: {sit = “escertkeMessage”,eckm: ESCertKE Message 3 }} ESCertKE Message 3 includes Certificate, Client Key exchange, Certificate Verify, Change Cipher Spec, Finished messages Certificate handshake message: • Handshake Type = 0x0b (Client Certificate) • Certificate: AE certificate Client Key Exchange message: • Handshake Type = 0x10 (Client Key Exchange) • Public key: ECDHE generated key Certificate Verify message: • Handshake Type = 0x0f (Certificate Verify) Change Cipher Spec message: • Content type = 0x14 (Change Cipher Spec) Finished handshake message: • Handshake Type = 0x14 (Client Finished) |
| 7 |  | IOP Check | The TLS server on CSE checks if the certificate of the Client is valid |
| 8 | Mca | PRO Check Primitive | The Registrar CSE sends an ESCertKE Message 2 in Notify response: • op = 5 (Notify) • to = AE-ID • from = CSE-ID • rqi = (token-string) pc: {seci: {sit = “escertkeMessage”,eckm: ESCertKE Message 4 }} ESCertKE Message 4 includes Change Cipher Spec, and Finished messages Server Change Cipher Spec message: • Content type = 0x14 (Change Cipher Spec) Server Finished message: • Handshake Type = 0x14 (Client Finished) |
| 9 |  | IOP Check | Check that The TLS client authenticated the Server by validating Verify Data |
| 10 |  | IOP Check | Check that AE and the Registrar CSE has generated and cached a pairwiseE2EKey |

## 8.5 HAIM Device Model

### 8.5.1 HAIM Light Device Creation

Interoperability Test Description

| Identifier: | TD_M2M_NH_102 |
| --- | --- |
| Objective: | AE1 creates a HAIM Light Device Model |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0023 [14], clause 5.5.27 |

Pre-test conditions:

AE1 has created an application resource <AE> on registrar CSE

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE1> resource • fr = AE-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| 6 |  | Stimulus | AE1 sends a request to create a <flexContainer> for binarySwitch |
| 7 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE1> resource/resource name of deviceLight • fr = AE-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 8 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE |
| 9 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE indicates successful operation |

NOTE:

Optional: Repeat steps 5-10 for additional deviceLight Modules

### 8.5.2 HAIM Light Device Status Read

Interoperability Test Description

| Identifier: | TD_M2M_NH_103 |
| --- | --- |
| Objective: | AE2 reads the status of a HAIM Light Device Model |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0023 [14], clauses 5.5.27, 5.3.12 |

Pre-test conditions:

AE2 has created an application resource <AE> on registrar CSE

AE1 has created a HAIM Light Device model

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 sends a request to retrieve a <flexContainer> for binarySwitch |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <deviceLight> resource/binarySwitch • fr = AE-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 4 |  | IOP Check | AE2 indicates successful operation |

NOTE:

Optional: Repeat steps 1-4 for additional deviceLight Modules

### 8.5.3 HAIM Light Device Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_104 |
| --- | --- |
| Objective: | AE2 turns the binarySwitch of a HAIM Light Device Model “ON” or “OFF” |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0023 [14], clause 5.5.27 |

Pre-test conditions:

AE2 has created an application resource <AE> on registrar CSE

AE1 has created a HAIM Light Device model

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/ URI of <deviceLight> resource/binarySwitch • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <flexContainer> resource with new value for powerState |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is updates in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |

NOTE:

Optional: Repeat steps 1-5 for additional device states and settings

### 8.5.4 HAIM Light Device Toggle Action

Interoperability Test Description

| Identifier: | TD_M2M_NH_105 |
| --- | --- |
| Objective: | AE2 toggles the state of a HAIM Light Device Model |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0023 [14], clause 5.5.27 |

Pre-test conditions:

AE2 has created an application resource <AE> on registrar CSE

AE1 has created a HAIM Light Device model

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/ URI of <deviceLight> resource/binarySwitch/toggle • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <flexContainer> resource for toggle action |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is updates in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE2 indicates successful operation. Check that the powerState of the binarySwitch is updated |

NOTE:

Optional: Repeat steps 1-5 for additional device states and settings

### 8.5.5 HAIM Power Outlet SubDevice Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_160 |
| --- | --- |
| Objective: | AE1 creates a Power Outlet SubDevice Model |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0023 [14], clause 5.4.1.2 |

Pre-test conditions:

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a <flexContainer> for deviceSmartPlug

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> for subDevicePowerOutlet |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE1> resource/resource name of deviceSmartPlug • fr = AE-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| 6 |  | Stimulus | AE1 sends a request to create a <flexContainer> for binarySwitch |
| 7 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE1> resource/resource name of subDevicePowerOutlet • fr = AE-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 8 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE |
| 9 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE indicates successful operation |

NOTE:

Optional: Repeat steps 6-10 for additional subDevicePowerOutlet Modules

### 8.5.6 HAIM Toggle Action Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_161 |
| --- | --- |
| Objective: | AE1 creates a Toggle Action Model |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0023 [14], clause 5.3.1.12 |

Pre-test conditions:

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a <flexContainer> for deviceSmartPlug

AE1 has created a <flexContainer> for binarySwtich as a child of deviceSmartPlug

Test Sequence

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> for Toggle |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE1> resource/resource name of binarySwtich • fr = AE-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.5.7 HAIM Device Properties Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_162 |
| --- | --- |
| Objective: | AE1 creates Device Properties Model |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0023 [14], clause 6.2.5 |

Pre-test conditions:

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a <flexContainer> for deviceLight

AE1 has created a <node> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 sends a request to create a [deviceInfo] resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <node> resource/ • fr = AE-ID • rqi = (token-string) • ty = 13 (mgmtObj) • pc = Serialized representation of [deviceInfo] resource with with properties set as attributes of the resource |
| 3 |  | IOP Check | Check if possible that the [deviceInfo] resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of [deviceInfo] resource |
| 5 |  | IOP Check | AE indicates successful operation |

## 8.6 Semantics management

### 8.6.1 Semantic Access Control Policy management

#### 8.6.1.1 Procedure for creating ACP triples when a new <accessControlPolicy> resource is created

Interoperability Test Description

| Identifier: | TD_M2M_NH_106 |
| --- | --- |
| Objective: | ACP triples are created when a new <accessControlPolicy> resource is created |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.2.1.5.2 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

The Registrar CSE has SGS available

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create an < accessControlPolicy> resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/<AE> • fr = AE-ID • rqi = (token-string) • ty = 1 (accessControlPolicy) • pc = Serialized representation of < accessControlPolicy > resource |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource is created in Registrar CSE Check if possible that Registrar CSE has created ACP Triples in SGS for the new <accessControlPolicy> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of < accessControlPolicy > resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.1.2 Procedure for updating ACP triples when a new <accessControlPolicy> resource is updated

Interoperability Test Description

| Identifier: | TD_M2M_NH_107 |
| --- | --- |
| Objective: | ACP triples are updated when an existing <accessControlPolicy> resource is updated |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.2.1.5.3 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

The Registrar CSE has SGS available

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to update a privileges attribute of {accessControlPolicyName} |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/{AE}/{accessControlPolicyName} • fr = AE-ID • rqi = (token-string) • ty = 1 (accessControlPolicy) • pc = Serialized representation of updated <accessControlPolicy> resource |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been updated in Registrar CSE Check if possible that Registrar CSE has updated corresponding ACP Triples in SGS |
| 4 | Mca | PRO Check Primitive | • Registrar CSE sends response containing: rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.1.3 Procedure for deleting ACP triples when an existing <accessControlPolicy> resource is deleted

Interoperability Test Description

| Identifier: | TD_M2M_NH_108 |
| --- | --- |
| Objective: | ACP triples are deleted when an existing <accessControlPolicy> resource is deleted |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.2.1.5.4 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

The Registrar CSE has SGS available

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy delete request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/{AE}/{accessControlPolicyName} • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been removed from registrar CSE Check if possible that Registrar CSE has deleted corresponding ACP Triples in SGS |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.1.4 Procedure for creating ACP-SD binding triples and SD relationship in SGS

Interoperability Test Description

| Identifier: | TD_M2M_NH_109 |
| --- | --- |
| Objective: | ACP-SD Binding Triples and SD relationship in SGS are created when AE creates a <semanticDescriptor> resource in Registrar CSE |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.2.1.5.5 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

The Registrar CSE has SGS available

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create an < semanticDescriptor> resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/<AE> • fr = AE-ID • rqi = (token-string) • ty = 24 (semanticDescriptor) • pc = Serialized representation of <semanticDescriptor> resource • acpi = URI of {accessControlPolicyName} |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE. Check if possible that Registrar CSE has created SD Relationship Triples and ACP-SD Binding Triples for the new <semanticDescriptor> in SGS |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.1.5 Procedure for updating ACP-SD binding triples in SGS

Interoperability Test Description

| Identifier: | TD_M2M_NH_110 |
| --- | --- |
| Objective: | ACP-SD Binding Triples are updated when the accessControlPolicyIDs attribute of a <semanticDescriptor> resource is updated |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.2.1.5.6 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource

The Registrar CSE has SGS available

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to update an accessControlPolicyIDs attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <semanticDescriptor> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. Check if possible that Registrar CSE has updated corresponding ACP-SD Binding Triples in SGS |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.1.6 Procedure for updating SD relationship triples in SGS

Interoperability Test Description

| Identifier: | TD_M2M_NH_111 |
| --- | --- |
| Objective: | SD Relationship Triples are updated when the descriptor attribute of a <semanticDescriptor> resource is changed |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.2.1.5.7 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource

The Registrar CSE has SGS available

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Update Request to update the descriptor attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <semanticDescriptor> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. Check if possible that Registrar CSE has updated old SD Relationship Triples and/or add new SD Relationship Triple in the SGS |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: • rsc = 2004 (UPDATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.1.7 Procedure for deleting SD relationship triples and ACP-SD binding triples in SGS

Interoperability Test Description

| Identifier: | TD_M2M_NH_112 |
| --- | --- |
| Objective: | SD Relationship Triples are deleted when the descriptor attribute of a <semanticDescriptor> resource is deleted |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.2.1.5.8 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}

AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource

The Registrar CSE has SGS available

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Delete Request to update the descriptor attribute of the resource |
| 2 | Check Mca | PRO Check Primitive | • op = 4 (Detete) • to = {CSEBaseName}/URI of <semanticDescriptor> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is deleted in Registrar CSE. Check if possible that Registrar CSE has removed SD Original Triples, SD Relationship Triples, and ACP-SD Binding Triples related to the <semanticDescriptor> from SGS |
| 4 | Check Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.6.2 Semantic Filtering and discovery

#### 8.6.2.1 Semantic Filtering and Discovery using <semanticFanOutPoint> resource

Interoperability Test Description

| Identifier: | TD_M2M_NH_113 |
| --- | --- |
| Objective: | AE discovers accessible resources residing in Registrar CSE using the <semanticFanOutPoint> |
| Configuration: | M2M_CFG_01 |
| References: | oneM2M TS-0034 [13], clause 7.4.1 |

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created <container> resources {container1} and {container2} in registrar CSE under <AE> resource

AE has created a group resource with semanticSupportIndicator attribute set to TRUE and memberIds set to {container1} and {container2} ids

AE has created <semanticDescriptor> as a child resources of a resources {container1} and {container2}

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Discovery request to discover the < semanticFanOutPoint > virtual resource of <group> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/<AE>/<group>/sfopt • from = AE-ID • rqi = (token-string) • fu=1 • smf=sparqlQuery1 • pc = empty |
| 4 | Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the <Container> resources addresses |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.2.2 Resource link-based Semantic Discovery

Interoperability Test Description +——————-+————————————————————————————————————–+ | Identifier: | TD_M2M_NH_114 | +——————-+————————————————————————————————————–+ | Objective: | AE discovers accessible resources residing in Registrar CSE using the resource link-based Semantic Discovery | +——————-+————————————————————————————————————–+ | Configuration:| M2M_CFG_01 | +——————-+————————————————————————————————————–+ | References: | oneM2M TS-0034 [13], clause 7.4.3 | +——————-+————————————————————————————————————–+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created <container> resources {container1} and {container2} in registrar CSE under <AE> resource

AE has created a group resource with semanticSupportIndicator attribute set to TRUE and memberIds set to {container1} and {container2} ids

AE has created <semanticDescriptor> resources as a child resources of a resources {container1} and {container2} named {descriptor1} and {descriptor2}

{descriptor1} has relatedSemantics attribute set to ID of {descriptor2}

{descriptor2} has relatedSemantics attribute set to ID of {descriptor1}

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request to discover the <container> resource using the semanticFilter filterCriteria |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/<AE> • from = AE-ID • rqi = (token-string) • fu=1 • smf=sparqlQuery1 • pc = empty |
| 4 | Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of data object containing the <Container> resources addresses |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.2.3 Semantic query

Interoperability Test Description

| Identifier: | TD_M2M_NH_115 |
| --- | --- |
| Objective: | AE performs a Semantic Query request in Registrar CSE using the semanticFilter filter criteria |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.3.3.18| +——————-+———————————————————————————————————————-+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE1 has created a container resource <container> on Registrar CSE

AE1 has created a <semanticDescriptor> as a child resource of a <container>

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 is requested to send a Semantic Query Operation request to query the <container> resource using a SPARQL query |
| 2 | Check Mca | PRO Check Primitive | Sent request contains: • op = 2 (Retrieve) • to = {CSEBaseName} • from = AE-ID • rqi = (token-string) • sqi = TRUE • smf=sparqlQuery1 • rcn = 10 (semantic content) • pc = empty |
| 3 | Check Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation semantic query result |
| 4 |  | IOP Check | AE indicates notification received |

### 8.6.3 Semantic Mashup management

#### 8.6.3.1 SemanticMashupJobProfile Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_116 |
| --- | --- |
| Objective: | AE creates a SemanticMashupJobProfile resource in Registrar CSE via a SemanticMashupJobProfile Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.49.2.1 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <semanticMashupJobProfile> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 40 (semanticMashupJobProfile) • pc = Serialized representation of <semanticMashupJobProfile> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupJobProfile> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.2 SemanticMashupJobProfile Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_117 |
| --- | --- |
| Objective: | AE retrieves information of a semanticMashupJobProfile resource via a semanticMashupJobProfile Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.49.2.2 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a semanticMashupJobProfile resource <semanticMashupJobProfile> as child resource of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticMashupJobProfile> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <semanticMashupJobProfile> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupJobProfile> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.3 SemanticMashupJobProfile Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_118 |
| --- | --- |
| Objective: | AE updates attribute in <semanticMashupJobProfile> resource via a semanticMashupJobProfile Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.49.2.3 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a semanticMashupJobProfile resource <semanticMashupJobProfile> as child resource of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticMashupJobProfile Update Request to update the memberFilter attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <semanticMashupJobProfile> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <semanticMashupJobProfile> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupJobProfile> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.4 SemanticMashupJobProfile Delete

Interoperability Test Description +——————-+—————————————————————————————————————————+ | Identifier: | TD_M2M_NH_119 | +——————-+—————————————————————————————————————————+ | Objective: | AE deletes semanticMashupJobProfile resource via a semanticMashupJobProfile Delete Request | +——————-+—————————————————————————————————————————+ | Configuration:| M2M_CFG_01 | +——————-+—————————————————————————————————————————+ | References: | oneM2M TS-0034 [13], clause 6.3.5
oneM2M TS-0004 [2], clause 7.4.49.2.4 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

AE has created a semanticMashupJobProfile resource <semanticMashupJobProfile> as child resource of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticMashupJobProfile Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <semanticMashupJobProfile> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticMashupJobProfile> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticMashupJobProfile> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.5 SemanticMashupInstance Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_120 |
| --- | --- |
| Objective: | AE creates a semanticMashupInstance resource in Registrar CSE via a semanticMashupInstance Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.50.2.1 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <semanticDescriptor> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 41 (semanticMashupInstance) • pc = Serialized representation of <semanticMashupInstance> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupInstance> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.6 SemanticMashupInstance Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_121 |
| --- | --- |
| Objective: | AE retrieves information of a semanticMashupInstance resource via a semanticMashupInstance Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.50.2.2 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

- AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticMashupInstance> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <semanticMashupInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.7 SemanticMashupInstance Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_122 |
| --- | --- |
| Objective: | AE updates attribute in <semanticMashupInstance> resource via a semanticMashupInstance Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.50.2.3 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

- AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticMashupInstance Update Request to update the smjpInputParameter attribute of the resource. |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <semanticMashupInstance> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <semanticMashupInstance> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupInstance> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.8 SemanticMashupInstance Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_123 |
| --- | --- |
| Objective: | AE deletes semanticMashupInstance resource via a semanticMashupInstance Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.50.2.4 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on Registrar CSE

- AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticMashupInstance Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/ URI of <semanticMashupInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticMashupInstance> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticMashupInstance> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.9 SemanticMashupResult Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_124 |
| --- | --- |
| Objective: | AE retrieves information of a semanticMashupResult resource via a semanticMashupResult Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.52.2.2 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource

<semanticMashupResult> resource is created as child resource of <semanticMashupInstance> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticMashupResult> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <semanticMashupInstance> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupResult> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.10 SemanticMashupResult Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_125 |
| --- | --- |
| Objective: | AE deletes semanticMashupResult resource via a semanticMashupResult Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.52.2.4 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource

<semanticMashupResult> resource is created as child resource of <semanticMashupInstance> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a semanticMashupResult Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/ URI of <semanticMashupInstance> resource/ URI of <semanticMashupResult> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticMashupResult> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticMashupResult> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

#### 8.6.3.11 Mashup Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_126 |
| --- | --- |
| Objective: | AE triggers a calculation and generation of the mashup result by sending a <semanticMashupInstance>/<mashup> Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.51.2.2 | +——————-+—————————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE retrieves a <mashup> resource in a <semanticMashupInstance> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <semanticMashupInstance> resource/msp • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that a <semanticMashupResult> resource is created under the <semanticMashupInstance> resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticMashupResult> resource |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.6.4 Ontology Repository management

#### 8.6.5.1 OntologyRepository Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_127 |
| --- | --- |
| Objective: | AE creates a OntologyRepository resource in Registrar CSE via a OntologyRepository Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.46.2.1 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <ontologyRepository> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 38 (ontologyRepository) • pc = Serialized representation of <ontologyRepository> resource |
| 3 |  | IOP Check | Check if possible that the <ontologyRepository> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <ontologyRepository> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.4.2 OntologyRepository Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_128 |
| --- | --- |
| Objective: | AE retrieves information of a ontologyRepository resource via a ontologyRepository Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.46.2.2 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a ontologyRepository resource <ontologyRepository> as child resource of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <ontologyRepository> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <ontologyRepository> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <ontologyRepository> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.6.4.3 OntologyRepository Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_129 |
| --- | --- |
| Objective: | AE updates attribute in <ontologyRepository> resource via a ontologyRepository Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.46.2.3 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a ontologyRepository resource <ontologyRepository> as child resource of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a ontologyRepository Update Request to update an attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <ontologyRepository> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <ontologyRepository> resource |
| 3 |  | IOP Check | Check if possible that the <ontologyRepository> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <ontologyRepository> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.4.4 OntologyRepository Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_130 |
| --- | --- |
| Objective: | AE deletes OntologyRepository resource via a OntologyRepository Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.46.2.4 | +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a ontologyRepository resource <ontologyRepository> as child of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a ontologyRepository Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <ontologyRepository> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <ontologyRepository> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <ontologyRepository> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

### 8.6.5 Semantic validation management

#### 8.6.5.1 Semantic validation independent of <semanticDescriptor> resource operation

Interoperability Test Description

| Identifier: | TD_M2M_NH_131 |
| --- | --- |
| Objective: | AE checks the validity of the <semanticDescriptor> resource via a <semanticValidation> Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.48.2.3 | +——————-+—————————————————————————————————————————-+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a ontologyRepository resource <ontologyRepository> as child of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a <semanticValidation> Update Request to check the validity of the <semanticDescriptor>resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <semanticDescriptor> resource/smv • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of <semanticValidation> parameters |
| 3 |  | IOP Check | Check if possible that the received <semanticDescriptor> resource with the semanticValidated attribute is set to ‘true’ in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.5.2 Semantic validation triggered when Create a semanticDescriptor resource

Interoperability Test Description

| Identifier: | TD_M2M_NH_132 |
| --- | --- |
| Objective: | AE creates a <semanticDescriptor> resource visa SemanticDescriptor Create Request and Registrar CSE checks the validity of the created <semanticDescriptor> resource |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.34.2.1 | +——————-+——————————————————————————————————————————————————————————–+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a < semanticDescriptor > Create Request with validationEnable attribute set to ‘true’ |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 24 (semanticDescriptor) • pc = Serialized representation of <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE Check if possible that the semanticValidated attribute of the <semanticDescriptor> is set to ‘true’ |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.6.6 Ontology Mapping management

#### 8.6.6.1 OntologyMapping Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_133 |
| --- | --- |
| Objective: | AE creates an OntologyMapping resource in Registrar CSE via an OntologyMapping Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.62.2.1| +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <ontologyMapping> resource |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 52 (ontologyMapping) • pc = Serialized representation of <ontologyMapping>resource |
| 3 |  | IOP Check | Check if possible that the <ontologyMapping> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <ontologyMapping> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.6.2 OntologyMapping Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_134 |
| --- | --- |
| Objective: | AE retrieves information of an ontology mapping result via a ontologyMapping Retrieve Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.62.2.2| +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a ontologyMapping resource <ontologyMapping> as child resource of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <ontologyMapping> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <ontologyMapping> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <ontologyMapping> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.6.6.3 OntologyMapping Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_135 |
| --- | --- |
| Objective: | AE updates attribute in <ontologyMapping> resource via a ontologyMapping Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.62.2.3| +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a ontologyMapping resource <ontologyMapping> as child resource of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a ontologyMapping Update Request to update the mappingPolicy attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <ontologyMapping> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <ontologyMapping> resource |
| 3 |  | IOP Check | Check if possible that the <ontologyMapping> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <ontologyMapping> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.6.6.4 OntologyMapping Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_136 |
| --- | --- |
| Objective: | AE deletes OntologyMapping resource via a OntologyMapping Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.62.2.4| +——————-+—————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a ontologyMapping resource <ontologyMapping> as child of <AE> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a ontologyMapping Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <ontologyMapping> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 |  | IOP Check | Check if possible that the <ontologyMapping> resource is deleted in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 5 |  | IOP Check | Check if possible that the <ontologyMapping> resource has been removed in Registrar CSE |
| 6 |  | IOP Check | AE indicates successful operation |

## 8.7 3GPP Interworking

### 8.7.1 Cellular IoT non-IP data delivery (NIDD)

#### 8.7.1.1 SCEF Configuration for NIDD

Interoperability Test Description

| Identifier: | TD_M2M_SH_27 |
| --- | --- |
| Objective: | IN-CSE establishes SCEF Configuration for NIDD |
| Configuration: | M2M_CFG_09 |
| References: | oneM2M TS-0026 [15], clause 7.1.1.1 |

Pre-test conditions:

UE hosts an ADN-AE node

IN-CSE has a <m2mServiceSubscriptionProfile> resource created as a child of <CSEBase> resource

<serviceSubscribedNode> resource is created as a child of <m2mServiceSubscriptionProfile>

Node-ID attribute of <serviceSubscribedNode> resource is set to M2M-Ext-ID of UE and niddRequired attribute is set to TRUE

SCEF identifier is pre-provisioned to IN-CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | IN-CSE issues a NIDD Configuration Request to SCEF for ADN-AE hosted on a UE |
| 2 | (T8) Mcn | PRO Check HTTP | • Method = POST • URI = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/ The {apiRoot} and{scsAsId} segments are configured based on Service Provider and MNO policies. • Payload shall include NiddConfiguration data structure with the following attributes included in the request: externalId, notificationDestination, duration, pdnEstablishmentOption, duration, pdnEstablishmentOption, reliableDataService, rdsPorts, supportedFeatures |
| 3 |  | IOP Check | Check if possible that the SCEF has successfully processes the NIDD Configuration Request |
| 4 | (T8) Mcn | PRO Check HTTP | SCEF responds for the NIDD Configuration Response: • Status code = 201 (CREATED) • Location header = {apiRoot}/3gpp- nidd /v1/{scsAsId}/configurations/{configurationId} • Payload shall include NiddConfiguration data structure with the following attributes included in the request: maximumPacketSize, status, self |
| 5 |  | IOP Check | IN-CSE indicates successful operation |

#### 8.7.1.2 SCEF-based Mobile Terminated NIDD

Interoperability Test Description

| Identifier: | TD_M2M_SH_28 |
| --- | --- |
| Objective: | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE |
| Configuration: | M2M_CFG_11 |
| References: | oneM2M TS-0026 [15], clause 7.1.1.2 |

Pre-test conditions:

IN-AE has created an Application Entity resource <AE> on IN-CSE

ADN-AE has created an Application Entity resource <AE> on IN-CSE

IN-CSE has a <m2mServiceSubscriptionProfile> resource created as a child of <CSEBase> resource

<serviceSubscribedNode> resource is created as a child of <m2mServiceSubscriptionProfile>

Node-ID attribute of <serviceSubscribedNode> resource is set to M2M-Ext-ID of UE and niddRequired attribute is set to TRUE

SCEF identifier is pre-provisioned to IN-CSE

NIDD configuration procedure is competed successfully

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | (Optional) IN-AE issues an arbitrary oneM2M request targeting an ADN-AE. |
| 2 |  | Stimulus | IN-CSE issues a SCEF-based Mobile Terminated (MT) NIDD Downlink Data Transfer Request |
| 3 | (T8) Mcn | PRO Check HTTP | • Method = POST • URI = {apiRoot}/3gpp- nidd /v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries • Payload shall include NiddDownlinkDataTransfer data structure with the following attributes included in the request: externalId, maximumLatency, priority, pdnEstablishmentOption, pdnEstablishmentOption, reliableDataService, rdsPorts, data (containing onem2m primitive) |
| 4 |  | IOP Check | Check if possible that the SCEF has successfully processes the NIDD Downlink Data Transfer Request |
| 5 | (T8) Mcn | PRO Check HTTP | SCEF responds for the NIDD Downlink Data Transfer Request: • Status code = 201 (CREATED) • Location header = {apiRoot}/3gpp- nidd /v1/{scsAsId}/configurations/{configurationId} • Payload shall include NiddConfiguration data structure with the following attributes included in the request: maximumPacketSize, status, self |
| 6 |  | IOP Check | In case the UE does not have an active NIDD PDN connection to the SCEF, check that SCEF buffered the request until the UE establishes the connection |
| 7 | (T8) Mcn | PRO Check HTTP | SCEF responds with NIDD Downlink Data Transfer Response: • Status code = 200 (OK) / 201 (CREATED, Buffered request) • Location header = {apiRoot}/3gpp- nidd /v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries/{downlinkDataDeliveryId} • Payload shall include NiddDownlinkDataTransfer data structure with the following attributes included in the request: deliviryStatus, self, requestedRetransmissionTime |
| 8 | (T8) Mcn | PRO Check HTTP | (Optional) SCEF returns a MT NIDD Downlink Data Delivery Status Notification to IN-CSE: • Method = POST • URI = {notification _uri } • Payload shall include a NiddDownlinkDataDeliveryStatusNotification data structure with the following attributes included in the request: niddDownlinkDataTransfer, deliveryStatus, requestedRetransmissionTime |
| 9 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the MT NIDD Downlink Data Delivery Status Acknowledgement: • Status code = 204 (NO CONTENT) |
| 10 |  | IOP Check | Check that ADN-AE on UE has executed the oneM2M request primitive accordingly |
| 11 |  | Stimulus | (Optional) ADN-AE hosted on the UE issues a MO NIDD Uplink Data Notification to deliver a oneM2M response primitive back to the Originator |
| 12 | (T8) Mcn | PRO Check HTTP | (Optional) SCEF sends for the NIDD Uplink Data Notification: • Method = POST • URI = {notification _uri } • Payload shall include NiddUplinkDataNotification data structure with the following attributes included in the request: niddConfiguration, externalId, reliableDataService, rdsPort, data |
| 13 | (T8) Mcn | PRO Check HTTP | (Optional) IN-CSE responds with MO NIDD Uplink Data Acknowledgement and sends oneM2M response primitive to IN-AE • Status code = 204 (NO CONTENT) |
| 14 |  | IOP Check | (Optional) Check that IN-AE received a corresponding oneM2M response primitive |

#### 8.7.1.3 SCEF-based Mobile Originated NIDD

Interoperability Test Description

| Identifier: | TD_M2M_SH_29 |
| --- | --- |
| Objective: | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE |
| Configuration: | M2M_CFG_11 |
| References: | oneM2M TS-0026 [15], clause 7.1.1.3 |

Pre-test conditions:

IN-AE has created an Application Entity resource <AE> on IN-CSE

ADN-AE has created an Application Entity resource <AE> on IN-CSE

IN-CSE has a <m2mServiceSubscriptionProfile> resource created as a child of <CSEBase> resource

<serviceSubscribedNode> resource is created as a child of <m2mServiceSubscriptionProfile>

Node-ID attribute of <serviceSubscribedNode> resource is set to M2M-Ext-ID of UE and niddRequired attribute is set to TRUE

SCEF identifier is pre-provisioned to IN-CSE

NIDD configuration procedure is competed successfully

RDS source and destination port numbers are pre-provisioned in ADN-AE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | ADN-AE issues MO NIDD Uplink Data Notification to deliver an arbitrary primitive to the IN-CSE |
| 2 | (T8) Mcn | PRO Check HTTP | SCEF triggers a MO NIDD Uplink Data Notification carrying request primitive: • Method = POST • URI = notification _uri } • Payload shall include NiddUplinkDataNotification data structure with the following attributes included in the request: niddConfiguration, externalId, reliableDataService, rdsPort, data (containing onem2m primitive) |
| 3 | (T8) Mcn | PRO Check HTTP | IN-CSE responds with MO NIDD Uplink Data Acknowledgement: • Status code = 204 (NO CONTENT) |
| 4 |  | IOP Check | Check if possible that SCEF has processed the MO NIDD Uplink Data Acknowledgement from the IN-CSE |
| 5 |  | PRO Check | SCEF sends an RDS acknowledgment to the UE: • Status code = 204 (NO CONTENT) |
| 6 |  | IOP Check | Check if possible that IN-CSE processes the oneM2M request primitive |
| 7 | (T8) Mcn | PRO Check HTTP | (Optional) If a response is required, IN-CSE generates a oneM2M response and sends a MT NIDD Downlink Data Transfer Request: • Method = POST • URI = {apiRoot}/3gpp- nidd /v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries • Payload shall include NiddDownlinkDataTransfer data structure with the following attributes included in the request:externalId, maximumLatency, priority, pdnEstablishmentOption, (optional) reliableDataService, rdsPort, data (containing response to oneM2M primitive) |
| 8 | (T8) Mcn | PRO Check HTTP | (Optional) Scef returns MT NIDD Downlink Data Transfer Response to IN-CSE: • Status code = 200 (OK) / 201 (Created) • URI = {apiRoot}/3gpp- nidd /v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries/{downlinkDataDeliveryId} Payload may include NiddDownlinkDataTransfer data structure with the following attributes included in the response: deliveryStatus, self, requestedRetransmissionTime |
| 9 |  | IOP Check | (Optional) Check if possible that SCEF has processed the request and delivered it to the targeted UE (Optional) Check if possible that UE has responded with an RDS acknowledgment |
| 10 | (T8) Mcn | PRO Check HTTP | (Optional) SCEF returns MT NIDD Downlink Data Delivery Status Notification to IN-CSE: • Method = POST • URI = {notification _uri } • Payload shall include NiddDownlinkDataDeliveryStatusNotification data structure with the following attributes included in the request: niddDownlinkDataTransfer, deliveryStatus, requestedRetransmissionTime |
| 11 | (T8) Mcn | PRO Check HTTP | (Optional) IN-CSE responds to SCEF: • Status code = 204 (NO CONTENT) |
| 12 |  | IOP Check | (Optional) Check if possible that ADN-AE has processed the oneM2M response primitive |

### 8.7.2 Monitoring events

#### 8.7.2.1 UE Reachability monitoring

Interoperability Test Description

| Identifier: | TD_M2M_SH_30 |
| --- | --- |
| Objective: | IN-AE monitors UE Reachability status |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0026 [15], clause 7.4.1 |

Pre-test conditions:

UE, SCEF and IN-CSE are attached to the underlying 3GPP network

IN-AE has created an Application Entity resource <AE> on IN-CSE

ADN-AE has created an Application Entity resource <AE> on IN-CSE

ADN-AE has created a Node resource <node> on IN-CSE representing UE

ADN-AE has created a Schedule resource <schedule> on IN-CSE under <node> resource. The networkCoordinated attribute is set to TRUE.

IN-AE has subscribed to the <schedule> resource by creating a child <subscription> resource.

IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = UE_REACHABILITY).

UE is in idle mode

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | UE transitions to connected mode |
| 2 |  | IOP Check | Check if possible that the 3GPP network entities (e.g. HSS) has detected the condition and sent a Monitoring Event Report to SCEF |
| 3 | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification UE forREACHABILITY to IN-CSE: • Method = POST • URI = {notification _uri_ } • Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, idleStatusInfo, reachabilityType) |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the UE Reachability Monitoring Notification: Status code = 204 (NO CONTENT) |
| 5 |  | IOP Check | Check if possible that if idleStatusInfo information is provided in the report, IN-CSE has updated scheduleElement attribute of the <schedule> resource |
| 6 | (T8) Mcn | PRO Check Primitive | IN-CSE sends a Notify message to IN-CSE: • op = 6 (Notify) • pc = serialized representation of the updated <schedule> resource |
| 7 |  | IOP Check | Check if possible that ADN-AE on UE has updated its local <schedule> resource (if applicable) |

#### 8.7.2.2 UE Availability after DDN Failure

Interoperability Test Description

| Identifier: | TD_M2M_SH_31 |
| --- | --- |
| Objective: | UE Availability after DDN Failure scenario |
| Configuration: | M2M_CFG_11 |
| References: | oneM2M TS-0026 [15], clause 7.4.2 |

Pre-test conditions:

UE, SCEF and IN-CSE are attached to the underlying 3GPP network

IN-AE has created an Application Entity resource <AE> on IN-CSE

ADN-AE has created an Application Entity resource <AE> on IN-CSE

ADN-AE has created a Node resource <node> on IN-CSE representing UE

ADN-AE has created a Schedule resource <schedule> on IN-CSE under <node> resource. The networkCoordinated attribute is set to TRUE.

IN-AE has subscribed to the <schedule> resource by creating a child <subscription> resource.

IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = AVAILABILITY_AFTER_DDN_FAILURE)

UE is in unreachable for Downlink data and in state that DDN Failure condition can be reproduced

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | IN-AE issues an arbitrary oneM2M request targeting an ADN-AE |
| 2 |  | IOP Check | Check that no response for UE paging is received Check if possible that if UE is in PSM mode, the UE subscription has been updated to reflect that a notification of availability should be sent after this DDN failure |
| 3 |  | Stimulus | UE contacts the network |
| 4 |  | IOP Check | Check if possible that SCEF has received a Monitoring Indication that the UE is available |
| 5 | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification for AVAILABILITYAFTER_DDN_FAILURE to IN-CSE: • Method = POST • URI = {notification _uri_ } • Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, idleStatusInfo) |
| 6 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the DDN Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 7 |  | IOP Check | Check if possible that IN-CSE has updated the <schedule> resource to indicate that UE is available and created new scheduleElement Check if possible that notification has been sent to the <schedule> resource subscribed entities |
| 8 |  | Stimulus | UE transitions to Idle |
| 9 | (T8) Mcn | PRO Check HTTP | SCEF sends a UE Reachability Monitoring Event Notification Request to IN-CSE: • Method = POST • URI = {notification _uri } • Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, idleStatusInfo) |
| 10 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the UE Reachability Monitoring Notification: Status code = 204 (NO CONTENT) |
| 11 |  | IOP Check | Check if possible that IN-CSE has updated the <schedule> resource to indicate that UE is idle Check if possible that notification has been sent to the <schedule> resource subscribed entities |

#### 8.7.2.3 UE Communication Failure

Interoperability Test Description

| Identifier: | TD_M2M_SH_32 |
| --- | --- |
| Objective: | UE Communication Failure scenario |
| Configuration: | M2M_CFG_11 |
| References: | oneM2M TS-0026 [15], clause 7.4.3 |

Pre-test conditions:

- UE, SCEF and IN-CSE are attached to the underlying 3GPP network

- IN-AE has created an Application Entity resource &lt;AE> on IN-CSE

- ADN-AE has created an Application Entity resource &lt;AE> on IN-CSE

- ADN-AE has created a Node resource &lt;node> on IN-CSE representing UE

- ADN-AE has created a Schedule resource &lt;schedule> on IN-CSE under &lt;node> resource. The networkCoordinated attribute is set to TRUE

- IN-AE has subscribed to the &lt;schedule> resource by creating a child &lt;subscription> resource.

- IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = COMMUNICATION\_FAILURE)

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | UE communication failure occurs |
| 2 |  | IOP Check | Check if possible SCEF has received a Monitoring Event Report |
| 3 | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification Report for COMMUNICATIONFAILURE to IN-CSE: • Method = POST • URI = {notification _uri_ } • Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, failureCause). |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the Monitoring Notification Report request: Status code = 204 (NO CONTENT) |
| 5 |  | IOP Check | Check if possible that IN-CSE has updated the <schedule> resource to indicate that UE is available and created new scheduleElement |
| 6 | (T8) Mcn | PRO Check HTTP | SCEF sends a UE Communication Failure Monitoring Event Notification Request to IN-CSE: • Method = POST • URI = {notification _uri } • Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, failureCause) |
| 7 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the UE Communication Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 8 |  | IOP Check | Check if possible that IN-CSE has updated the scheduleElement of the <schedule> resource to indicate that no communications are currently available Check if possible that notification has been sent to the <schedule> resource subscribed entities |

#### 8.7.2.4 Roaming Status

Interoperability Test Description

| Identifier: | TD_M2M_SH_33 |
| --- | --- |
| Objective: | Roaming status scenario |
| Configuration: | M2M_CFG_09 |
| References: | oneM2M TS-0026 [15], clause 7.4.6 |

Pre-test conditions:

- UE, SCEF and IN-CSE are attached to the underlying 3GPP network

- IN-CSE can make Roaming Status Reports requests

- ADN-AE has created an Application Entity resource &lt;AE> on IN-CSE

- ADN-AE has created a Node resource &lt;node> on IN-CSE representing UE. roamingStatus and networkID attributes of &lt;node> resource are configured.

- IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = ROAMING\_STATUS)

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | Roaming status of UE has changed |
| 2 |  | IOP Check | Check if possible SCEF has received a Monitoring Event Report |
| 3\ | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification Report for ROAMINGSTATUS to IN-CSE: • Method = POST • URI = {notification _uri_ } • Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, plmnId, roamingStatus) |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the DDN Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 5 |  | IOP Check | Check if possible that IN-CSE has updated roamingStatus and networkID attributes the <node> resource |

#### 8.7.2.5 Location updating triggered by retrieval

Interoperability Test Description

| Identifier: | TD_M2M_SH_34 |
| --- | --- |
| Objective: | Location Reporting scenario |
| Configuration: | M2M_CFG_09 |
| References: | oneM2M TS-0026 [15], clause 7.4.7.2 |

Pre-test conditions:

- UE, SCEF and IN-CSE are attached to the underlying 3GPP network

- ADN-AE has created an Application Entity resource &lt;AE> on IN-CSE

- IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = LOCATION\_REPORTING)

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | IN-AE sends a <locationPolicy> CREATE request |
| 2 | (T8) Mcn | PRO Check Primitive | • op = 1 (Create); • to = {CSEBaseName}; • fr = IN-AE-ID; • rqi = (token-string); • ty = 10 (LocationPolicy); • pc = Serialized representation of <locationPolicy> resource; ◦ locationSource = Network based; ◦ locationUpdatePeriod = 0; ◦ locationTargetID = M2M-Ext-ID of the UE; ◦ locationInformationType = position fix; ◦ retrieveLastKnownLocation = TRUE/FALSE |
| 3 |  | IOP Check | Check if possible that IN-CSE has created <locationPolicy> resource |
| 4 |  | Stimulus | AE is requested to send a Retrieve request for a <latest> content instance |
| 5 | (T8) Mcn | PRO Check Primitive | • op = 2 (Retrieve); • to = {CSEBaseName}/URI of <container> resource/la; • fr = IN-AE-ID; • rqi = (token-string); • pc = empty |
| 6 | (T8) Mcn | PRO Check HTTP | IN-CSE makes Monitoring Event Subscription request to retrieve current location of UE: • Method = POST; • URI = {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/; • Payload includes MonitoringEventSubscription with: externalId, notificationDestination, monitoringType, supportedFeatures, maximumNumberOfReports, monitorExpireTime, accuracy; • locationType = CURRENT*KNOWNLOCATION |
| 7 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Event Subscription Response to IN-CSE: • Status code = 201 (CREATED); • Location header = {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId}; • Payload includes NiddConfiguration with: monitoringEventReport, self |
| 8 |  | IOP Check | Check if possible that SCEF detected and retrieved location of UE |
| 9 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Event Report to IN-CSE: • Method = POST; • URI = {notification_uri}; • Payload includes MonitoringEventSubscription with: subscription, configResults, cancelId, monitoring, EventReports (externalID, monitoringType, locationInfo) |
| 10 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the DDN Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 11 |  | IOP Check | Check if possible that IN-CSE has created a new <contentInstance> child resource of the <container>; the <contentInstance> contains the UE’s current location |
| 12 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Monitoring Event Subscription to SCEF: • Method = POST; • URI = {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/; • Payload includes MonitoringEventSubscription with: subscription, configResults, cancelId, monitoringEventReports (externalID, monitoringType, locationInfo); • locationType = LAST*KNOWNLOCATION |
| 13 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Event Report to IN-CSE: • Method = POST; • Response code = 201 (CREATED); • URI = {apiRoot}/3gpp-monitoring-vent/v1/{scsAsId}/subscriptions/{subscriptionId}; • Payload includes MonitoringEventSubscription with: self, monitoringEventReport |
| 14 |  | IOP Check | Check if possible that SCEF receives the last known location information of UE |
| 15 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Notification Report to IN-CSE for LOCATION_REPORTING: • Method = POST; • URI = {notification_uri}; • Payload includes MonitoringEventSubscription with: subscription, configResults, cancelId, monitoringEventReports |
| 16 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the Monitoring Notification: Status code = 204 (NO CONTENT) |
| 17 |  | IOP Check | Check if possible that IN-CSE has created a new <contentInstance> child resource of the <container> containing the UE’s last known location |
| 18 | (T8) Mcn | PRO Check HTTP | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message; pc = Serialized representation of latest <contentInstance> resource, created after acquiring location info from SCEF |
| 19 |  | IOP Check | AE indicates successful operation |

### 8.7.3 3GPP Based Device triggering

#### 8.7.3.1 General Procedure for 3GPP Based Device Triggering

Interoperability Test Description

| Identifier: | TD_M2M_SH_35 |
| --- | --- |
| Objective: | IN-AE triggers ADN-AE hosted on UE |
| Configuration: | M2M_CFG_09 |
| References: | oneM2M TS-0026 [15], clause 7.5.1 |

Pre-test conditions:

- UE, SCEF and IN-CSE are attached to the underlying 3GPP network

- ADN-AE has created an Application Entity resource &lt;AE> on IN-CSE

- ADN-AE is available to receive the Device Trigger Requests (triggerEnable = "TRUE")

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | IN-AE sends a request to create a <triggerRequest> |
| 2\ | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/{AE-ID}/ • fr = AE-ID • rqi = (token-string) • ty = 47 (triggerRequest) • pc = Serialized representation of < triggerRequest > resource |
| 3 |  | IOP Check | Check if possible that IN-CSE has created <triggerRequest> resource |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Device Triggering request to the SCEF • Method = POST • URI = {apiRoot}/3gpp-device-triggering/v1/{scsAsId}/transactions • Payload shall include DeviceTriggering data structure with the following attributes included in the request: supportedFeatures, validityPeriod, triggerPayload, externalId, applicationPortID, notificationDestination, priority |
| 5 |  | IOP Check | Check that SCEF has responded to IN-CSE for Device Triggering request Check that SCEF has delivered the device trigger message to the UE hosting ADN-AE Check that SCEF has delivered the Device Triggering Delivery Report Notification request to IN-CSE |
| 6 |  | IOP Check | Check that IN-CSE has responded to SCEF the Device Triggering Delivery Report Notification request Check if possible that IN-CSE has updated triggerStatus attribute of <triggerRequest> resource Check that IN-CSE has responded to IN-AE for <triggerRequest> Create request |
| 7 |  | IOP Check | Check that ADN-AE has performed the trigger actions |

#### 8.7.3.2 3GPP Based Device Trigger Recall/Replace Procedure

Interoperability Test Description

| Identifier: | TD_M2M_SH_36 |
| --- | --- |
| Objective: | IN-AE recalls/replaces a trigger request targeting ADN-AE hosted on UE that has been already created in IN-CSE |
| Configuration: | M2M_CFG_09 |
| References: | oneM2M TS-0026 [15], clause 7.5.2 |

Pre-test conditions:

- UE, SCEF and IN-CSE are attached to the underlying 3GPP network

- ADN-AE has created an Application Entity resource &lt;AE> on IN-CSE

- ADN-AE is available to receive the Device Trigger Requests (triggerEnable = "TRUE")

- &lt;triggerRequest> resource targeting ADN-AE has been created in IN-CSE

- IN-CSE has already sent a device trigger request to the underlying 3GPP network

| Test Sequence |  |  |  |
| --- | --- | --- | --- |
| Step | RP | Type |  |
| 1 |  | Stimulus | IN-AE sends a request to UPDATE / DELETE a |
| 2 | Mca | PRO Check Primitive | - op = 3 (Update) / 4 (Delete) - to = {CSEBaseName}/{AE-ID}/{URI of resource} - fr = AE-ID - rqi = (token-string) - pc = Serialized representation of resource (for UPDATE only) |
| 3 |  | IOP Check | Check if possible that IN-CSE has created resource |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Trigger UPDATE (Replace) / DELETE (Recall) request to the SCEF: - Method = PUT / DELETE - URI = {apiRoot}/3gpp-device-triggering/v1/{scsAsId}/transactions - (for Trigger UPDATE only) Payload includes DeviceTriggering with: supportedFeatures, validityPeriod, triggerPayload, externalId, applicationPortID, notificationDestination, priority |
| 5 |  | IOP Check | Check that SCEF has recalled / replaced the trigger. Check that SCEF responded to IN-CSE for Device Trigger Recall / Replace. Check that SCEF has delivered the device-trigger message to the UE hosting ADN-AE. Check that SCEF has delivered the Device Triggering Delivery Report Notification request to IN-CSE. |
| 6 |  | IOP Check | Check if possible that IN-CSE has updated triggerStatus attribute of resource or deleted the resource. Check that IN-CSE has responded to IN-AE for UPDATE / DELETE request. |

### 8.7.4 Configuration of traffic patterns

Interoperability Test Description

| Identifier: | TD_M2M_SH_37 |
| --- | --- |
| Objective: | IN-CSE translates the oneM2M Node Traffic Pattern (TP) into a 3GPP Device Communication Pattern |
| Configuration: | M2M_CFG_09 |
| References: | oneM2M TS-0026 [15], clause 7.6 |

Pre-test conditions:

- UE, SCEF and IN-CSE are attached to the underlying 3GPP network

- ADN-AE hosted on UE has created an Application Entity resource &lt;AE> on IN-CSE

- IN-CSE has established relationship with MNO and is allowed to request Configuration of Device Communication Patterns.



                                                                                                   |

| Test Sequence |  |  |  |
| --- | --- | --- | --- |
| Step | RP | Type | Description |
| 1 |  | Stimulus | ADN-AE sends a request to create an activityPatternElements attribute in the resource |
| 2 | Mca | PRO Check Primitive | - op = 3 (Update) - to = {CSEBaseName}/{AE-ID} - fr = AE-ID - rqi = (token-string) - pc = Serialized representation of resource |
| 3 |  | IOP Check | Check if possible that IN-CSE has updated the resource |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Communication Patterns Configuration creation request to the SCEF: - Method = POST - URI = {apiRoot}/3gpp-cp-parameter-provisioning/v1/{scsAsId}/subscriptions - Payload includes DeviceTriggering with: externalId, supportedFeatures, cpParameterSets |
| 5 |  | IOP Check | Check if possible that underlying 3GPP network elements have stored the new/updated CP parameter. Check that SCEF has responded to IN-CSE for the Communication Patterns Configuration request |

### 8.7.5 Group message delivery using MBMS

#### 8.7.5.1 Create MBMS Group

Interoperability Test Description

| Identifier: | TD_M2M_SH_38 |
| --- | --- |
| Objective: | IN-AE creates a MBMS Group for handling group related requests |
| Configuration: | M2M_CFG_12 |
| References: | oneM2M TS-0026 [15], clause 7.7.3.1 |

Pre-test conditions:

Member Hosting CSE, SCEF and Group Hosting CSE are attached to the underlying 3GPP network

Member Hosting CSE on UE has created a <RemoteCSE> resource on Group Hosting CSE

IN-AE has created an Application Entity resource <AE> on Group Hosting CSE

The MBMS service area information provided by the MNO is configured in the oneM2M System

External Group Identifiers for the devices have been pre-provisioned in the oneM2M System

| Test Sequence |  |  |  |
| --- | --- | --- | --- |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-AE is requested to send a group Create Request |
| 2 | Mca | PRO Check Primitive | - op = 1 (Create) - to = {CSEBaseName} - fr = AE-ID - rqi = (token-string) - ty = 9 (group) - pc = Serialized representation of resource |
| 3 |  | IOP Check | Check if possible that the resource is created in Registrar CSE Check if possible that multicastType attribute of the Multicast Group Information is set to 3GPP_MBMS_group |
| 4 | Mca | PRO Check Primitive | Group Hosting CSE responds to IN-AE: - rsc = 2001 (CREATED) - rqi = (token-string) same as received in request message - pc = Serialized representation of resource |
| 5 | (T8) Mcn | PRO Check HTTP | Group Hosting CSE sends an Allocate TMGI Request to the SCEF: - Method = POST - URI = {apiRoot}/3gpp-group-message-delivery-mb2/v1/{scsAsId}/tmgi-allocation - Payload includes TMGIAllocation: externalGroupId, mbmsLocArea, supportedFeatures |
| 6 |  | IOP Check | Check that SCEF has delivered Allocate TMGI Response to Group Hosting CSE |
| 7 |  | IOP Check | Check if possible that the Group Hosting CSE has stored the tmgi and tmgiExpiration in the local Multicast Group |

#### 8.7.5.2 Group message delivery using MBMS

Interoperability Test Description

| Identifier: | TD_M2M_SH_39 |
| --- | --- |
| Objective: | IN-AE sends a request for accessing member resources to the Group Hosting CSE |
| Configuration: | M2M_CFG_12 |
| References: | oneM2M TS-0026 [15], clause 7.7.3.1 |

Pre-test conditions:

Member Hosting CSE on UE has created a <RemoteCSE> resource on Group Hosting CSE

<node> resource representing UE has been created on Group Hosting CSE

<schedule> resource has been created as a child of the <node> resource on Group Hosting CSE

IN-AE has created a <group> resource in Group Hosting CSE

<node> resource is the member of the <group>

Group Hosting CSE has created a <localMulticastGroup> in the Member Hosting CSE

Group Hosting CSE stores tmgi and tmgiExpiration in the local Multicast Group Informationl

| Test Sequence :=======+:========+:=====================+=============================================================================================================================================================================================== Step¦ RP ¦ Type ¦ Description |  |  |  |
| --- | --- | --- | --- |
| 1 |  | Stimulus | IN-AE is requested to send a Retrieve Request to the fanoutPoint of resource |
| 2 | Mca | PRO Check Primitive | - op = 2 (Retrieve) - to = {CSEBaseName}/{group}/fopt - fr = AE-ID - rqi = (token-string) |
| 3 | (T8) Mcn | PRO Check HTTP | Group Hosting CSE sends a Group Message Delivery Request to the SCEF: - Method = POST - URI = to {apiRoot}/3gpp-group-message-delivery-mb2/v1/{scsAsId}/tmgi-allocation{tmgi}/delivery-via-mbms - Payload shall include GMDViaMBMSByMb2 data structure with the following attributes: externalGroupId, mbmsLocArea, messageDeliveryStartTime, notificationDestination |
| 4 |  | IOP Check | Check that SCEF has responded to the Group Message. Check that SCEF has sent Group Message Delivery Notification to Group Hosting CSE. |
| 5 |  | IOP Check | Check that Group Hosting CSE has responded to the Group Message Delivery Notification |
| 6 |  | IOP Check | Check that Member Hosting CSE has sent response message within the scope of responseTimeWindow |
| 7 |  | IOP Check | Check that Group Hosting CSE has received the response messages from Member Hosting CSEs until responseTimeWindow expires and returned the aggregated group member responses to the IN-AE/CSE |

## 8.8 Advanced Subscriptions & Notifications management

### 8.8.1 Notification Target removal procedure

Interoperability Test Description

| Identifier: | TD_M2M_NH_137 |
| --- | --- |
| Objective: | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request |
| Configuration: | M2M_CFG_10 |

oneM2M TS-0004 [2], clause 7.4.33.2.4 | +——————-+—————————————————————————————————————————–+

Pre-test conditions:

AE1 has created an Application Entity resource <AE> on Registrar CSE

AE1 has created a <subscription> resource on Registrar CSE

AE1 has created a <notificationTargetPolicy> resource on Registrar CSE, where policyLabel = “default”, action = “accept”

AE2 has created an Application Entity resource <AE> on Registrar CSE

| Test Sequence |  |  |  |
| --- | --- | --- | --- |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a notificationTargetSelfReference Delete request |
| 2 | Mca | PRO Check Primitive | - op = 4 (Delete) - to = {CSEBaseName}/URI of resource/ntsr - fr = AE-ID - rqi = (token-string) - pc = empty |
| 3 | Mca | PRO Check Primitive | - rsc = 2002 (DELETED) - rqi = (token-string) same as received in request message - pc = empty |
| 4 |  | IOP Check | Check if possible that the Registrar CSE has removed AE2 from the notificationURI attribute in the resource |
| 5 |  | IOP Check | AE2 indicates successful operation |

### 8.8.2 NotificationTargetMgmtPolicyRef management

#### 8.8.2.1 NotificationTargetMgmtPolicyRef Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_138 |
| --- | --- |
| Objective: | AE creates a notificationTargetMgmtPolicyRef resource in registrar CSE via a notificationTargetMgmtPolicyRef Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.30.2.1 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

AE has created a <subscription> resource on registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a notificationTargetMgmtPolicyRef> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <subscription> resource • fr = AE-ID • rqi = (token-string) • ty = 25 (notificationTargetMgmtPolicyRef) • pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetMgmtPolicyRef> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.2.2 NotificationTargetMgmtPolicyRef Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_139 |
| --- | --- |
| Objective: | AE retrieves notificationTargetMgmtPolicyRef resource from Registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.30.2.2 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a subscription resource <subscription> on Registrar CSE

AE has created a <notificationTargetMgmtPolicyRef> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <notificationTargetMgmtPolicyRef> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <notificationTargetMgmtPolicyRef> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.8.2.3 NotificationTargetMgmtPolicyRef Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_140 |
| --- | --- |
| Objective: | AE updates information about a notificationTargetMgmtPolicyRef via notificationTargetMgmtPolicyRef>Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.30.2.3 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a subscription resource <subscription> on Registrar CSE

AE has created a subscription resource <notificationTargetMgmtPolicyRef> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a notificationTargetMgmtPolicyRef Update Request to update the notificationlPolicyID attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <notificationTargetMgmtPolicyRef> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <notificationTargetMgmtPolicyRef> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetMgmtPolicyRef> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.2.4 NotificationTargetMgmtPolicyRef Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_141 |
| --- | --- |
| Objective: | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.30.2.4 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a notificationTargetMgmtPolicyRef resource <notificationTargetMgmtPolicyRef> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a notificationTargetMgmtPolicyRef Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <notificationTargetMgmtPolicyRef> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <notificationTargetMgmtPolicyRef> resource has been removed in registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.8.3 NotificationTargetPolicy management

#### 8.8.3.1 NotificationTargetPolicy Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_142 |
| --- | --- |
| Objective: | AE creates a notificationTargetPolicy resource in registrar CSE via a notificationTargetPolicy Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.31.2.1 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <notificationTargetPolicy> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 26 (notificationTargetPolicy) • pc = Serialized representation of <notificationTargetPolicy> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <notificationTargetPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.3.2 NotificationTargetPolicy Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_143 |
| --- | --- |
| Objective: | AE retrieves notificationTargetPolicy resource from Registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.31.2.2 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <notificationTargetPolicy> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <notificationTargetPolicy> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <notificationTargetPolicy> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <notificationTargetPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.8.3.3 NotificationTargetPolicy Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_144 |
| --- | --- |
| Objective: | AE updates information about a notificationTargetPolicy via <notificationTargetPolicy> Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.31.2.3 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a notificationTargetPolicy resource <notificationTargetPolicy> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a notificationTargetPolicy Update Request to update the policyLabel attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <notificationTargetPolicy> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <Subscription> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <notificationTargetPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.3.4 NotificationTargetPolicy Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_145 |
| --- | --- |
| Objective: | AE removes notificationTargetPolicy via a <notificationTargetPolicy> Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.31.2.4 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a notificationTargetPolicy resource <notificationTargetPolicy> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a notificationTargetPolicy Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <notificationTargetPolicy> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource is deleted in registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.8.4 PolicyDeletionRules management

#### 8.8.4.1 PolicyDeletionRules Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_146 |
| --- | --- |
| Objective: | AE creates a policyDeletionRules resource in registrar CSE via a policyDeletionRules Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.32.2.1 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

AE has created a <notificationTargetPolicy> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <policyDeletionRules> |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/ URI of <notificationTargetPolicy> • fr = AE-ID • rqi = (token-string) • ty = 27 (policyDeletionRules) • pc = Serialized representation of <policyDeletionRules> resource |
| 3 |  | IOP Check | Check if possible that the <policyDeletionRules> resource is created in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <policyDeletionRules> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.4.2 PolicyDeletionRules Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_147 |
| --- | --- |
| Objective: | AE retrieves policyDeletionRules resource from Registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.32.2.2 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <notificationTargetPolicy> on Registrar CSE

AE has created a <policyDeletionRules> as a child of <notificationTargetPolicy> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <policyDeletionRules> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <policyDeletionRules> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <policyDeletionRules> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.8.4.3 PolicyDeletionRules Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_148 |
| --- | --- |
| Objective: | AE updates information about a policyDeletionRules via <policyDeletionRules> Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.32.2.3 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <notificationTargetPolicy> on Registrar CSE

AE has created a <policyDeletionRules> as a child of <notificationTargetPolicy> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a policyDeletionRules Update Request to update the deletionRulesRelation attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <policyDeletionRules> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <policyDeletionRules> resource |
| 3 |  | IOP Check | Check if possible that the <policyDeletionRules> resource is updated in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <policyDeletionRules> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.4.4 PolicyDeletionRules Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_149 |
| --- | --- |
| Objective: | AE removes policyDeletionRules via a <policyDeletionRules> Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.32.2.4 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <notificationTargetPolicy> on Registrar CSE

AE has created a <policyDeletionRules> as a child of <notificationTargetPolicy> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a policyDeletionRules Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <policyDeletionRules> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource has been removed in registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

### 8.8.5 CrossResourceSubscription management

#### 8.8.5.1 CrossResourceSubscription Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_150 |
| --- | --- |
| Objective: | AE creates a crossResourceSubscription resource in registrar CSE via a crossResourceSubscription Create Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.58.2.1 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an application resource <AE> on registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE sends a request to create a <crossResourceSubscription> where regularResourcesAsTarget = AE-ID |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName} • fr = AE-ID • rqi = (token-string) • ty = 48 (crossResourceSubscription) • pc = Serialized representation of <crossResourceSubscription> resource |
| 3 |  | IOP Check | Check if possible that the <crossResourceSubscription> resource is created in registrar CSE Check if possible that the <subscribtion> resource is created as a child of <AE> resource in registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <crossResourceSubscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.5.2 CrossResourceSubscription Retrieve

Interoperability Test Description

| Identifier: | TD_M2M_NH_151 |
| --- | --- |
| Objective: | AE retrieves crossResourceSubscription resource from Registrar CSE |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.58.2.2 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <crossResourceSubscription> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <crossResourceSubscription> |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <crossResourceSubscription> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2000 (OK) • rqi = (token-string) same as received in request message • pc = Serialized representation of <crossResourceSubscription> resource |
| 4 |  | IOP Check | AE indicates successful operation |

#### 8.8.5.3 CrossResourceSubscription Update

Interoperability Test Description

| Identifier: | TD_M2M_NH_152 |
| --- | --- |
| Objective: | AE updates information about a crossResourceSubscription via <crossResourceSubscription> Update Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.58.2.3 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <crossResourceSubscription> on Registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a crossResourceSubscription Update Request to update the regularResourcesAsTarget attribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <crossResourceSubscription> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <crossResourceSubscription> resource |
| 3 |  | IOP Check | Check if possible that the <crossResourceSubscription> resource is updated in Registrar CSE If regularResourcesAsTarget contains new target resources, check if possible that <subscription> resources are created to each new target resource |
| 4 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <crossResourceSubscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.5.4 CrossResourceSubscription Delete

Interoperability Test Description

| Identifier: | TD_M2M_NH_153 |
| --- | --- |
| Objective: | AE removes crossResourceSubscription via a <crossResourceSubscription> Delete Request |
| Configuration: | M2M_CFG_01 |

oneM2M TS-0004 [2], clause 7.4.32.2.4 | +——————-+——————————————————————————————————————————+

Pre-test conditions:

AE has created an Application Entity resource <AE> on Registrar CSE

AE has created a <crossResourceSubscription> on Registrar CSE

Registrar CSE has created <subscription> resources as targets of <crossResourceSubscription>

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE is requested to send a crossResourceSubscription Delete Request |
| 2 | Mca | PRO Check Primitive | • op = 4 (Delete) • to = {CSEBaseName}/URI of <crossResourceSubscription> resource • fr = AE-ID • rqi = (token-string) • pc = empty |
| 3 | Mca | PRO Check Primitive | • rsc = 2002 (DELETED) • rqi = (token-string) same as received in request message • pc = empty |
| 4 |  | IOP Check | Check if possible that the <crossResourceSubscription> resource is deleted in registrar CSE Check if possible that the target <subscription> resources are deleted in registrar CSE |
| 5 |  | IOP Check | AE indicates successful operation |

#### 8.8.5.5 Cross-Resource Notification

Interoperability Test Description

| Identifier: | TD_M2M_NH_154 |
| --- | --- |
| Objective: | AE receives a notification request from the HOST CSE |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0001 [1], clause 10.2.10.26 |

Pre-test conditions:

AE1 has created an application resource <AE> on registrar CSE

AE1 has created <crossResourceSubscription> on registrar CSE

AE1 has created a container1 resource <container> on registrar CSE

AE1 has created a container2 resource <container> on registrar CSE

AE2 has permissions to UPDATE the container1 and container2 created by AE1

Registrar CSE has created <subscription> resources as a child resource of a container1 and container2 as a target of <crossResourceSubscription> resource

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send an Update request to the container1 created by AE1. This triggers the timer as indicated in timeWindowSize attribute of <crossResourceSubscription> |
| 2 |  | Stimulus | AE2 is requested to send an Update request to the container2 created by AE1 within the time frame indicated in timeWindowSize after the 1st request. This triggers or causes the Registrar CSE to send a notification to AE1 |
| 3 | Check Mca | PRO Check Primitive | • op = 5 (Notify) • to = notificationURI of subscription resource • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if the notification representation |
| 4 | Check Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE1 indicates notification received |

## 8.9 Modbus Interworking

### 8.9.1 Modbus Thermometer Device Create

Interoperability Test Description

| Identifier: | TD_M2M_NH_163 |
| --- | --- |
| Objective: | AE1 creates Device Model for Modbus device |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0040 [16], clause 6.3 |

Pre-test conditions:

AE1 is running in Modbus IPE

AE1 has created an application resource <AE> on registrar CSE

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> resource for Modbus deviceThermometer |
| 2 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE1> resource • fr = AE1-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in Registrar CSE |
| 4 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| 6 |  | Stimulus | AE1 sends a request to create a <flexContainer> for temperature |
| 7 | Mca | PRO Check Primitive | • op = 1 (Create) • to = {CSEBaseName}/URI of <AE1> resource/resource name of Modbus deviceLight • fr = AE1-ID • rqi = (token-string) • ty = 28 (flexContainer) • pc = Serialized representation of <flexContainer> resource with proper containerDefinition and nodnProperties |
| 8 |  | IOP Check | Check if possible that the <flexContainer> resource is created in Registrar CSE |
| 9 | Mca | PRO Check Primitive | • rsc = 2001 (CREATED) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE indicates successful operation |
| Note |  |  | Optional: Repeat steps 5 – 10 for battery Module |

### 8.9.2 Retrieve data from a Modbus Thermometer device

Interoperability Test Description

| Identifier: | TD_M2M_NH_164 |
| --- | --- |
| Objective: | Modbus IPE reads data from Modbus device and updates Registrar CSE with the read data |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0040 [16], clause 6.5.1 |

Pre-test conditions:

AE1 is running in Modbus IPE

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a <flexContainer> for deviceThermometer

AE1 has created a <flexContainer> for temperature as a child of deviceThermometer

AE2 has created an application resource <AE> on registrar CSE

AE2 has created a <subscription> resource as a child of temperature

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 sends a request to retrieve a <flexContainer> resource for temperature |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of <deviceThermometer> resource/ temperature • fr = AE1-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • pc = Serialized representation of <flexContainer> resource • rsc = 2000 (OK) |
| 4 |  | IOP Check | AE1 indicates successful operation |
| 5 |  | Stimulus | Modbus IPE sends a request(s) to retrieve data from Modbus Thermometer device |
| 6 |  | IOP Check | Check if possible that Modbus IPE has successfully retrieved data from Modbus device |
| 7 |  | Stimulus | AE1 sends a request to update a <flexContainer> resource for deviceThermometer |
| 8 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of deviceThermometer/temperature • fr = AE1-ID • rqi = (token-string) • pc = Serialized representation of updated <flexContainer> resource |
| 9 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | Check if possible that the < flexContainer > resource for temperature is updated in Registrar CSE. Registrar CSE sends a notification to AE2 |
| 11 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = AE2-ID • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 12 | Mca | PRO Check Primitive | AE2 responds to notification • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 13 |  | IOP Check | AE2 indicates notification received |

### 8.9.3 Write data to a Modbus Thermometer device

Interoperability Test Description

| Identifier: | TD_M2M_NH_165 |
| --- | --- |
| Objective: | AE writes data into a Modbus device by updating <flexContainer> resource in Registrar CSE |
| Configuration: | M2M_CFG_10 |
| References: | oneM2M TS-0040 [16], clause 6.5.2 |

Pre-test conditions:

AE1 is running in Modbus IPE

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a <flexContainer> for deviceThermometer

AE1 has created a <flexContainer> for temperature as a child of deviceThermometer

AE2 has created an application resource <AE> on registrar CSE

AE2 has created a <subscription> resource as a child of temperature (notificationEventType = Blocking_Update)

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a flexContainer Update Request to update the any customAttribute of the resource |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of <flexContainer> resource • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of updated <flexContainer> resource |
| 3 |  | IOP Check | Check that Registrar CSE sent a notification to AE1 |
| 4 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = AE1-ID • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 5 |  | Stimulus | Modbus IPE sends a request(s) to write data to Modbus Thermometer device |
| 6 |  | IOP Check | Check if possible that Modbus IPE has successfully written data to Modbus device |
| 7 |  | Stimulus | AE1 sends a response to notification to Registrar CSE |
| 8 | Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 9 | Mca | PRO Check Primitive | Registrar CSE sends a response to AE2 • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE2 indicates successful operation |

## 8.10 NoDN Interworking

### 8.10.1 Retrieve data from a NoDN device

Interoperability Test Description

| Identifier: | TD_M2M_NH_166 |
| --- | --- |
| Objective: | NoDN IPE reads data from a NoDN device and updates Registrar CSE with the read data |
| Configuration: | M2M_CFG_10 |
| References: |  |

Pre-test conditions:

AE1 is running in NoDN IPE

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a <flexContainer> representing NoDN DeviceClass

AE1 has created a <flexContainer> for the ModuleClass as a child of the DeviceClass representing the NoDN device

AE2 has created an application resource <AE> on registrar CSE

AE2 has created a <subscription> resource as a child of the ModuleClass

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE1 sends a request to retrieve a <flexContainer> resource for the ModuleClass |
| 2 | Mca | PRO Check Primitive | • op = 2 (Retrieve) • to = {CSEBaseName}/URI of ModuleClass • fr = AE1-ID • rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | • pc = Serialized representation of <flexContainer> resource • rsc = 2000 (OK) |
| 4 |  | IOP Check | AE1 indicates successful operation |
| 5 |  | Stimulus | NoDN IPE retrieves data from NoDN device |
| 6 |  | IOP Check | Check if possible that NoDN IPE has successfully retrieved data from NoDN device |
| 7 |  | Stimulus | AE1 sends a request to update a <flexContainer> resource for the ModuleClass |
| 8 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of the ModuleClass • fr = AE1-ID • rqi = (token-string) • pc = Serialized representation of the updated ModuleClass |
| 9 | Mca | PRO Check Primitive | • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | Check if possible that the <flexContainer> resource for the ModuleClass is updated in Registrar CSE. Registrar CSE sends a notification to AE2 |
| 11 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = AE2-ID • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 12 | Mca | PRO Check Primitive | AE2 responds to notification • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 13 |  | IOP Check | AE2 indicates notification received |

### 8.10.2 Write data to a NoDN device

Interoperability Test Description

| Identifier: | TD_M2M_NH_167 |
| --- | --- |
| Objective: | AE writes data into a NoDN device by updating <flexContainer> resource in Registrar CSE |
| Configuration: | M2M_CFG_10 |
| References: |  |

Pre-test conditions:

AE1 is running in NoDN IPE

AE1 has created an application resource <AE> on registrar CSE

AE1 has created a <flexContainer> representing the NoDN DeviceClass

AE1 has created a <flexContainer> for the ModuleClass as a child of the DeviceClass representing the NoDN device

AE2 has created an application resource <AE> on registrar CSE

AE2 has created a <subscription> resource as a child of the ModuleClass (notificationEventType = Blocking_Update)

| Step | RP | Type | Description |
| --- | --- | --- | --- |
| 1 |  | Stimulus | AE2 is requested to send a flexContainer Update Request to update any customAttribute of the ModuleClass |
| 2 | Mca | PRO Check Primitive | • op = 3 (Update) • to = {CSEBaseName}/URI of the ModuleClass • fr = AE-ID • rqi = (token-string) • pc = Serialized representation of the updated ModuleClass |
| 3 |  | IOP Check | Check that Registrar CSE sent a notification to AE1 |
| 4 | Mca | PRO Check Primitive | • op = 5 (Notify) • to = AE1-ID • from = Registrar CSE-ID • rqi = (token-string) • pc = Serialized representation of Notification data object |
| 5 |  | Stimulus | NoDN IPE sends a request(s) to write data to NoDN device |
| 6 |  | IOP Check | Check if possible that NoDN IPE has successfully written data to NoDN device |
| 7 |  | Stimulus | AE1 sends a response to notification to Registrar CSE |
| 8 | Mca | PRO Check Primitive | Sent response contains: • rsc = 2000 (OK) • rqi = (token-string) same as received in request message |
| 9 | Mca | PRO Check Primitive | Registrar CSE sends a response to AE2 • rsc = 2004 (Updated) • rqi = (token-string) same as received in request message • pc = Serialized representation of the ModuleClass |
| 10 |  | IOP Check | AE2 indicates successful operation |

# History

| Publication history |  |  |
| --- | --- | --- |
| V1.0.0 | 29-Feb-2016 | Updated Release 1 - Publication |
| V2.3.2 | 12-Mar-2016 | Release 2A - Publication |
| V3.3.1 | 02-Sep-2021 | Release 3 - Publication |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V2.0.0 | 10-Nov-2016 | Initial draft release 2 based on TS-0013 V1.2.0 Implemented CR agreed at TST#25 TST-2016-0145R03-Discovery level offset TST-2016-0146R01-Flexcontainer Implemented CR agreed at TST#25.1 TST-2016-0203R01, flexcontainer-corrections TST-2016-0204, TS-0013_discovery_update TST-2016-0210R01-TS-0013_discovery_update |
| V2.0.1 | 23-Nov-2016 | Implemented CR agreed at TST#25.2 TST-2016-0214R01-TS-0013_updates |
| V2.1.0 | 11-Jan-2017 | Implemented CR agreed at TST#26.1 TST-2017-0003R01-TDs_for_mgmtCmd_and_execInstance |
| V2.2.0 | 28-Feb-2017 | Implemented CR agreed at TST#27 TST-2017-0026-TS-0013_update_of_the_TD_format TST-2017-0047-TDs_for_latest_oldest_retrieve_operation TST-2017-0049-TDs_for_access_controls |
| V2.3.0 | 28-Sept-2017 | Implemented CR agreed at TST#31 TST-2017-0237-CR-TS-0013_update_test_descriptions_for_flexContainer TST-2017-0228-semanticInterop-R2 |
| V2.3.1 | 21-Nov-2017 | Implemented CR agreed at TST#32 TST-2017-0257-TS-0013_editorialUpdates |
| V2.3.2 | 03-Dec-2017 | Release 2A publication |
| V2.4.0 | 30-May-2018 | Implemented CR agreed at TST#35 TST-2018-0078R03-TS-0013_Interop_Test_Cases_for_Notification_and_ResultContent |
| V2.5.0 | 19-June-2018 | Implemented CR agreed at TST#35.1a TST-2018-0106- TS-0013_New Configuration and Notification Test TST-2018-0107-TS-0013_Interop_Test_Cases_for_timeSeries_and_timeSeriesInstance TST-2018-0108R01-TS-0013_InteropTestCasesforLocationPolicy |
| V2.6.0 | 29-June-2018 | Implemented CR agreed at TST#35.2 TST-2018-0115-TS-0013_Interop_Test_Descriptions_for_announcement |
| V3.0.0 | 05-February-2020 | Initial draft release 3 based on TS-0013 V2.6.0 Implemented CR agreed at TDE#43.3 TDE-2020-0007R01-HAIM_InterOp |
| V3.1.0 | 13-October-2020 | Implemented CR agreed at TDE#47 6 Oct 2020 TDE-2020-0085-Interop_test_cases_for_security TDE-2020-0086R01-Interop_test_cases_for_semantics |
| V3.2.0 | 19-October-2020 | Implemented CR agreed at TDE#47 16 Oct 2020 TDE-2020-0091-Test Cases for Subscription and Notification management TDE-2020-0093R01-Interop test cases for schedule |
| V3.3.0 | 23-October-2020 | Implemented CR agreed at TDE#47 20 Oct 2020 TDE-2020-0095R02-Interop_Test_Cases_for_3GPP_Interworking TDE-2020-0096R01-Interop_test_cases_for_HAIM |
| V3.3.0 | July 2021 | Partners pre-preprocessing done by editHelp! e-mail: mailto:edithelp@etsi.org |
| V3.4.0 | 24-April-2024 | Updated table style for markdown conversion at TDE #63 |
| V3.4.1 | 26-August-2025 | This baseline is markdown converted baseline |