| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0013-v4.0.1 |
| Document Name: | Interoperability Testing |
| Date: | 2026-08-27 |
| Abstract: | The specification addresses the testing of the primitives on the oneM2M interfaces as specified in TS-0001 [1] and TS-0004 [2]. The purpose of the interoperability testing is to prove end-to-end functionality between Application Entities and Common Service Entities over the Mca and Mcc reference points |
| 'Template Version: January 2026 (do not modify) |  |

The present document is provided for future development work within oneM2M only. The Partners accept no liability for any use of this specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1.  Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at:  https://www.oneM2M.org

Copyright Notification

© 2026, oneM2M Partners Type 1.

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

Contents

No table of contents entries found.

# 1	Scope

The present document specifies Interoperability Test Descriptions (TDs) for the oneM2M Primitives as specified in oneM2M TS-0001 [1], oneM2M TS-0004 [2], the bindings oneM2M TS-0008 [3], oneM2M TS-0009 [4] and oneM2M TS-0010 [5].

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or nonspecific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

[1]	
      oneM2M TS-0001: "Functional Architecture- Release 3".

[2]	
      oneM2M TS-0004 "Service Layer Core protocol Specification - Release 3".

[3]	
      oneM2M TS-0008: "CoAP Protocol Binding Release 3".

[4]	
      oneM2M TS-0009: "HTTP Protocol Binding - Release 3".

[5]	
      oneM2M TS-0010: "MQTT Protocol Binding - Release 3".

[6]	
      oneM2M TS-0015: "Testing Framework".

[7]	
      oneM2M TS-0011: "Common Terminology".

[8]	
      IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[9]	
      IETF RFC 7230: "Hypertext Transfer Protocol (HTTP/1.1): Message Syntax and Routing".

[10]	
      oneM2M TS-0005: "Management Enablement (OMA) - Release 3".

[11]	
      oneM2M TS-0006: "Management Enablement (BBF) - Release 3".

[12]	
      oneM2M TS-0003: "Security Solutions - Release 3".

[13]	
      oneM2M TS-0034: "Semantics Support - Release 3".

[14]	
      oneM2M TS-0023: " Home Appliances Information Model and Mapping - Release 3".

[15]	
      oneM2M TS-0026: " 3GPP interworking - Release 4".

[16]	
      oneM2M TS-0040: " Modbus Interworking - Release 4".

## 2.2	Informative references

References are either specific (identified by date of publication and/or edition number or version number) or nonspecific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

[i.1]	
      oneM2M Drafting Rules.

NOTE: 	Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

[i.2]	
      BBF TR-069: "CPE WAN Management Protocol"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in oneM2M TS-0011 [7] and the following apply.

NOTE: 	A term defined in the present document takes precedence over the definition of the same term, if any, in oneM2M TS-0011 [7].

hosting CSE: CSE where the addressed resource is hosted

M2M service provider domain: part of the M2M System that is associated with a specific M2M Service Provider

mc: interface between the management server and the management client

NOTE: 	This interface can be realized by the existing device management technologies such as BBF TR-069 [i.2], OMA DM [10], etc.

receiver CSE: any CSE that receives a request

registrar CSE: CSE where an Application or another CSE has registered

registree: AE or CSE that registers with another CSE

resource: uniquely addressable entity in oneM2M architecture

transit CSE: any receiver CSE that is not a Hosting CSE

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ACP	Access Control Policy

AE	Application Entity

AE-ID	Application Entity Identifier

APP-ID	Application Identifier

BBF	BroadBand Forum

CFG	Configuration

CoAP	Constrained Application Protocol

CSE	Common Services Entity

CSE-ID	Common Service Entity Identifier

DM	Device Management

DTLS	Datagram Transport Layer Security

DUT	Device Under Test

FQDN	Fully Qualified Domain Name

HAIM	Home Appliances Information Model

HTTP	HyperText Transfer Protocol

IN	Infrastructure Node

IN-CSE	CSE which resides in the Infrastructure Node

IOP	Interoperability

IP	Internet Protocol

IPE	Interworking Proxy Entity

JSON	JavaScript Object Notation

LWM2M	Lightweight M2M

M2M	Machine to Machine

MA	Mandatory Announced

Mca	Reference Point for M2M Communication with AE

Mcc	Reference Point for M2M Communication with CSE

MH	Multi Hop

MO	Management Object

MQTT	Message Queuing Telemetry Transport

NB	Non-Blocking

NH	No Hop

NODN	Non oneM2M Device Node

OMA	Open Mobile Alliance

PRO	Protocol

PSK	Pre-Shared Key

RFC	Request for Comments

RP	Reference Point

RPC	Remote Procedure Calls

RQI	Request-ID

SE	Security

SH	Single Hop

SP	Service Provider

SUT	System Under Test

TCP	Transmission Control Protocol

TD	Test Description

TLS	Transport Layer Security

UDP	User Datagram Protocol

URI	Uniform Resource Identifier

XML	eXtensible Markup Language

# 4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1].

# 5	Testing conventions

## 5.1	The Test Description proforma

The testing methodology used in the present document is specified in the oneM2M TS-0015 [6].

A Test Description (TD) is a well detailed description of a process that aims to test one or more functionalities of an implementation. Applying to interoperability testing, these testing objectives address the interoperable functionalities between two or more vendor implementations.

In order to ensure the correct execution of an interoperability test, the following information should be provided by the test description:

- The proper configuration of the vendor implementations.

- The availability of additional equipment (protocol monitors, functional equipment, …) required to achieve the correct behaviour of the vendor implementations.

- The correct initial conditions.

- The correct sequence of the test events and test results.

In order to facilitate the specification of test cases an interoperability test description should include, at a minimum, the following fields as indicated table 5.1-1.

Table 5.1-1: Interoperability test description

| Identifier | A unique test description ID. |
| --- | --- |
| Objective | A concise summary of the test which should reflect the purpose of the test and enable readers to easily distinguish this test from any other test in the document. |
| References | A list of references to the base specification section(s), use case(s), requirement(s) and TP(s) which are either used in the test or define the functionality being tested. |
| Applicability | A list of features and capabilities which are required to be supported by the SUT in order to execute this test (e.g. if this list contains an optional feature to be supported, then the test is optional). |
| Configuration or Architecture | A list of all required equipment for testing and possibly also including a reference to an illustration of a test architecture or test configuration. |
| Pre-Test Conditions | A list of test specific pre-conditions that need to be met by the SUT including information about equipment configuration, i.e. precise description of the initial state of the SUT required to start executing the test sequence. |
| Test Sequence | An ordered list of equipment operation and observations. The test sequence may also contain the conformance checks as part of the observations. |

The test descriptions are provided in proforma tables. In order to ensure the correct execution of an interoperability test, the following information is provided in the test description:

- The configuration applied for the test.

- The need of additional equipment (protocol monitors, functional equipment, etc.) required to achieve the correct behaviour of the implementations.

- The initial conditions.

- The sequence of the test events and test results.

The following different types of test operator actions are considered during the test execution:

- A stimulus corresponds to an event that enforces a DUT to proceed with a specific protocol action, such as sending a message.

- A configure corresponds to an action to modify the DUT configuration.

- An IOP check consists of observing that one DUT behaves as described in the standard: i.e. resource creation, update, deletion, etc. For each IOP check in the Test Sequence, a result can be recorded. The overall IOP Verdict will be considered OK if all the IOP checks in the sequence are OK.

- In the context of Interoperability Testing with Conformance Checks, an additional step type, PRO checks can be used to verify the appropriate sequence and contents of protocol messages, this is helpful for debugging purposes. PRO Verdict will be PASS if all the PRO checks are PASS.

## 5.2	Test Description naming convention

| TD/<root>/<gr>/<nn> |  |  |
| --- | --- | --- |
| <root> = root | M2M | oneM2M |
| <gr> = group | NH | No Hop: Testing on Mca reference point |
|  | NB | Non-Blocking scenario |
|  | SH | Single Hop: management of remote resources on Mca + Mcc |
|  | MH | Multi Hop |
|  | SE | Security |
| <nn> = sequential number |  | 01 to 99 |

## 5.3	Test Settings

This clause contains some test requirements applied to the testing, some constraints, restrictions for executions or some recommendations.

In order to ease test setup and execution, the CSE and AE are requested to support the following settings:

- Security shall be disable as it is out of scope of this interoperability testing.

- Resource names are pre-provisioned, except for content instance resources that are automatically assigned by the hosting CSE.

- After each "Delete" primitive on a resource, the user shall check the resource is effectively deleted.

- Unless it is indicated in the test cases prerequisites, by default, all the applications shall have the required access rights to manage resources on the CSE.

In order to address the TBDs in the oneM2M CoAP binding specification (oneM2M TS-0008 [3]), basic XML and JSON media-type numbers shall be used in the contentFormat option.

In the test descriptions specified below, the following definitions of terms used for short-hand notation apply:

Serialized Representation: refers to either an XML or a JSON representation of data in text-string format as defined in clauses 8.3 and 8.4 of oneM2M TS-0004 [2].

Host Address: refers to the authority part of a target URI as defined in IETF RFC 3986 [8] and IETF RFC 7230 [9] which can be represented as an IP literal encapsulated within square brackets, an IPv4 address in dotted decimal form, or a registered name, and optionally extended by a port identifier.

## 5.4	Pre-conditions

### 5.4.1	Registration

The AE or CSE that originates the request has been successfully registered to its corresponding CSE. The registration of the AE includes the creation of <AE> resource under the <CSEBase> of its registrar CSE. The registration of the CSE includes the creation of <remoteCSE> resource representing itself under the <CSEBase> of its registrar CSE as well as the creation of <remoteCSE> resource representing the registrar CSE under its own <CSEBase> resource. The creation of <remoteCSE> resource representing the registrar CSE can be achieved by remotely retrieving the <CSEBase> resource of the registrar CSE.

### 5.4.2	Security

The Originator and the receiver have successfully established security association between each other. This may involve the exchange of key and the establishment of a security connection.

The security pre-condition also assumes that the originator has the appropriate access control privilege towards the requested resource.

### 5.4.3	Service Subscription

Service subscription means that the originator is allowed to be connected with the oneM2M system by contract between the owner of the application and the service provider of the oneM2M system. This may require a corresponding information record in the <m2mServiceSubscriptionProfile> resource.

### 5.4.4	ID allocation

ID allocation means that the Originator has already acquired usable identity, either from its registrar CSE or the IN-CSE of the oneM2M system. The ID may be CSE relative or SP relative. The ID is then further used as the identity of the Originator to perform access control, charging, etc.

### 5.4.5	Existence of resource

Existence of resource means the resource been addressed and has already been created.

### 5.4.6	Management Session between Management Server and Management Client

Before the device management using external technologies is executed, it is required that a management session has already been established between the Management Server and Management Client. If there is no existing management session, the IN-CSE shall request the establishment of a management session between the Management Server and Management Client.

## 5.5	Binding message convention

In HTTP/CoAP/MQTT binding messages, the present document defines the convention for <variable>:

- <resourceType> represents a resource name (i.e. resourceName attribute) of a resource instance in that resourceType. For example, <CSEBase>/<AE> can represent "CSE1base/AE1" in structured resource ID format.

- <parameter> represents a value of a oneM2M request/response parameter. For example, <Request ID> can represent "0001" value of the Request ID parameter. Parameter names are case sensitive and in long names as specified in oneM2M TS-0004 [2].

- <ID> represents an AE-ID or CSE-ID in MQTT Topic names.

The value will be given at an interoperability test event.

In oneM2M TS-0010 [5], all oneM2M request/response parameters are carried in the MQTT message payload since it has no message header concept. Therefore, the MQTT message payload needs to be described more than HTTP and CoAP messages to describe those parameters in clause 8. In HTTP and CoAP binding messages, payloads are described as "empty" or "<container> resource to be created" in a very abstract way.

Since the representation can be XML or JSON, payload should be abstract to support XML and JSON. The following example is an XML representation and its abstraction for creating a <container> resource.

| XML payload example for MQTT binding | <?xml version="1.0" encoding="UTF-8"?> <m2m:req xmlns:m2m="http://www.onem2m.org/xml/protocols" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="http://www.onem2m.org/xml/protocols CDT-requestPrimitive-v1_0_0.xsd"> <op>1</op> <to>CSE1Base</to> <fr>/CSE1/C_AE1</fr> <rqi>2001</rqi> <ty>3</ty> <nm>cont1</nm> <rti><rt>3</rt></rti> <pc> <cnt> <lbl>SmartMeter</lbl> <et>20141003T112033</et> </cnt> </pc> </m2m:req> |
| --- | --- |
| Abstracted payload example for MQTT binding | op = 1 to = CSE1Base fr = /CSE1/C_AE01 rqi = 3001 ty = 3 name = cont1 rti.rt = 3 pc.cnt.lbl = SmartMeter pc.cnt.et = 20141003T112033 |
| Abstracted payload example for MQTT binding adopting the payload convention | op = 1 to = <CSEBase> fr = <From> rqi = <Request ID> ty = 3 name = <Name> rti.rt = 3 pc = <Content> |

# 6	Test Description Summary

## 6.1	Release 4 features Tests list

| Nb | Category | Procedure/Resource | TD ID | TD Description |
| --- | --- | --- | --- | --- |
| 1 | Resource management Single Hop | Discovery | TD_M2M_NH_168 | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 1 (within) |
| 2 |  |  | TD_M2M_NH_169 | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 2 (contains) |
| 3 |  |  | TD_M2M_NH_170 | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 3 (intersects) |

## 6.2	Full Tests list

| Nb | Category | Procedure/Resource | TD ID | TD Description |
| --- | --- | --- | --- | --- |
| 1 | Resource management Single Hop | CSEBase Management | TD_M2M_NH_01 | AE retrieves the CSEBase resource |
| 2 |  | RemoteCSE | TD_M2M_NH_02 | Registree CSE registers to Registrar CSE |
| 3 |  |  | TD_M2M_NH_03 | Registree CSE retrieves RemoteCSE from Registrar CSE |
| 4 |  |  | TD_M2M_NH_04 | Registree CSE updates RemoteCSE from Registrar CSE |
| 5 |  |  | TD_M2M_NH_05 | Registree CSE deletes RemoteCSE from Registrar CSE |
| 6 |  | Application Entity | TD_M2M_NH_06 | AE registers to its registrar CSE via an AE Create Request |
| 7 |  |  | TD_M2M_NH_07 | AE retrieves <AE> resource via an AE Retrieve Request |
| 8 |  |  | TD_M2M_NH_08 | AE updates attribute in <AE> resource via an AE Update Request |
| 9 |  |  | TD_M2M_NH_09 | AE de-registers by deleting <AE> resource via an AE Delete Request |
| 10 |  | Container | TD_M2M_NH_10 | AE creates a container resource in registrar CSE via a container Create Request |
| 11 |  |  | TD_M2M_NH_11 | AE retrieves information of a container resource via a container Retrieve Request |
| 12 |  |  | TD_M2M_NH_12 | AE updates attribute in application resource via a container Update Request |
| 13 |  |  | TD_M2M_NH_13 | AE deletes a specific container resource via a container Delete Request |
| 14 |  | ContentInstance | TD_M2M_NH_14 | AE adds a contentInstance resource <contentInstance> to a specific container in Registrar CSE via a contentInstance Create Request and the registrar CSE updates the parent <container> resource with stateTag, and currentNrOfInstances, CurrentByteSize attributes correspondingly |
| 15 |  |  | TD_M2M_NH_15 | AE retrieves information of a contentInstance resource via a contentInstance Retrieve Request |
| 16 |  |  | TD_M2M_NH_17 | AE deletes contentInstance resource via a Delete Request and the registrar CSE updates the parent <container> resource with currentNrOfInstances, and CurrentByteSize attribute correspondingly |
| 17 |  |  | TD_M2M_NH_49 | AE deletes a <latest> resource in a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 18 |  |  | TD_M2M_NH_50 | AE deletes a <oldest> resource in a <container> resource and the Registrar CSE points an oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 19 |  |  | TD_M2M_NH_51 | AE sends a <contentInstance> CREATE request to a <container> which contains attribute currentNrOfInstances whose value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <contentInstance> from the parent <container> and then creates the requested <contentInstance> resource |
| 20 |  |  | TD_M2M_NH_102 | AE retrieves a <latest> resource of a <container> for which attribute locationID is configured, value of locationUpdatePeriod is marked '0' or not defined and locationSource attribute is 'Network Based'. |
| 21 |  |  | TD_M2M_NH_71 | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| 22 |  |  | TD_M2M_NH_72 | AE retrieves a <oldest> resource of a <container> and the Registrar CSE points a oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| 23 |  | Discovery | TD_M2M_NH_18 | AE discovers resources residing in Registrar CSE |
| 24 |  |  | TD_M2M_NH_19 | AE discovers accessible resources residing in Registrar CSE using the label filter criteria |
| 25 |  |  | TD_M2M_NH_20 | AE discovers accessible resources residing in Registrar CSE limiting the number of matching resources to the specified value. |
| 26 |  |  | TD_M2M_NH_21 | AE discovers accessible resources residing in Registrar CSE using multiple Filter Criteria |
| 27 |  |  | TD_M2M_NH_58 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 1 |
| 28 |  |  | TD_M2M_NH_59 | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 2 |
| 29 |  |  | TD_M2M_NH_60 | AE1 discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 3 |
| 30 |  |  | TD_M2M_NH_61 | AE discovers accessible resources residing in Registrar CSE using the offset filter criteria value set to 3 |
| 31 |  |  | TD_M2M_NH_62 | AE discovers all the accessible resources residing in Registrar CSE using the offset filter criteria |
| 32 |  |  | TD_M2M_NH_168 | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 1 (within) |
| 33 |  |  | TD_M2M_NH_169 | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 2 (contains) |
| 34 |  |  | TD_M2M_NH_170 | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 3 (intersects) |
| 35 |  | Subscription | TD_M2M_NH_22 | AE creates a subscription to Application Entity resource via subscription Create Request |
| 36 |  |  | TD_M2M_NH_23 | AE retrieves information about a subscription via subscription Retrieve Request such as expirationTime, labels, etc. |
| 37 |  |  | TD_M2M_NH_24 | AE updates information about a subscription via subscription Retrieve Request |
| 38 |  |  | TD_M2M_NH_25 | AE cancels subscription via an subscription Delete Request |
| 39 |  | AccessControlPolicy | TD_M2M_NH_26 | AE creates an accessControlPolicy resource |
| 40 |  |  | TD_M2M_NH_27 | AE retrieves accessControlPolicy resource |
| 41 |  |  | TD_M2M_NH_28 | AE updates attribute in accessControlPolicy resource |
| 42 |  |  | TD_M2M_NH_29 | AE deletes accessControlPolicy resource |
| 43 |  |  | TD_M2M_NH_30 | AE delete request is rejected due to accessControlPolicy |
| 44 |  |  | TD_M2M_NH_73 | AE delete request is rejected due to accessControlPolicy (accessControlOriginators) |
| 45 |  |  | TD_M2M_NH_74 | AE delete request is allowed due to accessControlPolicy |
| 46 |  | Group | TD_M2M_NH_31 | AE creates a group resource |
| 47 |  |  | TD_M2M_NH_32 | AE retrieves group resource |
| 48 |  |  | TD_M2M_NH_33 | AE updates attribute in group resource |
| 49 |  |  | TD_M2M_NH_34 | AE deletes group resource |
| 50 |  | Node | TD_M2M_NH_35 | AE creates a node resource |
| 51 |  |  | TD_M2M_NH_36 | AE retrieves node resource |
| 52 |  |  | TD_M2M_NH_37 | AE updates attribute in node resource |
| 53 |  |  | TD_M2M_NH_38 | AE deletes node resource |
| 54 |  | PollingChannel | TD_M2M_NH_39 | AE creates a <pollingChannel> resource in registrar CSE via a Create Request |
| 55 |  |  | TD_M2M_NH_40 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 56 |  |  | TD_M2M_NH_41 | AE updates attribute in pollingChannel resource via a Update Request |
| 57 |  |  | TD_M2M_NH_42 | AE deletes a pollingChannel resource via a Delete Request |
| 58 |  |  | TD_M2M_NH_43 | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| 59 |  | FanoutPoint | TD_M2M_NH_44 | AE creates a <contentInstance> resource in each group member |
| 60 |  |  | TD_M2M_NH_45 | AE retrieves the <container> resource from in each group member |
| 61 |  |  | TD_M2M_NH_46 | AE updates an <container> resource of each member resource |
| 62 |  |  | TD_M2M_NH_47 | AE deletes a <container> of each member |
| 63 |  | Notification | TD_M2M_NH_48 | AE receives a notification request from the HOST CSE |
| 64 |  |  | TD_M2M_NH_80 | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 |
| 65 |  |  | TD_M2M_NH_81 | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with 'Delete of Resource', Hosting CSE send notification to AE1 |
| 66 |  |  | TD_M2M_NH_82 | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with 'Create of Direct Child Resource', Hosting CSE send notification to AE1 |
| 67 |  |  | TD_M2M_NH_83 | AE2 sends DELETE request to the <contentInstance> which located under the subscribed-to resource. Since <subscription> resource has notificationEventType with 'Delete of Direct Child Resource', Hosting CSE send notification to AE1 |
| 68 |  |  | TD_M2M_NH_89 | AE creates <subscription> resources by sending Create Request to the fanOutPoint. Since AE has set notifyAggregation to 2, Hosting CSE aggregate notification and send aggregated notification to AE |
| 69 |  | FlexContainer | TD_M2M_NH_52 | AE creates a flexcontainer resource in Registrar CSE via a flexcontainer Create Request |
| 70 |  |  | TD_M2M_NH_53 | AE retrieves information of a flexContainer resource via a flexContainer Retrieve Request |
| 71 |  |  | TD_M2M_NH_54 | AE updates attribute in application resource via a flexContainer Update Request |
| 72 |  |  | TD_M2M_NH_55 | AE deletes a specific container resource via a container Delete Request |
| 73 |  |  | TD_M2M_NH_56 | AE receives a notification request on flexContainer update from the HOST CSE |
| 74 |  |  | TD_M2M_NH_57 | AE discovers accessible resources residing in Registrar CSE using attribute filter criteria which has a customAttribute name and value assigned to it. |
| 75 |  | External Management Operations | TD_M2M_NH_63 | AE creates a mgmtCmd resource |
| 76 |  |  | TD_M2M_NH_64 | AE retrieves mgmtCmd resource |
| 77 |  |  | TD_M2M_NH_65 | AE updates attribute (not with 'true' in execEnable attribute) in mgmtCmd resource |
| 78 |  |  | TD_M2M_NH_66 | AE updates attribute (with 'true' in execEnable attribute) in mgmtCmd resource |
| 79 |  |  | TD_M2M_NH_67 | AE deletes mgmtCmd resource |
| 80 |  |  | TD_M2M_NH_68 | AE retrieves execInstance resource |
| 81 |  |  | TD_M2M_NH_69 | AE updates attribute 'execDisable' to true in execInstance resource to cancel pending management command. |
| 82 |  |  | TD_M2M_NH_70 | AE deletes execInstance resource |
| 83 |  | SemanticDescriptor Management | TD_M2M_NH_75 | AE creates a SemanticDescriptor resource in Registrar CSE via a SemanticDescriptor Create Request |
| 84 |  |  | TD_M2M_NH_76 | AE retrieves information of a semanticDescriptor resource via a semanticDescriptor Retrieve Request |
| 85 |  |  | TD_M2M_NH_77 | AE updates attribute in <semanticDescriptor> resource via a semanticDescriptor Update Request |
| 86 |  |  | TD_M2M_NH_78 | AE deletes SemanticDescriptor resource via a SemanticDescriptor Delete Request |
| 87 |  | Semantic Resource Discovery | TD_M2M_NH_79 | AE discovers accessible resources residing in Registrar CSE using the semanticFilter filter criteria |
| 88 |  | ResultContent | TD_M2M_NH_84 | Check creation of <container> resource with result content set to 0(nothing) |
| 89 |  |  | TD_M2M_NH_85 | Check creation of <container> resource with result content set to 1(attributes) |
| 90 |  |  | TD_M2M_NH_86 | Check creation of <container> resource with result content set to 2(hierarchical address) |
| 91 |  |  | TD_M2M_NH_87 | Check creation of <container> resource with result content set to 3(hierarchical address and attributes) |
| 92 |  |  | TD_M2M_NH_88 | Check retrievability of <container> resource with result content set to 4(attributes and child resources) |
| 93 |  | timeSeries | TD_M2M_NH_90 | AE creates a <timeSeries> resource in registrar CSE via a Create Request |
| 94 |  |  | TD_M2M_NH_91 | AE retrieves information of a <timeSeries> resource via a Retrieve Request |
| 95 |  |  | TD_M2M_NH_92 | AE updates attribute in <timeSeries> resource via a Update Request |
| 96 |  |  | TD_M2M_NH_93 | AE deletes a <timeSeries> resource via a Delete Request |
| 97 |  | timeSeriesInstance | TD_M2M_NH_94 | AE sends Create Request of a <timeSeriesInstance> resource to a <timeSeries> resource in Registrar CSE. Registrar CSE creates the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| 98 |  |  | TD_M2M_NH_95 | AE retrieves information of a <timeSeriesInstance> resource via a Retrieve Request |
| 99 |  |  | TD_M2M_NH_96 | AE sends Delete Request of a <timeSeriesInstance> resource in Registrar CSE. Registrar CSE delete the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| 100 |  |  | TD_M2M_NH_97 | AE sends a <timeSeriesInstance> resource Create Request to a <timeSeries> resource which contains currentNrOfInstances value equals to that of maxNrOfInstances and Registrar CSE delets the oldest <timeSeriesInstance> resource from the <timeSeries> resource and then creates the requested <timeSeriesInstance> resource |
| 101 |  | LocationPolicy | TD_M2M_NH_98 | AE creates a <locationPolicy> resource in registrar CSE via a locationPolicy Create Request |
| 102 |  |  | TD_M2M_NH_99 | AE retrieves information of a <locationPolicy> resource via a locationPolicy Retrieve Request |
| 103 |  |  | TD_M2M_NH_100 | AE updates attribute in <locationPolicy> resource via a locationPolicy Update Request |
| 104 |  |  | TD_M2M_NH_101 | AE deletes a specific <locationPolicy> resource via a locationPolicy Delete Request |
| 105 |  | Schedule | TD_M2M_NH_155 | AE creates a <schedule> resource in Registrar CSE via a Schedule Create Request |
| 106 |  |  | TD_M2M_NH_156 | AE retrieves information of a <schedule> resource via a schedule Retrieve Request |
| 107 |  |  | TD_M2M_NH_157 | AE updates attribute in <schedule> resource via a schedule Update Request |
| 108 |  |  | TD_M2M_NH_158 | AE deletes Schedule resource via a Schedule Delete Request |
| 109 |  |  | TD_M2M_NH_159 | CSE sends a notification request to the AE when Schedule resource is configured |
| 110 | Non-Blocking | Synchronous request | TD_M2M_NB_01 | AE creates a container resource using non-blocking synchronous request in registrar CSE |
| 111 |  |  | TD_M2M_NB_02 | AE retrieves a Container resource using non-blocking synchronous request in registrar CSE |
| 112 |  |  | TD_M2M_NB_03 | AE updates a Container resource using non-blocking synchronous request in registrar CSE |
| 113 |  |  | TD_M2M_NB_04 | AE deletes a Container resource using non-blocking synchronous request |
| 114 |  | Asynchronous request | TD_M2M_NB_05 | AE creates a container resource using non-blocking asynchronous request |
| 115 |  |  | TD_M2M_NB_06 | AE retrieves a Container resource using non-blocking asynchronous request |
| 116 |  |  | TD_M2M_NB_07 | AE updates a Container resource using non-blocking asynchronous request |
| 117 |  |  | TD_M2M_NB_08 | AE deletes a Container resource using non-blocking asynchronous request |
| 118 | Single Hop | Retargeting | TD_M2M_SH_01 | AE creates a remote <Resource> resource |
| 119 |  |  | TD_M2M_SH_02 | AE retrieves a remote <Resource> resource |
| 120 |  |  | TD_M2M_SH_03 | AE updates a remote <Resource> resource |
| 121 |  |  | TD_M2M_SH_04 | AE delete a remote <Resource> resource |
| 122 |  | Discovery | TD_M2M_SH_09 | AE discovers accessible resources residing in the remote Hosting CSE using multiple Filter Criteria |
| 123 |  | Unauthorized operation | TD_M2M_SH_10 | AE delete request is rejected after access rights verification using retargeting. |
| 124 |  | Notification | TD_M2M_SH_11 | AE receives a notification request from the remote hosting CSE |
| 125 |  | mgmtObj | TD_M2M_SH_05 | AE creates a <mgmtObj> resource |
| 126 |  |  | TD_M2M_SH_06 | AE updates a <mgmtObj> resource |
| 127 |  |  | TD_M2M_SH_07 | AE retrieves a <mgmtObj> resource |
| 128 |  |  | TD_M2M_SH_08 | AE deletes a <mgmtObj> resource |
| 129 |  | Announcement | TD_M2M_SH_12 | AE1 announces itself to CSE2 |
| 130 |  |  | TD_M2M_SH_13 | AE1 announces a child container to CSE2 |
| 131 |  |  | TD_M2M_SH_14 | AE1 announces an Optional Announce attribute to CSE2 |
| 132 |  |  | TD_M2M_SH_15 | AE2 retrieves an Announced Resource |
| 133 |  |  | TD_M2M_SH_16 | AE2 retrieves the original resource representation of an announced resource |
| 134 |  |  | TD_M2M_SH_21 | ContainerAnnc Delete by updating announceTo attribute. AE1 deletes its announced child container from CSE2. |
| 135 |  |  | TD_M2M_SH_22 | ContainerAnnc Delete by deleting original resource. AE1 deletes its announced child container from CSE2. |
| 136 |  |  | TD_M2M_SH_23 | Announced attribute Create by addition to announcedAttribute attribute AE1 announces an announcable attribute of its child container to CSE2 |
| 137 |  |  | TD_M2M_SH_24 | Announced attribute Create by creation of a MA attribute at the original resource. AE1 announces an MA attribute of its child container to CSE2 |
| 138 |  |  | TD_M2M_SH_25 | Announced attribute Delete by deletion from announcedAttribute attribute. AE1 de-announces an announcable attribute of its child container to CSE2 |
| 139 |  |  | TD_M2M_SH_26 | Announced attribute Delete by deletion of a MA attribute at the original resource. AE1 de-announces an MA attribute (conditionally mandatory) of its child container to CSE2 |
| 140 |  | fanOut | TD_M2M_SH_17 | AE creates a <contentInstance> resource in each group member, where some memberIDs are on a remoteCSE |
| 141 |  |  | TD_M2M_SH_18 | AE retrieves a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 142 |  |  | TD_M2M_SH_19 | AE updates a <container> resource in each group member, where some memberIDs are on a remoteCSE |
| 143 |  |  | TD_M2M_SH_20 | AE deletes a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| 144 | Security | Secure AE Registration | TD_M2M_SE_01 | AE uses Provisioned Symmetric Key Security Association Establishment Framework to enable mutual authentication with the Registrar CSE. Registrar CSE performs AE authorization check on incoming AE registration request. |
| 145 |  | Authentication | TD_M2M_SE_02 | AE establishes mutual authentication with the Registrar CSE using Provisioned Symmetric Key Security Association Establishment Framework. |
| 146 |  |  | TD_M2M_SE_03 | AE establishes mutual authentication with the Registrar CSE using Certificate-Based Security Association Establishment Framework. |
| 147 |  | Authorization | TD_M2M_SE_04 | AE accesses <accessControlPolicy> resource using its selfPrivileges credentials |
| 148 |  |  | TD_M2M_SE_05 | AE accesses <AE> resource using its accessControlPolicyIDs attribute |
| 149 |  |  | TD_M2M_SE_06 | AE accesses <AE> resource using default access privileges |
| 150 |  |  | TD_M2M_SE_07 | AE accesses <AE> resource using default access privileges |
| 151 |  |  | TD_M2M_SE_08 | AE accesses <AE> resource using Direct Dynamic Authorization |
| 152 |  |  | TD_M2M_SE_09 | AE accesses <AE> resource using Indirect Dynamic Authorization |
| 153 |  | Key provisioning management | TD_M2M_SE_10 | A MEF Handshake procedure establishes a mutually authenticated TLS session for protecting the communication between an MEF Client and MEF using pre-provisioned certificates. |
| 154 |  |  | TD_M2M_SE_11 | A MEF Handshake procedure establishes a mutually authenticated TLS or DTLS session for protecting the communication between an MEF Client and MEF using pre-provisioned Master Credentials. |
| 155 |  |  | TD_M2M_SE_12 | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder |
| 156 |  |  | TD_M2M_SE_13 | The MEF Client retrieves MEF Client Configurations provided by the administrating stakeholder to the MEF. |
| 157 |  |  | TD_M2M_SE_14 | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels. |
| 158 |  |  | TD_M2M_SE_15 | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder |
| 159 |  |  | TD_M2M_SE_16 | Source MEF Client establishes a symmetric key with the MEF which can be retrieved for use by one or more Target MEF Clients |
| 160 |  |  | TD_M2M_SE_17 | The Target MEF Client to retrieve the Key Value from a MEF corresponding to a RelativeKeyID received by the Target MEF Client |
| 161 |  |  | TD_M2M_SE_18 | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels. |
| 162 |  |  | TD_M2M_SE_19 | Source MEF Client requests the MEF to stop distributing the registered key |
| 163 |  | End-to-End security management | TD_M2M_SE_20 | AE sends an arbitrary request primitive inside of ESPrim Object to CSE |
| 164 |  |  | TD_M2M_SE_21 | AE establishes a connection with the Registrar CSE using pairwiseE2EKey |
| 165 | HAIM | HAIM Device Model | TD_M2M_NH_102 | AE1 creates a HAIM Light Device Model |
| 166 |  |  | TD_M2M_NH_103 | AE2 reads the status of a HAIM Light Device Model |
| 167 |  |  | TD_M2M_NH_104 | AE2 turns the binarySwitch of a HAIM Light Device Model "ON" or "OFF" |
| 168 |  |  | TD_M2M_NH_105 | AE2 toggles the state of a HAIM Light Device Model |
| 169 |  |  | TD_M2M_NH_160 | AE1 creates a Power Outlet SubDevice Model |
| 170 |  |  | TD_M2M_NH_161 | AE1 creates a Toggle Action Model |
| 171 |  |  | TD_M2M_NH_162 | AE1 creates Device Properties Model |
| 172 | Semantics | Semantic Access Control Policy | TD_M2M_NH_106 | ACP triples are created when a new <accessControlPolicy> resource is created |
| 173 |  |  | TD_M2M_NH_107 | ACP triples are updated when an existing <accessControlPolicy> resource is updated |
| 174 |  |  | TD_M2M_NH_108 | ACP triples are deleted when an existing <accessControlPolicy> resource is deleted |
| 175 |  |  | TD_M2M_NH_109 | ACP-SD Binding Triples and SD relationship in SGS are created when AE creates a <semanticDescriptor> resource in Registrar CSE |
| 176 |  |  | TD_M2M_NH_110 | ACP-SD Binding Triples are updated when the accessControlPolicyIDs attribute of a <semanticDescriptor> resource is updated |
| 177 |  |  | TD_M2M_NH_111 | SD Relationship Triples are updated when the descriptor attribute of a <semanticDescriptor> resource is changed |
| 178 |  |  | TD_M2M_NH_112 | SD Relationship Triples are deleted when the descriptor attribute of a <semanticDescriptor> resource is deleted |
| 179 |  | Semantic Filtering and discovery | TD_M2M_NH_113 | AE discovers accessible resources residing in Registrar CSE using the <semanticFanOutPoint> |
| 180 |  |  | TD_M2M_NH_114 | AE discovers accessible resources residing in Registrar CSE using the resource link-based Semantic Discovery |
| 181 |  |  | TD_M2M_NH_115 | AE performs a Semantic Query request in Registrar CSE using the semanticFilter filter criteria |
| 182 |  | Semantic Mashup | TD_M2M_NH_116 | AE creates a SemanticMashupJobProfile resource in Registrar CSE via a SemanticMashupJobProfile Create Request |
| 183 |  |  | TD_M2M_NH_117 | AE retrieves information of a semanticMashupJobProfile resource via a semanticMashupJobProfile Retrieve Request |
| 184 |  |  | TD_M2M_NH_118 | AE updates attribute in <semanticMashupJobProfile> resource via a semanticMashupJobProfile Update Request |
| 185 |  |  | TD_M2M_NH_119 | AE deletes semanticMashupJobProfile resource via a semanticMashupJobProfile Delete Request |
| 186 |  |  | TD_M2M_NH_120 | AE creates a semanticMashupInstance resource in Registrar CSE via a semanticMashupInstance Create Request |
| 187 |  |  | TD_M2M_NH_121 | AE retrieves information of a semanticMashupInstance resource via a semanticMashupInstance Retrieve Request |
| 188 |  |  | TD_M2M_NH_122 | AE updates attribute in <semanticMashupInstance> resource via a semanticMashupInstance Update Request |
| 189 |  |  | TD_M2M_NH_123 | AE deletes semanticMashupInstance resource via a semanticMashupInstance Delete Request |
| 190 |  |  | TD_M2M_NH_124 | AE retrieves information of a semanticMashupResult resource via a semanticMashupResult Retrieve Request |
| 191 |  |  | TD_M2M_NH_125 | AE deletes semanticMashupResult resource via a semanticMashupResult Delete Request |
| 192 |  |  | TD_M2M_NH_126 | AE triggers a calculation and generation of the mashup result by sending a <semanticMashupInstance>/<mashup> Retrieve Request |
| 193 |  | Ontology Repository | TD_M2M_NH_127 | AE creates a OntologyRepository resource in Registrar CSE via a OntologyRepository Create Request |
| 194 |  |  | TD_M2M_NH_128 | AE retrieves information of a ontologyRepository resource via a ontologyRepository Retrieve Request |
| 195 |  |  | TD_M2M_NH_129 | AE updates attribute in <ontologyRepository> resource via a ontologyRepository Update Request |
| 196 |  |  | TD_M2M_NH_130 | AE deletes OntologyRepository resource via a OntologyRepository Delete Request |
| 197 |  | Semantic validation | TD_M2M_NH_131 | AE checks the validity of the <semanticDescriptor> resource via a <semanticValidation> Update Request |
| 198 |  |  | TD_M2M_NH_132 | AE creates a <semanticDescriptor> resource visa SemanticDescriptor Create Request and Registrar CSE checks the validity of the created <semanticDescriptor> resource |
| 199 |  | Ontology Mapping | TD_M2M_NH_133 | AE creates an OntologyMapping resource in Registrar CSE via an OntologyMapping Create Request |
| 200 |  |  | TD_M2M_NH_134 | AE retrieves information of an ontology mapping result via a ontologyMapping Retrieve Request |
| 201 |  |  | TD_M2M_NH_135 | AE updates attribute in <ontologyMapping> resource via a ontologyMapping Update Request |
| 202 |  |  | TD_M2M_NH_136 | AE deletes OntologyMapping resource via a OntologyMapping Delete Request |
| 203 | 3GPP Interworking | IP data delivery (NIDD) | TD_M2M_SH_27 | IN-CSE establishes SCEF Configuration for NIDD |
| 204 |  |  | TD_M2M_SH_28 | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE |
| 205 |  |  | TD_M2M_SH_29 | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE |
| 206 |  | Monitoring events | TD_M2M_SH_30 | IN-AE monitors UE Reachability status |
| 207 |  |  | TD_M2M_SH_31 | UE Availability after DDN Failure scenario |
| 208 |  |  | TD_M2M_SH_32 | UE Communication Failure scenario |
| 209 |  |  | TD_M2M_SH_33 | Roaming status scenario |
| 210 |  |  | TD_M2M_SH_34 | Location Reporting scenario |
| 211 |  | 3GPP Based Device triggering | TD_M2M_SH_35 | IN-AE triggers ADN-AE hosted on UE |
| 212 |  |  | TD_M2M_SH_36 | IN-AE recalls/replaces a trigger request targeting AND-AE hosted on UE that has been already created in IN-CSE |
| 213 |  | Configuration of traffic patterns | TD_M2M_SH_37 | IN-CSE translates the oneM2M Node Traffic Pattern (TP) into a 3GPP Device Communication Pattern |
| 214 |  | Group message delivery using MBMS | TD_M2M_SH_38 | IN-AE creates a MBMS Group for handling group related requests |
| 215 |  |  | TD_M2M_SH_39 | IN-AE sends a request for accessing member resources to the Group Hosting CSE |
| 216 | Advanced Subscriptions & Notifications | Notification Target removal | TD_M2M_NH_137 | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request |
| 217 |  | NotificationTargetMgmtPolicyRef | TD_M2M_NH_138 | AE creates a notificationTargetMgmtPolicyRef resource in registrar CSE via a notificationTargetMgmtPolicyRef Create Request |
| 218 |  |  | TD_M2M_NH_139 | AE retrieves notificationTargetMgmtPolicyRef resource from Registrar CSE |
| 219 |  |  | TD_M2M_NH_140 | AE updates information about a notificationTargetMgmtPolicyRef via notificationTargetMgmtPolicyRef>Update Request |
| 220 |  |  | TD_M2M_NH_141 | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request |
| 221 |  | NotificationTargetPolicy | TD_M2M_NH_142 | AE creates a notificationTargetPolicy resource in registrar CSE via a notificationTargetPolicy Create Request |
| 222 |  |  | TD_M2M_NH_143 | AE retrieves notificationTargetPolicy resource from Registrar CSE |
| 223 |  |  | TD_M2M_NH_144 | AE updates information about a notificationTargetPolicy via <notificationTargetPolicy> Update Request |
| 224 |  |  | TD_M2M_NH_145 | AE removes notificationTargetPolicy via a <notificationTargetPolicy> Delete Request |
| 225 |  | PolicyDeletionRules | TD_M2M_NH_146 | AE creates a policyDeletionRules resource in registrar CSE via a policyDeletionRules Create Request |
| 226 |  |  | TD_M2M_NH_147 | AE retrieves policyDeletionRules resource from Registrar CSE |
| 227 |  |  | TD_M2M_NH_148 | AE updates information about a policyDeletionRules via <policyDeletionRules> Update Request |
| 228 |  |  | TD_M2M_NH_149 | AE removes policyDeletionRules via a <policyDeletionRules> Delete Request |
| 229 |  | CrossResourceSubscription | TD_M2M_NH_150 | AE creates a crossResourceSubscription resource in registrar CSE via a crossResourceSubscription Create Request |
| 230 |  |  | TD_M2M_NH_151 | AE retrieves crossResourceSubscription resource from Registrar CSE |
| 231 |  |  | TD_M2M_NH_152 | AE updates information about a crossResourceSubscription via <crossResourceSubscription> Update Request |
| 232 |  |  | TD_M2M_NH_153 | AE removes crossResourceSubscription via a <crossResourceSubscription> Delete Request |
| 233 |  |  | TD_M2M_NH_154 | AE receives a notification request from the HOST CSE |
| 234 | Modbus Interworking | Modbus Thermometer device | TD_M2M_NH_163 | AE1 creates Device Model for Modbus device |
| 235 |  |  | TD_M2M_NH_164 | Modbus IPE reads data from Modbus device and updates Registrar CSE with the read data |
| 236 |  |  | TD_M2M_NH_165 | AE writes data into a Modbus device by updating <flexContainer> resource in Registrar CSE |
| 237 | NoDN Interworking | Generic IPE | TD_M2M_NH_166 | NoDN IPE reads data from a NoDN device and updates Registrar CSE with the read data |
| 238 |  |  | TD_M2M_NH_167 | AE writes data into a NoDN device by updating <flexContainer> resource in Registrar CSE |

# 7	Configuration

## 7.1	Test Configuration

### 7.1.1	No hop

#### 7.1.1.1	M2M_CFG_01

The AE manages resources on the registrar CSE (Hosting CSE)

oneM2M entities model

Figure 7.1.1.1-1: M2M_CFG_01

#### 7.1.1.2	M2M_CFG_02

oneM2M entities model

Figure 7.1.1.2-1: M2M_CFG_02

#### 7.1.1.3	M2M_CFG_10

oneM2M entities model

Figure 7.1.1.3-1: M2M_CFG_10

NOTE: 	For HAIM Model tests, clause 8.5, AE1 represents a native oneM2M device that implements the HAIM models or a combined IPE(AE) plus a non-oneM2M device node (nodn).

### 7.1.2	Single hop

#### 7.1.2.1	M2M_CFG_03

The AE manages resources on the remote CSE

oneM2M entities model

Figure 7.1.2.1-1: M2M_CFG_03

#### 7.1.2.2	M2M_CFG_04

oneM2M entities model

Figure 7.1.2.2-1: M2M_CFG_04

#### 7.1.2.3	M2M_CFG_05

oneM2M entities model

Figure 7.1.2.3-1: M2M_CFG_05

#### 7.1.2.4	M2M_CFG_08

This configuration concerns group management when the AE is using a group to fan out requests to multiple members. The connection between the AE and the Group Hosting CSE, the Group Hosting CSE and the Member Hosting CSE may be a multi hop connection following the definition in clause 7.1.3.

This configuration is mapped to cases including:

- AE sends a request addressing <group>/fanOutPoint in the Group Hosting CSE, the Group Hosting CSE then further fans out the request to each Member Hosting CSE.

- The Member Hosting CSE sends a notification to the Group Hosting CSE pertaining to the subscription made through the Group Hosting CSE. The Group Hosting CSE then further aggregates the notification and sends it back to the AE.

Figure 7.1.2.4-1: M2M_CFG_08

#### 7.1.2.5	M2M_CFG_09

This configuration concerns device management using external technologies.

This configuration is mapped to cases including:

- The AE sends a request addressing <mgmtObj> to IN-CSE. IN-CSE then further acts as a Management Server to send management commands to Managed Entity over the mc interface. The management command is defined in OMA DM, BBF TR069 or LWM2M.

Figure 7.1.2.5-1: M2M_CFG_09

#### 7.1.2.6	M2M_CFG_11

This configuration concerns device management using 3GPP network.

Figure 7.1.2.6-1: M2M_CFG_11

#### 7.1.2.7	M2M_CFG_12

This configuration concerns group management when the IN-AE is using a group to fan out requests to multiple members in 3GPP interworking scenraios. The connection between the IN-AE and the Group Hosting CSE, the Group Hosting CSE and the Member Hosting CSE may be a multi hop connection following the definition in clause 7.1.3.

This configuration is mapped to cases including:

- IN-AE sends a request addressing <group>/fanOutPoint in the Group Hosting CSE, the Group Hosting CSE then further fans out the request to each Member Hosting CSE through 3GPP network.

Figure 7.1.2.7-1: M2M_CFG_12

### 7.1.3	Multi hops

#### 7.1.3.1	M2M_CFG_06

oneM2M entities model

Figure 7.1.3.1-1: M2M_CFG_06

#### 7.1.3.2	M2M_CFG_07

oneM2M entities model

Figure 7.1.3.2-1: M2M_CFG_07

# 8	Test Descriptions

## 8.1	No Hop configuration testing

### 8.1.1	CSEBase Management

#### 8.1.1.1	CSEBase Retrieve on Mca

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_01 |
| Objective: |  |  | AE retrieves the CSEBase resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.3.2 oneM2M TS-0004 [2], clause 7.3.2 |
| Pre-test conditions: |  |  | CSEBase resource has been automatically created in CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a retrieve Request to CSE with name {CSEBaseName} |
| 2 | Mca | PRO Check Primitive | Operation (op) = 2 (Retrieve) To (to) = Resource-ID of requested <CSEBase> resource, assumed CSE-relative here From (from) = AE-ID of request originator Request Identifier (rqi) = (token-string) |
| 3 | Mca | PRO Check Primitive | Response Status Code (rsc) = 2000 (OK) Request Identifier (rqi) = same string as received in request message Content (pc) = Serialized Representation of <CSEBase> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.2	RemoteCSE Management

#### 8.1.2.1	RemoteCSE Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_02 |
| Objective: |  |  | Registree CSE registers to Registrar CSE |
| Configuration: |  |  | M2M_CFG_02 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.2.1 oneM2M TS-0004 [2], clause 7.3.3.2.1 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE Create request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = Registree CSE-ID rqi = (token-string) ty = 16 (RemoteCSE) pc = Serialized representation of <RemoteCSE> resource |
| 3 | Mcc | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <RemoteCSE> resource |
| 4 |  | IOP Check | Check if possible that the <remoteCSE> resource has been created in registrar CSE. |
| 5 |  | IOP Check | Check if possible that the corresponding <remoteCSE> resource has been also created in registree CSE. |
| 6 |  | IOP Check | Registree CSE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.2.2	remoteCSE Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_03 |
| Objective: |  |  | Registree CSE retrieves RemoteCSE from Registrar CSE |
| Configuration: |  |  | M2M_CFG_02 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.2.2 oneM2M TS-0004 [2], clause 7.3.3.2.2 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} Registree CSE has created a remoteCSE resource on registrar CSE with name {RemoteCSEName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE retrieve request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{remoteCSEName} fr = Registree CSE-ID rqi = (token-string) pc = empty |
| 3 | Mcc | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <RemoteCSE> resource |
| 4 |  | IOP Check | Registree CSE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.2.3	remoteCSE Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_04 |
| Objective: |  |  | Registree CSE updates RemoteCSE from Registrar CSE |
| Configuration: |  |  | M2M_CFG_02 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.2.3 oneM2M TS-0004 [2], clause 7.3.3.2.3 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} Registree CSE has created a remoteCSE resource on registrar CSE with name {RemoteCSEName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE update request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{remoteCSEName} fr = Registree CSE-ID rqi = (token-string) pc = Serialized representation of updated <RemoteCSE> resource |
| 3 |  | IOP Check | Check if possible that the <remoteCSE> resource has been updated in registrar CSE. |
| 4 | Mcc | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <RemoteCSE> resource |
| 5 |  | IOP Check | Registree CSE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.2.4	remoteCSE Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_05 |
| Objective: |  |  | Registree CSE deletes RemoteCSE from Registrar CSE |
| Configuration: |  |  | M2M_CFG_02 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.2.4 oneM2M TS-0004 [2], clause 7.3.3.2.4 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} Registree CSE has created a remoteCSE resource on registrar CSE with name {RemoteCSEName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | Registree CSE is requested to send a RemoteCSE delete request to Registrar CSE |
| 2 | Mcc | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{remoteCSEName} fr = Registree CSE-ID rqi = (token-string) pc = empty |
| 3 | Mcc | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <remoteCSE> resource has been removed from registrar CSE. |
| 5 |  | IOP Check | Check if possible that the <remoteCSE> resource is also removed from registree CSE. |
| 6 |  | IOP Check | Registree CSE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.3	Application Entity Registration

#### 8.1.3.1	AE Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_06 |
| Objective: |  |  | AE registers to its registrar CSE via an AE Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.1.1 oneM2M TS-0004 [2], clause 7.3.5.2.1 |
| Pre-test conditions: |  |  | CSEBase resource has been created in CSE with name {CSEBaseName} AE does not have an AE-ID, i.e. it registers from scratch |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a AE Create request to register to the Registrar CSE |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 2 (AE) pc = Serialized representation of <AE> resource |
| 3 |  | IOP Check | Check if possible that the <AE> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <AE> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.3.2	AE Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_07 |
| Objective: |  |  | AE retrieves <AE> resource via an AE Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.1.2 oneM2M TS-0004 [2], clause 7.3.5.2.2 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE}bgf |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a AE retrieve request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE} fr = AE-ID of request originator rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <AE> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.3.3	AE Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_08 |
| Objective: |  |  | AE updates attribute in <AE> resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.1.3 oneM2M TS-0004 [2], clause 7.3.5.2.3 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an AE Update Request |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{AE} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <AE> resource |
| 3 |  | IOP Check | Check if possible that the <AE> resource has been updated in registrar CSE. |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <AE> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.3.4	AE Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_09 |
| Objective: |  |  | AE de-registers by deleting <AE> resource via an AE Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.1.4 oneM2M TS-0004 [2], clause 7.3.5.2.4 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an AE Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{AE} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <AE> resource has been removed from registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.4	Container Management

#### 8.1.4.1	Container Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_10 |
| Objective: |  |  | AE creates a container resource in registrar CSE via a container Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.5.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <container> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE> resource fr = AE-ID rqi = (token-string) ty = 3 (Container) pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Check if possible that the <container> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.4.2	Container Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_11 |
| Objective: |  |  | AE retrieves information of a container resource via a container Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.2 oneM2M TS-0004 [2], clause 7.3.5.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <container> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <container> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.4.3	Container Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_12 |
| Objective: |  |  | AE updates attribute in application resource via a container Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.3 oneM2M TS-0004 [2], clause 7.3.5.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a container Update Request to update the lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <container> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the < container > resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.4.4	Container Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_13 |
| Objective: |  |  | AE deletes a specific container resource via a container Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.4 oneM2M TS-0004 [2], clause 7.3.5.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a container Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <container> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <container> resource is deleted in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <container> resource has been removed in registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.5	ContentInstance Management

#### 8.1.5.1	ContentInstance Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_14 |
| Objective: |  |  | AE adds a contentInstance resource <contentInstance> to a specific container in Registrar CSE via a contentInstance Create Request and the Registrar CSE updates the parent <container> resource with stateTag, currentNrOfInstances, and CurrentByteSize attributes correspondingly |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.19.2 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE AE has created a container resource <container> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a RETRIEVE request with resultContent set to 1 (default value) to retrieve the <container> resource and AE sends a request to create a < contentInstance > resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of < container > resource fr = AE-ID rqi = (token-string) ty = 4 (contentInstance) pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the <contentInstance> resource is created in Registrar CSE and AE sends a RETRIEVE request to the <container> resource to check that if the Registrar CSE has updated stateTag, currentNrOfInstances, and CurrentByteSize attribute correspondingly which is resulted from the successful creation of child <contentInstance> resource. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | AE indicates successful CREATE operation of <contentInstance> and indicates Registrar CSE has updated stateTag, currentNrOfInstances, and CurrentByteSize attribute correspondingly by checking the response of a <container> request to the <container> resource |
| IOP Verdict |  | Set verdict to pass if IOP check goal is achieved exactly, otherwise verdict fail is set with corresponding error message. |  |
| PRO Verdict |  |  |  |

#### 8.1.5.2	ContentInstance Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_15 |
| Objective: |  |  | AE retrieves information of a contentInstance resource via a contentInstance Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.19.3 oneM2M TS-0004 [2], clause 7.3.6.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created a contentInstance resource <contentInstance> as child resource of <container> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <contentInstance> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <contentInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <contentInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.5.3	ContentInstance Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_17 |
| Objective: |  |  | AE deletes contentInstance resource via a contentInstance Delete Request and the Registrar CSE updates the parent <container> resource with currentNrOfInstances, and CurrentByteSize attributes correspondingly |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.19.5 oneM2M TS-0004 [2], clause 7.3.6.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created a contentInstance resource <contentInstance> as child resource of <container> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a RETRIEVE request with resultContent set to 1 (default value) to retrieve the <container> resource and AE is requested to send a contentInstance Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <contentInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <contentInstance> resource is deleted in Registrar CSE and AE sends a RETRIEVE request to the parent <container> resource to check that if the Registrar CSE has updated currentNrOfInstances, and CurrentByteSize attribute correspondingly which is resulted from the successful deletion of child <contentInstance> resource. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <contentInstance> resource has been removed in registrar CSE. |
| 6 |  | IOP Check | AE indicates successful DELETE operation of <contentInstance> and indicates Registrar CSE has updated currentNrOfInstances, and CurrentByteSize attribute correspondingly |
| IOP Verdict |  | Set the verdict to pass if both the <contentInstance> is deleted and the Registrar CSE updated_currentNrOfInstances,_ and CurrentByteSize attribute. Otherwise, set the verdict to fail with corresponding error message. |  |
| PRO Verdict |  |  |  |

#### 8.1.5.4	<latest> ContentInstance Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_49 |
| Objective: |  |  | AE deletes a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.22.2 oneM2M TS-0004 [2], clause 7.4.28.2.5 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created more than one contentInstances <contentInstance> as child of <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE retrieves a <latest> resource in a <container> and then sends a DELETE request to the <latest> resource of the <container> |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <latest> resource of a <container> fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = representation of deleted <latest> resource of a <container> |
| 4 |  | IOP Check | AE indicates successful DELETE operation of a <latest> resource and AE sends a RETRIEVE request to <latest> resource of a <container> to check if the retrieved <latest> resource in the <container> is different with that one that was retrieved before DELETE request of the <latest> resource in terms of resourceID and resourceName attribute value. |
| IOP Verdict |  | Set the verdict to pass if IOP check goal is achieved, otherwise set the verdict to fail with corresponding error message. |  |
| PRO Verdict |  |  |  |

#### 8.1.5.5	<oldest> ContentInstance Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_50 |
| Objective: |  |  | AE deletes a <oldest> resource of a <container> and the Registrar CSE points an oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.22.2 oneM2M TS-0004 [2], clause 7.4.28.2.5 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created more than one contentInstances <contentInstance> as child of <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE retrieves a <oldest> resource of a <container> and AE sends a DELETE Request to the <oldest> resource of the <container> |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <oldest> resource of a <container> fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = representation of deleted <oldest> resource of a <container> |
| 4 |  | IOP Check | AE indicates successful DELETE operation of a <oldest> resource and AE sends a RETRIEVE request to <oldest> resource of a <container> to check if the retrieved <oldest> resource in the <container> is different with that one that was retrieved before DELETE request of the <oldest> resource in terms of resourceID and resourceName attribute values. |
| IOP Verdict |  | Set the verdict to pass if IOP check goal is achieved, otherwise set the verdict to fail with corresponding error message. |  |
| PRO Verdict |  |  |  |

#### 8.1.5.6	ContentInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <container> resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_51 |
| Objective: |  |  | AE sends a <contentInstance> CREATE request to a <container> which contains attribute currentNrOfInstances whose value equals to that of maxNrOfInstances and Registrar CSE deletes the oldest <contentInstance> from the parent <container> and then creates the requested <contentInstance> resource for the originator AE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.19.2 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE AE has created a container resource <container> (where the number of contentInstances equals to the value set in maxNrOfInstance) on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a RETRIEVE request with resultContent set to 1 (default value) to retrieve the <oldest> contentInstance resource and AE sends a request to create a <contentInstance> resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <container> resource fr = AE-ID rqi = (token-string) ty = 4 (contentInstance) pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the <oldest> resource of a <container> is deleted |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | AE indicates successful CREATE operation of <contentInstance> and indicates the representation of the recent <oldest> resource in the <container> is different with that of <oldest> resource retrieved at the beginning of test in terms of resourceID and resourceName attribute value |
| IOP Verdict |  | Set the verdict to pass if IOP check goal is achieved, otherwise set the verdict to fail with corresponding error message. |  |
| PRO Verdict |  |  |  |

#### 8.1.5.7	<latest> ContentInstance Retrieve

##### 8.1.5.7.1	Attribute locationID of the <container> resource configured

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_102 |
| Objective: |  |  | AE retrieves a <latest> resource of a <container> for which attribute locationID is configured, value of locationUpdatePeriod is marked '0' or not defined and locationSource attribute is 'Network Based'. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.22.1 oneM2M TS-0004 [2], clause 7.4.27.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created a <locationPolicy> resource on Registrar CSE having its resourceID and locationContainerID attribute set to locationID and resourceID attribute of the <container> resource respectively. In resource <locationPolicy>, value of locationUpdatePeriod is marked '0' or not defined and locationSource attribute is 'Network Based'. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <latest> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <container> resource/la fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of latest <contentInstance> resource, created after acquiring location info from Location Server, in response to Location Request made by the CSE, using the attributes stored in the <locationPolicy > resource. |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.1.5.7.2	Attribute locationID of the <container> resource not configured

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_71 |
| Objective: |  |  | AE retrieves a <latest> resource of a <container> and the Registrar CSE points a latest <contentInstance> among the existing contentInstances to the <latest> resource of the <container> |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.22.1 oneM2M TS-0004 [2], clause 7.4.27.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created multiple contentInstance resources <contentInstance> as child resource of <container> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <latest> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <container> resource/la fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of latest <contentInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.5.8	<oldest> ContentInstance Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_72 |
| Objective: |  |  | AE retrieves a <oldest> resource of a <container> and the Registrar CSE points a oldest <contentInstance> among the existing contentInstances to the <oldest> resource of the <container> |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.23.1 oneM2M TS-0004 [2], clause 7.4.28.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created multiple contentInstance resources <contentInstance> as child resource of <container> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <oldest> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <container> resource/ol fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of oldest <contentInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |

### 8.1.6	Discovery

#### 8.1.6.1	Discovery of all resources

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_18 |
| Objective: |  |  | AE discovers all accessible resources from registrar CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.2.3.13 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a discovery request to registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE-ID rqi = (token-string) fu=1 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing addresses of all discovered resources |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.6.2	Discovery with label filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_19 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using the label filter criteria |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.2.3.13 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} A <Container> resource with label "key1" is created on Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover the <Container> resource using the label filter criteria |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE-ID rqi = (token-string) fu=1 lbl=key1 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of the <Container> address |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.6.3	Discovery with limit filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_20 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE limiting the number of matching resources to the specified value. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.2.3.13 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover at most 2 resources in registrar CSE. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE-ID rqi = (token-string) fu=1 lim=2 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message cnst=1 cnot=2 pc = Serialized representation of data object containing the address of the <Container> address |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.6.4	Discovery with multiple filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_21 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using multiple Filter Criteria |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.2.3.13 |
| Pre-test conditions: |  |  | Two <Container> resources with labels "key1" and "key2" are created in Registrar CSE. A <Group> resources with labels "key1" and "key2" is created in Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using multiple filter criteria (label, resource type and limit) |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE-ID rqi = (token-string) fu=1 lbl=key1 lbl=key2 ty=3 lim=1 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of one of the <Container> resources |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.6.5	Discovery with level filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_58 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 1 |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.14 |
|  |  |  | A <Container> resource is created under both <AE> resources in Registrar CSE. A <ContentInstance> resource is created under both <Container> resources in Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using level filter criteria value set to 1 |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu=1 lvl=1 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of both <AE> resources |
| 4 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_59 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 2 |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.14 |
| Pre-test conditions: |  |  | <AE1> and <AE2> resources are created in Registrar CSE. A <Container> resource is created under both <AE> resources in Registrar CSE. A <ContentInstance> resource is created under both <Container> resources in Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using level filter criteria value set to 2 |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu=1 lvl=2 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of all <AE> and <Container> resources |
| 4 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_60 |
| Objective: |  |  | AE1 discovers accessible resources residing in Registrar CSE using the level filter criteria value set to 3 |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.14 |
| Pre-test conditions: |  |  | <AE1> and <AE2> resources are created in Registrar CSE. A <Container> resource is created under both <AE> resources in Registrar CSE. A <ContentInstance> resource is created under both <Container> resources in Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using level filter criteria value set to 3 |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu=1 lvl=3 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of all <AE>, <Container> and <ContentInstance>resources |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.6.6	Discovery with offset filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_61 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using the offset filter criteria value set to 3 |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.14 |
| Pre-test conditions: |  |  | <AE1> and <AE2> resources are created in Registrar CSE. A <Container> resource is created under both <AE> resources in Registrar CSE. A <ContentInstance> resource is created under both <Container> resources in Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using offset filter criteria value set to 3 |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu=1 ofst=3 pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing only 3 of the 6 <AE>, <container> and <contentInstance> resources hosted by the Registrar CSE |
| 4 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_62 |
| Objective: |  |  | AE discovers all the accessible resources residing in Registrar CSE using the offset filter criteria |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.14 |
| Pre-test conditions: |  |  | <AE1> and <AE2> resources are created in Registrar CSE. A <Container> resource is created under both <AE> resources in Registrar CSE. A <ContentInstance> resource is created under both <Container> resources in Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover specific resources located in Registrar CSE using offset filter criteria attribute value set to 0 (Default value) and limit filter Criteria attribute value set to 2. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu=1 lim=2 pc = empty |
| 3 |  | IOP Check | Registrar CSE sends success response to AE1 |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message cnst=1 cnot=2 pc = Serialized representation of data object containing the address of first 2 resources hosted by Registrar CSE |
| 5 |  | IOP Check | AE1 sends discovery request to Registrar CSE with offset filtercriteria value set to 2 and limit filter criteria attribute value set to 2 |
| 6 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu=1 ofst=2 lim=2 pc = empty |
| 7 |  | IOP Check | Registrar CSE sends success response to AE1 |
| 8 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message cnst=1 cnot=4 pc = Serialized representation of data object containing the address of next 2 resources hosted by Registrar CSE |
| 9 |  | IOP Check | AE1 sends discovery request to Registrar CSE with offset filtercriteria value set to 4 and limit filtercriteria attribute value set to 2 |
| 10 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu=1 ofst=4 lim=2 pc = empty |
| 11 |  | IOP Check | Registrar CSE sends success response to AE1 |
| 12 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message cnst =2 pc = Serialized representation of data object containing the address of last 2 resources hosted by Registrar CSE |
| 13 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.6.7	Discovery with geoQuery filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_168 |
| Objective: |  |  | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 1 (within) |
| Configuration: |  |  | M2M_CFG_01 |
| Applicable Release s |  |  | From Rel-4 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.17 |
| Pre-test conditions: |  |  | <AE1> and <AE2> resources are created in Registrar CSE. A <container> resource is created under the <AE2> resource in Registrar CSE. The <container> resource has location attribute set to type as 2(LineString) and coordinates as [[100.0, 0.0], [101.0, 1,0]]. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover resources located in Registrar CSE using geoQuery filter criteria |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu = 1 (Discovery Criteria) gq = {gmty=3(Polygon), geom=[[99.0,0.0],[99.0,1.0],[101.0,0.0],[101.0,1.0]], gsf=1(within)} pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing <container> which fulfill the geoQuery discovery hosted by the Registrar CSE |
| 4 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_169 |
| Objective: |  |  | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 2 (contains) |
| Configuration: |  |  | M2M_CFG_01 |
| Applicable Releases |  |  | From Rel-4 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.17 |
| Pre-test conditions: |  |  | <AE1> and <AE2> resources are created in Registrar CSE. A <container> resource is created under the <AE2> resource in Registrar CSE. The <container> resource has location attribute set to type as 3(Polygon) and coordinates as [99.0,0.0],[99.0,1.0],[101.0,0.0],[101.0,1.0]. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover resources located in Registrar CSE using geoQuery filter criteria |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu = 1 (Discovery Criteria) gq = {gmty=2(LineString), geom=[[100.0, 0.0], [101.0, 1,0]], gsf=2(contains)} pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing <container> which fulfill the geoQuery discovery hosted by the Registrar CSE |
| 4 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_170 |
| Objective: |  |  | AE discovers using geoQuery filter criteria when geoSpatialFunctionType set to 3 (intersects) |
| Configuration: |  |  | M2M_CFG_01 |
| Applicable Release |  |  | From Rel-4 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.3.3.17 |
| Pre-test conditions: |  |  | <AE1> and <AE2> resources are created in Registrar CSE. A <container> resource is created under the <AE2> resource in Registrar CSE. The <container> resource has location attribute set to type as 3(Polygon) and coordinates as [99.0,0.0],[99.0,1.0],[101.0,0.0],[101.0,1.0]. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request in order to discover resources located in Registrar CSE using geoQuery filter criteria |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE1-ID rqi = (token-string) fu = 1 (Discovery Criteria) gq = {gmty=3(Polygon), geom=[[101.0,0.0],[101.0,1.0],[103.0,0.0],[103.0,1.0]], gsf=3(intersects)} pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing <container> which fulfill the geoQuery discovery hosted by the Registrar CSE |
| 4 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.7	Subscription Management

#### 8.1.7.1	Subscription Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_22 |
| Objective: |  |  | AE creates a subscription to Application Entity resource via subscription Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.11.2 oneM2M TS-0004 [2], clause 7.3.7.2 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE AE has created a container resource <container> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a subscription Create request to the Registrar CSE |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <Container> resource fr = AE-ID rqi = (token-string) ty = 23 (Subscription) pc = Serialized representation of <Subscription> resource |
| 3 |  | IOP Check | Check if possible that the <Subscription> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <Subscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.7.2	Subscription Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_23 |
| Objective: |  |  | AE retrieves subscription resource from Registrar CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.11.3 oneM2M TS-0004 [2], clause 7.3.7.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created a subscription resource <subscription> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <subscription> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <Subscription> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <Subscription> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.7.3	Subscription Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_24 |
| Objective: |  |  | AE updates information about a subscription via subscription Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.11.4 oneM2M TS-0004 [2], clause 7.3.7.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created a subscription resource <subscription> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a subscription Update Request to update the lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <Subscription> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <Subscription> resource |
| 3 |  | IOP Check | Check if possible that the <subscription> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <Subscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.7.4	Subscription Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_25 |
| Objective: |  |  | AE cancels subscription via an subscription Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.11.5 oneM2M TS-0004 [2], clause 7.3.7.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE AE has created a subscription resource <subscription> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a subscription Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <Subscription> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <Subscription> resource is deleted in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <subscription> resource has been removed in registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.8	accessControlPolicy Management

#### 8.1.8.1	accessControlPolicy Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_26 |
| Objective: |  |  | AE creates an accessControlPolicy resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1] clause 10.2.21.1 oneM2M TS-0004 [2], clause 7.3.1.2 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy Create Request |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/{AE} fr = AE-ID rqi = (token-string) ty = 1 (accessControlPolicy) pc = Serialized representation of <accessControlPolicy > resource |
| 3 |  | IOP Check | Check if possible that the <container> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.8.2	accessControlPolicy Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_27 |
| Objective: |  |  | AE retrieves accessControlPolicy resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.21.2 oneM2M TS-0004 [2], clause 7.3.1.2 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a accessControlPolicy retrieve request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{accessControlPolicyName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.8.3	accessControlPolicy Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_28 |
| Objective: |  |  | AE updates attribute in accessControlPolicy resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.21.3 oneM2M TS-0004 [2], clause 7.3.1.2 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy update request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{AE}/{accessControlPolicyName} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <accessControlPolicy > resource |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been updated in registrar CSE. |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.8.4	accessControlPolicy Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_29 |
| Objective: |  |  | AE deletes accessControlPolicy resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.21.4 oneM2M TS-0004 [2], clause 7.3.1.2 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy delete request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{AE}/{accessControlPolicyName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been removed from registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.8.5	Unauthorized operation (Insufficient Access Rights, operations)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_30 |
| Objective: |  |  | AE delete request is rejected due to accessControlPolicy (accessControlOperations) |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0004 [2], clause 7.3.3.15 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}, and accessControlOperations with no delete privilege AE has created a <container> resource on registrar CSE under <AE>, with name {containerName} and accessControlPolicyIDs including proper identifier of accessControlPolicy resource. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a container Delete Request for resource <container> |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <container> resource has not been removed in registrar CSE. |
| 5 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.8.6	Unauthorized operation (Insufficient Access Rights, originators)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_73 |
| Objective: |  |  | AE delete request is rejected due to accessControlPolicy (accessControlOriginators) |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0004 [2], clause 7.3.3.15 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}, and accessControlOriginators with no privilege for AE. AE has created a <container> resource on registrar CSE under <AE>, with name {containerName} and accessControlPolicyIDs including proper identifier of accessControlPolicy resource. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a container Delete Request for resource <container> |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <container> resource has not been removed in registrar CSE. |
| 5 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.8.7	Authorized operation

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_74 |
| Objective: |  |  | AE delete request is allowed due to accessControlPolicy |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0004 [2], clause 7.3.3.15 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created a <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName}, and accessControlOperations with delete privilege and accessControlOriginators with privilege for AE. AE has created a <container> resource on registrar CSE under <AE>, with name {containerName} and accessControlPolicyIDs including proper identifier of accessControlPolicy resource. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a container Delete Request for resource <container> |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <container> resource has been removed in registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.9	Group Management

#### 8.1.9.1	Group Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_32 |
| Objective: |  |  | AE retrieves group resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.3 oneM2M TS-0004 [2], clause 7.4.14.2.2 |
| Pre-test conditions: |  |  | AE has created a <group> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a group Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (RETRIEVE) to = {CSEBaseName}/{group} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <group> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.9.2	Group Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_31 |
| Objective: |  |  | AE creates a group resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.2 oneM2M TS-0004 [2], clause 7.4.14.2.2 |
| Pre-test conditions: |  |  | void |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a group Create Request |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 9 (group) pc = Serialized representation of <group> resource |
| 3 |  | IOP Check | Check if possible that the <group> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <group> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.9.3	Group Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_33 |
| Objective: |  |  | AE updates attribute in group resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.4 oneM2M TS-0004 [2], clause 7.4.14.2.4 |
| Pre-test conditions: |  |  | AE has created a <group> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a group Update Request |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{group} fr = AE-ID rqi = (token-string) pc = Serialized representation of <group> resource |
| 3 |  | IOP Check | Check if possible that the <group> resource is updated in Registrar CSE.. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message pc = Serialized representation of <group> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.9.4	Group Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_34 |
| Objective: |  |  | AE deletes group resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.5 oneM2M TS-0004 [2], clause 7.4.14.2.5 |
| Pre-test conditions: |  |  | AE has created a <group> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a group Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (DELETE) to = {CSEBaseName}/{group} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check if possible that the <group> resource is deleted in Registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.10	Node Management

#### 8.1.10.1	Node Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_35 |
| Objective: |  |  | AE creates a node resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.14.1 oneM2M TS-0004 [2], clause 7.3.18.2.1 |
| Pre-test conditions: |  |  | void |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a node Create Request |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 14 (node) pc = Serialized representation of <node> resource |
| 3 |  | IOP Check | Check if possible that the <node> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <node> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.10.2	Node Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_36 |
| Objective: |  |  | AE retrieves node resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.14.2 oneM2M TS-0004 [2], clause 7.3.18.2.2 |
| Pre-test conditions: |  |  | AE has created a <node> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a node Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (RETRIEVE) to = {CSEBaseName}/{node} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <node> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.10.3	Node Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_37 |
| Objective: |  |  | AE updates attribute in node resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.14.3 oneM2M TS-0004 [2], clause 7.3.18.2.3 |
| Pre-test conditions: |  |  | AE has created a <node> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a node Update Request |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{node} fr = AE-ID rqi = (token-string) pc = Serialized representation of <node> resource |
| 3 |  | IOP Check | Check if possible that the <node> resource is updated in Registrar CSE.. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message pc = Serialized representation of <node> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.10.4	Node Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_38 |
| Objective: |  |  | AE deletes node resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.14.4 oneM2M TS-0004 [2], clause 7.3.18.2.4 |
| Pre-test conditions: |  |  | AE has created a <node> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a node Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (DELETE) to = {CSEBaseName}/{node} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check if possible that the <node> resource is deleted in Registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.11	PollingChannel Management

#### 8.1.11.1	PollingChannel Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_39 |
| Objective: |  |  | AE creates a <pollingChannel> resource in registrar CSE via a Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.13.2 oneM2M TS-0004 [2], clause 7.3.21.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a < pollingChannel > |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE> resource fr = AE-ID rqi = (token-string) ty = 15 (pollingChannel) pc = Serialized representation of < pollingChannel > resource |
| 3 |  | IOP Check | Check if possible that the < pollingChannel > resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of < pollingChannel > resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.11.2	PollingChannel Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_40 |
| Objective: |  |  | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.13.3 oneM2M TS-0004 [2], clause 7.3.21.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource < pollingChannel > on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a < pollingChannel > |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of < pollingChannel > resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of < pollingChannel > resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.11.3	pollingChannel Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_41 |
| Objective: |  |  | AE updates attribute in pollingChannel resource via a Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.13.4 oneM2M TS-0004 [2], clause 7.3.21.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a pollingChannel Update Request to update the lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of < pollingChannel > resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated < pollingChannel > resource |
| 3 |  | IOP Check | Check if possible that the < pollingChannel > resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of < pollingChannel > resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.11.4	pollingChannel Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_42 |
| Objective: |  |  | AE deletes a pollingChannel resource via a Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.13.5 oneM2M TS-0004 [2], clause 7.3.21.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a subscription Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of < pollingChannel > resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the < pollingChannel > resource is deleted in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the < pollingChannel > resource has been removed in registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.11.5	Long Polling on a PollingChannel Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_43 |
| Objective: |  |  | AE retrieves information of a pollingChannel resource via a Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.13.7 oneM2M TS-0004 [2], clause 7.3.22.2.2 |
| Pre-test conditions: |  |  | A pollingChannel resource < pollingChannel > has been created in application <AE> on the Registrar CSE A subscription to a <container> resource has been created using the <pollingChannel> as a notificationURI in the subscription. A single <contentInstance> resource is created in the subscribed to resource. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a pollingChannelURI Retrieve Request for <pollingChannelURI> |
| 2 | Mca | PRO Check Primitive | Sent RETRIEVE request contains To: <CSEBase>/<AE>/<pollingChannel>/pollingChannelURI Fr: AE-ID |
| 3 | Mca | PRO Check Primitive | Sent RETRIEVE response contains To: AE-ID Fr: CSE-ID Response Status Code: OK Cn: pending Notification request |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  |  | Repeat steps 1-2. There is no pending request. When the Request Expiration Timestamp expires Registrar sends response indicating "REQUEST_TIMEOUT" |
| 6 | Mca | PRO Check Primitive | Sent RETRIEVE response contains To: AE-ID Fr: CSE-ID Response Status Code: REQUEST_TIMEOUT |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.12	FanoutPoint Management

#### 8.1.12.1	FanoutPoint Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_44 |
| Objective: |  |  | AE creates a <contentInstance> resource in each group member |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.6 oneM2M TS-0004 [2], clause 7.3.14.3.1 |
| Pre-test conditions: |  |  | A group is created containing 2 members of type <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request to create <contentInstance> in each group member |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) ty = 4 (contentInstance) pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the <contentInstance> resource is created in each member hosting CSE |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = aggregated response |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  | Verify that the aggregate response includes responses from each member of the group |  |
| PRO Verdict |  |  |  |

#### 8.1.12.2	FanoutPoint Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_45 |
| Objective: |  |  | AE retrieves the <container> resource from in each group member |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.8 oneM2M TS-0004 [2], clause 7.3.14.3.2 |
| Pre-test conditions: |  |  | A group is created containing 2 members of type <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request to the fanoutPoint of <group> resource |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) |
| 3 |  | IOP Check |  |
| 4 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = aggregated response |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  | Verify that the aggregate response includes responses from each member of the group |  |
| PRO Verdict |  |  |  |

#### 8.1.12.3	FanoutPoint Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_46 |
| Objective: |  |  | AE updates an <container> resource of each member resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.9 oneM2M TS-0004 [2], clause 7.3.14.3.3 |
| Pre-test conditions: |  |  | A group is created containing 2 members of type <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Update Request to the fanoutPoint of <group> resource to lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Check if possible that both of the <container> resources have been updated in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message pc = aggregated response |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  | Verify that the aggregate response includes responses from each member of the group |  |
| PRO Verdict |  |  |  |

#### 8.1.12.4	FanoutPoint Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_47 |
| Objective: |  |  | AE deletes a <container> of each member |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.10 oneM2M TS-0004 [2], clause 7.3.14.3.4 |
| Pre-test conditions: |  |  | A group is created containing 2 members of type <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Delete 'oldest' Request to the fanoutPoint of <group> resource |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = aggregated response |
| 4 |  | Verify | Check if possible that the oldest <contentInstance> resource has been removed in registrar CSE. |
| 5 |  | Verify | AE indicates successful operation |
| IOP Verdict |  | Verify that the aggregate response includes responses from each member of the group |  |
| PRO Verdict |  |  |  |

### 8.1.13	Notification Management

#### 8.1.13.1	Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_48 |
| Objective: |  |  | AE receives a notification request from the HOST CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.12 oneM2M TS-0004 [2], clause 7.4.1 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE AE1 has created a container resource <container> on registrar CSE AE1 has created a <subscription> as a child resource of a <container> AE2 has created an application resource <AE> on registrar CSE AE2 has permissions to UPDATE the container created by AE1 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Update request to the <container> created by AE1. This triggers or causes the HOST CSE to send a notification to AE1. |
| 2 | Mca | PRO Check Primitive | op = 5 (Notify) to = notificationURI of subscription resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if the notification representation |
| 4 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE1 indicates notification received |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.13.2	Update Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_80 |
| Objective: |  |  | AE2 sends maxNrOfInstances UPDATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has specific setting in eventNotificationCriteria, Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [1], clause 9.6.8 oneM2M TS-0004 [2], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE1 has created a <container> resource on registrar CSE AE1 has created <subscription> resources under the <container> resource. AE1 has set attribute of eventNotificationCriteria to a specific condition (e.g. maxNrOfInstances in this scenario) and notificationEventType set to 1(Update of Resource) AE2 has permissions to UPDATE the <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Update Request to the <container>. This triggers or causes the Hosting CSE to send a notification to AE1. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = ID of <container> resource from = AE2-ID rqi = (token-string) pc = Serialized representation of maxNrOfInstances update |
| 3 |  | IOP Check | Hosting CSE successfully updated mxNrOfInsatnce of the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 5 |  | IOP Check | AE2 successfully received response of Update request |
| 6 | Mca | PRO Check Primitive | op = 5 (Notify) to = ID of <subscription> resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notify request which contain notificationEventType equal to 1(Update of Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| 10 | Mca | PRO Check Primitive | op = 3 (Update) to = ID of <container> resource from = AE2-ID rqi = (token-string) pc = Serialized representation of labels update(can be any other attribute) |
| 11 |  | IOP Check | Hosting CSE successfully updated labels of the <container> resource |
| 12 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 13 |  | IOP Check | AE2 successfully received response of Update request, notify request will not be sent |
| IOP Verdict |  | Check that the notification is only sent when there is update of maxNrOfInstances due to setting of the eventNotificationCriteria |  |
| PRO Verdict |  |  |  |

#### 8.1.13.3	Delete Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_81 |
| Objective: |  |  | AE2 sends DELETE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with 'Delete of Resource', Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [1], clause 9.6.8 oneM2M TS-0004 [2], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE1 has created a <container> resource on registrar CSE AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 2(Delete of Resource) AE2 has permissions to DELETE the <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Delete Request to the <container>. This triggers or causes the Hosting CSE to send a notification to AE1. |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = ID of <container> resource fr = AE2-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Hosting CSE successfully deleted the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | AE2 successfully received response of Delete request |
| 6 | Mca | PRO Check Primitive | op = 5 (Notify) to = ID of <subscription> resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notify request which contain notificationEventType equal to 2(Delete of Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.13.4	Creation of Direct Child Resource Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_82 |
| Objective: |  |  | AE2 sends <contentInstance> CREATE request to <container> which has been set to subscribed-to resource. Since <subscription> resource has notificationEventType with 'Create of Direct Child Resource', Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [1], clause 9.6.8 oneM2M TS-0004 [2], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE1 has created a <container> resource on registrar CSE AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 3(Create of Direct Child Resource) AE2 has permissions to CREATE child resource under the <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Create Request of <contentInstance> resource to the <container> resource. This triggers or causes the Hosting CSE to send a notification to AE1 |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = ID of <container> resource fr = AE2-ID rqi = (token-string) ty = 4 (contentInstance) pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <contentInstance> resource under the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | AE2 successfully received response of Create request |
| 6 | Mca | PRO Check Primitive | op = 5 (Notify) to = ID of <subscription> resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notify request which contain notificationEventType equal to 3(Create of Direct Child Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.13.5	Deletion of Direct Child Resource Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_83 |
| Objective: |  |  | AE2 sends DELETE request to the <contentInstance> which is located under the subscribed-to resource. Since <subscription> resource has notificationEventType with 'Delete of Direct Child Resource', Hosting CSE send notification to AE1 |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [1], clause 9.6.8 oneM2M TS-0004 [2], clause 6.3.4.2.19 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE1 has created a <container> resource on registrar CSE AE1 has created a <contentInstance> as a child resource of <container> created by AE1 AE1 has created a <subscription> under the <container> resource, with notificationEventType set to 4(Delete of Direct Child Resource) AE2 has permissions to DELETE the <contentInstance> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Delete Request to the <contentinstance>. This triggers or causes the Hosting CSE to send a notification to AE1. |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = ID of <contentInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Hosting CSE successfully deleted the <contentInstance> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | AE2 successfully received response of Delete request |
| 6 | Mca | PRO Check Primitive | op = 5 (Notify) to = ID of <subscription> resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notify request which contain notificationEventType equal to 4(Delete of Direct Child Resource) |
| 7 |  | IOP Check | AE1 successfully received Notify request |
| 8 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Hosting CSE successfully received response of Notify request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.13.6	Notifcation Aggregation

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_89 |
| Objective: |  |  | AE creates <subscription> resources by sending Create Request to the fanOutPoint. Since AE has set notifyAggregation to 2, Hosting CSE aggregate notification and send aggregated notification to AE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.11 oneM2M TS-0004 [2], clause 7.4.14.2.4 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created two <container> resources on registrar CSE AE has created a <group> resource with memberIDs set to two <container> resources. AE has set number in notifyAggregation to 2 AE has created <subscription> resources under the members of <group> resources by sending Create Request to the fanOutPoint. AE has set notificationForwardingURI and notificationEventType set to 3(Create of Direct Child Resource) |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of <contentInstance> resource to the fanOutPoint |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) ty = 4 (contentInstance) pc = Serialized representation of <subscription> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <contentInstance> resources in each member |
| 4 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = aggregated response |
| 5 |  | IOP Check | AE successfully received converged response |
| 6 | Mca | PRO Check Primitive | op = 5 (Notify) to = ID of <subscription> resource from = Registrar CSE-ID rqi = (token-string) pc = aggregated Notify request which contain the occurrence of child resource creation |
| 7 |  | IOP Check | AE successfully received aggregated Notify request |
| 8 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | Group Hosting CSE successfully received response of Notify request |
| IOP Verdict |  | Check that the response is aggregated by the group Hosting CSE and successfully parse to AE |  |
| PRO Verdict |  |  |  |

### 8.1.14	FlexContainer Management

#### 8.1.14.1	FlexContainer Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_52 |
| Objective: |  |  | AE creates a flexContainer resource in Registrar CSE via a flexContainer Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.29.1, 9.6.1.2.2 oneM2M TS-0004 [2], clause 7.4.37.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <flexContainer> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.14.2	FlexContainer Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_53 |
| Objective: |  |  | AE retrieves information of a flexContainer resource via a flexContainer Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clauses 10.2.29.2, 9.6.1.2.2 oneM2M TS-0004 [2], clause 7.4.37.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a flexContainer resource <flexContainer> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <flexContainer> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <flexContainer> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.14.3	FlexContainer Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_54 |
| Objective: |  |  | AE updates attribute in application resource via a flexContainer Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clauses 10.2.29.3, 9.6.1.2.2 oneM2M TS-0004 [2], clause 7.4.37.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a flexContainer resource <flexContainer> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a flexContainer Update Request to update the any customAttribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <flexContainer> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <flexContainer> resource |
| 3 |  | IOP Check | Check if possible that the < flexContainer > resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.14.4	FlexContainer Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_55 |
| Objective: |  |  | AE deletes a specific container resource via a container Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clauses 10.2.29.4, 9.6.1.2.2 oneM2M TS-0004 [2], clause 7.4.37.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a flexContainer resource <flexContainer> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a flexContainer Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <flexContainer> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <flexContainer> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.14.5	Notification Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_56 |
| Objective: |  |  | AE receives a notification request on flexContainer update from the HOST CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clauses 10.2.1.5, 9.6.1.2.2 oneM2M TS-0004 [2], clause 7.4.1 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on Registrar CSE AE1 has created a flexContainer resource <flexContainer> on Registrar CSE AE1 has created a <subscription> as a child resource of a <flexContainer> AE2 has created an application resource <AE> on Registrar CSE AE2 has permissions to UPDATE customAttributes of flexContainer. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a update request to <flexContainer> for updating customAttribute. This triggers or causes the HOST CSE to send a notification to AE1. |
| 2 | Mca | PRO Check Primitive | op = 5 (Notify) to = notificationURI of subscription resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if the notification representation |
| 4 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE1 indicates notification received |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.14.6	Discovery with attribute filter criteria over customAttributes

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_57 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using attribute filter criteria which has a customAttribute name and value assigned to it. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clauses 10.2.6, 9.6.1.2.2 oneM2M TS-0004 [2], clause 7.3.3.14 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a flexContainer resource <flexContainer> on Registrar CSE with customAttribute set to a specific value "x", created on Registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Discovery request in order to discover the <Container> resource using attribute filter criteria |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} fr = AE-ID rqi = (token-string) fu=1 atr=<nm>,<val> pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of the <flexContainer> address |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.15	External Management Operations Management

#### 8.1.15.1	mgmtCmd Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_63 |
| Objective: |  |  | AE creates a mgmtCmd resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.2 oneM2M TS-0004 [2], clause 7.4.16.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Create Request |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 12 (mgmtCmd) pc = Serialized representation of <mgmtCmd> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtCmd> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <mgmtCmd> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  | Set verdict to pass if IOP check goal is achieved exactly, otherwise verdict fail is set with corresponding error message. |  |
| PRO Verdict |  |  |  |

#### 8.1.15.2	mgmtCmd Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_64 |
| Objective: |  |  | AE retrieves mgmtCmd resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.3 oneM2M TS-0004 [2], clause 7.4.16.2.2 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (RETRIEVE) to = {CSEBaseName}/{mgmtCmd} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <mgmtCmd> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.15.3	mgmtCmd Update (Normal)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_65 |
| Objective: |  |  | AE updates attribute (not with 'true' in execEnable attribute) in mgmtCmd resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.4 oneM2M TS-0004 [2], clause 7.4.16.2.3.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Update Request |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{mgmtCmd} fr = AE-ID rqi = (token-string) pc = Serialized representation of <mgmtCmd> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtCmd> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <mgmtCmd> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.15.4	mgmtCmd Update (Execute)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_66 |
| Objective: |  |  | AE updates attribute (with 'true' in execEnable attribute) in mgmtCmd resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.6 oneM2M TS-0004 [2], clause 7.4.16.2.3.2 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Update Request |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{mgmtCmd} fr = AE-ID rqi = (token-string) pc = Serialized representation of <mgmtCmd> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtCmd> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <mgmtCmd> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.15.5	mgmtCmd Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_67 |
| Objective: |  |  | AE deletes mgmtCmd resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.5 oneM2M TS-0004 [2], clause 7.4.16.2.4 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a mgmtCmd Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (DELETE) to = {CSEBaseName}/{mgmtCmd} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check if possible that the <mgmtCmd> resource is deleted in Registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.15.6	execInstance Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_68 |
| Objective: |  |  | AE retrieves execInstance resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.8 oneM2M TS-0004 [2], clause 7.4.17.2.2 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE AE has executed the mgmtCmd resource <mgmtCmd> on Registrar CSE (update execEnable attribute with 'true') |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a execInstance Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (RETRIEVE) to = {CSEBaseName}/{mgmtCmd}/{execInstance} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <execInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.15.7	execInstance Update (Cancel)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_69 |
| Objective: |  |  | AE updates attribute 'execDisable' to true in execInstance resource to cancel pending management command. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.7 oneM2M TS-0004 [2], clause 7.4.17.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE AE has executed the mgmtCmd resource <mgmtCmd> on Registrar CSE (update execEnable attribute with 'true') |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a execInstance Update Request |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{mgmtCmd}/{execInstance} fr = AE-ID rqi = (token-string) pc = Serialized representation of <execInstance> resource |
| 3 |  | IOP Check | Check if possible that the <execInstance> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <execInstance> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.15.8	execInstance Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_70 |
| Objective: |  |  | AE deletes execInstance resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.9.9 oneM2M TS-0004 [2], clause 7.4.17.2.3 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a node resource <node> on Registrar CSE AE has created a mgmtCmd resource <mgmtCmd> on Registrar CSE AE has executed the mgmtCmd resource <mgmtCmd> on Registrar CSE (update execEnable attribute with 'true') |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a execInstance Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (DELETE) to = {CSEBaseName}/{mgmtCmd}/{execInstance} fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 4 |  | IOP Check | Check if possible that the <execInstance> resource is deleted in Registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.16	SemanticDescriptor Management

#### 8.1.16.1	SemanticDescriptor Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_75 |
| Objective: |  |  | AE creates a SemanticDescriptor resource in Registrar CSE via a SemanticDescriptor Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.1.2 oneM2M TS-0004 [2], clause 7.4.34.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created a container resource <container> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <semanticDescriptor> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of < container > resource fr = AE-ID rqi = (token-string) ty = 24 (semanticDescriptor) pc = Serialized representation of <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.16.2	SemanticDescriptor Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_76 |
| Objective: |  |  | AE retrieves information of a semanticDescriptor resource via a semanticDescriptor Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.1.3 oneM2M TS-0004 [2], clause 7.4.34.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticDescriptor> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <semanticDescriptor> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticDescriptor> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.16.3	SemanticDescriptor Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_77 |
| Objective: |  |  | AE updates attribute in <semanticDescriptor> resource via a semanticDescriptor Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.1.4 oneM2M TS-0004 [2], clause 7.4.34.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Update Request to update the descriptor attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <semanticDescriptor> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.16.4	SemanticDescriptor Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_78 |
| Objective: |  |  | AE deletes SemanticDescriptor resource via a SemanticDescriptor Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.1.5 oneM2M TS-0004 [2], clause 7.4.34.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticDescriptor resource <semanticDescriptor> as child of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <semanticDescriptor> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticDescriptor> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.17	Semantic Resource Discovery

#### 8.1.17.1	Discovery with semanticFilter filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_79 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using the semanticFilter filter criteria |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.4 oneM2M TS-0004 [2], clause 7.3.3.18 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on Registrar CSE AE1 has created a container resource <container> on Registrar CSE AE1 has created a <semanticDescriptor> as a child resource of a <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request to discover the <container> resource using the semanticFilter filterCriteria |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} from = AE-ID rqi = (token-string) fu=1 smf=sparqlQuery1 pc = empty |
| 3 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the <Container> address |
| 4 |  | IOP Check | AE1 indicates notification received |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.18	ResultContent

#### 8.1.18.1	ResultContent=0

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_84 |
| Objective: |  |  | Check creation of <container> resource with result content set to 0(nothing) |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 8.1.2 oneM2M TS-0004 [2], clause 6.3.4.2.7 |
| Pre-test conditions: |  |  | CSEBase resource has been created in CSE with name {CSEBaseName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a <container> resource Create Request to the Registrar CSE with rcn=0 |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 3 (container) rcn = 0 pc = Serialized representation of <container> resource which contain {containerName} as a rn |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE successfully received response of Create request |
| IOP Verdict |  | Check that the response body does not include any message |  |
| PRO Verdict |  |  |  |

#### 8.1.18.2	ResultContent=1

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_85 |
| Objective: |  |  | Check creation of <container> resource with result content set to 1(attributes) |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 8.1.2 oneM2M TS-0004 [2], clause 6.3.4.2.7 |
| Pre-test conditions: |  |  | CSEBase resource has been created in CSE with name {CSEBaseName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a <container> resource Create Request to the Registrar CSE with rcn=1 |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 3 (container) rcn = 1 pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource which contain attributes |
| 5 |  | IOP Check | AE successfully received response of Create request |
| IOP Verdict |  | Check that the response body include attributes |  |
| PRO Verdict |  |  |  |

#### 8.1.18.3	ResultContent=2

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_86 |
| Objective: |  |  | Check creation of <container> resource with result content set to 2(hierarchical address) |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 8.1.2 oneM2M TS-0004 [2], clause 6.3.4.2.7 |
| Pre-test conditions: |  |  | CSEBase resource has been created in CSE with name {CSEBaseName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a <container> resource Create Request to the Registrar CSE with rcn=2 |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 3 (container) rcn = 2 pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Hierarchical address of <container> resource |
| 5 |  | IOP Check | AE successfully received response of Create request |
| IOP Verdict |  | Check that the response body include hierarchical address |  |
| PRO Verdict |  |  |  |

#### 8.1.18.4	ResultContent=3

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_87 |
| Objective: |  |  | Check creation of <container> resource with result content set to 3(hierarchical address and attributes) |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 8.1.2 oneM2M TS-0004 [2], clause 6.3.4.2.7 |
| Pre-test conditions: |  |  | CSEBase resource has been created in CSE with name {CSEBaseName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a <container> Create Request to the Registrar CSE with rcn=3 |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 3 (container) rcn = 3 pc = Serialized representation of <container> resource |
| 3 |  | IOP Check | Hosting CSE successfully created the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource with hierarchical address |
| 5 |  | IOP Check | AE successfully received response of Create request |
| IOP Verdict |  | Check that the response body include hierarchical address and attributes |  |
| PRO Verdict |  |  |  |

#### 8.1.18.5	ResultContent=4

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_88 |
| Objective: |  |  | Check retrievability of <container> resource with result content set to 4(attributes and child resources) |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 8.1.2 oneM2M TS-0004 [2], clause 6.3.4.2.7 |
| Pre-test conditions: |  |  | CSEBase resource has been created in CSE with name {CSEBaseName} AE has created a <container> resource on registrar CSE AE has created two <contentInstance> resources under the <container> resources |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a <container> Retrieve Request to the Registrar CSE with rcn=4 |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = ID of <container> resource fr = AE-ID rqi = (token-string) rcn = 4 |
| 3 |  | IOP Check | Hosting CSE successfully received Retrieve request of the <container> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <container> and child resources |
| 5 |  | IOP Check | AE successfully received response of Retrieve request |
| IOP Verdict |  | Check that the response body include attributes and child resources |  |
| PRO Verdict |  |  |  |

### 8.1.19	timeSeries Management

#### 8.1.19.1	timeSeries Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_90 |
| Objective: |  |  | AE creates a <timeSeries> resource in registrar CSE via a Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.30.1 oneM2M TS-0004 [2], clause 7.4.38.2.1 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE> resource fr = AE-ID rqi = (token-string) ty = 29 (timeSeries) pc = Serialized representation of <timeSeries> resource |
| 3 |  | IOP Check | Registrar CSE successfully created the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.2	timeSeries Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_91 |
| Objective: |  |  | AE retrieves information of a <timeSeries> resource via a Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.30.2 oneM2M TS-0004 [2], clause 7.4.38.2.2 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE AE has created a <timeSereis> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <timeSeries> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully received Retrieve Request of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Retrieve Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.3	timeSeries Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_92 |
| Objective: |  |  | AE updates attribute in <timeSeries> resource via a Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.30.3 oneM2M TS-0004 [2], clause 7.4.38.2.3 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE AE has created a <timeSereis> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Update Request to the <timeSeries> resource to update the maxNrOfInstances |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <timeSeries> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of maxNrOfInstances update(can be any other attribute) |
| 3 |  | IOP Check | Registrar CSE successfully updated the maxNrOfInstances of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <timeSeries> resource |
| 5 |  | IOP Check | AE successfully received response of Update Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.19.4	timeSeries Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_93 |
| Objective: |  |  | AE deletes a <timeSeries> resource via a Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.30.4 oneM2M TS-0004 [2], clause 7.4.38.2.4 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE AE has created a <timeSeries> resource on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Delete Request of the <timeSeries> resource |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <timeSeries> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully deleted the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | AE successfully received response of Delete Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.20	timeSeriesInstance Management

#### 8.1.20.1	timeSeriesInstance Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_94 |
| Objective: |  |  | AE sends Create Request of a <timeSeriesInstance> resource to a <timeSeries> resource in Registrar CSE. Registrar CSE creates the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.31.1 oneM2M TS-0004 [2], clause 7.4.39.2.1 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE AE has created a <timeSeries> resource on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <timeSeries> resource fr = AE-ID rqi = (token-string) ty = 30 (timeSeriesInstance) pc = Serialized representation of <timeSeriesInstance> resource |
| 3 |  | IOP Check | Registrar CSE successfully created <timeSeriesInstance> resource and updated currentNrOfInstances and currentByteSize of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.20.2	timeSeriesInstance Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_95 |
| Objective: |  |  | AE retrieves information of a <timeSeriesInstance> resource via a Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.31.2 oneM2M TS-0004 [2], clause 7.4.39.2.2 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE AE has created a <timeSeries> resource on registrar CSE AE has created a <timeSeriesInstance> resource as child resource of the <timeSeries> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <timeSeriesInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully received Retrieve Request of the <timeSeriesInstance> resource |
| 4 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Retrieve Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.20.3	timeSeriesInstance Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_96 |
| Objective: |  |  | AE sends Delete Request of a <timeSeriesInstance> resource in Registrar CSE. Registrar CSE delete the <timeSeriesInstance> resource and updates the parent <timeSeries> resource with currentNrOfInstances and currentByteSize attributes correspondingly |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.31.4 oneM2M TS-0004 [2], clause 7.4.39.2.4 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE AE has created a <timeSeries> resource on registrar CSE AE has created a <timeSeriesInstance> resource as child resource of the <timeSeries> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Delete Requeest a <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <timeSeriesInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Registrar CSE successfully deleted <timeSeriesInstance> resource and updated currentNrOfInstances and currentByteSize of the <timeSeries> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | AE successfully received response of Delete Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.20.4	timeSeriesInstance Create when currentNrOfInstance equals to maxNrOfInstances in parent <timeSeries> resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_97 |
| Objective: |  |  | AE sends a <timeSeriesInstance> resource Create Request to a <timeSeries> resource which contains currentNrOfInstances value equals to that of maxNrOfInstances and Registrar CSE delets the oldest <timeSeriesInstance> resource from the <timeSeries> resource and then creates the requested <timeSeriesInstance> resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.31.1 oneM2M TS-0004 [2], clause 7.4.38.2.1 |
| Pre-test conditions: |  |  | AE has created a <AE> resource on Registrar CSE AE has created a <timeSeries> resource on registrar CSE AE has created several <timeSeriesInstance> resources and currentNrOfInstances of the <timeSeries> resource reach the maxNrOfInstances |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request of the <timeSeriesInstance> resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <timeSeries> resource fr = AE-ID rqi = (token-string) ty = 30 (timeSeriesInstance) pc = Serialized representation of <timeSeriesInstance> resource |
| 3 |  | IOP Check | Registrar CSE successfully deleted <timeSeriesInstance> resource with the oldest dataGenerationTime and created <timeSeriesInstance> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <timeSeriesInstance> resource |
| 5 |  | IOP Check | AE successfully received response of Create Request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.21	Location Management

#### 8.1.21.1	LocationPolicy Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_98 |
| Objective: |  |  | AE creates a <locationPolicy> resource in registrar CSE via a locationPolicy Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.1.1 oneM2M TS-0004 [2], clause 7.4.10.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <locationPolicy> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 10 (LocationPolicy) pc = Serialized representation of <locationPolicy> resource |
| 3 |  | IOP Check | Check if possible that the <locationPolicy> resource is created in registrar CSE. Check if possible that the <container> resource is created on registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <locationPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.21.2	LocationPolicy Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_99 |
| Objective: |  |  | AE retrieves information of a <locationPolicy> resource via a locationPolicy Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.1.2 oneM2M TS-0004 [2], clause 7.4.10.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a locationPolicy resource <locationPolicy> on Registrar CSE AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <locationPolicy> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <locationPolicy> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <locationPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.21.3	LocationPolicy Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_100 |
| Objective: |  |  | AE updates attribute in <locationPolicy> resource via a locationPolicy Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.1.3 oneM2M TS-0004 [2], clause 7.4.10.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a locationPolicy resource <locationPolicy> on Registrar CSE AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a locationPolicy Update Request to update the lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <locationPolicy> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <locationPolicy> resource |
| 3 |  | IOP Check | Check if possible that the < locationPolicy > resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <locationPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.21.4	LocationPolicy Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_101 |
| Objective: |  |  | AE deletes a specific <locationPolicy> resource via a locationPolicy Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.1.4 oneM2M TS-0004 [2], clause 7.4.10.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a locationPolicy resource <locationPolicy> on Registrar CSE AE has created a container resource <container> on Registrar CSE having its resourceID and locationID attribute set to locationContainerID and resourceID attribute of the <locationPolicy> resource respectively. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a locationPolicy Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <locationPolicy> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <locationPolicy> resource is deleted in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <locationPolicy> resource has been removed in registrar CSE. |
| 6 |  | IOP Check | Check if possible that the associated resources (e.g. <container>, <contentInstance> resources) are removed from the registrar CSE. |
| 7 |  | IOP Check | Check if possible that if the locationSource attribute and the locationUpdatePeriod attribute of the <locationPolicy> resource were set with appropriate value, the session with underlying network are torn down. |
| 8 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.1.22	Schedule management

#### 8.1.22.1	Schedule Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_155 |
| Objective: |  |  | AE creates a <schedule> resource in Registrar CSE via a Schedule Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.17.2 oneM2M TS-0004 [2], clause 7.4.9.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <schedule> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 18 (schedule) pc = Serialized representation of <schedule> resource |
| 3 |  | IOP Check | Check if possible that the <schedule> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <schedule> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.22.2	Schedule Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_156 |
| Objective: |  |  | AE retrieves information of a <schedule> resource via a schedule Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.17.3 oneM2M TS-0004 [2], clause 7.4.9.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a schedule resource <schedule> as child resource of <CSEBase> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <schedule> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <schedule> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <schedule> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.22.3	Schedule Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_157 |
| Objective: |  |  | AE updates attribute in <schedule> resource via a schedule Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.17.4 oneM2M TS-0004 [2], clause 7.4.9.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a schedule resource <schedule> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a schedule Update Request to update an attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <schedule> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <schedule> resource |
| 3 |  | IOP Check | Check if possible that the <schedule> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <schedule> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.22.4	Schedule Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_158 |
| Objective: |  |  | AE deletes <schedule> resource via a Schedule Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.17.5 oneM2M TS-0004 [2], clause 7.4.9.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <schedule> resource as child of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a schedule Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <schedule> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <schedule> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <schedule> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.1.22.5	Notification to AE with configured Schedule resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_159 |
| Objective: |  |  | CSE sends a notification request to the AE when <schedule> resource is configured |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [1], clause 9.6.9, 10.2.12 oneM2M TS-0004 [2], clause 7.4.1 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE AE1 has created a container resource <container> on registrar CSE AE1 has created a <subscription> as a child resource of a <container> AE1 has created a <schedule> as a child resource of a <subscription> AE2 has created an application resource <AE> on registrar CSE AE2 has permissions to UPDATE the container created by AE1 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Update request to the <container> created by AE1. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <container> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | The HOST CSE prepares a notification to AE1 and sends it within the time window indicated in scheduleElement attribute of <schedule> resource |
| 4 | Mca | PRO Check Primitive | op = 5 (Notify) to = notificationURI of subscription resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 5 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 6 |  | IOP Check | Check that AE1 has received the notification within the time window indicated in scheduleElement attribute of <schedule> resource |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.2	Non-blocking configuration testing

### 8.2.1	Synchronous request

#### 8.2.1.1	Container management

##### 8.2.1.1.1	Container Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_01 |
| Objective: |  |  | AE creates a <Container> resource using non-blocking synchronous request in registrar CSE. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  |  |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking synchronous request to create a <Container> resource in registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 1 (Create) to = {CSEBaseName} fr= AE-ID rqi = (token-string) rt = 1 (non-blocking synchronous) ty = 3 (container) pc = Serialized Representation of the <Container> resource |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = token-string) same as received in request message pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to wait then send a retrieve request to <Request> reference |
| 6 | Mca | PRO Check Primitive | Sent Retrieve request contains op = 2 (Retrieve) to = <Request> reference fr = AE-ID rqi = (token-string) pc = empty |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = <Request> resource with the parameter "requestStatus" set to 1 (COMPLETED) and the "operationResult" parameter containing the <Container> resource. |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.2.1.1.2	Container Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_02 |
| Objective: |  |  | AE retrieves a <container> resource using non-blocking synchronous request from registrar CSE. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created a <Container> resource in registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking synchronous request to retrieve the <Container> resource from registrar CSE. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName}/URI of <container> resource fr= AE-ID rqi = (token-string) rt = 1 (non-blocking synchronous) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = token-string) same as received in request message pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to send a retrieve request to <Request> reference |
| 6 | Mca | PRO Check Primitive | Sent Retrieve request contains op = 2 (Retrieve) to = <Request> reference fr = AE-ID rqi = (token-string) pc = empty |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = <Request> resource with the parameter "requestStatus" set to 1 (COMPLETED) and the "operationResult" parameter containing the <Container> resource. |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.2.1.1.3	Container Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_03 |
| Objective: |  |  | AE updates a <Container> resource using non-blocking synchronous request in registrar CSE. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created a <Container> resource in registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking synchronous request to update the <Container> resource. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 3 (Update) to = {CSEBaseName}/URI of <Container> resource fr= AE-ID rqi = (token-string) rt = 1 (non-blocking synchronous) pc = Serialized Representation of the updated <Container> resource |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = token-string) same as received in request message pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to wait then send a retrieve request to <Request> reference |
| 6 | Mca | PRO Check Primitive | Sent Retrieve request contains op = 2 (Retrieve) to = <Request> reference fr = AE-ID rqi = (token-string) pc = empty |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = <Request> resource with the parameter "requestStatus" set to 1 (COMPLETED) and the "operationResult" parameter containing the <Container> resource. |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.2.1.1.4	Container Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_04 |
| Objective: |  |  | AE deletes a Container resource using non-blocking synchronous request. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created <Container> resource on registrar CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking synchronous request to delete the <Container> resource. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 4 (Delete) to = {CSEBaseName}/URI of <container> resource fr= AE-ID rqi = (token-string) rt = 1 (non-blocking synchronous) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = token-string) same as received in request message pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to send a retrieve request to <Request> reference |
| 6 | Mca | PRO Check Primitive | Sent Retrieve request contains op = 2 (Retrieve) to = <Request> reference fr = AE-ID rqi = (token-string) pc = empty |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = <Request> resource with the parameter "requestStatus" set to 1 (COMPLETED) |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.2.2	Asynchronous request

#### 8.2.2.1	Container management

##### 8.2.2.1.1	Container Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_05 |
| Objective: |  |  | AE creates a <Container> resource using non-blocking asynchronous request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE is reachable on the URI: "AE-Notification-URI" |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to create the <Container> resource in registrar CSE. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 1 (Create) to = {CSEBaseName} fr= AE-ID rqi = (token-string) rt = 2 (non-blocking asynchronous) ty = 3 (container) nu= AE-Notification-URI oneM2M-RQI: Request-ID pc = Serialized Representation of the <Container> resource |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = token-string) same as received in request message pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains op = 5 (Notify) to = AE-Notification-URI fr = registrar CSE-ID rqi = (token-string) pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.2.2.1.2	Container Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_06 |
| Objective: |  |  | AE retrieves a <container> resource using non-blocking asynchronous request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created a <Container> resource on registrar CSE. AE is reachable on the URI: "AE-Notification-URI" |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to retrieve the <Container> resource from registrar CSE |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName}/URI of <container> resource fr = AE-ID rqi = (token-string) rt = 2 (non-blocking asynchronous) nu = AE-Notification-URI pc = empty |
| Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = (token-string) same as received in request message pc = Reference to the created <Request> resource |  |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains op = 5 (Notify) to = AE-Notification-URI fr = registrar CSE-ID rqi = (token-string) pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.2.2.1.3	Container Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_07 |
| Objective: |  |  | AE updates a <Container> resource using non-blocking asynchronous request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created a Container resource <Container> on registrar CSE AE is reachable on the URI: "AE-Notification-URI" |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to update the <Container> resource in registrar CSE. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 3 (Update) to = {CSEBaseName}/URI of <Container> resource fr = AE-ID rqi = (token-string) rt = 2 (non-blocking asynchronous) nu = AE-Notification-URI pc = Serialized Representation of the updated <Container> resource |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = token-string) same as received in request message pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains op = 5 (Notify) to = AE-Notification-URI fr = registrar CSE-ID rqi = (token-string) pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

##### 8.2.2.1.4	Container Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NB_08 |
| Objective: |  |  | AE deletes a Container resource using non-blocking asynchronous request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.4.1 oneM2M TS-0004 [2], clause 7.3.6.2.1 |
| Pre-test conditions: |  |  | AE has created a <Container> resource on registrar CSE AE is reachable on the URI: "AE-Notification-URI" |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a non-blocking asynchronous request to delete the <Container> resource in registrar CSE. |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 4 (Delete) to = {CSEBaseName}/URI of <container> resource fr = AE-ID rqi = (token-string) rt = 2 (non-blocking asynchronous) nu = AE-Notification-URI pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE creates an internal <Request> resource and sends acknowledgement response containing: rsc = 1000 (Accepted) rqi = token-string) same as received in request message pc = Reference to the created <Request> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | IOP Check | Registrar CSE sends notify request to AE |
| 6 | Mca | PRO Check Primitive | Sent request contains op = 5 (Notify) to = AE-Notification-URI fr = registrar CSE-ID rqi = (token-string) pc = Serialized representation of notification data object |
| 7 | Mca | PRO Check Primitive | AE sends notify response to Registrar CSE containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | Registrar CSE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.3	Single hop configuration testing

### 8.3.1	Retargeting

#### 8.3.1.1	RetargetingResource Create (Generic Test Description)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_01 |
| Objective: |  |  | AE creates a remote <Resource> resource |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  |  |
| Pre-test conditions |  |  | Parents resources need to be created on the hosting CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request to create <Resource> on the Hosting CSE. |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = URI of the parent resource fr = AE-ID rqi = (token-string) ty = <Resource> type number pc = Serialized representation of <Resource> resource |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE. |
| 4 | Mcc | PRO Check Primitive | op = 1 (Create) to = URI of the parent resource fr = AE-ID rqi = (token-string) ty = m2m:resourceType pc = Serialized representation of <Resource> resource |
| 5 |  | IOP Check | Check if possible that the <Resource> resource is created in the Hosting CSE. |
| 6 | Mcc | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <Resource> resource |
| 7 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE. |
| 8 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <Resource> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.1.2	<Resource> Create

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_01#01 | oneM2M TS-0001 [1], clause 10.2.4.1oneM2M TS-0004 [2], clause 7.3.5.2.1 |  |  |
| <contentInstance> | TD_M2M_SH_01#02 | oneM2M TS-0001 [1], clause 10.2.19.2oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <subscription> | TD_M2M_SH_01#03 | oneM2M TS-0001 [1], clause 10.2.11.2oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_01#04 | oneM2M TS-0001 [1], clause 10.2.21.1oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_01#05 | oneM2M TS-0001 [1], clause 10.2.7.2oneM2M TS-0004 [2], clause 7.3.12.2.1 |  |  |
| <pollingChannel> | TD_M2M_SH_01#06 | oneM2M TS-0001 [1], clause 10.2.13.2oneM2M TS-0004 [2], clause 7.3.21.2.1 |  |  |
| <fanOutPoint> | TD_M2M_SH_01#07 | oneM2M TS-0001 [1], clause 10.2.7.6oneM2M TS-0004 [2], clause 7.3.14.3.1 |  |  |
| <node> | TD_M2M_SH_01#08 | oneM2M TS-0001 [1], clause 10.2.14.1oneM2M TS-0004 [2], clause 7.3.18.2.1 |  |  |

#### 8.3.1.3	Resource Retrieve (Generic Test Description)

| Interoperability Test Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_02 |  |
| Objective: |  |  | AE retrieves a remote resource |  |
| Configuration: |  |  | M2M_CFG_03 |  |
| References: |  |  |  |  |
| Test Sequence |  |  |  |  |
| Step | RP | Type | Description |  |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request to retrieve on the remote Hosting CSE. |  |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = URI of the resource U fr = AE-ID rqi = (token-string) |  |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE. |  |
| 4 | Mcc | PRO Check Primitive | op = 2 (Retrieve) to URI of the resource fr = AE-ID rqi = (token-string) |  |
| 5 | Mcc | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of resource |  |
| 6 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE. |  |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of resource |  |
| 8 |  | IOP Check | AE indicates successful operation |  |
| IOP Verdict |  |  |  |  |
| PRO Verdict |  |  |  |  |

#### 8.3.1.4	<Resource> retrieve

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_02#01 | oneM2M TS-0001 [1], clause 10.2.4.2oneM2M TS-0004 [2], clause 7.3.5.2.2 |  |  |
| <contentInstance> | TD_M2M_SH_02#02 | oneM2M TS-0001 [1], clause 10.2.19.3oneM2M TS-0004 [2], clause 7.3.6.2.2 |  |  |
| <subscription> | TD_M2M_SH_02#03 | oneM2M TS-0001 [1], clause 10.2.11.3oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_02#04 | oneM2M TS-0001 [1], clause 10.2.21.2oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_02#05 | oneM2M TS-0001 [1], clause 10.2.7.3oneM2M TS-0004 [2], clause 7.3.12.2.2 |  |  |
| <pollingChannel> | TD_M2M_SH_02#06 | oneM2M TS-0001 [1], clause 10.2.13.3oneM2M TS-0004 [2], clause 7.3.21.2.2 |  |  |
| <fanOutPoint> | TD_M2M_SH_02#07 | oneM2M TS-0001 [1], clause 10.2.7.8oneM2M TS-0004 [2], clause 7.3.14.3.2 |  |  |
| <node> | TD_M2M_SH_02#08 | oneM2M TS-0001 [1], clause 10.2.14.2oneM2M TS-0004 [2], clause 7.3.18.2.2 |  |  |
| <remoteCSE> | TD_M2M_SH_02#09 | oneM2M TS-0001 [1], clause 10.2.2.3oneM2M TS-0004 [2], clause 7.3.3.2.3 |  |  |
| <ae> | TD_M2M_SH_02#10 | oneM2M TS-0001 [1], clause 10.2.1.2oneM2M TS-0004 [2], clause 7.3.5.2.2 |  |  |
| <CSEBase> | TD_M2M_SH_02#11 | oneM2M TS-0001 [1], clause 10.2.3.2oneM2M TS-0004 [2], clause 7.3.2 |  |  |

#### 8.3.1.5	Resource Update (Generic Test Description)

| Interoperability Test Description |  |  |  |  |
| --- | --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_03 |  |
| Objective: |  |  | AE updates a remote resource |  |
| Configuration: |  |  | M2M_CFG_03 |  |
| References: |  |  |  |  |
| Test Sequence |  |  |  |  |
| Step | RP | Type | Description |  |
| 1 |  | Stimulus | AE is requested to send an Update Request to update the on the Hosting CSE |  |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = URI of the resource fr = AE-ID rqi = (token-string) pc = Serialized representation of resource |  |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE |  |
| 4 | Mcc | PRO Check Primitive | op = 3 (Update) to = URI of the resource fr = AE-ID rqi = (token-string) pc = Serialized representation of resource |  |
| 5 |  | IOP Check | Check if possible that the resource is updated in the Hosting CSE. |  |
| 6 | Mcc | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message pc = Serialized representation of resource |  |
| 7 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE. |  |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message pc = Serialized representation of resource |  |
| 9 |  | IOP Check | AE indicates successful operation |  |
| IOP Verdict |  |  |  |  |
| PRO Verdict |  |  |  |  |

#### 8.3.1.6	<Resource> update

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_03#01 | oneM2M TS-0001 [1], clause 10.2.4.3oneM2M TS-0004 [2], clause 7.3.5.2.3 |  |  |
| <subscription> | TD_M2M_SH_03#02 | oneM2M TS-0001 [1], clause 10.2.11.4oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_03#03 | oneM2M TS-0001 [1], clause 10.2.21.3oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_03#04 | oneM2M TS-0001 [1], clause 10.2.7.4oneM2M TS-0004 [2], clause 7.3.12.2.3 |  |  |
| <pollingChannel> | TD_M2M_SH_03#05 | oneM2M TS-0001 [1], clause 10.2.13.4oneM2M TS-0004 [2], clause 7.3.21.2.3 |  |  |
| <fanOutPoint> | TD_M2M_SH_03#06 | oneM2M TS-0001 [1], clause 10.2.7.9oneM2M TS-0004 [2], clause 7.3.14.3.3 |  |  |
| <node> | TD_M2M_SH_03#07 | oneM2M TS-0001 [1], clause 10.2.14.3oneM2M TS-0004 [2], clause 7.3.18.2.3 |  |  |
| <remoteCSE> | TD_M2M_SH_03#08 | oneM2M TS-0001 [1], clause 10.2.2.3oneM2M TS-0004 [2], clause 7.3.3.2.3 |  |  |
| <ae> | TD_M2M_SH_03#09 | oneM2M TS-0001 [1], clause 10.2.1.3oneM2M TS-0004 [2], clause 7.3.5.2.3 |  |  |

#### 8.3.1.7	Resource Delete (Generic Test Description)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_04 |
| Objective: |  |  | AE delete a remote <Resource> resource |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  |  |
| Pre-test conditions: |  |  | Parents resources need to be created on the hosting CSE Resource <Resource> has been created in Hosting CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Delete Request to delete <Resource> on the Hosting CSE. |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = URI of the resource <Resource> fr = AE-ID rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE. |
| 4 | Mcc | PRO Check Primitive | op = 4 (Delete) to = URI of the resource <Resource> fr = AE-ID rqi = (token-string) |
| 5 |  | IOP Check | Check if possible that the <Resource> resource is deleted in the Hosting CSE. |
| 6 | Mcc | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 7 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE. |
| 8 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.1.8	<Resource> delete

| <Resource> | Identifier | Refs | IOP Verdict | PRO Verdict |
| --- | --- | --- | --- | --- |
| <container> | TD_M2M_SH_04#01 | oneM2M TS-0001 [1], clause 10.2.4.4oneM2M TS-0004 [2], clause 7.3.5.2.4 |  |  |
| <contentInstance> | TD_M2M_SH_04#02 | oneM2M TS-0001 [1], clause 10.2.19.5oneM2M TS-0004 [2], clause 7.3.6.2.4 |  |  |
| <subscription> | TD_M2M_SH_04#03 | oneM2M TS-0001 [1], clause 10.2.11.5oneM2M TS-0004 [2], clause 7.3.7.2 |  |  |
| <accessControlPolicy> | TD_M2M_SH_04#04 | oneM2M TS-0001 [1], clause 10.2.21.4oneM2M TS-0004 [2], clause 7.3.1.2 |  |  |
| <group> | TD_M2M_SH_04#05 | oneM2M TS-0001 [1], clause 10.2.7.5oneM2M TS-0004 [2], clause 7.3.12.2.4 |  |  |
| <pollingChannel> | TD_M2M_SH_04#06 | oneM2M TS-0001 [1], clause 10.2.13.5oneM2M TS-0004 [2], clause 7.3.21.2.4 |  |  |
| <fanOutPoint> | TD_M2M_SH_04#07 | oneM2M TS-0001 [1], clause 10.2.7.10oneM2M TS-0004 [2], clause 7.3.14.3.4 |  |  |
| <node> | TD_M2M_SH_04#08 | oneM2M TS-0001 [1], clause 10.2.14.4oneM2M TS-0004 [2], clause 7.3.18.2.4 |  |  |

#### 8.3.1.9	Discovery with multiple filter criteria

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_09 |
| Objective: |  |  | AE discovers accessible resources residing in the remote Hosting CSE using multiple Filter Criteria |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.6 oneM2M TS-0004 [2], clause 7.2.3.13 |
| Pre-test conditions: |  |  | Two <Container> resources with labels "key1" and "key2" are created in Hosting CSE. A <Group> resources with labels "key1" and "key2" is created in Hosting CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a discovery request to discover specific resources located in hosting CSE using multiple filter criteria (label, resource type and limit) |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = URI of hosting CSEBase fr = AE-ID rqi = (token-string) fu=1 lbl=key1 lbl=key2 rty=3 lim=1 pc = empty |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE. |
| 4 | Mcc | PRO Check Primitive | Forwarded request contains op = 2 (Retrieve) to = hosting CSEBase fr = AE-ID rqi = (token-string) fu=1 lbl=key1 lbl=key2 rty=3 lim=1 pc = empty |
| 5 |  | IOP Check | Check if possible that the response is sent by the hosting CSE to the registrar CSE. |
| 6 | Mcc | PRO Check Primitive | Hosting CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of one of the <Container> resources |
| 7 |  | IOP Check | Check if possible that the response is forwarded from the registrar CSE to AE |
| 6 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the address of one of the <Container> resources |
| 7 |  | IOP Check | AE indicates successful operation |

#### 8.3.1.10	Unauthorized operation (Insufficient Access Rights)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_10 |
| Objective: |  |  | AE delete request is rejected after access rights verification using retargeting. |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  | oneM2M TS-0004 [2], clause 7.3.1.2 |
| Pre-test conditions: |  |  | An <accessControlPolicy> resource with name {ACPName} has been created in remote hosting CSE, not allowing delete operation. AE has created an <AE> resource on registrar CSE with name {AEName} AE has created a <container> sub-resource in the <AE> resource with name {containerName} and having as accessControlPolicy-ID the ID of the remote <accessControlPolicy>. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Request to delete the <container> resource from the registrar CSE. |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = URI of addressed resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that a request is sent by the registrar CSE to the Hosting CSE to retrieve the corresponding remote <accessControlPolicy> resource. |
| 4 | Mcc | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = URI of addressed resource fr = Registrar CSE-ID rqi = (token-string) pc = empty |
| 5 |  | IOP Check | Check if possible that the response is sent by the hosting CSE to the registrar CSE. |
| 6 | Mcc | PRO Check Primitive | Hosting CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 7 |  | IOP Check | Check if possible that an access denied error response is sent by registrar CSE to AE |
| 8 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 9 |  | IOP Check | Check if possible that the <container> resource has not been deleted. |
| 10 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |

#### 8.3.1.11	Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_11 |
| Objective: |  |  | AE receives a notification request from the remote hosting CSE |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.12 oneM2M TS-0004 [2], clause 7.4.1 |
| Pre-test conditions: |  |  | A <container> resource has been created on hosting CSE AE has created an <AE> resource on registrar CSE AE has created a <subscription> resource for the <container> resource on the remote hosting CSE. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | A <contentInstance> sub-resource is created on the <container> resource. This triggers or causes the hosting CSE to send a notification to AE. |
| 2 | Mca | PRO Check Primitive | op = 5 (Notify) to = URI of AE resource from = Hosting CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if possible that the Notify request is forwarded by the registrar CSE to the AE-ID. |
| 4 | Mcc | PRO Check Primitive | op = 5 (Notify) to = AE from = Hosting CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 5 |  | IOP Check | Check if possible that the response is sent by the AE to the registrar CSE. |
| 6 | Mcc | PRO Check Primitive | AE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = empty |
| 7 |  | IOP Check | Check if possible that the response is forwarded by registrar CSE to Hosting CSE |
| 8 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = empty |
| 9 |  | IOP Check | Check if possible that the <container> resource has not been deleted. |
| 10 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege). |

### 8.3.2	<mgmtObj> Test Description

#### 8.3.2.1	<mgmtObj> Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_05 |
| Objective: |  |  | AE creates a <mgmtObj> resource |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.8.2 |
| Pre-test conditions: |  |  | Management Session between Management Server and Management Client |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Create Request to create an <mgmtObj> on IN-CSE. |
| 2 | Mca | PRO Check Primitive | op: 1 (CREATE) fr: AE-ID to: {CSEBaseName}/{node} rqi = (token-string) ty = 13 (mgmtObj) pc: Serialized representation of the <mgmtObj> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is created in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to create the corresponding MO using Add DM command. The mapping of <mgmtObj> and MO can be referenced from clause 5.3 of oneM2M TS-0005 [10]. |
|  |  | PRO Check BBF TR069 | Requests to create the corresponding information model using AddObject RPC. The mapping of <mgmtObj> and information model or RPC can be referenced from clause 7 of oneM2M TS-0006 [11]. |
|  |  | PRO Check OMA LWM2M | Requests to create the corresponding Objects using Create LWM2M Create operations. The mapping of <mgmtObj> and Object can be referenced from clause 6.3 of oneM2M TS-0005 [10]. |
| 5 |  | IOP Check | Check if possible that the corresponding MO for OMA DM, information model for BBF TR069 or Object for OMA LWM2M is created on the Managed Entity. |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK. Details can be found in clause 5.4 oneM2M TS-0005 [10]. |
|  |  | PRO Check BBF TR069 | Successful response of the RPC. Details can be found in clause 8.1 oneM2M TS-0006 [11]. |
|  |  | PRO Check OMA LWM2M | Response with status code 2.01 Created. Details can be found in clause 6.4 oneM2M TS-0005 [10]. |
| 7 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <mgmtObj> resource |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.2.2	<mgmtObj> Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_06 |
| Objective: |  |  | AE updates a <mgmtObj> resource |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.8.4 |
| Pre-test conditions: |  |  | Management Session between Management Server and Management Client |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Update Request to update an <mgmtObj> on IN-CSE. |
| 2 | Mca | PRO Check Primitive | op: 3 (UPDATE) fr: AE-ID to: {CSEBaseName}/{node}/{mgmtObj} rqi = (token-string) pc: Serialized representation of the <mgmtObj> resource |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is updated in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to update the corresponding MO using Replace DM command. The mapping of <mgmtObj> and MO can be referenced from clause 5.3 of oneM2M TS-0005 [10]. |
|  |  | PRO Check BBF TR069 | Requests to Update the corresponding information model using SetParameterValues RPC. The mapping of <mgmtObj> and information model or RPC can be referenced from clause 7 of oneM2M TS-0006 [11]. |
|  |  | PRO Check OMA LWM2M | Requests to Update the corresponding Objects using LWM2M Write operations. The mapping of <mgmtObj> and Object can be referenced from clause 6.3 of oneM2M TS-0005 [10]. |
| 5 |  | IOP Check | Check if possible that the corresponding MO for OMA DM, information model for BBF TR069 or Object for OMA LWM2M is Updated on the Managed Entity. |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK. Details can be found in clause 5.4 oneM2M TS-0005 [10]. |
|  |  | PRO Check BBF TR069 | Successful response of the RPC. Details can be found in clause 8.1 oneM2M TS-0006 [11]. |
|  |  | PRO Check OMA LWM2M | Response with status code 2.04 Changed. Details can be found in clause 6.4 oneM2M TS-0005 [10]. |
| 7 | Mca | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message pc = Serialized representation of <mgmtObj> resource |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.2.3	<mgmtObj> Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_07 |
| Objective: |  |  | AE retrieves a <mgmtObj> resource |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.8.3 |
| Pre-test conditions: |  |  | Management Session between Management Server and Management Client |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Retrieve Request to retrieve an <mgmtObj> on IN-CSE. |
| 2 | Mca | PRO Check Primitive | op = 2 (RETRIEVE) to = {CSEBaseName}/{node}/{mgmtObj} fr = AE-ID rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is retrieved in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to retrieve the corresponding MO using Get DM command. |
|  |  | PRO Check BBF TR069 | Requests to retrieve the corresponding information model using GetParametersValue RPC. |
|  |  | PRO Check OMA LWM2M | Requests to retrieve the corresponding Objects using Retrieve LWM2M Read operation. |
| 5 |  | IOP Check |  |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK with the information of the MO. Details can be found in clause 5.4 oneM2M TS-0005 [10]. |
|  |  | PRO Check BBF TR069 | Successful response of the RPC with the information about the management related information. Details can be found in clause 8.1 oneM2M TS-0006 [11]. |
|  |  | PRO Check OMA LWM2M | Response with status code 2.05 Content with the information of the Object. Details can be found in clause 6.4 oneM2M TS-0005 [10]. |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <mgmtObj> resource |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.2.4	<mgmtObj> Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_08 |
| Objective: |  |  | AE deletes a <mgmtObj> resource |
| Configuration: |  |  | M2M_CFG_03 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.8.5 |
| Pre-test conditions: |  |  | Management Session between Management Server and Management Client |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an <mgmtObj> Delete Request to delete an <mgmtObj> on IN-CSE. |
| 2 | Mca | PRO Check Primitive | op = 4 (DELETE) to = {CSEBaseName}/{node}/{mgmtObj} fr = AE-ID rqi = (token string) |
| 3 |  | IOP Check | Check if possible that the <mgmtObj> resource is deleted in IN-CSE |
| 4 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Requests to delete the corresponding MO using Delete DM command. |
|  |  | PRO Check BBF TR069 | Requests to delete the corresponding information model using DeleteObject RPC. |
|  |  | PRO Check OMA LWM2M | Requests to delete the corresponding Objects using LWM2M Delete operation. |
| 5 |  | IOP Check | Check if possible that the corresponding MO for OMA DM, information model for BBF TR069 or Object for OMA LWM2M is deleted on the Managed Entity. |
| 6 | mc | PRO Check Primitive | N/A |
|  |  | PRO Check OMA DM | Response with status code (200) OK. Details can be found in clause 5.4 oneM2M TS-0005 [10]. |
|  |  | PRO Check BBF TR069 | Successful response of the RPC. Details can be found in clause 8.1 oneM2M TS-0006 [11]. |
|  |  | PRO Check OMA LWM2M | Response with status code 2.02 Deleted. Details can be found in clause 6.4 oneM2M TS-0005 [10] |
| 7 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.3.3	Announcement Management

#### 8.3.3.1	AEAnnc Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_12 |
| Objective: |  |  | AE1 announces itself to CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  |  |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} CSE1 is registered to CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send an AE Update Request with announceTo attribute set to CSE2 CSE-ID |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{AE} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <AE> resource |
| 3 |  | IOP Check | Check if possible that the CREATE <AEannc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName2}/{CSEBaseName1} fr = CSE1-ID rqi = (token-string) ty = 10002 (AEAnnc) pc = Serialized representation of <AEAnnc> resource |
| 5 |  | IOP Check | Check if possible that the <AEAnnc> resource is created in CSE2 with only MA attributes. |
| 6 | Mcc | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <AEAnnc> resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <AE> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.3.2	ContainerAnnc Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_13 |
| Objective: |  |  | AE1 announces a child container to CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  |  |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} AE2 has created a <AE> resource on registrar CSE with name {AE2} CSE1 is registered to CSE2 <container> resource is created as a child of AE1 AE1 is announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with announceTo attribute set to CSE2 CSE-ID |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{container} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the CREATE <ContainerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName2}/{AE1Annc} fr = CSE1-ID rqi = (token-string) ty = 10003 (containerAnnc) pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is created in CSE2 with only MA attributes. |
| 6 | Mcc | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.3.3	ContainerAnnc Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_14 |
| Objective: |  |  | AE1 announces an Optional Announce attribute to CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  |  |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} AE2 has created a <AE> resource on registrar CSE with name {AE2} CSE1 is registered to CSE2 <container> resource is created as a child of AE1 AE1 is announced on CSE2 <container> is announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with announcedAttribute = maxNrOfInstances |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{container} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName2}/{ ContainerAnnc } fr = CSE1-ID rqi = (token-string) pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is update in CSE2 with maxNrOfInstances attributes. |
| 6 | Mcc | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca |  | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE1 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.3.4	ContainerAnnc Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_15 |
| Objective: |  |  | AE2 retrieves an Announced Resource |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  |  |
| Pre-test conditions: |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} AE2 has created a <AE> resource on registrar CSE with name {AE2} CSE1 is registered to CSE2 <container> resource is created as a child of AE1 AE1 is announced on CSE2 <container> is announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Retrieve Request for a < containerAnnc > |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName2}/URI of < containerAnnc > resource fr = AE2-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <containerAnnc> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  | Verify that this is a containAnnc resource |  |
| PRO Verdict |  |  |  |

#### 8.3.3.5	ContainerAnnc Retrieve Original

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_16 |
| Objective: |  |  | AE2 retrieves the original resource representation of an announced resource |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  |  |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} AE2 has created a <AE> resource on registrar CSE with name {AE2} CSE1 is registered to CSE2 <container> resource is created as a child of AE1 AE1 is announced on CSE2 <container> is announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a Retrieve Request to a < containerAnnc > with rcn = 7 |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName2}/URI of < containerAnnc > resource fr = AE2-ID rqi = (token-string) rcn = 7 (original) pc = empty |
| 3 |  | IOP Check | Check if possible that the GET <container> is sent from CSE2 to CSE1. |
| 4 | Mcc | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName1}/{ Container} fr = AE2-ID rqi = (token-string) pc = empty |
| 5 |  | IOP Check |  |
| 6 | Mcc | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 7 |  | IOP Check | Check if possible that the response is forwarded by the registrar CSE to the AE. |
| 8 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.3.6	ContainerAnnc Delete by updating announceTo attribute

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_21 |
| Objective: |  |  | AE1 deletes its announced child container from CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  | 10.2.13.4 |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} CSE1 is registered to CSE2 AE1 is announced on CSE2 <container> resource is created as a child of AE1 and announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a an container Update Request with announceTo attribute set to NULL |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName1}/{AE1}/{container} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the DELETE <containerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} fr = CSE1-ID rqi = (token-string) |
| 5 |  | IOP Check | Check if possible that the <containerAnnc> resource is deleted in CSE2 |
| 6 | Mcc | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

8.3.3.7 ContainerAnnc Delete by deleting original resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_22 |
| Objective: |  |  | AE1 deletes its announced child container from CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  | 10.2.13.4 |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} CSE1 is registered to CSE2 AE1 is announced on CSE2 <container> resource is created as a child of AE1 and announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a an container Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName1}/{AE1}/{container} fr = AE-ID rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the DELETE <containerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} fr = CSE1-ID rqi = (token-string) |
| 5 |  | IOP Check | Check if possible that the <containerAnnc> resource is deleted in CSE2 |
| 6 | Mcc | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 7 |  | IOP Check | CSE1 sends a DELETE response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

8.3.3.8 Announced attribute Create by addition to announcedAttribute attribute

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_23 |
| Objective: |  |  | AE1 announces an announcable attribute of its child container to CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  | 10.2.13.7 |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} CSE1 is registered to CSE2 AE1 is announced on CSE2 <container> resource is created as a child of AE1 and announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with announcedAttribute attribute containing currentNrOfInstances |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName1}/{AE1}/{container} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} fr = CSE1-ID rqi = (token-string) pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with currentNrOfInstances attribute. |
| 6 | Mcc | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

8.3.3.9 Announced attribute Create by creation of a MA attribute at the original resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_24 |
| Objective: |  |  | AE1 announces an MA attribute of its child container to CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  |  |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} CSE1 is registered to CSE2 AE1 is announced on CSE2 <container> resource is created as a child of AE1 and announced on CSE2 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a an <container> Update Request with labels attribute |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName1}/{AE1}/{container} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} fr = CSE1-ID rqi = (token-string) pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with labels attribute. |
| 6 | Mcc | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

8.3.3.10 Announced attribute Delete by deletion from announcedAttribute attribute

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_25 |
| Objective: |  |  | AE1 de-announces an announcable attribute of its child container to CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  | 10.2.13.8 |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} CSE1 is registered to CSE2 AE1 is announced on CSE2 <container> resource is created as a child of AE1 and announced on CSE2 with currentNrOfInstances attribute announced |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a <container> Update Request with announcedAttribute attribute set to NULL |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName1}/{AE1}/{container} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} fr = CSE1-ID rqi = (token-string) pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with no currentNrOfInstances attribute. |
| 6 | Mcc | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

8.3.3.11 Announced attribute Delete by deletion of a MA attribute at the original resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_26 |
| Objective: |  |  | AE1 de-announces an MA attribute (conditionally mandatory) of its child container to CSE2 |
| Configuration: |  |  | M2M_CFG_04 |
| References: |  |  |  |
| Pre-test conditions |  |  | <CSEBase> resource has been created in CSE1 with name {CSEBaseName1} AE1 has created a <AE> resource on registrar CSE with name {AE1} <CSEBase> resource has been created in CSE2 with name {CSEBaseName2} CSE1 is registered to CSE2 AE1 is announced on CSE2 <container> resource is created as a child of AE1 and announced on CSE2 with labels attribute announced |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a <container> Update Request with labels attribute set to NULL |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName1}/{AE1}/{container} fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <container> resource |
| 3 |  | IOP Check | Check if possible that the UPDATE <ContainerAnnc> is sent from CSE1 to CSE2. |
| 4 | Mcc | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName2}/{AE1Annc}/{containerAnnc} fr = CSE1-ID rqi = (token-string) pc = Serialized representation of < containerAnnc > resource |
| 5 |  | IOP Check | Check if possible that the < containerAnnc > resource is updated in CSE2 with no labels attribute. |
| 6 | Mcc | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of < containerAnnc > resource |
| 7 |  | IOP Check | CSE1 sends a UPDATED response to the AE1. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.3.4	Single Hop <fanOutPoint> operations

#### 8.3.4.1	Create <fanOutPoint>

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_17 |
| Objective: |  |  | AE creates a <contentInstance> resource in each group member, where some memberIDs are on a remoteCSE |
| Configuration: |  |  | M2M_CFG_08 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.7.7 oneM2M TS-0004 [2], clause 7.4.15.2, 7.4.15.3 |
| Pre-test conditions |  |  | Two or more resources of type <container> exist on the member hosting CSE A group exists containing these two members of type <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Create Request to create <contentInstance> in each group member |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) ty = 4 (contentInstance) pc = Serialized representation of <contentInstance> resource |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar/Group Hosting CSE to the Member Hosting CSE. |
| 4 | Mcc | PRO Check Primitive | op = 1 (Create) to = {MemberCSEBaseName}/{subgroupId}/fopt or {MemberCSEBaseName}/{memberId} fr = AE-ID rqi = (token-string) gid = (grpId-token-string) ty = 4 (contentInstance) pc = Serialized representation of <contentInstance> resource |
| 5 |  | IOP Check | Check if possible that the <contentInstance> resource is created in the Member Hosting CSE. |
| 6 | Mcc | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message gid = (grpId-token-string) same as received in request message pc = Serialized representation of <contentInstance> resource or <aggregated response> |
| 7 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE. |
| 8 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <aggregated response> |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.4.2	Retrieve <fanOutPoint>

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_18 |
| Objective: |  |  | AE retrieves a resource from each group member, where some memberIDs are on a remoteCSE |
| Configuration: |  |  | M2M_CFG_08 |
| References: |  |  |  |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request to the fanoutPoint of resource |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar/group hosting CSE to the Member Hosting CSE. |
| 4 | Mcc | PRO Check Primitive | op = 2 (Retrieve) to = {MemberCSEBaseName}/{subgroupId}/fopt or {MemberCSEBaseName}/{memberId} fr = AE-ID rqi = (token-string) gid = (grpId-token-string) |
| 5 | Mcc | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message gid = (grpId-token-string) same as received in request message pc = Serialized representation of resource |
| 6 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE. |
| 7 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of |
| 8 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.4.3	Update <fanOutPoint>

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_19 |
| Objective: |  |  | AE updates a resource in each group member, where some memberIDs are on a remoteCSE |
| Configuration: |  |  | M2M_CFG_08 |
| References: |  |  |  |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Update Request to the fanoutPoint of resource to lifetime of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) pc = Serialized representation of resource |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar/group hosting CSE to the Member Hosting CSE. |
| 4 | Mcc | PRO Check Primitive | op = 3 (Update) to = {MemberCSEBaseName}/{subgroupId}/fopt or {MemberCSEBaseName}/{memberId} fr = AE-ID rqi = (token-string) pc = Serialized representation of resource |
| 5 |  | IOP Check | Check if possible that the resource is updated in the Hosting CSE. |
| 6 | Mcc | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message gid = (grpId-token-string) same as received in request message pc = Serialized representation of resource or |
| 7 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE. |
| 8 | Mca | PRO Check Primitive | rsc = 2004 (CHANGED) rqi = (token-string) same as received in request message pc = Serialized representation of |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.3.4.4	Delete <fanOutPoint>

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_20 |
| Objective: |  |  | AE deletes a <contentInstance> resource from each group member, where some memberIDs are on a remoteCSE |
| Configuration: |  |  | M2M_CFG_08 |
| References: |  |  |  |
| Pre-test conditions: |  |  | Two or more resources of type <container> exist on the member hosting CSE Each <container> has at least 1 <contentInstance> A group exists containing these two members of type <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Delete 'oldest' Request to the fanoutPoint of <group> resource |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{group}/fopt/ol fr = AE-ID rqi = (token-string) |
| 3 |  | IOP Check | Check if possible that the request is forwarded by the registrar CSE to the Hosting CSE. |
| 4 | Mcc | PRO Check Primitive | op = 4 (Delete) to = {MemberCSEBaseName}/{subgroupId}/fopt/ol or {MemberCSEBaseName}/{memberId}/ol fr = AE-ID rqi = (token-string) gid = (grpId-token-string) |
| 5 |  | IOP Check | Check if possible that the <Resource> resource is deleted in the Hosting CSE. |
| 6 | Mcc | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message gid = (grpId-token-string) same as received in request message |
| 7 |  | IOP Check | Check that the response is aggregated by the group hosting CSE and sent to the AE. |
| 8 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = Serialized representation of <aggregated_response> |
| 9 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.4	Security management

### 8.4.1	Secure AE Registration

#### 8.4.1.1	PSK Security Association Establishment Framework

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_01 |
| Objective: |  |  | AE uses Provisioned Symmetric Key Security Association Establishment Framework to enable mutual authentication with the Registrar CSE. Registrar CSE performs AE authorization check on incoming AE registration request. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.2.2.1 oneM2M TS-0001 [1], clauses 9.6.29, 9.6.19, 9.16.20 |
| Pre-test conditions: |  |  | AE and Registrar CSE are pre-Provisioned with Kpsa = 123456,KpsaId = test@onem2m.com and Cipher Suites = TLS_PSK_WITH_AES_128_CBC_SHA256, TLS_PSK_WITH_AES_128_CCM_8 Registrar CSE is provisioned with Service Subscribed Profile and Service Subscribed Node Resources. Service Subscribed Node contains csi <Registrar CSE-ID> and rlk < URI of serviceSubscribedAppRule > attributes. Registrar CSE is configured with <serviceSubscribedAppRule> resource having a CredentialD, APP-ID and AE-ID with the following values: <m2m:asar rn="asar"> <aci>00-test@onem2m.com</aci> <aai>APP01</aai> <aae>AE-ID</aae> </m2m:asar> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a primitive to the Registrar CSE |
| 2 | Mca | PRO Check Primitive | Security Association Establishment |
|  |  | PRO Check TCP | TLS Handshake Cipher Suite:TLS_PSK_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 KpsaId = test@onem2m.com |
|  |  | PRO Check UDP | DTLS Handshake Cipher Suite:TLS_PSK_WITH_AES_128_CCM_8 Version: DTLS v1.2 KpsaId = test@onem2m.com |
| 3 |  | IOP Check | Check if possible that Handshake was successful |
| 4 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 2 (AE) pc = Serialized representation of <AE> resource |
| 5 |  | IOP Check | Check that APP-ID, AE-ID, Credential ID are in <serviceSubscribedAppRule> Check if possible that the <AE> resource is created in registrar CSE. |
| 6 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <AE> resource |
| 7 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.4.2	Authentication

#### 8.4.2.1	Authentication using the Provisioned Symmetric Key Security Association Establishment Framework with TLS

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 02 |
| Objective: |  |  | AE establishes mutual authentication with the Registrar CSE using Provisioned Symmetric Key Security Association Establishment Framework. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.2.2.1 |
| Pre-test conditions: |  |  | AE and Registrar CSE are pre-Provisioned with Kpsa = 123456,KpsaId = test@onem2m.com and Cipher Suites = TLS_PSK_WITH_AES_128_CBC_SHA256 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The TLS client on AE sends a Client Hello Handshake message |
| 2 | Mca | PRO Check TCP | Client Hello handshake message Handshake Type = 0x01 (Client Hello) Cipher Suite:TLS_PSK_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 |
| 3 | Mca | PRO Check TCP | Server Hello handshake message Handshake Type = 0x02 (Server Hello) Cipher Suite:TLS_PSK_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 Server Hello Done handshake message Handshake Type = 0x0e (Server Hello Done) |
| 4 |  | Stimulus | The TLS client on AE sends Client Key Exchange, Change Cipher Spec, Finished messages |
| 5 | Mca | PRO Check TCP | The TLS client Key Exchange handshake message Handshake Type = 0x10 (Client Key Exchange) psk_identity = test@onem2m.com Version: TLS v1.2 Client Change Cipher Spec message Content type = 0x14 (Change Cipher Spec) Client Finished handshake message Handshake Type = 0x14 (Client Finished) Version: TLS v1.2 |
| 6 |  | IOP Check | Check that The TLS server authenticated the Client by validating Verify Data Check that AE associated the established TLS session with the CSE-ID |
| 7 | Mca | PRO Check TCP | Server New Session Ticket handshake message Handshake Type = 0x04 (New Session Ticket) psk_identity = test@onem2m.com Version: TLS v1.2 Server Change Cipher Spec message Content type = 0x14 (Change Cipher Spec) Server Finished handshake message Handshake Type = 0x14 (Client Finished) Version: TLS v1.2 |
| 8 |  | IOP Check | Check that The TLS client authenticated the Server by validating Verify Data |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.2.2	Authentication using the Certificate-Based Security Association Establishment Framework with TLS

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 03 |
| Objective: |  |  | AE establishes mutual authentication with the Registrar CSE using Certificate-Based Security Association Establishment Framework. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.2.2.2 |
| Pre-test conditions: |  |  | The Registrar CSE uses the CSE-ID certificate signed by a root CA certificate AE uses the AE-ID certificate signed by a root CA certificate Cipher Suite = TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The TLS client on AE sends a client Hello Handshake message |
| 2 | Mca | PRO Check TCP | The TLS client sends a Hello handshake message to the TLS server Handshake Type = 0x01 (Client Hello) Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 |
| 3 | Mca | PRO Check TCP | The TLS server sends Hello, Certificate, Key Exchange, Certificate Request, Hello Done messages to the TLS client Server Hello handshake message Handshake Type = 0x02 (Server Hello) Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 Server Certificate handshake message Handshake Type = 0x0b (Server Certificate) Certificate: the Registrar CSE certificate Server Key Exchange handshake message Handshake Type = 0x0c (Server Key Exchange) Public key: ECDHE generated key Server Certificate Request handshake message Handshake Type = 0x0d (Certificate Request) Server Hello Done handshake message Handshake Type = 0x0e (Server Hello Done) |
| 4 |  | IOP Check | The TLS client on AE checks if the certificate of the Server is valid |
| 5 |  | Stimulus | The TLS client on AE sends Certificate, Client Key exchange, Certificate Verify, Change Cipher Spec, Finished messages |
| 6 | Mca | PRO Check TCP | Client Certificate handshake message Handshake Type = 0x0b (Client Certificate) Certificate: AE certificate Client Key Exchange message Handshake Type = 0x10 (Client Key Exchange) Public key: ECDHE generated key Client Certificate Verify message Handshake Type = 0x0f (Certificate Verify) Client Change Cipher Spec message Content type = 0x14 (Change Cipher Spec) Client Finished handshake message Handshake Type = 0x14 (Client Finished) |
| 7 |  | IOP Check | The TLS server on CSE checks if the certificate of the Client is valid |
| 8 | Mca | PRO Check TCP | The TLS server sends New Session Ticket, Change Cipher Spec, and Finished messages to the TLS client Server New Session Ticket message Handshake Type = 0x04 (New Session Ticket) Server Change Cipher Spec message Content type = 0x14 (Change Cipher Spec) Server Finished message Handshake Type = 0x14 (Client Finished) Version: TLS v1.2 |
| 9 |  | IOP Check | Check that The TLS client authenticated the Server by validating Verify Data |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.4.3	Authorization

#### 8.4.3.1	Authorization using selfPrivileges

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 04 |
| Objective: |  |  | AE accesses <accessControlPolicy> resource using its selfPrivileges credentials |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1] clause 9.6.2.0 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created an <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} selfPrivileges attribute of {accessControlPolicyName} contains the following access control tuple: acor = AE-ID acop = 63 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{accessControlPolicyName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.3.2	Authorization using accessControlPolicy privileges

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 05 |
| Objective: |  |  | AE accesses <AE> resource using its accessControlPolicyIDs attribute |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1] clause 9.6.2.0 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created an <AE> resource on registrar CSE with name {AE} accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} accessControlPolicyIDs attribute of {AE} is set to resource id of {accessControlPolicyName} privileges attribute of {accessControlPolicyName} contains the following access control tuple: acor = AE-ID acop = 34 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an AE Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE is requested to send an AE Delete Request |
| 6 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{AE} fr = AE-ID rqi = (token-string) pc = empty |
| 7 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 8 |  | IOP Check | Check if possible that the <AE> resource has not been removed in registrar CSE. |
| 9 |  | IOP Check | AE indicates unsuccessful operation (Delete error - no privilege) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.3.3	Authorization using default access privileges (owner is configured)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 06 |
| Objective: |  |  | AE accesses <AE> resource using default access privileges |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1] clause 9.6.2.0 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created an <AE> resource on registrar CSE with name {AE} <container> resource has been created in registrar CSE under <AE> resource with name {containerName} accessControlPolicyIDs attribute of {containerName} is NULL owner attribute of {containerName} = AE-ID |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a container Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE2 is requested to send a container Retrieve Request |
| 6 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{containerName} fr = AE2-ID rqi = (token-string) pc = empty |
| 7 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 8 |  | IOP Check | AE indicates unsuccessful operation (Retrieve error - no privilege) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.3.4	Authorization using default access privileges (owner is not configured)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 07 |
| Objective: |  |  | AE accesses <AE> resource using default access privileges |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1] clause 9.6.2.0 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created an <AE> resource on registrar CSE with name {AE} <container> resource has been created in registrar CSE under <AE> resource with name {containerName} accessControlPolicyIDs attribute of {containerName} is NULL owner attribute of {containerName} is not set creator attribute of {containerName} = AE-ID |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a ContainerRetrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| 5 |  | Stimulus | AE2 is requested to send a Container Retrieve Request |
| 6 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{containerName} fr = AE2-ID rqi = (token-string) pc = empty |
| 7 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 8 |  | IOP Check | AE indicates unsuccessful operation (Retrieve error - no privilege) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.3.5	Direct Dynamic Authorization

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 08 |
| Objective: |  |  | AE accesses <AE> resource using Direct Dynamic Authorization |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [1] clause 7.3.2.2 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created an <AE> resource on registrar CSE with name {AE} <container> resource has been created in registrar CSE under <AE> resource with name {containerName} Arbitrary set of <accessControlPolicy> resources are linked to the {containerName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Container Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that Tokens or Token-Ids have not been included in the request |
| 4 |  | IOP Check | Check if possible that CSE selected a DAS Server based on accessControlRules linked to the requested resource |
| 5 | Mca | PRO Check Primitive | Registrar CSE sends a Notify request to the DAS server: op = 6 (Notify) pc: securityInfo: Direct Dynamic Authorization Originator = AE-ID Originator Resource Type = 3 (Container) Operation = 2 (Retrieve) |
| 6 |  | IOP Check | Check that if the DAS Server issued token(s), they conform to the Token structure (TS-0003, clause 7.3.2.4) |
| 7 | Mca | PRO Check Primitive | The DAS server responds to the Registrar CSE: op = 6 (Notify response) pc: securityInfo: Direct Dynamic Authorization (optional) token(s): authorization token(s) (optional) dynamicACPInfo: information for creating accessControlPolicy dynamicaly |
| 8 |  | IOP Check | Check that if token(s) present in response content, the token is validated in the Registrar CSE successfully (TS-0003, clause 7.3.2.5) |
| 9 |  | IOP Check | Check that if dynamicACPInfo present in response content, the Registrar CSE created <accessControlPolicy> resource matching the dynamicACPInfo. |
| 10 | Mca | PRO Check Primitive | If access is granted, the Registrar CSE responds to the AE: rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource If access is not granted, the Registrar CSE responds to the AE: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 11 |  | IOP Check | If access is granted, AE indicates successful operation, otherwise AE indicates unsuccessful operation (Retrieve error - no privilege) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.3.6	Indirect Dynamic Authorization

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 09 |
| Objective: |  |  | AE accesses <AE> resource using Indirect Dynamic Authorization |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [1] clause 7.3.2.3 |
| Pre-test conditions: |  |  | CSEBase resource has been created in registrar CSE with name {CSEBaseName} AE has created an <AE> resource on registrar CSE with name {AE} <container> resource has been created in registrar CSE under <AE> resource with name {containerName} Arbitrary set of <accessControlPolicy> resources are linked to the {containerName} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Container Retrieve Request |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message tqf: DAS Server PoA pc = empty |
| 4 |  | IOP Check | AE indicates unsuccessful operation (Retrieve error - no privilege) |
| 5 |  | Stimulus | AE is requested to send a token request to the DAS using original request data. AuthorSignIndicator parameter is optional. |
| 6 |  | IOP Check | Check that if the DAS Server issued token(s), they conform to the Token structure (TS-0003, clause 7.3.2.4) |
| 7 |  | Stimulus | AE is requested to send a Container Retrieve Request with additional token(s) information |
| 8 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{AE}/{containerName} fr = AE-ID rqi = (token-string) (optional) tkns: token(s) if ESData-protected Token(s) are provided (optional) tids: token Id(s) if ESData-protected Token(s) are not provided pc = empty |
| 9 | Mca | PRO Check Primitive | If the request in step 7 includes token Id(s), the Registrar CSE sends a Notify request to the DAS Server: op = 6 (Notify) securityInfo Type: Indirect Dynamic Authorization pc: tids: token Id(s) |
| 10 | Mca | PRO Check Primitive | The DAS server responds to the Registrar CSE: op = 6 (Notify response) pc: securityInfo: Indirect Dynamic Authorization token(s): authorization token(s) corresponding token Id(s) |
| 12 |  | IOP Check | Check that the token(s) are validated in the Registrar CSE successfully (TS-0003, clause 7.3.2.5) |
| 13 |  | IOP Check | If access is granted, AE indicates successful operation, otherwise AE indicates unsuccessful operation (Retrieve error - no privilege) |
| 14 | Mca | PRO Check Primitive | If access is granted, the Registrar CSE responds to the AE: rsc = 2000 (OK) ltids: Local-Token-ID(s) tkns: Token(s) rqi = (token-string) same as received in request message pc = Serialized representation of <container> resource If access is not granted, the Registrar CSE responds to the AE: rsc = 4103 (ACCESS_DENIED) rqi = (token-string) same as received in request message pc = empty |
| 15 |  | IOP Check | If access is granted, AE indicates successful operation, otherwise AE indicates unsuccessful operation (Retrieve error - no privilege) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.4.4	Key provisioning management

#### 8.4.4.1	MEF Handshake Procedure using certificates

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_10 |
| Objective: |  |  | A MEF Handshake procedure establishes a mutually authenticated TLS session for protecting the communication between an MEF Client and MEF using pre-provisioned certificates. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.2 |
| Pre-test conditions: |  |  | The MEF Client and MEF have been provisioned with certificates and Cipher Suite = TLS_PSK_WITH_AES_128_CBC_SHA256 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | MEF Client and MEF establish the TLS or DTLS session using the certificate-based TLS handshake |
| 2 |  | IOP Check | Check that MEF Handshake is successful Check that the MEF's certificate is verified against the set of provisioned MEF certificate trust anchors (as described in TS-0003 [12]) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.2	MEF Handshake Procedure using Master Credentials

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 11 |
| Objective: |  |  | A MEF Handshake procedure establishes a mutually authenticated TLS or DTLS session for protecting the communication between an MEF Client and MEF using pre-provisioned Master Credentials. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.2 |
| Pre-test conditions: |  |  | The MEF Client and MEF have been provisioned with Kpm = 123456, KpmID = psk_identity, and Cipher Suites = TLS_PSK_WITH_AES_128_CBC_SHA256, TLS_PSK_WITH_AES_128_CCM_8 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | MEF Client and MEF establish the TLS or DTLS session using the certificate-based TLS handshake |
| 2 | Mca | PRO Check TCP/UDP | psk_identity = test@onem2m.com psk = 123456 |
| 3 |  | IOP Check | Check that MEF Handshake is successful |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.3	MEF Client Registration Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 12 |
| Objective: |  |  | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.3 |
| Pre-test conditions: |  |  | The MEF Client, and MEF have been provisioned with the parameters described in TS-0003 [12], clause 8.3.7 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Client Registration request |
| 3 | Mca | PRO Check TCP/UDP | MEF-FQDN = FQDN of the MEF adminFQDN = FQDN of the administrating stakeholder expirationTime = time when the registration shall expire |
| 4 |  | IOP Check | Check if possible that MEF has created a MEF Client Registration record |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Client Registration response MEFClientRegID = Identifier for the new MEF Client Registration expirationTime = time when the MEF Client Registration record shall expire MEF Client ID = Identifier of the MEF Client adminFQDN = FQDN of the administrating stakeholder |
| 6 |  | IOP Check | Check if possible that MEF Client has stored parameters provided by the MEF |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.4	MEF Client Configuration Retrieval Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 13 |
| Objective: |  |  | The MEF Client retrieves MEF Client Configurations provided by the administrating stakeholder to the MEF. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.4 |
| Pre-test conditions: |  |  | The MEF Client has previously performed the MEF Client Registration procedure to create the MEF Client Registration record. The MEF Client Registration record is not expired. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Client Configuration Retrieval request |
| 3 | Mca | PRO Check TCP/UDP | MEF-FQDN = FQDN of the MEF, from MEF Instruction Configuration MEFClientRegID = Identifier for the MEF Client registration record being updated |
| 4 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Client Configuration Retrieval response MEFClientCfg = MEF Client Configuration currently associated with the identified MEF Client registration record |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.5	MEF Client Configuration Update Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 14 |
| Objective: |  |  | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.5 |
| Pre-test conditions: |  |  | The MEF Client has previously performed the MEF Client Registration procedure to create the MEF Client Registration record. The MEF Client Registration record is not expired. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client shall send a MEF Client Registration Update request |
| 3 | Mca | PRO Check TCP/UDP | MEF-FQDN = FQDN of the MEF MEFClientRegID = Identifier for the MEF Client registration record being updated (optional) expirationTime = time when the MEF Client registration record shall expire (optional) labels = labels to aid discovery of the MEF Client registration record NOTE: At least one of expirationTime and labels shall be included. |
| 4 |  | IOP Check | Check if possible that MEF has updated the MEF Client Registration record with the proposed values |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Client Registration Update response (optional) expirationTime = time when the MEF Client registration record shall expire (optional) labels = labels to aid discovery of the MEF Client registration record NOTE: The response only includes expirationTime and/or labels if those parameters were present in the corresponding request. |
| 6 |  | IOP Check | Check if possible that MEF Client has stored parameters provided by the MEF |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.6	MEF Client De-Registration Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 15 |
| Objective: |  |  | The MEF Client registers with the MEF to confirm that it is willing to use the services of the MEF, under the authorization of the administrating stakeholder |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.6 |
| Pre-test conditions: |  |  | The MEF Client has previously performed the MEF Client Registration procedure to create the MEF Client Registration record. The MEF Client Registration record is not expired. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Client De-Registration request |
| 3 | Mca | PRO Check TCP/UDP | MEF-FQDN = FQDN of the MEF MEFClientRegID = Identifier for the MEF Client Registration record being ended |
| 4 |  | IOP Check | Check if possible that MEF has deleted the information associated with the identified MEF Client Registration record |
| 5 |  | IOP Check | The MEF sends a MEF Client Registration Update response. The MEF Client indicates the success of the operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.7	MEF Key Registration Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 16 |
| Objective: |  |  | Source MEF Client establishes a symmetric key with the MEF which can be retrieved for use by one or more Target MEF Clients |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.7 |
| Pre-test conditions: |  |  | The Source MEF Client is provided with (or has otherwise determined) the information in the MEF Key Registration Configuration (TS-0003 [12], clause 8.3.7.3) The Source MEF Client has performed the MEF Client Registration procedure (TS-0003 [12], clause 8.3.5.2.3) with the MEF for the administrating stakeholder identified in the MEF Key Registration Configuration |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The Source MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Key Registration request |
| 3 | Mca | PRO Check TCP/UDP | MEF-FQDN = FQDN of the MEF expirationTime = time when the MEF Client Registration shall expire adminFQDN = Identifier for the administrating stakeholder SUID = The Security Usage Identifier limiting the security feature in which the symmetric key may be used (optional) targetIDs = list of identifiers for the initial set of Target MEF Clients authorized to retrieve the symmetric key (optional) Key Value = output symmetric key value which is self-generated by the Source MEF Client |
| 4 |  | IOP Check | If the MEF Key Registration request included Key Value, check that MEF has stored the value. Otherwise, MEF generates Key Value from the (D)TLS session using TLS Key Export. |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Key Registration response RelativeKeyID = the relative part of the Key Identifier associated with the Key Registration expirationTime = time when the MEF Client Registration record shall expire Source MEF Client ID = Identifier of the Source MEF Client adminFQDN = FQDN of the administrating stakeholder SUID = the Security Usage Identifier limiting the security feature in which the symmetric key may be used targetIDs =list of identifiers for the initial set of Target MEF Clients authorized to retrieve the symmetric key |
| 6 |  | IOP Check | Check if possible that the Source MEF Client and MEF has stored the output symmetric key value and corresponding Key Identifier |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.8	MEF Key Retrieval Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 17 |
| Objective: |  |  | The Target MEF Client to retrieve the Key Value from a MEF corresponding to a RelativeKeyID received by the Target MEF Client |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.8 |
| Pre-test conditions: |  |  | The Target MEF Client has performed the MEF Client Credential Configuration with the MEF, including configuration of the MEF Key Retrieval URI The Source MEF Client has performed the MEF Key Registration procedure with the MEF, resulting in a registered Key Value and assigned RelativeKeyID for a specific administrating stakeholder and Security Usage Identifier The Target MEF Client received a Key Identifier from the Initiating-MEF Client in a security feature with the SUID which the Source MEF Client provided to the MEF during the MEF Key Registration procedure The Target MEF Client may expect that it is authorized to obtain the corresponding output symmetric key value. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Key Retrieval request |
| 3 | Mca | PRO Check Primitive | RelativeKeyID = The relative part of the Key Identifier received from the Source MEF Client in a security feature |
| 4 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Key Retrieval response expirationTime = time when the Key Registration shall expire Source MEF Client ID = Identifier of the Source MEF Client adminFQDN = Identifier for the administrating stakeholder SUID = the Security Usage Identifier limiting the security feature in which the symmetric key may be used Key Value = The registered value of the output symmetric key |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.9	MEF Key Registration Update Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 18 |
| Objective: |  |  | MEF Client updates the MEF Client registration by any combination of extending the expirationTime of the MEF Client Registration record or updating the labels. |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.9 |
| Pre-test conditions: |  |  | The MEF Client has previously performed the MEF Key Registration procedure to create the key registration The key registration is not expired |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client shall send a MEF Key Registration Update request |
| 3 | Mca | PRO Check TCP/UDP | MEF-FQDN = FQDN of the MEF RelativeKeyID = the relative part of the Key Identifier associated with the Key Registration (optional) expirationTime = time when the Key Registration shall expire (optional) labels = labels to aid discovery of the registered key (optional) targetIDs = proposed list of identifiers for the set of Target MEF Clients authorized to retrieve the symmetric key NOTE: At least one of expirationTime, labels or targetIDs shall be included. |
| 4 |  | IOP Check | Check if possible that MEF has updated the metadata with the proposed values |
| 5 | Mca | PRO Check TCP/UDP | The MEF sends a MEF Key Registration Update response (optional) expirationTime = current time when the key registration shall expire (optional) labels = Updated list of labels to aid discovery of the Key Registration, if any (optional) targetIDs = current list of identifiers for the initial set of Target MEF Clients authorized to retrieve the symmetric key NOTE: The response includes only those parameters that were present in the corresponding request. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.4.10	MEF Key De-Registration Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 19 |
| Objective: |  |  | Source MEF Client requests the MEF to stop distributing the registered key |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.3.5.2.10 |
| Pre-test conditions: |  |  | The MEF Client has previously performed the MEF Key Registration procedure to create the key registration. The key registration is not expired. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | The MEF Client establishes a TLS (or DTLS) connection with the MEF by performing the MEF Handshake procedure |
| 2 |  | Stimulus | The MEF Client sends a MEF Key De-Registration request |
| 3 | Mca | PRO Check TCP/UDP | MEF-FQDN = FQDN of the MEF RelativeKeyID = the relative part of the Key Identifier associated with the Key Registration |
| 4 |  | IOP Check | Check if possible that MEF has deleted the information associated with the identified key registration |
| 5 |  | IOP Check | The MEF sends a MEF Client De-Registration response. The MEF client indicates success of the operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.4.5	End-to-End security management

#### 8.4.5.1	End-to-End Security of Primitives (ESPrim) Architecture

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_ 20 |
| Objective: |  |  | AE sends an arbitrary request primitive inside of ESPrim Object to CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [12], clause 8.4.2 |
| Pre-test conditions: |  |  | AE and CSE has established a secure ESPrim connection, so that both are able to extract ESPrim Objects sent from each other AE has produced an ESPrim Object from the serialization of the arbitrary request primitive |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a NOTIFY Request Message with ESPrim Object |
| 2 | Mca | PRO Check Primitive | op = 5 (Notify) to = {CSEBaseName} from = AE-ID rqi = (token-string) pc: {seci: {sit = "esprimObject", epo: serialized ESPrim Object }} |
| 3 |  | IOP Check | Check if possible that the CSE successfully extracted the inner request primitive. Check if possible that the CSE successfully processed the inner request primitive. |
| 4 | Mca | PRO Check Primitive | The CSE sends a NOTIFY response to the AE op = 5 (Notify) to = AE-ID from = CSE-ID rqi = (token-string) pc: {seci: {sit = "esprimObject", epo: serialized ESPrim Object }} |
| 5 |  | IOP Check | Check that the AE successfully extracted the inner response primitive. Check that the AE successfully processed the inner response primitive. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.4.5.2	End-to-End Certificate-based Key Establishment (ESCertKE)

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SE_21 |
| Objective: |  |  | AE establishes a connection with the Registrar CSE using pairwiseE2EKey |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0003 [Error! Reference source not found. ], clause 8.7.2 |
| Pre-test conditions: |  |  | Both the Registrar CSE and AE support ESCertKE and are provisioned with private key and certificates. Both entities are configured with the information needed for the authentication and identification. Cipher Suite = TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends an ESCertKE Message 1 in Notify request |
| 2 | Mca | PRO Check Primitive | op = 5 (Notify) to = {CSEBaseName} from = AE-ID rqi = (token-string) pc: {seci: {sit = "escertkeMessage",eckm: ESCertKE Message 1 }} ESCertKE Message 1 includes TLS a Client Hello handshake message: Handshake Type = 0x01 (Client Hello) Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 |
| 3 | Mca | PRO Check Primitive | The Registrar CSE sends an ESCertKE Message 2 in Notify response op = 5 (Notify) to = AE-ID from = CSE-ID rqi = (token-string) pc: {seci: {sit = "escertkeMessage",eckm: ESCertKE Message 2 }} ESCertKE Message 2 includes Server Hello, Certificate, Server Key Exchange, Certificate Request, Server Hello Done messages Server Hello handshake message Handshake Type = 0x02 (Server Hello) Cipher Suite: TLS_ECDHE_ECDSA_WITH_AES_128_CBC_SHA256 Version: TLS v1.2 Certificate handshake message Handshake Type = 0x0b (Server Certificate) Certificate: the Registrar CSE certificate Server Key Exchange handshake message Handshake Type = 0x0c (Server Key Exchange) Public key: ECDHE generated key Certificate Request handshake message Handshake Type = 0x0d (Certificate Request) Server Hello Done handshake message Handshake Type = 0x0e (Server Hello Done) |
| 4 |  | IOP Check | The TLS client on AE checks if the certificate of the Server is valid |
| 5 |  | Stimulus | AE sends an ESCertKE Message 3 in Notify request |
| 6 | Mca | PRO Check Primitive | op = 5 (Notify) to = {CSEBaseName} from = AE-ID rqi = (token-string) pc: {seci: {sit = "escertkeMessage",eckm: ESCertKE Message 3 }} ESCertKE Message 3 includes Certificate, Client Key exchange, Certificate Verify, Change Cipher Spec, Finished messages Certificate handshake message Handshake Type = 0x0b (Client Certificate) Certificate: AE certificate Client Key Exchange message Handshake Type = 0x10 (Client Key Exchange) Public key: ECDHE generated key Certificate Verify message Handshake Type = 0x0f (Certificate Verify) Change Cipher Spec message Content type = 0x14 (Change Cipher Spec) Finished handshake message Handshake Type = 0x14 (Client Finished) |
| 7 |  | IOP Check | The TLS server on CSE checks if the certificate of the Client is valid |
| 8 | Mca | PRO Check Primitive | The Registrar CSE sends an ESCertKE Message 2 in Notify response op = 5 (Notify) to = AE-ID from = CSE-ID rqi = (token-string) pc: {seci: {sit = "escertkeMessage",eckm: ESCertKE Message 4 }} ESCertKE Message 4 includes Change Cipher Spec, and Finished messages Server Change Cipher Spec message Content type = 0x14 (Change Cipher Spec) Server Finished message Handshake Type = 0x14 (Client Finished) |
| 9 |  | IOP Check | Check that The TLS client authenticated the Server by validating Verify Data |
| 10 |  | IOP Check | Check that AE and the Registrar CSE has generated and cached a pairwiseE2EKey |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.5	HAIM Device Model

### 8.5.1	HAIM Light Device Creation

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_102 |
| Objective: |  |  | AE1 creates a HAIM Light Device Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| 6 |  | Stimulus | AE1 sends a request to create a <flexContainer> for binarySwitch |
| 7 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource/resource name of deviceLight fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 8 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 9 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE indicates successful operation |
| Note |  | Optional: Repeat steps 5-10 for additional deviceLight Modules |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.5.2	HAIM Light Device Status Read

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_103 |
| Objective: |  |  | AE2 reads the status of a HAIM Light Device Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27, 5.3.12 |
| Pre-test conditions: |  |  | AE2 has created an application resource <AE> on registrar CSE AE1 has created a HAIM Light Device model |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 sends a request to retrieve a <flexContainer> for binarySwitch |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <deviceLight> resource/binarySwitch fr = AE-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 4 |  | IOP Check | AE2 indicates successful operation |
| Note |  | Optional: Repeat steps 1-4 for additional deviceLight Modules |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.5.3	HAIM Light Device Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_104 |
| Objective: |  |  | AE2 turns the binarySwitch of a HAIM Light Device Model "ON" or "OFF" |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27 |
| Pre-test conditions: |  |  | AE2 has created an application resource <AE> on registrar CSE AE1 has created a HAIM Light Device model |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/ URI of <deviceLight> resource/binarySwitch fr = AE-ID rqi = (token-string) pc = Serialized representation of <flexContainer> resource with new value for powerState |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is updates in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| Note |  | Optional: Repeat steps 1-5 for additional device states and settings |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.5.4	HAIM Light Device Toggle Action

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_105 |
| Objective: |  |  | AE2 toggles the state of a HAIM Light Device Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.5.27 |
| Pre-test conditions: |  |  | AE2 has created an application resource <AE> on registrar CSE AE1 has created a HAIM Light Device model |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 sends a request to create a <flexContainer> for deviceLight |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/ URI of <deviceLight> resource/binarySwitch/toggle fr = AE-ID rqi = (token-string) pc = Serialized representation of <flexContainer> resource for toggle action |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is updates in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE2 indicates successful operation. Check that the powerState of the binarySwitch is updated. |
| Note |  | Optional: Repeat steps 1-5 for additional device states and settings |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.5.5	HAIM Power Outlet SubDevice Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_160 |
| Objective: |  |  | AE1 creates a Power Outlet SubDevice Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.4.1.2 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE AE1 has created a <flexContainer> for deviceSmartPlug |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> for subDevicePowerOutlet |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource/resource name of deviceSmartPlug fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| 6 |  | Stimulus | AE1 sends a request to create a <flexContainer> for binarySwitch |
| 7 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource/resource name of subDevicePowerOutlet fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 8 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 9 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE indicates successful operation |
| Note |  | Optional: Repeat steps 6-10 for additional subDevicePowerOutlet Modules |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.5.6	HAIM Toggle Action Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_161 |
| Objective: |  |  | AE1 creates a Toggle Action Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 5.3.1.12 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE AE1 has created a <flexContainer> for deviceSmartPlug AE1 has created a <flexContainer> for binarySwtich as a child of deviceSmartPlug |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> for Toggle |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource/resource name of binarySwtich fr = AE-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.5.7	HAIM Device Properties Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_162 |
| Objective: |  |  | AE1 creates Device Properties Model |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0023 [14], clause 6.2.5 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE AE1 has created a <flexContainer> for deviceLight AE1 has created a <node> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to create a [deviceInfo] resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <node> resource/ fr = AE-ID rqi = (token-string) ty = 13 (mgmtObj) pc = Serialized representation of [deviceInfo] resource with with properties set as attributes of the resource |
| 3 |  | IOP Check | Check if possible that the [deviceInfo] resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of [deviceInfo] resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.6	Semantics management

### 8.6.1	Semantic Access Control Policy management

#### 8.6.1.1	Procedure for creating ACP triples when a new <accessControlPolicy> resource is created

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_106 |
| Objective: |  |  | ACP triples are created when a new <accessControlPolicy> resource is created |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.2.1.5.2 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE The Registrar CSE has SGS available |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create an < accessControlPolicy_>_ resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/<AE> fr = AE-ID rqi = (token-string) ty = 1 (accessControlPolicy) pc = Serialized representation of < accessControlPolicy > resource |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource is created in Registrar CSE. Check if possible that Registrar CSE has created ACP Triples in SGS for the new <accessControlPolicy> resource |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of < accessControlPolicy > resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.1.2	Procedure for updating ACP triples when a new <accessControlPolicy> resource is updated

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_107 |
| Objective: |  |  | ACP triples are updated when an existing <accessControlPolicy> resource is updated |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.2.1.5.3 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} The Registrar CSE has SGS available |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to update a privileges attribute of {accessControlPolicyName} |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/{AE}/{accessControlPolicyName} fr = AE-ID rqi = (token-string) ty = 1 (accessControlPolicy) pc = Serialized representation of updated <accessControlPolicy> resource |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been updated in Registrar CSE Check if possible that Registrar CSE has updated corresponding ACP Triples in SGS |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.1.3	Procedure for deleting ACP triples when an existing <accessControlPolicy> resource is deleted

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_108 |
| Objective: |  |  | ACP triples are deleted when an existing <accessControlPolicy> resource is deleted |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.2.1.5.4 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} The Registrar CSE has SGS available |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send an accessControlPolicy delete request to Registrar CSE |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/{AE}/{accessControlPolicyName} fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <accessControlPolicy> resource has been removed from registrar CSE. Check if possible that Registrar CSE has deleted corresponding ACP Triples in SGS |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.1.4	Procedure for creating ACP-SD binding triples and SD relationship in SGS

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_109 |
| Objective: |  |  | ACP-SD Binding Triples and SD relationship in SGS are created when AE creates a <semanticDescriptor> resource in Registrar CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.2.1.5.5 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} The Registrar CSE has SGS available |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create an < semanticDescriptor_>_ resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/<AE> fr = AE-ID rqi = (token-string) ty = 24 (semanticDescriptor) pc = Serialized representation of <semanticDescriptor> resource acpi = URI of {accessControlPolicyName} |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE. Check if possible that Registrar CSE has created SD Relationship Triples and ACP-SD Binding Triples for the new <semanticDescriptor> in SGS |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.1.5	Procedure for updating ACP-SD binding triples in SGS

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_110 |
| Objective: |  |  | ACP-SD Binding Triples are updated when the accessControlPolicyIDs attribute of a <semanticDescriptor> resource is updated |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.2.1.5.6 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource The Registrar CSE has SGS available |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to update an accessControlPolicyIDs attribute of the resource |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <semanticDescriptor> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. Check if possible that Registrar CSE has updated corresponding ACP-SD Binding Triples in SGS |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.1.6	Procedure for updating SD relationship triples in SGS

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_111 |
| Objective: |  |  | SD Relationship Triples are updated when the descriptor attribute of a <semanticDescriptor> resource is changed |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.2.1.5.7 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource The Registrar CSE has SGS available |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Update Request to update the descriptor attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <semanticDescriptor> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. Check if possible that Registrar CSE has updated old SD Relationship Triples and/or add new SD Relationship Triple in the SGS. |
| 4 | Mca | PRO Check Primitive | Registrar CSE sends response containing: rsc = 2004 (UPDATED) rqi = (token-string) same as received in request message pc = Serialized representation of <accessControlPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.1.7	Procedure for deleting SD relationship triples and ACP-SD binding triples in SGS

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_112 |
| Objective: |  |  | SD Relationship Triples are deleted when the descriptor attribute of a <semanticDescriptor> resource is deleted |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.2.1.5.8 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE accessControlPolicy resource has been created in registrar CSE under <AE> resource with name {accessControlPolicyName} AE has created a semanticDescriptor resource <semanticDescriptor> as child resource of <AE> resource The Registrar CSE has SGS available |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticDescriptor Delete Request to update the descriptor attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 4 (Detete) to = {CSEBaseName}/URI of <semanticDescriptor> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is deleted in Registrar CSE. Check if possible that Registrar CSE has removed SD Original Triples, SD Relationship Triples, and ACP-SD Binding Triples related to the <semanticDescriptor> from SGS |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.6.2	Semantic Filtering and discovery

#### 8.6.2.1	Semantic Filtering and Discovery using <semanticFanOutPoint> resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_113 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using the <semanticFanOutPoint> |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.4.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created <container> resources {container1} and {container2} in registrar CSE under <AE> resource AE has created a group resource with semanticSupportIndicator attribute set to TRUE and memberIds set to {container1} and {container2} ids AE has created <semanticDescriptor> as a child resources of a resources {container1} and {container2} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Discovery request to discover the < semanticFanOutPoint > virtual resource of <group> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/<AE>/<group>/sfopt from = AE-ID rqi = (token-string) fu=1 smf=sparqlQuery1 pc = empty |
| 4 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the <Container> resources addresses |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.2.2	Resource link-based Semantic Discovery

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_114 |
| Objective: |  |  | AE discovers accessible resources residing in Registrar CSE using the resource link-based Semantic Discovery |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.4.3 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE AE has created <container> resources {container1} and {container2} in registrar CSE under <AE> resource AE has created a group resource with semanticSupportIndicator attribute set to TRUE and memberIds set to {container1} and {container2} ids AE has created <semanticDescriptor> resources as a child resources of a resources {container1} and {container2} named {descriptor1} and {descriptor2} {descriptor1} has relatedSemantics attribute set to ID of {descriptor2} {descriptor2} has relatedSemantics attribute set to ID of {descriptor1} |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Discovery request to discover the <container> resource using the semanticFilter filterCriteria |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/<AE> from = AE-ID rqi = (token-string) fu=1 smf=sparqlQuery1 pc = empty |
| 4 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of data object containing the <Container> resources addresses |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.2.3	Semantic query

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_115 |
| Objective: |  |  | AE performs a Semantic Query request in Registrar CSE using the semanticFilter filter criteria |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.4 oneM2M TS-0004 [2], clause 7.3.3.18 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on Registrar CSE AE1 has created a container resource <container> on Registrar CSE AE1 has created a <semanticDescriptor> as a child resource of a <container> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 is requested to send a Semantic Query Operation request to query the <container> resource using a SPARQL query |
| 2 | Mca | PRO Check Primitive | Sent request contains op = 2 (Retrieve) to = {CSEBaseName} from = AE-ID rqi = (token-string) sqi = TRUE smf=sparqlQuery1 rcn = 10 (semantic content) pc = empty |
| 3 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation semantic query result |
| 4 |  | IOP Check | AE indicates notification received |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.6.3	Semantic Mashup management

#### 8.6.3.1	SemanticMashupJobProfile Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_116 |
| Objective: |  |  | AE creates a SemanticMashupJobProfile resource in Registrar CSE via a SemanticMashupJobProfile Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.3.2 oneM2M TS-0004 [2], clause 7.4.49.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <semanticMashupJobProfile> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 40 (semanticMashupJobProfile) pc = Serialized representation of <semanticMashupJobProfile> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupJobProfile> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.2	SemanticMashupJobProfile Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_117 |
| Objective: |  |  | AE retrieves information of a semanticMashupJobProfile resource via a semanticMashupJobProfile Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.3.3 oneM2M TS-0004 [2], clause 7.4.49.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupJobProfile resource <semanticMashupJobProfile> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticMashupJobProfile> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <semanticMashupJobProfile> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupJobProfile> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.3	SemanticMashupJobProfile Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_118 |
| Objective: |  |  | AE updates attribute in <semanticMashupJobProfile> resource via a semanticMashupJobProfile Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.3.4 oneM2M TS-0004 [2], clause 7.4.49.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupJobProfile resource <semanticMashupJobProfile> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticMashupJobProfile Update Request to update the memberFilter attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <semanticMashupJobProfile> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <semanticMashupJobProfile> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupJobProfile> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.4	SemanticMashupJobProfile Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_119 |
| Objective: |  |  | AE deletes semanticMashupJobProfile resource via a semanticMashupJobProfile Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.3.5 oneM2M TS-0004 [2], clause 7.4.49.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupJobProfile resource <semanticMashupJobProfile> as child of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticMashupJobProfile Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <semanticMashupJobProfile> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticMashupJobProfile> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticMashupJobProfile> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.5	SemanticMashupInstance Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_120 |
| Objective: |  |  | AE creates a semanticMashupInstance resource in Registrar CSE via a semanticMashupInstance Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.4.2 oneM2M TS-0004 [2], clause 7.4.50.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <semanticDescriptor> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 41 (semanticMashupInstance) pc = Serialized representation of <semanticMashupInstance> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupInstance> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.6	SemanticMashupInstance Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_121 |
| Objective: |  |  | AE retrieves information of a semanticMashupInstance resource via a semanticMashupInstance Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.4.3 oneM2M TS-0004 [2], clause 7.4.50.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticMashupInstance> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <semanticMashupInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupInstance> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.7	SemanticMashupInstance Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_122 |
| Objective: |  |  | AE updates attribute in <semanticMashupInstance> resource via a semanticMashupInstance Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.4.4 oneM2M TS-0004 [2], clause 7.4.50.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticMashupInstance Update Request to update the smjpInputParameter attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <semanticMashupInstance> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <semanticMashupInstance> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupInstance> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.8	SemanticMashupInstance Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_123 |
| Objective: |  |  | AE deletes semanticMashupInstance resource via a semanticMashupInstance Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.4.5 oneM2M TS-0004 [2], clause 7.4.50.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticMashupInstance Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/ URI of <semanticMashupInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticMashupInstance> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticMashupInstance> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.9	SemanticMashupResult Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_124 |
| Objective: |  |  | AE retrieves information of a semanticMashupResult resource via a semanticMashupResult Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.6.2 oneM2M TS-0004 [2], clause 7.4.52.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource <semanticMashupResult> resource is created as child resource of <semanticMashupInstance> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <semanticMashupResult> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <semanticMashupInstance> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupResult> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.10	SemanticMashupResult Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_125 |
| Objective: |  |  | AE deletes semanticMashupResult resource via a semanticMashupResult Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.6.3 oneM2M TS-0004 [2], clause 7.4.52.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource <semanticMashupResult> resource is created as child resource of <semanticMashupInstance> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a semanticMashupResult Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/ URI of <semanticMashupInstance> resource/ URI of <semanticMashupResult> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <semanticMashupResult> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <semanticMashupResult> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.3.11	Mashup Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_126 |
| Objective: |  |  | AE triggers a calculation and generation of the mashup result by sending a <semanticMashupInstance>/<mashup> Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.5.2 oneM2M TS-0004 [2], clause 7.4.51.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticMashupInstance resource <semanticMashupInstance> as child resource of <CSEBase> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE retrieves a <mashup> resource in a <semanticMashupInstance> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <semanticMashupInstance> resource/msp fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that a <semanticMashupResult> resource is created under the <semanticMashupInstance> resource |
| 4 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticMashupResult> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.6.4	Ontology Repository management

#### 8.6.5.1	OntologyRepository Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_127 |
| Objective: |  |  | AE creates a OntologyRepository resource in Registrar CSE via a OntologyRepository Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.7.2 oneM2M TS-0004 [2], clause 7.4.46.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <ontologyRepository> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 38 (ontologyRepository) pc = Serialized representation of <ontologyRepository> resource |
| 3 |  | IOP Check | Check if possible that the <ontologyRepository> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <ontologyRepository> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.4.2	OntologyRepository Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_128 |
| Objective: |  |  | AE retrieves information of a ontologyRepository resource via a ontologyRepository Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.7.3 oneM2M TS-0004 [2], clause 7.4.46.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a ontologyRepository resource <ontologyRepository> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <ontologyRepository> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <ontologyRepository> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <ontologyRepository> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.4.3	OntologyRepository Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_129 |
| Objective: |  |  | AE updates attribute in <ontologyRepository> resource via a ontologyRepository Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.7.4 oneM2M TS-0004 [2], clause 7.4.46.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a ontologyRepository resource <ontologyRepository> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a ontologyRepository Update Request to update an attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <ontologyRepository> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <ontologyRepository> resource |
| 3 |  | IOP Check | Check if possible that the <ontologyRepository> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <ontologyRepository> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.4.4	OntologyRepository Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_130 |
| Objective: |  |  | AE deletes OntologyRepository resource via a OntologyRepository Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.7.5 oneM2M TS-0004 [2], clause 7.4.46.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a ontologyRepository resource <ontologyRepository> as child of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a ontologyRepository Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <ontologyRepository> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <ontologyRepository> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <ontologyRepository> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.6.5	Semantic validation management

#### 8.6.5.1	Semantic validation independent of <semanticDescriptor> resource operation

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_131 |
| Objective: |  |  | AE checks the validity of the <semanticDescriptor> resource via a <semanticValidation> Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.10.2 oneM2M TS-0004 [2], clause 7.4.48.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a semanticDescriptor resource <semanticDescriptor> as child of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a <semanticValidation> Update Request to check the validity of the <semanticDescriptor>resource |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <semanticDescriptor> resource/smv fr = AE-ID rqi = (token-string) pc = Serialized representation of <semanticValidation> parameters |
| 3 |  | IOP Check | Check if possible that the received <semanticDescriptor> resource with the semanticValidated attribute is set to 'true' in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.5.2	Semantic validation triggered when Create a semanticDescriptor resource

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_132 |
| Objective: |  |  | AE creates a <semanticDescriptor> resource visa SemanticDescriptor Create Request and Registrar CSE checks the validity of the created <semanticDescriptor> resource |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 7.10.3 oneM2M TS-0004 [2], clause 7.4.34.2.1 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a < semanticDescriptor > Create Request with validationEnable attribute set to 'true' |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 24 (semanticDescriptor) pc = Serialized representation of <semanticDescriptor> resource |
| 3 |  | IOP Check | Check if possible that the <semanticDescriptor> resource is created in Registrar CSE. Check if possible that the semanticValidated attribute of the <semanticDescriptor> is set to 'true'. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <semanticDescriptor> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.6.6	Ontology Mapping management

#### 8.6.6.1	OntologyMapping Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_133 |
| Objective: |  |  | AE creates an OntologyMapping resource in Registrar CSE via an OntologyMapping Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.10.2 oneM2M TS-0004 [2], clause 7.4.62.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <ontologyMapping> resource |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 52 (ontologyMapping) pc = Serialized representation of <ontologyMapping>resource |
| 3 |  | IOP Check | Check if possible that the <ontologyMapping> resource is created in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <ontologyMapping> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.6.2	OntologyMapping Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_134 |
| Objective: |  |  | AE retrieves information of an ontology mapping result via a ontologyMapping Retrieve Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.10.3 oneM2M TS-0004 [2], clause 7.4.62.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a ontologyMapping resource <ontologyMapping> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <ontologyMapping> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <ontologyMapping> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <ontologyMapping> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.6.3	OntologyMapping Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_135 |
| Objective: |  |  | AE updates attribute in <ontologyMapping> resource via a ontologyMapping Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.10.4 oneM2M TS-0004 [2], clause 7.4.62.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a ontologyMapping resource <ontologyMapping> as child resource of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a ontologyMapping Update Request to update the mappingPolicy attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <ontologyMapping> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <ontologyMapping> resource |
| 3 |  | IOP Check | Check if possible that the <ontologyMapping> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <ontologyMapping> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.6.6.4	OntologyMapping Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_136 |
| Objective: |  |  | AE deletes OntologyMapping resource via a OntologyMapping Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0034 [13], clause 6.10.5 oneM2M TS-0004 [2], clause 7.4.62.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a ontologyMapping resource <ontologyMapping> as child of <AE> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a ontologyMapping Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <ontologyMapping> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 |  | IOP Check | Check if possible that the <ontologyMapping> resource is deleted in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 5 |  | IOP Check | Check if possible that the <ontologyMapping> resource has been removed in Registrar CSE. |
| 6 |  | IOP Check | AE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.7	3GPP Interworking

### 8.7.1	Cellular IoT non-IP data delivery (NIDD)

#### 8.7.1.1	SCEF Configuration for NIDD

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_27 |
| Objective: |  |  | IN-CSE establishes SCEF Configuration for NIDD |
| Configuration: |  |  | M2M_CFG_09 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.1.1.1 |
| Pre-test conditions: |  |  | UE hosts an ADN-AE node IN-CSE has a <m2mServiceSubscriptionProfile> resource created as a child of <CSEBase> resource <serviceSubscribedNode> resource is created as a child of <m2mServiceSubscriptionProfile> Node-ID attribute of <serviceSubscribedNode> resource is set to M2M-Ext-ID of UE and niddRequired attribute is set to TRUE SCEF identifier is pre-provisioned to IN-CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-CSE issues a NIDD Configuration Request to SCEF for ADN-AE hosted on a UE |
| 2 | (T8) Mcn | PRO Check HTTP | Method = POST URI = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/ The {apiRoot} and_{scsAsId}_ segments are configured based on Service Provider and MNO policies. Payload shall include NiddConfiguration data structure with the following attributes included in the request: externalId, notificationDestination, duration, pdnEstablishmentOption, duration, pdnEstablishmentOption, reliableDataService, rdsPorts, supportedFeatures |
| 3 |  | IOP Check | Check if possible that the SCEF has successfully processes the NIDD Configuration Request |
| 4 | (T8) Mcn | PRO Check HTTP | SCEF responds for the NIDD Configuration Response: Status code = 201 (CREATED) Location header = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/{configurationId} Payload shall include NiddConfiguration data structure with the following attributes included in the request: maximumPacketSize, status, self |
| 5 |  | IOP Check | IN-CSE indicates successful operation. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

8.7.1.2 SCEF-based Mobile Terminated NIDD

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_28 |
| Objective: |  |  | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE |
| Configuration: |  |  | M2M_CFG_11 |
| References: |  |  | oneM2M TS-0026 [], clause 7.1.1.2 |
| Pre-test conditions: |  |  | IN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created an Application Entity resource <AE> on IN-CSE IN-CSE has a <m2mServiceSubscriptionProfile> resource created as a child of <CSEBase> resource <serviceSubscribedNode> resource is created as a child of <m2mServiceSubscriptionProfile> Node-ID attribute of <serviceSubscribedNode> resource is set to M2M-Ext-ID of UE and niddRequired attribute is set to TRUE SCEF identifier is pre-provisioned to IN-CSE NIDD configuration procedure is competed successfully |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | (Optional) IN-AE issues an arbitrary oneM2M request targeting an ADN-AE. |
| 2 |  | Stimulus | IN-CSE issues a SCEF-based Mobile Terminated (MT) NIDD Downlink Data Transfer Request |
| 3 | (T8) Mcn | PRO Check HTTP | Method = POST URI = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries Payload shall include NiddDownlinkDataTransfer data structure with the following attributes included in the request: externalId, maximumLatency, priority, pdnEstablishmentOption, pdnEstablishmentOption, reliableDataService, rdsPorts, data (containing onem2m primitive) |
| 4 |  | IOP Check | Check if possible that the SCEF has successfully processes the NIDD Downlink Data Transfer Request |
| 5 | (T8) Mcn | PRO Check HTTP | SCEF responds for the NIDD Downlink Data Transfer Request: Status code = 201 (CREATED) Location header = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/{configurationId} Payload shall include NiddConfiguration data structure with the following attributes included in the request: maximumPacketSize, status, self |
| 6 |  | IOP Check | In case the UE does not have an active NIDD PDN connection to the SCEF, check that SCEF buffered the request until the UE establishes the connection |
| 7 | (T8) Mcn | PRO Check HTTP | SCEF responds with NIDD Downlink Data Transfer Response: Status code = 200 (OK) / 201 (CREATED, Buffered request) Location header = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries/{downlinkDataDeliveryId} Payload shall include NiddDownlinkDataTransfer data structure with the following attributes included in the request: deliviryStatus, self, requestedRetransmissionTime. |
| 8 | (T8) Mcn | PRO Check HTTP | (Optional) SCEF returns a MT NIDD Downlink Data Delivery Status Notification to IN-CSE: Method = POST URI = {notification_uri} Payload shall include a NiddDownlinkDataDeliveryStatusNotification data structure with the following attributes included in the request: niddDownlinkDataTransfer, deliveryStatus, requestedRetransmissionTime |
| 9 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the MT NIDD Downlink Data Delivery Status Acknowledgement: Status code = 204 (NO CONTENT) |
| 10 |  | IOP Check | Check that ADN-AE on UE has executed the oneM2M request primitive accordingly |
| 11 |  | Stimulus | (Optional) ADN-AE hosted on the UE issues a MO NIDD Uplink Data Notification to deliver a oneM2M response primitive back to the Originator |
| 12 | (T8) Mcn | PRO Check HTTP | (Optional) SCEF sends for the NIDD Uplink Data Notification: Method = POST URI = {notification_uri} Payload shall include NiddUplinkDataNotification data structure with the following attributes included in the request: niddConfiguration, externalId, reliableDataService, rdsPort, data |
| 13 | (T8) Mcn | PRO Check HTTP | (Optional) IN-CSE responds with MO NIDD Uplink Data Acknowledgement and sends oneM2M response primitive to IN-AE Status code = 204 (NO CONTENT) |
| 14 |  | IOP Check | (Optional) Check that IN-AE received a corresponding oneM2M response primitive. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

8.7.1.3 SCEF-based Mobile Originated NIDD

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_29 |
| Objective: |  |  | IN-AE sends a downlink non-IP data to a UE hosting ADN-AE |
| Configuration: |  |  | M2M_CFG_11 |
| References: |  |  | oneM2M TS-0026 [], clause 7.1.1.3 |
| Pre-test conditions: |  |  | IN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created an Application Entity resource <AE> on IN-CSE IN-CSE has a <m2mServiceSubscriptionProfile> resource created as a child of <CSEBase> resource <serviceSubscribedNode> resource is created as a child of <m2mServiceSubscriptionProfile> Node-ID attribute of <serviceSubscribedNode> resource is set to M2M-Ext-ID of UE and niddRequired attribute is set to TRUE SCEF identifier is pre-provisioned to IN-CSE NIDD configuration procedure is competed successfully RDS source and destination port numbers are pre-provisioned in ADN-AE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AND-AE issues MO NIDD Uplink Data Notification to deliver an arbitrary primitive to the IN-CSE |
| 2 | (T8) Mcn | PRO Check HTTP | SCEF triggers a MO NIDD Uplink Data Notification carrying request primitive Method = POST URI = notification_uri} Payload shall include NiddUplinkDataNotification data structure with the following attributes included in the request: niddConfiguration, externalId, reliableDataService, rdsPort, data (containing onem2m primitive) |
| 3 | (T8) Mcn | PRO Check HTTP | IN-CSE responds with MO NIDD Uplink Data Acknowledgement Status code = 204 (NO CONTENT) |
| 4 |  | IOP Check | Check if possible that SCEF has processed the MO NIDD Uplink Data Acknowledgement from the IN-CSE |
| 5 |  | PRO Check | SCEF sends an RDS acknowledgment to the UE Status code = 204 (NO CONTENT) |
| 6 |  | IOP Check | Check if possible that IN-CSE processes the oneM2M request primitive |
| 7 | (T8) Mcn | PRO Check HTTP | (Optional) If a response is required, IN-CSE generates a oneM2M response and sends a MT NIDD Downlink Data Transfer Request Method = POST URI = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries Payload shall include NiddDownlinkDataTransfer data structure with the following attributes included in the request:externalId, maximumLatency, priority, pdnEstablishmentOption, (optional) reliableDataService, rdsPort, data (containing response to oneM2M primitive) |
| 8 | (T8) Mcn | PRO Check HTTP | (Optional) Scef returns MT NIDD Downlink Data Transfer Response to IN-CSE Status code = 200 (OK) / 201 (Created) URI = {apiRoot}/3gpp-nidd/v1/{scsAsId}/configurations/{configurationId}/downlink-data-deliveries/{downlinkDataDeliveryId} Payload may include NiddDownlinkDataTransfer data structure with the following attributes included in the response: deliveryStatus, self, requestedRetransmissionTime |
| 9 |  | IOP Check | (Optional) Check if possible that SCEF has processed the request and delivered it to the targeted UE (Optional) Check if possible that UE has responded with an RDS acknowledgment |
| 10 | (T8) Mcn | PRO Check HTTP | (Optional) SCEF returns MT NIDD Downlink Data Delivery Status Notification to IN-CSE Method = POST URI = {notification_uri} Payload shall include NiddDownlinkDataDeliveryStatusNotification data structure with the following attributes included in the request: niddDownlinkDataTransfer, deliveryStatus, requestedRetransmissionTime |
| 11 | (T8) Mcn | PRO Check HTTP | (Optional) IN-CSE responds to SCEF Status code = 204 (NO CONTENT) |
| 12 |  | IOP Check | (Optional) Check if possible that ADN-AE has processed the oneM2M response primitive |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.7.2	Monitoring events

#### 8.7.2.1	UE Reachability monitoring

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_30 |
| Objective: |  |  | IN-AE monitors UE Reachability status |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.4.1 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network IN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created a Node resource <node> on IN-CSE representing UE ADN-AE has created a Schedule resource <schedule> on IN-CSE under <node> resource. The networkCoordinated attribute is set to TRUE. IN-AE has subscribed to the <schedule> resource by creating a child <subscription> resource. IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = UE_REACHABILITY). UE is in idle mode |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | UE transitions to connected mode |
| 2 |  | IOP Check | Check if possible that the 3GPP network entities (e.g. HSS) has detected the condition and sent a Monitoring Event Report to SCEF |
| 3 | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification UE for_REACHABILITY to IN-CSE: Method = POST URI = {notification_uri} Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, idleStatusInfo, reachabilityType) |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the UE Reachability Monitoring Notification: Status code = 204 (NO CONTENT) |
| 5 |  | IOP Check | Check if possible that if idleStatusInfo information is provided in the report, IN-CSE has updated scheduleElement attribute of the <schedule> resource |
| 6 | (T8) Mcn | PRO Primitive | IN-CSE sends a Notify message to IN-CSE: op = 6 (Notify) pc = serialized representation of the updated <schedule> resource |
| 7 |  | IOP Check | Check if possible that ADN-AE on UE has updated its local <schedule> resource (if applicable) |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.7.2.2	UE Availability after DDN Failure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_31 |
| Objective: |  |  | UE Availability after DDN Failure scenario |
| Configuration: |  |  | M2M_CFG_11 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.4.2 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network IN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created a Node resource <node> on IN-CSE representing UE ADN-AE has created a Schedule resource <schedule> on IN-CSE under <node> resource. The networkCoordinated attribute is set to TRUE. IN-AE has subscribed to the <schedule> resource by creating a child <subscription> resource. IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = AVAILABILITY_AFTER_DDN_FAILURE) UE is in unreachable for Downlink data and in state that DDN Failure condition can be reproduced |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-AE issues an arbitrary oneM2M request targeting an ADN-AE. |
| 2 |  | IOP Check | Check that no response for UE paging is received. Check if possible that if UE is in PSM mode, the UE subscription has been updated to reflect that a notification of availability should be sent after this DDN failure |
| 3 |  | Stimulus | UE contacts the network |
| 4 |  | IOP Check | Check if possible that SCEF has received a Monitoring Indication that the UE is available |
| 5 | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification for AVAILABILITY_AFTER_DDN_FAILURE to IN-CSE: Method = POST URI = {notification_uri} Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, idleStatusInfo) |
| 6 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the DDN Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 7 |  | IOP Check | Check if possible that IN-CSE has updated the <schedule> resource to indicate that UE is available and created new scheduleElement. Check if possible that notification has been sent to the <schedule> resource subscribed entities. |
| 8 |  | Stimulus | UE transitions to Idle |
| 9 | (T8) Mcn | PRO Check HTTP | SCEF sends a UE Reachability Monitoring Event Notification Request to IN-CSE: Method = POST URI = {notification_uri} Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, idleStatusInfo). |
| 10 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the UE Reachability Monitoring Notification: Status code = 204 (NO CONTENT) |
| 11 |  | IOP Check | Check if possible that IN-CSE has updated the <schedule> resource to indicate that UE is idle. Check if possible that notification has been sent to the <schedule> resource subscribed entities. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.7.2.3	UE Communication Failure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_32 |
| Objective: |  |  | UE Communication Failure scenario |
| Configuration: |  |  | M2M_CFG_11 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.4.3 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network IN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created a Node resource <node> on IN-CSE representing UE ADN-AE has created a Schedule resource <schedule> on IN-CSE under <node> resource. The networkCoordinated attribute is set to TRUE IN-AE has subscribed to the <schedule> resource by creating a child <subscription> resource. IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = COMMUNICATION_FAILURE) |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | UE communication failure occurs |
| 2 |  | IOP Check | Check if possible SCEF has received a Monitoring Event Report |
| 3 | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification Report for COMMUNICATION_FAILURE to IN-CSE: Method = POST URI = {notification_uri} Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, failureCause). |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the Monitoring Notification Report request: Status code = 204 (NO CONTENT) |
| 5 |  | IOP Check | Check if possible that IN-CSE has updated the <schedule> resource to indicate that UE is available and created new scheduleElement. |
| 6 | (T8) Mcn | PRO Check HTTP | SCEF sends a UE Communication Failure Monitoring Event Notification Request to IN-CSE: Method = POST URI = {notification_uri} Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, failureCause). |
| 7 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the UE Communication Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 8 |  | IOP Check | Check if possible that IN-CSE has updated the scheduleElement of the <schedule_>_ resource to indicate that no communications are currently available. Check if possible that notification has been sent to the <schedule> resource subscribed entities. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.7.2.4	Roaming Status

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_33 |
| Objective: |  |  | Roaming status scenario |
| Configuration: |  |  | M2M_CFG_09 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.4.6 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network IN-CSE can make Roaming Status Reports requests ADN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE has created a Node resource <node> on IN-CSE representing UE. roamingStatus and networkID attributes of <node> resource are configured. IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = ROAMING_STATUS) |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | Roaming status of UE has changed |
| 2 |  | IOP Check | Check if possible SCEF has received a Monitoring Event Report |
| 3 | (T8) Mcn | PRO Check HTTP | SCEF receives the report and sends Monitoring Notification Report for ROAMING_STATUS to IN-CSE: Method = POST URI = {notification_uri} Payload shall include MonitoringNotification data structure with the following attributes included in the request: subscription, configResults, cancelInd, monitoringEventReports (externalIDs, monitoringType, plmnId, roamingStatus). |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the DDN Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 5 |  | IOP Check | Check if possible that IN-CSE has updated roamingStatus and networkID attributes the <node> resource. |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.7.2.5	Location updating triggered by retrieval

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_34 |
| Objective: |  |  | Location Reporting scenario |
| Configuration: |  |  | M2M_CFG_09 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.4.7.2 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network ADN-AE has created an Application Entity resource <AE> on IN-CSE IN-CSE has subscribed to to the SCEF to receive notifications (monitoringType = LOCATION_REPORTING) |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-AE sends a < locationPolicy> CREATE request |
| 2 | (T8) Mcn | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = IN-AE-ID rqi = (token-string) ty = 10 (LocationPolicy) pc = Serialized representation of <locationPolicy> resource locationSource = Netwrork based locationUpdatePeriod = 0 locationTargetID = M2M-Ext-ID of the UE locationInformationType = position fix retrieveLastKnownLocation = TRUE/FALSE |
| 3 |  | IOP Check | Check if possible that IN-CSE has created <locationPolicy> resource |
| 4 |  | Stimulus | AE is requested to send a Retrieve Request for a <latest> content instance. |
| 5 | (T8) Mcn | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <container> resource/la fr = IN-AE-ID rqi = (token-string) pc = empty |
| 6 | (T8) Mcn | PRO Check HTTP | IN-CSE makes Monitoring Event Subscription request to retrieve current location of UE Method = POST URI = {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/ Payload shall include_MonitoringEventSubscription_ data structure with the following attributes included in the request: externalId, notificationDestination, monitoringType, supportedFeatures, maximumNumberOfReports, monitorExpireTime, accuracy. locationType = CURRENT_KNOWNLOCATION |
| 7 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Event Subscription Response message to the IN-CSE Status code = 201 (CREATED) Location header = {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/{subscriptionId} Payload shall include NiddConfiguration data structure with the following attributes included in the request: monitoringEventReport, self |
| 8 |  | IOP Check | Check if possible that SCEF detected and retrieved location of UE. |
| 9 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Event Report message to the IN-CSE Method = POST URI = {notification_uri} Payload shall include_MonitoringEventSubscription_ data structure with the following attributes included in the request: subscription, configResults, cancelid,monitoring, EventReports (externalID, monitoringType, locatonInfo) |
| 10 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the DDN Failure Monitoring Notification: Status code = 204 (NO CONTENT) |
| 11 |  | IOP Check | Check if possible that IN-CSE has created a new <contentInstance> child resource of the <container>. The <contentInstance> contains the UE's current location |
| 12 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Monitoring Event Subscription message to the SCEF Method = POST URI = {apiRoot}/3gpp-monitoring-event/v1/{scsAsId}/subscriptions/ Payload shall include MonitoringEventSubscription with the following attributes included in the request: subscription, configResults, cancelid,monitoringEventReports (externalID, monitoringType, locatonInfo) locationType = LAST_KNOWNLOCATION |
| 13 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Event Report message to the IN-CSE Method = POST Response code = 201 (CREATED) URI = {apiRoot}/3gpp-monitoring-vent/v1/{scsAsId}/subscriptions/{subscriptionId} Payload shall include_MonitoringEventSubscription_ data structure with the following attributes included in the request: self, monitoringEnentReport |
| 14 |  | IOP Check | Check if possible that SCEF receives a last known location information of UE |
| 15 | (T8) Mcn | PRO Check HTTP | SCEF sends a Monitoring Notification Report message to the IN-CSE for LOCATION_REPORTING: Method = POST URI = {notification_uri} Payload shall include_MonitoringEventSubscription_ data structure with the following attributes included in the request: subscription, configResults, cancelid,monitoringEventReports |
| 16 | (T8) Mcn | PRO Check HTTP | IN-CSE responds to the Monitoring Notification: Status code = 204 (NO CONTENT) |
| 17 |  | IOP Check | Check if possible that IN-CSE has created a new <contentInstance > child resource of the <container > containing the UE's last known location in this <contentInstance> |
| 18 | (T8) Mcn | PRO Check HTTP | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of latest <contentInstance> resource, created after acquiring location info from SCEF |
| 19 |  | IOP Check | AE indicates successful operation |
| NOTE: Steps: 12-17 are only applicable, if retrieveLastKnownLocation is set to TRUE |  |  |  |
| PRO Verdict |  |  |  |

### 8.7.3	3GPP Based Device triggering

#### 8.7.3.1	General Procedure for 3GPP Based Device Triggering

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_35 |
| Objective: |  |  | IN-AE triggers ADN-AE hosted on UE |
| Configuration: |  |  | M2M_CFG_09 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.5.1 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network ADN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE is available to receive the Device Trigger Requests (triggerEnable = "TRUE") |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-AE sends a request to create a <triggerRequest> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/{AE-ID}/ fr = AE-ID rqi = (token-string) ty = 47 (triggerRequest) pc = Serialized representation of < triggerRequest > resource |
| 3 |  | IOP Check | Check if possible that IN-CSE has created <triggerRequest> resource |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Device Triggering request to the SCEF Method = POST URI = {apiRoot}/3gpp-device-triggering/v1/{scsAsId}/transactions Payload shall include DeviceTriggering data structure with the following attributes included in the request: supportedFeatures, validityPeriod, triggerPayload, externalId, applicationPortID, notificationDestination, priority |
| 5 |  | IOP Check | Check that SCEF has responded to IN-CSE for Device Triggering request Check that SCEF has delivered the device trigger message to the UE hosting ADN-AE Check that SCEF has delivered the Device Triggering Delivery Report Notification request to IN-CSE |
| 6 |  | IOP Check | Check that IN-CSE has responded to SCEF the Device Triggering Delivery Report Notification request Check if possible that IN-CSE has updated triggerStatus attribute of <triggerRequest> resource Check that IN-CSE has responded to IN-AE for <triggerRequest> Create request |
| 7 |  | IOP Check | Check that ADN-AE has performed the trigger actions |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.7.3.2	3GPP Based Device Trigger Recall/Replace Procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_36 |
| Objective: |  |  | IN-AE recalls/replaces a trigger request targeting AND-AE hosted on UE that has been already created in IN-CSE |
| Configuration: |  |  | M2M_CFG_09 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.5.2 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network ADN-AE has created an Application Entity resource <AE> on IN-CSE ADN-AE is available to receive the Device Trigger Requests (triggerEnable = "TRUE") <triggerRequest> resource targeting ADN-AE has been created in IN-CSE IN-CSE has already sent a device trigger request to the underlying 3GPP network |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-AE sends a request to UPDATE/DELETE a <triggerRequest> |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) / 4 (Delete) to = {CSEBaseName}/{AE-ID}/{URI of <triggerRequest> resource} fr = AE-ID rqi = (token-string) pc = Serialized representation of < triggerRequest > resource (for UPDATE only) |
| 3 |  | IOP Check | Check if possible that IN-CSE has created <triggerRequest> resource |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Trigger UPDATE (Replace) / DELETE (Recall) Request to the SCEF Method = PUT/DELETE URI = {apiRoot}/3gpp-device-triggering/v1/{scsAsId}/transactions (For Trigger UPDATE only) Payload shall include DeviceTriggering data structure with the following attributes included in the request: supportedFeatures, validityPeriod, triggerPayload, externalId, applicationPortID_,_ notificationDestination, priority |
| 5 |  | IOP Check | Check that SCEF has recalled/replaced the trigger Check that SCEF responded to IN-CSE for Device Trigger Recall/Replace Check that SCEF has delivered the device trigger message to the UE hosting ADN-AE Check that SCEF has delivered the Device Triggering Delivery Report Notification request to IN-CSE |
| 6 |  | IOP Check | Check if possible that IN-CSE has updated triggerStatus attribute of <triggerRequest> resource / deleted <triggerRequest> resource Check that IN-CSE has responded to IN-AE for <triggerRequest> UPDATE/DELETE request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.7.4	Configuration of traffic patterns

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_37 |
| Objective: |  |  | IN-CSE translates the oneM2M Node Traffic Pattern (TP) into a 3GPP Device Communication Pattern |
| Configuration: |  |  | M2M_CFG_09 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.6 |
| Pre-test conditions: |  |  | UE, SCEF and IN-CSE are attached to the underlying 3GPP network ADN-AE hosted on UE has created an Application Entity resource <AE> on IN-CSE IN-CSE has established relationship with MNO and is allowed to request Configuration of Device Communication Patterns. |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | ADN-AE sends a request to create an activityPatternElements attribute in <AE> resource |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/{AE-ID} fr = AE-ID rqi = (token-string) pc = Serialized representation of <AE> resource |
| 3 |  | IOP Check | Check if possible that IN-CSE has updated <AE > resource |
| 4 | (T8) Mcn | PRO Check HTTP | IN-CSE sends a Communication Patterns Configuration creation request to the SCEF Method = POST URI = {apiRoot}/3gpp-cp-parameter-provisioning/v1/{scsAsId}/subscriptions Payload shall include DeviceTriggering data structure with the following attributes included in the request: externalId, supportedFeatures, cpParameterSets |
| 5 |  | IOP Check | Check if possible that underlying 3GPP network elements has stored the new/updated CP parameter Check that SCEF has responded to IN-CSE for Communication Patterns Configuration request |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.7.5	Group message delivery using MBMS

#### 8.7.5.1	Create MBMS Group

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_38 |
| Objective: |  |  | IN-AE creates a MBMS Group for handling group related requests |
| Configuration: |  |  | M2M_CFG_12 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.7.3.1 |
| Pre-test conditions: |  |  | Member Hosting CSE, SCEF and Group Hosting CSE are attached to the underlying 3GPP network Member Hosting CSE on UE has created a <RemoteCSE> resource on Group Hosting CSE IN-AE has created an Application Entity resource <AE> on Group Hosting CSE The MBMS service area information provided by the MNO is configured in the oneM2M System External Group Identifiers for the devices have been pre-provisioned in the oneM2M System |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-AE is requested to send a group Create Request |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 9 (group) pc = Serialized representation of <group> resource |
| 3 |  | IOP Check | Check if possible that the <group> resource is created in Registrar CSE. Check if possible that multicastType attribute of the Multicast Group Information is set to 3GPP_MBMS_group |
| 4 | Mca | PRO Check Primitive | Group Hosting CSE responds to IN-AE: rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <group> resource |
| 5 | (T8) Mcn | PRO Check HTTP | Group Hosting CSE sends a Allocate TMGI Request to the SCEF Method = POST URI = {apiRoot}/3gpp-group-message-delivery-mb2 /v1/{scsAsId}/ tmgi-allocation Payload shall include TMGIAllocation data structure with the following attributes included in the request: externalGroupId, mbmsLocArea, supportedFeatures |
| 6 |  | IOP Check | Check that SCEF has delivered Allocate TMGI Response to Group Hosting CSE |
| 7 |  | IOP Check | Check if possible that the Group Hosting CSE has stored the tmgi and tmgiExpiration in the local Multicast Group Information Check that Group Hosting CSE has sent < localMulticastGroup_>_ creation requests to the Member Hosting CSE Check that Member Hosting CSE has created < localMulticastGroup_>_ resource |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.7.5.2	Group message delivery using MBMS

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_SH_39 |
| Objective: |  |  | IN-AE sends a request for accessing member resources to the Group Hosting CSE |
| Configuration: |  |  | M2M_CFG_12 |
| References: |  |  | oneM2M TS-0026 [15], clause 7.7.3.1 |
| Pre-test conditions: |  |  | Member Hosting CSE, SCEF and Group Hosting CSE are attached to the underlying 3GPP network Member Hosting CSE on UE has created a <RemoteCSE> resource on Group Hosting CSE <node> resource representing UE has been created on Group Hosting CSE <schedule > resource has been created as a child of the <node> resource on Group Hosting CSE IN-AE has created a <group> resource in Group Hosting CSE <node> resource is the member of the <group> Group Hosting CSE has created a < localMulticastGroup_>_ in the Member Hosting CSE Group Hosting CSE stores tmgi and tmgiExpiration in the local Multicast Group Information |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | IN-AE is requested to send a Retrieve Request to the fanoutPoint of <group> resource |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/{group}/fopt fr = AE-ID rqi = (token-string) |
| 3 | (T8) Mcn | PRO Check HTTP | Group Hosting CSE sends a Group Message Delivery Request to the SCEF Method = POST URI = to {apiRoot}/3gpp-group-message-delivery-mb2 /v1/{scsAsId}/ tmgi-allocation{tmgi }/delivery-via-mbms Payload shall include GMDViaMBMSByMb2data structure with the following attributes included in the request: externalGroupId, mbmsLocArea, messageDeliveryStartTime, notificationDestination |
| 4 |  | IOP Check | Check that SCEF has responded to the Group Message Check that SCEF has sent Group Message Delivery Notification to Group Hosting CSE |
| 5 |  | IOP Check | Check that Group Hosting CSE has responded to the Group Message Delivery Notification |
| 6 |  | IOP Check | Check that Member Hosting CSE has sent response message within the scope of responseTimeWindow |
| 7 |  | IOP Check | Check that Group Hosting CSE has received the response messages from Member Hosting CSEs until responseTimeWindow expires and returned the aggregated group member responses to the IN-AE/CSE |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.8	Advanced Subscriptions & Notifications management

### 8.8.1	Notification Target removal procedure

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_137 |
| Objective: |  |  | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.8 oneM2M TS-0004 [2], clause 7.4.33.2.4 |
| Pre-test conditions: |  |  | AE1 has created an Application Entity resource <AE> on Registrar CSE AE1 has created a <subscription> resource on Registrar CSE AE1 has created a <notificationTargetPolicy> resource on Registrar CSE, where policyLabel = "default", action = "accept" AE2 has created an Application Entity resource <AE> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a notificationTargetSelfReference Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <subscription> resource/ntsr fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the registrar CSE has removed the AE2 in the notificationURI attribute in the <subscription> resource |
| 5 |  | IOP Check | AE2 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.8.2	NotificationTargetMgmtPolicyRef management

#### 8.8.2.1	NotificationTargetMgmtPolicyRef Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_138 |
| Objective: |  |  | AE creates a notificationTargetMgmtPolicyRef resource in registrar CSE via a notificationTargetMgmtPolicyRef Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.10 oneM2M TS-0004 [2], clause 7.4.30.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE AE has created a <subscription> resource on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a notificationTargetMgmtPolicyRef> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <subscription> resource fr = AE-ID rqi = (token-string) ty = 25 (notificationTargetMgmtPolicyRef) pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetMgmtPolicyRef> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.2.2	NotificationTargetMgmtPolicyRef Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_139 |
| Objective: |  |  | AE retrieves notificationTargetMgmtPolicyRef resource from Registrar CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.11 oneM2M TS-0004 [2], clause 7.4.30.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a subscription resource <subscription> on Registrar CSE AE has created a <notificationTargetMgmtPolicyRef> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <notificationTargetMgmtPolicyRef> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <notificationTargetMgmtPolicyRef> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.2.3	NotificationTargetMgmtPolicyRef Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_140 |
| Objective: |  |  | AE updates information about a notificationTargetMgmtPolicyRef via notificationTargetMgmtPolicyRef>Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.12 oneM2M TS-0004 [2], clause 7.4.30.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a subscription resource <subscription> on Registrar CSE AE has created a subscription resource <notificationTargetMgmtPolicyRef> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a notificationTargetMgmtPolicyRef Update Request to update the notificationlPolicyID attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <notificationTargetMgmtPolicyRef> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <notificationTargetMgmtPolicyRef> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetMgmtPolicyRef> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <notificationTargetMgmtPolicyRef> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.2.4	NotificationTargetMgmtPolicyRef Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_141 |
| Objective: |  |  | AE removes notificationTargetMgmtPolicyRef via a notificationTargetMgmtPolicyRef Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.12 oneM2M TS-0004 [2], clause 7.4.30.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a notificationTargetMgmtPolicyRef resource <notificationTargetMgmtPolicyRef> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a notificationTargetMgmtPolicyRef Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <notificationTargetMgmtPolicyRef> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <notificationTargetMgmtPolicyRef> resource has been removed in registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

Editor's Note: it should be added some tests to verify the functionality brought by the resource <notificationTargetMgmtPolicyRef> is accomplished.

### 8.8.3	NotificationTargetPolicy management

#### 8.8.3.1	NotificationTargetPolicy Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_142 |
| Objective: |  |  | AE creates a notificationTargetPolicy resource in registrar CSE via a notificationTargetPolicy Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.14 oneM2M TS-0004 [2], clause 7.4.31.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <notificationTargetPolicy> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 26 (notificationTargetPolicy) pc = Serialized representation of <notificationTargetPolicy> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <notificationTargetPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.3.2	NotificationTargetPolicy Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_143 |
| Objective: |  |  | AE retrieves notificationTargetPolicy resource from Registrar CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.15 oneM2M TS-0004 [2], clause 7.4.31.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <notificationTargetPolicy> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <notificationTargetPolicy> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <notificationTargetPolicy> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <notificationTargetPolicy> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.3.3	NotificationTargetPolicy Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_144 |
| Objective: |  |  | AE updates information about a notificationTargetPolicy via <notificationTargetPolicy> Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.16 oneM2M TS-0004 [2], clause 7.4.31.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a notificationTargetPolicy resource <notificationTargetPolicy> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a notificationTargetPolicy Update Request to update the policyLabel attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <notificationTargetPolicy> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <Subscription> resource |
| 3 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <notificationTargetPolicy> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.3.4	NotificationTargetPolicy Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_145 |
| Objective: |  |  | AE removes notificationTargetPolicy via a <notificationTargetPolicy> Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.17 oneM2M TS-0004 [2], clause 7.4.31.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a notificationTargetPolicy resource <notificationTargetPolicy> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a notificationTargetPolicy Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <notificationTargetPolicy> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource is deleted in registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

Editor's Note: It should be added sopme tests to verify the functionality brought by the resource <notificationTargetPolicy> is accomplished.

### 8.8.4	PolicyDeletionRules management

#### 8.8.4.1	PolicyDeletionRules Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_146 |
| Objective: |  |  | AE creates a policyDeletionRules resource in registrar CSE via a policyDeletionRules Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.18 oneM2M TS-0004 [2], clause 7.4.32.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE AE has created a <notificationTargetPolicy> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <policyDeletionRules> |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/ URI of <notificationTargetPolicy> fr = AE-ID rqi = (token-string) ty = 27 (policyDeletionRules) pc = Serialized representation of <policyDeletionRules> resource |
| 3 |  | IOP Check | Check if possible that the <policyDeletionRules> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <policyDeletionRules> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.4.2	PolicyDeletionRules Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_147 |
| Objective: |  |  | AE retrieves policyDeletionRules resource from Registrar CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.19 oneM2M TS-0004 [2], clause 7.4.32.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <notificationTargetPolicy> on Registrar CSE AE has created a <policyDeletionRules> as a child of <notificationTargetPolicy> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <policyDeletionRules> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <policyDeletionRules> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <policyDeletionRules> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.4.3	PolicyDeletionRules Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_148 |
| Objective: |  |  | AE updates information about a policyDeletionRules via <policyDeletionRules> Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.20 oneM2M TS-0004 [2], clause 7.4.32.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <notificationTargetPolicy> on Registrar CSE AE has created a <policyDeletionRules> as a child of <notificationTargetPolicy> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a policyDeletionRules Update Request to update the deletionRulesRelation attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <policyDeletionRules> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <policyDeletionRules> resource |
| 3 |  | IOP Check | Check if possible that the <policyDeletionRules> resource is updated in Registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <policyDeletionRules> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.4.4	PolicyDeletionRules Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_149 |
| Objective: |  |  | AE removes policyDeletionRules via a <policyDeletionRules> Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.21 oneM2M TS-0004 [2], clause 7.4.32.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <notificationTargetPolicy> on Registrar CSE AE has created a <policyDeletionRules> as a child of <notificationTargetPolicy> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a policyDeletionRules Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <policyDeletionRules> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <notificationTargetPolicy> resource has been removed in registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

Editor's Note: It should be added some tests to verify the functionality brought by the resource <policyDeletionRules> is accomplished.

### 8.8.5	CrossResourceSubscription management

#### 8.8.5.1	CrossResourceSubscription Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_150 |
| Objective: |  |  | AE creates a crossResourceSubscription resource in registrar CSE via a crossResourceSubscription Create Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.22 oneM2M TS-0004 [2], clause 7.4.58.2.1 |
| Pre-test conditions: |  |  | AE has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE sends a request to create a <crossResourceSubscription> where regularResourcesAsTarget = AE-ID |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName} fr = AE-ID rqi = (token-string) ty = 48 (crossResourceSubscription) pc = Serialized representation of <crossResourceSubscription> resource |
| 3 |  | IOP Check | Check if possible that the <crossResourceSubscription> resource is created in registrar CSE Check if possible that the <subscribtion> resource is created as a child of <AE> resource in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <crossResourceSubscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.5.2	CrossResourceSubscription Retrieve

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_151 |
| Objective: |  |  | AE retrieves crossResourceSubscription resource from Registrar CSE |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.23 oneM2M TS-0004 [2], clause 7.4.58.2.2 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <crossResourceSubscription> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a Retrieve Request for a <crossResourceSubscription> |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <crossResourceSubscription> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc =2000 (OK) rqi = (token-string) same as received in request message pc = Serialized representation of <crossResourceSubscription> resource |
| 4 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.5.3	CrossResourceSubscription Update

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_152 |
| Objective: |  |  | AE updates information about a crossResourceSubscription via <crossResourceSubscription> Update Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.24 oneM2M TS-0004 [2], clause 7.4.58.2.3 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <crossResourceSubscription> on Registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a crossResourceSubscription Update Request to update the regularResourcesAsTarget attribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <crossResourceSubscription> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <crossResourceSubscription> resource |
| 3 |  | IOP Check | Check if possible that the <crossResourceSubscription> resource is updated in Registrar CSE. If regularResourcesAsTarget contains new target resources, check if possible that <subscription> resources are created to each new target resource. |
| 4 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <crossResourceSubscription> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.5.4	CrossResourceSubscription Delete

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_153 |
| Objective: |  |  | AE removes crossResourceSubscription via a <crossResourceSubscription> Delete Request |
| Configuration: |  |  | M2M_CFG_01 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.21 oneM2M TS-0004 [2], clause 7.4.32.2.4 |
| Pre-test conditions: |  |  | AE has created an Application Entity resource <AE> on Registrar CSE AE has created a <crossResourceSubscription> on Registrar CSE Registrar CSE has created <subscription> resources as targets of <crossResourceSubscription> |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE is requested to send a crossResourceSubscription Delete Request |
| 2 | Mca | PRO Check Primitive | op = 4 (Delete) to = {CSEBaseName}/URI of <crossResourceSubscription> resource fr = AE-ID rqi = (token-string) pc = empty |
| 3 | Mca | PRO Check Primitive | rsc = 2002 (DELETED) rqi = (token-string) same as received in request message pc = empty |
| 4 |  | IOP Check | Check if possible that the <crossResourceSubscription> resource is deleted in registrar CSE. Check if possible that the target <subscription> resources are deleted in registrar CSE. |
| 5 |  | IOP Check | AE indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

#### 8.8.5.5	Cross-Resource Notification

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_154 |
| Objective: |  |  | AE receives a notification request from the HOST CSE |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0001 [1], clause 10.2.10.26 |
| Pre-test conditions: |  |  | AE1 has created an application resource <AE> on registrar CSE AE1 has created <crossResourceSubscription> on registrar CSE AE1 has created a container1 resource <container> on registrar CSE AE1 has created a container2 resource <container> on registrar CSE AE2 has permissions to UPDATE the container1 and container2 created by AE1 Registrar CSE has created <subscription> resources as a child resource of a container1 and container2 as a target of <crossResourceSubscription> resource |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send an Update request to the container1 created by AE1. This triggers the timer as indicated in timeWindowSize attribute of <crossResourceSubscription> |
| 2 |  | Stimulus | AE2 is requested to send an Update request to the container2 created by AE1 within the time frame indicated in timeWindowSize after the 1st request. This triggers or causes the Registrar CSE to send a notification to AE1. |
| 3 | Mca | PRO Check Primitive | op = 5 (Notify) to = notificationURI of subscription resource from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 3 |  | IOP Check | Check if the notification representation |
| 4 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 5 |  | IOP Check | AE1 indicates notification received |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.9	Modbus Interworking

### 8.9.1	Modbus Thermometer Device Create

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_163 |
| Objective: |  |  | AE1 creates Device Model for Modbus device |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0040 [], clause 6.3 |
| Pre-test conditions: |  |  | AE1 is running in Modbus IPE AE1 has created an application resource <AE> on registrar CSE |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to create a <flexContainer> resource for Modbus deviceThermometer |
| 2 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource fr = AE1-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition |
| 3 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 4 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 5 |  | IOP Check | AE indicates successful operation |
| 6 |  | Stimulus | AE1 sends a request to create a <flexContainer> for temperature |
| 7 | Mca | PRO Check Primitive | op = 1 (Create) to = {CSEBaseName}/URI of <AE1> resource/resource name of Modbus deviceLight fr = AE1-ID rqi = (token-string) ty = 28 (flexContainer) pc = Serialized representation of <flexContainer> resource with proper containerDefinition and nodnProperties |
| 8 |  | IOP Check | Check if possible that the <flexContainer> resource is created in registrar CSE. |
| 9 | Mca | PRO Check Primitive | rsc = 2001 (CREATED) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE indicates successful operation |
| Note |  | Optional: Repeat steps 5-10 for battery Module |  |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.9.2	Retrieve data from a Modbus Thermometer device

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_164 |
| Objective: |  |  | Modbus IPE reads data from Modbus device and updates Registrar CSE with the read data |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0040 [], clause 6.5.1 |
| Pre-test conditions: |  |  | AE1 is running in Modbus IPE AE1 has created an application resource <AE> on registrar CSE AE1 has created a <flexContainer> for deviceThermometer AE1 has created a <flexContainer> for temperature as a child of deviceThermometer AE2 has created an application resource <AE> on registrar CSE AE2 has created a <subscription> resource as a child of temperature |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to retrieve a <flexContainer> resource for temperature |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of <deviceThermometer> resource/ temperature fr = AE1-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | pc = Serialized representation of <flexContainer> resource rsc = 2000 (OK) |
| 4 |  | IOP Check | AE1 indicates successful operation |
| 5 |  | Stimulus | Modbus IPE sends a request(s) to retrieve data from Modbus Thermometer device |
| 6 |  | IOP Check | Check if possible that Modbus IPE has successfully retrieved data from Modbus device |
| 7 |  | Stimulus | AE1 sends a request to update a <flexContainer> resource for deviceThermometer |
| 8 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of deviceThermometer/temperature fr = AE1-ID rqi = (token-string) pc = Serialized representation of updated <flexContainer> resource |
| 9 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | Check if possible that the < flexContainer > resource for temperature is updated in Registrar CSE. Registrar CSE sends a notification to AE2 |
| 11 | Mca | PRO Check Primitive | op = 5 (Notify) to = AE2-ID from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 12 | Mca | PRO Check Primitive | AE2 responds to notification rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 13 |  | IOP Check | AE2 indicates notification received |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.9.3	Write data to a Modbus Thermometer device

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_165 |
| Objective: |  |  | AE writes data into a Modbus device by updating <flexContainer> resource in Registrar CSE |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  | oneM2M TS-0040 [], clause 6.5.2 |
| Pre-test conditions: |  |  | AE1 is running in Modbus IPE AE1 has created an application resource <AE> on registrar CSE AE1 has created a <flexContainer> for deviceThermometer AE1 has created a <flexContainer> for temperature as a child of deviceThermometer AE2 has created an application resource <AE> on registrar CSE AE2 has created a <subscription> resource as a child of temperature (notificationEventType = Blocking_Update) |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a flexContainer Update Request to update the any customAttribute of the resource. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of <flexContainer> resource fr = AE-ID rqi = (token-string) pc = Serialized representation of updated <flexContainer> resource |
| 3 |  | IOP Check | Check that Registrar CSE sent a notification to AE1 |
| 4 | Mca | PRO Check Primitive | op = 5 (Notify) to = AE1-ID from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 5 |  | Stimulus | Modbus IPE sends a request(s) to write data to Modbus Thermometer device |
| 6 |  | IOP Check | Check if possible that Modbus IPE has successfully written data to Modbus device |
| 7 |  | Stimulus | AE1 sends a response to notification to Registrar CSE |
| 8 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 9 | Mca | PRO Check Primitive | Registrar CSE sends a response to AE2 rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | AE2 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

## 8.10	NoDN Interworking

### 8.10.1	Retrieve data from a NoDN device

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_166 |
| Objective: |  |  | NoDN IPE reads data from a NoDN device and updates Registrar CSE with the read data |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  |  |
| Pre-test conditions: |  |  | AE1 is running in NoDN IPE AE1 has created an application resource <AE> on registrar CSE AE1 has created a <flexContainer> representing NoDN DeviceClass AE1 has created a <flexContainer> for the ModuleClass as a child of the DeviceClass representing the NoDN device AE2 has created an application resource <AE> on registrar CSE AE2 has created a <subscription> resource as a child of the ModuleClass |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE1 sends a request to retrieve a <flexContainer> resource for the ModuleClass |
| 2 | Mca | PRO Check Primitive | op = 2 (Retrieve) to = {CSEBaseName}/URI of ModuleClass fr = AE1-ID rqi = (token-string) |
| 3 | Mca | PRO Check Primitive | pc = Serialized representation of <flexContainer> resource rsc = 2000 (OK) |
| 4 |  | IOP Check | AE1 indicates successful operation |
| 5 |  | Stimulus | NoDN IPE retrieves data from NoDN device |
| 6 |  | IOP Check | Check if possible that NoDN IPE has successfully retrieved data from NoDN device |
| 7 |  | Stimulus | AE1 sends a request to update a <flexContainer> resource for the ModuleClass |
| 8 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of the ModuleClass fr = AE1-ID rqi = (token-string) pc = Serialized representation of the updated ModuleClass |
| 9 | Mca | PRO Check Primitive | rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of <flexContainer> resource |
| 10 |  | IOP Check | Check if possible that the <flexContainer> resource for the ModuleClass is updated in Registrar CSE. Registrar CSE sends a notification to AE2 |
| 11 | Mca | PRO Check Primitive | op = 5 (Notify) to = AE2-ID from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 12 | Mca | PRO Check Primitive | AE2 responds to notification rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 13 |  | IOP Check | AE2 indicates notification received |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

### 8.10.2	Write data to a NoDN device

| Interoperability Test Description |  |  |  |
| --- | --- | --- | --- |
| Identifier: |  |  | TD_M2M_NH_167 |
| Objective: |  |  | AE writes data into a NoDN device by updating <flexContainer> resource in Registrar CSE |
| Configuration: |  |  | M2M_CFG_10 |
| References: |  |  |  |
| Pre-test conditions: |  |  | AE1 is running in NoDN IPE AE1 has created an application resource <AE> on registrar CSE AE1 has created a <flexContainer> representing the NoDN DeviceClass AE1 has created a <flexContainer> for the ModuleClass as a child of the DeviceClass representing the NoDN device AE2 has created an application resource <AE> on registrar CSE AE2 has created a <subscription> resource as a child of the ModuleClass (notificationEventType = Blocking_Update) |
| Test Sequence |  |  |  |
| Step | RP | Type | Description |
| 1 |  | Stimulus | AE2 is requested to send a flexContainer Update Request to update any customAttribute of the ModuleClass. |
| 2 | Mca | PRO Check Primitive | op = 3 (Update) to = {CSEBaseName}/URI of the ModuleClass fr = AE-ID rqi = (token-string) pc = Serialized representation of the updated ModuleClass |
| 3 |  | IOP Check | Check that Registrar CSE sent a notification to AE1 |
| 4 | Mca | PRO Check Primitive | op = 5 (Notify) to = AE1-ID from = Registrar CSE-ID rqi = (token-string) pc = Serialized representation of Notification data object |
| 5 |  | Stimulus | NoDN IPE sends a request(s) to write data to NoDN device |
| 6 |  | IOP Check | Check if possible that NoDN IPE has successfully written data to NoDN device |
| 7 |  | Stimulus | AE1 sends a response to notification to Registrar CSE |
| 8 | Mca | PRO Check Primitive | Sent response contains rsc = 2000 (OK) rqi = (token-string) same as received in request message |
| 9 | Mca | PRO Check Primitive | Registrar CSE sends a response to AE2 rsc = 2004 (Updated) rqi = (token-string) same as received in request message pc = Serialized representation of the ModuleClass |
| 10 |  | IOP Check | AE2 indicates successful operation |
| IOP Verdict |  |  |  |
| PRO Verdict |  |  |  |

# History

| Publication history |  |  |
| --- | --- | --- |
| V1.0.0 | 29-Feb-2016 | Updated Release 1 - Publication |
| V2.3.2 | 12-Mar-2016 | Release 2A - Publication |
| V3.3.0 | Sept-2021 | Release 3 - Publication |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V4.0.0 | 19-July-2021 | Implemented CR agreed at TDE#50.1 TDE-2021-0039-TS-0013_Geo-query_interop_test_case_Rel-4 |
| V4.0.1 | 27-August-2026 | Implemented CR agreed at TDE#75.4 TDE-2026-0047-TS-0013_-_Conversion_to_markdown_from_v4_0_0 |