| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | oneM2M-TS-0018-V-4.6.1 |
| Document Name: | Test Suite Structure and Test Purposes |
| Date: | 2025-09-07 |
| Abstract: | The Test Suite Structure and Test Purposes document for conformance testing consists of: - Defining the test suite structure by grouping the test purposes according to different criteria; - Specifying test purposes for conformance test. A test purpose is an informal description of the expected test behaviour. |
| Template Version: 23 February 2015 (Do not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1. Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners' Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at: http//www.oneM2M.org

Copyright Notification

© 2016, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

1 Scope
2 References
    2.1 Normative references
    2.2 Informative references
3 Definitions and abbreviations
    3.1 Definitions
    3.2 Abbreviations
4 Conventions
5 Prerequisites and Test Configurations
    5.1 Test Configurations
6 Test Suite Structure (TSS)
    6.1 Test groups
    6.1.1 Functional Entity Groups
        6.1.2 Common Services Functions Subgroups
        6.1.3 oneM2M Resource Primitives specific Operations
    6.2 Test Suite Structure (TSS) for oneM2M
7 Test Purposes (TP)
    7.1 Introduction
        7.1.1 TP definition conventions
        7.1.2 TP Identifier naming conventions
        7.1.3 Rules for the behaviour description
        7.1.4 ICS reference
PICS Items
    7.2 Test Purposes for oneM2M Service Primitives
        7.2.1 Group AE (AE)
            7.2.1.1 General Capability (GEN)
                7.2.1.1.1 CREATE Operation
                    TP/oneM2M/AE/GEN/CRE/001
                    TP/oneM2M/AE/GEN/CRE/002
                7.2.1.1.2 UPDATE Operation
                    TP/oneM2M/AE/GEN/UPD/001
                    TP/oneM2M/AE/GEN/UPD/002
                7.2.1.1.3 RETRIEVE Operation
                    TP/oneM2M/AE/GEN/RET/001
                    TP/oneM2M/AE/GEN/RET/002
                7.2.1.1.4 DELETE Operation
                    TP/oneM2M/AE/GEN/DEL/001
                    TP/oneM2M/AE/GEN/DEL/002
            7.2.1.2 Registration (REG)
                7.2.1.2.1 CREATE Operation
                    TP/oneM2M/AE/REG/CRE/001
                    TP/oneM2M/AE/REG/CRE/002
                    Editor's Note: Reference need to be checked
                7.2.1.2.2 DELETE Operation
                    TP/oneM2M/AE/REG/DEL/001
            7.2.1.3 Data Management and Repository (DMR)
                7.2.1.3.1 CREATE Operation
                    TP/oneM2M/AE/DMR/CRE/001
                    TP/oneM2M/AE/DMR/CRE/002
                    TP/oneM2M/AE/DMR/CRE/003
                    TP/oneM2M/AE/DMR/CRE/004
                7.2.1.3.2 UPDATE Operation
                    TP/oneM2M/AE/DMR/UPD/001
                    TP/oneM2M/AE/DMR/UPD/002
                7.2.1.3.3 RETRIEVE Operation
                    TP/oneM2M/AE/DMR/RET/001
                    TP/oneM2M/AE/DMR/RET/002
                    TP/oneM2M/AE/DMR/RET/003
                    TP/oneM2M/AE/DMR/RET/004
                    TP/oneM2M/AE/DMR/RET/005
                7.2.1.3.4 DELETE Operation
                    TP/oneM2M/AE/DMR/DEL/001
                    TP/oneM2M/AE/DMR/DEL/002
                    TP/oneM2M/AE/DMR/DEL/003
                    TP/oneM2M/AE/DMR/DEL/004
            7.2.1.4 Subscription and Notification (SUB)
                7.2.1.4.1 CREATE Operation
                    TP/oneM2M/AE/SUB/CRE/001
                    TP/oneM2M/AE/SUB/CRE/002
                    TP/oneM2M/AE/SUB/CRE/003
                7.2.1.4.2 NOTIFY Operation
                    TP/oneM2M/AE/SUB/NTF/001
                    TP/oneM2M/AE/SUB/NTF/002
            7.2.1.5 Communication Management and Delivery Handling (CMDH)
                7.2.1.5.1 Resource pollingChannel (PCH)
                    TP/oneM2M/AE/PCH/001
                    TP/oneM2M/AE/PCH/002
            7.2.1.6 Ontology based Interworking IPE (OBI)
                7.2.1.6.1 Initialization sequence
                7.2.1.6.2 CREATE Operation
                7.2.1.6.3 RETRIEVE Operation
            7.2.1.7 Home Appliance Information Model and Mapping (HAIM)
                7.2.1.7.1 CREATE Operation
            7.2.1.8 Security (SEC)
                7.2.1.8.1 Authentication
                7.2.1.8.1.1 ESPrim (ESP)
                    7.2.1.8.1.1.1 NOTIFY Operation
                    7.2.1.8.1.1.2 RETRIEVE Operation
                    7.2.1.8.1.2.1 CREATE Operation
                    7.2.1.8.1.3.1 NOTIFY Operation
        7.2.2 Group CSE (CE)
            7.2.2.1 General Capability (GEN)
                7.2.2.1.1 CREATE Operation
                    TP/oneM2M/CSE/GEN/CRE/001
                    TP/oneM2M/CSE/GEN/CRE/002
                    TP/oneM2M/CSE/GEN/CRE/003
                    TP/oneM2M/CSE/GEN/CRE/004
                7.2.2.1.2 UPDATE Operation
                    TP/oneM2M/CSE/GEN/UPD/001
                    TP/oneM2M/CSE/GEN/UPD/002
                    TP/oneM2M/CSE/GEN/UPD/003
                    TP/oneM2M/CSE/GEN/UPD/004
                7.2.2.1.3 RETRIEVE Operation
                    TP/oneM2M/CSE/GEN/RET/001
                    TP/oneM2M/CSE/GEN/RET/002
                    TP/oneM2M/CSE/GEN/RET/003
                    TP/oneM2M/CSE/GEN/RET/004
                7.2.2.1.4 DELETE Operation
                    TP/oneM2M/CSE/GEN/DEL/001
                    TP/oneM2M/CSE/GEN/DEL/002
                    TP/oneM2M/CSE/GEN/DEL/003
                    TP/oneM2M/CSE/GEN/DEL/004
            7.2.2.2 Registration (REG)
                7.2.2.2.1 RETRIEVE Operation
                    TP/oneM2M/CSE/REG/RET/001
                    TP/oneM2M/CSE/REG/RET/002
                    TP/oneM2M/CSE/REG/RET/003
                    TP/oneM2M/CSE/REG/RET/004
                    TP/oneM2M/CSE/REG/RET/005
                    TP/oneM2M/CSE/REG/RET/006
                    TP/oneM2M/CSE/REG/RET/007
                    TP/oneM2M/CSE/REG/RET/008
                    TP/oneM2M/CSE/REG/RET/009
                    TP/oneM2M/CSE/REG/RET/010
                7.2.2.2.2 CREATE Operation
                    TP/oneM2M/CSE/REG/CRE/001
                    TP/oneM2M/CSE/REG/CRE/002
                    TP/oneM2M/CSE/REG/CRE/003
                    TP/oneM2M/CSE/REG/CRE/004
                    TP/oneM2M/CSE/REG/CRE/005
                    TP/oneM2M/CSE/REG/CRE/006
                    TP/oneM2M/CSE/REG/CRE/007
                    TP/oneM2M/CSE/REG/CRE/008
                    TP/oneM2M/CSE/REG/CRE/009
                    TP/oneM2M/CSE/REG/CRE/010
                    TP/oneM2M/CSE/REG/CRE/011
                    TP/oneM2M/CSE/REG/CRE/012
                    TP/oneM2M/CSE/REG/CRE/013
                    TP/oneM2M/CSE/REG/CRE/014
                    TP/oneM2M/CSE/REG/CRE/015
                    TP/oneM2M/CSE/REG/CRE/016
                    TP/oneM2M/CSE/REG/CRE/017
                    TP/oneM2M/CSE/REG/CRE/018
                    TP/oneM2M/CSE/REG/CRE/019
                    TP/oneM2M/CSE/REG/CRE/021
                    TP/oneM2M/CSE/REG/CRE/022
                    TP/oneM2M/CSE/REG/CRE/023
                    TP/oneM2M/CSE/REG/CRE/024
                    TP/oneM2M/CSE/REG/CRE/025
                    TP/oneM2M/CSE/REG/CRE/026
                    TP/oneM2M/CSE/REG/CRE/027
                    TP/oneM2M/CSE/REG/CRE/028
                    TP/oneM2M/CSE/REG/CRE/032
                7.2.2.2.3 DELETE Operation
                    TP/oneM2M/CSE/REG/DEL/001
                    TP/oneM2M/CSE/REG/DEL/002
                    TP/oneM2M/CSE/REG/DEL/003
                    TP/oneM2M/CSE/REG/DEL/004
                7.2.2.2.4 UPDATE Operation
                    TP/oneM2M/CSE/REG/UPD/001
                    TP/oneM2M/CSE/REG/UPD/003
            7.2.2.3 Data Management and Repository Function (DMR)
                7.2.2.3.1 RETRIEVE Operation
                    TP/oneM2M/CSE/DMR/RET/001
                    TP/oneM2M/CSE/DMR/RET/002
                    TP/oneM2M/CSE/DMR/RET/003
                    TP/oneM2M/CSE/DMR/RET/004
                    TP/oneM2M/CSE/DMR/RET/005
                    TP/oneM2M/CSE/DMR/RET/006
                    TP/oneM2M/CSE/DMR/RET/007
                    TP/oneM2M/CSE/DMR/RET/008
                    TP/oneM2M/CSE/DMR/RET/009
                    TP/oneM2M/CSE/DMR/RET/010
                    TP/oneM2M/CSE/DMR/RET/011
                    TP/oneM2M/CSE/DMR/RET/012
                    TP/oneM2M/CSE/DMR/RET/013
                    TP/oneM2M/CSE/DMR/RET/014
                    TP/oneM2M/CSE/DMR/RET/015
                    TP/oneM2M/CSE/DMR/RET/016
                    TP/oneM2M/CSE/DMR/RET/017
                    TP/oneM2M/CSE/DMR/RET/018
                    TP/oneM2M/CSE/DMR/RET/019
                    TP/oneM2M/CSE/DMR/RET/020
                    TP/oneM2M/CSE/DMR/RET/021
                    TP/oneM2M/CSE/DMR/RET/022
                    TP/oneM2M/CSE/DMR/RET/023
                    TP/oneM2M/CSE/DMR/RET/024
                    TP/oneM2M/CSE/DMR/RET/025
                    TP/oneM2M/CSE/DMR/RET/028
                    TP/oneM2M/CSE/DMR/RET/029
                7.2.2.3.2 UPDATE Operation
                    TP/oneM2M/CSE/DMR/UPD/001
                    TP/oneM2M/CSE/DMR/UPD/002
                    TP/oneM2M/CSE/DMR/UPD/003
                    TP/oneM2M/CSE/DMR/UPD/004
                    TP/oneM2M/CSE/DMR/UPD/005
                    TP/oneM2M/CSE/DMR/UPD/006
                    TP/oneM2M/CSE/DMR/UPD/007
                    TP/oneM2M/CSE/DMR/UPD/008
                    TP/oneM2M/CSE/DMR/UPD/009
                    TP/oneM2M/CSE/DMR/UPD/010
                    TP/oneM2M/CSE/DMR/UPD/011
                    TP/oneM2M/CSE/DMR/UPD/012
                    TP/oneM2M/CSE/DMR/UPD/013
                    TP/oneM2M/CSE/DMR/UPD/014
                    TP/oneM2M/CSE/DMR/UPD/015
                    TP/oneM2M/CSE/DMR/UPD/016
                    TP/oneM2M/CSE/DMR/UPD/017
                    TP/oneM2M/CSE/DMR/UPD/018
                7.2.2.3.3 CREATE Operation
                    TP/oneM2M/CSE/DMR/CRE/001
                    TP/oneM2M/CSE/DMR/CRE/002
                    TP/oneM2M/CSE/DMR/CRE/003
                    TP/oneM2M/CSE/DMR/CRE/004
                    TP/oneM2M/CSE/DMR/CRE/005
                    TP/oneM2M/CSE/DMR/CRE/006
                    TP/oneM2M/CSE/DMR/CRE/007
                    TP/oneM2M/CSE/DMR/CRE/008
                    TP/oneM2M/CSE/DMR/CRE/009
                    TP/oneM2M/CSE/DMR/CRE/010
                    TP/oneM2M/CSE/DMR/CRE/011
                    TP/oneM2M/CSE/DMR/CRE/012
                    TP/oneM2M/CSE/DMR/CRE/013
                    TP/oneM2M/CSE/DMR/CRE/014
                    TP/oneM2M/CSE/DMR/CRE/015
                    TP/oneM2M/CSE/DMR/CRE/016
                    TP/oneM2M/CSE/DMR/CRE/017
                    TP/oneM2M/CSE/DMR/CRE/018
                    TP/oneM2M/CSE/DMR/CRE/021
                    TP/oneM2M/CSE/DMR/CRE/022
                    TP/oneM2M/CSE/DMR/CRE/023
                7.2.2.3.4 DELETE Operation
                    TP/oneM2M/CSE/DMR/DEL/001
                    TP/oneM2M/CSE/DMR/DEL/002
                    TP/oneM2M/CSE/DMR/DEL/003
                    TP/oneM2M/CSE/DMR/DEL/004
                    TP/oneM2M/CSE/DMR/DEL/005
                    TP/oneM2M/CSE/DMR/DEL/006
                    TP/oneM2M/CSE/DMR/DEL/007
                    TP/oneM2M/CSE/DMR/DEL/008
                    TP/oneM2M/CSE/DMR/DEL/009
                    TP/oneM2M/CSE/DMR/DEL/010
                7.2.2.3.5 BASIC Operation
            7.2.2.4 Subscription and Notification (SUB)
                7.2.2.4.1 CREATE Operation
                    TP/oneM2M/CSE/SUB/CRE/001
                    TP/oneM2M/CSE/SUB/CRE/002
                    TP/oneM2M/CSE/SUB/CRE/003
                    TP/oneM2M/CSE/SUB/CRE/004
                    TP/oneM2M/CSE/SUB/CRE/005
                    TP/oneM2M/CSE/SUB/CRE/006
                    TP/oneM2M/CSE/SUB/CRE/007
                    TP/oneM2M/CSE/SUB/CRE/008
                7.2.2.4.2 DELETE Operation
                    TP/oneM2M/CSE/SUB/DEL/001
                    TP/oneM2M/CSE/SUB/DEL/002
                    TP/oneM2M/CSE/SUB/DEL/003
                7.2.2.4.3 UPDATE Operation
                    TP/oneM2M/CSE/SUB/UPD/001
                    TP/oneM2M/CSE/SUB/UPD/002
                    TP/oneM2M/CSE/SUB/UPD/003
                    TP/oneM2M/CSE/SUB/UPD/004
                    TP/oneM2M/CSE/SUB/UPD/005
                    TP/oneM2M/CSE/SUB/UPD/006
                    TP/oneM2M/CSE/SUB/UPD/007
                    TP/oneM2M/CSE/SUB/UPD/008
                    TP/oneM2M/CSE/SUB/UPD/009
                7.2.2.4.4 NOTIFY Operation
                    TP/oneM2M/CSE/SUB/NTF/001
                    TP/oneM2M/CSE/SUB/NTF/002
                    TP/oneM2M/CSE/SUB/NTF/003
                    TP/oneM2M/CSE/SUB/NTF/004
                    TP/oneM2M/CSE/SUB/NTF/005
                    TP/oneM2M/CSE/SUB/NTF/006
                    TP/oneM2M/CSE/SUB/NTF/007
                    TP/oneM2M/CSE/SUB/NTF/008
                    TP/oneM2M/CSE/SUB/NTF/009
                    TP/oneM2M/CSE/SUB/NTF/010
                    TP/oneM2M/CSE/SUB/NTF/011
                    TP/oneM2M/CSE/SUB/NTF/012
                    TP/oneM2M/CSE/SUB/NTF/013
                    TP/oneM2M/CSE/SUB/NTF/014
                    TP/oneM2M/CSE/SUB/NTF/015
                    TP/oneM2M/CSE/SUB/NTF/016
                    TP/oneM2M/CSE/SUB/NTF/017
                    TP/oneM2M/CSE/SUB/NTF/018
                    TP/oneM2M/CSE/SUB/NTF/019
                    TP/oneM2M/CSE/SUB/NTF/020
                    TP/oneM2M/CSE/SUB/NTF/021
                    TP/oneM2M/CSE/SUB/NTF/022
                    TP/oneM2M/CSE/SUB/NTF/023
                    TP/oneM2M/CSE/SUB/NTF/024
            7.2.2.5 Group Management (GMG)
                7.2.2.5.1 CREATE Operation
                    TP/oneM2M/CSE/GMG/CRE/001
                    TP/oneM2M/CSE/GMG/CRE/002
                    TP/oneM2M/CSE/GMG/CRE/003
                    TP/oneM2M/CSE/GMG/CRE/004
                    TP/oneM2M/CSE/GMG/CRE/005
                    TP/oneM2M/CSE/GMG/CRE/006
                    TP/oneM2M/CSE/GMG/CRE/007
                    TP/oneM2M/CSE/GMG/CRE/008
                7.2.2.5.2 UPDATE Operation
                    TP/oneM2M/CSE/GMG/UPD/001
                    TP/oneM2M/CSE/GMG/UPD/002
                    TP/oneM2M/CSE/GMG/UPD/003
                    TP/oneM2M/CSE/GMG/UPD/004
                    TP/oneM2M/CSE/GMG/UPD/005
                    TP/oneM2M/CSE/GMG/UPD/006
                    TP/oneM2M/CSE/GMG/UPD/007
                    TP/oneM2M/CSE/GMG/UPD/008
                    TP/oneM2M/CSE/GMG/UPD/009
                    TP/oneM2M/CSE/GMG/UPD/010
                7.2.2.5.3 RETRIEVE Operation
                    TP/oneM2M/CSE/GMG/RET/001
                7.2.2.5.4 BASIC OPERATION
                    TP/oneM2M/CSE/GMG/001
                    TP/oneM2M/CSE/GMG/002
                    TP/oneM2M/CSE/GMG/003
                    TP/oneM2M/CSE/GMG/004
                    TP/oneM2M/CSE/GMG/005
                    TP/oneM2M/CSE/GMG/006
                    TP/oneM2M/CSE/GMG/007
            7.2.2.6 Discovery (DIS)
                7.2.2.6.1 RETRIEVE Operation
                    TP/oneM2M/CSE/DIS/001
                    TP/oneM2M/CSE/DIS/002
                    TP/oneM2M/CSE/DIS/003
                    TP/oneM2M/CSE/DIS/004
                    TP/oneM2M/CSE/DIS/005
                    TP/oneM2M/CSE/DIS/006
                    TP/oneM2M/CSE/DIS/007
                    TP/oneM2M/CSE/DIS/008
                    TP/oneM2M/CSE/DIS/009
                    TP/oneM2M/CSE/DIS/031
                    TP/oneM2M/CSE/DIS/032
            7.2.2.7 Location (LOC)
                7.2.2.7.1 CREATE Operation
                    TP/oneM2M/CSE/LOC/CRE/001
                    TP/oneM2M/CSE/LOC/CRE/002
                    TP/oneM2M/CSE/LOC/CRE/003
                    TP/oneM2M/CSE/LOC/CRE/004
                    TP/oneM2M/CSE/LOC/CRE/005
                    TP/oneM2M/CSE/LOC/CRE/006
                    TP/oneM2M/CSE/LOC/CRE/007
                    TP/oneM2M/CSE/LOC/CRE/008
                    TP/oneM2M/CSE/LOC/CRE/009
                7.2.2.7.2 UPDATE Operation
                    TP/oneM2M/CSE/LOC/UPD/001
                7.2.2.7.3 RETRIEVE Operation
                    TP/oneM2M/CSE/LOC/RET/001
                    TP/oneM2M/CSE/LOC/RET/002
                    TP/oneM2M/CSE/LOC/RET/003
                    TP/oneM2M/CSE/LOC/RET/004
                    TP/oneM2M/CSE/LOC/RET/005
                7.2.2.7.4 NOTIFY Operation
                    TP/oneM2M/CSE/LOC/NTF/001
                    TP/oneM2M/CSE/LOC/NTF/002
            7.2.2.8 Device Management (DMG)
                7.2.2.8.1 Resource timeSeries (TS)
                    TP/oneM2M/CSE/TS/001
                    TP/oneM2M/CSE/TS/002
                    TP/oneM2M/CSE/TS/003
                    TP/oneM2M/CSE/TS/004
                    TP/oneM2M/CSE/TS/005
            7.2.2.9 Communication Management and Delivery Handling(CMDH)
                7.2.2.9.1 Resource pollingChannel (PCH)
                    TP/oneM2M/CSE/PCH/001
                    TP/oneM2M/CSE/PCH/002
                    TP/oneM2M/CSE/PCH/003
                    TP/oneM2M/CSE/PCH/004
                    TP/oneM2M/CSE/PCH/005
                    TP/oneM2M/CSE/PCH/006
                7.2.2.9.2 Response Type (RT)
                    7.2.2.9.2.1 nonBlockingRequestSynch (NBS)
                    TP/oneM2M/CSE/RT/NBS/001
                    TP/oneM2M/CSE/RT/NBS/002
                    TP/oneM2M/CSE/RT/NBS/003
                    7.2.2.9.2.2 nonBlockingRequestAsynch (NBA)
                    TP/oneM2M/CSE/RT/NBA/001
                    TP/oneM2M/CSE/RT/NBA/002
                    TP/oneM2M/CSE/RT/NBA/003
                    TP/oneM2M/CSE/RT/NBA/004
                    TP/oneM2M/CSE/RT/NBA/005
            7.2.2.10 Security(SEC)
                7.2.2.10.1 CREATE Operation
                    TP/oneM2M/CSE/SEC/ACP/CRE/001
                    TP/oneM2M/CSE/SEC/ACP/CRE/002
                    TP/oneM2M/CSE/SEC/ACP/CRE/003
                    TP/oneM2M/CSE/SEC/ACP/CRE/004
                    TP/oneM2M/CSE/SEC/ACP/CRE/005
                7.2.2.10.2 UPDATE Operation
                    TP/oneM2M/CSE/SEC/ACP/UPD/001
                    TP/oneM2M/CSE/SEC/ACP/UPD/002
                7.2.2.10.3 BASIC OPERATION
                    TP/oneM2M/CSE/SEC/ACP/001
                    TP/oneM2M/CSE/SEC/ACP/002
                    TP/oneM2M/CSE/SEC/ACP/003
                    TP/oneM2M/CSE/SEC/ACP/004
                    TP/oneM2M/CSE/SEC/ACP/005
                    TP/oneM2M/CSE/SEC/ACP/006
                    TP/oneM2M/CSE/SEC/ACP/007
                    TP/oneM2M/CSE/SEC/ACP/008
                    TP/oneM2M/CSE/SEC/ACP/009
                    TP/oneM2M/CSE/SEC/ACP/010
                    TP/oneM2M/CSE/SEC/ACP/011
                    TP/oneM2M/CSE/SEC/ACP/012
                    TP/oneM2M/CSE/SEC/ACP/013
                    TP/oneM2M/CSE/SEC/ACP/014
                    TP/oneM2M/CSE/SEC/ACP/015
                    TP/oneM2M/CSE/SEC/ACP/016
                    TP/oneM2M/CSE/SEC/ACP/017
                7.2.2.10.4 Authentication
                    7.2.2.10.4.1 ESPrim (ESP)
                        7.2.2.10.4.1.1 NOTIFY Operation
                        7.2.2.10.4.1.2 RETRIEVE Operation
                    7.2.2.10.4.2 Roles (ROL)
                        7.2.2.10.4.2.1 CREATE Operation
                        7.2.2.10.4.2.2 RETRIEVE Operation
                        7.2.2.10.4.2.3 UPDATE Operation
                        7.2.2.10.4.2.4 NOTIFY Operation
                    7.2.2.10.4.3 DAS (DAS)
                        7.2.2.10.4.3.1 NOTIFY Operation
                    7.2.2.10.4.4 ESCertKE (ESC)
                        7.2.2.10.4.4.1 NOTIFY Operation
            7.2.2.11 Resource Announcement (ANNC)
                7.2.2.11.1 BASIC Operation
                    TP/oneM2M/CSE/ANNC/001
                7.2.2.11.2 CREATE Operation
                    TP/oneM2M/CSE/ANNC/CRE/001
                    TP/oneM2M/CSE/ANNC/CRE/002
                    TP/oneM2M/CSE/ANNC/CRE/003
                    TP/oneM2M/CSE/ANNC/CRE/004
                    TP/oneM2M/CSE/ANNC/CRE/005
                    TP/oneM2M/CSE/ANNC/CRE/006
                    TP/oneM2M/CSE/ANNC/CRE/007
                    TP/oneM2M/CSE/ANNC/CRE/008
                    TP/oneM2M/CSE/ANNC/CRE/009
                7.2.2.11.3 RETRIEVE Operation
                    TP/oneM2M/CSE/ANNC/RET/001
                    TP/oneM2M/CSE/ANNC/RET/002
                7.2.2.11.4 UPDATE Operation
                    TP/oneM2M/CSE/ANNC/UPD/001
                    TP/oneM2M/CSE/ANNC/UPD/002
                    TP/oneM2M/CSE/ANNC/UPD/003
                    TP/oneM2M/CSE/ANNC/UPD/004
                    TP/oneM2M/CSE/ANNC/UPD/005
                    TP/oneM2M/CSE/ANNC/UPD/006
                    TP/oneM2M/CSE/ANNC/UPD/007
                    TP/oneM2M/CSE/ANNC/UPD/008
                    TP/oneM2M/CSE/ANNC/UPD/009
                    TP/oneM2M/CSE/ANNC/UPD/010
                    TP/oneM2M/CSE/ANNC/UPD/011
                    TP/oneM2M/CSE/ANNC/UPD/012
                    TP/oneM2M/CSE/ANNC/UPD/013
                    TP/oneM2M/CSE/ANNC/UPD/014
                7.2.2.11.5 DELETE Operation
                    TP/oneM2M/CSE/ANNC/DEL/001
                    TP/oneM2M/CSE/ANNC/DEL/002
            7.2.2.12 3GPP interworking (3GPP)
                7.2.2.12.1 BASIC Operation
                    TP/oneM2M/CSE/3GPP/TRIG/001
                    TP/oneM2M/CSE/3GPP/TRIG/002
                    TP/oneM2M/CSE/3GPP/TRIG/003
                    TP/oneM2M/CSE/3GPP/TRIG/004
                    TP/oneM2M/CSE/3GPP/TRIG/005
                    TP/oneM2M/CSE/3GPP/TRIG/006
                    TP/oneM2M/CSE/3GPP/TRIG/007
                    TP/oneM2M/CSE/3GPP/TRIG/008
                    TP/oneM2M/CSE/3GPP/TRIG/009
                    TP/oneM2M/CSE/3GPP/TRIG/010
                    TP/oneM2M/CSE/3GPP/TRIG/011
                    TP/oneM2M/CSE/3GPP/TRIG/012
                    TP/oneM2M/CSE/3GPP/TRIG/013
                    TP/oneM2M/CSE/3GPP/TRIG/014
                7.2.2.12.2 SCEF-based NIDD
                    TP/oneM2M/CSE/3GPP/NIDD/001
                    TP/oneM2M/CSE/3GPP/NIDD/002
                    TP/oneM2M/CSE/3GPP/NIDD/003
                    TP/oneM2M/CSE/3GPP/NIDD/004
                    TP/oneM2M/CSE/3GPP/NIDD/005
                    TP/oneM2M/CSE/3GPP/NIDD/006
                    TP/oneM2M/CSE/3GPP/NIDD/007
                    TP/oneM2M/CSE/3GPP/NIDD/008
                    TP/oneM2M/CSE/3GPP/NIDD/009
            7.2.2.13 FlexContainer (FLXC)
                7.2.2.13.1 CREATE Operation
                    TP/oneM2M/CSE/FLXC/CRE/001
                    TP/oneM2M/CSE/FLXC/CRE/002
                    TP/oneM2M/CSE/FLXC/CRE/003
                    TP/oneM2M/CSE/FLXC/CRE/004
                7.2.2.13.2 UPDATE Operation
                    TP/oneM2M/CSE/FLXC/UPD/001
                    TP/oneM2M/CSE/FLXC/UPD/002
                    TP/oneM2M/CSE/FLXC/UPD/003
                    TP/oneM2M/CSE/FLXC/UPD/004
                    TP/oneM2M/CSE/FLXC/UPD/005
                    TP/oneM2M/CSE/FLXC/UPD/006
                    TP/oneM2M/CSE/FLXC/UPD/007
                    TP/oneM2M/CSE/FLXC/UPD/008
                7.2.2.13.3 RETRIEVE Operation
                    TP/oneM2M/CSE/FLXC/RET/001
                    TP/oneM2M/CSE/FLXC/RET/002
                7.2.2.13.4 DELETE Operation
                    TP/oneM2M/CSE/FLXC/DEL/001
                    TP/oneM2M/CSE/FLXC/DEL/002
            7.2.2.14 Dynamic Authorization (DAU)
                7.2.1.14.1 NOTIFY Operation
                    TP/oneM2M/CSE/DAU/NOT/001
                    TP/oneM2M/CSE/DAU/NOT/002
                    TP/oneM2M/CSE/DAU/NOT/003
                    TP/oneM2M/CSE/DAU/NOT/004
                    TP/oneM2M/CSE/DAU/NOT/005
                    TP/oneM2M/CSE/DAU/NOT/006
                    TP/oneM2M/CSE/DAU/NOT/007
                    TP/oneM2M/CSE/DAU/NOT/008
            7.2.2.15 Semantic (SEM)
                7.2.2.15.1 CREATE Operation
                    TP/oneM2M/CSE/SEM/CRE/001
                    TP/oneM2M/CSE/SEM/CRE/002
                    TP/oneM2M/CSE/SEM/CRE/003
                    TP/oneM2M/CSE/SEM/CRE/004
                    TP/oneM2M/CSE/SEM/CRE/005
                    TP/oneM2M/CSE/SEM/CRE/006
                    TP/oneM2M/CSE/SEM/CRE/007
                    TP/oneM2M/CSE/SEM/CRE/008
                    TP/oneM2M/CSE/SEM/CRE/009
                    TP/oneM2M/CSE/SEM/CRE/010
                    TP/oneM2M/CSE/SEM/CRE/011
                    TP/oneM2M/CSE/SEM/CRE/012
                    TP/oneM2M/CSE/SEM/CRE/013
                    TP/oneM2M/CSE/SEM/CRE/014
                    TP/oneM2M/CSE/SEM/CRE/015
                    TP/oneM2M/CSE/SEM/CRE/016
                    TP/oneM2M/CSE/SEM/CRE/017
                7.2.2.15.2 UPDATE Operation
                    TP/oneM2M/CSE/SEM/UPD/001
                    TP/oneM2M/CSE/SEM/UPD/002
                    TP/oneM2M/CSE/SEM/UPD/003
                    TP/oneM2M/CSE/SEM/UPD/004
                    TP/oneM2M/CSE/SEM/UPD/005
                    TP/oneM2M/CSE/SEM/UPD/006
                    TP/oneM2M/CSE/SEM/UPD/007
                    TP/oneM2M/CSE/SEM/UPD/008
                    TP/oneM2M/CSE/SEM/UPD/009
                    TP/oneM2M/CSE/SEM/UPD/010
                    TP/oneM2M/CSE/SEM/UPD/011
                    TP/oneM2M/CSE/SEM/UPD/012
                    TP/oneM2M/CSE/SEM/UPD/013
                    TP/oneM2M/CSE/SEM/UPD/014
                    TP/oneM2M/CSE/SEM/UPD/015
                7.2.2.15.3 RETRIEVE Operation
                    TP/oneM2M/CSE/SEM/RET/001
                    TP/oneM2M/CSE/SEM/RET/002
                    TP/oneM2M/CSE/SEM/RET/003
                    TP/oneM2M/CSE/SEM/RET/004
                    TP/oneM2M/CSE/SEM/RET/005
                    TP/oneM2M/CSE/SEM/RET/006
                    TP/oneM2M/CSE/SEM/RET/007
                    TP/oneM2M/CSE/SEM/RET/008
                    TP/oneM2M/CSE/SEM/RET/009
                    TP/oneM2M/CSE/SEM/RET/010
                    TP/oneM2M/CSE/SEM/RET/011
                    TP/oneM2M/CSE/SEM/RET/012
                    TP/oneM2M/CSE/SEM/RET/013
                    TP/oneM2M/CSE/SEM/RET/014
                    TP/oneM2M/CSE/SEM/RET/015
                    TP/oneM2M/CSE/SEM/CRE/016
                    TP/oneM2M/CSE/SEM/CRE/017
                7.2.2.15.4 DELETE Operation
                    TP/oneM2M/CSE/SEM/DEL/001
                    TP/oneM2M/CSE/SEM/DEL/002
                    TP/oneM2M/CSE/SEM/DEL/003
            7.2.2.16 Initial Service Provisioning (ISP)
                7.2.2.16.1 BASIC Operation
            7.2.2.17 Service Charging and Accounting (CSA)
                7.2.2.17.1 Event Based Charging (EBC)
            7.2.2.18 Service Subscription Management(SSM)
                7.2.2.18.1 Service Subscription User Profile(SSUP)
                7.2.2.18.2 Service Subscription Profile(SSP)
                    7.2.2.18.2.1 Basic Operation
                    7.2.2.18.2.2 UPDATE
                7.2.2.18.3 Service Subscribed App Rule(SSAR)
                7.2.2.18.4 Service Subscribed Node(SSN)
                    7.2.2.18.4.1 Basic Operation
                    7.2.2.18.4.2 CREATE Operation
            7.2.2.19 Primitive Profile (PP)
                7.2.2.19.1 Basic Operation
                    TP/oneM2M/CSE/PP/001
                    TP/oneM2M/CSE/PP/002
                    TP/oneM2M/CSE/PP/003
                    TP/oneM2M/CSE/PP/004
                    TP/oneM2M/CSE/PP/005
                    TP/oneM2M/CSE/PP/006
                    TP/oneM2M/CSE/PP/007
                    TP/oneM2M/CSE/PP/008
                    TP/oneM2M/CSE/PP/009
                    TP/oneM2M/CSE/PP/010
                    TP/oneM2M/CSE/PP/011
                    TP/oneM2M/CSE/PP/012
                    TP/oneM2M/CSE/PP/013
                    TP/oneM2M/CSE/PP/014
                    TP/oneM2M/CSE/PP/015
                    TP/oneM2M/CSE/PP/016
                    TP/oneM2M/CSE/PP/017
                    TP/oneM2M/CSE/PP/018
                    TP/oneM2M/CSE/PP/019
                    TP/oneM2M/CSE/PP/020
                    TP/oneM2M/CSE/PP/021
                    TP/oneM2M/CSE/PP/022
                    TP/oneM2M/CSE/PP/023
            7.2.2.20 Transaction Management(TMG)
                7.2.2.20.1 CREATE Operation
                    TP/oneM2M/CSE/TMG/CRE/001
                    TP/oneM2M/CSE/TMG/CRE/002
                    TP/oneM2M/CSE/TMG/CRE/003
                    TP/oneM2M/CSE/TMG/CRE/004
                    TP/oneM2M/CSE/TMG/CRE/005
                7.2.2.20.2 UPDATE Operation
                    TP/oneM2M/CSE/TMG/UPD/001
                    TP/oneM2M/CSE/TMG/UPD/002
                    TP/oneM2M/CSE/TMG/UPD/003
                    TP/oneM2M/CSE/TMG/UPD/004
                    TP/oneM2M/CSE/TMG/UPD/005
                    TP/oneM2M/CSE/TMG/UPD/006
                    TP/oneM2M/CSE/TMG/UPD/007
                7.2.2.20.3 DELETE Operation
                    TP/oneM2M/CSE/TMG/DEL/001
            7.2.2.21 Resource Schedule (SCH)
                7.2.2.21.1 BASIC Operation
            7.2.2.22 Process Management(PMG)
                7.2.2.22.1 CREATE Operation
                7.2.2.22.2 DELETE Operation
                7.2.2.22.3 UPDATE Operation
            7.2.2.23 Software Management(SM)
                7.2.2.23.1 BASIC Operation
                    TP/oneM2M/CSE/SM/003
                    TP/oneM2M/CSE/SM/004
                    TP/oneM2M/CSE/SM/005
                    TP/oneM2M/CSE/SM/006
                    TP/oneM2M/CSE/SM/007
                    TP/oneM2M/CSE/SM/011
                    TP/oneM2M/CSE/SM/012
    A.1 MQTT Protocol Conformance Test Requirement
    B.1 TP template
History

# 1	Scope

The present document specifies oneM2M test suite structure and test purposes that are designed to evaluate the conformity of oneM2M implementations to the oneM2M specifications. It also specifies

Guidelines for description of test behaviours and definition of test purpose structure,

Notations to define a test purpose for conformance testing, and

Test configurations for conformance testing.

# 2	References

## 2.1	Normative references

References are either specific (identified by date of publication and/or edition number or version number) or nonspecific. For specific references, only the cited version applies. For non-specific references, the latest version of the referenced document (including any amendments) applies.

The following referenced documents are necessary for the application of the present document.

- [1] oneM2M TS-0001: "Functional Architecture".

- [2] oneM2M TS-0004: "Service Layer Core Protocol Specification".

- [3] oneM2M TS-0015: "Testing Framework".

- [4] oneM2M TS-0017: "Implementation Conformance Statements".

- [5] oneM2M TS-0026: "3GPP Interworking".

- [6] oneM2M TS-0034: "Semantics Support".

- [7] oneM2M TS-0023: "Home Appliances Information Model and Mapping".

- [8] oneM2M TS-0030: "Ontology based Interworking".

## 2.2	Informative references

The following referenced documents are not necessary for the application of the present document but they assist the user with regard to a particular subject area.

- [i.1] oneM2M Drafting Rules > NOTE: Available at http://www.onem2m.org/images/files/oneM2M-Drafting-Rules.pdf.

- [i.2] oneM2M TS-0010: "MQTT Protocol Binding".

- [i.3] oneM2M TS-0031: "oneM2M Features".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TS-0015 [3] Testing Framework and the following ones apply:

valid <OPERATION> Request : a oneM2M Request primitive that only contains all mandatory primitive parameters with correct values which are not required to be explicitly specified in the test purpose. If specific values need to be specified, parameters need to be explicitly indicated in the test purpose. Operation parameter is set to <OPERATION>.

valid Response : a oneM2M Response primitive that only contains all mandatory primitive parameters with correct values which are not required to be explicitly specified in the test purpose. If specific values need to be specified, parameters need to be explicitly indicated in the test purpose. Request Identifier parameter contains the Request Identifier value of the corresponding Request.

valid <ATTRIBUTE_NAME> : correct value for the <ATTRIBUTE_NAME> attribute which is not required to be explicitly specified in the test purpose

Editor's Note: This may contain additional information.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TS-0001[1] and the follwing apply:

ADN	Application Dedicated Node

ADN-AE	AE which resides in the Application Dedicated Node

AE	Application Entity

ASN	Application Service Node

CSE	Common Service Entity

CSF	Common Service Function

IN	Infrastructure Node

IN-CSE	CSE which resides in the Infrastructure Node

IUT	Implementation Under Test

MN	Middle Node

SUT	System Under Test

# 4	Conventions

The key words "Shall", "Shall not", "May", "Need not", "Should", "Should not" in the present document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5	Prerequisites and Test Configurations

## 5.1	Test Configurations

Test configurations are defined to test different entities such as CSE and AE etc.

Figure 5.1-1 and Figure 5.1-2 show CSE test configurations where CSE as a IUT can be tested against TTCN-3 Test System, which is acting as AE or AE and CSE respectively. Test system may include as well other entities which are necessary for each configuration, i.e. a second AE (AE2) for receiving notifications These entities are represented with dashed rectangles.

Figure 5.1-3 shows AE test configuration (CF03) for ADN-AE testing scenario where ADN-AE is IUT and TTCN-3 Test System is acting as a CSE.

Figure 5.1-4 depicts test configurations between two CSEs, where one CSE is acting as a Test System, the other is SUT.

Figure 5.1-5 shows test configurations for 3GPP interworking testing scenario where 3GPP SCEF is working in a Test System.

Figure 5.1-1: Test configuration 1 (CF01)

Figure 5.1-2: Test configuration 2 (CF02)

Figure 5.1-3: Test configuration 3 (CF03)

Figure 5.1-4: Test configuration 4 (CF04)

Figure 5.1-5 : Test configuration 5 (CF05)

Figure 5.1-6: Test configuration 6 (CF06)

Figure 5.1-7 : Test configuration 7 (CF07)

# 6	Test Suite Structure (TSS)

## 6.1	Test groups

The test groups are organized in 2 levels. The first level defines the functional entity of oneM2M. The second level defines the common services functions which are represented in oneM2M functional architecture.

## 6.1.1	Functional Entity Groups

The functional entity groups identify the types of entity which are specified in oneM2M functional architecture(TS-0001 [1]).

- AE

- CSE

### 6.1.2	Common Services Functions Subgroups

The common services functions subgroups identify the general procedures in each common services functions specified in oneM2M functional architecture (TS-0001 [1]):

- Registration

- Data Management and Repository

- Subscription and Notification

- Group Management

- Discovery

- Location

- Device Management

- Communication Management and Delivery Handling

- Security

### 6.1.3	oneM2M Resource Primitives specific Operations

A group of resource type specific procedures are defined in oneM2M functional architecture (TS-0001 [1]) and service layer core protocol (TS-0004 [2]). Those procedures can be potentially abstracted with requirements for generating test purposes for conformance testing purpose. Those resource type specific procedures are the complementary procedures to the general procedures defined in oneM2M Common Service Functions (CSF) and will use for subgroups of structures for oneM2M tests. A list of those procedures are including:

- Resource announcement

- timeSeries

- location

- 3GPP interworking

- Semantic

- HAIM

- Dynamic Authorization

- FlexContainer

## 6.2	Test Suite Structure (TSS) for oneM2M

The Test Suite Structure (TSS) for oneM2M will be present as below:

- Group 1: General Capability (GEN):

  - Subgroup 1.1: Protocol Bindings

  - Subgroup 1.2: Serializations

- Group 2: AE(AE):

  - Subgroup 2.1: General Capability (GEN)

  - Subgroup 2.2: Registration(REG)

  - Subgroup 2.3: Data Management and Repository(DMR)

  - Subgroup 2.4: Subscription and Notification(SUB)

  - Subgroup 2.5: Communication Management and Delivery Handling(CMDH)

  - Subgroup 2.5.1: Request Message Polling(PCH)

- Group 3: CSE(CE):

  - Subgroup 3.1: General Capability (GEN)

  - Subgroup 3.2: Registration(REG)

  - Subgroup 3.3: Data Management and Repository(DMR)

  - Subgroup 3.4: Subscription and Notification(SUB)

  - Subgroup 3.5: Group Management(GMG)

  - Subgroup 3.6: Discovery(DIS)

  - Subgroup 3.7: Location(LOC)

  - Subgroup 3.8: Device Management(DMG)

  - Subgroup 3.8.1: Data Management using TimeSeries(TS)

  - Subgroup 3.9: Communication Management and Delivery Handling(CMDH)

  - Subgroup 3.9.1: Request Message Polling(PCH)

  - Subgroup 3.9.2: Response Type(RT)

  - Subgroup 3.10: Security(SEC)

  - Subgroup 3.11: Resource Announcement (ANNC)

  - Subgroup 3.12: 3GPP Interworking(3GPP)

  - Subgroup 3.13: FlexContainer (FLXC)

  - Subgroup 3.14: Dynamic Authorization (DAU)

  - Subgroup 3.15: Semantic (SEM)

  - Subgroup 3.16: Initial Service Provisioning (ISP)

  - Subgroup 3.17: Service Charging and Accounting (CSA)

  - Subgroup 3.18: Service Subscription Management(SSM)

  - Subgroup 3.19: Primitive Profile (PP)

  - Subgroup 3.20: Transaction Management (TMG)

  - Subgroup 3.21: Resource Schedule (SCH)

  - Subgroup 3.22: Process Management (PMG)

  - Subgroup 3.23: Software Management (SM)

# 7	Test Purposes (TP)

## 7.1	Introduction

### 7.1.1	TP definition conventions

The TP definition is constructed according to Test Suite Structure & Test Purposes(TSS&TP) in Testing Framework(TS-0015 [3]).

### 7.1.2	TP Identifier naming conventions

The identifier of the TP is constructed according to table 7.1.2-1.

Table 7.1.2-1: TP naming convention for oneM2M

| Name | Option | Interpretation |
| --- | --- | --- |
| TP///[/][/]/_ |  |  |
| = root | oneM2M | oneM2M |
| = entity | AE | Application Entity |
|  | CSE | Common Services Entity |
| = group | GEN | Common Service Function |
|  | REG |  |
|  | DMR |  |
|  | SUB |  |
|  | GMG |  |
|  | DIS |  |
|  | LOC |  |
|  | DMG |  |
|  | CMDH |  |
|  | SEC |  |
|  | ANNC |  |
|  | FLXC |  |
|  | DAU |  |
|  | SEM |  |
|  | ISP |  |
| = sub-group | ACP | AccessControlPolicy |
| = operation | CRE | Create |
|  | UPD | Update |
|  | RET | Retrieve |
|  | DEL | Delete |
|  | NTF | Notify |
| = sequential number | 001 to 999 | 023 |
| = permutation part | May be number, short name of attribute or resource type depending on the permutated part in the TP. If there are more than one permutated variables, they should be separated by slash '/'. | Enumerated type attribute and its value is represented as /, e.g. TP/oneM2M/CSE/DMR/RET/020_RCN/0 When resourceType attribute is involved in a TP Id, the resourceType shall be put in first order followed by any other attribute(s), if any: // e.g. TP/oneM2M/CSE/DMR/CRE/015_CNT/ACPI |

### 7.1.3	Rules for the behaviour description

The description of the TP is built according to TS-0015 [3] Testing Framework.

In addition, the following rules apply:

Primitive parameters shall use their Parameter name as indicated in TS-0004 [2] table 8.2.2-1

Resource attributes shall use their Attribute name as indicated in TS-0004 [2] tables 8.2.3 followed by "attribute".

When_Content_ request parameter contains a resource representation, such representation shall contain all mandatory attributes and could contain some optional attributes. Only those attributes and/or child resources required by the test purpose will be explicitly indicated.

When_Content_ response parameter contains a resource representation, such representation shall contain attributes and/or child resources according to the Result Content parameter of the corresponding request. Only those attributes and/or child resources required by the test purpose will be explicitly indicated.

### 7.1.4	ICS reference

Table 7.1.4-1: Node

| Item | Name of field | Reference | Status | Support |
| --- | --- | --- | --- | --- |
| 1 | ASN (Application Service Node) | 5.1.2 | o.1 |  |
| 2 | ADN (Application Dedicated Node) | 5.1.2 | o.1 |  |
| 3 | IN (Infrastructure Node) | 5.1.2 | o.1 |  |
| 4 | MN (Middle Node) | 5.1.2 | o.1 |  |
| o.1: Exactly one item shall be supported |  |  |  |  |

Table 7.1.4-2: Functional entity type

| Prerequisite: A.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Item | Name of field | Reference | Status | Support |
| 1 | AE | 5.1.1 | c.1 |  |
| 2 | CSE | 5.1.1 | c.2 |  |
| c.1: If A.1-1 or A.1-2 then m else o c.2: If A.1-2 x else m |  |  |  |  |

Table: Table 7.1.4-: Mnemonics for PICS reference

# PICS Items

| Mnemonic | PICS item |
| --- | --- |
| PICS_AE | TS-0017 [4], clause A.5.1.1/1 |
| PICS_CSE | TS-0017 [4], clause A.5.1.1/2 |
| PICS_ASN_CSE | TS-0017 [4], clause A.5.1.2/1 |
| PICS_ADN | TS-0017 [4], clause A.5.1.2/7 |
| PICS_IN_CSE | TS-0017 [4], clause A.5.1.2/3 |
| PICS_MN_CSE | TS-0017 [4], clause A.5.1.2/2 |
| PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT | TS-0017 [4], clause A.5.2.1/2 |
| PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT | TS-0017 [4], clause A.5.2.1/1 |
| PICS_SP_RELATIVE_RESOURCE_ID | TS-0017 [4], clause A.5.2.1 |
| PICS_ABSOLUTE_RESOURCE_ID | TS-0017 [4], clause A.5.2.1 |
| PICS_ACP_SUPPORT | TS-0017 [4], clause A.5.3.1/10 |
| PICS_CB_CST | TS-0017 [4], clause A.5.7.1/3 |
| PICS_CB_NL | TS-0017 [4], clause A.5.7.1/4 |
| PICS_CSR_RN | TS-0017 [4], clause A.5.7.2/10 |
| PICS_CSR_ET | TS-0017 [4], clause A.5.7.2/11 |
| PICS_CSR_LBL | TS-0017 [4], clause A.5.7.2/2 |
| PICS_CSR_POA | TS-0017 [4], clause A.5.7.2/6 |
| PICS_CSR_NL | TS-0017 [4], clause A.5.7.2/9 |
| PICS_CSR_CST | TS-0017 [4], clause A.5.7.2/5 |
| PICS_CSR_RR | TS-0017 [4], clause A.5.7.2/12 |
| PICS_AE_LBL | TS-0017 [4], clause A.5.7.3/2 |
| PICS_AE_APN | TS-0017 [4], clause A.5.7.3/5 |
| PICS_AE_POA | TS-0017 [4], clause A.5.7.3/6 |
| PICS_AE_NL | TS-0017 [4], clause A.5.7.3/8 |
| PICS_AE_CSZ | TS-0017 [4], clause A.5.7.3/9 |
| PICS_CNT_ACPI | TS-0017 [4], clause A.5.7.5/1 |
| PICS_CNT_MNI | TS-0017 [4], clause A.5.7.5/6 |
| PICS_CNT_MBS | TS-0017 [4], clause A.5.7.5/7 |
| PICS_CNT_MIA | TS-0017 [4], clause A.5.7.5/8 |
| PICS_CNT_OR | TS-0017 [4], clause A.5.7.5/10 |
| PICS_CNT_LI | TS-0017 [4], clause A.5.7.5/9 |
| PICS_CNT_RN | TS-0017 [4], clause A.5.7.5/11 |
| PICS_CNT_ET | TS-0017 [4], clause A.5.7.5/12 |
| PICS_CNT_LBL | TS-0017 [4], clause A.5.7.5/2 |
| PICS_CNT_CR | TS-0017 [4], clause A.5.7.5/5 |
| PICS_CIN_CNF | TS-0017 [4], clause A.5.7.6/5 |
| PICS_CIN_RN | TS-0017 [4], clause A.5.7.6/7 |
| PICS_CIN_ET | TS-0017 [4], clause A.5.7.6/8 |
| PICS_CIN_LBL | TS-0017 [4], clause A.5.7.6/1 |
| PICS_CIN_CR | TS-0017 [4], clause A.5.7.6/4 |
| PICS_ACP_LBL | TS-0017 [4], clause A.5.7.4/1 |
| PICS_SUB_ACPI | TS-0017 [4], clause A.5.7.7/2 |
| PICS_SUB_LBL | TS-0017 [4], clause A.5.7.7/1 |
| PICS_SUB_ENC | TS-0017 [4], clause A.5.7.7/3 |
| PICS_SUB_EXC | TS-0017 [4], clause A.5.7.7/4 |
| PICS_SUB_GPI | TS-0017 [4], clause A.5.7.7/5 |
| PICS_SUB_NFU | TS-0017 [4], clause A.5.7.7/6 |
| PICS_SUB_BN | TS-0017 [4], clause A.5.7.7/7 |
| PICS_SUB_RL | TS-0017 [4], clause A.5.7.7/8 |
| PICS_SUB_PN | TS-0017 [4], clause A.5.7.7/10 |
| PICS_SUB_NSP | TS-0017 [4], clause A.5.7.7/11 |
| PICS_SUB_LN | TS-0017 [4], clause A.5.7.7/12 |
| PICS_SUB_NCT | TS-0017 [4], clause A.5.7.7/13 |
| PICS_SUB_NEC | TS-0017 [4], clause A.5.7.7/14 |
| PICS_SUB_CR | TS-0017 [4], clause A.5.7.7/15 |
| PICS_SUB_SU | TS-0017 [4], clause A.5.7.7/16 |
| PICS_GRP_LBL | TS-0017 [4], clause A.5.7.8/2 |
| PICS_GRP_ACPI | TS-0017 [4], clause A.5.7.8/1 |
| PICS_GRP_MACP | TS-0017 [4], clause A.5.7.8/6 |
| PICS_GRP_GN | TS-0017 [4], clause A.5.7.8/9 |
| PICS_SCA | TS-0017 [4], clause A.5.6 |

## 7.2	Test Purposes for oneM2M Service Primitives

### 7.2.1	Group AE (AE)

#### 7.2.1.1	General Capability (GEN)

##### 7.2.1.1.1	CREATE Operation

###### TP/oneM2M/AE/GEN/CRE/001

| TP Id | TP/oneM2M/AE/GEN/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the creation of a <container> resource using unstructured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid Container CREATE Request to CSE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+———————————————————————————————-+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/CRE/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/CRE/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/CRE/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

###### TP/oneM2M/AE/GEN/CRE/002

| TP Id | TP/oneM2M/AE/GEN/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the creation of a <container> resource using structured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 3 (container) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | To set to HIERARCHICA_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 3 (container) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/CRE/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/CRE/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/CRE/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

##### 7.2.1.1.2	UPDATE Operation

###### TP/oneM2M/AE/GEN/UPD/001

| TP Id | TP/oneM2M/AE/GEN/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the update of a <container> resource using unstructured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid UPDATE Request containing  | |
                     | Toset to CONTAINER_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid labels attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid UPDATE request to CSE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/UPD/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/UPD/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/UPD/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID. |  |

###### TP/oneM2M/AE/GEN/UPD/002

| TP Id | TP/oneM2M/AE/GEN/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the update of a <container> resource using structured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid UPDATE Request containing  | |
                     | Toset to CONTAINER_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid labels attribute  | |
                     |} | |

- +------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid UPDATE request to CSE containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | Container resource containing  | |
                     | valid labels attribute  | |
                     |} | |
+———————-+——————————————————————————————+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/UPD/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/UPD/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/UPD/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID. |  |

##### 7.2.1.1.3	RETRIEVE Operation

###### TP/oneM2M/AE/GEN/RET/001

| TP Id |  | TP/oneM2M/AE/GEN/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the retrieval of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform RETRIEVE operation on container resource } |  |  |

- +-----------------------------------------------------------------+-----------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | Toset to CONTAINER_RESOURCE_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------+-----------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid RETRIEVE request to CSE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+—————————————————————–+———————————————————————————————–+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/RET/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/RET/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/RET/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/AE/GEN/RET/002

| TP Id |  | TP/oneM2M/AE/GEN/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends the retrieval of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF03 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform RETRIEVE operation on container resource } |  |  |

- +-------------------------------------------------------------------+---------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | Toset to CONTAINER_RESOURCE_ADDRESS  | |
                     |} | |

- +-------------------------------------------------------------------+---------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid RETRIEVE request to CSE containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+——————————————————————-+———————————————————————————————+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/RET/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/RET/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/RET/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

##### 7.2.1.1.4	DELETE Operation

###### TP/oneM2M/AE/GEN/DEL/001

| TP Id | TP/oneM2M/AE/GEN/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the deletion of a <container> resource using unstructured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use NON_HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform DELETE operation on container resource } |  |

- +----------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid DELETE Request containing  | |
                     | Toset to CONTAINER_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid DELETE request to CSE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+———————————————————————————————-+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/DEL/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/DEL/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/DEL/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/AE/GEN/DEL/002

| TP Id | TP/oneM2M/AE/GEN/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the deletion of a <container> resource using structured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT being configured to use HIERARCHICAL_RESOURCE_ADDRESS and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform DELETE operation on container resource } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid DELETE Request containing  | |
                     | Toset to CONTAINER_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid DELETE request to CSE containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/AE/GEN/DEL/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/AE/GEN/DEL/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/AE/GEN/DEL/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

#### 7.2.1.2	Registration (REG)

##### 7.2.1.2.1	CREATE Operation

###### TP/oneM2M/AE/REG/CRE/001

| TP Id | TP/oneM2M/AE/REG/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an AE initial registration request with no AE-ID-STEM provided when it is started |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case C, and clause 9.6.19 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT never being registered and the IUT being switched off and the IUThaving got a valid APP-ID } |  |

- +---------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | Toset to CSE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 2 (AE) and   | |
                     | Content containing  | |
                     |AE resource representation  | |
                     |} | |

- +---------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | Resource Type set to 2 (AE) and  | |
                     | To set to CSE_RESOURCE_ADDRESS and  | |
                     | From set to empty and  | |
                     | Content containing  | |
                     |AE resource representation  | |
                     |} | |
+———————-+———————————————————————————————+————-+

###### TP/oneM2M/AE/REG/CRE/002

| TP Id | TP/oneM2M/AE/REG/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a registration CREATE Request with the value of the attribute ATTRIBUTE_NAME of the AE resource |  |
| Reference | TS-0004 [2], clause 7.4.6.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT never being registered and the IUT being switched off and the IUThaving got a valid APP-ID } |  |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 2 (AE) and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | ATTRIBUTE_NAME attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid CREATE Request containing  | IUT => CSE |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | AE resourcecontaining  | |
                     | valid_ATTRIBUTE_NAME_ attribute  | |
                     |} | |
+———————-+—————————————————————————————————————+————-+

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/REG/CRE/002_RN | TS-0004 [2], clause 2 | resourceName |
| TP/oneM2M/AE/REG/CRE/002_ET | TS-0004 [2], clause 2 | expirationTime |
| TP/oneM2M/AE/REG/CRE/002_LBL | TS-0004 [2], clause 2 | labels |
| TP/oneM2M/AE/REG/CRE/002_APN | TS-0004 [2], clause 2 | appName |
| TP/oneM2M/AE/REG/CRE/002_API | TS-0004 [2], clause 2 | App-ID |
| TP/oneM2M/AE/REG/CRE/002_POA | TS-0004 [2], clause 2 | pointOfAccess |
| TP/oneM2M/AE/REG/CRE/002_OR | TS-0004 [2], clause 2 | ontologyRef |
| TP/oneM2M/AE/REG/CRE/002_NL | TS-0004 [2], clause 2 | nodeLink |
| TP/oneM2M/AE/REG/CRE/002_RR | TS-0004 [2], clause 2 | requestReachability |
| TP/oneM2M/AE/REG/CRE/002_CSZ | TS-0004 [2], clause 2 | contentSerialization |

###### Editor's Note: Reference need to be checked

##### 7.2.1.2.2	DELETE Operation

###### TP/oneM2M/AE/REG/DEL/001

| TP Id | TP/oneM2M/AE/REG/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends AE deregistration request to CSE |  |
| Reference | TS-0001 [1], clause 10.1.4.2.2 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being in the "initial state" the IUT having registered to CSE and the IUThaving privileges to perform DELETE operation on the resource AE to CSE } |  |

- +---------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid DELETE Request containing| |
                     | To set to AE_RESOURCE_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid DELETE Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+—————————————————————————+————-+

#### 7.2.1.3	Data Management and Repository (DMR)

##### 7.2.1.3.1	CREATE Operation

###### TP/oneM2M/AE/DMR/CRE/001

| TP Id | TP/oneM2M/AE/DMR/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Container creation request when it is triggered |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.1, TS-0004 [2], clause 7.2.2.1 and 7.4.7.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUThaving privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 3 (container) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |container resource representation  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 3 (container) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |container resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————–+————-+

###### TP/oneM2M/AE/DMR/CRE/002

| TP Id | TP/oneM2M/AE/DMR/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a ContentInstance creation request when it is triggered |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.1, TS-0004 [2], clause 7.2.2.1 and 7.4.8.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE. |  |
| Initial conditions | with { the IUT being registeredand the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform CREATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 4 (contentInstance) and  | |
                     | Content containing  | |
                     |contentInstance resource representation  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 4 (contentInstance) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |contentInstance resource representation  | |
                     |} | |
+———————-+——————————————————————————————————————-+————-+

###### TP/oneM2M/AE/DMR/CRE/003

| TP Id | TP/oneM2M/AE/DMR/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a ContentInstance creation request with optional attribute ATTRIBUTE_NAME |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.1, TS-0004 [2], clause 7.2.2.1 and 7.4.8.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS through preconfiguration request and the IUThaving privileges to perform CREATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 4 (contentInstance) and  | |
                     | Content containing  | |
                     | ContentInstance resource c ontaining  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 4 (contentInstance) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | ContentInstance resource c ontaining  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |
+———————-+——————————————————————————————————————-+————-+

| TP Id | PICS Selection | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/CRE/003_CNF | PICS_CIN_CNF | contentInfo |
| TP/oneM2M/AE/DMR/CRE/003_RN | PICS_CIN_RN | resourceName |
| TP/oneM2M/AE/DMR/CRE/003_ET | PICS_CIN_ET | expirationTime |
| TP/oneM2M/AE/DMR/CRE/003_LBL | PICS_CIN_LBL | labels |
| TP/oneM2M/AE/DMR/CRE/003_CR | PICS_CIN_CR | creator |

###### TP/oneM2M/AE/DMR/CRE/004

| TP Id | TP/oneM2M/AE/DMR/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Container creation request with optional attribute ATTRIBUTE_NAME when it is triggered |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.1, TS-0004 [2], clause 7.2.2.1 and 7.4.7.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUThaving privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 3 (container) and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 3 (container) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |
+———————-+—————————————————————————————————————–+————-+

| TP Id | PICS Selection | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/CRE/004_ACPI | PICS_CNT_ACPI | accessControlPolicyIDs |
| TP/oneM2M/AE/DMR/CRE/004_MNI | PICS_CNT_MNI | maxNrOfInstances |
| TP/oneM2M/AE/DMR/CRE/004_MBS | PICS_CNT_MBS | maxByteSize |
| TP/oneM2M/AE/DMR/CRE/004_MIA | PICS_CNT_MIA | maxInstanceAge |
| TP/oneM2M/AE/DMR/CRE/004_OR | PICS_CNT_OR | ontologyRef |
| TP/oneM2M/AE/DMR/CRE/004_LI | PICS_CNT_LI | locationID |
| TP/oneM2M/AE/DMR/CRE/004_RN | PICS_CNT_RN | resourceName |
| TP/oneM2M/AE/DMR/CRE/004_ET | PICS_CNT_ET | expirationTime |
| TP/oneM2M/AE/DMR/CRE/004_LBL | PICS_CNT_LBL | labels |
| TP/oneM2M/AE/DMR/CRE/004_CR | PICS_CNT_CR | creator |

Editor's Note: Combination of optional attributes that are included in the Content of container creation request can be extended to generate more test cases based on this test case.

##### 7.2.1.3.2	UPDATE Operation

###### TP/oneM2M/AE/DMR/UPD/001

| TP Id | TP/oneM2M/AE/DMR/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an UPDATE Request with the value of the attribute ATTRIBUTE_NAME of the AE resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered containing a RW attribute ATTRIBUTE_NAME } |  |

- +--------------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     | the IUT is triggered to send a valid UPDATE Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | AE resourcecontaining  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid UPDATE Request containing  | IUT => CSE |
                     | To set to AE _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | AE resourcecontaining  | |
                     | valid_ATTRIBUTE_NAME_ attribute | |
                     |} | |
+———————-+————————————————————————————————–+————-+

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/001_ET | TS-0004 [2], clause 7.4.6.1 | expirationTime |
| TP/oneM2M/AE/DMR/UPD/001_LBL | TS-0004 [2], clause 7.4.6.1 | labels |
| TP/oneM2M/AE/DMR/UPD/001_APN | TS-0004 [2], clause 7.4.6.1 | appName |
| TP/oneM2M/AE/DMR/UPD/001_POA | TS-0004 [2], clause 7.4.6.1 | pointOfAccess |
| TP/oneM2M/AE/DMR/UPD/001_OR | TS-0004 [2], clause 7.4.6.1 | ontologyRef |
| TP/oneM2M/AE/DMR/UPD/001_NL | TS-0004 [2], clause 7.4.6.1 | nodeLink |
| TP/oneM2M/AE/DMR/UPD/001_RR | TS-0004 [2], clause 7.4.6.1 | requestReachability |
| TP/oneM2M/AE/DMR/UPD/001_CSZ | TS-0004 [2], clause 7.4.6.1 | contentSerialization |

###### TP/oneM2M/AE/DMR/UPD/002

| TP Id | TP/oneM2M/AE/DMR/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an UPDATE Request with the value of the attribute ATTRIBUTE_NAME of the <container> resource |  |
| Reference | TS-0004 [2], clause 7.4.7.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered containing the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform UPDATE operation on resource CONTAINER_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     | the IUT is triggered to send a valid UPDATE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | container resourcecontaining  | |
                     | valid_ATTRIBUTE_NAME_ attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid UPDATE Request containing  | IUT => CSE |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resourcecontaining  | |
                     | valid_ATTRIBUTE_NAME_ attribute  | |
                     |} | |
+———————-+————————————————————————————————————–+————-+

| TP Id | Reference | ATTRIBUTE_NAME |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/UPD/002_ET | TS-0004 [2], clause 7.4.7.2.3 | expirationTime |
| TP/oneM2M/AE/DMR/UPD/002_LBL | TS-0004 [2], clause 7.4.7.2.3 | labels |
| TP/oneM2M/AE/DMR/UPD/002_MNI | TS-0004 [2], clause 7.4.7.2.3 | maxNrOfInstances |
| TP/oneM2M/AE/DMR/UPD/002_MBS | TS-0004 [2], clause 7.4.7.2.3 | maxByteSize |
| TP/oneM2M/AE/DMR/UPD/002_MIA | TS-0004 [2], clause 7.4.7.2.3 | maxInstanceAge |

##### 7.2.1.3.3	RETRIEVE Operation

###### TP/oneM2M/AE/DMR/RET/001

| TP Id | TP/oneM2M/AE/DMR/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a RETRIEVE Request on the TARGET_RESOURCE_ADDRESS to CSE |  |
| Reference | TS-0001 [1], claus e 10.1.2, TS-0004 [2], clause 7.2.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE. |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the IUT having privileges to perform RETRIEVE operation on resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     |the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------------------+-------------+

- |then {   |  |
                     | the IUT sends a valid RETRIEVE Request containing  |IUT => CSE |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+———————————————————————————————+————-+

| TP Id | Reference | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/AE/DMR/RET/001_CB | TS-0001 [1], clause 10.2.3.2 | 5 (CSEBase) |
| TP/oneM2M/AE/DMR/RET/001_AE | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/AE/DMR/RET/001_CNT | TS-0001 [1], clause 10.2.4.2 | 3 (Container) |

###### TP/oneM2M/AE/DMR/RET/002

| TP Id | TP/oneM2M/AE/DMR/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a RETRIEVE Request of oldest virtual resource |  |
| Reference | TS-0001 [1], clause 10.2.23.1, TS-0004 [2], clause 7.4.29.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     |the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/oldest  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid RETRIEVE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/oldest and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

###### TP/oneM2M/AE/DMR/RET/003

| TP Id | TP/oneM2M/AE/DMR/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a RETRIEVE Request of latest virtual resource |  |
| Reference | TS-0001 [1], clause 10.2.22.1, TS-0004 [2], clause 7.4.28.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     |the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/latest  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid RETRIEVE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/latest and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

###### TP/oneM2M/AE/DMR/RET/004

| TP Id | TP/oneM2M/AE/DMR/RET/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <container> resource RETRIEVE Request to CSE |  |
| Reference | TS-0004 [2], clause 7.4.7.2.2 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform RETRIEVE operation on container resource } |  |

- +----------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     |the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------+-------------+

- |then {   |  |
                     | the IUT sends a valid RETRIEVE Request containing  |IUT => CSE |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+—————————————————————————-+————-+

###### TP/oneM2M/AE/DMR/RET/005

| TP Id | TP/oneM2M/AE/DMR/RET/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <contentInstance> resource RETRIEVE Request to CSE |  |
| Reference | TS-0004 [2], clause 7.4.8.2.2 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform RETRIEVE operation on contentInstance resource } |  |

- +-----------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     |the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to CONTENTINSTANCE_RESOURCE_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------+-------------+

- |then {   |  |
                     | the IUT sends a valid RETRIEVE Request containing  |IUT => CSE |
                     | To set to CONTENTINSTANCE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+———————————————————————————–+————-+

##### 7.2.1.3.4	DELETE Operation

###### TP/oneM2M/AE/DMR/DEL/001

| TP Id | TP/oneM2M/AE/DMR/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a DELETE Request of oldest virtual resource |  |
| Reference | TS-0001 [1], clause 10.2.23.2, TS-0004 [2], clause 7.4.29.2.5 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     |the IUT is triggered to send a valid DELETE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/oldest  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid DELETE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/oldest and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

###### TP/oneM2M/AE/DMR/DEL/002

| TP Id | TP/oneM2M/AE/DMR/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a DELETE Request of latest virtual resource |  |
| Reference | TS-0001 [1], clause 10.2.22.2, TS-0004 [2], clause 7.4.28.2.5 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid DELETE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/oldest  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid DELETE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/latest and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

###### TP/oneM2M/AE/DMR/DEL/003

| TP Id | TP/oneM2M/AE/DMR/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <container> resource DELETE request to CSE |  |
| Reference | TS-0004 [2], clause 7.4.7.2.4 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and and the IUT being switched on and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUThaving privileges to perform DELETE operation on container resource } |  |

- +----------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid DELETE Request containing | |
                     | To set to CONTAINER_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid DELETE Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+—————————————————————————-+————-+

###### TP/oneM2M/AE/DMR/DEL/004

| TP Id | TP/oneM2M/AE/DMR/DEL/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <contentInstance> resource DELETE request to CSE |  |
| Reference | TS-0004 [2], clause 7.4.8.2.4 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the CSE having created a container resource and the CSE having created a contentInstance resource under container resource and the IUT having privileges to perform DELETE operation on contentInstance resource } |  |

- +---------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid DELETE Request containing  | |
                     | To set to CONTENTINSTANCE_RESOURCE_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid DELETE Request containing  | |
                     | To set to CONTENTINSTANCE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |
+———————-+———————————————————————————+————-+

#### 7.2.1.4	Subscription and Notification (SUB)

##### 7.2.1.4.1	CREATE Operation

###### TP/oneM2M/AE/SUB/CRE/001

| TP Id | TP/oneM2M/AE/SUB/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a subscription creation request |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.1, TS-0004 [2], clause 7.2.2.1 and 7.4.9.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched off and the IUThaving privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | Toset to AE_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     |subscription resource representation  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | Toset to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |subscription resource representation  | |
                     |} | |
+———————-+——————————————————————————————————————-+————-+

###### TP/oneM2M/AE/SUB/CRE/002

| TP Id | TP/oneM2M/AE/SUB/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a subscription creation request with optional attribute ATTRIBUTE_NAME when it is triggered |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.11.2, TS-0004 [2], clause 7.2.2.1 and 7.4.9.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUThaving privileges to perform CREATE operation on resource AE_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | notificationURI attribute and  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | notificationURI attribute and  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |
+———————-+——————————————————————————————————————–+————-+

| TP Id | PICS Selection | Reference | ATTRIBUTE_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/AE/SUB/CRE/002_ACPI | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.9.1 | accessControlPolicyIDs |
| TP/oneM2M/AE/SUB/CRE/002_ENC | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.9.1 | eventNotificationCriteria |
| TP/oneM2M/AE/SUB/CRE/002_NCT | PICS_SUB_NCT | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.9.1 | notificationContentType |
| TP/oneM2M/AE/SUB/CRE/002_NEC | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.9.1 | notificationEventCat |
| TP/oneM2M/AE/SUB/CRE/002_SU | PICS_SUB_SU | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.9.1 | subscriberURI |

###### TP/oneM2M/AE/SUB/CRE/003

| TP Id | TP/oneM2M/AE/SUB/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <group>/fopt subscription creation request with optional attributes notificationForwardingURI and groupID when it is triggered |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.7.10, TS-0004 [2], clause 7.4.14.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand and the IUT having a <group> resource as TARGET_RESOURCE_ADDRESS containing members which are subscribable the IUThaving privileges to perform CREATE operation on <group> members } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | groupID, notificationURI attribute and  | |
                     | notificationForwardingURI attribute set  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | groupID, notificationURI attribute and  | |
                     | notificationForwardingURI attribute set  | |
                     |} | |
+———————-+———————————————————————————————————————————————–+————-+

##### 7.2.1.4.2	NOTIFY Operation

###### TP/oneM2M/AE/SUB/NTF/001

| TP Id | TP/oneM2M/AE/SUB/NTF/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify Response to the hosting CSE when receiving a Notify request containing a single notification |  |
| Reference | TS-0001 [1], clause 6.1.12, TS-0004 [2], clause 7.5.1.2 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT having been registered and the IUT having created subscription resource under the CSE and the IUT being reachable through a URL ACCESSIBLE_URL } |  |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE |
                     | the IUT receives a NOTIFY Request from hosting CSE containing  | |
                     | Content containing  | |
                     | notification message representation  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid NOTIFY Response to the hosting CSE containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————–+————-+

###### TP/oneM2M/AE/SUB/NTF/002

| TP Id | TP/oneM2M/AE/SUB/NTF/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify Response to the hosting CSE when receiving a Notify request containing aggregated notifications |  |
| Reference | TS-0001 [1], clause 6.2.12, TS-0004 [2], clause 7.5.1.2 |  |
| Config Id | CF03 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT having been registered and the IUT having created subscription resource at <group>/fopt with notificationForwardingURI set and the IUT being reachable through a URL ACCESSIBLE_URL } |  |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE |
                     | the IUT receives a NOTIFY Request from hosting CSE containing  | |
                     | Content containing  | |
                     |aggregatedNotification message representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid Response to the hosting CSE containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+——————————————————————————————————————–+————-+

#### 7.2.1.5	Communication Management and Delivery Handling (CMDH)

##### 7.2.1.5.1	Resource pollingChannel (PCH)

###### TP/oneM2M/AE/PCH/001

| TP Id | TP/oneM2M/AE/PCH/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which performs polling sends the Notify request to <pollingChannelURI> Hosting CSE after receiving response using polling channel |  |
| Reference | TS-0001 [1], clause 10.2.13.8 & TS-0004 [2], clause 7.4.22.2.5 |  |
| Config Id | CF03 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT be ing in the "initial state" and the IUT having registered to a CSE1 and the IUT having a <pollingChannel> resource and the IUT having sent a retrieve Request to the CSE To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and the CSE having re ceived the Request from Originator containing To set to AE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a polling Response from the CSE containing  | |
                     |Response Status Code set to 2000 (OK)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a NOTIFY Request to the CSE containing  | |
                     | To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |

| Note: Based on figure 10.2.13.1-1 in TS-0001[1] step 004 and 005 |  |  |
| --- | --- | --- |

###### TP/oneM2M/AE/PCH/002

| TP Id | TP/oneM2M/AE/PCH/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an RETRIEVE Request to <pollingChannelURI> resource See the Note below |  |
| Reference | TS-0001 [1], clause 10.2.13.6, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the AEhaving a child <pollingChannel> resource } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to POLLINGCHANNELURI _RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid RETRIEVE Request c ontaining  | |
                     | To set to POLLINGCHANNELURI _RESOURCE_ADDRESS  | |
                     |From set to AE_ID  | |
                     |} | |

| Note: Based on figure 10.2.5.12-1 in TS-0001[1] step 001. |  |  |
| --- | --- | --- |

#### 7.2.1.6	Ontology based Interworking IPE (OBI)

##### 7.2.1.6.1	Initialization sequence

TP/oneM2M/AE/OBI/INI/001

| TP Id | TP/oneM2M/AE/OBI/INI/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT starts the Initialization sequence of an Ontology based Interworking IPE and retrieves the formal description of the ontology |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.16, TS-0004 [2], clause 7.2.2.1 and 7.4.37, TS-0030 [8], clause 6.2.1.2 item 1. |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| Initial conditions | with { the IUT being registeredand the IUT being in the "initial state" and the IUThaving privileges to perform RETRIEVE operation on formal description of the ontology at AE_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   |NA |
                     | the IUT is triggered to send a request to get the formal description of the Ontology  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |Ontology server <= IUT|
                     | the IUT sends a valid HTTP GET Request to Ontology server containing  | |
                     | anURI  | |
                     | indicating the address where to get the Ontology  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————-+———————-+

##### 7.2.1.6.2	CREATE Operation

TP/oneM2M/AE/OBI/CRE/001

| TP Id | TP/oneM2M/AE/OBI/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an AE initial registration request when it is started |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case C, and clause 9.6.19, TS-0030 [8], clause 6.2.1.2 |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE_IPE |  |
| Initial conditions | with { the IUT never being registered and the IUT being in the "initial state" and the IUThaving got a valid APP-ID } |  |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | Toset to CSE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 2 (AE) and  | |
                     | Content containing  | |
                     |AE resource representation  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request to CSE containing  | |
                     | Resource Type set to 2 (AE) and  | |
                     | To set to CSE_RESOURCE_ADDRESS and  | |
                     | From set to empty and  | |
                     | Content containing  | |
                     |AE-IPE resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————-+————-+

##### 7.2.1.6.3	RETRIEVE Operation

TP/oneM2M/AE/OBI/RET/001

| TP Id | TP/oneM2M/AE/OBI/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request resource and responds 2000 (OK) status |  |
| Reference | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2, TS-0030 [8], clause 6.2.2.2.1. |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE_IPE |  |
| Initial conditions | with { the IUT being registeredand the IUT being in the "initial state" and the IUThaving retrieved and parsed the formal description of the ontology and the IUThaving performed DISCOVERY of non-oneM2M devices } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid RETRIEVE Request from CSE containing  | |
                     | To set to AE_IPE_ID and  | |
                     | From set to CSE_RESOURCE_ADDRESS and  | |
                     | no Content   | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Resource Type set to 28 (flexContainer) and   | |
                     | Content containing  | |
                     | Non-oneM2M AE resource representation as a specialization of <flexContainer>  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

TP/oneM2M/AE/OBI/RET/002

| TP Id | TP/oneM2M/AE/OBI/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request resource with the optional ATTRIBUTE |  |
| Reference | TS-0001 [1], clause 10.2.3.2and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2, TS-0030 [8], clause 6.2.2.2.1. |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE_IPE |  |
| Initial conditions | with { the IUT being registeredand the IUT being in the "initial state" and the IUThaving retrieved and parsed the formal description of the ontology and the IUThaving performed DISCOVERY of non-oneM2M devices } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Resource Type set to 28 (flexContainer) and   | |
                     | Content containing  | |
                     | Non-oneM2M resource as a specialization of <flexContainer> containing  | |
                     | valid ATTRIBUTE attribute  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

TP/oneM2M/AE/OBI/RET/003

| TP Id | TP/oneM2M/AE/OBI/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT has performed DISCOVERY of non-oneM2M devices during initialization sequence |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.16, TS-0004 [2], clause 7.2.2.1 and 7.4.37, TS-0030 [8], clause 6.2.1.2 item 5. |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE_IPE, PICS_AE_IPE_DISCOVERY |  |
| Initial conditions | with { the IUT being registeredand the IUT being in the "initial state" and the IUThaving retrieved and parsed the formal description of the ontology and the IUThaving performed DISCOVERY of non-oneM2M devices and the IUT having discovered a non-oneM2M device with a valid address value indicating NON_ONEM2M_AE_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid RETRIEVE Request from CSE containing  | |
                     | To set to NON_ONEM2M_AE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | non-oneM2M devices resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————-+————-+

TP/oneM2M/AE/OBI/RET/004

| TP Id | TP/oneM2M/AE/OBI/RET/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT continues to discover the devices in the non-oneM2M area |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.16, TS-0004 [2], clause 7.2.2.1 and 7.3.3.14, TS-0030 [8], clause 6.2.2.1. |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE_IPE, PICS_AE_IPE_DISCOVERY |  |
| Initial conditions | with { the IUT being registeredand the IUT being in the "initial state" and the IUThaving retrieved and parsed the formal description of the ontology and the IUThaving performed DISCOVERY of non-oneM2M devices and the IUT having discovered a new non-oneM2M device with a valid address value indicating NON_ONEM2M_NEW_AE_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid RETRIEVE Request from CSE containing  | |
                     | To set to NON_ONEM2M_NEW_AE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | non-oneM2M devices resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————–+————-+

TP/oneM2M/AE/OBI/RET/005

| TP Id | TP/oneM2M/AE/OBI/RET/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT continues to discover the devices in the non-oneM2M area |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.16, TS-0004 [2], clause 7.2.2.1 and 7.3.3.14, TS-0030 [8], clause 6.2.2.1. |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE_IPE, PICS_AE_IPE_DISCOVERY |  |
| Initial conditions | with { the IUT being registeredand the IUT being in the "initial state" and the IUThaving retrieved and parsed the formal description of the ontology and the IUThaving performed DISCOVERY of non-oneM2M devices and An existing non-oneM2M (NON_ONEM2M_NEW_AE_RESOURCE_ADDRESS) device was switched off } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid RETRIEVE Request from CSE containing  | |
                     | To set to NON_ONEM2M_NEW_AE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————–+————-+

TP/oneM2M/AE/OBI/RET/006

| TP Id | TP/oneM2M/AE/OBI/RET/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts "on demand" discovery |  |
| Reference | TS-0001 [1], clause 10.1.1.1 and 10.2.4.16, TS-0004 [2], clause 7.2.2.1 and 7.3.3.14, TS-0030 [8], clause 6.2.2.1. |  |
| Config Id | CF07 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE_IPE, PICS_AE_IPE_DISCOVERY |  |
| Initial conditions | with { the IUT being registeredand the IUT being in the "initial state" and the IUThaving retrieved and parsed the formal description of the ontology and the IUThaving performed DISCOVERY of non-oneM2M devices } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to IPE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 28 (flexContainer) and   | |
                     | From set to CSE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsageset to 1 (IPE On-demand Discovery)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————–+————-+

#### 7.2.1.7	Home Appliance Information Model and Mapping (HAIM)

##### 7.2.1.7.1	CREATE Operation

TP/oneM2M/AE/HAIM/CRE/001

| TP Id | TP/oneM2M/AE/HAIM/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates a [_ _HAIM_DEVICE] resource as a specialization of the <flexContainer> resource on the hosting CSE using the appropriate 'DeviceClass ID' |  |
| Reference | TS-0001 [1], clause 9.6.1.2.2.3, and TS-0023 [7], clauses 5.4, 6.2.2 and 6.4.2 |  |
| Config Id | CF03 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS and the IUT exposing a controlling interface for a HAIM_DEVICE } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     | Content set to [HAIM_DEVICE] resource  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | Content containing  | |
                     | [HAIM_DEVICE] resource as a specialization of <flexContainer> containing  | |
                     | containerDefinition attributeset to  | |
                     | "org.onem2m.home.device._HAIM_DEVICE"_  | |
                     |} | |
+———————-+————————————————————————————————————————————————————-+————-+

| TP Id | HAIM_DEVICE |
| --- | --- |
| TP/oneM2M/AE/HAIM/CRE/001_AIR | deviceAirConditioner |
| TP/oneM2M/AE/HAIM/CRE/001_CLO | deviceClothesWasher |
| TP/oneM2M/AE/HAIM/CRE/001_ELE | deviceElectricVehicleCharger |
| TP/oneM2M/AE/HAIM/CRE/001_LIG | deviceLight |
| TP/oneM2M/AE/HAIM/CRE/001_MIC | deviceMicrogeneration |
| TP/oneM2M/AE/HAIM/CRE/001_OVE | deviceOven |
| TP/oneM2M/AE/HAIM/CRE/001_REF | deviceRefrigerator |
| TP/oneM2M/AE/HAIM/CRE/001_ROB | deviceRobotCleaner |
| TP/oneM2M/AE/HAIM/CRE/001_SMA | deviceSmartElectricMeter |
| TP/oneM2M/AE/HAIM/CRE/001_STO | deviceStorageBattery |
| TP/oneM2M/AE/HAIM/CRE/001_TEL | deviceTelevision |
| TP/oneM2M/AE/HAIM/CRE/001_THE | deviceThermostat |
| TP/oneM2M/AE/HAIM/CRE/001_WAT | deviceWaterHeater |

TP/oneM2M/AE/HAIM/CRE/002

| TP Id | TP/oneM2M/AE/HAIM/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates a [_ _HAIM_MODULE] resource as a specialization of the <flexContainer> resource and as a child of [HAIM_DEVICE] resouce on the hosting CSE using the appropriate ModuleClass ID |  |
| Reference | TS-0001 [1], clause 9.6.1.2.2.3, and TS-0023 [7], clauses 5.3, 5.4, 6.2.2 and 6.4.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS and the IUT exposing a controlling interface for a HAIM_DEVIC E } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     | Content set to [HAIM_DEVICE] resource  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | Content containing  | |
                     | [HAIM_DEVICE] resource containing  | |
                     | [HAIM_MODULE] resource as a specialization of  | |
                     | <flexContainer> as an instance of the HAIM_MODULE  | |
                     | ModuleClass containing  | |
                     | containerDefinition attributeset to  | |
                     | "org.onem2m.home. moduleclass._HAIM_MODULE"_  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————+————-+

| TP Id | HAIM_MODULE | HAIM_DEVICE |
| --- | --- | --- |
| TP/oneM2M/AE/HAIM/CRE/002_SEL | binarySwitch | deviceElectricVehicleCharger |
| TP/oneM2M/AE/HAIM/CRE/002_SLI | binarySwitch | deviceLight |
| TP/oneM2M/AE/HAIM/CRE/002_SWA | binarySwitch | deviceWaterHeater |
| TP/oneM2M/AE/HAIM/CRE/002_FEL | faultDetection | deviceElectricVehicleCharger |
| TP/oneM2M/AE/HAIM/CRE/002_REL | runState | deviceElectricVehicleCharger |
| TP/oneM2M/AE/HAIM/CRE/002_BEL | battery | deviceElectricVehicleCharger |
| TP/oneM2M/AE/HAIM/CRE/002_BST | battery | deviceStorageBattery |
| TP/oneM2M/AE/HAIM/CRE/002_VEL | electricVehicleConnector | deviceElectricVehicleCharger |
| TP/oneM2M/AE/HAIM/CRE/002_GMI | energyGeneration | deviceMicrogeneration |
| TP/oneM2M/AE/HAIM/CRE/002_CSM | energyConsumption | deviceSmartElectricMeter |
| TP/oneM2M/AE/HAIM/CRE/002_TTH | temperature | deviceThermostat |

TP/oneM2M/AE/HAIM/CRE/003

| TP Id | TP/oneM2M/AE/HAIM/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates the appropriate Properties, according to the HAIM_DEVICE model, as [customAttribute] attributes of a [deviceInfo] resource which is a child-resource of the <node> resource linked by the nodelink attribute of the HAIM_DEVICE model |  |
| Reference | TS-0001 [1], clause 9.6.1.2.2.3, and TS-0023 [7], clauses 5.4, 5.6 and 6.2.5 |  |
| Config Id | CF03 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS and the IUT exposing a controlling interface for a HAIM_DEVICE } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     | Content set to [HAIM_DEVICE] resource  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | Content containing  | |
                     | [HAIM_DEVICE] resource containing  | |
                     | nodelink attribute containing  | |
                     | link to a <node> resource on the same hosting CSE containing  | |
                     | [deviceInfo] resource  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | HAIM_DEVICE |
| --- | --- |
| TP/oneM2M/AE/HAIM/CRE/003_AIR | deviceAirConditioner |
| TP/oneM2M/AE/HAIM/CRE/003_CLO | deviceClothesWasher |
| TP/oneM2M/AE/HAIM/CRE/003_ELE | deviceElectricVehicleCharger |
| TP/oneM2M/AE/HAIM/CRE/003_LIG | deviceLight |
| TP/oneM2M/AE/HAIM/CRE/003_MIC | deviceMicrogeneration |
| TP/oneM2M/AE/HAIM/CRE/003_OVE | deviceOven |
| TP/oneM2M/AE/HAIM/CRE/003_REF | deviceRefrigerator |
| TP/oneM2M/AE/HAIM/CRE/003_ROB | deviceRobotCleaner |
| TP/oneM2M/AE/HAIM/CRE/003_SMA | deviceSmartElectricMeter |
| TP/oneM2M/AE/HAIM/CRE/003_STO | deviceStorageBattery |
| TP/oneM2M/AE/HAIM/CRE/003_TEL | deviceTelevision |
| TP/oneM2M/AE/HAIM/CRE/003_THE | deviceThermostat |
| TP/oneM2M/AE/HAIM/CRE/003_WAT | deviceWaterHeater |

TP/oneM2M/AE/HAIM/CRE/004

| TP Id | TP/oneM2M/AE/HAIM/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates the appropriate Properties, according to the HAIM_MODULE model, as [customAttribute] attributes of the [HAIM_MODULE] resource on the hosting CSE, with [customAttribute] name set to the prefix 'prop' plus the Property name |  |
| Reference | TS-0001 [1], clause 9.6.1.2.2.3, and TS-0023 [7], clauses 5.3 and 6.2.5 |  |
| Config Id | CF03 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS and the IUT exposing a controlling interface for a HAIM_DEVICE } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     | Content set to [HAIM_DEVICE] resource  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |CSE <= IUT |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | [HAIM_DEVICE ] resource containing  | |
                     | [HAIM_MODULE ] resource as a specialization of  | |
                     | <flexContainer> containing  | |
                     | appropriate [customAttribute] attributes with name set to  | |
                     | "prop_"_ prefix plus the Property name  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | HAIM_DEVICE | HAIM_MODULE |
| --- | --- | --- |
| TP/oneM2M/AE/HAIM/CRE/004_VBT | deviceElectricVehicleCharger | battery |
| TP/oneM2M/AE/HAIM/CRE/004_SBT | deviceStorageBattery | battery |
| TP/oneM2M/AE/HAIM/CRE/004_VCN | deviceElectricVehicleCharger | electricVehicleConnector |

TP/oneM2M/AE/HAIM/CRE/005

| TP Id | TP/oneM2M/AE/HAIM/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates the appropriate DataPoints, according to the HAIM_MODULE model, as [customAttribute] attributes of the [HAIM_MODULE] resource on the hosting CSE, with [customAttribute] name set to the Datapoint name |  |
| Reference | TS-0001 [1], clause 9.6.1.2.2.3, and TS-0023 [7], clauses 5.3 and 6.2.6 |  |
| Config Id | CF03 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS and the IUT exposing a controlling interface for a HAIM_DEVICE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     | Content set to [HAIM_DEVICE] resource  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |CSE <= IUT |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | [HAIM_DEVICE ] resource containing  | |
                     | [HAIM_MODULE ] resource as a specialization of  | |
                     | <flexContainer> containing  | |
                     | appropriate [customAttribute] attributes with name set to  | |
                     | the DataPoint name  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————+————-+

| TP Id | HAIM_DEVICE | HAIM_MODULE |
| --- | --- | --- |
| TP/oneM2M/AE/HAIM/CRE/005_ACT | deviceAirConditioner | temperature |
| TP/oneM2M/AE/HAIM/CRE/005_ACU | deviceAirConditioner | turbo |
| TP/oneM2M/AE/HAIM/CRE/005_ACW | deviceAirConditioner | airflow |
| TP/oneM2M/AE/HAIM/CRE/005_CWS | deviceClothesWasher | spinLevel |
| TP/oneM2M/AE/HAIM/CRE/005_CWW | deviceClothesWasher | waterFlow |
| TP/oneM2M/AE/HAIM/CRE/005_VCB | deviceElectricVehicleCharger | battery |
| TP/oneM2M/AE/HAIM/CRE/005_VCE | deviceElectricVehicleCharger | electricVehicleConnector |
| TP/oneM2M/AE/HAIM/CRE/005_LBR | deviceLight | brightness |
| TP/oneM2M/AE/HAIM/CRE/005_LCL | deviceLight | colour |
| TP/oneM2M/AE/HAIM/CRE/005_LCS | deviceLight | colourSaturation |
| TP/oneM2M/AE/HAIM/CRE/005_LFD | deviceLight | faultDetection |
| TP/oneM2M/AE/HAIM/CRE/005_RDS | deviceRefrigerator | doorStatus |
| TP/oneM2M/AE/HAIM/CRE/005_RPS | deviceRefrigerator | powerSave |
| TP/oneM2M/AE/HAIM/CRE/005_EEC | deviceSmartElectricMeter | energyConsumption |
| TP/oneM2M/AE/HAIM/CRE/005_TVA | deviceTelevision | mediaInput |
| TP/oneM2M/AE/HAIM/CRE/005_TVV | deviceTelevision | audioVolume |
| TP/oneM2M/AE/HAIM/CRE/005_TVB | deviceTelevision | binarySwitch |
| TP/oneM2M/AE/HAIM/CRE/005_TVC | deviceTelevision | televisionChannel |
| TP/oneM2M/AE/HAIM/CRE/005_WHB | deviceWaterHeater | boiler |
| TP/oneM2M/AE/HAIM/CRE/005_WHC | deviceWaterHeater | clock |
| TP/oneM2M/AE/HAIM/CRE/005_WHH | deviceWaterHeater | hotWaterSupply |
| TP/oneM2M/AE/HAIM/CRE/005_WHR | deviceWaterHeater | runState |

TP/oneM2M/AE/HAIM/CRE/006

| TP Id | TP/oneM2M/AE/HAIM/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates the child Action module resources according to the SDT_MODULE model |  |
| Reference | TS-0001 [1], clause 9.6.1.2.2.3, and TS-0023 [7], clauses 5.3, 5.4, 5.5, 6.2, 6.4 |  |
| Config Id | CFG03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_??? |  |
| Initial conditions | with { the IUT being registeredand the IUT being switched on and the IUThaving privileges to perform CREATE operation on resource TARGET_RESOURCE_ADDRESS and the IUT having created a resource at SDT_DEVICE_ADDRESS of type SDT_DEVICE containing child SDT_MODULE resource at SDT_MODULE_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                      | } | |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                      | the IUT sends a valid CREATE Request containing  | |
                      | To set to SDT_MODULE _RESOURCE_ADDRESS and  | |
                      | Content containing  | |
                      | ACTION_NAME resource as a specialization of  | |
                      | <flexContainer> containing  | |
                      | CUSTOM_ATTRIBUTES attributes and  | |
                      | containerDefinition attributeset to  | |
                      | "org.onem2m._DOMAIN_ .action._ACTION_NAME_ " and  | |
                      | resourceName attributeset to ACTION_NAME  | |
                      |} | |
+———————–+—————————————————————————————————————-+————-+

| TP Id | DOMAIN | HAIM**_DEVICE** | CUSTOM_ ATTRIBUTES (of action) | HAIMMODULE(parent resource) | ACTION_NAME |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/AE/HAIM/CRE/006_STPR | Common | device3DPrinter | none | 3Dprinter | start3Dprint |
| TP/oneM2M/AE/HAIM/CRE/006_SPPR | Common | device3DPrinter | none | 3Dprinter | stop3Dprint |
| TP/oneM2M/AE/HAIM/CRE/006_UPV | Common | deviceAudioReceiver,device3DDisplay,deviceSetTopBox,deviceTelevision,deviceHandheldPTTTerminal,deviceTrainborneTerminal,deviceCardRecharingMachine,deviceSmartGate,deviceSmartScreenDoor | none | audioVolume | upVolume |
| TP/oneM2M/AE/HAIM/CRE/006_DWV | Common | deviceAudioReceiver,device3DDisplay,deviceSetTopBox,deviceTelevision,deviceHandheldPTTTerminal,deviceTrainborneTerminal,deviceCardRecharingMachine,deviceSmartGate,deviceSmartScreenDoor | none | audioVolume | downVolume |
| TP/oneM2M/AE/HAIM/CRE/006_TBS | Common | device3DPrinter, deviceAudioReceiver,deviceLight,deviceMultiFunctionPrinter,devicePrinter,deviceScanner,deviceSmartPlug,deviceSwitch,device3DDisplay,device3DScanner,subDeviceCuff,subDevicePowerOutlet | none | binarySwitch | toggle |
| TP/oneM2M/AE/HAIM/CRE/006_TBS | City | deviceOutdoorLamp,deviceSmartElectricMeter,deviceSmartGasMeter,deviceSmartWaterMeter | none | binarySwitch | toggle |
| TP/oneM2M/AE/HAIM/CRE/006_TBS | Health | deviceBloodPressureMonitor | none | binarySwitch | toggle |
| TP/oneM2M/AE/HAIM/CRE/006_TBS | Home | deviceAirConditioner,deviceAirPurifier,deviceClothesDryer,deviceClothesWasher,deviceClothesWasherDryer,deviceCoffeeMachine,deviceCookerHood,deviceDehumidifier,deviceDishWasher,deviceFan,deviceHomeCCTV,deviceHumidifier,deviceKettle,deviceMicrogeneration,deviceOven,deviceRefrigerator,deviceRiceCooker,deviceRobotCleaner,deviceSetTopBox,deviceSteamCloset,deviceStorageBattery,deviceTelevision,deviceWaterHeater,deviceBottleWarmer,deviceGarbageDisposal,deviceWaterPurifier | none | binarySwitch | toggle |
| TP/oneM2M/AE/HAIM/CRE/006_TBS | Vehicular | deviceElectricVehicleCharger | none | binarySwitch | toggle |
| TP/oneM2M/AE/HAIM/CRE/006_TBS | Railway | deviceHandheldPTTTerminal,deviceCardRecharingMachine,deviceSmartGate,deviceSmartScreenDoor | none | binarySwitch | toggle |
| TP/oneM2M/AE/HAIM/CRE/006_DNV | Common | subDevicePowerOutlet | none | numberValue | decrementNumberValue |
| TP/oneM2M/AE/HAIM/CRE/006_INV | Common | subDevicePowerOutlet | none | numberValue | incrementNumberValue |
| TP/oneM2M/AE/HAIM/CRE/006_RNV | Common | subDevicePowerOutlet | none | numberValue | resetNumberValue |
| TP/oneM2M/AE/HAIM/CRE/006_OOL | Common | deviceDoor,waterValve | none | openLevel | open |
| TP/oneM2M/AE/HAIM/CRE/006_OOL | Home | deviceWindowShade | none | openLevel | open |
| TP/oneM2M/AE/HAIM/CRE/006_COL | Common | deviceDoor,waterValve | none | openLevel | close |
| TP/oneM2M/AE/HAIM/CRE/006_COL | Home | deviceWindowShade | none | openLevel | close |
| TP/oneM2M/AE/HAIM/CRE/006_CPC | Railway | deviceHandheldPTTTerminal,deviceTrainborneTerminal | none | phoneCall | call |
| TP/oneM2M/AE/HAIM/CRE/006_APC | Railway | deviceHandheldPTTTerminal,deviceTrainborneTerminal | none | phoneCall | answer |
| TP/oneM2M/AE/HAIM/CRE/006_HPC | Railway | deviceHandheldPTTTerminal,deviceTrainborneTerminal | none | phoneCall | hangup |
| TP/oneM2M/AE/HAIM/CRE/006_NTPC | Common | deviceCamera | none | playerControl | nextTrack |
| TP/oneM2M/AE/HAIM/CRE/006_NTPC | Home | deviceHomeCCTV,deviceTelevision | none | playerControl | nextTrack |
| TP/oneM2M/AE/HAIM/CRE/006_PTPC | Common | deviceCamera | none | playerControl | previousTrack |
| TP/oneM2M/AE/HAIM/CRE/006_PTPC | Home | deviceHomeCCTV,deviceTelevision | none | playerControl | previousTrack |
| TP/oneM2M/AE/HAIM/CRE/006_MSS | Common | deviceGenericSensor | none | smokeSensor | mute |
| TP/oneM2M/AE/HAIM/CRE/006_MSS | Home | deviceHomeCCTV | none | smokeSensor | mute |
| TP/oneM2M/AE/HAIM/CRE/006_UCTC | Home | deviceSetTopBox,deviceTelevision | none | televisionChannel | upChannel |
| TP/oneM2M/AE/HAIM/CRE/006_DCTC | Home | deviceSetTopBoxdeviceTelevision | none | televisionChannel | downChannel |
| TP/oneM2M/AE/HAIM/CRE/006_RTM | Common | deviceServiceButton | none | textMessage | resetTextMessage |
| TP/oneM2M/AE/HAIM/CRE/006_ACT | Common | deviceLight,deviceThermostat | none | timer | activateClockTimer |
| TP/oneM2M/AE/HAIM/CRE/006_ACT | City | deviceOutdoorLamp | none | timer | activateClockTimer |
| TP/oneM2M/AE/HAIM/CRE/006_ACT | Home | deviceAirConditioner,deviceAirPurifier,deviceClothesWasher,deviceClothesWasherDryer,deviceDehumidifier,deviceOven,deviceRiceCooker,deviceRobotCleaner,deviceWindowShade,deviceWaterPurifier | none | timer | activateClockTimer |
| TP/oneM2M/AE/HAIM/CRE/006_DCT | Common | deviceLight,deviceThermostat | none | timer | deactivateClockTimer |
| TP/oneM2M/AE/HAIM/CRE/006_DCT | City | deviceOutdoorLamp | none | timer | deactivateClockTimer |
| TP/oneM2M/AE/HAIM/CRE/006_DCT | Home | deviceAirConditioner,deviceAirPurifier,deviceClothesWasher,deviceClothesWasherD ryer,deviceDehumidifier,deviceOven,deviceRiceCooker,deviceRobotCleaner,deviceWindowShade,deviceWaterPurifier | none | timer | deactivateClockTimer |
| TP/oneM2M/AE/HAIM/CRE/006_RRG | City | deviceWeatherStation | none | rainGauge | reset |
| TP/oneM2M/AE/HAIM/CRE/006_RDMA | management | flexNode | rebootType | dmAgent | reboot |
| TP/oneM2M/AE/HAIM/CRE/006_DPDMA | management | flexNode | resut, name, version, url | dmAgent | deployPackage |
| TP/oneM2M/AE/HAIM/CRE/006_RDMIO | management | flexNode | resut, address | dmDataModelIO | readIO |
| TP/oneM2M/AE/HAIM/CRE/006_WDMIO | management | flexNode | resut, address, payload | dmDataModelIO | writeIO |
| TP/oneM2M/AE/HAIM/CRE/006_UDMF | management | flexNode | Result, url, version | dmFirmware | updateFirmware |
| TP/oneM2M/AE/HAIM/CRE/006_TDMF | management | flexNode | resut | dmFirmware | toggle |
| TP/oneM2M/AE/HAIM/CRE/006_ADMS | management | flexNode | none | dmSoftware | activate |
| TP/oneM2M/AE/HAIM/CRE/006_DDMS | management | flexNode | none | dmSoftware | deactivate |
| TP/oneM2M/AE/HAIM/CRE/006_RDMEL | management | flexNode | Start, end | dmEventLog | retrieveLog |
| TP/oneM2M/AE/HAIM/CRE/006_IDMP | management | flexNode | none | dmPackage | install |
| TP/oneM2M/AE/HAIM/CRE/006_UDMP | management | flexNode | none | dmPackage | uninstall |
| TP/oneM2M/AE/HAIM/CRE/006_UPDMP | management | flexNode | Version, url | dmPackage | update |
| TP/oneM2M/AE/HAIM/CRE/006_EDMC | management | flexNode | resut | dmCapability | enable |
| TP/oneM2M/AE/HAIM/CRE/006_DDMC | management | flexNode | resut | dmCapability | disable |
| TP/oneM2M/AE/HAIM/CRE/006_FDMS | management | flexNode | none | dmStorage | format |
| TP/oneM2M/AE/HAIM/CRE/006_UDMS | management | flexNode | none | dmStorage | unmount |

TP/oneM2M/AE/HAIM/CRE/007

| TP Id | TP/oneM2M/AE/HAIM/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds correctly when its ACTION_NAME resource is triggered. |  |
| Reference | TS-0001 [1], clause 9.6.1.2.2.3, and TS-0023 [7], clauses 5.3, 5.8 and 6.2.6 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered to CSE and the IUT exposing a controlling interface for an ACTION_NAME and the IUT having created on the CSE a SDT_MODULE at SDT_MODULE_ADDRESS SDT_MODULE containing DATA_POINT set to a valid value and the IUT having created on the CSE an ACTION_NAME at ACTION_NAME_ADDRESS the IUT having created on the CSE a <subscription> at ACTION_NAME_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE |
                      | The IUT receives a valid NOTIFY Request containing  | |
                      | operation set to UPDATE  | |
                      | subscriptionReference set to resource identifier of ACTION_NAME  | |
                      | containing valid ACTION_NAME representation containing  | |
                      | CUSTOM_ATTRIBUTE set to valid value  | |
                      |} | |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                      | the IUT sends a valid UPDATE Request containing  | |
                      | To set to SDT_MODULE_ADDRESS and  | |
                      | Content containing SDT_MODULE resource containing  | |
                      | a valid_DATA_POINT_ attribute  | |
                      | the IUT sends a valid NOTIFY Response containing  | |
                      | response status code 2000  | |
                      |} | |
+———————–+———————————————————————————————————–+————-+

| TP Id | CUSTOM_ ATTRIBUTES (of HAIM module or ACTION module) | **HAIM _MODULE(parent resource)** | ACTION_NAME |
| --- | --- | --- | --- |
| TP/oneM2M/AE/HAIM/UPD/007_STPR | none | 3Dprinter | start3Dprint |
| TP/oneM2M/AE/HAIM/UPD/007_SPPR | none | 3Dprinter | stop3Dprint |
| TP/oneM2M/AE/HAIM/UPD/006_UPV | volumePercentage | audioVolume | upVolume |
| TP/oneM2M/AE/HAIM/UPD/006_DWV | volumePercentage | audioVolume | downVolume |
| TP/oneM2M/AE/HAIM/UPD/006_TBS | state | binarySwitch | toggle |
| TP/oneM2M/AE/HAIM/UPD/006_DNV | numberValue | numberValue | decrementNumberValue |
| TP/oneM2M/AE/HAIM/UPD/006_INV | numberValue | numberValue | incrementNumberValue |
| TP/oneM2M/AE/HAIM/UPD/006_RNV | numberValue | numberValue | resetNumberValue |
| TP/oneM2M/AE/HAIM/UPD/006_OOL | openLevel | openLevel | open |
| TP/oneM2M/AE/HAIM/UPD/006_COL | openLevel | openLevel | close |
| TP/oneM2M/AE/HAIM/UPD/006_CPC | callState | phoneCall | call |
| TP/oneM2M/AE/HAIM/UPD/006_APC | callState | phoneCall | answer |
| TP/oneM2M/AE/HAIM/UPD/006_HPC | callState | phoneCall | hangup |
| TP/oneM2M/AE/HAIM/UPD/006_NTPC | none | playerControl | nextTrack |
| TP/oneM2M/AE/HAIM/UPD/006_PTPC | none | playerControl | previousTrack |
| TP/oneM2M/AE/HAIM/UPD/006_MSS | dismantled | smokeSensor | mute |
| TP/oneM2M/AE/HAIM/UPD/006_UCTC | channelNumber | televisionChannel | upChannel |
| TP/oneM2M/AE/HAIM/UPD/006_DCTC | channelNumber | televisionChannel | downChannel |
| TP/oneM2M/AE/HAIM/UPD/006_RTM | textMessage | textMessage | resetTextMessage |
| TP/oneM2M/AE/HAIM/UPD/006_ACT | none | timer | activateClockTimer |
| TP/oneM2M/AE/HAIM/UPD/006_DCT | none | timer | deactivateClockTimer |
| TP/oneM2M/AE/HAIM/UPD/006_RRG | height | rainGauge | reset |
| TP/oneM2M/AE/HAIM/UPD/006_RDMA | state | dmAgent | reboot |
| TP/oneM2M/AE/HAIM/UPD/006_DPDMA | state | dmAgent | deployPackage |
| TP/oneM2M/AE/HAIM/UPD/006_RDMIO | resut | dmDataModelIO | readIO |
| TP/oneM2M/AE/HAIM/UPD/006_WDMIO | resut | dmDataModelIO | writeIO |
| TP/oneM2M/AE/HAIM/UPD/006_UDMF | resut | dmFirmware | updateFirmware |
| TP/oneM2M/AE/HAIM/UPD/006_TDMF | resut, primaryState, secondaryState | dmFirmware | toggle |
| TP/oneM2M/AE/HAIM/UPD/006_ADMS | state | dmSoftware | activate |
| TP/oneM2M/AE/HAIM/UPD/006_DDMS | state | dmSoftware | deactivate |
| TP/oneM2M/AE/HAIM/UPD/006_RDMEL | data | dmEventLog | retrieveLog |
| TP/oneM2M/AE/HAIM/UPD/006_IDMP | state | dmPackage | install |
| TP/oneM2M/AE/HAIM/UPD/006_UDMP | state | dmPackage | uninstall |
| TP/oneM2M/AE/HAIM/UPD/006_UPDMP | state | dmPackage | update |
| TP/oneM2M/AE/HAIM/UPD/006_EDMC | resut, currentState | dmCapability | enable |
| TP/oneM2M/AE/HAIM/UPD/006_DDMC | resut, currentState | dmCapability | disable |
| TP/oneM2M/AE/HAIM/UPD/006_FDMS | status | dmStorage | format |
| TP/oneM2M/AE/HAIM/UPD/006_UDMS | mounts | dmStorage | unmount |

#### 7.2.1.8	Security (SEC)

##### 7.2.1.8.1	Authentication

##### 7.2.1.8.1.1	ESPrim (ESP)

###### 7.2.1.8.1.1.1	NOTIFY Operation

TP/oneM2M/AE/SEC/ESP/NTF/001

| TP Id | TP/oneM2M/AE/SEC/ESP/NTF/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a request for receiverESPrimRandObject, if the e2eSecInfo attribute of the Receiver's <remoteCSE> resource in CSE2 does not include a sharedReceiverESPrimRandObject |  |
| Reference | TS-0003 8.4.2 - step B.2D, TS-0004 7.5.1.2.11 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having established a pairwiseESPrimKey with the Receiver (CSE1) and the IUT having sent a RETRIEVE Request on the e2eSecInfo attribute in resource TARGET_REMOTE_CSE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE2 |
                     | the IUT receives a valid Response from CSE2 containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     |<remoteCSE>resource containing   | |
                     | e2eSecInfo attribute containing  | |
                     | no sharedReceiverESPrimRandObject parameter  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE1 <= IUT |
                     | the IUT sends a valid NOTIFY Request to Receiver CSE(CSE1) containing  | |
                     | To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 3  | |
                     | (receiverESPrimRandObject Request)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————–+————-+

TP/oneM2M/AE/SEC/ESP/NTF/002

| TP Id | TP/oneM2M/AE/SEC/ESP/NTF/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a request for receiverESPrimRandObject to the Receiver if the_sharedReceiverESPrimRandObject_ parameter in the Receiver's <remoteCSE> resource located in CSE2 has expired |  |
| Reference | TS-0003 8.4.2 - step B.2D, TS-0004 7.5.1.2.11 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having established a pairwiseESPrimKey with the Receiver CSE(CSE1) and the IUT having sent a RETRIEVE Request on the e2eSecInfo on resource TARGET_REMOTE_CSE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE2 |
                     | the IUT receives a valid Response from CSE2 containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     |<remoteCSE> resource containing   | |
                     | e2eSecInfo attribute containing  | |
                     | sharedReceiverESPrimRandObject parameter containing  | |
                     | ESPrimRandExpiry field set to an expired date  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE1 <= IUT |
                     | the IUT sends a valid NOTIFY Request to Receiver CSE containing  | |
                     | To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 3 (receiverESPrimRandObject  | |
                     | Request)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————–+————-+

TP/oneM2M/AE/SEC/ESP/NTF/003

| TP Id |  | TP/oneM2M/AE/SEC/ESP/NTF/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a valid ESPrim object to the Receiver CSE |  |
| Reference |  | TS-0003 8.4.2 - step C.4, TS-0004 7.5.1.2.13, TS-0001 11.4.2 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUThaving received receiverESPrimRandObject and the IUT having established a sessionESPrimKey with the Receiver CSE } |  |  |

- +-----------------------------------------------------------------------------------------------+-----------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid NOTIFY Request containing  | |
                     | To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 5 (ESPrim Object) and   | |
                     | esprimObject element  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------+-----------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid NOTIFY Request to the Receiver CSE containing To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 5 (ESPrim Object) and   | |
                     | esprimObject element set to a JSON Web Encryption (JWE)  | |
                     | Compact Serialization on inner request primitive  | |
                     |} | |
+———————-+———————————————————————————————–+—————————————————–+————-+

###### 7.2.1.8.1.1.2	RETRIEVE Operation

TP/oneM2M/AE/SEC/ESP/RET/001

| TP Id | TP/oneM2M/AE/SEC/ESP/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a valid request for e2eSecInfo attribute of the Receiver's <remoteCSE> resource in the CSE Registered with Receiver CSE |  |
| Reference | TS-0003 8.4.2 - step B.2A, TS-0004 7.5.1.2.11 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUThaving established a pairwiseESPrimKey with the Receiver CSE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE1 <= IUT |
                     | the IUT sends a valid RETRIEVE Request to CSE1 containing  | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | remoteCSE resourcecontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |
+———————-+————————————————————————————————————————————–+————-+

TP/oneM2M/AE/SEC/ESP/RET/002

| TP Id | TP/oneM2M/AE/SEC/ESP/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification containing an inner "receiverESPrimRandObject request" to the Receiver if the sharedReceiverESPrimRandObject parameter is not present in the <remoteCSE> resource in the CSE Registered with Receiver CSE |  |
| Reference | TS-0003 8.4.2 - step B.2D, TS-0004 7.5.1.2.11 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having established a pairwiseESPrimKey with the Originator and the <remoteCSE> resource at TARGET_REMOTE_CSE_ADDRESS contain ing e2eSecInfo attribute with no sharedReceiverESPrimRandObject parameter } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing   | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFICATION Request containing  | |
                     | To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 3 (receiverESPrimRandObject  | |
                     | Request)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ESP/RET/003

| TP Id | TP/oneM2M/AE/SEC/ESP/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification containing an inner "receiverESPrimRandObject request" to the Receiver if the sharedReceiverESPrimRandObject in the <remoteCSE> resource is expired |  |
| Reference | TS-0003 8.4.2 - step B.2D, TS-0004 7.5.1.2.11 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having established a pairwiseESPrimKey with the Originator and the <remoteCSE> resource at TARGET_REMOTE_CSE_ADDRESS contain ing e2eSecInfo attribute with an expired sharedReceiverESPrimRandObject parameter } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing   | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFICATION Request containing  | |
                     | To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 3 (receiverESPrimRandObject  | |
                     | Request)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————+————-+

7.2.1.8.1.2 Roles (ROL)

###### 7.2.1.8.1.2.1	CREATE Operation

7.2.1.8.1.3 ESCertKE (ESC)

###### 7.2.1.8.1.3.1	NOTIFY Operation

TP/oneM2M/AE/SEC/ESC/NTF/001

| TP Id |  | TP/oneM2M/AE/SEC/ESC/NTF/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notify request with ESCertKE message 1 when triggered |  |
| Reference |  | TS-0003 8.7.2.2 - step C.1.a, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Terminating End-Point's Certificate Info & Identity } |  |  |

- +-------------------------------------------------+-------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid NOTIFY Request containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 1 (TLS Message: Client Hello) | |
                     |} | |

- +-------------------------------------------------+-------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Request to CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 1 (TLS Message: Client Hello) | |
                     |} | |
+———————-+————————————————-+——————————————————————-+————-+

TP/oneM2M/AE/SEC/ESC/NTF/002

| TP Id |  | TP/oneM2M/AE/SEC/ESC/NTF/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT sends a notify response with ESCertKE message 2 when it receives a notify request with ESCertKE message 1 |  |
| Reference |  | TS-0003 8.7.2.2 - step C.1.b, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id |  | CF03 |  |
| PICS Selection |  | PICS_AE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Initiating End-Point's Certificate Info & Identity } |  |  |

- +-----+---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE1 |
                     | the IUT receives a valid NOTIFY Request from CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE1_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 1 (TLS Message: Client Hello) | |
                     |} | |

- +-----+---------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Response to CSE containing  | |
                     | To set to INITIATING_ENDPOINT_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 2 (TLS  | |
                     | Messages: Server Hello, Certificate*,  | |
                     | ServerKeyExchange*, CertificateRequest*,  | |
                     | ServerHelloDone)  | |
                     |} | |
+———————-+—–+—————————————————————————————————————+————-+

TP/oneM2M/AE/SEC/ESC/NTF/003

| TP Id | TP/oneM2M/AE/SEC/ESC/NTF/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notify request with ESCertKE message 3 when it receives a notify response with ESCertKE message 2 |  |
| Reference | TS-0003 8.7.2.2 - step C.1.c, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE, |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Terminating End-Point's Certificate Info & Identity and the IUT having sent a notify request to the Terminating End-Point with ESCertKE Message 1 } |  |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE1 |
                     | the IUT receives a valid NOTIFY Response from CSE1 containing  | |
                     | To set to INITIATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE1_ID and  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 2 (TLS  | |
                     | Messages: Server Hello, Certificate*,  | |
                     | ServerKeyExchange*, CertificateRequest*,  | |
                     | ServerHelloDone)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Request to CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 3 (TLS  | |
                     | Messages: Certificate*, ClientKeyExchange,  | |
                     | CertificateVerify*, [ChangeCipherSpec], Finished)  | |
                     |} | |
+———————-+—————————————————————————————————————+————-+

TP/oneM2M/AE/SEC/ESC/NTF/004

| TP Id | TP/oneM2M/AE/SEC/ESC/NTF/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notify response with ESCertKE message 4 when it receives a notify request with ESCertKE message 3 |  |
| Reference | TS-0003 8.7.2.2 - step C.1.d, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id | CF03 |  |
| PICS Selection | PICS_AE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Initiating End-Point's Certificate Info & Identity and the IUT having sent a notify request to the Initiating End-Point with ESCertKE Message 2 } |  |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE1 |
                     | the IUT receives a valid NOTIFY Request from CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE1_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 3 (TLS  | |
                     | Messages: Certificate*, ClientKeyExchange,  | |
                     | CertificateVerify*, [ChangeCipherSpec], Finished)  | |
                     |  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Response to CSE1 containing  | |
                     | To set to INITIATING_ENDPOINT_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 4 (TLS  | |
                     | Messages: ChangeCipherSpec], Finished)  | |
                     |} | |
+———————-+—————————————————————————————————————+————-+

### 7.2.2	Group CSE (CE)

#### 7.2.2.1	General Capability (GEN)

##### 7.2.2.1.1	CREATE Operation

###### TP/oneM2M/CSE/GEN/CRE/001

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +------+------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE request from AE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to 3 (container) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |

- +------+------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+——+————————————————————————————————+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/CRE/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/CRE/002

| TP Id |  | TP/oneM2M/CSE/GEN/CRE/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +--------+----------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE request from AE containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to 3 (container) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |

- +--------+----------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+——–+———————————————————————————————-+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/CRE/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/CRE/003

| TP Id | TP/oneM2M/CSE/GEN/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a <container> resource using shortcut structured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE request from AE containing  | |
                     | To set to SHORTCUT_H IERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to 3 (container) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+——————————————————————————————————-+————-+

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/CRE/003_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/CRE/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/CRE/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/CRE/004

| TP Id | TP/oneM2M/CSE/GEN/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a <container> resource using structured resource identifier with hybrid addressing |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the IUT having created a <group> resource GROUP_RESOURCE_ADDRESS containing membersIDs attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE request from AE containing  | |
                     | To set to HYBRID __HIERARCHICAL_ _ RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to 3 (container) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+———————————————————————————————————————+————-+

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |  |
| --- | --- | --- |
| TP/oneM2M/CSE/GEN/CRE/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID (see note) |  |
| TP/oneM2M/CSE/GEN/CRE/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |  |
| TP/oneM2M/CSE/GEN/CRE/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |  |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus "/fopt" |  | CSE_RELATIVE_RESOURCE_ID of group plus "/fopt" ¦ |

##### 7.2.2.1.2	UPDATE Operation

###### TP/oneM2M/CSE/GEN/UPD/001

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +----------+----------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE request from AE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid labels attribute  | |
                     |} | |

- +----------+----------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+———-+———————————————————————————————-+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/UPD/002

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +----------+--------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE request from AE containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid l abels attribute  | |
                     |} | |

- +----------+--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+———-+——————————————————————————————–+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/UPD/003

| TP Id |  | TP/oneM2M/CSE/GEN/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the update of a <container> resource using shortcut structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +-+-----------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE request from AE containing  | |
                     | To set to SHORTCUT_H IERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid l abels attribute  | |
                     |} | |

- +-+-----------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+-+—————————————————————————————————–+————-+

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/003_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/UPD/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/UPD/004

| TP Id | TP/oneM2M/CSE/GEN/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the update of a <container> resource using structured resource identifier with hybrid addressing |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the IUT having created a <group> resource GROUP_RESOURCE_ADDRESS containing membersIDs attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE request from AE containing  | |
                     | To set to HYBRID __HIERARCHICAL_RESOURCE_ADDRESS_ and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid l abels attribute  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+——————————————————————————————————————-+————-+

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/UPD/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID (see note) |
| TP/oneM2M/CSE/GEN/UPD/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/UPD/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus "/fopt" |  |

##### 7.2.2.1.3	RETRIEVE Operation

###### TP/oneM2M/CSE/GEN/RET/001

| TP Id |  | TP/oneM2M/CSE/GEN/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +-------+-------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE request from AE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +-------+-------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |
+———————-+——-+————————————————————————————————-+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/RET/002

| TP Id |  | TP/oneM2M/CSE/GEN/RET/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the retrieval of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +---------+-----------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE request from AE containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +---------+-----------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |
+———————-+———+———————————————————————————————–+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/RET/003

| TP Id | TP/oneM2M/CSE/GEN/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the retrieval of a <container> resource using shortcut structured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE request from AE containing  | |
                     | To set to SHORTCUT_H IERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |
+———————-+——————————————————————————————————–+————-+

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/003_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/RET/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/RET/004

| TP Id | TP/oneM2M/CSE/GEN/RET/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the retrieval of a <container> resource using structured resource identifier with hybrid addressing |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the IUT having created a <group> resource GROUP_RESOURCE_ADDRESS containing membersIDs attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE request from AE containing  | |
                     | To set to HYBRID __HIERARCHICAL_RESOURCE_ADDRESS_ and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | container resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————-+————-+

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/RET/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID (see note) |
| TP/oneM2M/CSE/GEN/RET/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/RET/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus "/fopt" |  |

##### 7.2.2.1.4	DELETE Operation

###### TP/oneM2M/CSE/GEN/DEL/001

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using unstructured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE, PICS_UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +------+------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE request from AE containing  | |
                     | To set to NON_HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +------+------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+——+————————————————————————————————+————-+

| TP Id | NON_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/001_CSR | UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/001_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/001_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Unstructured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/DEL/002

| TP Id |  | TP/oneM2M/CSE/GEN/DEL/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a <container> resource using structured resource identifier |  |
| Reference |  | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +--------+----------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE request from AE containing  | |
                     | To set to HIERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +--------+----------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+——–+———————————————————————————————-+————-+

| TP Id | HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/002_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/002_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/002_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/DEL/003

| TP Id | TP/oneM2M/CSE/GEN/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the deletion of a <container> resource using shortcut structured resource identifier |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE request from AE containing  | |
                     | To set to SHORTCUT_H IERARCHICAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+——————————————————————————————————-+————-+

| TP Id | SHORTCUT_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/003_CSR/ | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/003_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/003_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the Structured-CSE-Relative-Resource-ID |  |

###### TP/oneM2M/CSE/GEN/DEL/004

| TP Id | TP/oneM2M/CSE/GEN/DEL/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the deletion of a <container> resource using structured resource identifier with hybrid addressing |  |
| Reference | TS-0001 [1], clause 9.3.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE, PICS_STRUCTURED_CSE_RELATIVE_RESOURCE_ID_FORMAT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <container> resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE request from AE containing  | |
                     | To set to HYBRID __HIERARCHICAL_RESOURCE_ADDRESS_ and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+———————————————————————————————————————+————-+

| TP Id | HYBRID_HIERARCHICAL_RESOURCE_ADDRESS |
| --- | --- |
| TP/oneM2M/CSE/GEN/DEL/004_CSR | STRUCTURED_CSE_RELATIVE_RESOURCE_ID |
| TP/oneM2M/CSE/GEN/DEL/004_SPR | SP_RELATIVE_RESOURCE_ID, (see note) |
| TP/oneM2M/CSE/GEN/DEL/004_ABS | ABSOLUTE_RESOURCE_ID, (see note) |
| NOTE: These addresses are constructed with the UNSTRUCTURED_CSE_RELATIVE_RESOURCE_ID of group plus "/fopt" |  |

#### 7.2.2.2	Registration (REG)

##### 7.2.2.2.1	RETRIEVE Operation

###### TP/oneM2M/CSE/REG/RET/001

| TP Id | TP/oneM2M/CSE/REG/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <CSEBase> resource and responds with all attributes that have multiplicity equals to 1 |  |
| Reference | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content   | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | CSEBase resource representation  | |
                     |} | |
+———————-+——————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/REG/RET/002

| TP Id | TP/oneM2M/CSE/REG/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <CSEBase> resource with the optional ATTRIBUTE |  |
| Reference | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource and the IUT having a CSEBase resource containing the valid ATTRIBUTE attribute } |  |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | CSEBase resource containing  | |
                     | valid ATTRIBUTE attribute  | |
                     |} | |
+———————-+———————————————————————————————————–+————-+

| TP Id | PICS Selection | Reference | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/002_CST | PICS_CB_CST | TS-0001 9.6.3 | cseType |
| TP/oneM2M/CSE/REG/RET/002_NL | PICS_CB_NL | TS-0001 9.6.3 | nodeLink |

###### TP/oneM2M/CSE/REG/RET/003

| TP Id | TP/oneM2M/CSE/REG/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <AE> resource with attributes multiplicity equals to 1 |  |
| Reference | TS-0001 [1], clause 10.2.1.2, and clause 9.6.5, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the AE resource } |  |

- +------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID   | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |
+———————-+————————————————————————————————————+————-+

###### TP/oneM2M/CSE/REG/RET/004

| TP Id | TP/oneM2M/CSE/REG/RET/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <AE> resource with optional ATTRIBUTE |  |
| Reference | TS-0001 [1], clause 10.2.1.2 and clause 9.6.5, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing valid ATTRIBUTE attribute and the AE having privileges to perform RETRIEVE operation on the AE resource } |  |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | AE resource containing ATTRIBUTE attribute  | |
                     |} | |
+———————-+———————————————————————————————————–+————-+

| TP Id | PICS Selection | Reference | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/004_LBL | PICS_AE_LBL | TS-0001 [1], clause 9.6.5 | labels |
| TP/oneM2M/CSE/REG/RET/004_APN | PICS_AE_APN | TS-0001 [1], clause 9.6.5 | appName |
| TP/oneM2M/CSE/REG/RET/004_POA | PICS_AE_POA | TS-0001 [1], clause 9.6.5 | pointOfAccess |
| TP/oneM2M/CSE/REG/RET/004_NL | PICS_AE_NL | TS-0001 [1], clause 9.6.5 | nodeLink |
| TP/oneM2M/CSE/REG/RET/004_CSZ | PICS_AE_CSZ | TS-0001 [1], clause 9.6.5 | contentSerialization |

###### TP/oneM2M/CSE/REG/RET/005

| TP Id | TP/oneM2M/CSE/REG/RET/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <CSEBase> resource including the cseType attribute |  |
| Reference | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | CSEBase resource containing  | |
                     | cseType attribute set to 1 (IN_CSE)  | |
                     |} | |
+———————-+———————————————————————————————————–+————-+

###### TP/oneM2M/CSE/REG/RET/006

| TP Id | TP/oneM2M/CSE/REG/RET/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <remoteCSE> resource |  |
| Reference | TS-0001 [1], clause 10.1.1.2.1 and, clause 9.6.4 TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the ORIGINATOR and the ORIGINATOR having privileges to perform RETRIEVE operation on the remoteCSE resource } |  |

- +---------------------------------------------------------------------------------------------------------+-----------------+

- |when {   |IUT <= ORIGINATOR|
                     | the IUT receives a valid RETRIEVE Request from ORIGINATOR containing  | |
                     | To set to REMOTECSE_RESOURCE_ADDRESS and  | |
                     | From set to ORIGINATOR_ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------+-----------------+

- |then {   |IUT => ORIGINATOR|
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | remoteCSE resource representation  | |
                     |} | |
+———————-+———————————————————————————————————+—————–+

###### TP/oneM2M/CSE/REG/RET/007

| TP Id | TP/oneM2M/CSE/REG/RET/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <remoteCSE> resource with optional ATTRIBUTE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4, TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the ORIGINATOR and the ORIGINATOR having privileges to perform RETRIEVE operation on the remoteCSE resource } |  |

- +---------------------------------------------------------------------------------------------------------+-----------------+

- |when {   |IUT <= ORIGINATOR|
                     | the IUT receives a valid RETRIEVE Request from ORIGINATOR containing  | |
                     | To set to REMOTECSE_RESOURCE_ADDRESS and  | |
                     | From set to ORIGINATOR_ID   | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------+-----------------+

- |then {   |IUT => ORIGINATOR|
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | remoteCSE resource containing  | |
                     | ATTRIBUTE attribute  | |
                     |} | |
+———————-+———————————————————————————————————+—————–+

| TP Id | PICS Selection | Reference | ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/RET/007_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4 | labels |
| TP/oneM2M/CSE/REG/RET/007_CST | PICS_CSR_CST | TS-0001 [1], clause 9.6.4 | cseType |
| TP/oneM2M/CSE/REG/RET/007_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4 | pointOfAccess |
| TP/oneM2M/CSE/REG/RET/007_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4 | nodeLink |

###### TP/oneM2M/CSE/REG/RET/008

| TP Id | TP/oneM2M/CSE/REG/RET/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a retrieval request of <CSEBase> resource and responds with supportedResourceTypes attribute containing a list of the supported resources and pointOfAccess containing the list of physical addresses to be used by Registree to connect to this CSE (e.g. IP address, FQDN) |  |
| Reference | TS-0001 [1], clause 10.2.3.2 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the CSEBase resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | CSEBase resource containing  | |
                     | supportedResourceType set to PX_SRT  | |
                     | pointOfAccess set to PX_POA  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/REG/RET/009

| TP Id |  | TP/oneM2M/CSE/REG/RET/009 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> retrieve request on TARGET_REMOTE_CSE_ADDRESS |  |
| Reference |  | TS-0001 [1], clause 10.2.2.2, TS-0004 [2], clause 7.4.5.2.3 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT is triggered to send a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESS } |  | NA |  |
| then { the IUT sends a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESSand From set to CSE_ID and no Content } |  | IUT => CSE |  |

###### TP/oneM2M/CSE/REG/RET/010

| TP Id |  | TP/oneM2M/CSE/REG/RET/010 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> retrieve request |  |
| Reference |  | TS-0001 [1], clause 10.2.2.2, TS-0004 [2], clause 7.4.5.2.3 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT receives a valid RETRIEVE Request containing To set to TARGET_REMOTE_CSE_ADDRESSand From set to CSE_ID and no Content } |  | IUT <= CSE |  |
| then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Contentcontaining remoteCSE resource representation } |  | IUT => CSE |  |

##### 7.2.2.2.2	CREATE Operation

###### TP/oneM2M/CSE/REG/CRE/001

| TP Id | TP/oneM2M/CSE/REG/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned SP_ _ AE_ID |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case b, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS SELECTION |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to SP_RELATIVE_AE_ID and  | |
                     | Resource Typeset to 2(AE) and  | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | SP_ RELATIVE _ AE_ID | PICS S ELECTION |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/001_CAE | Release 1 | Starting with "C" | PICS_CSE |
| TP/oneM2M/CSE/REG/CRE/001_SAE | Release 2 | Starting with "S" | PICS_IN_CSE |

###### TP/oneM2M/CSE/REG/CRE/002

| TP Id | TP/oneM2M/CSE/REG/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned AE_ID of AE-ID-STEM format |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case d, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS SELECTION |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID_STEM and  | |
                     | Resource Typeset to 2 (AE) and   | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | AE_ID _ STEM | PICS Selection |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/002_CAE | Release 1 | Starting with "C" | PICS_CSE |
| TP/oneM2M/CSE/REG/CRE/002_SAE | Release 2 | Starting with "S" | PICS_IN_CSE |

Editor's Note: Test purpose TP/oneM2M/CSE/REG/CRE/002 is duplicated with TP/oneM2M/CSE/REG/CRE/026 in terms of testing AE re-registration in TS-0001 10.1.1.2.2. case d. Therefore, test purpose TP/oneM2M/CSE/REG/CRE/026 is going to be removed.

###### TP/oneM2M/CSE/REG/CRE/003

| TP Id | TP/oneM2M/CSE/REG/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided preprovisioned S-AE-ID-STEM |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case b, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the IN-CSE } |  |

- +------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | From set to S-AE-ID-STEM and  | |
                     | Resource Typeset to 2(AE) and   | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => IN-CSE|
                     | the IUT sends a valid CREATE Request to IN-CSE containing  | |
                     | From set to SP-RELATIVE-AE-ID and  | |
                     | Content containing  | |
                     | AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID and  | |
                     | labels attribute set to "Credential-IDs:None"  | |
                     |} | |

| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Valid AE-ID-Stem starting with "S" |  |  |
| --- | --- | --- |

Editor's Note: need to clarify if there is duplication.

###### TP/oneM2M/CSE/REG/CRE/004

| TP Id |  | TP/oneM2M/CSE/REG/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (allowed App-ID, not allowed C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 - step 3 and clause 9.6.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT not allowing to register an AE containing allowed App-ID attribute indicating APP-ID and no t allowed AE-ID attribute indicating C-AE-ID-STEM } |  |  |

- +------------------------------------------+------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and | |
                     | From set to C-AE-ID-STEM and  | |
                     | Content containing  | |
                     |AE resource containing  | |
                     | App-ID attribute set to APP-ID  | |
                     |} | |

- +------------------------------------------+------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the AE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4126(APP_RULE_VALIDATION_FAILED)  | |
                     |} | |
+———————-+——————————————+——————————————————————————————+————-+

###### TP/oneM2M/CSE/REG/CRE/005

| TP Id |  | TP/oneM2M/CSE/REG/CRE/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE registration (allowed App-ID, S-AE-ID-STEM not provided by AE) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 case a, and clause 9.6.19 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state"and the IUT having registered to the IN-CSE and and the IUT allowing to register an AE containing App-ID attributeset to APP-ID } |  |  |

- +--------------------------------------------------+----------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and | |
                     | From set to "S" and  | |
                     | Content containing  | |
                     |AE resource containing  | |
                     | App-ID attribute set to APP-ID  | |
                     |} | |

- +--------------------------------------------------+----------------------------------------------------------------------------------+-------------+

- |then {   |IUT => IN-CSE|
                     | the IUT sends a valid CREATE Request to the IN-CSEcontaining  | |
                     | Resource Type set to 10002 (AEAnnc) and  | |
                     | From set to SP-RELATIVE-CSE-ID followed by '/S' and  | |
                     | Content containing  | |
                     |AEAnnc resource containing  | |
                     | App-ID attribute set to APP-ID and  | |
                     | valid link attribute and  | |
                     | labels attribute indicating 'Credential-ID:None'  | |
                     |} | |
+———————-+————————————————–+———————————————————————————-+————-+ Editor's Note: Double check the value of attribute nodeLink!!

###### TP/oneM2M/CSE/REG/CRE/006

| TP Id | TP/oneM2M/CSE/REG/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and provided character 'S' in AE_ID_Stem |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the IN-CSE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | From set to character 'S' and  | |
                     | Resource Typeset to 2(AE) and  | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => IN-CSE|
                     | the IUT sends a valid CREATE Request to IN_CSE containing  | |
                     | From set to SP-RELATIVE-CSE-ID followed by '/S' and  | |
                     | Content containing  | |
                     | AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID  | |
                     |} | |

| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{AE_ID_Stem} AE_ID_Stem = "S" |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/REG/CRE/007

| TP Id | TP/oneM2M/CSE/REG/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an response of AEANNC create request during AE registration with provided character 'S' in AE_ID_Stem |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the IN-CSE and the IUT having received a valid AE create request containing From set to "S"and the IUT having sent a valid AEANNC create to IN_CSE } |  |

- +------------------------------------------------------------------------------------------------------------------+----------------+

- |when{  |IUT <- IN-CSE|
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------+----------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | ResourceID attribute set to S-AE-ID-STEM and  | |
                     | AE-ID attribute set to S-AE-ID-STEM  | |
                     |} | |

| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM} S-AE-ID-STEM = Valid AE-ID-Stem assigned by IN-CSE |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/REG/CRE/008

| TP Id |  | TP/oneM2M/CSE/REG/CRE/008 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer request to the IN-CSE |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 - case b, step 5 and clause 9.6.19 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" the IUT having registered to the IN-CSE and the AE having already been registered containing AE-ID set to S-AE-ID-STEM and the AE having deregistered } |  |  |

- +----+--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and | |
                     | From set to S-AE-ID-STEM and  | |
                     | Content containing  | |
                     |AE resource representation  | |
                     |} | |

- +----+--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => IN-CSE|
                     | the IUT sends a valid UPDATE Request to the IN-CSE containing  | |
                     | From set to SP-RELATIVE-AE-ID and  | |
                     | Content containing  | |
                     |AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID and  | |
                     | labels attribute indicating 'Credential-ID:None'  | |
                     |} | |

| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE |  |  |  |
| --- | --- | --- | --- |

###### TP/oneM2M/CSE/REG/CRE/009

| TP Id | TP/oneM2M/CSE/REG/CRE/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), communication between MN-CSE and IN-CSE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case b, step 5-8 |  |
| Config Id | CF03 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the MN-CSE and the AE having already been registered containing AE-ID set to S-AE-ID-STEM and the AE having deregistered } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT<= MN-CSE |
                     | the IUT receives a valid UPDATE Request from the MN-CSE containing  | |
                     | To set to AEANNC_RESOURCE_ADDRESS and  | |
                     | From set to SP-RELATIVE-AE-ID and  | |
                     | Content containing  | |
                     |AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID and  | |
                     | labels attribute indicating 'Credential-ID:None'  | |
                     |} | |

| then { | IUT => MN-CSE the IUT updates the AEANNC_RESOURCE_ADDRESS resource and the IUT sends a valid Response to MN-CSE containing Response Status Code set to 2004 (UPDATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels attribute indicating 'Credential-ID:None' } |  |
| --- | --- | --- |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE |  |  |

###### TP/oneM2M/CSE/REG/CRE/010

| TP Id | TP/oneM2M/CSE/REG/CRE/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer response to the AE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case b, step 8 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the MN-CSE and the AE having already been registered containing AE-ID set to S-AE-ID-STEM and the AE having deregistered and the AE having sent a registration containing From set to S-AE-ID-STEM and the IUT having sent a valid AEAnnc UPDATE Request to IN-CSE } |  |

- +-----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= IN-CSE|
                     | the IUT receives a valid UPDATE Response from IN-CSE containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     |AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID and  | |
                     | labels attribute indicating 'Credential-ID:None'   | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response to the AE containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | AE-ID attribute set to S-AE-ID-STEM  | |
                     |} | |

| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/REG/CRE/011

| TP Id | TP/oneM2M/CSE/REG/CRE/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and asking AE_ID_Stem by indicating AE_ID_Stem starting with character 'C' |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case c, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to 'C' and  | |
                     | Resource Typeset to 2(AE) and   | |
                     | Content containing  | |
                     | AE resource representation   | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | ResourceID attribute indicating a value  | |
                     | starting with 'C'  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/REG/CRE/012

| TP Id | TP/oneM2M/CSE/REG/CRE/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an AE registration with the optional attribute OPTIONAL_ATTRIBUTE provided |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 and clause 9.6.5-2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a CSEBase resource } |  |

- +-------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 2 (AE) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |AE resource containing  | |
                     | valid OPTIONAL_ATTRIBUTE attribute   | |
                     |} | |

- +-------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     |AE resource containing  | |
                     | valid OPTIONAL_ATTRIBUTE attribute  | |
                     |} | |
+———————-+——————————————————————————————-+————-+

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/012_AE/LBL | PICS_AE_LBL | TS-0001 [1], clause 9.6.5-2 | labels |
| TP/oneM2M/CSE/REG/CRE/012_AE/APN | PICS_AE_APN | TS-0001 [1], clause 9.6.5-2 | AppName |
| TP/oneM2M/CSE/REG/CRE/012_AE/POA | PICS_AE_POA | TS-0001 [1], clause 9.6.5-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/012_AE/NL | PICS_AE_NL | TS-0001 [1], clause 9.6.5-2 | nodeLink |
| TP/oneM2M/CSE/REG/CRE/012_AE/OR | PICS_AE_OR | TS-0001 [1], clause 9.6.5-2 | ontologyRef |

###### TP/oneM2M/CSE/REG/CRE/013

| TP Id | TP/oneM2M/CSE/REG/CRE/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <remoteCSE> resource with OPTIONAL_ATTRIBUTE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4 TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id | CF04 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from CSE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16(remoteCSE) and Content containing remoteCSE resource representation containing OPTIONAL_ATTRIBUTE attribute } | IUT <= CSE |
| then { | IUT => CSE the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing remoteCSE resource representation } |  |

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/013_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4 | labels |
| TP/oneM2M/CSE/REG/CRE/013_CST | PICS_CSR_CST | TS-0001 [1], clause 9.6.4 | cseType |
| TP/oneM2M/CSE/REG/CRE/013_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/013_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4 | nodeLink |

###### TP/oneM2M/CSE/REG/CRE/014

| TP Id | TP/oneM2M/CSE/REG/CRE/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the request to the remoteCSE according to pointOfAccess in the <remoteCSE> resource |  |
| Reference | TS-0001 [1], clause 9.6.4 and clause 9.3.2.2.3, TS-0004 [2], clause 6.2.3 |  |
| Config Id | CF02 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUThaving created the remoteCSE resource containing POINT_OF_ACCESS attribute } |  |

- +----------------------------------------------------------------------------------------------------+-----------------+

- |when {   |IUT <= Originator|
                     | the IUT receives a valid REQUEST from Originator containing  | |
                     | To set to RESOURCE_ADDRESS and  | |
                     | From set to Originator_ID   | |
                     |} | |

- +----------------------------------------------------------------------------------------------------+-----------------+

- |then {   |IUT => CSE |
                     | the IUT sends the REQUEST to POINT_OF_ACCESS  | |
                     |} | |
+———————-+—————————————————————————————————-+—————–+

###### TP/oneM2M/CSE/REG/CRE/015

| TP Id | TP/oneM2M/CSE/REG/CRE/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the request to the CSE according to pointOfAccess in the <remoteCSE> resource and forward the response back to the originator |  |
| Reference | TS-0001 [1], clause 9.6.4 and clause 9.3.2.2.3, TS-0004 [2], clause 6.2.3 |  |
| Config Id | CF02 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE , PICS_SP_RELATIVE_RESOURCE_ID, PICS_ABSOLUTE_RESOURCE_ID |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having received a valid REQUEST from Originator and The IUT having sent the REQUEST to the remoteCSE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid RESPONSE from the remoteCSE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |then {   |IUT => Originator|
                     | the IUT sends the RESPONSE to the Originator  | |
                     |} | |
+———————-+——————————————————————————————————————————————–+—————–+

###### TP/oneM2M/CSE/REG/CRE/016

| TP Id |  | TP/oneM2M/CSE/REG/CRE/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects an AE registration (not allowed App-ID) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 - step 3 and clause 9.6.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT not allowing to register an AE containing not allowed App-ID attribute indicating APP-ID } |  |  |

- +------------------------------------------------------------------+------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and | |
                     | Content containing  | |
                     |AE resource containing  | |
                     | App-ID attribute set to APP-ID  | |
                     |} | |

- +------------------------------------------------------------------+------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the AE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4126(APP_RULE_VALIDATION_FAILED)  | |
                     |} | |
+———————-+——————————————————————+——————————————————————+————-+

###### TP/oneM2M/CSE/REG/CRE/017

| TP Id | TP/oneM2M/CSE/REG/CRE/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a create request of <AE> resource that doesn't include the MANDATORY_ATTRIBUTE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having preconfigured AE_ID_stem } |  |

- +--------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Typeset to 2(AE) and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | no _MANDATORY ATTRIBUTE attribute   | |
                     |} | |

- +--------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD Request)  | |
                     |} | |
+———————-+————————————————————————————————–+————-+

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/017_API | TS-0001 [1], clause 9.6.5 | App-ID |
| TP/oneM2M/CSE/REG/CRE/017_RR | TS-0001 [1], clause 9.6.5 | requestReachability |

###### TP/oneM2M/CSE/REG/CRE/018

| TP Id | TP/oneM2M/CSE/REG/CRE/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <remoteCSE> resource with attributes multiplicity equals to 1 |  |
| Reference | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id | CF04 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid CREATE Request from CSE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Resource Typeset to 16(remoteCSE) and  | |
                     | Content containing  | |
                     | remoteCSE resource representation  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | remoteCSE resource representation  | |
                     |} | |
+———————-+————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/REG/CRE/019

| TP Id | TP/oneM2M/CSE/REG/CRE/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <remoteCSE> resource with attributes multiplicity equals to 1 without the preconfigured CSE-ID |  |
| Reference | TS-0001 [1], clause 10.1.1.2.1 and clause 9.6.4 TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id | CF04 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid CREATE Request from CSE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | no From and  | |
                     | Resource Typeset to 16(remoteCSE) and  | |
                     | Content containing  | |
                     | remoteCSE resource representation  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | remoteCSE resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————————–+————-+

TP/oneM2M/CSE/REG/CRE/020

| TP Id | TP/oneM2M/CSE/REG/CRE/020 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a create request of <AE> resource with attributes multiplicity equals to 1 and asking AE_ID_Stem by indicating AE_ID_Stem starting with character 'S' |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | From set to 'S' and  | |
                     | Resource Typeset to 2(AE) and   | |
                     | Content containing  | |
                     | AE resource representation   | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | ResourceID attribute set to S-AE-ID-STEM  | |
                     |} | |

| NOTE: S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/REG/CRE/021

| TP Id | TP/oneM2M/CSE/REG/CRE/021 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the create request of <CSEBase> resource |  |
| Reference | TS-0001 [1], clause 10.2.3.1 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 5 (CSEBase) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | CSEBase resource representation  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+———————————————————————————————————–+————-+

###### TP/oneM2M/CSE/REG/CRE/022

| TP Id | TP/oneM2M/CSE/REG/CRE/022 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an AE registration (allowed App-ID, C-AE-ID-STEM not provided by AE) |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case C and clause 9.6.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE containing App-ID attribute indicating APP-ID and AE-ID attribute indicating C-AE-ID-STEM } |  |

- +----------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 2 (AE) and  | |
                     | From set to 'C' and  | |
                     | Content containing  | |
                     |AE resource containing  | |
                     | App-ID attribute set to APP-ID  | |
                     |} | |

- +----------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT creates the AE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+———————————————————————————-+————-+

###### TP/oneM2M/CSE/REG/CRE/023

| TP Id |  | TP/oneM2M/CSE/REG/CRE/023 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects registration of already registered AE (C-AE-ID-STEM provided by AE) |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 - step 4 and clause 9.6.19 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state"and the IUT having registered the AE with AE-ID attribute set to C-AE-ID-STEM } |  |  |

- +----------------------------------------------------------+---------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE) and | |
                     |To set to TARGE_CSE_RESOURCE_ADDRESS and  | |
                     | From set to C-AE-ID-STEM and  | |
                     | Content containing  | |
                     | AE resource representation  | |
                     |} | |

- +----------------------------------------------------------+---------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the AE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4105 (CONFLICT)  | |
                     |} | |
+———————-+———————————————————-+———————————————————————————+————-+

###### TP/oneM2M/CSE/REG/CRE/024

| TP Id |  | TP/oneM2M/CSE/REG/CRE/024 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a CSE registration request with attributes multiplicity equals to 1 |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT is triggered to send a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | NA |  |
| then { the IUT sends a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource representation } |  | IUT => CSE |  |

###### TP/oneM2M/CSE/REG/CRE/025

| TP Id |  | TP/oneM2M/CSE/REG/CRE/025 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with attributes multiplicity equals to 1 |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |

- +-+--------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Resource Type set to 16 (remoteCSE) and  | |
                     | Content containing  | |
                     | remoteCSE resource representation  | |
                     |} | |

- +-+--------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED) and  | |
                     |Content containing  | |
                     | remoteCSE resource representation  | |
                     |  | |
                     |and the IUT creates the <remoteCSE> resource  | |
                     |} | |
+———————-+-+——————————————————————————————–+————-+

###### TP/oneM2M/CSE/REG/CRE/026

| TP Id |  | TP/oneM2M/CSE/REG/CRE/026 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a CSE registration request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE or PICS_ASN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT is triggered to send a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS an d Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | NA |  |
| then { the IUT sends a valid CREATE Request containing To set to CSEBASE_RESOURCE_ADDRESS and From set to CSE_ID and Resource Type set to 16 (remoteCSE) and Content containing remoteCSE resource containing cseType attribute set to 2 (MN_CSE) and valid attribute OPTIONAL_ATTRIBUTE } |  | IUT => CSE |  |

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/026_RN | PICS_CSR_RN | TS-0001 [1], clause 9.6.4-2 | resourceName |
| TP/oneM2M/CSE/REG/CRE/026_ET | PICS_CSR_ET | TS-0001 [1], clause 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/CRE/026_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/CRE/026_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/026_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |
| TP/oneM2M/CSE/REG/CRE/026_CST | PICS_CSR_CST | TS-0001 [1], clause 9.6.4-2 | cseType |

###### TP/oneM2M/CSE/REG/CRE/027

| TP Id |  | TP/oneM2M/CSE/REG/CRE/027 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with cseType attribute set to 'MN_CSE' |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |

- +-+--------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Resource Type set to 16 (remoteCSE) and  | |
                     | Content containing  | |
                     | remoteCSE resource containing  | |
                     | cseType attribute set to 2 (MN_CSE)  | |
                     |} | |

- +-+--------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED) and  | |
                     |Content containing  | |
                     | remoteCSE resource containing  | |
                     | cseType attribute set to 2 (MN_CSE)  | |
                     |  | |
                     | and the IUT creates a <remoteCSE> resource  | |
                     |} | |
+———————-+-+——————————————————————————————–+————-+

###### TP/oneM2M/CSE/REG/CRE/028

| TP Id |  | TP/oneM2M/CSE/REG/CRE/028 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a CSE registration request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.1, TS-0004 [2], clause 7.4.5.2.2 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the CSE having privileges to perform CREATE operation on the resource CSEBASE_RESOURCE_ADDRESS } |  |  |

- +-+--------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Resource Type set to 16 (remoteCSE) and  | |
                     | Content containing  | |
                     | remoteCSE resource containing  | |
                     | cseType attribute set to 2 (MN_CSE) and  | |
                     | valid attribute OPTIONAL_ATTRIBUTE  | |
                     |} | |

- +-+--------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED) and  | |
                     |Content containing  | |
                     | remoteCSE resource containing  | |
                     | cseType attribute set to 2 (MN_CSE) and  | |
                     | valid attribute OPTIONAL_ATTRIBUTE  | |
                     |  | |
                     |and the IUT creates a <remoteCSE> resource  | |
                     |} | |
+———————-+-+——————————————————————————————–+————-+

| TP Id | PICS Selection | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/CRE/028_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/CRE/028_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/CRE/028_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |

TP/oneM2M/CSE/REG/CRE/029

| TP Id | TP/oneM2M/CSE/REG/CRE/029 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an AE registration (preprovisioned S-AE-ID-STEM provided by AE), communication between MN-CSE and IN-CSE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case a, step 5-8 |  |
| Config Id | CF04 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT<= MN-CSE |
                     | the IUT receives a valid CREATE Request from the CSE containing  | |
                     | Resource Type set to 10002 (AEAnnc) and  | |
                     | From set to SP-RELATIVE-AE-ID and  | |
                     | Content containing  | |
                     |AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID and  | |
                     | labels attribute indicating 'Credential-ID:None'  | |
                     |} | |

| then { | IUT => MN-CSE the IUT creates the AEANNC_RESOURCE_ADDRESS resource and the IUT sends a valid Response to CSE containing Response Status Code set to 2001 (CREATED) and Content containing AEAnnc resource containing link attribute set to SP-RELATIVE-AE-ID and labels indicating 'Credential-ID:None' } |  |
| --- | --- | --- |
| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM} S-AE-ID-STEM = Valid AE-ID-Stem starting with "S" |  |  |

TP/oneM2M/CSE/REG/CRE/030

| TP Id | TP/oneM2M/CSE/REG/CRE/030 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an AE re-registration (allowed M2M-SP-assigned AE-ID, S-AE-ID-STEM provided by AE), transfer response to the AE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 - case b, step 8 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the MN-CSE and the AE having sent a registration containing From set to S-AE-ID-STEM and the IUT having sent a valid AEAnnc CREATE Request to IN-CSE } |  |

- +-----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= IN-CSE|
                     | the IUT receives a valid Response from IN-CSE containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     |AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID and  | |
                     | labels attribute indicating 'Credential-ID:None'   | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response to the AE containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | AE-ID attribute set to S-AE-ID-STEM  | |
                     |} | |

| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM } S-AE-ID-STEM = Value starting with "S" assigned by IN-CSE |  |  |
| --- | --- | --- |

TP/oneM2M/CSE/REG/CRE/031

| TP Id | TP/oneM2M/CSE/REG/CRE/031 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts an AE registration (S-AE-ID-STEM not provided by AE), communication between MN-CSE and IN-CSE |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 case a, TS-0004 [2], clause 7.4.5.2.1 |  |
| Config Id | CF04 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" } |  |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= MN-CSE|
                     | the IUT receives a valid CREATE Request from MN-CSE containing  | |
                     | Resource Type set to 10002 (AEAnnc) and  | |
                     | From set to SP-RELATIVE-AE-ID and  | |
                     | Content containing  | |
                     | AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID and  | |
                     | labels indicating 'Credential-ID:None'  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => MN-CSE|
                     | the IUT creates the AEANNC_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response to CSE containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     |AEAnnc resource containing  | |
                     | link attribute set to SP-RELATIVE-AE-ID* and  | |
                     | labels indicating 'Credential-ID:None'  | |
                     |} | |

| NOTE: SP-RELATIVE-AE-ID = {SP-RELATIVE-CSE-ID}/{AE-ID-STEM} SP-RELATIVE-AE-ID* = {SP-RELATIVE-CSE-ID}/{S-AE-ID-STEM} AE-ID-STEM = "S" S-AE-ID-STEM = Valid AE-ID-Stem starting with "S" |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/REG/CRE/032

| TP Id | TP/oneM2M/CSE/REG/CRE/032 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects an AE registration for CSEs for which the associated <serviceSubscribeNode> resource is not linked with the <serviceSubscribedAppRule> via the ruleLinks attribute of the <serviceSubscribedNode> resource |  |
| Reference | TS-0001 [1], clause 10.2.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having the serviceSubscribedNode resource For the CSE containing ruleLinks attribute not set } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing Resource Type set to 2 (AE)  | |
                     | } | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the AE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4126(APP_RULE_VALIDATION_FAILED)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————–+————-+

##### 7.2.2.2.3	DELETE Operation

###### TP/oneM2M/CSE/REG/DEL/001

| TP Id | TP/oneM2M/CSE/REG/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the delete request of <CSEBase> resource. |  |
| Reference | TS-0001 [1], clause 10.2.3.4 and [1], clause 9.6.3, TS-0004 [2], clause 7.4.3.2.4 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+—————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/REG/DEL/002

| TP Id | TP/oneM2M/CSE/REG/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that IUT accepts a <remoteCSE> delete request on TARGET_REMOTE_CSE_ADDRESS, and deletes the <remoteCSE> resource |  |
| Reference | TS-0001 [1], clause 10.2.2.4 and clause 9.6.4, TS-0004 [2], clause 7.4.4.2.1 |  |
| Config Id | CF04 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the CSE and the CSE originator having privileges to perform DELETE operation on the remoteCSE resource } |  |

- +------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid DELETE Request from CSE containing  | |
                     | To set to REMOTECSE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     |no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |and the IUT deletes the remoteCSE resource  | |
                     |} | |
+———————-+——————————————————————————————————————+————-+

###### TP/oneM2M/CSE/REG/DEL/003

| TP Id |  | TP/oneM2M/CSE/REG/DEL/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE de-registration. |  |
| Reference |  | TS-0001 [1], clause 10.1.4.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform DELETE operation on the resource AE } |  |  |

- +------------------------------------+---------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +------------------------------------+---------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT deletes the AE_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+————————————+—————————————+————-+

###### TP/oneM2M/CSE/REG/DEL/004

| TP Id |  | TP/oneM2M/CSE/REG/DEL/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> delete request on TARGET_REMOTE_CSE_ADDRESS |  |
| Reference |  | TS-0001 [1], clause 10.2.2.4, TS-0004 [2], clause 7.4.5.2.5 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS and the IUT having privileges to perform DELETE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT is triggered to send a valid DELETE Request containing To set to TARGET_REMOTE_CSE_ADDRESSS } |  | NA |  |
| then { the IUT sends a valid DELETE Request containing To set to TARGET_REMOTE_CSE_ADDRESSand From set to CSE_ID and no Content } |  | IUT => CSE |  |

TP/oneM2M/CSE/REG/DEL/005

| TP Id |  | TP/oneM2M/CSE/REG/DEL/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE de-registration when AE-ID is starting with "S" |  |
| Reference |  | TS-0001 [1], clause 10.1.4.2.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing From set to "S" and the AE having privileges to perform DELETE operation on the resource AE } |  |  |

- +--------+-------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +--------+-------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT deletes the AE_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+——–+——————————————————————-+————-+

TP/oneM2M/CSE/REG/DEL/006

| TP Id |  | TP/oneM2M/CSE/REG/DEL/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts an AE de-registration when AE-ID is starting with "S" |  |
| Reference |  | TS-0001 [1], clause 10.1.1.2.2 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 2 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing From set to "S" and the AE having privileges to perform DELETE operation on the resource AE } |  |  |

- +--------+-------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +--------+-------------------------------------------------------------------+-------------+

- |then {   |IUT => IN-CSE|
                     | the IUT sends a valid UPDATE Request containing  | |
                     | Contentcontaining  | |
                     | AEANNC_RESOURCE_ADDRESS resource containing  | |
                     | link attribute set to "INACTIVE"  | |
                     |} | |
+———————-+——–+——————————————————————-+————-+

TP/oneM2M/CSE/REG/DEL/007

| TP Id | TP/oneM2M/CSE/REG/DEL/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the response from IN-CSE for the AE de-registration when AE-ID is starting with "S" |  |
| Reference | TS-0001 [1], clause 10.1.1.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing From set to "S" and the AE having privileges to perform DELETE operation on the resource AE and the AE having sent a valid DELETE Request at AE_RESOURCE_ADDRESS resource and the IUT having sent a valid UPDATE Request to IN-CSE containing Content containing AEANNC_RESOURCE_ADDRESS resource containing link attribute set to empty } |  |

- +-------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= IN-CSE|
                     | the IUT receives a valid Response from IN-CSE containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT deletes the AE_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+————————————————————————————————-+————-+

##### 7.2.2.2.4	UPDATE Operation

###### TP/oneM2M/CSE/REG/UPD/001

| TP Id | TP/oneM2M/CSE/REG/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of <CSEBase> resource. |  |
| Reference | TS-0001 [1], clause 10.2.3.3 and clause 9.6.3, TS-0004 [2], clause 7.4.3.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE } |  |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to CSEBASE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | CSEBase resource representation  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+———————————————————————————————————–+————-+ TP/oneM2M/CSE/REG/UPD/002

| TP Id |  | TP/oneM2M/CSE/REG/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts a <remoteCSE> update request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.2.2.3, TS-0004 [2], clause 7.4.5.2.4 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS containing a RW OPTIONAL_ATTRIBUTE attribute set to VALUE_1 and the CSE having privileges to perform UPDATE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |

- +-----+------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE |
                     | the IUT receives a valid UPDATE Request containing  | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | remoteCSE resource containing  | |
                     | OPTIONAL_ATTRIBUTE attribute set to VALUE_2  | |
                     |} | |

- +-----+------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2004 (UPDATED) and  | |
                     |Content containing  | |
                     | remoteCSE resource containing  | |
                     | OPTIONAL_ATTRIBUTE attribute set to VALUE_2  | |
                     |} | |
+———————-+—–+——————————————————————————————+————-+

| TP Id | Reference | OPTIONAL_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/REG/UPD/002_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/UPD/002_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/UPD/002_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |

###### TP/oneM2M/CSE/REG/UPD/003

| TP Id |  | TP/oneM2M/CSE/REG/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT sends a <remoteCSE> update request with OPTIONAL_ATTRIBUTE attribute |  |
| Reference |  | TS-0001 [1], clause 10.2.2.3, TS-0004 [2], clause 7.4.5.2.4 |  |
| Config Id |  | CF04 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_MN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having created a resource TARGET_REMOTE_CSE_ADDRESS containing a RW OPTIONAL_ATTRIBUTE attribute set to VALU_1 and the IUT having privileges to perform UPDATE operation on the resource TARGET_REMOTE_CSE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT is triggered to send a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESS an d Content containing remoteCSE resource containing attribute OPTIONAL_ATTRIBUTE set to VALUE_2 } |  | NA |  |
| then { the IUT sends a valid UPDATE Request containing To set to TARGET_REMOTE_CSE_ADDRESSand From set to CSE_ID and Content containing remoteCSE resource containing attribute OPTIONAL_ATTRIBUTE set to VALUE_2 } |  | IUT => CSE |  |

| TP Id | PICS Selection | Reference | OPTIONAL_ ATTRIBUTE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/REG/UPD/003_ET | PICS_CSR_ET | TS-0001 [1], clause 9.6.4-2 | expirationTime |
| TP/oneM2M/CSE/REG/UPD/003_LBL | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | labels |
| TP/oneM2M/CSE/REG/UPD/003_POA | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | pointOfAccess |
| TP/oneM2M/CSE/REG/UPD/003_NL | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | nodeLink |
| TP/oneM2M/CSE/REG/UPD/003_RR | PICS_CSR_RR | TS-0001 [1], clause 9.6.4-2 | requestReachability |

#### 7.2.2.3	Data Management and Repository Function (DMR)

##### 7.2.2.3.1	RETRIEVE Operation

###### TP/oneM2M/CSE/DMR/RET/001

| TP Id |  | TP/oneM2M/CSE/DMR/RET/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +-------------------------------------------------+-----------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +-------------------------------------------------+-----------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |
+———————-+————————————————-+—————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/001_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/001_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/001_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/001_CIN | Release 1 | TS-0001 [1], clause 10.2.4.2 | 4 (contentInstance) |
| TP/oneM2M/CSE/DMR/RET/001_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/001_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/001_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/001_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/001_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/001_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/002

| TP Id | TP/oneM2M/CSE/DMR/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to retrieve the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference | TS-0001 [1], clause 10.1.2 - item 1) |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+———————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DMR/RET/003

| TP Id | TP/oneM2M/CSE/DMR/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to retrieve a resource TARGET_RESOURCE_ADDRESS under AE without having privileges for the RETRIEVE operation |  |
| Reference | TS-0001 [1], clause 10.1.2 - item 2) |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having no privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/003_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/003_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/003_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/003_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/003_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/003_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/003_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/003_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/004

| TP Id |  | TP/oneM2M/CSE/DMR/RET/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("To" option) |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +----------------------+--------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                              | the IUT receives a valid RETRIEVE Request from AE containing  | |
                              | To set to TARGET_ATTRIBUTE_ADDRESS and  | |
                              | From set to AE_ID and  | |
                              |no Content  | |
                              |} | |

- +----------------------+--------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                              | the IUT sends a valid Response containing  | |
                              | Response Status Code set to 2000 (OK) and  | |
                              | Content containing  | |
                              | RESOURCE_TYPE resource containing  | |
                              | valid ATTRIBUTE attribute and  | |
                              | no other attribute  | |
                              |} | |

| Notes: TARGET_ATTRIBUTE_ADDRESS is represented as TARGET_RESOURCE_ADDRESS#ATTRIBUTE where '#' is present between TARGET_RESOURCE_ADDRESS and ATTRIBUTE. ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below. |  |  |  |
| --- | --- | --- | --- |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/004_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/004_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/004_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/004_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/004_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/004_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/004_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/004_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/004_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/005

| TP Id |  | TP/oneM2M/CSE/DMR/RET/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("Content" option) |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +----------------+--------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | ATTRIBUTE name  | |
                     |} | |

- +----------------+--------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid ATTRIBUTE attribute and  | |
                     | no other attribute  | |
                     |} | |

| Notes: ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |
| --- | --- | --- | --- |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/005_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/005_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/005_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/005_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/005_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/005_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/005_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/005_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/005_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/006

| TP Id |  | TP/oneM2M/CSE/DMR/RET/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully multiple attributes of TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +-----------------------------+-------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontainin   | |
                     | ATTRIBUTE_1 name and  | |
                     | ATTRIBUTE_2 name  | |
                     |} | |

- +-----------------------------+-------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid ATTRIBUTE_1 attribute and  | |
                     | valid ATTRIBUTE_2 attribute and  | |
                     | no other attribute  | |
                     |} | |

| Notes: ATTRIBUTE_1 indicates common attribute for all RESOURCE_TYPE resources listed in the table below ATTRIBUTE_2 indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |
| --- | --- | --- | --- |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_ TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/006_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/006_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/006_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/006_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/006_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/006_PCH | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/006_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/006_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/006_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/007

| TP Id | TP/oneM2M/CSE/DMR/RET/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource ("single" attribute case) |  |
| Reference | TS-0001 [1], clause 10.1.2 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resourcecontaining no attribute ATTRIBUTE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | ATTRIBUTE name  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/007_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/RET/007_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/007_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/RET/007_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/RET/007_SCH/LBL | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/RET/007_PCH/LBL | Release 2 | TS-0001 [1], clause 10.2.13.3 | 15 (pollingChallenge) | labels |
| TP/oneM2M/CSE/DMR/RET/007_TS/LBL | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) | labels |
| TP/oneM2M/CSE/DMR/RET/007_TRAM/LBL | Release 4 | TS-0001 [1], clause10.2.18.3 | 50(transactionMgmt) | labels |
| TP/oneM2M/CSE/DMR/RET/007_TRAC/LBL | Release 4 | TS-0001 [1], clause10.2.18.7 | 51(transaction) | labels |

Editor Note: A common ATTRIBUTE has been chosen for all types of resource. A note could replace ATTRIBUTE column.

###### TP/oneM2M/CSE/DMR/RET/008

| TP Id | TP/oneM2M/CSE/DMR/RET/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attributes of the TARGET_RESOURCE_ADDRESS resource ("multiple" attributes case) |  |
| Reference | TS-0001 [1], clause 10.1.2 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the AE resource containing no ATTRIBUTE_1 attribute and no ATTRIBUTE_2 attribute and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | ATTRIBUTE_1 name a nd   | |
                     | ATTRIBUTE_2 name  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_1 | ATTRIBUTE_2 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/008_CNT/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_SUB/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_ACP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_GRP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_SCH/LBL/AT | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_TS/LBL/AT | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/008_TRAM/LBL/TEXT | Release 4 | TS-0001 [1], clause10.2.18.3 | 50(transactionMgmt) | labels | transactionExecuteTime |
| TP/oneM2M/CSE/DMR/RET/008_TRAC/LBL/TEXT | Release 4 | TS-0001 [1], clause10.2.18.7 | 51(transaction) | labels | transactionExecuteTime |

Editor Note: The references to the TS-0001 for the operations (CREATE, RETRIEVE) procedures of the resource type <schedule> have not been found. To check whether these procedures are missing or they have been added in newer versions (TS-0001 v1.12.0 as basis)

###### TP/oneM2M/CSE/DMR/RET/009

| TP Id | TP/oneM2M/CSE/DMR/RET/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the RETRIEVE Request of a latest resource target to a container resource that has no direct child contentInstance resources |  |
| Reference | TS-0001 [1], clause 10.2.22.1, TS-0004 [2], clause 7.4.28.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no direct child contentInstance resources } |  |
| Expected behaviour | Test events | Direction |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/DMR/RET/010

| TP Id | TP/oneM2M/CSE/DMR/RET/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the RETRIEVE Request of the oldest contentInstance target to a <container> resource |  |
| Reference | TS-0001 [1], clause 10.2.23.1, TS-0004 [2], clause 7.4.29.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform RETRIEVE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS/ol and From set to AE_ID and no Content } | IUT <= AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing contentInstance resource containing resourceName attributeset to CONTENT_INSTANCE_1 } | IUT => AE |

###### TP/oneM2M/CSE/DMR/RET/011

| TP Id | TP/oneM2M/CSE/DMR/RET/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the RETRIEVE Request of an oldest resource target to a container resource that has no direct child contentInstance resources. |  |
| Reference | TS-0001 [1], clause 10.2.23.1, TS-0004 [2], clause 7.4.29.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no direct child contentInstance resources } |  |
| Expected behaviour | Test events | Direction |

- +-------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+——————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DMR/RET/012

| TP Id |  | TP/oneM2M/CSE/DMR/RET/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the RETRIEVE Request of the latest contentInstance target to a <container> resource |  |
| Reference |  | TS-0001 [1], clause 10.2.22.1, TS-0004 [2], clause 7.4.28.2.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 1 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform RETRIEVE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |  |

- +--+----------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/la and  | |
                     | From set to AE_ID and   | |
                     |no Content  | |
                     |} | |

- +--+----------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a Response containing  | |
                     |Response Status Code set to 2000 (OK) and  | |
                     |Content containing  | |
                     | contentInstance resource containing  | |
                     | resourceName attributeset to CONTENT_INSTANCE_2  | |
                     |} | |
+———————-+–+—————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DMR/RET/013

| TP Id |  | TP/oneM2M/CSE/DMR/RET/013 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the TARGET_RESOURCE_ADDRESS resource under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +----------------------------------------+-------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +----------------------------------------+-------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |
+———————-+—————————————-+——————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/013_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/013_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/013_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/013_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/013_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/013_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/013_DLV | Release 4 | TS-0001 [1], clause 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/013_LCP | Release 3 | TS-0001 [1], clause 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/013_MGC | Release 4 | TS-0001 [1], clause 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/013_NOD | Release 4 | TS-0001 [1], clause 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/013_REQ | Release 2 | TS-0001 [1], clause 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/013_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/013_STCL | Release 4 | TS-0001 [1], clause 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/013_STCG | Release 4 | TS-0001 [1], clause 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/013_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/013_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/013_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/014

| TP Id | TP/oneM2M/CSE/DMR/RET/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to retrieve a resource TARGET_RESOURCE_ADDRESS under CSEBase without having privileges for the RETRIEVE operation |  |
| Reference | TS-0001 [1], clause 10.1.2 - item 2) |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having no privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/014_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/014_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/014_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/014_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/014_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/014_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/014_DLV | Release 4 | TS-0001 [1], clause 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/014_LCP | Release 3 | TS-0001 [1], clause 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/014_MGC | Release 4 | TS-0001 [1], clause 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/014_NOD | Release 4 | TS-0001 [1], clause 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/014_REQ | Release 2 | TS-0001 [1], clause 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/014_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/014_STCL | Release 4 | TS-0001 [1], clause 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/014_STCG | Release 4 | TS-0001 [1], clause 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/014_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/014_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/014_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/015

| TP Id |  | TP/oneM2M/CSE/DMR/RET/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("To" option) under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +-------------+----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_ATTRIBUTE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +-------------+----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid ATTRIBUTE attribute and  | |
                     | no other attribute  | |
                     |} | |

| Notes: TARGET_ATTRIBUTE_ADDRESS is represented as TARGET_RESOURCE_ADDRESS # ATTRIBUTE ATTRIBUTE indicates common attribute for all_RESOURCE_TYPE_ resources listed in the table below |  |  |  |
| --- | --- | --- | --- |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/015_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/015_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/015_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/015_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/015_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/015_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/015_DLV | Release 4 | TS-0001 [1], clause 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/015_LCP | Release 3 | TS-0001 [1], clause 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/015_MGC | Release 4 | TS-0001 [1], clause 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/015_NOD | Release 4 | TS-0001 [1], clause 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/015_REQ | Release 2 | TS-0001 [1], clause 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/015_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/015_STCL | Release 4 | TS-0001 [1], clause 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/015_STCG | Release 4 | TS-0001 [1], clause 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/015_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/015_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/015_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

Editor's Note: Update the reference of schedule resource to TS-0001 9.6.9

###### TP/oneM2M/CSE/DMR/RET/016

| TP Id |  | TP/oneM2M/CSE/DMR/RET/016 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully the ATTRIBUTE of TARGET_RESOURCE_ADDRESS resource ("Content" option) under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS} |  |  |

- +--------+---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining   | |
                     | ATTRIBUTE name  | |
                     |} | |

- +--------+---------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid ATTRIBUTE attribute and  | |
                     | no other attribute  | |
                     |} | |

| Notes: ATTRIBUTE indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |
| --- | --- | --- | --- |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/016_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/016_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/016_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/016_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/016_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/016_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/016_DLV | Release 4 | TS-0001 [1], clause 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/016_LCP | Release 3 | TS-0001 [1], clause 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/016_MGC | Release 4 | TS-0001 [1], clause 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/016_NOD | Release 4 | TS-0001 [1], clause 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/016_REQ | Release 2 | TS-0001 [1], clause 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/016_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/016_STCL | Release 4 | TS-0001 [1], clause 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/016_STCG | Release 4 | TS-0001 [1], clause 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/016_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/016_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/016_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/017

| TP Id |  | TP/oneM2M/CSE/DMR/RET/017 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully multiple attributes of TARGET_RESOURCE_ADDRESS resource under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +--------------------+---------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | ATTRIBUTE_1 name  | |
                     | and ATTRIBUTE_2 name  | |
                     |} | |

- +--------------------+---------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid ATTRIBUTE_1 attribute and  | |
                     | valid ATTRIBUTE_2 attribute and  | |
                     | no other attribute  | |
                     |} | |

| Notes: ATTRIBUTE_1 indicates common attribute for all RESOURCE_TYPE resources listed in the table below ATTRIBUTE_2 indicates common attribute for all RESOURCE_TYPE resources listed in the table below |  |  |  |
| --- | --- | --- | --- |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/017_ACP | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/017_AE | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) |
| TP/oneM2M/CSE/DMR/RET/017_CNT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/017_CSR | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/RET/017_SUB | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/017_GRP | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/017_DLV | Release 4 | TS-0001 [1], clause 10.2.5.3 | 6 (delivery) |
| TP/oneM2M/CSE/DMR/RET/017_LCP | Release 3 | TS-0001 [1], clause 10.2.10.1.2 | 10 (locationPolicy) |
| TP/oneM2M/CSE/DMR/RET/017_MGC | Release 4 | TS-0001 [1], clause 10.2.8.3 | 12 (mgmtCmd) |
| TP/oneM2M/CSE/DMR/RET/017_NOD | Release 4 | TS-0001 [1], clause 10.2.14.2 | 14 (node) |
| TP/oneM2M/CSE/DMR/RET/017_REQ | Release 2 | TS-0001 [1], clause 10.2.20.2 | 17 (request) |
| TP/oneM2M/CSE/DMR/RET/017_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/017_STCL | Release 4 | TS-0001 [1], clause 10.2.15.11 | 21 (statsCollect) |
| TP/oneM2M/CSE/DMR/RET/017_STCG | Release 4 | TS-0001 [1], clause 10.2.15.3 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/RET/017_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/017_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/017_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) |

###### TP/oneM2M/CSE/DMR/RET/018

| TP Id | TP/oneM2M/CSE/DMR/RET/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource ("single" attribute case) under CSEBase |  |
| Reference | TS-0001 [1], clause 10.1.2 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resourcecontaining no attribute ATTRIBUTE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | ATTRIBUTE name  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/018_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/018_AE/LBL | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) | labels |
| TP/oneM2M/CSE/DMR/RET/018_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/RET/018_CSR/LBL | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) | labels |
| TP/oneM2M/CSE/DMR/RET/018_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/RET/018_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/RET/018_DLV/LBL | Release 4 | TS-0001 [1], clause 10.2.5.3 | 6 (delivery) | labels |
| TP/oneM2M/CSE/DMR/RET/018_LCP/LBL | Release 3 | TS-0001 [1], clause 10.2.10.1.2 | 10 (locationPolicy) | labels |
| TP/oneM2M/CSE/DMR/RET/018_MGC/LBL | Release 4 | TS-0001 [1], clause 10.2.8.3 | 12 (mgmtCmd) | labels |
| TP/oneM2M/CSE/DMR/RET/018_NOD/LBL | Release 4 | TS-0001 [1], clause 10.2.14.2 | 14 (node) | labels |
| TP/oneM2M/CSE/DMR/RET/018_REQ/LBL | Release 2 | TS-0001 [1], clause 10.2.20.2 | 17 (request) | labels |
| TP/oneM2M/CSE/DMR/RET/018_SCH/LBL | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/RET/018_STCL/LBL | Release 4 | TS-0001 [1], clause 10.2.15.11 | 21 (statsCollect) | labels |
| TP/oneM2M/CSE/DMR/RET/018_STCG/LBL | Release 4 | TS-0001 [1], clause 10.2.15.3 | 22 (statsConfig) | labels |
| TP/oneM2M/CSE/DMR/RET/018_TS | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) | labels |
| TP/oneM2M/CSE/DMR/RET/018_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgmt) | labels |
| TP/oneM2M/CSE/DMR/RET/018_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) | labels |

###### TP/oneM2M/CSE/DMR/RET/019

| TP Id | TP/oneM2M/CSE/DMR/RET/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to retrieve a non-existing attribute of the TARGET_RESOURCE_ADDRESS resource ("multiple" attributes case) under CSEBase |  |
| Reference | TS-0001 [1], clause 10.1.2 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the <CSEBase> resourcecontaining no ATTRIBUTE_1 attribute and no ATTRIBUTE_2 attribute and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | ATTRIBUTE_1 name  | |
                     | and ATTRIBUTE_2 name  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————+————-+

| TP Id | PARENT_R ELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_1 | ATTRIBUTE_2 |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/019_ACP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.21.2 | 1 (accessControlPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_AE/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.1.2 | 2 (AE) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_CNT/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.4.2 | 3 (container) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_CSR/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.2.2 | 16 (remoteCSE) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_SUB/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.11.3 | 23 (subscription) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_GRP/LBL/AT | Release 1 | TS-0001 [1], clause 10.2.7.3 | 9 (group) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_DLV/LBL/AT | Release 4 | TS-0001 [1], clause 10.2.5.3 | 6 (delivery) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_LCP/LBL/AT | Release 3 | TS-0001 [1], clause 10.2.10.1.2 | 10 (locationPolicy) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_MGC/LBL/AT | Release 4 | TS-0001 [1], clause 10.2.8.3 | 12 (mgmtCmd) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_NOD/LBL/AT | Release 4 | TS-0001 [1], clause 10.2.14.2 | 14 (node) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_REQ/LBL/AT | Release 3 | TS-0001 [1], clause 10.2.20.2 | 17 (request) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_SCH/LBL/AT | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_STCL/LBL/AT | Release 4 | TS-0001 [1], clause 10.2.15.11 | 21 (statsCollect) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_STCG/LBL/AT | Release 4 | TS-0001 [1], clause 10.2.15.3 | 22 (statsConfig) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_TS/LBL/AT | Release 3 | TS-0001 [1], clause 10.2.30.2 | 29 (timeSeries) | labels | announceTo |
| TP/oneM2M/CSE/DMR/RET/019_TRAM/LBL/TEXT | Release 4 | TS-0001 [1], clause 10.2.18.3 | 50(transactionMgm) | labels | transactionExecuteTime |
| TP/oneM2M/CSE/DMR/RET/019_TRAC/LBL/TEXT | Release 4 | TS-0001 [1], clause 10.2.18.7 | 51(transaction) | labels | transactionExecuteTime |

###### TP/oneM2M/CSE/DMR/RET/020

| TP Id | TP/oneM2M/CSE/DMR/RET/020 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a RETRIEVE Request target to TARGET_ RESOURCE_ADDRESS resource when the Result Content set to RESULT_CONTENT |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 7.3.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Contentand  | |
                     | Result Content set to RESULT_CONTENT  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+——————————————————————————————————————————+————-+

| TP Id | Reference | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/020_RCN/0 | TS-0004 [2], clause 6.3.4.2.7 | 0 (Nothing) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/2 |  | 2 (hierarchical address) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/3 |  | 3 (attributes and hierarchichal address) |
| TP/oneM2M/CSE/DMR/RET/020_RCN/9 |  | 9 (modified attributes) |

###### TP/oneM2M/CSE/DMR/RET/021

| TP Id |  | TP/oneM2M/CSE/DMR/RET/021 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS} |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { the IUT receives a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID andno Contentand Result Content set to 1 (attributes)} |  | IUT <= AE |
|  |  |  | then { the IUT sends a valid Response containing Response Status Code set to 2000 (OK) and Content containing RESOURCE_TYPE resource containing attributes and no hierarchichal address and no child resources and no child resource references} |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/021_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/021_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/021_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/021_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/021_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/021_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/021_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/021_TRAM | Release 4 | 50 (TransactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/021_TRAC | Release 4 | 51 (Transaction) |

###### TP/oneM2M/CSE/DMR/RET/022

| TP Id | TP/oneM2M/CSE/DMR/RET/022 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only attributes and child resources of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 4 (attributes and child resources) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Contentand  | |
                     | Result Content set to 4 (attributes and child resources)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | attributes and  | |
                     | no hierarchichal address and  | |
                     | child resources and  | |
                     | no child resource references  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/022_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/022_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/022_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/022_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/022_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/022_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/022_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/022_TRAM | Release 4 | 50 (TransactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/022_TRAC | Release 4 | 51 (Transaction) |

###### TP/oneM2M/CSE/DMR/RET/023

| TP Id | TP/oneM2M/CSE/DMR/RET/023 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only attributes and child resource references of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 5 (attributes and child resource references) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content and  | |
                     | Result Content set to 5 (attributes and child resource references)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | attributes and  | |
                     | no hierarchichal address and  | |
                     | no child resources and  | |
                     | child resource references  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/023_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/023_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/023_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/023_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/023_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/023_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/023_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/023_TRAM | Release 4 | 50 (TransactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/023_TRAC | Release 4 | 51 (Transaction) |

###### TP/oneM2M/CSE/DMR/RET/024

| TP Id | TP/oneM2M/CSE/DMR/RET/024 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only child resource references of TARGET_RESOURCE_ADDRESS resource when Result Content is set to 6 (child resource references) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Contentand  | |
                     | Result Content set to 6 (child resource references)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | resourceRefList element containing  | |
                     | child resource reference  | |
                     |} | |
+———————-+———————————————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/024_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/024_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/024_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/024_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/024_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/024_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/024_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/024_TRAM | Release 4 | 50 (TransactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/024_TRAC | Release 4 | 51 (Transaction) |

###### TP/oneM2M/CSE/DMR/RET/025

| TP Id | TP/oneM2M/CSE/DMR/RET/025 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only child resources of TARGET_RESOURCE_ADDRESS resource when Result Content is set to 8 (child resources) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Contentand  | |
                     | Result Content set to 8 (child resources)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | CHILD_RESOURCE_TYPE resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/025_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/025_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/025_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/025_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/025_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/025_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/025_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/RET/025_TRAM | Release 4 | 50 (TransactionMgmt) |
| TP/oneM2M/CSE/DMR/RET/025_TRAC | Release 4 | 51 (Transaction) |

TP/oneM2M/CSE/DMR/RET/026

| TP Id | TP/oneM2M/CSE/DMR/RET/026 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a RETRIEVE request to ORIGINAL_RESOURCE_ADDRESS resource when receiving a RETRIEVE request with Result Content set to 7 (original resource) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered the CSE and the IUT having a resource ANNC_ORIGINAL_RESOURCE_ADDRESS containing link set to ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource ANNC_ORIGINAL_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to ANNC_ORIGINAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Contentand  | |
                     | Result Content set to 7 (original resource)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid RETRIEVE Request to CSE containing  | |
                     | To set to ORIGINAL_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID or IUT CSE ID?? and  | |
                     |no Content  | |
                     |} | |
+———————-+———————————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/026_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/026_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/026_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/026_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/026_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/026_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/026_PCH | Release 3 | 15 (pollingChannel) |

TP/oneM2M/CSE/DMR/RET/027

| TP Id | TP/oneM2M/CSE/DMR/RET/027 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully the representation of the original resource of the ANNC_ORIGINAL_RESOURCE_ADDRESS resource when receiving a RETRIEVE request with Result Content is set to 7 (original resource) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered the CSE and the IUT having a resource ANNC_ORIGINAL_RESOURCE_ADDRESS containing link set to ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource ANNC_ORIGINAL_RESOURCE_ADDRESS and the IUT having received a valid RETRIEVE Request containing To set to ANNC_ORIGINAL_RESOURCE_ADDRESS and Result Content set to 7 (original resource) and the IUT having sent a valid RETRIEVE Request containing To set to ORIGINAL_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid Response from CSE containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | a valid resource representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a valid Response to AE containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | a valid resource representation  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/RET/027_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/RET/027_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/RET/027_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/RET/027_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/RET/027_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/RET/027_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/RET/027_PCH | Release 3 | 15 (pollingChannel) |

###### TP/oneM2M/CSE/DMR/RET/028

| TP Id | TP/oneM2M/CSE/DMR/RET/028 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully returns the privileges of the ORIGINATOR on the TARGET_RESOURCE_ADDRESS when the RETRIEVE request contains Result Content set to permissions |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7, clause 7.5.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having resource at TARGET_RESOURCE_ADDRESS and AE having the privileges to perform the RETRIEVE operation on TARGET_RESOURCE_ADDRESS and AE having privileges to perform the RETRIEVE operation on the <accessControlPolicy> resources mentioned in accessControlPolicyIDs attribute of TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to 12 (permissions)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | permissions containing  | |
                     | representation of access control privileges  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/DMR/RET/029

| TP Id | TP/oneM2M/CSE/DMR/RET/029 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a RETRIEVE request from the AE containing Result Content set to permissions when AE has no RETRIEVE privileges on the <accessControlPolicy> resources associated with the TARGET_RESOURCE_ADDRESS resource |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having resource at TARGET_RESOURCE_ADDRESS and AE having the privileges to perform the RETRIEVE operation on TARGET_RESOURCE_ADDRESS and AE not having the privileges to perform the RETRIEVE operation on <accessControlPolicy> resources mentioned in accessControlPolicyIDs attribute of TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content and  | |
                     | Result Content set to 12 (permissions)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4130 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————–+————-+

##### 7.2.2.3.2	UPDATE Operation

###### TP/oneM2M/CSE/DMR/UPD/001

| TP Id | TP/oneM2M/CSE/DMR/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the value of the attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT updates the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |
+———————-+—————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYP E | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/001_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_SCH/LBL | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_PCH/LBL | Release 2 | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_TS/LBL | Release 3 | TS-0001 [1], clause 10.2.13.4 | 29 (timeSeries) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_TRAM/LBL | Release 4 | TS-0001 [1], clause 10.2.18.4 | 50 (transactionMgmt) | labels |
| TP/oneM2M/CSE/DMR/UPD/001_TRAC/LBL | Release 4 | TS-0001 [1], clause 10.2.18.8 | 51 (transaction) | labels |

###### TP/oneM2M/CSE/DMR/UPD/002

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT adds successfully the attribute ATTRIBUTE_NAME to the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing no optional RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resourcecontaining ATTRIBUTE_NAME attribute set to VALUE_1 } |  | IUT <= AE |  |
| then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resourcecontaining ATTRIBUTE_NAME attribute set to VALUE_1 } |  | IUT => AE |  |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/002_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_GRP/LBL | Release 1 | PICS_GRP_LBL | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_SCH/LBL | Release 3 |  | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_PCH/LBL | Release 2 |  | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_TS/LBL | Release 3 |  | TS-0001 [1], clause 10.2.30.3 | 29 (timeSeries) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_TRAM/LBL | Release 4 |  | TS-0001 [1], clause 10.2.18.4 | 50 (transactionMgmt) | labels |
| TP/oneM2M/CSE/DMR/UPD/002_TRAC/LBL | Release 4 |  | TS-0001 [1], clause 10.2.18.8 | 51 (transaction) | labels |

###### TP/oneM2M/CSE/DMR/UPD/003

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes successfully the attribute ATTRIBUTE_NAME from the TARGET_RESOURCE_ADDRESS resource |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing an optional RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resourcecontaining ATTRIBUTE_NAME attribute set to NULL } |  | IUT <= AE |  |
| then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resourcecontaining ATTRIBUTE_NAME attribute set to NULL } |  | IUT => AE |  |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/003_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_GRP/LBL | Release 1 | PICS_GRP_LBL | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_SCH/LBL | Release 3 |  | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_PCH/LBL | Release 2 |  | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_TS/LBL | Release 3 |  | TS-0001 [1], clause 10.2.30.3 | 29 (timeSeries) | labels |
| TP/oneM2M/CSE/DMR/UPD/003_TRAM/LBL | Release 4 |  | TS-0001 [1], clause 10.2.18.4 | 50 (transactionMgmt) | lables |
| TP/oneM2M/CSE/DMR/UPD/003_TRAC/LBL | Release 4 |  | TS-0001 [1], clause 10.2.18.8 | 51 (transaction) | lables |

###### TP/oneM2M/CSE/DMR/UPD/004

| TP Id | TP/oneM2M/CSE/DMR/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the value of the attribute ATTRIBUTE_NAME_1 , and creates the ATTRIBUTE_NAME_2 and deletes the ATTRIBUTE_NAME_3 of the TARGET_RESOURCE_ADDRESS resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME_1 attribute set to VALUE_1 and no optional RW ATTRIBUTE_NAME_2 attribute and an optional RW ATTRIBUTE_NAME_3 attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
| when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resourcecontaining ATTRIBUTE_NAME_1 attibute set to VALUE_2 and ATTRIBUTE_NAME_2 attribute set to VALUE_3 and ATTRIBUTE_NAME_3 attribute set to NULL } | IUT <= AE |  |
| then { the IUT updates the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resourcecontaining ATTRIBUTE_NAME_1 attribute set to VALUE_2 and ATTRIBUTE_NAME_2 attribute set to VALUE_ 3 and ATTRIBUTE_NAME_3 attribute set to NULL } | IUT => AE |  |
| Editor's Note: Clarification is required in term of the test objective!! |  |  |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME_1 | ATTRIBUTE_NAME_2 | ATTRIBUTE_NAME_3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/004_CNT/ET/MNI/LBL | Release 1 | PICS_CNT_MNI and PICS_CNT_LBL | TS-0001 [1], clause 10.2.4.3 | 3 (container) | expirationTime | maxNrOfInstances | labels |
| TP/oneM2M/CSE/DMR/UPD/004_ACP/PVS/PV/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | selfPrivileges | privileges | labels |
| TP/oneM2M/CSE/DMR/UPD/004_ACP/PV/AT/LBL | Release 3 | PICS_ACP_AT and PICS_ACP_LBL | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | privileges | announceTo | labels |
| TP/oneM2M/CSE/DMR/UPD/004_SUB/ET/LBL/EXC | Release 1 | PICS_SUB_LBL and PICS_SUB_EXC | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | expirationTime | labels | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/004_GRP/ET/GN/LBL | Release 1 | PICS_GRP_GN and PICS_GRP_LBL | TS-0001 [1], clause 10.2.7.4 | 9 (group) | expirationTime | groupName | labels |
| TP/oneM2M/CSE/DMR/UPD/004_SCH/SE/AT/LBL | Release 3 |  | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | scheduleElement | announceTo | labels |
| TP/oneM2M/CSE/DMR/UPD/004_TS/ET/MNI/LBL | Release 3 |  | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | expirationTime | maxNrOfInstances | labels |

###### TP/oneM2M/CSE/DMR/UPD/005

| TP Id | TP/oneM2M/CSE/DMR/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to update an attribute of a TARGET_RESOURCE_ADDRESS resource when the resource does not exist |  |
| Reference | TS-0001 [1], clause 10.1.3 - item 15) |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having created a resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing valid representation resourcecontaining valid attribute } | IUT <= AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 4004 (NOT_FOUND) } | IUT => AE |

###### TP/oneM2M/CSE/DMR/UPD/006

| TP Id | TP/oneM2M/CSE/DMR/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to update the attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource without having privileges for the UPDATE operation |  |
| Reference | TS-0001 [1], clause 10.1.3 - 16) |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a RW ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having no privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resourcecontaining  | |
                     | ATTRIBUTE_NAME set to VALUE_2  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not update the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/006_CNT/LBL | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_ACP/LBL | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_SUB/LBL | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_GRP/LBL | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_SCH/LBL | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | labels |
| TP/oneM2M/CSE/DMR/UPD/005_TS/LBL | Release 3 | TS-0001 [1], clause 10.2.30.3 | 29 (timeSeries) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_TRAM/LBL | Release 4 | TS-0001 [1], clause 10.2.18.4 | 50 (transactionMgmt) | labels |
| TP/oneM2M/CSE/DMR/UPD/006_TRAC/LBL | Release 4 | TS-0001 [1], clause 10.2.18.8 | 51 (transaction) | labels |

###### TP/oneM2M/CSE/DMR/UPD/007

| TP Id | TP/oneM2M/CSE/DMR/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to update a non-RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a non-RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resourcecontaining  | |
                     | valid ATTRIBUTE_NAME attribute  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not update the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/007_CNT/CT | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_ACP/CT | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_SUB/CT | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_GRP/CT | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_SCH/CT | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_PCH/CT | Release 2 | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_TS/CT | Release 3 | TS-0001 [1], clause 10.2.30.3 | 29 (timeSeries) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_TRAM/CT | Release 4 | TS-0001 [1], clause 10.2.18.4 | 50 (transactionMgmt) | creationTime |
| TP/oneM2M/CSE/DMR/UPD/007_TRAC/CT | Release 4 | TS-0001 [1], clause 10.2.18.8 | 51 (transaction) | creationTime |

###### TP/oneM2M/CSE/DMR/UPD/008

| TP Id | TP/oneM2M/CSE/DMR/UPD/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to delete a mandatory RW attribute ATTRIBUTE_NAME of a TARGET_RESOURCE_ADDRESS resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a mandatory RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resourcecontaining ATTRIBUTE_NAME attribute set to NULL } | IUT <= AE |
|  | then { the IUT does not update the TARGET_RESOURCE_ADDRESS resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } | IUT => AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/008_ACP/PVS | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | selfPrivileges |
| TP/oneM2M/CSE/DMR/UPD/008_SUB/NU | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | notificationURI |
| TP/oneM2M/CSE/DMR/UPD/008_GRP/MNI | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/008_SCH/SCE | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | expirationTime |

Editor's Note: Release 2 test cases which are having expirationTime as attribute_name need to be verified.

###### TP/oneM2M/CSE/DMR/UPD/009

| TP Id | TP/oneM2M/CSE/DMR/UPD/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to update a RW attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource with an UNACCEPTABLE_VALUE |  |
| Reference | TS-0001 [1], clause 10.1.3 - item 3) |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a valid RW ATTRIBUTE_NAME attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resourcecontaining  | |
                     | not supported ATTRIBUTE_NAME attribute  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not update the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE_NAME |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/009_CNT/EXC | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_ACP/EXC | Release 1 | TS-0001 [1], clause 10.2.21.3 | 1 (accessControlPolicy) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_SUB/MNI | Release 1 | TS-0001 [1], clause 10.2.11.4 | 23 (subscription) | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/009_GRP/EXC | Release 1 | TS-0001 [1], clause 10.2.7.4 | 9 (group) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_SCH/EXC | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_PCH/EXC | Release 2 | TS-0001 [1], clause 10.2.13.4 | 15 (pollingChannel) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_TS/EXC | Release 3 | TS-0001 [1], clause 10.2.30.3 | 29 (timeSeries) | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/009_TRAM/EXT | Release 4 | TS-0001 [1],, clause 10.2.18.4 | 50 (transactionMgmt) | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/009_TRAC/EXT | Release 4 | TS-0001 [1],, clause 10.2.18.8 | 51 (transaction) | expirationTime |

###### TP/oneM2M/CSE/DMR/UPD/010

| TP Id | TP/oneM2M/CSE/DMR/UPD/010 |  |
| --- | --- | --- |
| Test objective | Check that the stateTag attribute of a RESOURCE_TYPE resource is increased when an update operation has been performed on its child resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing CHILD_RESOURCE_TYPE resource containing ATTRIBUTE_NAME attribute set to VALUE_1 and the AE having privileges to perform UPDATE operation on the TARGET_CHILD_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_CHILD_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | CHILD_RESOURCE_TYPE resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT increments the stateTag attribute of RESOURCE_TYPE resource   | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+———————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/010_CNT | Release 1 | TS-0001 [1], clause 10.2.4.3 | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/010_TS | Release 3 | TS-0001 [1], clause 10.2.30.3 | 29 (timeSeries) |

###### TP/oneM2M/CSE/DMR/UPD/011

| TP Id | TP/oneM2M/CSE/DMR/UPD/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE Request of an existing RESOURCE_TYPE resource with error "OPERATION_NOT_ALLOWED" |  |
| Reference | TS-0001 [1], clause 10.2.19.4, TS-0004 [2], clause 7.3.2.1 and clause 7.4.8.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a _PARENT RESOURCE_TYPE resource containing a RESOURCE_TYPE resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Expected behaviour |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE representation } |  |
|  | then { the IUT does not update the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 4005 (OPERATION_NOT_ALLOWED) } |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | PARENT_RESOURCE_TYPE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/011_CNT/CIN | Release 1 | TS-0001 [1], clause 10.2.4.3 | 4 (contentInstance) | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/011_TS/TSI | Release 3 | TS-0001 [1], clause 10.2.31.3 | 30 (timeSeriesInstance) | 29 (timeSeries) |

###### TP/oneM2M/CSE/DMR/UPD/012

| TP Id | TP/oneM2M/CSE/DMR/UPD/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE Request of a latest resource as a direct child of a <container> resource with error "OPERATION_NOT_ALLOWED" |  |
| Reference | TS-0001 [1], clause 10.2.22, TS-0004 [2], clause 7.4.28.2.4 and clause 7.3.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing a contentInstance resource containing valid creationTime attribute set to the most recent resource creation time } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives an UPDATE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/la and  | |
                     | From set to AE_ID and  | |
                     |no Content   | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT does not update the CONTAINER_RESOURCE_ADDRESS/la resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/DMR/UPD/013

| TP Id | TP/oneM2M/CSE/DMR/UPD/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE Request of an oldest resource as a direct child of a <container> resource with error "OPERATION_NOT_ALLOWED" |  |
| Reference | TS-0001 [1], clause 10.2.23, TS-0004 [2], clause 7.4.29.2.4 and clause 7.3.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing contentInstance resource(s) } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/ol and  | |
                     | From set to AE_ID and  | |
                     |no Content   | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT does not update the CONTAINER_RESOURCE_ADDRESS/ol resource  | |
                     | and the IUT sends a Response containing  | |
                     |Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/DMR/UPD/014

| TP Id | TP/oneM2M/CSE/DMR/UPD/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the value of the optional attribute OPTIONAL_ATTRIBUTE of the RESOURCE _ resource |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE _ resource containing OPTIONAL_ATTRIBUTE attribute set to VALUE } | IUT <= AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resourcecontaining O_PTIONAL_ATTRIBUTE_ attribute set to VALUE } | IUT => AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/ACPI | Release 1 | PICS_CNT_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | container | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/ET | Release 1 | PICS_CNT_ET | TS-0001 [1], clause 9.6.1.3.1-1 | container | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | container | labels |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MNI | Release 1 | PICS_CNT_MNI | TS-0001 [1], clause 9.6.6-2 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MBS | Release 1 | PICS_CNT_MBS | TS-0001 [1], clause 9.6.6-2 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/MIA | Release 1 | PICS_CNT_MIA | TS-0001 [1], clause 9.6.6-2 | container | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/014_CNT/OR | Release 1 | PICS_CNT_OR | TS-0001 [1], clause 9.6.6-2 | container | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | accessControlPolicy | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | accessControlPolicy | labels |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ACPI | Release 1 | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | subscription | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | subscription | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | subscription | labels |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/ENC | Release 1 | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8-2 | subscription | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/EXC | Release 1 | PICS_SUB_EXC | TS-0001 [1], clause 9.6.8-2 | subscription | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/GPI | Release 1 | PICS_SUB_GPI | TS-0001 [1], clause 9.6.8-2 | subscription | groupID |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NFU | Release 1 | PICS_SUB_NFU | TS-0001 [1], clause 9.6.8-2 | subscription | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/BN | Release 1 | PICS_SUB_BN | TS-0001 [1], clause 9.6.8-2 | subscription | batchNotify |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/RL | Release 1 | PICS_SUB_RL | TS-0001 [1], clause 9.6.8-2 | subscription | rateLimit |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/PN | Release 1 | PICS_SUB_PN | TS-0001 [1], clause 9.6.8-2 | subscription | pendingNotification |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NSP | Release 1 | PICS_SUB_NSP | TS-0001 [1], clause 9.6.8-2 | subscription | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/LN | Release 1 | PICS_SUB_LN | TS-0001 [1], clause 9.6.8-2 | subscription | latestNotify |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NCT | Release 1 | PICS_SUB_NCT | TS-0001 [1], clause 9.6.8-2 | subscription | notificationContentType |
| TP/oneM2M/CSE/DMR/UPD/014_SUB/NEC | Release 1 | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8-2 | subscription | notificationEventCat |
| TP/oneM2M/CSE/DMR/UPD/014_STCG/LBL | Release 4 |  | TS-0001 [1], clause 9.6.23-2 | statsConfig | labels |
| TP/oneM2M/CSE/DMR/UPD/014_STCG/ACPI | Release 4 |  | TS-0001 [1], clause 9.6.23-2 | statsConfig | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_TS/ACPI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/014_TS/LBL | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | labels |
| TP/oneM2M/CSE/DMR/UPD/014_TS/MNI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | maximumNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/014_TS/MBS | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/014_TS/MIA | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/014_TS/PEI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | periodicInterval |
| TP/oneM2M/CSE/DMR/UPD/014_TS/MDD | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | missingDataDetect |
| TP/oneM2M/CSE/DMR/UPD/014_TS/MDN | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | missingDataMaxNr |
| TP/oneM2M/CSE/DMR/UPD/014_TS/MDT | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | missingDataDetectTimer |
| TP/oneM2M/CSE/DMR/UPD/014_TS/OR | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | ontologyRef |

###### TP/oneM2M/CSE/DMR/UPD/015

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/015 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT updates successfully the value of the optional attribute OPTIONAL_ATTRIBUTE of the RESOURCE _ resource under CSEBase |  |
| Reference |  | TS-0001 [1], clause 10.1.3 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource containing no OPTIONAL_ATTRIBUTE attribute and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +--+------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE _ resource containing  | |
                     | OPTIONAL_ATTRIBUTE attribute set to VALUE  | |
                     |} | |

- +--+------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT updates the RESOURCE_TYPE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resourcecontaining  | |
                     | O_PTIONAL_ATTRIBUTE_ attribute set to VALUE  | |
                     |} | |
+———————-+–+————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/015_CSR/LBL | Release 1 | PICS_CSR_LBL | TS-0001 [1], clause 9.6.4-2 | remoteCSE | labels |
| TP/oneM2M/CSE/DMR/UPD/015_CSR/POA | Release 1 | PICS_CSR_POA | TS-0001 [1], clause 9.6.4-2 | remoteCSE | pointOfAccess |
| TP/oneM2M/CSE/DMR/UPD/015_CSR/NL | Release 1 | PICS_CSR_NL | TS-0001 [1], clause 9.6.4-2 | remoteCSE | nodeLink |
| TP/oneM2M/CSE/DMR/UPD/015_AE/LBL | Release 1 | PICS_AE_LBL | TS-0001 [1], clause 9.6.5-2 | AE | labels |
| TP/oneM2M/CSE/DMR/UPD/015_AE/ACPI | Release 1 | PICS_AE_ACPI | TS-0001 [1], clause 9.6.5-2 | AE | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_AE/APN | Release 1 | PICS_AE_APN | TS-0001 [1], clause 9.6.5-2 | AE | AppName |
| TP/oneM2M/CSE/DMR/UPD/015_AE/POA | Release 1 | PICS_AE_POA | TS-0001 [1], clause 9.6.5-2 | AE | pointOfAccess |
| TP/oneM2M/CSE/DMR/UPD/015_AE/OR | Release 1 | PICS_AE_OR | TS-0001 [1], clause 9.6.5-2 | AE | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 9.6.6-2 | container | labels |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/ACPI | Release 1 | PICS_CNT_ACPI | TS-0001 [1], clause 9.6.6-2 | container | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/MNI | Release 1 | PICS_CNT_MNI | TS-0001 [1], clause 9.6.6-2 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/MBS | Release 1 | PICS_CNT_MBS | TS-0001 [1], clause 9.6.6-2 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/MIA | Release 1 | PICS_CNT_MIA | TS-0001 [1], clause 9.6.6-2 | container | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/OR | Release 1 | PICS_CNT_OR | TS-0001 [1], clause 9.6.6-2 | container | ontologyRef |
| TP/oneM2M/CSE/DMR/UPD/015_CNT/LI | Release 1 | PICS_CNT_LI | TS-0001 [1], clause 9.6.6-2 | container | locationID |
| TP/oneM2M/CSE/DMR/UPD/015_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | labels |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/ACPI | Release 1 | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.8-2 | subscription | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 9.6.8-2 | subscription | labels |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/ENC | Release 1 | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8-2 | subscription | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/EXC | Release 1 | PICS_SUB_EXC | TS-0001 [1], clause 9.6.8-2 | subscription | expirationCounter |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/GPI | Release 1 | PICS_SUB_GPI | TS-0001 [1], clause 9.6.8-2 | subscription | groupID |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/NFU | Release 1 | PICS_SUB_NFU | TS-0001 [1], clause 9.6.8-2 | subscription | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/BN | Release 1 | PICS_SUB_BN | TS-0001 [1], clause 9.6.8-2 | subscription | batchNotify |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/RL | Release 1 | PICS_SUB_RL | TS-0001 [1], clause 9.6.8-2 | subscription | rateLimit |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/PN | Release 1 | PICS_SUB_PN | TS-0001 [1], clause 9.6.8-2 | subscription | pendingNotification |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/NSP | Release 1 | PICS_SUB_NSP | TS-0001 [1], clause 9.6.8-2 | subscription | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/LN | Release 1 | PICS_SUB_LN | TS-0001 [1], clause 9.6.8-2 | subscription | latestNotify |
| TP/oneM2M/CSE/DMR/UPD/015_SUB/NEC | Release 1 | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8-2 | subscription | notificationEventCat |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/LBL | Release 1 | PICS_GRP_LBL | TS-0001 [1], clause 9.6.13-2 | group | labels |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/ACPI | Release 1 | PICS_GRP_ACPI | TS-0001 [1], clause 9.6.13-2 | group | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/MACP | Release 1 | PICS_GRP_MACP | TS-0001 [1], clause 9.6.13-2 | group | membersAccessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_GRP/GN | Release 1 | PICS_GRP_GN | TS-0001 [1], clause 9.6.13-2 | group | groupName |
| TP/oneM2M/CSE/DMR/UPD/015_NOD/LBL | Release 4 |  | TS-0001 [1], clause 9.6.18-2 | node | labels |
| TP/oneM2M/CSE/DMR/UPD/015_NOD/ACPI | Release 4 |  | TS-0001 [1], clause 9.6.18-2 | node | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_NOD/HCL | Release 4 |  | TS-0001 [1], clause 9.6.18-2 | node | hostedCSELink |
| TP/oneM2M/CSE/DMR/UPD/015_TS/ACPI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/UPD/015_TS/LBL | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | labels |
| TP/oneM2M/CSE/DMR/UPD/015_TS/MNI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | maximumNrOfInstances |
| TP/oneM2M/CSE/DMR/UPD/015_TS/MBS | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/015_TS/MIA | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | maxInstanceAge |
| TP/oneM2M/CSE/DMR/UPD/015_TS/PEI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | periodicInterval |
| TP/oneM2M/CSE/DMR/UPD/015_TS/MDD | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | missingDataDetect |
| TP/oneM2M/CSE/DMR/UPD/015_TS/MDN | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | missingDataMaxNr |
| TP/oneM2M/CSE/DMR/UPD/015_TS/MDT | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | missingDataDetectTimer |
| TP/oneM2M/CSE/DMR/UPD/015_TS/OR | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | timeSeries | ontologyRef |

###### TP/oneM2M/CSE/DMR/UPD/016

| TP Id | TP/oneM2M/CSE/DMR/UPD/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the value of the attribute MANDATORY_ATTRIBUTE of the RESOURCE _ resource under CSEBase |  |
| Reference | TS-0001 [1], clause 10.1.3 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE under the CSEBase resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE _ resource containing MANDATORY_ATTRIBUTE attribute set to VALUE } | IUT <= AE |
|  | then { the IUT updates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2004 (UPDATED) and Content containing RESOURCE_TYPE resourcecontaining MANDATORY __ATTRIBUTE_ attribute set to VALUE } | IUT => AE |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | MANDATORY_ATTRIBUTE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/016_CSR/ET | Release 1 | TS-0001 [1], clause 9.6.4-2 | remoteCSE | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_CSR/RR | Release 1 | TS-0001 [1], clause 9.6.4-2 | remoteCSE | requestReachability |
| TP/oneM2M/CSE/DMR/UPD/016_AE/ET | Release 1 | TS-0001 [1], clause 9.6.5-2 | AE | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_AE/RR | Release 1 | TS-0001 [1], clause 9.6.5-2 | AE | requestReachability |
| TP/oneM2M/CSE/DMR/UPD/016_CNT/ET | Release 1 | TS-0001 [1], clause 9.6.6-2 | container | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_ACP/ET | Release 1 | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_ACP/PV | Release 1 | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | privileges |
| TP/oneM2M/CSE/DMR/UPD/016_ACP/PVS | Release 1 | TS-0001 [1], clause 9.6.2-2 | accessControlPolicy | selfPrivileges |
| TP/oneM2M/CSE/DMR/UPD/016_SUB/ET | Release 1 | TS-0001 [1], clause 9.6.8-2 | subscription | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_SUB/NU | Release 1 | TS-0001 [1], clause 9.6.8-2 | subscription | notificationURI |
| TP/oneM2M/CSE/DMR/UPD/016_SUB/NCT | Release 1 | TS-0001 [1], clause 9.6.8-2 | subscription | notificationContentType |
| TP/oneM2M/CSE/DMR/UPD/016_GRP/ET | Release 1 | TS-0001 [1], clause 9.6.13-2 | group | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_GRP/MNM | Release 1 | TS-0001 [1], clause 9.6.13-2 | group | maxNrOfMembers |
| TP/oneM2M/CSE/DMR/UPD/016_GRP/MID | Release 1 | TS-0001 [1], clause 9.6.13-2 | group | memberIDs |
| TP/oneM2M/CSE/DMR/UPD/016_NOD/ET | Release 4 | TS-0001 [1], clause 9.6.18-2 | node | expirationTime |
| TP/oneM2M/CSE/DMR/UPD/016_NOD/NI | Release 4 | TS-0001 [1], clause 9.6.18-2 | node | nodeID |
| TP/oneM2M/CSE/DMR/UPD/016_TS/ET | Release 3 | TS-0001 [1], clause 9.6.36-2 | timeSeries | expirationTime |

###### TP/oneM2M/CSE/DMR/UPD/017

| TP Id | TP/oneM2M/CSE/DMR/UPD/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the value of the announceTo optional attribute of the RESOURCE _ resource |  |
| Reference | TS-0001 [1], clause 10.2.18.4and clause 9.6.1.3.2-1, TS-0004 [2], clause 7.3.3.10 |  |
| Config Id | CF02 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the PARENT_RESOURCE resource containing RESOURCE_TYPE resource containing announceTo attribute set to NULL and the IUT having announced the PARENT_RESOURCE resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid CREATE Request containing  | |
                     | To set ANNC_TARGET_CSE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resourcecontaining  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+—————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/DMR/UPD/017_ACP | accesControlPolicy |
| TP/oneM2M/CSE/DMR/UPD/017_CNT | container |
| TP/oneM2M/CSE/DMR/UPD/017_GRP | group |
| TP/oneM2M/CSE/DMR/UPD/017_LCP | locationPolicy |
| TP/oneM2M/CSE/DMR/UPD/017_MGO | mgmtObj |
| TP/oneM2M/CSE/DMR/UPD/017_NOD | node |
| TP/oneM2M/CSE/DMR/UPD/017_SCH | schedule |
| TP/oneM2M/CSE/DMR/UPD/017_TS | timeSeries |
| TP/oneM2M/CSE/DMR/UPD/017_TSI | timeSeriesInstance |

###### TP/oneM2M/CSE/DMR/UPD/018

| TP Id | TP/oneM2M/CSE/DMR/UPD/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the value of the announcedAttribute optional attribute of the RESOURCE _ resource |  |
| Reference | TS-0001 [1], clause 10.2.18.8 and clause 9.6.1.3.2-1, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and announcedAttribute attribute set to NULL and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announcedAttribute attribute set to OA_ATTRIBUTE  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | The IUT sends a valid UPDATE Request containing  | |
                     | To set to ANNC_TARGET_CSE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resourcecontaining  | |
                     | OA_ATTRIBUTE attribute set to VALUE  | |
                     |} | |
+———————-+——————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/018_CNT/MBS | container | maxByteSize |
| TP/oneM2M/CSE/DMR/UPD/018_GRP/MT | Group | memberType |
| TP/oneM2M/CSE/DMR/UPD/018_LCP/LOI | locationPolicy | locationContainerID |
| TP/oneM2M/CSE/DMR/UPD/018_MGO/DC | mgmtObj | description |
| TP/oneM2M/CSE/DMR/UPD/018_NOD/HCL | Node | hostedCSELink |
| TP/oneM2M/CSE/DMR/UPD/018_SCH/SE | schedule | scheduleElement |
| TP/oneM2M/CSE/DMR/UPD/018_TS/MBS | timeSeries | maxByteSize |

TP/oneM2M/CSE/DMR/UPD/019

| TP Id | TP/oneM2M/CSE/DMR/UPD/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a UPDATE Request target to TARGET_ RESOURCE_ADDRESS resource when the Result Content set to RESULT_CONTENT |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to RESULT_CONTENT  | |
                     | Contentand  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid attribute  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+——————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/2 | Release 2 | 2 (hierarchical address) |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/3 | Release 2 | 3 (attributes and hierarchichal address) |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/4 | Release 3 | 4 (attributes and child resources) |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/5 | Release 3 | 5 (attributes and child resource references) |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/6 | Release 3 | 6 (child resource references) |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/7 | Release 3 | 7 (original resource) |
| TP/oneM2M/CSE/DMR/UPD/019_RCN/8 | Release 3 | 8 (child resources) |

TP/oneM2M/CSE/DMR/UPD/020

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/020 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns nothing of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 0 (nothing) |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +--+---------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to 0 (nothing)  | |
                     | Contentand  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid attribute  | |
                     |} | |

- +--+---------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | no Content  | |
                     |} | |
+———————-+–+———————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/020_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/020_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/UPD/020_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/UPD/020_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/UPD/020_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/UPD/020_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/UPD/020_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/UPD/020_TRAM | Release 4 | 50 (TransactionMgmt) |
| TP/oneM2M/CSE/DMR/UPD/020_TRAC | Release 4 | 51 (Transaction) |

TP/oneM2M/CSE/DMR/UPD/021

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/021 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { / the IUT being in the "initial state"/ and the IUT having registered the AE/ and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing / a child resource / and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS/} |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { / the IUT receives a valid UPDATE Request from AE containing/ To set to TARGET_RESOURCE_ADDRESS and/ From set to AE_ID and/ Result Content set to 1 (attributes)/ Contentand/ RESOURCE_TYPE resource containing/ valid attribute/} |  | IUT <= AE |
|  |  |  | then { / the IUT sends a valid Response containing/ Response Status Code set to 2004 (UPDATED) and/ Content containing/ RESOURCE_TYPE resource containing/ attributes and/ no hierarchichal address and/ no child resources and/ no child resource references/} |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/021_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/021_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/UPD/021_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/UPD/021_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/UPD/021_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/UPD/021_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/UPD/021_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/UPD/021_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/UPD/021_TRAC | Release 4 | 51 (transaction) |

TP/oneM2M/CSE/DMR/UPD/022

| TP Id |  | TP/oneM2M/CSE/DMR/UPD/022 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only modified attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 9 (modified attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { / the IUT being in the "initial state"/ and the IUT having registered the AE/ and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing/ a child resource/ and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS/} |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { / the IUT receives a valid UPDATE Request from AE containing/ To set to TARGET_RESOURCE_ADDRESS and/ From set to AE_ID and/ Result Content set to 9 (modified attributes)/ Contentand/ RESOURCE_TYPE resource containing/ valid ATTRIBUTE attribute set to VALUE/} |  | IUT <= AE |
|  |  |  | then { / the IUT sends a valid Response containing/ Response Status Code set to 2004 (UPDATED) and/ Content containing/ RESOURCE_TYPE resource containing/ modified attributes and/ no ATTRIBUTE attribute and/ no hierarchichal address and/ no child resources and/ no child resource references/} |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/UPD/022_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/UPD/022_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/UPD/022_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/UPD/022_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/UPD/022_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/UPD/022_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/UPD/022_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/UPD/022_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/UPD/022_TRAC | Release 4 | 51 (transaction) |

##### 7.2.2.3.3	CREATE Operation

###### TP/oneM2M/CSE/DMR/CRE/001

| TP Id | TP/oneM2M/CSE/DMR/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource on the TARGET_RESOURCE_ADDRESS of type PARENT_RESOURCE_TYPE with resource name not provided |  |
| Reference | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
| when { the IUT receives a valid CREATE Request from AE containing Resource Type set to RESOURCE_TYPE and To set to TARGET_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing no resourceName attribute } | IUT <= AE |  |
| then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing Valid resourceName attribute } | IUT => AE |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | PARENT_RESOURCE_TYPE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/CB | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/AE | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/CB | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/AE | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CB | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/AE | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CNT | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/ACP | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/CSR | Release 1 | TS-0001 [1], clause 9.6.4-1 | 23 (subscription) | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/CRE/001_CIN/CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 4 (contentInstance) | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/CB | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/AE | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/GRP | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/001_SCH/CB | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_SCH/SUB | Release 2 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/001_PCH/AE | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/SCH | Release 3 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/001_NOD/CB | Release 4 | TS-0001 [1], clause 9.6.3-1 | 14 (node) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_MGC/CB | Release 4 | TS-0001 [1], clause 9.6.3-1 | 12 (mgmtCmd) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_LCP/CB | Release 3 | TS-0001 [1], clause 9.6.3-1 | 10 (locationPolicy) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_STCG/CB | Release 4 | TS-0001 [1], clause 9.6.3-1 | 22 (statsConfig) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_STCL/CB | Release 4 | TS-0001 [1], clause 9.6.3-1 | 21 (statsCollect) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_ASAR/CB | Release 3 | TS-0001 [1], clause 9.6.3-1 | 19(serviceSubscribedAppRule) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/AEA | Release 3 | TS-0001 [1], clause 9.6.5-1 | 23 (subscription) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_CNT/AEA | Release 3 | TS-0001 [1], clause 9.6.5-1 | 3 (container) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_GRP/AEA | Release 3 | TS-0001 [1], clause 9.6.5-1 | 9 (group) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_ACP/AEA | Release 3 | TS-0001 [1], clause 9.6.5-1 | 1 (accessControlPolicy) | 10002 (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/001_EVCF/STCG | Release 4 | TS-0001 [1], clause 9.6.23-1 | 7 (eventConfig) | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/CRE/001_SUB/STCG | Release 4 | TS-0001 [1], clause 9.6.23-1 | 23 (subscription) | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/CRE/001_TS/CB | Release 3 | TS-0001 [1], clause 9.6.36-1 | 29 (timeSeries) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_TS/AE | Release 3 | TS-0001 [1], clause 9.6.36-1 | 29 (timeSeries) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_TS/CSR | Release 3 | TS-0001 [1], clause 9.6.36-1 | 29 (timeSeries) | 16 (remoteCSE) |
| TP/oneM2M/CSE/DMR/CRE/001_TS/AEA | Release 3 | TS-0001 [1],clause 9.6.36-1 | 29 (timeSeries) | 10002 (AEAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TSI/TS | Release 3 | TS-0001 [1], clause 9.6.37-1 | 30 (timeSeriesInstance) | 30 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAM/CB | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 50 (transactionMgmt) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAM/AE | Release 4 | TS-0001 [], Table 9.6.1.1-1 | 50 (transactionMgmt) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAM/CSR | Release 4 | TS-0001 [], Table 9.6.1.1-1 | 50 (transactionMgmt) | 16 (RemoteCSE) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/ACP | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 1 (AccessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/AE | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 2 (AE) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/CNT | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 3 (Container) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/CIN | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 4 (ContentInstance) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/CB | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 5 (CSEBase) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/GRP | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 9 (Group) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/LCP | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10 (LocationPolicy) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/MSSP | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 11 (M2MServiceSubscriptionProfile) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/MGC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 12 (MgmtCmd) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/MGO | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 13 (MgmtObj) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/MGO | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 13 (MgmtObj) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/NOD | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 14 (Node) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/CSR | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 16 (RemoteCSE) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/SCH | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 18 (Schedule) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/ASAR | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 19 (ServiceSubscribedAppRule) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/SVSN | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 20 (ServiceSubscribedNode) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/STCL | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 21 (StatsCollect) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/STCG | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 22 (StatsConfig) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/SUB | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 23 (Subscription) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/SMD | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 24 (SemanticDescriptor) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/NTPR | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 25 (NotificationTargetMgmtPolicyRef) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/NTP | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 26 (NotificationTargetPolicy) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/PDR | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 27 (PolicyDeletionRules) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/TS | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 29 (TimeSeries) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/TSI | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 30 (TimeSeriesInstance) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/ROL | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 31 (Role) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/TK | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 32 (Token) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/TS | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 29 (TimeSeries) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/ACPANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10001 (AccessContraolPolicyAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/AEANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10002 (AEAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/CNTANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10003 (ContainerAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/CINANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10001 (ContentInstanceAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/GRPANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10009 (GroupAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/LCPANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10010 (LocationPolicyAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/MGOANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10013 (MgmtObjAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/NODANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10014 (NodeAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/CSRANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10016 (RemoteCSEAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/SCHANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10018 (ScheduleAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/SMDANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10024 (AccessContraolPolicyAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/TSANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10029 (TimeSeriesAnnc) |
| TP/oneM2M/CSE/DMR/CRE/001_TRAC/TSIANNC | Release 4 | TS-0001 [1], Table 9.6.1.1-1 | 51 (transaction) | 10030 (TimeSeriesInstanceAnnc) |

###### TP/oneM2M/CSE/DMR/CRE/002

| TP Id | TP/oneM2M/CSE/DMR/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with a non-existing resource name provided by AE |  |
| Reference | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the TARGET_RESOURCE_ADDRESScontaining no child resource containing resourceName attribute set to NAME } |  |
| Expected behaviour | Test events | Direction |
| when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Resource Type set to RESOURCE_TYPE and From set to AE-ID and Content containing RESOURCE_TYPE resource containing resourceName attribute set to NAME } | IUT <= AE |  |
| then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } | IUT => AE |  |

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/002_CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/002_ACP | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/002_SUB | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/002_GRP | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/002_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/002_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/002_TS | Release 3 | TS-0001 [1], clause 10.2.30.1 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/002_FLXC | Release 2 | TS-0001 [1], clause 10.2.29.1 | 28 (flexContainer) |
| TP/oneM2M/CSE/DMR/CRE/002_TRAM | Release 4 | TS-0001 [1], clause 10.1.2, Step-2 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/CRE/002_TRAC | Release 4 | TS-0001 [1], clause 10.1.2, Step-2 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/CRE/003

| TP Id | TP/oneM2M/CSE/DMR/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of a RESOURCE_TYPE resource with an existing resource name provided by AE |  |
| Reference | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the TARGET_RESOURCE_ADDRESS containing a child resource having resourceName attribute set to NAME } |  |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to RESOURCE_TYPE and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | resourceName attribute set to NAME  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4105 (CONFLICT)  | |
                     |} | |
+———————-+——————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/003_CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/003_ACP | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/003_SUB | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/003_GRP | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/003_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/003_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/003_TS | Release 3 | TS-0001 [1], clause 10.2.30.1 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/003_TRAM | Release 4 | TS-0001 [1], clause 10.1.2, Step-2 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/CRE/003_TRAC | Release 4 | TS-0001 [1], clause 10.2.30.1 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/CRE/004

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the creation of a RESOURCE_TYPE resource when AE has no privileges |  |
| Reference |  | TS-0001 [1], clause 10.1.1.1 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having no privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +----------------------+-----------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to RESOURCE_TYPE and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource  | |
                     |} | |

- +----------------------+-----------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the RESOURCE_TYPE resource   | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————-+———————————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/004_CNT | Release 1 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/004_ACP | Release 1 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/004_SUB | Release 1 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/004_GRP | Release 1 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/004_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/004_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/004_TS | Release 3 | TS-0001 [1], clause 10.2.30.1 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/004_TRAM | Release 4 | TS-0001 [1], clause 10.1.2, Step-2 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/CRE/004_TRAC | Release 4 | TS-0001 [1], clause 10.2.30.1 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/CRE/005

| TP Id | TP/oneM2M/CSE/DMR/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the CREATE Request of container resource when the RW ATTRIBUTE is provided with an invalid value |  |
| Reference | TS-0001 [1], clause 10.2.4.1, TS-0004 [2], clause 7.4.7.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to 3 (container) and Content containing an invalid RW ATTRIBUTE attribute } | IUT <= AE |
|  | then { the IUT does not create the container resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } | IUT => AE |

| TP Id | Reference | ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/005_MNI | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/005_MBS | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/005_MIA | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | maxInstanceAge |

###### TP/oneM2M/CSE/DMR/CRE/006

| TP Id | TP/oneM2M/CSE/DMR/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns a default value to RW ATTRIBUTE in the response of a RESOURCE_TYPE resource CREATE Request where no a RW ATTRIBUTE is provided by AE |  |
| Reference | TS-0001 [1], clause 10.2.4.1, TS-0004 [2], clause 7.4.7.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Resource Type set to RESOURCE_TYPE and Content containing RESOURCE_TYPE resource representation } | IUT <= AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) and Content containing RESOURCE_TYPE resource containing valid ATTRIBUTE attribute } | IUT => AE |

| TP Id | PARENT _ RELEASE | Reference | RESOURCE_TYPE | ATTRIBUTE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/006_CNT/MNI | Release 1 | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | container | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/006_CNT/MBS | Release 1 | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | container | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/006_CNT/MIA | Release 1 | TS-0001 [1], clause 9.6.6, TS-0004 [2], clause 7.4.7.1 | container | maxInstanceAge |
| TP/oneM2M/CSE/DMR/CRE/006_TS/MNI | Release 3 | TS-0001 [1], clause 9.6.36, TS-0004 [2], clause 7.4.38.1 | timeSeries | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/006_TS/MBS | Release 3 | TS-0001 [1], clause 9.6.36, TS-0004 [2], clause 7.4.38.1 | timeSeries | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/006_TS/MIA | Release 3 | TS-0001 [1], clause 9.6.36, TS-0004 [2], clause 7.4.38.1 | timeSeries | maxInstanceAge |

###### TP/oneM2M/CSE/DMR/CRE/007

| TP Id | TP/oneM2M/CSE/DMR/CRE/007 |  |
| --- | --- | --- |
| Test objective | Checks that the IUT accepts a newly created RESOURCE_TYPE when the currentNrOfInstances exceeds the field value set in maxNrOfInstances in the _PARENT RESOURCE_TYPE resource by removing enough of the oldest RESOURCE_TYPE resources to allow the creation of the new RESOURCE_TYPE resource |  |
| Reference | TS-0001 [1], clause 10.2.19.2, TS-0004 [2], clause 7.4.8.2 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource containing currentNrOfInstances attribute set to MAX_NUMBER_OF_INSTANCES and maxNrOfInstances attribute set to MAX_NUMBER_OF_INSTANCES and the AE having privileges to perform CREATE operation on the resource PARENT __RESOURCE_ADDRESS_ } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to RESOURCE_TYPE and  | |
                     |Content containing  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT creates the RESOURCE_TYPE resource  | |
                     | and the IUT removes the oldest RESOURCE_TYPE resources   | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | PARENT_RE LEASE | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/007_CNT/CIN | Release 1 | container | contentInstance |
| TP/oneM2M/CSE/DMR/CRE/007_TS/TSI | Release 3 | timeSeries | timeSeriesInstance |

###### TP/oneM2M/CSE/DMR/CRE/008

| TP Id | TP/oneM2M/CSE/DMR/CRE/008 |  |
| --- | --- | --- |
| Test objective | Checks that the IUT accepts a newly created RESOURCE_TYPE resource when the currentByteSize exceeds the field value set in maxByteSize in the _PARENT RESOURCE_TYPE resource by removing enough of the oldest RESOURCE_TYPE resources to allow the creation of the new RESOURCE_TYPE resource |  |
| Reference | TS-0001 [1], clause 10.2.19.2 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a _PARENT RESOURCE_TYPE resource containing maxByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and currentByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |Resource Type set to RESOURCE_TYPE and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource with size less than  | |
                     | MAX_BYTE_SIZE_OF DATA  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {  |IUT => AE |
                     |the IUT creates the RESOURCE_TYPE resource  | |
                     |and the IUT removes the oldest contentInstance resources  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————————————–+———————-+

| TP Id | PARENT_RE LEASE | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/008_CNT/CIN | Release 1 | container | contentInstance |
| TP/oneM2M/CSE/DMR/CRE/008_TS/TSI | Release 3 | timeSeries | timeSeriesInstance |

###### TP/oneM2M/CSE/DMR/CRE/009

| TP Id | TP/oneM2M/CSE/DMR/CRE/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT increaments and then copies the field value of attribute stateTag in parent container resource when contentInstance resource is created as the direct child of the parent container |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2, TS-0004 [2], clause 7.4.7.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource CONTAINER_RESOURCE_ADDRESScontaining stateTag attribute and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |Resource Type set to 4 (contentInstance) and  | |
                     |Content containing  | |
                     | contentInstance resource representation   | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT increments stateTag attribute of CONTAINER_RESOURCE_ADDRESS resource  | |
                     |and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DMR/CRE/010

| TP Id | TP/oneM2M/CSE/DMR/CRE/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the CREATE Request of a container resource named "la" as a direct child of a container resource |  |
| Reference | TS-0001 [1], clause 10.2.22, TS-0004 [2], clause 7.4.28.2.2 and clause 7.3.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |Content containing  | |
                     | Container resource containing  | |
                     | resourceName attribute set to "la"  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT does not create the container resource   | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DMR/CRE/011

| TP Id | TP/oneM2M/CSE/DMR/CRE/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the CREATE Request of a container resource named "ol" as a direct child of a container resource |  |
| Reference | TS-0001 [1], clause 10.2.23, TS-0004 [2], clause 7.4.29.2.2 and clause 7.3.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource CONTAINER_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |Content containing  | |
                     |container resource containing  | |
                     | resourceName attribute set to "ol"  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT does not create the container resource   | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DMR/CRE/012

| TP Id | TP/oneM2M/CSE/DMR/CRE/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with the optional attribute OPTIONAL_ATTRIBUTE provided |  |
| Reference | TS-0001 [1], clause 10.1.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE-ID and Content containing RESOURCE_TYPE resource containing valid OPTIONAL_ATTRIBUTE attribute } | IUT <= AE |
|  | then { the IUT creates the RESOURCE_TYPE resource and the IUT sends a valid Response containing Response Status Code set to 2001 (CREATED) } | IUT => AE |

| TP Id | PARENT_RELEASE | PICS Selection | Reference | RESOURCE_TYPE | OPTIONAL_ATTRIBUTE |
| --- | --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/ACPI | Release 1 | PICS_CNT_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | 3 (container) | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/ET | Release 1 | PICS_CNT_ET | TS-0001 [1], clause 9.6.1.3.1-1 | 3 (container) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/LBL | Release 1 | PICS_CNT_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 3 (container) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/MNI | Release 1 | PICS_CNT_MNI | TS-0001 [1], clause 9.6.6-2 | 3 (container) | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/MBS | Release 1 | PICS_CNT_MBS | TS-0001 [1], clause 9.6.6-2 | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/MIA | Release 1 | PICS_CNT_MIA | TS-0001 [1], clause 9.6.6-2 | 3 (container) | maxInstanceAge |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/OR | Release 1 | PICS_CNT_OR | TS-0001 [1], clause 9.6.6-2 | 3 (container) | ontologyRef |
| TP/oneM2M/CSE/DMR/CRE/012_CNT/CR | Release 1 | PICS_CNT_CR | TS-0001 [1], clause 9.6.6-2 | 3 (container) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/ET | Release 1 | PICS_CIN_ET | TS-0001 [1], clause 9.6.1.3.1-1 | 4 (contentInstance) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/LBL | Release 1 | PICS_CIN_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 4 (contentInstance) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/CR | Release 1 | PICS_CIN_CR | TS-0001 [1], clause 9.6.7-2 | 4 (contentInstance) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/CNF | Release 1 | PICS_CIN_CNF | TS-0001 [1], clause 9.6.7-2 | 4 (contentInstance) | contentInfo |
| TP/oneM2M/CSE/DMR/CRE/012_CIN/OR | Release 1 | PICS_CIN_OR | TS-0001 [1], clause 9.6.7-2 | 4 (contentInstance) | ontologyRef |
| TP/oneM2M/CSE/DMR/CRE/012_ACP/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | 1 (accesControlPolicy) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_ACP/LBL | Release 1 | PICS_ACP_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 1 (accesControlPolicy) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/ACPI | Release 1 | PICS_SUB_ACPI | TS-0001 [1], clause 9.6.1.3.2-1 | 23 (subscription) | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/ET | Release 1 | N/A | TS-0001 [1], clause 9.6.1.3.1-1 | 23 (subscription) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/LBL | Release 1 | PICS_SUB_LBL | TS-0001 [1], clause 9.6.1.3.2-1 | 23 (subscription) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/ENC | Release 1 | PICS_SUB_ENC | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | eventNotificationCriteria |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/EXC | Release 1 | PICS_SUB_EXC | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | expirationCounter |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/GPI | Release 1 | PICS_SUB_GPI | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | groupID |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NFU | Release 1 | PICS_SUB_NFU | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationForwardingURI |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/BN | Release 1 | PICS_SUB_BN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | batchNotify |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/RL | Release 1 | PICS_SUB_RL | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | rateLimit |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/PSN | Release 1 | PICS_SUB_PSN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | preSubscriptionNotify |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/PN | Release 1 | PICS_SUB_PN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | pendingNotification |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NSP | Release 1 | PICS_SUB_NSP | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationStoragePriority |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/LN | Release 1 | PICS_SUB_LN | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | latestNotify |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NCT | Release 1 | PICS_SUB_NCT | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationContentType |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/NEC | Release 1 | PICS_SUB_NEC | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | notificationEventCat |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/CR | Release 1 | PICS_SUB_CR | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_SUB/SU | Release 1 | PICS_SUB_SU | TS-0001 [1], clause 9.6.8-2 | 23 (subscription) | subscriberURI |
| TP/oneM2M/CSE/DMR/CRE/012_STCG/CR | Release 4 |  | TS-0001 [1], clause 9.6.23-2 | 22 (statsConfig) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_TS/ACPI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | accessControlPolicyIDs |
| TP/oneM2M/CSE/DMR/CRE/012_TS/ET | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_TS/LBL | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_TS/CR | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | creator |
| TP/oneM2M/CSE/DMR/CRE/012_TS/MNI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | maxNrOfInstances |
| TP/oneM2M/CSE/DMR/CRE/012_TS/MBS | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | maxByteSize |
| TP/oneM2M/CSE/DMR/CRE/012_TS/MIA | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | maxInstanceAge |
| TP/oneM2M/CSE/DMR/CRE/012_TS/PEI | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | periodicInterval |
| TP/oneM2M/CSE/DMR/CRE/012_TS/MDD | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | missingDataDetect |
| TP/oneM2M/CSE/DMR/CRE/012_TS/MDN | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | missingDataMaxNr |
| TP/oneM2M/CSE/DMR/CRE/012_TS/MDT | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | missingDataDetectTimer |
| TP/oneM2M/CSE/DMR/CRE/012_TS/OR | Release 3 |  | TS-0001 [1], clause 9.6.36-2 | 29 (timeSeries) | ontologyRef |
| TP/oneM2M/CSE/DMR/CRE/012_TSI/ET | Release 3 |  | TS-0001 [1], clause 9.6.37-2 | 30 (timeSeriesInstance) | expirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_TSI/LBL | Release 3 |  | TS-0001 [1], clause 9.6.37-2 | 30 (timeSeriesInstance) | labels |
| TP/oneM2M/CSE/DMR/CRE/012_TSI/SNR | Release 3 |  | TS-0001 [1], clause 9.6.37-2 | 30 (timeSeriesInstance) | sequenceNr |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TLTM | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionLockTime |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TEXT | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionExecuteTime |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TCT | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionCommitTime |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TEPT | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionExpirationTime |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TMD | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionMode |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TLTP | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionLockType |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TMR | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionMaxRetries |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TMH | Release 4 |  | TS-0004 [2], Table 7.4.60.1-363 | 50 (transactionMgmt) | transactionManagementHandling |
| TP/oneM2M/CSE/DMR/CRE/012_TRAC/TLTM | Release 4 |  | TS-0004 [2], Table 7.4.61.1-367 | 51 (transaction) | transactionLockTime |
| TP/oneM2M/CSE/DMR/CRE/012_TRAC/TEXT | Release 4 |  | TS-0004 [2], Table 7.4.61.1-367 | 51 (transaction) | transactionExecuteTime |
| TP/oneM2M/CSE/DMR/CRE/012_TRAC/TCT | Release 4 |  | TS-0004 [2], Table 7.4.61.1-367 | 51 (transaction) | transactionCommitTime |
| TP/oneM2M/CSE/DMR/CRE/012_TRAM/TLTP | Release 4 |  | TS-0004 [2], Table 7.4.61.1-367 | 51 (transaction) | transactionLockType |

###### TP/oneM2M/CSE/DMR/CRE/013

| TP Id | TP/oneM2M/CSE/DMR/CRE/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the CREATE Request of a RESOURCE_TYPE resource with error "NOT_ACCEPTABLE" when contentSize exceeds maxByteSize of the PARENT_RESOURCE_TYPE |  |
| Reference | TS-0001 [1], clause 10.2.19, TS-0004 [2], clause 7.4.7.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource having maxByteSize attribute set to MAX_BYTE_SIZE } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to PARENT_RESOURCE_ADDRESS and From set to AE_ID and Content containing RESOURCE_TYPE resource containing valid content with size larger than MAX_BYTE_SIZE } | IUT <= AE |
|  | then { the IUT sends a valid Response containing Response Status Code set to 5207 (NOT_ACCEPTABLE) } | IUT => AE |

| TP Id | PARENT_RE LEASE | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/013_CNT/CIN | Release 1 | container | contentInstance |
| TP/oneM2M/CSE/DMR/CRE/013_TS/TSI | Release 3 | timeSeries | timeSeriesInstance |

###### TP/oneM2M/CSE/DMR/CRE/014

| TP Id | TP/oneM2M/CSE/DMR/CRE/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the currentByteSize attribute in a PARENT_RESOURCE_TYPE when a new RESOURCE_TYPE is created |  |
| Reference | TS-0001 [1], clause 10.2.19.2, TS-0004 [2], clause 7.4.8.2 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a PARENT_RESOURCE_TYPE resource at PARENT_RESOURCE_ADDRESS containing maxByteSize attribute set to MAX_BYTE_SIZE_OF_DATA and currentByteSize attribute set to 0 and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to RESOURCE_TYPE and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | valid content attribute with size equal to DATA_SIZE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT increments currentByteSize attribute of PARENT_RESOURCE_ADDRESS resource by DATA_SIZE  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+—————————————————————————————————————-+————-+

| TP Id | PARENT_RE LEASE | PARENT_RESOURCE_TYPE | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/014_CNT/CIN | Relase 1 | container | contentInstance |
| TP/oneM2M/CSE/DMR/CRE/014_TS/TSI | Release 3 | timeSeries | timeSeriesInstance |

###### TP/oneM2M/CSE/DMR/CRE/015

| TP Id | TP/oneM2M/CSE/DMR/CRE/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully after a CREATE request an empty content of created resource when the ResultContent set to 0 (Nothing) |  |
| Reference | TS-0001 [1], clause 8.1.2, clause 8.1.3 and clause 10.1.1.1, TS-0004 [2], clause 6.3.4.2.7 |  |
| Parent Release | PARENT_RELEASE |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to RESOURCE_TYPE and  | |
                     |Result Content set to 0 (nothing) and  | |
                     |Contentcontaining  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED) and  | |
                     |no Content  | |
                     |} | |
+———————-+——————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/015_CNT | Release 2 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/015_GRP | Release 2 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/015_ACP | Release 2 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/015_SUB | Release 2 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/015_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/015_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/015_TS | Release 3 | TS-0001 [1], clause 10.2.30.1 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/015_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.2 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/CRE/015_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.6 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/CRE/016

| TP Id | TP/oneM2M/CSE/DMR/CRE/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully after a CREATE request only hierarchichal address of the created resource when the Result Content is set to 2 (hierarchical address) |  |
| Reference | TS-0001 [1], clause 8.1.2, clause 8.1.3 and clause 10.1.1.1, TS-0004 [2], clause 6.3.4.2.7 |  |
| Parent Release | PARENT_RELEASE |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |Resource Type set to RESOURCE_TYPE and  | |
                     | Result Content set to 2 (hierarchical address) and  | |
                     | Contentcontaining  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | Hierarchical address of the RESOURCE_TYPE resource  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/016_CNT | Release 2 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/016_GRP | Release 2 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/016_ACP | Release 2 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/016_SUB | Release 2 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/016_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/016_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/016_TS | Release 3 | TS-0001 [1], clause 10.2.30.1 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/016_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.2 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/CRE/016_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.6 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/CRE/017

| TP Id | TP/oneM2M/CSE/DMR/CRE/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully after a CREATE request only attributes and hierarchichal address of the created resource when Result Content is set 3 (attributes and hierarchichal address) |  |
| Reference | TS-0001 [1], clause 8.1.2, clause 8.1.3 and clause 10.1.1.1, TS-0004 [2], clause 6.3.4.2.7 |  |
| Parent Release | PARENT_RELEASE |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to RESOURCE_TYPE and  | |
                     | Result Content set to 3 (attributes and hierarchichal address_)_ and  | |
                     | Contentcontaining  | |
                     | RESOURCE_TYPE resource representation  | |
                     |  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | resource element containing  | |
                     | Hierarchical address of the RESOURCE_TYPE resourceand  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/017_CNT | Release 2 | TS-0001 [1], clause 10.2.4.1 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/017_GRP | Release 2 | TS-0001 [1], clause 10.2.7.2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/017_ACP | Release 2 | TS-0001 [1], clause 10.2.21.1 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/017_SUB | Release 2 | TS-0001 [1], clause 10.2.11.2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/017_PCH | Release 2 | TS-0001 [1], clause 10.2.13.2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/017_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/017_TS | Release 3 | TS-0001 [1], clause 10.2.30.1 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/017_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.2 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/CRE/017_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.6 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/CRE/018

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/018 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a CREATE request when the ResultContent is set to RESULT_CONTENT |  |
| Reference |  | TS-0001 [1], clause 8.1.2 & TS-0004 [2], clause 7.3.2.1 |  |
| Parent Release |  | Release 2 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |  |
| Expected behaviour | Test events |  | Direction |
| when { the IUT receives a valid CREATE Request from AE containing To set to AE_RESOURCE_ADDRESS and From set to AE_ID and Result Content set to RESULT_CONTENT } |  | IUT <= AE |  |
| then { the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } |  | IUT => AE |  |

| TP Id | Reference | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/4 | TS-0004 [2], clause 6.3.4.2.7 | 4 (attributes and child resources) |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/5 |  | 5 (attributes and child resources references) |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/6 |  | 6 (child resrouces references) |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/7 |  | 7 (original resource) |
| TP/oneM2M/CSE/DMR/CRE/018_RCN/8 |  | 8 (child resources) |

TP/oneM2M/CSE/DMR/CRE/019

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/019 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { / the IUT being in the "initial state"/ and the IUT having registered the AE/ and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE/ and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS/} |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { / the IUT receives a valid CREATE Request from AE containing/ To set to TARGET_RESOURCE_ADDRESS and/ From set to AE_ID and/ Result Content set to 1 (attributes)/ Contentcontaining/ RESOURCE_TYPE resource representation/} |  | IUT <= AE |
|  |  |  | then { / the IUT sends a valid Response containing/ Response Status Code set to 2001 (CREATED) and/ Content containing/ RESOURCE_TYPE resource containing/ attributes and/ no hierarchichal address and/ no child resources and/ no child resource references/} |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/019_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/019_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/019_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/019_PCH | Release 2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/019_SUB | Release 2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/019_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/019_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/CRE/019_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/CRE/019_TRAC | Release 4 | 51 (transaction) |

TP/oneM2M/CSE/DMR/CRE/020

| TP Id |  | TP/oneM2M/CSE/DMR/CRE/020 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only modified attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 9 (modified attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { / the IUT being in the "initial state"/ and the IUT having registered the AE/ and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE/ and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS/} |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { / the IUT receives a valid CREATE Request from AE containing/ To set to TARGET_RESOURCE_ADDRESS and/ From set to AE_ID and/ Result Content set to 1 (attributes)/ Contentcontaining/ RESOURCE_TYPE resource representation/} |  | IUT <= AE |
|  |  |  | then { / the IUT sends a valid Response containing/ Response Status Code set to 2001 (CREATED) and/ Content containing/ RESOURCE_TYPE resource containing/ modified attributes/} |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/020_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/CRE/020_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/CRE/020_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/CRE/020_PCH | Release 2 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/CRE/020_SUB | Release 2 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/CRE/020_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/CRE/020_TS | Release 3 | 29 (timeSeries) |

###### TP/oneM2M/CSE/DMR/CRE/021

| TP Id | TP/oneM2M/CSE/DMR/CRE/021 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource on the TARGET_RESOURCE_ADDRESS of type announced PARENT_RESOURCE_TYPE |  |
| Reference | TS-0001 [1], clause 9.6.5 |  |
| Parent Release | Release 3 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the CSE and the IUT having created the PARENT_RESOURCE_TYPE and the Originator having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid CREATE Request from CSE containing  | |
                     | Resource Type set to RESOURCE_TYPE and  | |
                     |To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to CSE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | no resourceName attribute  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     |the IUT creates the RESOURCE_TYPE resource   | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | Valid resourceName attribute  | |
                     |} | |
+———————-+————————————————————————————————————————————-+————-+

| TP Id | Reference | RESOURCE_TYPE | PARENT_RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/021_CNTA/AEA | TS-0001 [1], clause 9.6.5-1 | (containerAnnc) | (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/021_GRPA/AEA | TS-0001 [1], clause 9.6.5-1 | (groupAnnc) | (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/021_ACPA/AEA | TS-0001 [1], clause 9.6.5-1 | (accessControlPolicyAnnc) | (AEannc) |
| TP/oneM2M/CSE/DMR/CRE/021_TSA/AEA | TS-0001 [1], clause 9.6.5-1 | (timeSeriesAnnc) | (AEannc) |

###### TP/oneM2M/CSE/DMR/CRE/022

| /TP Id |  | TP/oneM2M/CSE/DMR/CRE/022 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with the optional attribute announceTo provided |  |
| Reference |  | TS-0001 [1], clause 10.2.18.4 and clause 9.6.1.3.2-1, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release |  | Release 3 |  |
| Config Id |  | CF02 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { / the IUT being in the "initial state"/ and the IUT having registered the AE/ and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE/ and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS/} |  |  |
| Expected behaviour | Test events |  | Direction |
|  | when { / the IUT receives a valid CREATE Request from AE containing/ To set to PARENT_RESOURCE_ADDRESS and/ From set to AE-ID and/ Content containing/ RESOURCE_TYPE resource containing/ announceTo attribute set to ANNC_TARGET_CSE_ADDRESS/} |  | IUT <= AE |
|  | then {/ The IUT sends a valid CREATE Request containing/ To set to ANNC_TARGET_CSE_ADDRESS and/ Content containing/ RESOURCE_TYPE announced variant resourcecontaining/ MA (mandatory) attributes/} |  | /IUT => CSE/// |

| TP Id | RESOURCE_TYPE | PICS |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/022_ACP | 1 (accesControlPolicy) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/022_CNT | 3 (container) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/022_CIN | 4 (contentInstance) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/022_GRP | 9 (group) |  |
| TP/oneM2M/CSE/DMR/CRE/022_LCP | 10 (locationPolicy) |  |
| TP/oneM2M/CSE/DMR/CRE/022_MGO | 13 (mgmtObj) |  |
| TP/oneM2M/CSE/DMR/CRE/022_NOD | 14 (node) |  |
| TP/oneM2M/CSE/DMR/CRE/022_SCH | 18 (schedule) |  |
| TP/oneM2M/CSE/DMR/CRE/022_TS | 29 (timeSeries) |  |
| TP/oneM2M/CSE/DMR/CRE/022_TSI | 30 (timeSeriesInstance) |  |

###### TP/oneM2M/CSE/DMR/CRE/023

| TP Id | TP/oneM2M/CSE/DMR/CRE/023 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a RESOURCE_TYPE resource with the optional attribute announcedAttribute provided |  |
| Reference | TS-0001 [1], clause 10.2.18.4 and clause 9.6.1.3.2-1, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |
| Expected behaviour | Test events | Direction |

- +------------------------------------------------------------------------------------+-----------------------------------------+

- |then {  |IUT => CSE |
                     |The IUT sends a valid CREATE Request containing  | |
                     |To set to ANNC_TARGET_CSE_ADDRESS and  | |
                     |Content containing  | |
                     |RESOURCE_TYPE announced variant resourcecontaining  | |
                     |MA (mandatory) attributes and  | |
                     |OA_ATTRIBUTE attribute set to VALUE  | |
                     |} | |
+———————-+————————————————————————————+—————————————–+

| TP Id | RESOURCE_TYPE | PICS |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/CRE/023_CNT | 3 (container) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/023_CIN | 4 (contentInstance) | PICS_IN_PROFILE |
| TP/oneM2M/CSE/DMR/CRE/023_GRP | 9 (group) |  |
| TP/oneM2M/CSE/DMR/CRE/023_LCP | 10 (locationPolicy) |  |
| TP/oneM2M/CSE/DMR/CRE/023_MGO | 13 (mgmtObj) |  |
| TP/oneM2M/CSE/DMR/CRE/023_NOD | 14 (node) |  |
| TP/oneM2M/CSE/DMR/CRE/023_SCH | 18 (schedule) |  |
| TP/oneM2M/CSE/DMR/CRE/023_TS | 29 (timeSeries) |  |
| TP/oneM2M/CSE/DMR/CRE/023_TSI | 30 (timeSeriesInstance) |  |

##### 7.2.2.3.4	DELETE Operation

###### TP/oneM2M/CSE/DMR/DEL/001

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts the deletion of a RESOURCE_TYPE resource |  |
| Reference |  | TS-0001 [1], clause 10.1.4 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +-----------------------------------------------+---------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +-----------------------------------------------+---------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT deletes the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+———————————————–+———————————————————+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/001_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/001_ACP | Release 1 | TS-0001 [1], clause 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/001_SUB | Release 1 | TS-0001 [1], clause 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/001_CIN | Release 1 | TS-0001 [1], clause 10.2.4.1 | 4 (contentInstance) |
| TP/oneM2M/CSE/DMR/DEL/001_GRP | Release 1 | TS-0001 [1], clause 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/001_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/001_PCH | Release 2 | TS-0001 [1], clause 10.2.13.5 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/001_STCG | Release 4 | TS-0001 [1], clause 10.2.15.5 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/DEL/001_TS | Release 3 | TS-0001 [1], clause 10.2.30.4 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/001_TSI | Release 3 | TS-0001 [1], clause 10.2.31.4 | 30 (timeSeriesInstance) |
| TP/oneM2M/CSE/DMR/DEL/001_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.5 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/001_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.9 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/DEL/002

| TP Id | TP/oneM2M/CSE/DMR/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS without having privileges for the DELETE operation |  |
| Reference | TS-0001 [1], clause 10.1.4 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having no privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not delete the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————–+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/002_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/002_ACP | Release 1 | TS-0001 [1], clause 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/002_SUB | Release 1 | TS-0001 [1], clause 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/002_GRP | Release 1 | TS-0001 [1], clause 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/002_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/002_TS | Release 3 | TS-0001 [1], clause 10.2.30.4 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/002_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.5 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/002_TRAC | Release 4 | TS-0001 [1], clause 10.2.18.9 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/DEL/003

| TP Id | TP/oneM2M/CSE/DMR/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries to delete the resource TARGET_RESOURCE_ADDRESS which does not exist |  |
| Reference | TS-0001 [1], clause 10.1.4 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not delete the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+——————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/DMR/DEL/004

| TP Id | TP/oneM2M/CSE/DMR/DEL/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when AE tries to delete a child resource of a RESOURCE_TYPE resource when this resource has been already deleted |  |
| Reference | TS-0001 [1], clause 10.1.4 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS and the AE having deleted the TARGET_RESOURCE_ADDRESS resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_CHILD_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     | } | |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+————————————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/004_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/004_ACP | Release 1 | TS-0001 [1], clause 10.2.21.4 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/004_SUB | Release 1 | TS-0001 [1], clause 10.2.11.5 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/004_GRP | Release 1 | TS-0001 [1], clause 10.2.7.5 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/004_SCH | Release 3 | TS-0001 [1], clause 10.2.40.2 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/004_STCG | Release 4 | TS-0001 [1], clause 10.2.15.5 | 22 (statsConfig) |
| TP/oneM2M/CSE/DMR/DEL/004_TS | Release 3 | TS-0001 [1], clause 10.2.30.4 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/004_TRAM | Release 4 | TS-0001 [1], clause 10.2.18.5 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/004_TRAC | Release 4 | TS-0001 [1], clause 10.2.30.4 | 51 (transaction) |

###### TP/oneM2M/CSE/DMR/DEL/005

| TP Id | TP/oneM2M/CSE/DMR/DEL/005 |  |
| --- | --- | --- |
| Test objective | Check that the stateTag attribute of a RESOURCE_TYPE resource is increased when a child resource is deleted |  |
| Reference | TS-0001 [1], clause 10.1.4 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a RESOURCE_TYPE resource containing a child resource and the AE having privileges to perform DELETE operation on the TARGET_CHILD_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_CHILD_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and   | |
                     |no Content  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT increments the stateTag attribute of the RESOURCE_TYPE resource   | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/005_CNT | Release 1 | TS-0001 [1], clause 10.2.4.4 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/005_TS | Release 3 | TS-0001 [1], clause 10.2.30.4 | 29 (timeSeries) |

###### TP/oneM2M/CSE/DMR/DEL/006

| TP Id | TP/oneM2M/CSE/DMR/DEL/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT decreases the field value of attribute currentNrOfInstances and currentByteSize of parent RESOURCE_TYPE when a latest RESOURCE_TYPE resource is deleted successfully |  |
| Reference | TS-0001 [1], clause 10.2.19.5, TS-0004 [2], clause 7.4.8.2.4 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a _PARENT RESOURCE_TYPE resource containing currentNrOfInstances attribute set to NR_OF_INSTANCES and currentByteSize attribute set to BYTE_SIZE_OF_DATA and a RESOURCE_TYPE resource TARGET_RESOURCE_ADDRESS and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT decreases the currentNrOfInstances attribute of _PARENT RESOURCE_TYPE resource  | |
                     | and the IUT decreases the currentByteSize attribute of _PARENT RESOURCE_TYPE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————–+————-+ Editor's Note: Need to clarify if needed to split into two separate test purposes.

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE | PARENT_ RESOURCE_TYPE |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/006_CNT/CIN | Release 1 | TS-0001 [1], clause 10.2.4.4 | 4 (contentInstance) | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/006_TS/TSI | Release 3 | TS-0001 [1], clause 10.2.30.4 | 30 (timeSeriesInstance) | 29 (timeSeries) |

###### TP/oneM2M/CSE/DMR/DEL/007

| TP Id | TP/oneM2M/CSE/DMR/DEL/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the DELETE Request of the latest contentInstance resource target to a container resource |  |
| Reference | TS-0001 [1], clause 10.2.22.2, TS-0004 [2], clause 7.4.28.2.5 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform DELETE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |

- +------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/la and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT deletes the CONTENT_INSTANCE_2_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+——————————————————————————————————+————-+

###### TP/oneM2M/CSE/DMR/DEL/008

| TP Id | TP/oneM2M/CSE/DMR/DEL/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the DELETE request of a latest resource in a container resource that has no direct child contentInstance resources |  |
| Reference | TS-0001 [1], clause 10.2.22.2, TS-0004 [2], clause 7.4.28.2.5 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no contentInstance resources } |  |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/la and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+———————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/DMR/DEL/009

| TP Id | TP/oneM2M/CSE/DMR/DEL/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the DELETE Request of the oldest contentInstance resource target to a container resource |  |
| Reference | TS-0001 [1], clause 10.2.23.2, TS-0004 [2], clause 7.4.29.2.5 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a container resource containing CONTENT_INSTANCE_1 contentInstance resource and CONTENT_INSTANCE_2 contentInstance resource and the AE having privileges to perform DELETE operation on the container resource and creationTime attribute of CONTENT_INSTANCE_1 < creationTime attribute of CONTENT_INSTANCE_2 } |  |

- +------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT deletes the CONTENT_INSTANCE_1_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+——————————————————————————————————+————-+

###### TP/oneM2M/CSE/DMR/DEL/010

| TP Id | TP/oneM2M/CSE/DMR/DEL/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the DELETE request of an oldest resource in a container resource that has no direct child contentInstance resources |  |
| Reference | TS-0001 [1], clause 10.2.23.2, TS-0004 [2], clause 7.4.29.2.5 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource containing no contentInstance resources } |  |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS/ol and  | |
                     | From set to AE_ID and  | |
                     |no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+———————————————————————————————————————————-+————-+ TP/oneM2M/CSE/DMR/DEL/011

| TP Id | TP/oneM2M/CSE/DMR/DEL/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a DELETE Request target to TARGET_ RESOURCE_ADDRESS resource when the Result Content set to RESULT_CONTENT |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to RESULT_CONTENT  | |
                     | no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+——————————————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | RESULT_CONTENT |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/011_RCN/2 | Release 2 | 2 (hierarchical address) |
| TP/oneM2M/CSE/DMR/DEL/011_RCN/3 | Release 2 | 3 (attributes and hierarchichal address) |
| TP/oneM2M/CSE/DMR/DEL/011_RCN/7 | Release 3 | 7 (original resource) |
| TP/oneM2M/CSE/DMR/DEL/011_RCN/9 | Release 2 | 9 (modified attributes) |

TP/oneM2M/CSE/DMR/DEL/012

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/012 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns nothing of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 0 (nothing) |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | PARENT_RELEASE |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +--+---------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to 0 (nothing)  | |
                     | no Content  | |
                     |} | |

- +--+---------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED) and  | |
                     | no Content  | |
                     |} | |
+———————-+–+———————————————————————————————————+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/012_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/012_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/012_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/012_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/012_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/012_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/012_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/012_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/012_TRAC | Release 4 | 51 (transaction) |

TP/oneM2M/CSE/DMR/DEL/013

| TP Id |  | TP/oneM2M/CSE/DMR/DEL/013 |  |  |
| --- | --- | --- | --- | --- |
| Test objective |  | Check that the IUT returns successfully only attributes of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 1 (attributes) |  |  |
| Reference |  | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.3.2.7 |  |  |
| Config Id |  | CF01 |  |  |
| Parent Release |  | PARENT_RELEASE |  |  |
| PICS Selection |  | PICS_CSE |  |  |
| Initial conditions | with { / the IUT being in the "initial state"/ and the IUT having registered the AE/ and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing/ a child resource/ and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS/} |  |  |  |
| Expected behaviour | Test events |  |  | Direction |
|  |  | when { / the IUT receives a valid DELETE Request from AE containing/ To set to TARGET_RESOURCE_ADDRESS and/ From set to AE_ID and/ Result Content set to 1 (attributes)/ no Content/} |  | IUT <= AE |
|  |  |  | then { / the IUT sends a valid Response containing/ Response Status Code set to 2002 (DELETED) and/ Content containing/ RESOURCE_TYPE resource containing/ attributes and/ no hierarchichal address and/ no child resources and/ no child resource references/} |  |

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/013_CNT | Release 2 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/013_GRP | Release 2 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/013_ACP | Release 2 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/013_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/013_PCH | Release 3 | 15 (pollingChannel) |
| TP/oneM2M/CSE/DMR/DEL/013_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/013_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/013_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/013_TRAC | Release 4 | 51 (transaction) |

TP/oneM2M/CSE/DMR/DEL/014

| TP Id | TP/oneM2M/CSE/DMR/DEL/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only attributes and child resources of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 4 (attributes and child resources) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to 4 (attributes and child resources)  | |
                     | no Content  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | attributes and  | |
                     | child resources  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/014_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/014_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/014_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/014_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/014_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/014_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/014_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/014_TRAC | Release 4 | 51 (transaction) |

TP/oneM2M/CSE/DMR/DEL/015

| TP Id | TP/oneM2M/CSE/DMR/DEL/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only attributes and child resources references of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 5(attributes and child resources references) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to 5 (attributes and child resources references)  | |
                     | no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | attributes and  | |
                     | child resource references  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/015_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/015_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/015_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/015_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/015_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/015_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/015_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/015_TRAC | Release 4 | 51 (transaction) |

TP/oneM2M/CSE/DMR/DEL/016

| TP Id | TP/oneM2M/CSE/DMR/DEL/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only child resources references of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 6(child resources references) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to 6 (child resources references)  | |
                     | no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED) and  | |
                     | Content containing  | |
                     | resourceRefList elementcontaining  | |
                     | child resource references  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/016_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/016_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/016_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/016_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/016_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/016_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/016_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/016_TRAC | Release 4 | 51 (transaction) |

TP/oneM2M/CSE/DMR/DEL/017

| TP Id | TP/oneM2M/CSE/DMR/DEL/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully only child resources of TARGET_RESOURCE_ADDRESS resource when the Result Content is set to 8(child resources) |  |
| Reference | TS-0001 [1], clause 8.1.2, TS-0004 [2], clause 6.3.4.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing a child resource and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Result Content set to 8 (child resources)  | |
                     | no Content  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED) and  | |
                     | Content containing  | |
                     |CHILD_RESOURCE_TYPE resource representation  | |
                     |  | |
                     |} | |
+———————-+——————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | RESOURCE_TYPE |
| --- | --- | --- |
| TP/oneM2M/CSE/DMR/DEL/017_CNT | Release 3 | 3 (container) |
| TP/oneM2M/CSE/DMR/DEL/017_GRP | Release 3 | 9 (group) |
| TP/oneM2M/CSE/DMR/DEL/017_ACP | Release 3 | 1 (accessControlPolicy) |
| TP/oneM2M/CSE/DMR/DEL/017_SCH | Release 3 | 18 (schedule) |
| TP/oneM2M/CSE/DMR/DEL/017_SUB | Release 3 | 23 (subscription) |
| TP/oneM2M/CSE/DMR/DEL/017_TS | Release 3 | 29 (timeSeries) |
| TP/oneM2M/CSE/DMR/DEL/017_TRAM | Release 4 | 50 (transactionMgmt) |
| TP/oneM2M/CSE/DMR/DEL/017_TRAC | Release 4 | 51 (transaction) |

##### 7.2.2.3.5	BASIC Operation

TP/oneM2M/CSE/DMR/001

| TP Id | TP/oneM2M/CSE/DMR/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the request to the hosting CSE when the To parameter contains the hosting CSE-ID which is known by the IUT (a descendant CSE or its Registrar CSE) |  |
| Reference | TS-0001 [1], clause 8.2.1.1 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to a Hosting CSE and the IUT having registered the AE and the Hosting CSE containing a resource representation on TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on the Hosting CSE } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from the AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | the IUT sends a OPERATION Request to the Hosting CSE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DMR/001_CRE | CREATE |
| TP/oneM2M/CSE/DMR/001_RET | RETRIEVE |
| TP/oneM2M/CSE/DMR/001_UPD | UPDATE |
| TP/oneM2M/CSE/DMR/001_DEL | DELETE |

TP/oneM2M/CSE/DMR/002

| TP Id | TP/oneM2M/CSE/DMR/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the response from the hosting CSE after retargeting an OPERATION Request |  |
| Reference | TS-0001 [1], clause 8.2.1.1 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the Hosting CSE and the IUT having registered the AE and the Hosting CSE containing a resource representation on TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on the Hosting CSE and the IUT having received a valid OPERATION Request from the AE containing To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID and the IUT sends the OPERATION Request to the Hosting CSE } |  |

- +----------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE |
                     | the IUT receives a valid OPERATION Response from the Hosting CSE  | |
                     |} | |

- +----------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response to the AE  | |
                     |} | |
+———————-+—————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DMR/002_CRE | CREATE |
| TP/oneM2M/CSE/DMR/002_RET | RETRIEVE |
| TP/oneM2M/CSE/DMR/002_UPD | UPDATE |
| TP/oneM2M/CSE/DMR/002_DEL | DELETE |

#### 7.2.2.4	Subscription and Notification (SUB)

##### 7.2.2.4.1	CREATE Operation

###### TP/oneM2M/CSE/SUB/CRE/001

| TP Id | TP/oneM2M/CSE/SUB/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the <subscription> resource when the target subscribed-to RESOURCE_TYPE resource is not subscribable |  |
| Reference | TS-0001 [1], clause 10.2.11.2 & TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | PARENT_RELEASE |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT having a resource PARENT_RESOURCE_ADDRESS not being subscribable RESOURCE_TYPE resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     |subscription resource representation  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the subscription resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5203 (TARGET_NOT_SUBSCRIBABLE) and  | |
                     | no Content  | |
                     |} | |
+———————-+—————————————————————————————————————————————-+————-+

| TP Id | PARENT_RELEASE | Reference | RESOURCE_TYPE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/SUB/CRE/001_CIN | Release 1 | TS-0001 [1], clause 9.6.7 | 4 (contentInstance) |
| TP/oneM2M/CSE/SUB/CRE/001_SUB | Release 1 | TS-0001 [1], clause 9.6.8 | 23 (subscription) |
| TP/oneM2M/CSE/SUB/CRE/001_TSI | Release 3 | TS-0001 [1], clause 9.6.37 | 30 (timeSeriesInstance) |

###### TP/oneM2M/CSE/SUB/CRE/002

| TP Id | TP/oneM2M/CSE/SUB/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the <subscription> resource when the originator does not have privileges for retrieving the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute pointing to accessControlPolicy resource containing privileges attribute containing accessControlRule attribute containing accessControlOriginators attribute indicating AE-IDand accessControlOperations attribute not indicating RETRIEVE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     |subscription resource representation  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the subscription resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)and  | |
                     | no Content  | |
                     |} | |
+———————-+—————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/CRE/003

| TP Id | TP/oneM2M/CSE/SUB/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the <subscription> resource when the notificationURI is not the originator and the IUT cannot send the Notify request to the notificationURI |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and the IUT not being able to send the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | From set to AE-ID and  | |
                     | Content containg  | |
                     | subscription resourcecontaining  | |
                     | notificationURI attribute set to NOTI_URI_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the subscription resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5204  | |
                     |(SUBSCRIPTION_VERIFICATION_INITIATION_FAILED)and  | |
                     | no Content  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/CRE/004

| TP Id | TP/oneM2M/CSE/SUB/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT stores Originator ID in the notification creator attribute when a <SUBSCRIPTION> creation request which needs verification is received and the notificationURI is not the Originator |  |
| Reference | TS-0001 [1], clause 10.2.12.1, TS-0004 [2], clause 7.4.9.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid CREATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 23 (subscription) and  | |
                     | Content containing  | |
                     | subscription resourcecontaining  | |
                     | notificationURI attribute set to AE2_RESOURCE_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE2 |
                     | the IUT sends a valid NOTIFY Request containing  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | verificationRequest attribute set to TRUE and  | |
                     | creator attribute set to AE1_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/CRE/005

| TP Id | TP/oneM2M/CSE/SUB/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the subscriber resource when notificationEventType attribute is set to "Create_of_Direct_Child_Resource" and a create operation has been performed on the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 9.6.8-3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 3 (Create_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid CREATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | CHILD_RESOURCE_TYPE resource containing  | |
                     | resourceName attribute set to NAME   | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2001 (CREATED)  |IUT -> AE2  |
                     | and the IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | Notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | CHILD_RESOURCE_TYPE resourcecontaining  | |
                     | All attributes  | |
                     |} | |
+———————-+——————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/CRE/006

| TP Id | TP/oneM2M/CSE/SUB/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a subscription resource with the eventNotificationCriteria attribute which includes a CONDITION_TAG condition provided |  |
| Reference | TS-0001 [1], clause 9.6.8 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | valid_CONDITION_TAG_ condition  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     |the IUT creates the subscription resource   | |
                     |and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————–+————-+

| TP Id | Reference | CONDITION_TAG |
| --- | --- | --- |
| TP/oneM2M/CSE/SUB/CRE/006_CRB | TS-0001 [1], clause 9.6.8 | createdBefore |
| TP/oneM2M/CSE/SUB/CRE/006_CRA | TS-0001 [1], clause 9.6.8 | createdAfter |
| TP/oneM2M/CSE/SUB/CRE/006_MS | TS-0001 [1], clause 9.6.8 | modifiedSince |
| TP/oneM2M/CSE/SUB/CRE/006_US | TS-0001 [1], clause 9.6.8 | unmodifiedSince |
| TP/oneM2M/CSE/SUB/CRE/006_STS | TS-0001 [1], clause 9.6.8 | stateTagSmaller |
| TP/oneM2M/CSE/SUB/CRE/006_STB | TS-0001 [1], clause 9.6.8 | stateTagBigger |
| TP/oneM2M/CSE/SUB/CRE/006_EXB | TS-0001 [1], clause 9.6.8 | expireBefore |
| TP/oneM2M/CSE/SUB/CRE/006_EXA | TS-0001 [1], clause 9.6.8 | expireAfter |
| TP/oneM2M/CSE/SUB/CRE/006_SZA | TS-0001 [1], clause 9.6.8 | sizeAbove |
| TP/oneM2M/CSE/SUB/CRE/006_SZB | TS-0001 [1], clause 9.6.8 | sizeBelow |
| TP/oneM2M/CSE/SUB/CRE/006_ET | TS-0001 [1], clause 9.6.8 | eventType |
| TP/oneM2M/CSE/SUB/CRE/006_OM | TS-0001 [1], clause 9.6.8 | operationMonitor |
| TP/oneM2M/CSE/SUB/CRE/006_ATR | TS-0001 [1], clause 9.6.8 | attribute |
| TP/oneM2M/CSE/SUB/CRE/006_ATR | TS-0001 [1], clause 9.6.8 | attribute |
| TP/oneM2M/CSE/SUB/CRE/006_CRT | TS-0001 [1], clause 9.6.8 | childResourceType |

###### TP/oneM2M/CSE/SUB/CRE/007

| TP Id | TP/oneM2M/CSE/SUB/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a subscription resource with the eventNotificationCriteria attribute which includes a missingData condition provided |  |
| Reference | TS-0001 [1], clause 9.6.8, 9.6.36 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a TARGET_RESOURCE_ADDRESS of type <timeSeries > and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | valid missingData condition  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT creates the subscription resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/CRE/008

| TP Id | TP/oneM2M/CSE/SUB/CRE/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a subscription resource with the optional attributes notificationForwardingURI and groupID provided |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.7.10, TS-0004 [2], clause 7.4.14.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource as TARGET_RESOURCE_ADDRESS containing members which are subscribable and the Originatorhaving privileges to perform CREATE operation on <group> members } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | groupID_,_ notificationURI attribute, and  | |
                     | notificationForwardingURI attribute set   | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT creates the subscription resource on the <group> members containing  | |
                     | groupID, notificationURI attribute and  | |
                     | notificationForwardingURI attribute set identical to notificationURI  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+—————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SUB/CRE/009

| TP Id | TP/oneM2M/CSE/SUB/CRE/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to 6 (Trigger_Received_For_AE_Resource) in combination of NOTIFICATION_EVENT_TYPE |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to 6  | |
                     | (Trigger_Received_For_AE_Resource) and  | |
                     | NOTIFICATION_EVENT_TYPE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+————————————————————————————————————————————————————–+————-+

| TP Id | NOTIFICATION_EVENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/009_NET/1 | 1 (Update_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/009_NET/2 | 2 (Delete_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/009_NET/3 | 3 (Create_of_Direct_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/009_NET/4 | 4 (Delete_of_Direct_Child_Resource ) |
| TP/oneM2M/CSE/SUB/CRE/009_NET/5 | 5 (Retrieve_of_Container_Resource_With_No_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/009_NET/7 | 7 (Blocking_Update) |

TP/oneM2M/CSE/SUB/CRE/010

| TP Id | TP/oneM2M/CSE/SUB/CRE/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to 7 (Blocking Update) in combination of NOTIFICATION_EVENT_TYPE |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to 7 (Blocking Update)  | |
                     | and NOTIFICATION_EVENT_TYPE  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

| TP Id | NOTIFICATION_EVENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/010_NET/1 | 1 (Update_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/010_NET/2 | 2 (Delete_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/010_NET/3 | 3 (Create_of_Direct_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/010_NET/4 | 4 (Delete_of_Direct_Child_Resource ) |
| TP/oneM2M/CSE/SUB/CRE/010_NET/5 | 5 (Retrieve_of_Container_Resource_With_No_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/010_NET/6 | 6 (Trigger_Received_For_AE_Resource) |

TP/oneM2M/CSE/SUB/CRE/011

| TP Id | TP/oneM2M/CSE/SUB/CRE/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to 6 (Trigger_Received_For_AE_Resource) and notificationContentType is set to NOTIFICATION_CONTENT_TYPE |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to 6  | |
                     | (Trigger_Received_For_AE_Resource) and  | |
                     | notificationContentType attribute set to  | |
                     | NOTIFICATION_CONTENT_TYPE   | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+————-+

| TP Id | NOTIFICATION_CONTENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/011_NCT/1 | 1 (All Attributes) |
| TP/oneM2M/CSE/SUB/CRE/011_NCT/2 | 2 (Modified Attributes) |
| TP/oneM2M/CSE/SUB/CRE/011_NCT/3 | 3 (ResourceID) |

TP/oneM2M/CSE/SUB/CRE/012

| TP Id | TP/oneM2M/CSE/SUB/CRE/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to 7 (Blocking Update) and notificationContentType attribute set to NOTIFICATION_CONTENT_TYPE |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to 7 (Blocking Update)  | |
                     | and  | |
                     | notificationContentType attribute set to  | |
                     | NOTIFICATION_CONTENT_TYPE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————-+————-+

| TP Id | NOTIFICATION_CONTENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/012_NCT/1 | 1 (All attributes) |
| TP/oneM2M/CSE/SUB/CRE/012_NCT/3 | 3 (ResourceID) |
| TP/oneM2M/CSE/SUB/CRE/012_NCT/4 | 4 (Trigger Payload) |

TP/oneM2M/CSE/SUB/CRE/013

| TP Id | TP/oneM2M/CSE/SUB/CRE/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to NOTIFICATION_EVENT_TYPE and notificationContentType set to 4 (Trigger Payload) |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to  | |
                     | NOTIFICATION_EVENT_TYPE and  | |
                     | notificationContentType attribute set to 4 (Trigger Payload)  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+———————————————————————————————————————————————————-+————-+

| TP Id | NOTIFICATION_EVENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/013_NET/1 | 1 (Update_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/013_NET/2 | 2 (Delete_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/013_NET/3 | 3 (Create_of_Direct_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/013_NET/4 | 4 (Delete_of_Direct_Child_Resource ) |
| TP/oneM2M/CSE/SUB/CRE/013_NET/5 | 5 (Retrieve_of_Container_Resource_With_No_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/013_NET/7 | 7 (Blocking_Update) |

TP/oneM2M/CSE/SUB/CRE/014

| TP Id | TP/oneM2M/CSE/SUB/CRE/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to NOTIFICATION_EVENT_TYPE and notificationContentType set to 2(modified attributes) |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving** privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to  | |
                     | NOTIFICATION_EVENT_TYPE and  | |
                     | notificationContentType attribute set to 2 (Modified Attributes)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+————————————————————————————————————————————————————-+————-+

| TP Id | NOTIFICATION_EVENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/014_NET/2 | 2 (Delete_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/014_NET/3 | 3 (Create_of_Direct_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/014_NET/4 | 4 (Delete_of_Direct_Child_Resource ) |
| TP/oneM2M/CSE/SUB/CRE/014_NET/5 | 5 (Retrieve_of_Container_Resource_With_No_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/014_NET/6 | 6 (Trigger_Received_For_AE_Resource) |

TP/oneM2M/CSE/SUB/CRE/015

| TP Id | TP/oneM2M/CSE/SUB/CRE/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to NOTIFICATION_EVENT_TYPE and notificationContentType set to 3 (ResourceID) |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving** privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to  | |
                     | NOTIFICATION_EVENT_TYPE and  | |
                     | notificationContentType attribute set to 3 (ResourceID)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+—————————————————————————————————————————————————–+————-+

| TP Id | NOTIFICATION_EVENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/015_NET/6 | 6 (Trigger_Received_For_AE_Resource) |
| TP/oneM2M/CSE/SUB/CRE/015_NET/7 | 7 (Blocking_Update ) |
| TP/oneM2M/CSE/SUB/CRE/015_NET/8 | 8 (Report_on_missing_data_points) |

TP/oneM2M/CSE/SUB/CRE/016

| TP Id | TP/oneM2M/CSE/SUB/CRE/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to NOTIFICATION_EVENT_TYPE and notificationContentType set to 5 (TimeSeries notification) |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to  | |
                     | NOTIFICATION_EVENT_TYPE and  | |
                     | notificationContentType attribute set to 5 (TimeSeries notification)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————+————-+

| TP Id | NOTIFICATION_EVENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/016_NET/1 | 1 (Update_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/016_NET/2 | 2 (Delete_of_Resource) |
| TP/oneM2M/CSE/SUB/CRE/016_NET/3 | 3 (Create_of_Direct_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/016_NET/4 | 4 (Delete_of_Direct_Child_Resource ) |
| TP/oneM2M/CSE/SUB/CRE/016_NET/5 | 5 (Retrieve_of_Container_Resource_With_No_Child_Resource) |
| TP/oneM2M/CSE/SUB/CRE/016_NET/6 | 6 (Trigger_Received_For_AE_Resource) |
| TP/oneM2M/CSE/SUB/CRE/016_NET/7 | 7 (Blocking_Update ) |

TP/oneM2M/CSE/SUB/CRE/017

| TP Id | TP/oneM2M/CSE/SUB/CRE/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a CREATE request when the notificationEventType is set to NOTIFICATION_EVENT_TYPE and notificationContentType set to 1 (All Attributes) |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.10.2, TS-0004 [2], clause 7.4.8.2.1, 6.3.4.2.18-19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource as TARGET_RESOURCE_ADDRESS which is subscribable and the Originatorhaving** privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to subscription and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to  | |
                     | NOTIFICATION_EVENT_TYPE and  | |
                     | notificationContentType attribute set to 1 (All Attributes)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST) and  | |
                     | no Content  | |
                     |} | |
+———————-+———————————————————————————————————————————————————+————-+

| TP Id | NOTIFICATION_EVENT_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/CRE/017_NET/6 | 6 (Trigger_Received_For_AE_Resource) |
| TP/oneM2M/CSE/SUB/CRE/017_NET/7 | 7 (Blocking_Update ) |
| TP/oneM2M/CSE/SUB/CRE/017_NET/8 | 8 (Report_on_missing_data_points) |

##### 7.2.2.4.2	DELETE Operation

###### TP/oneM2M/CSE/SUB/DEL/001

| TP Id | TP/oneM2M/CSE/SUB/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the AE2_RESOURCE_ADDRESS of the subscriberURI attribute when the SUBSCRIPTION_RESOURCE_ADDRESS <subscription> resource is deleted |  |
| Reference | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.4 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription resource containing subscriberURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid DELETE Request from AE1 containing  | |
                     | To set to SUBSCRIPTION_RESOURCE_ADDRESS and  | |
                     | From set to AE1_RESOURCE_ADDRESS  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2002 (DELETED)  |IUT -> AE2 |
                     | and the IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | subscriptionDeletion attribute set to TRUE and  | |
                     | subscriptionReference attribute set to  | |
                     |AE2_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/DEL/002

| TP Id | TP/oneM2M/CSE/SUB/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the subscriber resource when eventNotificationCriteria/notificationEventType attribute is set to "Delete_of_Resource" and an delete operation has been performed the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 9.6.8-3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 2 (Delete_of_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid DELETE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2002 (DELETED)  |IUT -> AE2 |
                     | and the IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | a valid representation attribute  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/DEL/003

| TP Id | TP/oneM2M/CSE/SUB/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the subscriber resource when eventNotificationCriteria/notificationEventType attribute is set to "Delete_of_Direct_Child_Resource" and an delete operation has been performed on a child resource of the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 9.6.8-3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing a CHILD_RESOURCE_ADDRESS child resource and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 4 (Delete_of_Direct_Child_Resource) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid DELETE Request from AE1 containing  | |
                     | To set to CHILD_RESOURCE_ADDRESS and  | |
                     | From set to AE1_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2002 (DELETED)  |IUT -> AE2 |
                     | and the IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | CHILD_RESOURCE_TYPE resource  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————————–+————-+

##### 7.2.2.4.3	UPDATE Operation

###### TP/oneM2M/CSE/SUB/UPD/001

| TP Id | TP/oneM2M/CSE/SUB/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends Notify request to the subscriber resource when an update operation has been performed on the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 10.2.11.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME set to VALUE_2  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2004 (UPDATED)  |IUT -> AE2  |
                     | and the IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | All attributes  | |
                     |} | |
+———————-+——————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SUB/UPD/002

| TP Id | TP/oneM2M/CSE/SUB/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to "modified attributes" and an update operation has been performed on the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESSand notificationContentType attribute set to 2 (modified attributes) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2004 (UPDATED)  |IUT -> AE2 |
                     | and the IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/UPD/003

| TP Id | TP/oneM2M/CSE/SUB/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the subscriber resource when the notificationContentType attribute is set to "ResourceID" and an update operation has been performed on the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and notificationContentType attribute set to 3 (ResourceID) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | < subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2004 (UPDATED)  |IUT -> AE2 |
                     | and the IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | subscribed-to ResourceID  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/UPD/004

| TP Id | TP/oneM2M/CSE/SUB/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT decreases the expirationCounter attribute of a subscription resource when the Hosting CSE of the subscribed-to resource successfully sends the notification request to subscriber resource(s) |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to MAX_NUMBER_NOTIFICATION and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESS a nd  | |
                     | From set to AE1_RESOURCE_ADDRESSand  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT decrease s the expirationCounter attribute of the subscription resource  |  |
                     | and the IUT sends a valid Response containing  |IUT -> AE2 |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | and the IUT sends a NOTIFY Request  | |
                     |  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/UPD/005

| TP Id | TP/oneM2M/CSE/SUB/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT deletes the subscription resource when the the expirationCounter meets zero |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and expirationCounter attribute set to 1 and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +---------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESSand  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |

- +---------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT decrease s the expirationCounter attribute of the subscription resource |  |
                     | and the IUT deletes the subscription resource  |IUT -> AE2 |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | and the IUT sends a NOTIFY Request  | |
                     |} | |
+———————-+———————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/UPD/006

| TP Id | TP/oneM2M/CSE/SUB/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT doesn't send a Notify request to the subscriber resource when the "attribute" condition tag of eventNotificationCriteria attribute is set to LIST_OF_ATTRIBUTE and doesn't contain the ATTRIBUTE_NAME attribute updated |  |
| Reference | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to LIST_OF_ATTRIBUTE containing no ATTRIBUTE_NAME attribute and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESSand  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | and the IUT does not send a NOTIFY Request  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/UPD/007

| TP Id | TP/oneM2M/CSE/SUB/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends batched notifications to the subscriber resource when the number value of the batchNotify attribute is set to NUMBER and when this number have been reached |  |
| Reference | TS-0001 [1], clause 10.2.12.1 |  |
| Config Id | CF01 |  |
| Parent release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing number attribute set to NUMBERand duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (NUMBER- 1) valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+---------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | a valid ATTRIBUTE_NAME attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+---------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2004 (UPDATED)  |IUT -> AE2 |
                     | and the IUT sends NUMBER times a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | a valid notificationEvent attribute  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————+—————+

###### TP/oneM2M/CSE/SUB/UPD/008

| TP Id | TP/oneM2M/CSE/SUB/UPD/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the number value of the batchNotify attribute is set to NUMBER and when this number has been reached |  |
| Reference | TS-0001 [1], clause 10.2.12.1 |  |
| Config Id | CF01 |  |
| Parent release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing number attribute set to NUMBERand duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received (NUMBER- 1) valid UPDATE Requests from AE1_RESOURCE_ADDRESS to subscribed-to resource and the timer has not reached the TIME_LIMIT duration value } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESSand  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | a valid ATTRIBUTE_NAME attribute  | |
                     | } | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2004 (UPDATED)  |IUT -> AE2  |
                     | and the IUT sends a NOTIFY Request contaning  |  |
                     | Content containing  | |
                     | the latest notification message containing  | |
                     | a valid notificationEvent attribute  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/UPD/009

| TP Id | TP/oneM2M/CSE/SUB/UPD/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the subscriber resource when the "attribute" condition tag of eventNotificationCriteria attribute is set to ATTRIBUTE_NAME and an update operation has been performed on the subscribed-to resource |  |
| Reference | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing attribute condition tag set to ATTRIBUTE_NAME and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_2  | |
                     | } | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2004 (UPDATED)  |IUT -> AE2  |
                     | and the IUT sends a NOTIFY Request contaning  |  |
                     | Content containing  |  |
                     | notification message containing  |  |
                     | notificationEvent attribute containing  |  |
                     | representation attribute containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | All attributes  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————+————-+

##### 7.2.2.4.4	NOTIFY Operation

###### TP/oneM2M/CSE/SUB/NTF/001

| TP Id | TP/oneM2M/CSE/SUB/NTF/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the < subscription_>_ resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Codeset to 4101  | |
                     |(SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the subscription resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4101  | |
                     |(SUBSCRIPTION_CREATOR_HAS_NO_PRIVILEGE)and  | |
                     | no Content  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/002

| TP Id | TP/oneM2M/CSE/SUB/NTF/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the < subscription_>_ resource when the notificationURI is not the originator and the IUT have received the Notify response containing Response Status Code indicating SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE |  |
| Reference | TS-0001 [1], clause 10.2.11.2, TS-0004 [2], clause 7.4.8.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation and notificationURI not being the originator and the IUT having sent the NOTIFY Request to the NOTI_URI_ADDRESS containing verificationRequest set to TRUE } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 5205  | |
                     |(SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the subscription resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5205  | |
                     |(SUBSCRIPTION_HOST_HAS_NO_PRIVILEGE)and  | |
                     | no Content  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/003

| TP Id | TP/oneM2M/CSE/SUB/NTF/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends batched notifications to the subscriber resource when the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference | TS-0001 10.2.12.1 |  |
| Config Id | CF01 |  |
| Parent release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME attribute set to VALUE_1 and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and batchNotify attribute containing duration attrinbute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to subscribed-to resource and timer has not reached TIME_LIMIT } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the timer expires after the TIME_LIMIT duration value  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE2 |
                     | The IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | a valid notificationEvent attribute  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SUB/NTF/004

| TP Id | TP/oneM2M/CSE/SUB/NTF/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the latest notification to the subscriber resource when latestNotify is set to TRUE, the duration value of the batchNotify attribute is set to TIME_LIMIT and when this timer expires |  |
| Reference | TS-0001 [1], clause 10.2.12.1 |  |
| Config Id | CF01 |  |
| Parent release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1and subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and latestNotify attribute set to TRUE and batchNotify attribute containing duration attribute set to TIME_LIMIT and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to subscribed-to resource and timer has not reached TIME_LIMIT } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the timer expires after the TIME_LIMIT duration value  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE2 |
                     | The IUT sends a NOTIFY Request contaning  | |
                     | Content containing  | |
                     | the latest notification message containing  | |
                     | a valid notificationEvent attribute   | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/005

| TP Id | TP/oneM2M/CSE/SUB/NTF/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the latest cached notification after IUT escapes from connectionless state when the pendingNotification is set to 1 (sendLatest) |  |
| Reference | TS-0001 [1], clause 9.6.8 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and pendingNotification attribute set to 1 (sendLatest) and the AE2 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT being in the "connectionless state" with AE2 and the IUT having frequently received from AE1 a valid UPDATE Requests to subscribed-to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT escapes from the "connectionless state"  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE2 |
                     | The IUT sends a valid NOTIFY Request contaning  | |
                     | Content containing  | |
                     | the latest notification message  | |
                     |} | |
+———————-+——————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/006

| TP Id | TP/oneM2M/CSE/SUB/NTF/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the notifications of cached notification messages after IUT escapes from connectionless state when the pendingNotification is set to 2 (sendAllPending) |  |
| Reference | TS-0001 [1], clause 9.6.8 |  |
| Config Id | CF01 |  |
| Parent release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource having notificationURI attribute set to AE2_RESOURCE_ADDRESS and pendingNotification attribute set to 2 (sendAllPending) and the AE2 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT being in the "connectionless state" with AE2 and the IUT having received from AE1 a NUMBER of valid UPDATE Requests to subscribed- to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT escapes from the "connectionless state"  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE2 |
                     | The IUT sends NUMBER times a valid NOTIFY Request contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | a valid notificationEvent attribute  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/007

| TP Id | TP/oneM2M/CSE/SUB/NTF/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the notification to the AE according to pointOfAccess in the <AE> resource |  |
| Reference | TS-0001 [1], clause 9.6.5 and clause 9.3.2.3.1 TS-0004 [2], clause 7.3.3.9 |  |
| Config Id | CF02 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUThaving created the AE resource containing POINT_OF_ACCESS attribute and REQUEST_REACHABILITYset to true } |  |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- CSE |
                     | the IUT receives a valid NOTIFY from CSE containing  | |
                     | To set to AE_RESOURCE_ADDRESS  | |
                     | From set to CSE_ID   | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends the valid NOTIFY to POINT_OF_ACCESS  | |
                     |} | |
+———————-+———————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/008

| TP Id | TP/oneM2M/CSE/SUB/NTF/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the notification to the AE according to pointOfAccess in the <AE> resource and forward the response back to the CSE |  |
| Reference | TS-0001 [1], clause 9.6.5 and clause 9.3.2.3.1, TS-0004 [2], clause 7.3.3.9 |  |
| Config Id | CF02 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having received a valid NOTIFY from CSE and The IUT having sent the NOTIFY to the AE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                                                            | the IUT receives a valid NOTIFY RESPONSE from the AE  | |
                                                            |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> CSE |
                                                            | the IUT sends the valid NOTIFY RESPONSE to the CSE  | |
                                                            |} | |
+————————————————————-+————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/009

| TP Id | TP/oneM2M/CSE/SUB/NTF/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an aggregatedNotification to the AE according to URI provided in notificationForwardingURI attribute when an event occurs on <group> resource with its members having group related subscriptions |  |
| Reference | TS-0001 [1], clause 9.6.8 and clause 10.2.7.11, TS-0004 [2], clause 7.5.1.2.6 |  |
| Config Id | CF02 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and and the IUT having a <group> resource with members allowing the AE to perform required operation and <subscription> created on these members with notificationForwardingURI attribute set identical to notificationURI } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                                                               | the IUT receives a single notifications containing notificationForwardingURI, it  | |
                                                               |forms an aggregatedNotification   | |
                                                               |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                                                               | the IUT sends the Notify request containing  | |
                                                               | aggregatedNotification message representation to the AE  | |
                                                               |} | |
+—————————————————————-+——————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/010

| TP Id | TP/oneM2M/CSE/SUB/NTF/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT doesn't send a Notify request to the subscriber resource when "childResourceType" condition tag of eventNotificationCriteria attribute is set to LIST_OF_RESOURCE_TYPE and this list doesn't contain the CHILD_RESOURCE_TYPE created in the request |  |
| Reference | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8, 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having created a subscription resource on SUBSCRIBED_TO_RESOURCE_ADDRESS resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType element set to 3 (Create_of_Direct_Child_Resource) and childResourceType element set to LIST_OF_RESOURCE_TYPE and AE1 having create privileges SUBSCRIBED_TO_RESOURCE_ADDRESS resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid CREATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_IDand  | |
                     | Content containing  | |
                     | CHILD_RESOURCE_TYPE resource representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     | and the IUT does not send a NOTIFY Request to  | |
                     |AE2_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/011

| TP Id | TP/oneM2M/CSE/SUB/NTF/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a Notify request to the subscriber resource when "childResourceType" condition tag of eventNotificationCriteria attribute is set to LIST_OF_RESOURCE_TYPE and this list contains the CHILD_RESOURCE_TYPE created in the request |  |
| Reference | TS-0001 [1], clause 9.6.8, TS-0004 [2], clause 7.4.8, 7.5.1.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having created a subscription resource on SUBSCRIBED_TO_RESOURCE_ADDRESS resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType element set to 3 (Create_of_Direct_Child_Resource) and childResourceType element set to LIST_OF_RESOURCE_TYPE and AE1 having create privileges SUBSCRIBED_TO_RESOURCE_ADDRESS resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid CREATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_ID and  | |
                     | Content containing  | |
                     | CHILD_RESOURCE_TYPE resource representation  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response containing  |IUT -> AE2 |
                     | Response Status Code set to 2001 (CREATED)  | |
                     | and the IUT sends a NOTIFY Request to AE2_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/012

| TP Id | TP/oneM2M/CSE/SUB/NTF/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully sends a cross-resource notification to the notificationURI when notifications from resources in regularResourcesAsTarget are received within required TIME_LIMIT window |  |
| Reference | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 and the IUThaving created a <crossResourceSubsription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS_1 and CONTAINER_RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |  |
                     | the IUT receives a valid UPDATE Request (Req1) from AE1contaning  |IUT <- AE1  |
                     | To set to CONTAINER_RESOURCE_ADDRESS_1 and  |  |
                     | From set to AE_ID and  |  |
                     | Content containing  |  |
                     | container resource containing  |  |
                     | valid labels attribute  |  |
                     | and the IUT receives a valid UPDATE Request (Req2) from AE1contaning  |IUT <- AE1 |
                     | To set to CONTAINER_RESOURCE_ADDRESS_2 and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid labels attribute  | |
                     | and timer has not reached TIME_LIMIT  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response to AE1 for Req1  |IUT -> AE1  |
                     | and the IUT sends a valid Response to AE1 for Req2  |IUT -> AE1  |
                     | and the IUT sends a NOTIFY Request to AE2 generated from  |IUT -> AE2 |
                     | crossResourceSubsription on CROSS_RESOURCE_SUBS_ADDRESS  | |
                     | containing  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | subscriptionReference attribute set to  | |
                     | CROSS_RESOURCE_SUBS_ADDRESS  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————-+————-+

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/012_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/012_TWT/2 | 2 (SLIDINGWINDOW) |

###### TP/oneM2M/CSE/SUB/NTF/013

| TP Id | TP/oneM2M/CSE/SUB/NTF/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not send a cross-resource notification to the notificationURI when a notification from resource in regularResourcesAsTarget is not received within required TIME_LIMIT window |  |
| Reference | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 and the IUThaving created a <crossResourceSubsription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS_1 and CONTAINER_RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |  |
                     | the IUT receives a valid UPDATE Request (Req1) from AE1contaning  |IUT <- AE1  |
                     | To set to CONTAINER_RESOURCE_ADDRESS_1 and  |  |
                     | From set to AE_ID and  |  |
                     | Content containing  |  |
                     | container resource containing  |  |
                     | valid labels attribute  |  |
                     | and the IUT receives a valid UPDATE Request (Req2) from AE1contaning  |IUT <- AE1 |
                     | To set to CONTAINER_RESOURCE_ADDRESS_2 and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid labels attribute  | |
                     | and timer has reached TIME_LIMIT  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE1  |
                     | the IUT sends a valid Response to AE1 for Req1  |IUT -> AE1  |
                     | and the IUT sends a valid Response to AE1 for Req2  | |
                     | and the IUT does not send a NOTIFY Request to AE2  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————–+————-+

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/013_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/013_TWT/2 | 2 (SLIDINGWINDOW) |

###### TP/oneM2M/CSE/SUB/NTF/014

| TP Id | TP/oneM2M/CSE/SUB/NTF/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a cross-resource notification to the notificationURI when notifications from resources in regularResourcesAsTarget are received within required TIME_LIMIT window having different NOTIFICATION_EVENT_TYPE values for each of the target resources |  |
| Reference | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 and the IUThaving created a <crossResourceSubsription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing regularResourcesAsTarget attribute containing CONTAINER_RESOURCE_ADDRESS_1 and CONTAINER_RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and eventNotificationCriteriaSet containing EVENT_NOTIFICATION_CRITERIA_1 set to 1 (Update_of_resource) and EVENT_NOTIFICATION_CRITERIA_2 set to 3 (Create_of_Direct_Child_Resource) and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |  |  |
                     |when {  |  |
                     | the IUT receives a valid UPDATE Request (Req1) from AE1contaning  |IUT <- AE1  |
                     | To set to CONTAINER_RESOURCE_ADDRESS_1 and  |  |
                     | From set to AE_ID and  |  |
                     | Content containing  |  |
                     | container resource containing  |  |
                     | valid labels attribute  |  |
                     | and the IUT receives a valid CREATE Request (Req2) from AE1contaning  |IUT <- AE1 |
                     | To set to CONTAINER_RESOURCE_ADDRESS_2 and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | a valid container resource representation  | |
                     | and timer has not reached TIME_LIMIT  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response to AE1 for Req1  |IUT -> AE1  |
                     | and the IUT sends a valid Response to AE1 for Req2  |IUT -> AE1  |
                     | and the IUT sends a NOTIFY Request to AE2 generated from  |IUT -> AE2 |
                     | crossResourceSubscription CROSS_RESOURCE_SUBS_ADDRESS  | |
                     | containing  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | subscriptionReference attribute set to CROSS_SUBS_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/014_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/014_TWT/2 | 2 (SLIDINGWINDOW) |

###### TP/oneM2M/CSE/SUB/NTF/015

| TP Id | TP/oneM2M/CSE/SUB/NTF/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully sends a cross-resource notification to the notificationURI when notifications from resources in subscriptionResourcesAsTarget are received within required TIME_LIMIT window |  |
| Reference | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 containing a subscription resource on SUBS_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 containing a subscription resource on SUBS_RESOURCE_ADDRESS_2 and the IUThaving created a <crossResourceSubscription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing subscriptionResourcesAsTarget attribute containing SUBS _RESOURCE_ADDRESS_1 and SUBS _RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |  |
                     | the IUT receives a valid UPDATE Request (Req1) from AE1contaning  |IUT <- AE1  |
                     | To set to AE1_RESOURCE_ADDRESS_1 and  |  |
                     | From set to AE_ID and  |  |
                     | Content containing  |  |
                     | AE resource containing  |  |
                     | valid labels attribute  |  |
                     | and the IUT receives a valid UPDATE Request (Req2) from AE1contaning  |IUT <- AE1 |
                     | To set to AE1_RESOURCE_ADDRESS_2 and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | valid labels attribute  | |
                     | and timer has not reached TIME_LIMIT  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response to AE1 for Req1  |IUT -> AE1  |
                     | and the IUT sends a valid Response to AE1 for Req2  |IUT -> AE1  |
                     | and the IUT sends a NOTIFY Request to AE1 generated from  |IUT -> AE1  |
                     | SUBS_RESOURCE_ADDRESS_1  |  |
                     | and the IUT sends a NOTIFY Request to AE1 generated from  |IUT -> AE1  |
                     | SUBS_RESOURCE_ADDRESS_2  |  |
                     | and the IUT sends a NOTIFY Request to AE2 generated from  |IUT -> AE2 |
                     | crossResourceSubscription CROSS_RESOURCE_SUBS_ADDRESS  | |
                     | containing  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | subscriptionReference attribute set to  | |
                     | CROSS_SUBS_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————+————-+

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/015_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/015_TWT/2 | 2 (SLIDINGWINDOW) |

###### TP/oneM2M/CSE/SUB/NTF/016

| TP Id | TP/oneM2M/CSE/SUB/NTF/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not send a cross-resource notification to the notificationURI when a notification from resource in subscriptionResourcesAsTarget is not received within required TIME_LIMIT window |  |
| Reference | TS-0001 [1], clause 9.6.58, TS-0004 [2], clause 7.5.1.2.18 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 containing a container resource on CONTAINER_RESOURCE_ADDRESS_1 containing a subscription resource on SUBS_RESOURCE_ADDRESS_1 a container resource on CONTAINER_RESOURCE_ADDRESS_2 containing a subscription resource on SUBS_RESOURCE_ADDRESS_2 and the IUThaving created a <crossResourceSubscription> resource on CROSS_RESOURCE_SUBS_ADDRESS containing subscriptionResourcesAsTarget attribute containing SUBS _RESOURCE_ADDRESS_1 and SUBS _RESOURCE_ADDRESS_2 and timeWindowType attribute set to TIME_WINDOW_TYPE and timeWindowSize attribute set to DURATION and notificationURI attribute set to AE2_RESOURCE_ADDRESS and the IUT having registered the AE2 } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |  |
                     | the IUT receives a valid UPDATE Request (Req1) from AE1contaning  |IUT <- AE1  |
                     | To set to AE1_RESOURCE_ADDRESS_1 and  |  |
                     | From set to AE_ID and  |  |
                     | Content containing  |  |
                     | AE resource containing  |  |
                     | valid labels attribute  |  |
                     | and the IUT receives a valid UPDATE Request (Req2) from AE1contaning  |IUT <- AE1 |
                     | To set to AE1_RESOURCE_ADDRESS_2 and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | valid labels attribute  | |
                     | and timer has reached TIME_LIMIT  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response to AE1 for Req1  |IUT -> AE1  |
                     | and the IUT sends a valid Response to AE1 for Req2  |IUT -> AE1  |
                     | and the IUT sends a NOTIFY Request to AE1 generated from  |IUT -> AE1  |
                     | SUBS_RESOURCE_ADDRESS_1  |  |
                     | and the IUT sends a NOTIFY Request to AE1 generated from  |IUT -> AE1 |
                     | SUBS_RESOURCE_ADDRESS_2  | |
                     | and the IUT does not send a NOTIFY Request to AE2  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————-+————-+

| TP Id | TIME_WINDOW_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SUB/NTF/016_TWT/1 | 1 (PERIODICWINDOW) |
| TP/oneM2M/CSE/SUB/NTF/016_TWT/2 | 2 (SLIDINGWINDOW) |

###### TP/oneM2M/CSE/SUB/NTF/017

| TP Id | TP/oneM2M/CSE/SUB/NTF/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully sends notification request to the notificationURI when an update request is received for the target subscribed-to resource with notificationEventType set to "Blocking_Update" |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME set to VALUE_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE2  |
                     | the IUT sends a NOTIFY Request to AE2 contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | All attributes  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/018

| TP Id | TP/oneM2M/CSE/SUB/NTF/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully sends notification request to the notificationURI when an update request is received for the target subscribed-to resource with notificationEventType set to "Blocking_Update" when the attribute condition tag matches the modified attributes |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) attribute set to ATTRIBUTE_NAME and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME set to VALUE_2  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE2  |
                     | the IUT sends a NOTIFY Request to AE2 contaning  | |
                     | Content containing  | |
                     | notification message containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | All attributes  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SUB/NTF/019

| TP Id | TP/oneM2M/CSE/SUB/NTF/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not send notification request to the notificationURI when an update request is received for the target subscribed-to resource with notificationEventType set to "Blocking_Update" but the attribute condition tag does not match the modified attributes |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME_1 set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) attribute set to ATTRIBUTE_NAME_2 and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME_1 set to VALUE_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   | |
                     | the IUT does not send a NOTIFY Request to AE2_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/020

| TP Id | TP/oneM2M/CSE/SUB/NTF/020 |  |
| --- | --- | --- |
| Test objective | Check that the IUT blocks update request for the target subscribed-to resource when an previous update request is received with notificationEventType set to "Blocking_Update" and notification process is still not completed |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent a valid NOTIFY Request to AE2_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESSand  | |
                     | From set to AE1_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | <subscribed-to type> resourcecontaining  | |
                     | ATTRIBUTE_NAME set to VALUE_2  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   | |
                     | the IUT does not update SUBSCRIBED_TO_RESOURCE_ADDRESS  | |
                     | and the IUT does not send a NOTFIY Request to  | |
                     |AE2_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/021

| TP Id | TP/oneM2M/CSE/SUB/NTF/021 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs update request to the target subscribed-to resource with notificationEventType set to "Blocking_Update" |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent the NOTIFY Request to the AE2_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE2 |
                     | the IUT receives a valid NOTIFY Response from AE2 containing  | |
                     | ResponseStatusCode set to OK (2000)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE1  |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/022

| TP Id | TP/oneM2M/CSE/SUB/NTF/022 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update request to the target subscribed-to resource when notificationEventType is set to "Blocking_Update" and the IUT have received the Notify response containing Response Status Code indicating TARGET_NOT_REACHABLE |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent a NOTIFY Request to AE2_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE2 |
                     | the IUT receives a valid NOTIFY Response from AE2 containing  | |
                     | ResponseStatusCode set to TARGET_NOT_REACHABLE (5103)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE1  |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5107 (REMOTE_ENTITY_NOT_REACHABLE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SUB/NTF/023

| TP Id | TP/oneM2M/CSE/SUB/NTF/023 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update request to the target subscribed-to resource when notificationEventType is set to "Blocking_Update" and the IUT have received the Notify response containing Response Status Code indicating OPERATION_NOT_ALLOWED |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing ATTRIBUTE_NAME set to VALUE_1 and subscription child resource containing notificationURI attribute set to AE2_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource and the IUT having received a valid UPDATE Request from AE1_RESOURCE_ADDRESS to SUBSCRIBED_TO_RESOURCE_ADDRESS and the IUT having sent a NOTIFY Request to AE2_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE2 |
                     | the IUT receives a valid NOTIFY Response from AE2 containing  | |
                     | ResponseStatusCode set to OPERATION_NOT_ALLOWED (4005)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE1  |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4127 (OPERATION_DENIED_BY_REMOTE_ENTITY)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SUB/NTF/024

| TP Id | TP/oneM2M/CSE/SUB/NTF/024 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of a subscription resource with notificationEventType set to "Blocking_Update" when the subscribed-to resource has already a subscription resource with this notificationEventType |  |
| Reference | TS-0004 [1], clause 7.5.1.2.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a <subscribed-to type> resource containing subscription child resource containing eventNotificationCriteria attribute containing notificationEventType set to 7 (Blocking_Update) and the AE1 having privileges to perform RETRIEVE operation on the subscribed-to resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE1 |
                     | the IUT receives a valid CREATE Request from AE1 containing  | |
                     | To set to SUBSCRIBED_TO_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | a subscription resource containing  | |
                     | eventNotificationCriteria attribute containing  | |
                     | notificationEventType set to 7 (Blocking_Update)   | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE1  |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4124  | |
                     |(BLOCKING_SUBSCRIPTION_ALREADY_EXISTS)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————+————-+

#### 7.2.2.5	Group Management (GMG)

##### 7.2.2.5.1	CREATE Operation

###### TP/oneM2M/CSE/GMG/CRE/001

| TP Id | TP/oneM2M/CSE/GMG/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the group resource when member ID exceed max number of members |  |
| Reference | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.3.13 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to | |
                     | LIST_OF_MEMBER_RESOURCE_IDS and  | |
                     | maxNrOfMembers attribute set to MAX_NR_MEMBERS less  | |
                     | than the number of memerIDs set in memberIDs attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 6010  | |
                     | (MAX_NUMBER_OF_MEMBER_EXCEEDED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/GMG/CRE/002

| TP Id | TP/oneM2M/CSE/GMG/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the group resource when the memberType cannot be retrieved due to lack of privilege |  |
| Reference | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.3.13 |  |
| Parent Release | Release 1 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and IUT registered with remote CSE CSE1 and the AE having created a resource at MEMBER_RESOURCE_ADDRESS_1 at another CSE1 containing resourceType set to RESOURCE_TYPE and the IUT not having privileges to perform RETRIEVE operation on the MEMBER_RESOURCE_ADDRESS_1 } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberType attribute set to RESOURCE_TYPE and  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the group resource  | |
                     | and the IUT sends a valid valid Response containing  | |
                     | Response Status Code set to 5105  | |
                     |(RECEIVER_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/GMG/CRE/003

| TP Id | TP/oneM2M/CSE/GMG/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT detects the presence of duplicate member IDs during the creation of the <group> resource and removes the duplicate member IDs prior to creation of the <group> resource |  |
| Reference | TS-0001 [1], clause 10.2.7.2 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having created a resource at MEMBER_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives an valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS,  | |
                     | MEMBER_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT creates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS  | |
                     |} | |

| Note: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/GMG/CRE/004

| TP Id | TP/oneM2M/CSE/GMG/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT validates the resource type during the creation of the <group> resource when memberType attribute is not 'mixed' |  |
| Reference | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE } |  |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to MEMBER_RESOURCE_ADDRESS  | |
                     | and  | |
                     | memberType attribute set to RESOURCE_TYPE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT creates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to TRUE  | |
                     |} | |

| Note: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/GMG/CRE/005

| TP Id | TP/oneM2M/CSE/GMG/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is SET_MIXED |  |
| Reference | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS having resourceType attribute RESOURCE_TYPE_1 } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS  | |
                     | and memberType attribute set to RESOURCE_TYPE_2  | |
                     | and consistencyStrategy attribute set to SET_MIXED  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT creates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to TRUE and  | |
                     | memberType attribute set to MIXED  | |
                     |} | |

| Note: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/GMG/CRE/006

| TP Id | TP/oneM2M/CSE/GMG/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles unsuccessful validation of the resource type during the creation of the <group> resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_MEMBER |  |
| Reference | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type group and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing   | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2 and  | |
                     | memberType attribute set to RESOURCE_TYPE_1 and | |
                     | consistencyStrategy attribute set to ABANDON_MEMBER  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT creates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to TRUE and  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1  | |
                     |} | |

| Note: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/GMG/CRE/007

| TP Id | TP/oneM2M/CSE/GMG/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles unsuccessful validation of the resource type during the creation of the group resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_GROUP |  |
| Reference | TS-0001 [1], clause 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS allowing a child resource type <group> and the AE having privileges to perform CREATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2 having resourceType attribute RESOURCE_TYPE_2 } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2 and  | |
                     | memberType attribute set to RESOURCE_TYPE_1 and | |
                     | consistencyStrategy attribute set to ABANDON_GROUP  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not create the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4110  | |
                     |(GROUP_MEMBER_TYPE_INCONSISTENT)  | |
                     |} | |

| Note: TARGET_RESOURCE_ADDRESS : <CSEBase>, <remoteCSE> MEMBER_RESOURCE_ADDRESS : any oneM2M resource type |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/GMG/CRE/008

| TP Id | TP/oneM2M/CSE/GMG/CRE/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates a <group> resource with empty memberIDs attribute. |  |
| Reference | TS-0001 [1], clause 9.6.13, 10.2.7.2, TS-0004 [2], clause 7.4.14.2.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation } |  |

- +---------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 9 (group) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to empty  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | group resource representation  | |
                     |} | |
+———————-+—————————————————————————————————+————-+

##### 7.2.2.5.2	UPDATE Operation

###### TP/oneM2M/CSE/GMG/UPD/001

| TP Id | TP/oneM2M/CSE/GMG/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update of the group resource when the memberType cannot be retrieved due to lack of privilege. |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.3.13 |  |
| Parent Release | Release 1 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and IUT registered with remote CSE CSE1 and the IUThaving a resource at MEMBER_RESOURCE_ADDRESS_1 containing resourceType set to RESOURCE_TYPE and the IUT having created a <group> resource at GROUP_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE and the AE having privileges to perform UPDATE operation on the GROUP_RESOURCE_ADDRESS and the AEhaving created a resource at MEMBER_RESOURCE_ADDRESS_2 at another CSE1 and the IUT not having privileges to perform RETRIEVE operation on MEMBER_RESOURCE_ADDRESS_2 } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid valid Response containing  | |
                     | Response Status Code set to 5105  | |
                     |(RECEIVER_HAS _NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/GMG/UPD/002

| TP Id | TP/oneM2M/CSE/GMG/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT detects the presence of duplicate member IDs during an update of the <group> resource and removes the duplicate member IDs prior to updating the <group> resource |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 and the IUT having created a <group> resource at GROUP_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and the AE having privileges to perform UPDATE operation on the GROUP_RESOURCE_ADDRESS and the IUT having created a resource at MEMBER_RESOURCE_ADDRESS_2 } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to GROUP_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_2,  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT updates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/GMG/UPD/003

| TP Id | TP/oneM2M/CSE/GMG/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT validates the resource type during an UPDATE of the <group> resource when memberType attribute is not 'mixed'. |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 memberType attribute set to RESOURCE_TYPE_1 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2having resourceType attribute RESOURCE_TYPE_1 } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT updates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to TRUE  | |
                     |} | |
+———————-+——————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/GMG/UPD/004

| TP Id | TP/oneM2M/CSE/GMG/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the < group_>_ resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is SET_MIXED |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to SET_MIXED and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2* *having resourceType attribute RESOURCE_TYPE_2 }** |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT updates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to TRUE and  | |
                     | memberType attribute set to MIXED  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/GMG/UPD/005

| TP Id | TP/oneM2M/CSE/GMG/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles unsuccessful validation of the resource type during an UPDATE of the group resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_MEMBER |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2having resourceType attribute RESOURCE_TYPE_2 } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT updates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to TRUE and  | |
                     | memberIDs attribute set to | |
                     | MEMBER_RESOURCE_ADDRESS_1  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/GMG/UPD/006

| TP Id | TP/oneM2M/CSE/GMG/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles unsuccessful validation of the resource type during the update of the group resource when memberType attribute is not 'mixed' and the consistencyStrategy attribute is ABANDON_GROUP |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_GROUP and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2having resourceType attribute RESOURCE_TYPE_2 } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT does not update the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4110  | |
                     |(GROUP_MEMBER_TYPE_INCONSISTENT)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/GMG/UPD/007

| TP Id | TP/oneM2M/CSE/GMG/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT detects when the number of memberIDs exceeds the limitation of maxNrOfMembers |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and maxNrOfMembers attribute set to 2 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_3having resourceType attributeset to RESOURCE_TYPE_1 } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2,  | |
                     | MEMBER_RESOURCE_ADDRESS_3  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not update the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 6010  | |
                     |(MAX_NUMBER_OF_MEMBER_EXCEEDED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/GMG/UPD/008

| TP Id | TP/oneM2M/CSE/GMG/UPD/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT detects when the value provided for maxNrOfMembers attribute is less than the value of the currentNrOfMembers attribute |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and maxNrOfMembers attribute set to 2 and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1having resourceType attribute set to RESOURCE_TYPE_1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_2having resourceType attribute set to RESOURCE_TYPE_1 } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing group resource containing  | |
                     | maxNrOfMembers attribute set to 1  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT does not update the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 6010  | |
                     |(MAX_NUMBER_OF_MEMBER_EXCEEDED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/GMG/UPD/009

| TP Id | TP/oneM2M/CSE/GMG/UPD/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles validation of the resource type during an UPDATE of the <group> resource when memberIDs contains a sub-group on a temporarily unreachable Hosting CSE |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to TRUE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the remoteCSE having a group resource at MEMBER_RESOURCE_ADDRESS_2 containing memberType attribute set to RESOURCE_TYPE_1 and the remoteCSE having onlineStatus set to False } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT updates the group resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to FALSE and  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/GMG/UPD/010

| TP Id | TP/oneM2M/CSE/GMG/UPD/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT handles validation of the resource type during an UPDATE of the group resource when memberIDs contains a sub-group on a a previously unreachable Hosting CSE that has become reachable |  |
| Reference | TS-0001 [1], clause 10.2.7.4, TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the remoteCSE and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing memberIDs attribute set to MEMBER_RESOURCE_ADDRESS_1, MEMBER_RESOURCE_ADDRESS_2 and memberType attribute set to RESOURCE_TYPE_1 and memberTypeValidated attribute set to FALSE and consistencyStrategy attribute set to ABANDON_MEMBER and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the AE having a subscription to TARGET_RESOURCE_ADDRESS and the IUT having a resource at MEMBER_RESOURCE_ADDRESS_1 having resourceType attribute set to RESOURCE_TYPE_1 and the remoteCSE having a group resource at MEMBER_RESOURCE_ADDRESS_2 containing memberType attribute set to RESOURCE_TYPE_2 and the remoteCSE having onlineStatus set to False } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid UPDATE Request from remoteCSE containing  | |
                     | To set to REMOTE_CSE_ADDRESS and  | |
                     | From set to CSE-ID and  | |
                     | Content containing  | |
                     | remoteCSE resource containing  | |
                     | onlineStatus attribute set to TRUE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a NOTIFY Request containing  | |
                     | Content containing  | |
                     | group resource containing  | |
                     | memberTypeValidated attribute set to TRUE and  | |
                     | memberIDs attribute set to  | |
                     | MEMBER_RESOURCE_ADDRESS_1,  | |
                     | MEMBER_RESOURCE_ADDRESS_2  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————+————-+

##### 7.2.2.5.3	RETRIEVE Operation

###### TP/oneM2M/CSE/GMG/RET/001

| TP Id | TP/oneM2M/CSE/GMG/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT performs a RETRIEVE request for each resource in memberIDs with a relative address appended to fanOutPoint that includes a virtual resource |  |
| Reference | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform RETRIEVE operation and memberType attribute set to 3 (container) and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType attribute set to container and and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType attribute set to container and and MEMBER_RESOURCE_ADDRESS1 and MEMBER_RESOURCE_ADDRESS2 containing a child resource containing resourceType attribute set to contentInstance } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt/la and  | |
                     | From set to AE-ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | TaggregatedResponse message containing  | |
                     | Response for MEMBER_RESOURCE_ADDRESS1/la,  | |
                     | Response for MEMBER_RESOURCE_ADDRESS2/la  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

##### 7.2.2.5.4	BASIC OPERATION

###### TP/oneM2M/CSE/GMG/001

| TP Id | TP/oneM2M/CSE/GMG/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT allows a <group>/fopt OPERATION when the Originator has OPERATION permission specified in membersAccessControlPolicyIDs attribute in the group resource |  |
| Reference | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | From set to AE-ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing  | |
                     | Response for MEMBER_RESOURCE_ADDRESS1,  | |
                     | Response for MEMBER_RESOURCE_ADDRESS2  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/001_CRE | CREATE |
| TP/oneM2M/CSE/GMG/001_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/001_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/001_DEL | DELETE |

###### TP/oneM2M/CSE/GMG/002

| TP Id | TP/oneM2M/CSE/GMG/002 |  |
| --- | --- | --- |
| Test objective | Check that IUT denies a <group>/fopt OPERATION when the Originator does not have OPERATION permission specified in membersAccessControlPolicyIDs in the group resource |  |
| Reference | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | From set to AE-ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103  | |
                     | (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/002_CRE | CREATE |
| TP/oneM2M/CSE/GMG/002_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/002_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/002_DEL | DELETE |

###### TP/oneM2M/CSE/GMG/003

| TP Id | TP/oneM2M/CSE/GMG/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT allows a <group>/fopt OPERATION when the Originator has OPERATION permission specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the <group> resource |  |
| Reference | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to empty and accessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | From set to AE-ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set 2000(OK)and  | |
                     | Content containing  | |
                     | aggregatedResponse message containing  | |
                     | Response for MEMBER_RESOURCE_ADDRESS1,  | |
                     | Response for MEMBER_RESOURCE_ADDRESS2  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/003_CRE | CREATE |
| TP/oneM2M/CSE/GMG/003_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/003_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/003_DEL | DELETE |

###### TP/oneM2M/CSE/GMG/004

| TP Id | TP/oneM2M/CSE/GMG/004 |  |
| --- | --- | --- |
| Test objective | Check that IUT rejects a <group>/fopt OPERATION when the Originator does not have OPERATION permission specified in accessControlPolicyIDs and the membersAccessControlPolicyIDs is empty in the group resource |  |
| Reference | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to empty and accessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE privileges to perform all operations except OPERATION and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | From set to AE-ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103  | |
                     | (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/004_CRE | CREATE |
| TP/oneM2M/CSE/GMG/004_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/004_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/004_DEL | DELETE |

###### TP/oneM2M/CSE/GMG/005

| TP Id | TP/oneM2M/CSE/GMG/005 |  |
| --- | --- | --- |
| Test objective | Check that IUT performs an OPERATION request for each resource in memberIDs with no relative address appended to it |  |
| Reference | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE privileges to perform OPERATION and memberType attribute set to AE AE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceTypeset to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceTypeset to AE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | From set to AE-ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse message containing  | |
                     | Response for MEMBER_RESOURCE_ADDRESS1 and  | |
                     | Response for MEMBER_RESOURCE_ADDRESS2  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/005_CRE | CREATE |
| TP/oneM2M/CSE/GMG/005_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/005_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/005_DEL | DELETE |

###### TP/oneM2M/CSE/GMG/006

| TP Id | TP/oneM2M/CSE/GMG/006 |  |
| --- | --- | --- |
| Test objective | Check that IUT perfomrs an OPERATION request for each resource in memberIDs with a relative address appended to it |  |
| Reference | TS-0001 [1], clause 10.2.7.7 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE and memberType attribute set to AE and memberIDs attribute set to MEMBER_RESOURCE_ADDRESS1, MEMBER_RESOURCE_ADDRESS2 and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing resourceType attribute set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing resourceType attribute set to AE and the IUT having a resource at MEMBER_RESOURCE_ADDRESS1 containing a child resource containing resourceType attribute set to container and resourceName attribute set to NAME and the IUT having a resource at MEMBER_RESOURCE_ADDRESS2 containing a child resource containing resourceType attribute set to container and resourceName attribute set to NAME } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt/NAME and  | |
                     | From set to AE-ID  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse message containing  | |
                     | Response for MEMBER_RESOURCE_ADDRESS1 and  | |
                     | Response for MEMBER_RESOURCE_ADDRESS2  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/GMG/006_CRE | CREATE |
| TP/oneM2M/CSE/GMG/006_RET | RETRIEVE |
| TP/oneM2M/CSE/GMG/006_UPD | UPDATE |
| TP/oneM2M/CSE/GMG/006_DEL | DELETE |

###### TP/oneM2M/CSE/GMG/007

| TP Id | TP/oneM2M/CSE/GMG/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT denies a <group>/fanOutPoint RETRIEVE when there is no memberID in <group> resource |  |
| Reference | TS-0004 [2], clause 7.4.14.2.4 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a group resource at TARGET_RESOURCE_ADDRESS containing membersAccessControlPolicyIDs attribute set to ACP_RESOURCE_ID indicating to allow the AE to perform all operations and no memberID attribute } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/fopt and  | |
                     | From set to AE-ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4109 (NO_MEMBER)  | |
                     |} | |
+———————-+———————————————————————————————————————————————+————-+

#### 7.2.2.6	Discovery (DIS)

##### 7.2.2.6.1	RETRIEVE Operation

###### TP/oneM2M/CSE/DIS/001

| TP Id | TP/oneM2M/CSE/DIS/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully a list all discovered resource addresses |  |
| Reference | TS-0001 [1], clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |

- +------------------------------------------------------------------------------+----------------------+

- |when {  | |
                     | the IUT receives a valid RETRIEVE Request from AE containing | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +------------------------------------------------------------------------------+----------------------+

- |then {  | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList representation  | |
                     |} | |
+———————-+——————————————————————————+———————-+

###### TP/oneM2M/CSE/DIS/002

| TP Id | TP/oneM2M/CSE/DIS/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully appropriate list of discovered resource when the filter criteria is provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation on the child resource } |  |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList representation  | |
                     |} | |
+———————-+———————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DIS/003

| TP Id | TP/oneM2M/CSE/DIS/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty address list when no result matching with filter criteria is discovered |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation on the child resource and FILTER_CRITERIA_CONDITIONS not matching child resource } |  |

- +-----------------------------------------------------------------------------------------------+----------------------+

- |when {  | |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------+----------------------+

- |then {   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList element set to empty list  | |
                     |} | |
+———————-+———————————————————————————————–+———————-+

###### TP/oneM2M/CSE/DIS/004

| TP Id | TP/oneM2M/CSE/DIS/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully a list of discovered resource addresses with Non- hierarchical addressing form when the Desired Identifier Result Type is provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   | |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Desired Identifier Result Type set to 2 (unstructured) and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList representation containing  | |
                     | unstructured addresses of discovered resources  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————+———————-+

###### TP/oneM2M/CSE/DIS/005

| TP Id | TP/oneM2M/CSE/DIS/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a discovery requests to the resource TARGET_RESOURCE_ADDRESS when AE has no privilege to perform the discovery operation on the children/descendant of resource TARGET_RESOURCE_ADDRESS |  |
| Reference | TS-0001 [1], clause 10.2.6.2 and clause 7.2.3.14, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing child resources and the AE having no privileges to perform DISCOVERY operation on the children/ descendant of resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUTreceives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList element set to empty list  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/DIS/006

| TP Id | TP/oneM2M/CSE/DIS/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE sends requests to discover the resource TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference | TS-0001 [1], clause 10.2.6.2, TS-0004 [2], clause 7.3.3.14 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   | |
                     | the IUTreceives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+———————-+

###### TP/oneM2M/CSE/DIS/007

| TP Id | TP/oneM2M/CSE/DIS/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds the originator with an error when the originator sends a request including an invalid format of filter criteria to discover the resource TARGET_RESOURCE_ADDRESS |  |
| Reference | TS-0001 [1], clause 10.2.6, TS-0004 [2], clause 7.3.3.6 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS not being  | |
                     |valid format  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————+———————-+

###### TP/oneM2M/CSE/DIS/008

| TP Id | TP/oneM2M/CSE/DIS/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds the originator with a success response when the originator sends a request to discover the resource TARGET_RESOURCE_ADDRESS including two conflicted filter criteria of different type FILTER_CRITERIA_CONDITION_1 and FILTER_CRITERIA_CONDITION_ 2 (E.G. createBefore < createdAfter ) |  |
| Reference | TS-0001 [1], clause 6.2.5, TS-0004 [2], clause 7.3.3.17 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation on the child resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 1 (Discovery Criteria)and  | |
                     | having conflict conditions   | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList element set to empty list  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | REQ Reference | FILTER_CRITERIA_CONDITION_1 | FILTER_CRITERIA_CONDITION_2 |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/DIS/008_CRB/CRA | REQ-0004-07241 | createdBefore | createdAfter |
| TP/oneM2M/CSE/DIS/008_MS/US | REQ-0004-07242 | unmodifiedSince | modifiedSince |
| TP/oneM2M/CSE/DIS/008_STS/STB | REQ-0004-07243 | stateTagSmaller | stateTagBigger |
| TP/oneM2M/CSE/DIS/008_EXB/EXA | REQ-0004-07244 | expireBefore | expireAfter |

###### TP/oneM2M/CSE/DIS/009

| TP Id | TP/oneM2M/CSE/DIS/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty address list when resources match the filter criteria but they do not include DISCOVERY permission |  |
| Reference | TS-0001 [1], clause 9.6.2.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE not having privileges to perform DISCOVERY operation on the child resource and FILTER_CRITERIA_CONDITIONS matching the child resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList element set to empty list  | |
                     |} | |
+———————-+————————————————————————————————————————–+————-+

TP/oneM2M/CSE/DIS/010

| TP Id | TP/oneM2M/CSE/DIS/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully a list of discovered resource addresses with hierarchical addressing form when the Desired Identifier Result Type is provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY operation } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   |  |
                     | the IUT receives a valid RETRIEVE Request from AE containing  |  |
                     | To set to TARGET_RESOURCE_ADDRESS and  |  |
                     | From set to AE-ID and  |  |
                     | Desired Identifier Result Type set to 1 (structured) and  |IUT <- AE |
                     | Filter Criteria containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {  |  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2000 (OK) and  |  |
                     | Content containing  |IUT -> AE |
                     | URIList representation containing  | |
                     | structured addresses of discovered resources  | |
                     |} | |
+———————-+————————————————————————————————————————————————————–+———————-+

TP/oneM2M/CSE/DIS/011

| TP Id | TP/oneM2M/CSE/DIS/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully a list of discovered resource addresses with hierarchical addressing form when the Desired Identifier Result Type is not provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resourceand the AE having privileges to perform DISCOVERY operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   |  |
                     | the IUT receives a valid RETRIEVE Request from AE containing  |  |
                     | To set to TARGET_RESOURCE_ADDRESS and  |  |
                     | From set to AE-ID and  |  |
                     | no Desired Identifier Result Type parameter and  |IUT <- AE |
                     | Filter Criteria containing  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2000 (OK) and  |  |
                     | Contentcontaining  |IUT -> AE |
                     | URIList representation containing  | |
                     | structured addresses of discovered resources  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————+———————-+

TP/oneM2M/CSE/DIS/012

| TP Id | TP/oneM2M/CSE/DIS/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs a Discovery-based OPERATION |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS_1 and a child resource at CONTAINER_RESOURCE_ADDRESS_2 and the AE having privileges to perform DISCOVERY and OPERATION on children resources } |  |

- +-------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------+----------------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_1,  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_2  | |
                     |} | |
+———————-+————————————————————————————————-+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/012_CRE | CREATE |
| TP/oneM2M/CSE/DIS/012_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/012_DEL | DELETE |

TP/oneM2M/CSE/DIS/013

| TP Id | TP/oneM2M/CSE/DIS/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs a Discovery-based OPERATION when the filter criteria is provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS_1 and a child resource at CONTAINER_RESOURCE_ADDRESS_2 and the AE having privileges to perform DISCOVERY and OPERATION on children resources and FILTER_CRITERIA_CONDITIONS matching child resource at CONTAINER_RESOURCE_ADDRESS_1 } |  |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     |containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_1  | |
                     |} | |
+———————-+————————————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/013_CRE | CREATE |
| TP/oneM2M/CSE/DIS/013_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/013_DEL | DELETE |

TP/oneM2M/CSE/DIS/014

| TP Id | TP/oneM2M/CSE/DIS/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty content when no result matching with filter criteria is discovered in a Discovery-based OPERATION |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY and OPERATION on child resource and FILTER_CRITERIA_CONDITIONS not matching child resource } |  |

- +------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |} | |
+———————-+——————————————————————————————————————+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/014_CRE | CREATE |
| TP/oneM2M/CSE/DIS/014_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/014_DEL | DELETE |

TP/oneM2M/CSE/DIS/015

| TP Id | TP/oneM2M/CSE/DIS/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs a Discovery-based OPERATION using Non- hierarchical addressing form when the Result Content and Desired Identifier Result Type is provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Result Content set to 11 (discovery result resource references) and  | |
                     | Desired Identifier Result Typeset to 2 (unstructured) and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | URIList containing  | |
                     | unstructured addresses of discovered resources  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————–+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/015_CRE | CREATE |
| TP/oneM2M/CSE/DIS/015_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/015_DEL | DELETE |

TP/oneM2M/CSE/DIS/016

| TP Id | TP/oneM2M/CSE/DIS/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a Discovery-based OPERATION request to the resource TARGET_RESOURCE_ADDRESS when AE has no privilege to perform the discovery operation |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing child resources and the AE having no privileges to perform DISCOVERY and OPERATION on children resources } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUTreceives a valid Discovery-based OPERATION Request from  | |
                     | AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |  | |
                     |} | |
+———————-+———————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/016_CRE | CREATE |
| TP/oneM2M/CSE/DIS/016_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/016_DEL | DELETE |

TP/oneM2M/CSE/DIS/017

| TP Id | TP/oneM2M/CSE/DIS/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a Discovery-based OPERATION request to the resource TARGET_RESOURCE_ADDRESS when AE has privilege to perform the discovery operation but AE has no privilege to perform the OPERATION request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS_1 and a child resource at CONTAINER_RESOURCE_ADDRESS_2 and the AE having privileges to perform DISCOVERY on children resources and the AE having no privileges to perform OPERATION on children resources } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUTreceives a valid Discovery-based OPERATION Request from  | |
                     | AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT does not perform OPERATION  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK)  | |
                     | aggregatedResponse containing  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_1  | |
                     |containing  | |
                     | Response Status Code set to 4103(ACCESS_DENIED)  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_2  | |
                     |containing  | |
                     | Response Status Code set to 4103(ACCESS_DENIED)  | |
                     |  | |
                     |  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/017_CRE | CREATE |
| TP/oneM2M/CSE/DIS/017_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/017_DEL | DELETE |

TP/oneM2M/CSE/DIS/018

| TP Id | TP/oneM2M/CSE/DIS/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE sends Discovery-based OPERATION requests to the resource TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   |IUT <- AE |
                     | the IUTreceives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4(Discovery-based Operation)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/018_CRE | CREATE |
| TP/oneM2M/CSE/DIS/018_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/018_DEL | DELETE |

TP/oneM2M/CSE/DIS/019

| TP Id | TP/oneM2M/CSE/DIS/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds to the originator with an error when the originator sends a Discovery-based OPERATION request including an invalid format of filter criteria to discover the resource TARGET_RESOURCE_ADDRESS |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from  | |
                     | AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS not being valid  | |
                     | format  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————-+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/019_CRE | CREATE |
| TP/oneM2M/CSE/DIS/019_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/019_DEL | DELETE |

TP/oneM2M/CSE/DIS/020

| TP Id | TP/oneM2M/CSE/DIS/020 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds to the originator with a success response when the originator sends a Discovery-based OPERATION including two conflicted filter criteria of different type FILTER_CRITERIA_CONDITION_1 and FILTER_CRITERIA_CONDITION_ 2 (E.G. createBefore < createdAfter ) |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 and clause 7.3.3.17 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUTreceives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_IDand  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation) and  | |
                     | having conflict conditions   | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | COLSPAN | FILTER_CRITERIA_CONDITION_1 | FILTER_CRITERIA_CONDITION_2 | OPERATION |
| --- | --- | --- | --- | --- |
| TP/oneM2M/CSE/DIS/020_CRB/CRA/CRE | COLSPAN | createdBefore | createdAfter | CREATE |
| TP/oneM2M/CSE/DIS/020_MS/US/CRE | COLSPAN | unmodifiedSince | modifiedSince | CREATE |
| TP/oneM2M/CSE/DIS/020_STS/STB/CRE | COLSPAN | stateTagSmaller | stateTagBigger | CREATE |
| TP/oneM2M/CSE/DIS/020_EXB/EXA/CRE | COLSPAN | expireBefore | expireAfter | CREATE |
| TP/oneM2M/CSE/DIS/020_CRB/CRA/UPD | COLSPAN | createdBefore | createdAfter | UPDATE |
| TP/oneM2M/CSE/DIS/020_MS/US/UPD | COLSPAN | unmodifiedSince | modifiedSince | UPDATE |
| TP/oneM2M/CSE/DIS/020_STS/STB/UPD | COLSPAN | stateTagSmaller | stateTagBigger | UPDATE |
| TP/oneM2M/CSE/DIS/020_EXB/EXA/UPD | COLSPAN | expireBefore | expireAfter | UPDATE |
| TP/oneM2M/CSE/DIS/020_CRB/CRA/DEL | createdBefore | createdAfter | DELETE |  |
| TP/oneM2M/CSE/DIS/020_MS/US/DEL | unmodifiedSince | modifiedSince | DELETE |  |
| TP/oneM2M/CSE/DIS/020_STS/STB/DEL | stateTagSmaller | stateTagBigger | DELETE |  |
| TP/oneM2M/CSE/DIS/020_EXB/EXA/DEL | expireBefore | expireAfter | DELETE |  |

TP/oneM2M/CSE/DIS/021

| TP Id | TP/oneM2M/CSE/DIS/021 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty content when resources match the filter criteria but they do not include Discovery-based OPERATION permission. |  |
| Reference | TS-0001 [1], clause 9.6.2.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE not having privileges to perform DISCOVERY on child resource and the AE having privileges to perform OPERATION on child resource and FILTER_CRITERIA_CONDITIONS matching the child resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |  | |
                     |} | |
+———————-+—————————————————————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/021_CRE | CREATE |
| TP/oneM2M/CSE/DIS/021_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/021_DEL | DELETE |

TP/oneM2M/CSE/DIS/022

| TP Id | TP/oneM2M/CSE/DIS/022 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs a Discovery-based OPERATION using hierarchical addressing form when the Result Content and Desired Identifier Result Type is provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   |  |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  |  |
                     | To set to TARGET_RESOURCE_ADDRESS and  |  |
                     | From set to AE-ID and  |IUT <- AE |
                     | Result Content set to 11 (discovery result resource references) and  | |
                     | Desired Identifier Result Type set to 1 (structured) and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2000(OK) and  |  |
                     | Content containing  |IUT -> AE |
                     | URIList containing  | |
                     | structured addresses of discovered resources  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————-+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/022_CRE | CREATE |
| TP/oneM2M/CSE/DIS/022_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/022_DEL | DELETE |

TP/oneM2M/CSE/DIS/023

| TP Id | TP/oneM2M/CSE/DIS/023 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs a Discovery-based OPERATION using hierarchical addressing form when the Desired Identifier Result Type is not provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6, TS-0004 [2], clause 7.2.3.13 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {   |  |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  |  |
                     | To set to TARGET_RESOURCE_ADDRESS and  |  |
                     | From set to AE-ID and  |IUT <- AE |
                     | Result Content set to 11 (discovery result resource references) and  | |
                     | no Desired Identifier Result Type parameter and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |  |
                     | the IUT sends a valid Response containing  |  |
                     | Response Status Code set to 2000(OK) and  |  |
                     | Content containing  |IUT -> AE |
                     | URIList containing  | |
                     | structured addresses of discovered resources  | |
                     |} | |
+———————-+———————————————————————————————————————————————————-+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/023_CRE | CREATE |
| TP/oneM2M/CSE/DIS/023_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/023_DEL | DELETE |

TP/oneM2M/CSE/DIS/024

| TP Id | TP/oneM2M/CSE/DIS/024 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds to the originator with an error when resources match the filter criteria in a Discovery-based OPERATION request but including an invalid Content |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform DISCOVERY and OPERATION on child resource and FILTER_CRITERIA_CONDITIONS matching the child resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from  | |
                     | AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content not being valid and  | |
                     | Filter Criteria set to FILTER_CRITERIA_CONDITIONS containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK)  | |
                     | aggregatedResponse containing  | |
                     | Response for child resource containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/024_CRE | CREATE |
| TP/oneM2M/CSE/DIS/024_UPD | UPDATE |

TP/oneM2M/CSE/DIS/025

| TP Id | TP/oneM2M/CSE/DIS/025 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs a Discovery-based OPERATION when resources match the filter criteria containing geoQuery condition provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS_1 containing location attribute not set to LOCATIONand a child resource at CONTAINER_RESOURCE_ADDRESS_2 containing location attribute set to LOCATION and the AE having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteriacontaining  | |
                     | geoQuery set to LOCATION and  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_2  | |
                     |} | |
+———————-+——————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/025_CRE | CREATE |
| TP/oneM2M/CSE/DIS/025_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/025_DEL | DELETE |

TP/oneM2M/CSE/DIS/026

| TP Id | TP/oneM2M/CSE/DIS/026 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty content when no result matching with filter criteria containing geoQuery condition provided in a Discovery-based OPERATION |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource containing location attribute set to LOCATION and the AE having privileges to perform DISCOVERY and OPERATION on child resource and FILTER_CRITERIA_CONDITIONS not matching child resource location attribute } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteriaset to FILTER_CRITERIA_CONDITIONS containing  | |
                     | geoQuery not matching LOCATION and  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |} | |
+———————-+——————————————————————————————————————————————-+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/026_CRE | CREATE |
| TP/oneM2M/CSE/DIS/026_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/026_DEL | DELETE |

TP/oneM2M/CSE/DIS/027

| TP Id | TP/oneM2M/CSE/DIS/027 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds to the originator with an error when the originator sends a Discovery-based OPERATION request including an invalid format of filter criteria containing geoQuery condition to discover the resource TARGET_RESOURCE_ADDRESS |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having a container resource TARGET_RESOURCE_ADDRESS containing a child resource containing location attribute set to LOCATION and the AE having privileges to perform DISCOVERY and_OPERATION_ on child resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | geoQuery not being valid format  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————-+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/027_CRE | CREATE |
| TP/oneM2M/CSE/DIS/027_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/027_DEL | DELETE |

TP/oneM2M/CSE/DIS/028

| TP Id | TP/oneM2M/CSE/DIS/028 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty content when the originator sends a Discovery-based OPERATION request including filter criteria containing geQuery condition but resources do not have location attribute |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource containing no location attribute and the AE having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteriacontaining  | |
                     | geoQuery set to LOCATION and  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————+———————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/028_CRE | CREATE |
| TP/oneM2M/CSE/DIS/028_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/028_DEL | DELETE |

TP/oneM2M/CSE/DIS/029

| TP Id | TP/oneM2M/CSE/DIS/029 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts Discovery-based OPERATION requests to the resource TARGET_RESOURCE_ADDRESS when AE has privilege to perform the discovery operation on the children/descendant but AE has no privilege to perform discovery operation of parent resource TARGET_RESOURCE_ADDRESS |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS_1 and a child resource at CONTAINER_RESOURCE_ADDRESS_2 and the AE having privileges to perform DISCOVERY and OPERATION on children resources and the AE having no privileges to perform DISCOVERY on parent resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     | | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_1,  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_2  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/029_CRE | CREATE |
| TP/oneM2M/CSE/DIS/029_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/029_DEL | DELETE |

TP/oneM2M/CSE/DIS/030

| TP Id | TP/oneM2M/CSE/DIS/030 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty content in a Discovery-based OPERATION when resource has status marked as INACTIVE |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 1 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and registrationStatus set t o 2 (INACTIVE) and the AE2 having a resource TARGET_RESOURCE_ADDRESS containing a child resource and the AE1 having privileges to perform DISCOVERY and OPERATION on child resource } |  |

- +------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <- AE |
                     | the IUTreceives a valid OPERATION Request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID and  | |
                     | Filter Criteria containing  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |  | |
                     |} | |
+———————-+————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/030_CRE | CREATE |
| TP/oneM2M/CSE/DIS/030_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/030_DEL | DELETE |

###### TP/oneM2M/CSE/DIS/031

| TP Id | TP/oneM2M/CSE/DIS/031 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully appropriate list of discovered resources when resources match the filter criteria containing geoQuery condition provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS containing location attribute set to LOCATIONand and the AE having privileges to perform DISCOVERY on child resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based RETRIEVE Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteriacontaining  | |
                     | geoQuery set to LOCATION and  | |
                     | filterUsageset to 1 (Discovery Criteria)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList representation  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/DIS/032

| TP Id | TP/oneM2M/CSE/DIS/032 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty content when no result matching with filter criteria containing geoQuery condition provided in the request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS containing location attribute set to LOCATION1 and and the AE having privileges to perform DISCOVERY on child resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based RETRIEVE Request from AE  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteriacontaining  | |
                     | geoQuery set to LOCATION2 and  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList element set to empty list  | |
                     |} | |
+———————-+———————————————————————————————————————————-+————-+

TP/oneM2M/CSE/DIS/033

| TP Id | TP/oneM2M/CSE/DIS/033 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty address list when resources match the geoQuery filter criteria but they do not include DISCOVERY permission |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS containing location attribute set to LOCATIONand and the AE not h aving privileges to perform DISCOVERY on child resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteriacontaining  | |
                     | geoQuery set to LOCATION and  | |
                     | filterUsage set to 1 (Discovery Criteria)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | URIList element set to empty list  | |
                     |} | |
+———————-+———————————————————————————————————————————–+————-+

TP/oneM2M/CSE/DIS/034

| TP Id | TP/oneM2M/CSE/DIS/034 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a Discovery-based OPERATION request containing geoQuery filter criteria when AE has privilege to perform the discovery operation but AE has no privilege to perform the OPERATION request |  |
| Reference | TS-0001 [1], clause 8.1.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 and clause 6.6.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS_1 containing location attribute set to LOCATION a child resource at CONTAINER_RESOURCE_ADDRESS_2 containing location attribute set to LOCATION and the AE having privileges to perform DISCOVERY on children resources and the AE not h aving privileges to perform OPERATION on children resources } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from  | |
                     | AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria containing  | |
                     | geoQuery set to LOCATION  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT -> AE |
                     | the IUT does not perform OPERATION  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK)  | |
                     | aggregatedResponse containing  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_1  | |
                     | containing  | |
                     | Response Status Code set to 4103 (ACCESS_DENIED)  | |
                     | Response for CONTAINER_RESOURCE_ADDRESS_2  | |
                     | containing  | |
                     | Response Status Code set to 4103 (ACCESS_DENIED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/034_CRE | CREATE |
| TP/oneM2M/CSE/DIS/034_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/034_DEL | DELETE |

TP/oneM2M/CSE/DIS/035

| TP Id | TP/oneM2M/CSE/DIS/035 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the empty content when resources match the geoQuery filter criteria but they do not include Discovery-based OPERATION permission. |  |
| Reference | TS-0001 [1], clause 9.6.2.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing a child resource at CONTAINER_RESOURCE_ADDRESS_1 containing location attribute set to LOCATION and the AE not having privileges to perform DISCOVERY on child resource and the AE having privileges to perform OPERATION on child resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria containing  | |
                     | geoQuery set to LOCATION  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |  | |
                     |} | |
+———————-+————————————————————————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/035_CRE | CREATE |
| TP/oneM2M/CSE/DIS/035_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/035_DEL | DELETE |

TP/oneM2M/CSE/DIS/036

| TP Id | TP/oneM2M/CSE/DIS/036 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a Discovery-based OPERATION request to the resource TARGET_RESOURCE_ADDRESS when AE has no privilege to perform the discovery operation |  |
| Reference | TS-0001 [1], clause 9.6.2.3 and clause 10.2.6.4, TS-0004 [2], clause 7.3.3.14 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource TARGET_RESOURCE_ADDRESS containing child resources and the AE not having privileges to perform DISCOVERY on child resources and the AE not having privileges to perform OPERATION on child resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <- AE |
                     | the IUT receives a valid Discovery-based OPERATION Request from  | |
                     | AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria containing  | |
                     | geoQuery set to LOCATION  | |
                     | filterUsage set to 4 (Discovery-based Operation)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT -> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | aggregatedResponse containing empty list  | |
                     |  | |
                     |} | |
+———————-+———————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/DIS/036_CRE | CREATE |
| TP/oneM2M/CSE/DIS/036_UPD | UPDATE |
| TP/oneM2M/CSE/DIS/036_DEL | DELETE |

#### 7.2.2.7	Location (LOC)

##### 7.2.2.7.1	CREATE Operation

###### TP/oneM2M/CSE/LOC/CRE/001

| TP Id | TP/oneM2M/CSE/LOC/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the locationPolicy resource when locationSource attribute is set by Network Based and Location Server is not reachable |  |
| Reference | TS-0001 [1], clause 8.5 and clause 10.2.9, TS-0004 [2], clause 7.4.10 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUThaving no connection to Location Server and the IUT having a CSEBase resource allowing the AE to perform CREATE operation and } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid CREATE Request from AE containing To set to CSEBASE_RESOURCE_ADDRESS and From set to AE-ID and Content containing locationPolicy resource representation } | IUT <= AE |
|  | then { the IUT does not create the locationPolicy resource and the IUT sends a valid Response containing Response Status Code set to 4000 (BAD_REQUEST) } | IUT => AE |

###### TP/oneM2M/CSE/LOC/CRE/002

| TP Id | TP/oneM2M/CSE/LOC/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of locationPolicy resource with locationSource attribute and locationUpdatePeriod attribute provided |  |
| Reference | TS-0001 [1], clause 10.2.9.2, TS-0004 [2], clause 7.4.11.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 10 (locationPolicy) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |locationSource attributeset to 1 (Network_based) and  | |
                     |valid locationUpdatePeriod attribute and  | |
                     |valid locationTargetID attributeand  | |
                     |valid locationServer attribute   | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT c reates the locationPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+———————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/LOC/CRE/003

| TP Id | TP/oneM2M/CSE/LOC/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of locationPolicy resource when AE has no privilege to perform the creation request for the locationPolicy resource |  |
| Reference | TS-0001 [1], clause 10.2.9.2, TS-0004 [2], clause 7.4.10.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the AE having no privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     |the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 10 (locationPolicy) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |locationSource attribute set to 1 (Network_based) and  | |
                     |valid locationUpdatePeriod attribute and  | |
                     |valid locationTargetID attributeand  | |
                     |valid locationServer attribute  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the locationPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/LOC/CRE/004

| TP Id | TP/oneM2M/CSE/LOC/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when receiving a creation request of locationPolicy resource target to the TARGET_RESOURCE_ADDRESS which does not exist in the Hosting CSE |  |
| Reference | TS-0001 [1], clause 10.2.9.2, TS-0004 [2], clause 7.4.10.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the originator AE and the IUT having not yet created a resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     |the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 10 (locationPolicy) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |locationSource attributeset to 1 (Network_based) and  | |
                     |valid locationUpdatePeriod attribute and  | |
                     |valid locationTargetID attributeand  | |
                     |valid locationServer attribute  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the locationPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/LOC/CRE/005

| TP Id | TP/oneM2M/CSE/LOC/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates a container resource with CONTAINER_NAME included in the associated locationPolicy create request |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.10.2.1, TS-0004 [2], clause 7.3.3.3 and clause 7.4.11.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having no created container having resourceName set to CONTAINER_NAME and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | ResourceType set to 10 (locationPolicy) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | locationContainerName attribute set to CONTAINER_NAME  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT creates the container resource with Content containing  | |
                     | container resource representation containing  | |
                     | resourceName attribute set to CONTAINER_NAME and  | |
                     | locationID attribute set to resourceID of the associated locationPolicy resource  | |
                     |and the IUT c reates the locationPolicy resource with Content containing  | |
                     | locationPolicy resource representation containing  | |
                     | locationContainerName attribute set to CONTAINER_NAME  | |
                     | and locationContainerId attribute set to resourceID of the created container resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |

| Note that the IUT sets the locationContainerID attribute in locationPolicy resource to the resourceID of the created <container> resource |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/LOC/CRE/006

| TP Id | TP/oneM2M/CSE/LOC/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates a subscription resource with SUBSCRIPTION_NAME provided by the originator as a child resource of the container linked with a locationPolicy resource |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.9.2, TS-0004 [2], clause 7.3.3.3 and clause 7.4.10.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing resourceID set to NOTI_URI_ADDRESS and the IUThaving created a resource LOCATIONPOLICY_ADDRESS of type <locationPolicy > containing locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD and locationSource set to 1 (Network_based) and locationTargetID set to LOCATION_TARGET_ID and locationServer set to LOCATION_SERVER_ADDRESS and locationContainerID set to CONTAINER_ADDRESSand and the IUT having created a resource CONTAINER_ADDRESS of type <container > containing locationID set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | ResourceType set to 23 (subscription) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |notificationURI attribute set to NOTI_URI_ADDRESS and  | |
                     |eventNotificationCriteria attribute containing  | |
                     | notificationEventType element set to 3 (Create_of_Direct_Child_Resource)  | |
                     | and notificationContentType attribute set to 1 (All Attributes)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT creates the subscription resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED) and  | |
                     |Content containing  | |
                     | subscription resource representation  | |
                     |} | |

| Note that the IUT sets the locationContainerID attribute in <locationPolicy> resource to the resourceID of the created <container> resource |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/LOC/CRE/007

| TP Id | TP/oneM2M/CSE/LOC/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that IUT accepts the creation of a locationPolicy resource with attribute locationSource set to Device-based and locationUpdatePeriod set to POSITIVE_UPDATE_PERIOD |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.9.2, TS-0004 [2], clause 7.3.3.3 and clause 7.4.10.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having location positioning modules and the originator being a PX_ASN_AE and the IUT being a PX_ASN_CSE and the IUT having registered the PX_ASN_AE and the ASN-AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= ASN-AE|
                     |the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 10 (locationPolicy) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |locationSource attributeset to 2 (Device_based) and  | |
                     |locationUpdatePeriod attribute set to POSITIVE_UPDATE_PERIOD  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => ASN-AE|
                     | the IUT creates the locationPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | locationPolicy resource representation  | |
                     |} | |
+———————-+————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/LOC/CRE/008

| TP Id | TP/oneM2M/CSE/LOC/CRE/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a locationPolicy resource with attribute locationSource set to Sharing-based |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.9.2, TS-0004 [2], clause 7.3.3.3 and clause7.4.10.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the originator being a PX_ADN_AE and the IUT being a PX_MN_CSE and the ADN-AE being no location positioning modules deployed and the ADN-AE being no Network-based positioning capabilities and the IUT having registered the PX_ADN_AE and the ADN-AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= ADN-AE|
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 10 (locationPolicy) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |locationSource attributeset to 3 (Sharing_based)  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => ADN-AE|
                     | the IUT creates the locationPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | locationPolicy resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/LOC/CRE/009

| TP Id | TP/oneM2M/CSE/LOC/CRE/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the locationPolicy resource when locationSource attribute is set to Network-Based and neither locationTargetID nor locationServer is provided. |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.9.2, TS-0004 [2], clause 7.3.3.3 and clause7.4.10.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 10 (locationPolicy) and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     |locationSource attributeset to 1 (Network_based)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the locationPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————+————-+

##### 7.2.2.7.2	UPDATE Operation

###### TP/oneM2M/CSE/LOC/UPD/001

| TP Id | TP/oneM2M/CSE/LOC/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects AE to update the locationPolicy resource identified with LOCATION_ID when receiving an update request containing an invalid attribute |  |
| Reference | TS-0001 [1], clause 10.2.9.4, TS-0004 [2], clause 7.3.3.4 and clause 7.4.10.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type locationPolicy containing an optional RW attribute locationUpdatePeriod and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives an UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |Content containing  | |
                     |locationUpdatePeriod attribute set to INVALID_LOCATION_UPDATE_PERIOD  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not update the locationPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————-+————-+

##### 7.2.2.7.3	RETRIEVE Operation

###### TP/oneM2M/CSE/LOC/RET/001

| TP Id | TP/oneM2M/CSE/LOC/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with a container resource containing attribute locationID set to resourceID of the associated locationPolicy resource |  |
| Reference | TS-0001 [1], clause 10.2.9.3, TS-0004 [2], clause 7.4.10.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUThaving created a resource LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to TARGET_RESOURCE_ADDRESSand locationSource attribute set to 1 (Network_based) and locationTargetID attributeset to LOCATION_TARGET_ID and locationServer attribute set to LOCATION_SERVER_ADDRESS and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type container containing locationID attribute set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2000 (OK) and  | |
                     |Content containing  | |
                     | container resource containing  | |
                     | locationID attribute set to LOCATIONPOLICY_ADDRESS  | |
                     |} | |
+———————-+————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/LOC/RET/002

| TP Id | TP/oneM2M/CSE/LOC/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification message including the location Information to the originator when the originator sends a retrieve request target to the <latest> resource of CONTAINER_ADDRESS container associated with a <locationPolicy> resource |  |
| Reference | TS-0001 [1], clause 8.5.2,clause 10.2.9.3 and clause 10.2.10.2.2, TS-0004 [2], clause 7.4.10.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUThaving created a resource LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to CONTAINER_ADDRESSand locationUpdatePeriod attribute set to ZERO (or NULL) and locationSource attribute set to LOCATION_SOURCE and the IUT having created a resource CONTAINER_ADDRESS of type <container > containing locationID attribute set to LOCATIONPOLICY_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource CONTAINER_ADDRESS and the AE having created a resource of type subscription as a child of the resource CONTAINER_ADDRESS of type container } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a RETRIEVE Request from AE containing  | |
                     | To set to CONTAINER_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid NOTIFY Response containing  | |
                     |Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | notification attribute containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | contentInstance resource containing location information  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | Reference | LOCATION_SOURCE |
| --- | --- | --- |
| TP/oneM2M/CSE/LOC/RET/002_LOS/1 | TS-0001 [1], clause 9.6.10 and 10.2.9TS-0004 [2], clause 7.4.10 and 6.3.4.2.3 | 1 (Network_based) |
| TP/oneM2M/CSE/LOC/RET/002_LOS/2 | ROWSPAN | 2 (Device_based) |
| TP/oneM2M/CSE/LOC/RET/002_LOS/3 | ROWSPAN | 3 (Sharing_based) |

###### TP/oneM2M/CSE/LOC/RET/003

| TP Id | TP/oneM2M/CSE/LOC/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with a <locationPolicy> resource containing attribute locationContainerID set to resourceID of the associated <container> resource |  |
| Reference | TS-0001 [1], clause 10.2.9.3, TS-0004 [2], clause 7.4.10.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUThaving created a resource TARGET_RESOURCE_ADDRESS of type locationPolicy containing locationContainerID attribute set to CONTAINER_ADDRESSand locationSource attribute set to 1 (Network_based) and locationTargetID attributeset to LOCATION_TARGET_ID and locationServer attribute set to LOCATION_SERVER_ADDRESS and the IUT having created a resource CONTAINER_ADDRESS of type container containing locationID attribute set to TARGET_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2000 (OK) and  | |
                     |Content containing  | |
                     | locationPolicy representation containing  | |
                     |locationContainerID attribute set to CONTAINER_ADDRESS  | |
                     |} | `|
+———————-+———————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/LOC/RET/004

| TP Id | TP/oneM2M/CSE/LOC/RET/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the locationStatus attribute with the error status code in the locationPolicy resource when the location information of the requested node is failed to be retrieved due to NSE |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.9.3, TS-0004 [2], clause 7.4.10.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUThaving created a resource TARGET_RESOURCE_ADDRESS of type locationPolicy and the AE having privileges to perform RETRIEVE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2000 (OK) and  | |
                     |Content containing  | |
                     | locationPolicy representationcontaining  | |
                     | locationStatus attribute set to LOCATION_PROTOCOL_SPECIFIC_RESULT_CODE  | |
                     |} | |

| Note: the error status code LOCATION_PROTOCOL_SPECIFIC_RESULT_CODE is location protocol specific, e.g. in OMA Mobile Location Protocol the reslt code is set to 6 indicating POSITION METHOD FAILURE. Note that the failure is caused by the NSE such as location server. |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/LOC/RET/005

| TP Id | TP/oneM2M/CSE/LOC/RET/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error message "NOT_FOUND" when receiving a retrieve request target to an non-existing locationPolicy resource |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.9.3, TS-0004 [2], clause 7.3.3.2 and clause 7.4.10.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUThaving not created LOCATIONPOLICY_ADDRESS of type locationPolicy under the AE a resource and the IUT having privileges to perform RETRIEVE operation on locationPolicy resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a RETRIEVE Request from AE containing  | |
                     | To set to LOCATIONPOLICY_ADDRESSand  | |
                     | From set to AE_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4004 (NOT_FOUND)  | |
                     |} | |
+———————-+————————————————————————————————————————————–+————-+

##### 7.2.2.7.4	NOTIFY Operation

###### TP/oneM2M/CSE/LOC/NTF/001

| TP Id | TP/oneM2M/CSE/LOC/NTF/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification message including the location Information to the originator with frequency of POSITIVE_UPDATE_PERIOD set in field of_locationUpdatePeriod_ |  |
| Reference | TS-0001 [1], clause 8.5.2 and clause 10.2.10.1.1, TS-0004 [2], clause 7.3.3.3 and clause 7.4.11.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the originator being a ADN-AE and the IUT being a MN-CSE and the ADN-AE being no location positioning modules deployed and the ADN-AE being no Network-based positioning capabilities and the IUT having registered the ADN-AE and the IUT having created ORIGINATOR_LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to ORIGINATOR_CONTAINER_ADDRESS and locationUpdatePeriod attribute set to POSITIVE_UPDATE_PERIOD and locationSource attribute set to 3 (Sharing-based) and the IUThaving created ORIGINATOR_CONTAINER_ADDRESS of type container containing locationID attribute set to ORIGINATOR_LOCATIONPOLICY_ADDRESS and the IUThaving created a resource of type subscription as a child resource of ORIGINATOR_CONTAINER_ADDRESS of type container and the IUT having registered a Node being a minimum hop with the originator compared to other nodes registered with the hosting CSE and the IUThaving created under the Node a child resource CLOSEST_NODE_CONTAINER_ADDRESS of type <container> containing a latest contentInstance resource containing the latest location information and the IUT having privileges to perform RETRIEVE operation on the resource CLOSEST_NODE_CONTAINER_ADDRESS of type of container } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a latest resource of container from the CLOSEST_NODE_CONTAINER_ADDRESS containing  | |
                     | latest location information  | |
                     |and the IUT creates a contentInstance as a child resource of ORIGINATOR_CONTAINER_ADDRESS of type <container> containing  | |
                     |the location information that retrieved  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => ADN-AE|
                     |the IUT sends a valid Response containing  | |
                     |Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | notification representation containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | contentInstance resource containing  | |
                     |location information  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/LOC/NTF/002

| TP Id | TP/oneM2M/CSE/LOC/NTF/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification message including the location Information to the originator with frequency of POSITIVE_UPDATE_PERIOD set in field of locationUpdatePeriod |  |
| Reference | TS-0001 [1], clause 8.5.2, clause 10.2.10.1.1 and clause 10.2.10.2.2, TS-0004 [2], clause 7.4.11.2 |  |
| Parent Release | Release 3 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUThaving created a resource LOCATIONPOLICY_ADDRESS of type locationPolicy containing locationContainerID attribute set to CONTAINER_ADDRESSand locationUpdatePeriod attribute set to POSITIVE_UPDATE_PERIOD and locationSource attribute set to LOCATION_SOURCE and the IUT having created a resource CONTAINER_ADDRESS of type container containing locationID attribute set to LOCATIONPOLICY_ADDRESS and the AE having created a resource of type subscription as a child of the resource CONTAINER_ADDRESS of type container and the IUT having privileges to retrieve the location information from the NSE } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= NSE |
                     | the IUT receives a valid Response from NSE containing  | |
                     | location-protocol-specific-result-code of positioning procedure execution and  | |
                     |location-response containing  | |
                     |location information  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     |the IUT transforms the location-response into a oneM2M Response primitive and sends a valid NOTIFY Response containing  | |
                     |Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | notification representation containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | contentInstance representation  | |
                     |} | |

| Note that: the attribute location-protocol-specific-result-code and location-response are location acquisition protocol specific and the exact name of these two attributes have to follow the specification of corresponding location protocol e.g. OMA Mobile Location Protocol. |  |  |
| --- | --- | --- |

Table Referenced values for parameter LOCATION_SOURCE

| TP Id | Reference | LOCATION_SOURCE |
| --- | --- | --- |
| TP/oneM2M/CSE/LOC/NTF/010_LOS/1 | TS-0001 [1], clause 9.6.10 and 10.2.10TS-0004 [2], clause 7.4.11 and 6.3.4.2.3 | 1 (Network_based) |
| TP/oneM2M/CSE/LOC/NTF/010_LOS/2 | ROWSPAN | 2 (Device_based) |
| TP/oneM2M/CSE/LOC/NTF/010_LOS/3 | ROWSPAN | 3 (Sharing_based) |

Table Referenced values for parameter location-protocol-specific-result-code and location-response

| location-protocol-specific-result-code | location-response | Location protocol | Reference |
| --- | --- | --- | --- |
| [Result code] | [Content] | OMA Mobile Location Protocol | Not Present |
| [HTTP Response code] | <TerminalLocation > representation containingattributes address indicating the address of targeted node where location information is retrieved_, locationRetrievalStatus_ indicating the status of retrieval of the location information for the target node , and_currentLocation_ containing the actual location data of the target node | OMA RESTful NetAPI | TS-0004 [2], Annex G |
| [HTTP Response code] | <SubscriptionNotification > representation | OMA RESTful NetAPI | Location Server Open RESTful API Specification |

#### 7.2.2.8	Device Management (DMG)

##### 7.2.2.8.1	Resource timeSeries (TS)

###### TP/oneM2M/CSE/TS/001

| TP Id | TP/oneM2M/CSE/TS/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT inserts the dataGenerationTime informantion of a missing data point and increases the missingDataCurrentNr attribute when a missing data point is detected |  |
| Reference | TS-0001 [1], clause 10.2.39, TS-0004 [2], clause 7.5.1.2.9 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having created a timeSeries resource containing missingDataDetect attribute set to true and a valid periodicInterval attribute and the AE having privileges to perform RETRIEVE operation on the timeSeries resource and the IUT having detected a missing data point } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TIMESERIES_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | timeSeries resourcecontaining  | |
                     | missingDataList attribute containing  | |
                     | dataGenerationTime element of the missing data point and  | |
                     | missingDataCurrentNr attributeset to one  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/TS/002

| TP Id | TP/oneM2M/CSE/TS/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT removes the oldest element in MissingDataList when MissingDataCurrentNr reaches MissingDataMaxNr to enable insertion of a new missing data point |  |
| Reference | TS-0001 [1], clause 10.2.39, TS-0004 [2], clause 7.5.1.2.9 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having created a timeSeries resource containing missingDataDetect attribute set to true and a valid periodicInterval attribute missingDataMaxNr attributeset to one and the AE having privileges to perform RETRIEVE operation on the timeSeries resource and the IUT having detected a missing data point } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUTdetects a new missing data pointand   | |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TIMESERIES_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | timeSeries resourcecontaining  | |
                     | missingDataList attribute containing  | |
                     | dataGenerationTime element of the new missing data point and  | |
                     | missingDataCurrentNr attributeset to one  | |
                     |} | |
+———————-+——————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/TS/003

| TP Id | TP/oneM2M/CSE/TS/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification with missing data points information when the number of missing data points detected becomes equal to the minimum specified missing number of the Time Series Data specified in missingData condition of the subscription resource |  |
| Reference | TS-0001 [1], clause 10.2.39, TS-0004 [2], clause 7.5.1.2.9 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a timeSeries resource containing missingDataDetect attribute set to true and a valid periodicInterval attribute and a subscription resource containing notificationURI attribute set to AE1_RESOURCE_ADDRESS and eventNotificationCriteria attribute containing missingData element containing number element set to MISSING_DATA_POINT_NR and duration element set to MISSING_DATA_POINT_DURATION and the AE1 having privileges to perform RETRIEVE operation on the timeSeries resource and the AE2 having privileges to perform CREATE operation on the timeSeries resource and the IUT having detected MISSING_DATA_POINT_NR-1 missing data points from AE2 } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT detects a new missing data point within MISSING_DATA_POINT_DURATION timer  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid NOTIFY request containing  | |
                     | Content containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | timeSeries resourcecontaining  | |
                     | missingDataList attributecontaining  | |
                     | dataGenerationTime of missing data points and  | |
                     | currentMissingDataNr attributeset to MISSING_DATA_POINT_NR  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/TS/004

| TP Id | TP/oneM2M/CSE/TS/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not send notification when the total number of missing data points does not exceed the minimum specified missing number of the Time Series Data specified in missingData condition of the subscription resource during the window duration specified in the same missingData condition |  |
| Reference | TS-0001 [1], clause 10.2.39, TS-0004 [2], clause 7.5.1.2.9 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a timeSeries resource containing missingDataDetect attribute set to true and a valid periodicInterval attribute and a subscription resource containing notificationURI attribute set to AE1_RESOURCE_ADDRESS and eventNotificationcriteria attribute containing missingData element containing number element set to MISSING_DATA_POINT_NR and duration element set to MISSING_DATA_POINT_DURATION and the AE1 having privileges to perform RETRIEVE operation on the timeSeries resource and the AE2 having privileges to perform CREATE operation on the timeSeries resource and the IUT having detected MISSING_DATA_POINT_NR-1 missing data points from AE2 } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT does not detect a new missing data point within MISSING_DATA_POINT_DURATION timer  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT does not sends a NOTIFY request and  | |
                     | the IUT restarts missing data point counter and timer  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————————————————-+————-+ Editor's Note: This test case is not available on the ATS. Currently no idea to implementate internal timer, but will try to find a way to solve this problem.

###### TP/oneM2M/CSE/TS/005

| TP Id | TP/oneM2M/CSE/TS/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a final notification with the missing data points information when subscription is terminated |  |
| Reference | TS-0001 [1], clause 10.2.39, TS-0004 [2], clause 7.5.1.2.9 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having created a timeSeries resource containing missingDataDetect attribute set to true and a valid periodicInterval attribute and a subscription resource containing notificationURI attribute set to AE1_RESOURCE_ADDRESS and eventNotificationcriteria attribute containing missingData element containing number element set to MISSING_DATA_POINT_NR and duration element set to MISSING_DATA_POINT_DURATION and the AE1 having privileges to perform RETRIEVE operation on the timeSeries resource and the AE1 having privileges to perform DELETE operation on the subscription resource and the AE2 having privileges to perform CREATE operation on the timeSeries resource and the IUT having detected MISSING_DATA_POINT_NR-1 missing data points from AE2 } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid DELETE Request from AE1 containing  | |
                     | To set to SUBSCRIPTION_RESOURCE_ADDRESS and  | |
                     | From set to AE1-ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |  |
                     | the IUT sends a valid Response containing  |IUT => AE1  |
                     | Response Status Code set to 2000 (OK) and  |  |
                     | and the IUT sends a NOTIFY Request contaning  |IUT => AE2  |
                     | Content containing  |  |
                     | notificationEvent attribute containing  |  |
                     | representation attribute containing  |  |
                     | timeSeries resourcecontaining  |  |
                     | missingDataList attributecontaining  |  |
                     | dataGenerationTime of missing data points and  |  |
                     | currentMissingDataNr attributeset to MISSING_DATA_POINT_NR - 1  | |
                     |} | |
+———————-+———————————————————————————————————————————————-+————-+

#### 7.2.2.9	Communication Management and Delivery Handling(CMDH)

##### 7.2.2.9.1	Resource pollingChannel (PCH)

###### TP/oneM2M/CSE/PCH/001

| TP Id | TP/oneM2M/CSE/PCH/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which hosts <pollingChannel> resource forwards a Notify request successfully to a target AE, once the IUT receives a polling request from AE (retrieve request to <pollingChannelURI> resource) See the Note below. |  |
| Reference | TS-0001 [1], clause 10.2.13.6, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having register ed an AE1 and the AE having a child <pollingChannel> resource and the IUT having re ceived a retrieve request from the AE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a NOTIFY Request from the CSE containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID } | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT send s a RETRIEVE Response to the AE containing  | |
                     |Response Status Code set to 2000 (OK)  | |
                     |Contentcontaining  | |
                     | the request re ceived from the CSE  | |
                     |} | |

| Note: Based on figure 10.2.5.12-1 in TS-0001[1] step 001, 002 and 003 in order. In this case, IUT is pending AE's polling request since there was no request to AE |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/PCH/002

| TP Id | TP/oneM2M/CSE/PCH/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which hosts <pollingChannel> resource forwards a Notify request successfully to a target AE, once the IUT receives a polling request from the AE |  |
| Reference | TS-0001 [1], clause 10.2.13.6, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having register ed an AE and the AE having a child <pollingChannel> resource and the IUT having receive d a NOTIFY Request from CSE containing To set to AE_RESOURCE_ADDRESS and From set to CSE_ID } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a retrieve Request from the AE containing  | |
                     | To set to POLLINGCHANNELURI _RESOURCE_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT send s a Response to the AE containing  | |
                     |Response Status Code set to 2000 (OK)  | |
                     |Contentcontaining  | |
                     | the Request re ceived from the CSE  | |
                     |} | |

| Note: Based on figure 10.2.13.1-1 in TS-0001[1] step 002 is followed by 001 and 003 The CSE's request to AE is pended and will be sent as soon as AE sends polling request to CSE. |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/PCH/003

| TP Id | TP/oneM2M/CSE/PCH/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT performs both forwarding the response to the CSE and sending response to AE after receiving a Notify Request from the AE sent to the <pollingChannelURI> resource |  |
| Reference | TS-0001 [1], clause 10.2.13.8, TS-0004 [2], clause 7.4.22.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having register ed an AE and the AE1 having a child <pollingChannel> resource and the IUT having re ceived a retrieve Request from the AE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and the IUT having re ceived a NOTIFY Request from the CSE containing To set to AE_RESOURCE_ADDRESS and From set to CSE_ID and the IUT send s a Response to the AE containing Response Status Code set to 2000 (OK) Contentcontaining the Request re ceived from the CSE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a NOTIFY Request from the AE containing  | |
                     | To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE  |
                     | the IUT sends a Response to the CSE  |IUT => AE |
                     |Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |the IUT sends the valid NOTIFY Response to the AE  | |
                     |} | |

| Note: Based on figure 10.2.13.1-1 in TS-0001[1] step 005 and 006 |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/PCH/004

| TP Id | TP/oneM2M/CSE/PCH/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT which performs polling sends the Notify request to <pollingChannelURI> Hosting CSE after receiving response using polling channel |  |
| Reference | TS-0001 [1], clause 10.2.13.8, TS-0004 [2], clause 7.4.22.2.5 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to a CSE and the IUT having a <pollingChannel> resource and the IUT having sent a retrieve Request to the CSE To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and the CSE having re ceived the Request containing To set to CSE_RESOURCE_ADDRESS and From set to ORIGINATOR_ID } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE1 |
                     | the IUT receives a polling Response from the CSE containing  | |
                     |Response Status Code set to 2000 (OK)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a NOTIFY Request to the CSE containing  | |
                     | To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |

| Note: Based on figure 10.2.13.1-1 in TS-0001[1] step 004 and 005 |  |  |
| --- | --- | --- |

###### TP/oneM2M/CSE/PCH/005

| TP Id | TP/oneM2M/CSE/PCH/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a <pollingChannel> OPERATION of the AE when AE-ID is not same as the AE-ID of the parent resource |  |
| Reference | TS-0001 [1], clause 10.2.13.2, TS-0004 [2], clause 7.4.21 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having register ed an AE1 and the IUT having registered an AE2 and the IUThaving created a pollingChannel resource on the AE1 } |  |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE2 |
                     |the IUT receives a valid OPERATION Request from the AE2 containing  | |
                     | To set to POLLING_CHANNEL_RESOURCE_ADDRESS and  | |
                     | From set to AE2_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE2 |
                     | the IUT send s a valid Response containing  | |
                     |Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+——————————————————————————————————————–+————-+

| TP Id | Reference | OPERATION |
| --- | --- | --- |
| TP/oneM2M/CSE/PCH/005_RET | TS-0001 10.2.13.3 | RETRIEVE |
| TP/oneM2M/CSE/PCH/005_UPD | TS-0001 10.2.13.4 | UPDATE |
| TP/oneM2M/CSE/PCH/005_DEL | TS-0001 10.2.13.5 | DELETE |

###### TP/oneM2M/CSE/PCH/006

| TP Id | TP/oneM2M/CSE/PCH/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the response with a status to the CSE1 when the request expires according to its Request Expiration Timestamp |  |
| Reference | TS-0001 [1], clause 10.2.13.6 |  |
| Config Id | CF02 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_PCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having register ed an AE and the AE having a child <pollingChannel> resource and the IUT having receive d a RETRIEVE Request from AE containing To set to POLLINGCHANNELURI_RESOURCE_ADDRESS and Request Expiration Timestamp set to VALUE } |  |

- +-------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | Request Expiration Timestamp expires  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | and the IUT sends a Response to the AE containing  | |
                     |Response Status Code set to 4008 (REQUEST_TIMEOUT)  | |
                     |} | |
+———————-+————————————————————————————————————————-+————-+

##### 7.2.2.9.2	Response Type (RT)

###### 7.2.2.9.2.1	nonBlockingRequestSynch (NBS)

###### TP/oneM2M/CSE/RT/NBS/001

| TP Id | TP/oneM2M/CSE/RT/NBS/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects a Non-Blocking Synchronous Request OPERATION if the IUT does not support the <request> resource |  |
| Reference | TS-0001 [1], clause 8.2.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_SYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | Resource Type set to 3 (container)and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | ResponseType set to 1 (nonBlockingRequestSynch)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5206 (NON_BLOCKING_REQUEST_NOT_SUPPORTED)  | |
                     |} | |
+———————-+——————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBS/001_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBS/001_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBS/001_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBS/001_DEL | DELETE |

Editor's Note: Need to be clarified that does it make sense to have such TPs on the NBR group when no support of NBR from the IUT is required.

###### TP/oneM2M/CSE/RT/NBS/002

| TP Id | TP/oneM2M/CSE/RT/NBS/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds to a Non-Blocking Synchronous Request OPERATION with the reference of a <request> resource provided within the Content parameter of the Response |  |
| Reference | TS-0001 [1], clause 8.2.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_SYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | Resource Type set to 3 (container)and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | ResponseType set to 1 (nonBlockingRequestSynch)  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 1001 (ACCEPTED for n onBlockingRequestSynch) and  | |
                     | Content containing  | |
                     | URI attribute set to REQUEST_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBS/002_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBS/002_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBS/002_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBS/002_DEL | DELETE |

###### TP/oneM2M/CSE/RT/NBS/003

| TP Id | TP/oneM2M/CSE/RT/NBS/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully the <request> resource after receiving a Non-Blocking Synchronous Request OPERATION and it contains the expected result in operationResult attribute |  |
| Reference | TS-0001 [1], clause 8.2.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_SYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having received a Non-Blocking Synchronous Request OPERATION and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type <request> under the AE resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | ResponseTypeset to 3 (blockingRequest)  | |
                     |no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | request resource representation containing  | |
                     | operationResult attribute containing  | |
                     | expected response to OPERATION  | |
                     |  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBS/003_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBS/003_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBS/003_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBS/003_DEL | DELETE |

###### 7.2.2.9.2.2	nonBlockingRequestAsynch (NBA)

###### TP/oneM2M/CSE/RT/NBA/001

| TP Id | TP/oneM2M/CSE/RT/NBA/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds to a Non-Blocking Asynchronous Request OPERATION without a reference to a resource containing the context of the request if the IUT does not support the <request> resource |  |
| Reference | TS-0001 [1], clause 8.2.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | Resource Type set to 3 (container)and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | ResponseType set to 2 (nonBlockingRequestAsynch)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     |the IUT sends a valid Response containing  | |
                     | Response Status Code set to 1002 (ACCEPTED for nonBlockingRequestAsynch) and  | |
                     | no Content  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBA/001_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBA/001_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBA/001_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBA/001_DEL | DELETE |

Editor's Note: Need to be clarified that does it make sense to have such TPs on the NBR group when no support of NBR from the IUT is required.

###### TP/oneM2M/CSE/RT/NBA/002

| TP Id | TP/oneM2M/CSE/RT/NBA/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds to a Non-Blocking Asynchronous Request OPERATION with the reference of a <request> resource provided within the Content parameter of the Response |  |
| Reference | TS-0001 [1], clause 8.2.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | Resource Type set to 3 (container)and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | ResponseType set to 2 (nonBlockingRequestAsynch)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 1002 (ACCEPTED for nonBlockingRequestAsynch) and  | |
                     | Content containing  | |
                     | URI attribute set to REQUEST_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBA/002_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBA/002_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBA/002_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBA/002_DEL | DELETE |

###### TP/oneM2M/CSE/RT/NBA/003

| TP Id | TP/oneM2M/CSE/RT/NBA/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the result of the requested Non-Blocking Asynchronous OPERATION as notification to the Originator when no notification target list is provided |  |
| Reference | TS-0001 [1], clause 8.1.2, 8.2.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | Resource Type set to 3 (container)and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | ResponseType set to 2 (nonBlockingRequestAsynch)  | |
                     | NotificationURI not set   | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     |the IUT sends a valid Response (Ack)  | |
                     | and the IUT sends a valid NOTIFY Request containing  | |
                     | Content containing  | |
                     | ResponsePrimitive representation indicating the response of the original Non-Blocking Asynchronous OPERATION containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/RT/NBA/003_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/RT/NBA/003_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/RT/NBA/003_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/RT/NBA/003_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/RT/NBA/004

| TP Id | TP/oneM2M/CSE/RT/NBA/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not send the result of the requested Non-Blocking Asynchronous OPERATION as notification to the Originator when an empty notification target list is provided |  |
| Reference | TS-0001 [1], clause 8.1.2, 8.2.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the Originator having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | Resource Type set to 3 (container)and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | ResponseType set to 2 (nonBlockingRequestAsynch) and  | |
                     | NotificationURI set to empty list  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     |the IUT sends a valid Response (Ack)  | |
                     | and the IUT does not send a NOTIFY Response  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/RT/NBA/004_CRE | CREATE |
| TP/oneM2M/CSE/RT/NBA/004_UPD | UPDATE |
| TP/oneM2M/CSE/RT/NBA/004_RET | RETRIEVE |
| TP/oneM2M/CSE/RT/NBA/004_DEL | DELETE |

###### TP/oneM2M/CSE/RT/NBA/005

| TP Id | TP/oneM2M/CSE/RT/NBA/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the result of the requested Non-Blocking Asynchronous OPERATION as notification to the notification targets indicated in responseType/notificationURI attribute of the request |  |
| Reference | TS-0001 [1], clause 8.1.2, 8.2.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_NON_BLOCKING_REQUEST_ASYNCH |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the AE1 having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | Resource Type set to 3 (container)and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Response Type parameter containing  | |
                     | responseTypeValue set to 2 (nonBlockingRequestAsynch) and  | |
                     | NotificationURI set to AE1_RESOURCE_ADDRESS and AE2_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE1  |
                     | the IUT sends a valid Response (Ack)  |  |
                     | and the IUT sends a valid NOTIFY Request containing to AE1  |  |
                     | Content containing  |IUT => AE1  |
                     | ResponsePrimitive representation indicating the response of the original Non-Blocking Asynchronous OPERATION containing  |  |
                     | Response Status Code set to RESPONSE_STATUS_CODE  |  |
                     | and the IUT sends a valid NOTIFY Request containing to AE2  |  |
                     | Content containing  |  |
                     | ResponsePrimitive representation indicating the response of the original Non-Blocking Asynchronous OPERATION containing  |IUT => AE2 |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/RT/NBA/005_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/RT/NBA/005_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/RT/NBA/005_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/RT/NBA/005_DEL | DELETE | 2002 (DELETED) |

#### 7.2.2.10	Security(SEC)

##### 7.2.2.10.1	CREATE Operation

###### TP/oneM2M/CSE/SEC/ACP/CRE/001

| TP Id | TP/oneM2M/CSE/SEC/ACP/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a accessControlPolicy resource with privileges attribute having multiple access control rules |  |
| Reference | TS-0001 [1], clause 9.6.2-2 and clause, 10.2.21 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_IDand  | |
                     | Content containing  | |
                     | accessControlPolicy resource containing  | |
                     | privileges attribute containing  | |
                     | accessControlRule attribute containing  | |
                     | ACCESS_CONTROL_RULE_1 and  | |
                     | ACCESS_CONTROL_RULE_2  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT creates the accessControlPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+—————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEC/ACP/CRE/002

| TP Id | TP/oneM2M/CSE/SEC/ACP/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a accessControlPolicy resource with selfPrivileges attribute having multiple access control rules |  |
| Reference | TS-0001 [1], clause 9.6.2-2 and clause 10.2.21 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_IDand  | |
                     | Content containing  | |
                     | accessControlPolicy resource containing  | |
                     | selfPrivileges attribute containing  | |
                     | accessControlRule attribute containing  | |
                     | ACCESS_CONTROL_RULE_1 and  | |
                     | ACCESS_CONTROL_RULE_2  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT creates the accessControlPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+——————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEC/ACP/CRE/003

| TP Id | TP/oneM2M/CSE/SEC/ACP/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the creation of a accessControlPolicy resource with privileges attribute having empty rules |  |
| Reference | TS-0001 [1], clause 9.6.2 and clause 10.2.21 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_IDand  | |
                     | Content containing  | |
                     | accessControlPolicy resource containing  | |
                     | privileges attribute set to empty list  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT creates the accessControlPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+———————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEC/ACP/CRE/004

| TP Id | TP/oneM2M/CSE/SEC/ACP/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the creation of a accessControlPolicy resource with selfPrivileges attribute having empty rules |  |
| Reference | TS-0001 [1], clause 9.6.2and clause 10.2.21 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_IDand  | |
                     | Content containing  | |
                     | accessControlPolicy resource containing  | |
                     | selfPrivileges attribute set to empty list  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the accessControlPolicy resource  | |
                     | and the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+———————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEC/ACP/CRE/005

| TP Id | TP/oneM2M/CSE/SEC/ACP/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when an AE creates an accessControlPolicy resource having accessControlOriginators element set to a group resource identifier which contains AE resource as member |  |
| Reference | TS-0001 [1], clause 9.6.2 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the AE1 having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having registered the AE2 and the IUT having created a group resource at GROUP_RESOURCE_ADDRESS containing memberIDs set to AE2_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid CREATE Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | accessControlPolicy resource containing  | |
                     | privileges containing  | |
                     | accessControlRule element containing  | |
                     | accessControlOriginators element set to GROUP_RESOURCE_ADDRESS  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————+————-+

##### 7.2.2.10.2	UPDATE Operation

###### TP/oneM2M/CSE/SEC/ACP/UPD/001

| TP Id | TP/oneM2M/CSE/SEC/ACP/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the accessControlPolicyIDs attribute of the <AE> resource when the AE has privileges for UPDATE operation in any selfPrivileges of the <accessControlPolicy> resources which this attribute originally indicates |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing accessControlPolicyID attribute set to ACP_ID_1 and the AE having privileges to perform UPDATE operation on the AE_RESOURCE_ADDRESS in selfPrivileges } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | accessControlPolicyIDs attribute set to ACP_ID_2  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT updates the accessControlPolicyIDs attribute of AE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | AE resourcecontaining  | |
                     | accessControlPolicyID attribute set to ACP_ID_2  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEC/ACP/UPD/002

| TP Id | TP/oneM2M/CSE/SEC/ACP/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries an UPDATE operation on the accessControlPolicyIDs attribute without having privileges for such operation in any selfPrivileges of the <accessControlPolicy> resources which this attribute originally indicates |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing accessControlPolicyID attribute set to ACP_ID_1 and the AE having no privileges to perform UPDATE operation on the AE_RESOURCE_ADDRESS in selfPrivileges } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | AE resource containing  | |
                     | accessControlPolicyID attribute set to ACP_ID_2  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not update the accessControlPolicyIDs attribute of AE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————————-+————-+

##### 7.2.2.10.3	BASIC OPERATION

###### TP/oneM2M/CSE/SEC/ACP/001

| TP Id | TP/oneM2M/CSE/SEC/ACP/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when an ORIGINATOR tries an OPERATION without having privileges for performing the OPERATION on the TARGET_RESOURCE_TYPE resource |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the ORIGINATOR having no privileges to perform an OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |when {   |IUT <= ORIGINATOR|
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to ORIGINATOR_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |then {   |IUT => ORIGINATOR|
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+—————–+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/001_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/001_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/001_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/001_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/002

| TP Id | TP/oneM2M/CSE/SEC/ACP/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when the AE tries an OPERATION on its <AE> resource which has multiple accessControlPolicyID attribute |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 and clause 9.6.2.1-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <Container> resource containing accessControlPolicyID attribute containing ACP_ID_1 and ACP_ID_2 and the AE having privileges to perform an OPERATION on itself given by ACP_ID_2 only } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/002_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/002_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/002_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/002_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/003

| TP Id | TP/oneM2M/CSE/SEC/ACP/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when an allowed DOMAIN tries an OPERATION on a AE_RESOURCE_TYPE resource. |  |
| Reference | TS-0001 [1], clause 9.6.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the DOMAIN having privileges to perform an OPERATION on the resource AE_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= DOMAIN|
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to DOMAIN  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => DOMAIN|
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+———————————————————————————————————+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/003_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/003_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/003_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/003_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/004

| TP Id | TP/oneM2M/CSE/SEC/ACP/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when any ORIGINATOR tries an OPERATION on a AE_RESOURCE_TYPE resource for which all originators are allowed |  |
| Reference | TS-0001 [1], clause 9.6.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and any originator (*) having privileges to perform an OPE RATION on the resource AE_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |when {   |IUT <= ORIGINATOR|
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to ORIGINATOR  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |then {   |IUT => ORIGINATOR|
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————+—————–+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/004_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/004_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/004_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/004_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/005

| TP Id | TP/oneM2M/CSE/SEC/ACP/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource and that the request is within accessControlTimeWindow context |  |
| Reference | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AEhaving privileges to perform OPERATION on the AE_RESOURCE_ADDRESSwithin TIME_WINDOW } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | within TIME_WINDOW  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/005_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/005_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/005_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/005_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/006

| TP Id | TP/oneM2M/CSE/SEC/ACP/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries an_OPERATION_ on a AE_RESOURCE_TYPE resource out of accessControlTimeWindow context |  |
| Reference | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AEhaving privileges to perform OPERATION on the AE_RESOURCE_ADDRESSwithin TIME_WINDOW } |  |

- +------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | out of TIME_WINDOW  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/006_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/006_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/006_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/006_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/007

| TP Id | TP/oneM2M/CSE/SEC/ACP/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource and that the request is within accessControlLocationRegion context |  |
| Reference | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a child locationPolicy resource containing locationContainerID attribute set to CONTAINER_ID and a child container resource containing locationID attribute set to LOCATION_POLICY_ID and contentInstance resource containing content attribute set to LOCATION and the AEhaving privileges to perform OPERATION on the AE_RESOURCE_ADDRESSwithin LOCATION_RESTRICTION } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | and LOCATION within LOCATION_RESTRICTION  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/007_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/007_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/007_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/007_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/008

| TP Id | TP/oneM2M/CSE/SEC/ACP/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlLocationRegion context |  |
| Reference | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AEhaving privileges to perform OPERATION on the AE_RESOURCE_ADDRESSwithin LOCATION_RESTRICTION } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | out of LOCATION_RESTRICTION  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/008_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/008_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/008_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/008_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/009

| TP Id | TP/oneM2M/CSE/SEC/ACP/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when the AE tries an OPERATION on a TARGET_RESOURCE_TYPE resource and that the request is within accessControlIpAddresses context |  |
| Reference | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AEhaving privileges to perform OPERATION on the AE_RESOURCE_ADDRESSwithin IP_ADDRESS_RESTRICTION } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | and AE_IP_ADDRESSwithin IP_ADDRESS_RESTRICTION  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/009_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/009_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/009_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/009_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/010

| TP Id | TP/oneM2M/CSE/SEC/ACP/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE tries an OPERATION on a AE_RESOURCE_TYPE resource out of accessControlIpAddresses context |  |
| Reference | TS-0001 [1], clause 9.6.2.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AEhaving privileges to perform OPERATION on the AE_RESOURCE_ADDRESSwithin IP_ADDRESS_RESTRICTION } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | and AE_IP_ADDRESS out of IP_ADDRESS_RESTRICTION  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/010_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/010_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/010_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/010_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/011

| TP Id | TP/oneM2M/CSE/SEC/ACP/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when the AE tries an OPERATION on its child <container> resource whose accessControlPolicyID attribute is not set and AE has privileges for such OPERATION on its associated accessControlPolicy resource |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <Container> resource containing no accessControlPolicyID attribute and the AE having privileges to perform an OPERATION on the resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/011_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/011_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/011_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/011_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/012

| TP Id | TP/oneM2M/CSE/SEC/ACP/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when an ORIGINATOR tries an OPERATION on a <Container> resource with no accessControlPolicyID associated and such ORIGINATOR having no privileges for performing the OPERATION on the <Container> resource |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 1 |  |
| PICS Selection | PICS_CSE, PICS_ACP_SUPPORT |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having a child <Container> resource containing no accessControlPolicyID attribute and the ORIGINATOR having no privileges to perform an OPERATION on the <Container> resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |when {   |IUT <= ORIGINATOR|
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to ORIGINATOR_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-----------------+

- |then {   |IUT => ORIGINATOR|
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————–+—————–+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/012_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/012_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/012_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/012_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/013

| TP Id | TP/oneM2M/CSE/SEC/ACP/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when the creator AE tries an OPERATION on the AE resource whose accessControlPolicyIDs attribute is not set |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the <AE> resource having no accessControlPolicyIDs attribute } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/013_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/013_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/013_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/013_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/014

| TP Id | TP/oneM2M/CSE/SEC/ACP/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when an AE2 tries an OPERATION on the AE1 resource whose accessControlPolicyIDs attribute is not set |  |
| Reference | TS-0001 [1], clause 9.6.1.3.2-1 |  |
| Config Id | CF01 |  |
| Parent release | Release 1 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the <AE> resource of AE1 having no accessControlPolicyIDs attribute } |  |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE2 |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE1_RESOURCE_ADDRESS and  | |
                     | From set to AE2_ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE2 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/014_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/014_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/014_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/014_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/015

| TP Id | TP/oneM2M/CSE/SEC/ACP/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds successfully when an AE tries an OPERATION on the AE1 resource whose accessControlPolicyIDs attribute is set |  |
| Reference | TS-0001 [1], clause 9.6.2 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having <group> resource GRP1 having AE2 as memberIds and the <AE> resource of AE1 having accessControlPolicyIDs attribute set and this referenced <accessControlPolicy> having atleast single ACR accessControlOriginators attribute having resourceID of <group> resource GRP1 and accessControlOperations to OPERATION } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE2 |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE1_RESOURCE_ADDRESS and  | |
                     | From set to AE2_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE2 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/015_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SEC/ACP/015_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/015_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/015_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/SEC/ACP/016

| TP Id | TP/oneM2M/CSE/SEC/ACP/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when an AE tries an OPERATION on the AE1 resource whose accessControlPolicyIDs attribute is set |  |
| Reference | TS-0001 [1], clause 9.6.2 |  |
| Config Id | CF01 |  |
| Parent release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having registered the AE3 and the IUT having <group> resource GRP1 having AE3 as member only and the <AE> resource of AE1 having accessControlPolicyIDs attribute set and this referenced <accessControlPolicy> having at least single ACR accessControlOriginators attribute having resourceID of <group> resource GRP1 and accessControlOperations to OPERATION } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE2 |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to AE1_RESOURCE_ADDRESS and  | |
                     | From set to AE2_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE2 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SEC/ACP/016_CRE | CREATE |
| TP/oneM2M/CSE/SEC/ACP/016_RET | RETRIEVE |
| TP/oneM2M/CSE/SEC/ACP/016_UPD | UPDATE |
| TP/oneM2M/CSE/SEC/ACP/016_DEL | DELETE |

###### TP/oneM2M/CSE/SEC/ACP/017

| TP Id | TP/oneM2M/CSE/SEC/ACP/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts the OPERATION request on <accessControlPolicy> resource when at least one of the access control rules defined in the selfPrivileges attribute allows the operation |  |
| Reference | TS-0001 [1], clause 10.2.3.1, TS-0004 [2], clause 7.4.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a Hosting CSE and the IUT having registered the AE as ORIGINATOR and the IUT having an <accessControlPolicy> resource at TARGET_RESOURCE_ADDRESS containing selfPrivileges attribute containing access control rule(s) to allow the OPERATION } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SEC/ACP/017_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SEC/ACP/017_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SEC/ACP/017_DEL | DELETE | 2002 (DELETED) |

##### 7.2.2.10.4	Authentication

###### 7.2.2.10.4.1	ESPrim (ESP)

7.2.2.10.4.1.1	NOTIFY Operation

TP/oneM2M/CSE/SEC/ESP/NTF/001

| TP Id | TP/oneM2M/CSE/SEC/ESP/NTF/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a new ESPrimRandObject when requested |  |
| Reference | TS-0003 8.4.2 - step B.2C.2, TS-0004 7.5.1.2.11 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having established a pairwiseESPrimKey with the Originator (AE1) } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid NOTIFY request from AE1 containing To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE1_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 3 (receiverESPrimRandObject  | |
                     | Request)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid NOTIFY Response containing  | |
                     | To set to TARGET_ORIGINATOR_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 4 (receiverESPrimRandObject  | |
                     | Response) and  | |
                     | esprimRandObject element containing  | |
                     | receiverESPrimRandObject  | |
                     |} | |
+———————-+————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ESP/NTF/002

| TP Id | TP/oneM2M/CSE/SEC/ESP/NTF/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with a valid ESPrim object when it receives a request |  |
| Reference | TS-0003 8.4.2 - step C.9, TS-0004 7.6.2 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having established a sessionESPrimKey with the Originator (AE1) } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid NOTIFY Request from AE1 containing To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to AE1_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 5 (ESPrim Object) and  | |
                     | valid esprimObject element  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid NOTIFY Response containing  | |
                     | To set to RECEIVER_RECEIVER_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 5 (ESPrim Object) and  | |
                     | valid esprimObject element  | |
                     |} | |
+———————-+————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ESP/NTF/003

| TP Id | TP/oneM2M/CSE/SEC/ESP/NTF/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully detects AE impersonation in an End-to-End Security of Primitives |  |
| Reference | TS-0003 7.2.2 - step 4, TS-0004 7.6.2 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having established a sessionESPrimKey with with the Originator (AE1) } |  |

- +-------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE1 |
                     | the IUT receives a valid NOTIFY Request from AE1 containing   | |
                     | To set to CSE_ID and  | |
                     | From set to a value other than AE1_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 5 (ESPrim Object) and  | |
                     | valid esprimObject element  | |
                     |} | |

- +-------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response containing  | IUT => AE1 |
                     | Response Status Code set to  | |
                     | 4116 (ESPRIM_IMPERSONATION_ERROR)  | |
                     |} | |
+———————-+————————————————————————————-+————-+

7.2.2.10.4.1.2	RETRIEVE Operation

TP/oneM2M/CSE/SEC/ESP/RET/001

| TP Id | TP/oneM2M/CSE/SEC/ESP/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a valid request for e2eSecInfo attribute of the Receiver's <remoteCSE> resource in the CSE Registered with Receiver CSE |  |
| Reference | TS-0003 8.4.2 - step B.2A, TS-0004 7.5.1.2.11 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUThaving established a pairwiseESPrimKey with the Receiver CSE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing  | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE <= IUT |
                     | the IUT sends a valid RETRIEVE Request to Target CSE containing  | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | remoteCSE resourcecontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |
+———————-+————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/SEC/ESP/RET/002

| TP Id | TP/oneM2M/CSE/SEC/ESP/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification containing an inner "receiverESPrimRandObject request" to the Receiver if the sharedReceiverESPrimRandObject parameter is not present in the <remoteCSE> resource in the CSE Registered with Receiver CSE |  |
| Reference | TS-0003 8.4.2 - step B.2D, TS-0004 7.5.1.2.11 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and and the IUT being switched on a nd the IUT having established a pairwiseESPrimKey with the Originator and the <remoteCSE> resource at TARGET_REMOTE_CSE_ADDRESS containing e2eSecInfo attribute with no sharedReceiverESPrimRandObject parameter } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing   | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFICATION Request containing  | |
                     | To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 3 (receiverESPrimRandObject  | |
                     | Request)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ESP/RET/003

| TP Id | TP/oneM2M/CSE/SEC/ESP/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notification containing an inner "receiverESPrimRandObject request" to the Receiver if the sharedReceiverESPrimRandObject in the <remoteCSE> resource is expired |  |
| Reference | TS-0003 8.4.2 - step B.2B, TS-0004 7.5.1.2.11 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT having established a pairwiseESPrimKey with the Originator and and the <remoteCSE> resource at TARGET_REMOTE_CSE_ADDRESS contain ing e2eSecInfo attribute with an expired sharedReceiverESPrimRandObject parameter } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid RETRIEVE Request containing   | |
                     | To set to TARGET_REMOTE_CSE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | attributeList elementcontaining  | |
                     | e2eSecInfo attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFICATION Request containing  | |
                     | To set to TARGET_RECEIVER_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 3 (receiverESPrimRandObject  | |
                     | Request)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————+————-+

###### 7.2.2.10.4.2	Roles (ROL)

7.2.2.10.4.2.1	CREATE Operation

TP/oneM2M/CSE/SEC/ROL/CRE/001

| TP Id | TP/oneM2M/CSE/SEC/ROL/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a CREATE <role> resource request to the Role Repository (CSE1) when triggered |  |
| Reference | TS-0003 7.4.2.2 - step 3 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT acting as an Authorization Authority } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to ROLE_REPOSITORY_ADDRESS (CSE1) and  | |
                     | Resource Type set to 31 (role) and  | |
                     | Content containing  | |
                     | < role> resource representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid CREATE Request to CSE1 containing  | IUT => CSE1 |
                     | To set to ROLE_REPOSITORY_ADDRESS and  | |
                     | Resource Type set to 31 (role) and  | |
                     | Content containing  | |
                     | <role> resource representation  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

TP/oneM2M/CSE/SEC/ROL/CRE/002

| TP Id | TP/oneM2M/CSE/SEC/ROL/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully creates the <role> resource requested by the Authorization Authority |  |
| Reference | TS-0003 7.4.2.2 - step 4 and 5 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and and the IUT being switched on and and the IUT acting as a Role Repository } |  |

- +------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE1 |
                     | the IUT receives a valid CREATE Request from CSE1 (AA) containing   | |
                     | From set to CSE1_ID and  | |
                     | To set to ROLE_REPOSITORY_ADDRESS and  | |
                     | Resource Type set to 31 (role) and  | |
                     | Content containing  | |
                     | <role> resource representation  | |
                     |} | |

- +------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT creates the <role> resource  | IUT => CSE1 |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | <role> resource representation  | |
                     |} | |
+———————-+——————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ROL/CRE/003

| TP Id | TP/oneM2M/CSE/SEC/ROL/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a CREATE <token> resource request to the Token Repository when triggered |  |
| Reference | TS-0003 7.4.2.3 - step 2 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT acting as an Authorization Authority and the IUT having assigned a role for the Originator } |  |

- +---------------------------------------------------------------------------------------+-------------+

- |when {  |NA |
                     | the IUT is triggered to send a valid CREATE Request containing  | |
                     | To set to TOKEN_REPOSITORY_ADDRESS (CSE1) and  | |
                     | Resource Type set to 32 (token) and  | |
                     | Content containing  | |
                     | <token> resource representation  | |
                     |} | |

- +---------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid CREATE Request to CSE containing  | IUT => CSE1 |
                     | To set to TOKEN _REPOSITORY_ADDRESS and  | |
                     | Resource Type set to 32 (token) and  | |
                     | Content containing  | |
                     | <token> resource representation  | |
                     |} | |
+———————-+—————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ROL/CRE/004

| TP Id | TP/oneM2M/CSE/SEC/ROL/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully creates the <token> resource requested by the Authorization Authority |  |
| Reference | TS-0003 7.4.2.3 - step 3 and 4 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT acting as a Token Repository and the Authorization Authority having assigned a role for the Originator } |  |

- +-------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE1 |
                     | the IUT receives a valid CREATE Request from CSE1 (AA) containing   | |
                     | From set to CSE1_ID and  | |
                     | To set to TOKEN_REPOSITORY_ADDRESS and  | |
                     | Resource Type set to 32 (token) and  | |
                     | Content containing  | |
                     | <token> resource representation  | |
                     |} | |

- +-------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT creates the <token> resource  | IUT => CSE1 |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | <token> resource representation  | |
                     |} | |
+———————-+——————————————————————————————-+————-+

7.2.2.10.4.2.2	RETRIEVE Operation

TP/oneM2M/CSE/SEC/ROL/RET/001

| TP Id | TP/oneM2M/CSE/SEC/ROL/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the <role> resource of the Originator |  |
| Reference | TS-0003 7.4.2.2 - step 8 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT acting as a Role Repository and the Authorization Authority having assigned a role for the Originator at ROLE_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE1 |
                     | the IUT receives a RETRIEVE Request containing   | |
                     | From set to CSE1_ID and  | |
                     | To set to ROLE_RESOURCE_ADDRESS and   | |
                     | no Content  | |
                     |} | |

- +-----------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response containing  | IUT => CSE1 |
                     | Response Status Code set to 2000 (OK) and   | |
                     | Content containing  | |
                     | valid <role> resource representation  | |
                     |} | |
+———————-+—————————————————————————–+————-+

TP/oneM2M/CSE/SEC/ROL/RET/002

| TP Id | TP/oneM2M/CSE/SEC/ROL/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the <token> resource of the Originator |  |
| Reference | TS-0003 7.4.2.3 - step 8 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT acting as a Token Repository and the Authorization Authority having assigned a role for the Originator and the IUT having a token associated to the role of the Originator at TOKEN_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE1 |
                     | the IUT receives a RETRIEVE Request containing   | |
                     | From set to CSE1_ID and  | |
                     | To set to TOKEN_RESOURCE_ADDRESS and   | |
                     | no Content  | |
                     |} | |

- +--------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response containing  | IUT => CSE1 |
                     | Response Status Code set to 2000 (OK) and   | |
                     | Content containing  | |
                     | valid <token> resource representation  | |
                     |} | |
+———————-+————————————————————————–+————-+

TP/oneM2M/CSE/SEC/ROL/RET/003

| TP Id | TP/oneM2M/CSE/SEC/ROL/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <role> Retrieve request to the Role Repository after receiving a request on a resource protected by role based access control |  |
| Reference | TS-0003 7.4.3 - step 2 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on a nd the Authorization Authority having assigned a role for the Originator a nd the Authorization Authority having created a token associated to the role of the Originator a nd the IUT having a <container> resource at RESOURCE_TARGET_ADDRESS protected by role based access control a nd the IUThaving granted the Originator access to the <container> resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE1 |
                     | the IUT receives a valid RETRIEVE Request containing   | |
                     | From set to AE1_ID and  | |
                     | To set to RESOURCE_TARGET_ADDRESS and   | |
                     | Role ID parameter set to address the <role> resource assigned to the Originator  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid RETRIEVE Request to CSE1 containing  | IUT => CSE1 |
                     | From set to CSE_ID and  | |
                     | To set to Role ID and   | |
                     | no Content  | |
                     |} | |
+———————-+———————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/SEC/ROL/RET/004

| TP Id | TP/oneM2M/CSE/SEC/ROL/RET/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a <token> Retrieve request to the Token Repository after receiving a request on a resource protected by role based access control |  |
| Reference | TS-0003 7.4.3 - step 2 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the Authorization Authority having assigned a role for the Originator and the Authorization Authority having created a token associated to the role of the Originator and the IUT having a <container> resource at RESOURCE_TARGET_ADDRESS protected by role based access control procedure an d the IUThaving granted the Originator access to the <container> resource } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE1 |
                     | the IUT receives a valid RETRIEVE Request containing   | |
                     | From set to AE1_ID and  | |
                     | To set to RESOURCE_TARGET_ADDRESS and   | |
                     | Role ID parameter set to address the <role> resource assigned to the Originator  | |
                     | Token ID parameter set to address the <token> resource assigned to the role of the Originator  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid RETRIEVE Request to CSE1 containing  | IUT => CSE1 |
                     | From set to CSE_ID and  | |
                     | To set to Token_ID and   | |
                     | no Content  | |
                     |} | |
+———————-+————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ROL/RET/005

| TP Id | TP/oneM2M/CSE/SEC/ROL/RET/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns the result of a requeston a resource protected by role based access control, when the appropriate credentials are provided in the request |  |
| Reference | TS-0003 7.4.3 - step 7 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the Authorization Authority having assigned a role for the Originator and the Authorization Authority having created a token associated to the role of the Originator and the IUT having a <container> resource at RESOURCE_TARGET_ADDRESS protected by role based access control procedure and the IUThaving granted the Originator access to the <container> resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE1 |
                     | the IUT receives a valid RETRIEVE Request containing   | |
                     | From set to AE1_ID and  | |
                     | To set to RESOURCE_TARGET_ADDRESS and   | |
                     | Role ID parameter set to address the <role> resource assigned to the Originator  | |
                     | Token ID parameter set to address the <token> resource assigned to the role of the Originator  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a RETRIEVE Response containing  | IUT => AE1 |
                     | Response Status Code set to 2000 (OK) and   | |
                     | Content containing  | |
                     | <container> resource  | |
                     |} | |
+———————-+——————————————————————————————————————————————————-+————-+

7.2.2.10.4.2.3	UPDATE Operation

TP/oneM2M/CSE/SEC/ROL/UPD/001

| TP Id | TP/oneM2M/CSE/SEC/ROL/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the tokenLink attribute of the <role> resource after creating its associated <token> resource |  |
| Reference | TS-0003 7.4.2.3 - step 5 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on and the IUT acting as an Authorization Authority and the IUT having assigned a role for the Originator and the IUT having created a token associated to the role of the Originator at TOKEN_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE2 |
                     | the IUT receives a valid CREATE Response from CSE2 (TR) containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | <token> resource representation  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid UPDATE Request to CSE1 (RR) containing  | IUT => CSE1 |
                     | To set to ROLE_REPOSITORY_ADDRESS and  | |
                     | Resource Type set to 32 (token) and  | |
                     | Content containing  | |
                     | <token> resource representation containing  | |
                     | tokenLink attribute set to TOKEN_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+—————————————————————————————————————–+————-+

7.2.2.10.4.2.4	NOTIFY Operation

TP/oneM2M/CSE/SEC/ROL/NTF/001

| TP Id | TP/oneM2M/CSE/SEC/ROL/NTF/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT notifies the Originator about a token issuance associated to its role |  |
| Reference | TS-0003 7.4.2.3 - step 6 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being switched on the IUT being in the "initial state" and the IUT acting as an Authorization Authority and the IUT having assigned a role for the Originator and the IUT having created a token associated to the role of the Originator } |  |

- +---------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= CSE2 |
                     | the IUT receives a valid CREATE Response from CSE2 (TR) containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | <token> resource representation  | |
                     |} | |

- +---------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid NOTIFY Request to CSE1 (Originator) containing | IUT => CSE1 |
                     | Content containing  | |
                     | notificationEvent attribute containing  | |
                     | representation attribute containing  | |
                     | <token> resource representation  | |
                     |} | |
+———————-+—————————————————————————————+————-+

###### 7.2.2.10.4.3	DAS (DAS)

7.2.2.10.4.3.1	NOTIFY Operation

###### 7.2.2.10.4.4	ESCertKE (ESC)

7.2.2.10.4.4.1	NOTIFY Operation

TP/oneM2M/CSE/SEC/ESC/NTF/001

| TP Id | TP/oneM2M/CSE/SEC/ESC/NTF/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notify request with ESCertKE message 1 when triggered |  |
| Reference | TS-0003 8.7.2.2 - step C.1.a, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Terminating End-Point's Certificate Info & Identity } |  |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |NA |
                     | the IUT is triggered to send a valid NOTIFY Request containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 1 (TLS Message: Client Hello)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Request to CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE1 ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 1 (TLS Message: Client Hello)  | |
                     |} | |
+———————-+———————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ESC/NTF/002

| TP Id | TP/oneM2M/CSE/SEC/ESC/NTF/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notify response with ESCertKE message 2 when it receives a notify request with ESCertKE message 1 |  |
| Reference | TS-0003 8.7.2.2 - step C.1.b, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Initiating End-Point's Certificate Info & Identity } |  |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE1 |
                     | the IUT receives a valid NOTIFY Request from CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE1_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 1 (TLS  | |
                     | Message: Client Hello)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Response to CSE1 containing  | |
                     | To set to INITIATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 2 (TLS  | |
                     | Messages: Server Hello, Certificate*,  | |
                     | ServerKeyExchange, CertificateRequest, ServerHelloDone)  | |
                     |} | |
+———————-+———————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ESC/NTF/003

| TP Id | TP/oneM2M/CSE/SEC/ESC/NTF/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notify request with ESCertKE message 3 when it receives a notify response with ESCertKE message 2 |  |
| Reference | TS-0003 8.7.2.2 - step C.1.c, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Terminating End-Point's Certificate Info & Identity and the IUT having sent a notify request to the Terminating End-Point with ESCertKE Message 1 } |  |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE1 |
                     | the IUT receives a valid NOTIFY Response from CSE1 containing  | |
                     | To set to INITIATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE1_ID and  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 2 (TLS  | |
                     | Messages: Server Hello, Certificate*,  | |
                     | ServerKeyExchange, CertificateRequest,  | |
                     | ServerHelloDone)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Request to CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 3 (TLS  | |
                     | Messages: Certificate*, ClientKeyExchange,  | |
                     | CertificateVerify*, [ChangeCipherSpec], Finished)  | |
                     |} | |
+———————-+—————————————————————————————————————+————-+

TP/oneM2M/CSE/SEC/ESC/NTF/004

| TP Id | TP/oneM2M/CSE/SEC/ESC/NTF/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a notify response with ESCertKE message 4 when it receives a notify request with ESCertKE message 3 |  |
| Reference | TS-0003 8.7.2.2 - step C.1.d, TS-0001 11.4.3, TS-0004 6.3.5.48 |  |
| Config Id | CF04 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT is being registered and the IUT being switched on and the IUT having been provisioned with the corresponding private key and certificate and the IUT having configured Initiating End-Point's Certificate Info & Identity and the IUT having sent a notify request to the Initiating End-Point with ESCertKE Message 2 } |  |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE1 |
                     | the IUT receives a valid NOTIFY Request from CSE1 containing  | |
                     | To set to TERMINATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE1_ID and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 3 (TLS  | |
                     | Messages: Certificate*, ClientKeyExchange,  | |
                     | CertificateVerify*, [ChangeCipherSpec], Finished)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE1 |
                     | the IUT sends a valid NOTIFY Response to CSE containing  | |
                     | To set to INITIATING_ENDPOINT_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Contentcontaining  | |
                     | securityInfo element containing  | |
                     | securityInfoType element set to 6 (ESCertKE Message) and  | |
                     | escertkeMessage element containing  | |
                     | base64 representation of ESCertKE Message 4 (TLS  | |
                     | Messages: [ChangeCipherSpec], Finished)  | |
                     |} | |
+———————-+—————————————————————————————————————+————-+

#### 7.2.2.11	Resource Announcement (ANNC)

##### 7.2.2.11.1	BASIC Operation

###### TP/oneM2M/CSE/ANNC/001

| TP Id | TP/oneM2M/CSE/ANNC/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT provides confirmation of the announcement of a resource to the originator of the initiating request after a successful resource announcement procedure |  |
| Reference | TS-0001 [1], clause 10.2.18.1, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having created the PARENT_RESOURCE resource containing RESOURCE_TYPE resource and the IUT having announced the PARENT_RESOURCE resource and the AE having privileges to perform an OPERATION operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and The IUT having sent a valid CREATE Request containing To set REMOTE_CSE_RESOURCE_ADDRESSand Content containing RESOURCE_TYPE announced variant resourcecontaining MA (mandatory) attributes } |  |
| Expected behaviour | Test events | Direction |
|  | when { the IUT receives a valid Response from ANNC_TARGET_CSE containing Response Status Code set to 2001 (CREATED) containing valid RESOURCE_TYPE announced variant resource representation } | IUT <= CSE |
|  | then { The IUT sends a valid Response containing Response Status Code set to RESPONSE_STATUS_CODE and Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_RESOURCE_ADDRESS } | IUT => AE |

| TP Id | RESOURCE_TYPE | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/ANNC/001_ACP/UPD | 1 (accesControlPolicy) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_ACP/CRE | 1 (accesControlPolicy) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_CNT/UPD | 3 (container) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_CNT/CRE | 3 (container) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_GRP/UPD | 9 (group) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_GRP/CRE | 9 (group) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_LCP/UPD | 10 (locationPolicy) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_LCP/CRE | 10 (locationPolicy) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_MGO/UPD | 13 (mgmtObj) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_MGO/CRE | 13 (mgmtObj) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_NOD/UPD | 14 (node) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_NOD/CRE | 14 (node) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_SCH/UPD | 18 (schedule) | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/ANNC/001_SCH/CRE | 18 (schedule) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_CIN/CRE | 4 (contentInstance) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_TS/UPD | 29 (timeSeries) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_TS/CRE | 29 (timeSeries) | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/ANNC/001_TSI/CRE | 30 (timeSeriesInstance) | CREATE | 2001 (CREATED) |

##### 7.2.2.11.2	CREATE Operation

###### TP/oneM2M/CSE/ANNC/CRE/001

| TP Id | TP/oneM2M/CSE/ANNC/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the accessControlPolicyIDs attribute is announced when accessControlPolicyIDs attribute of the original resource is not present |  |
| Reference | TS-0001 [1], clause 10.2.18.4 and clause 9.6.26.3-1, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | no accessControlPolicyIDs attribute and  | |
                     | no anouncedAttribute attribute and  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to ANNC_PARENT_RESOURCE and  | |
                     | Content containing  | |
                     | containerAnnc resource containing  | |
                     | MA (mandatory) attributes and  | |
                     | valid accessControlPolicyIDs attribute  | |
                     |} | |
+———————-+—————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/ANNC/CRE/002

| TP Id | TP/oneM2M/CSE/ANNC/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and IUT having not been announced to announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the IUT having not been announced nor registered to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request containing Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ID and the IUT having sent a valid CREATE Request containing To set to ANNC_TARGET_CSE_ID and Content containing CSEBaseAnnc resource } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT => CSE |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |CSE => IUT |
                     | and the IUT sends a valid CREATE Request containing  | |
                     | To set to CSE_BASE_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/002_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/002_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/002_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/002_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/CRE/002_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/CRE/003

| TP Id | TP/oneM2M/CSE/ANNC/CRE/003 |  |
| --- | --- | --- |
| Test objective | Host CSE of original resource removes an announcement target CSE from the announceTo attribute if it does not return successful CREATE response for the announced resource (for a CREATE initiating resource announcement) |  |
| Reference | TS-0001 [1], clause 10.2.18.4 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created and announced the container resource and the AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having received a valid CREATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE-ID and Content containing contentInstance resource containing announceTo attribute set to ANNC_TARGET_CSE_ID and The IUT having sent a valid CREATE Request containing To set to REMOTE_CSE_RESOURCE_ADDRESS and Content containing contentInstanceAnnc resource containing MA (mandatory) attributes } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid Response from ANNC_TARGET_CSE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     |The IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)and  | |
                     | Content containing  | |
                     | contentInstance resourcecontaining  | |
                     | announceTo attribute set to EMPTH  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/ANNC/CRE/004

| TP Id | TP/oneM2M/CSE/ANNC/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided and its parent resource having been announced to the announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created and announced the PARENT_RESOURCE resource and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to REMOTE_CSE_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/004_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/004_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/004_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/CRE/004_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/004_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/004_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/004_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/004_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/CRE/004_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/CRE/005

| TP Id | TP/oneM2M/CSE/ANNC/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided and IUT having been already announced to announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having registered to the CSE and the IUT having created the PARENT_RESOURCE resource and the IUT having been announced to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to CSE_BASE_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resourcecontaining  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/005_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/005_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/005_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/005_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/005_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/005_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/005_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/CRE/005_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/CRE/006

| TP Id | TP/oneM2M/CSE/ANNC/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT removes successfully an entry from announceTo attribute of the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided when parent has not been announced at the announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created the PARENT_RESOURCE resource and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | The IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)and  | |
                     | Content containing  | |
                     | contentInstance resourcecontaining  | |
                     | announceTo attribute set to EMPTY  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/006_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/CRE/006_TSI | 30 (timeSeriesInstance) |

###### TP/oneM2M/CSE/ANNC/CRE/007

| TP Id | TP/oneM2M/CSE/ANNC/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo optional attribute is provided and IUT having not been announced to announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having create d the PARENT_RESOURCE resource and the IUT having not been announced nor registered to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS and the IUT having received a valid CREATE Request containing To set to PARENT_RESOURCE_ADDRESS Content containing RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ID and the IUT having sent a valid CREATE Request containing To set to ANNC_TARGET_CSE_ID and Content containing CSEBaseAnnc resource representation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to CSE_BASE_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/007_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/007_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/CRE/007_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/CRE/007_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/CRE/007_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/CRE/007_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/CRE/007_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/CRE/007_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/CRE/008

| TP Id | TP/oneM2M/CSE/ANNC/CRE/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource during its creation when announceTo and announcedAttribute optional attributes are provided and its parent resource having been announced to the announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.6, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | OA_ATTRIBUTE attribute set to VALUE and  | |
                     | announcedAttribute attribute set to OA_ATTRIBUTE and  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to REMOTE_CSE_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | MA (mandatory) attributes and  | |
                     | OA_ATTRIBUTE attribute set to VALUE  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/CRE/008_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/CRE/008_CIN/CNF | 4 (contentInstance) | contentInfo |
| TP/oneM2M/CSE/ANNC/CRE/008_GRP/MT | 9 (group) | memberType |
| TP/oneM2M/CSE/ANNC/CRE/008_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/CRE/008_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/CRE/008_NOD/HCL | 14 (node) | hostCSELink |
| TP/oneM2M/CSE/ANNC/CRE/008_SCH/SE | 18 (schedule) | scheduleElement |
| TP/oneM2M/CSE/ANNC/CRE/008_TS/MBS | 29 (timeSeries) | maxByteSize |
| TP/oneM2M/CSE/ANNC/CRE/008_TSI/DGT | 30 (timeSeriesInstance) | dataGenerationTime |

###### TP/oneM2M/CSE/ANNC/CRE/009

| TP Id | TP/oneM2M/CSE/ANNC/CRE/009 |  |
| --- | --- | --- |
| Test objective | Check that the labels attribute is announced when such attribute is present in the original resource |  |
| Reference | TS-0001 [1], clause 10.2.18.4 and 9.6.26.3-1, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created and announced the PARENT_RESOURCE to the announcement target CSE and the AE having privileges to perform CREATE operation on the resource PARENT_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | no anouncedAttribute attribute and  | |
                     | labels attribute set to VALUE and  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to ANNC_PARENT_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | containerAnnc resource containing  | |
                     | MA (mandatory) attributes and  | |
                     | labels attribute set to VALUE  | |
                     |} | |
+———————-+———————————————————————————————————+————-+

##### 7.2.2.11.3	RETRIEVE Operation

###### TP/oneM2M/CSE/ANNC/RET/001

| TP Id | TP/oneM2M/CSE/ANNC/RET/001 |  |  |
| --- | --- | --- | --- |
| Test objective | Host of <resourceAnnc> returns the <resourceAnnc> representation in a RETRIEVE response |  |  |
| Reference | TS-0001 [1], clause 10.2.18.2 |  |  |
| Parent Release | Release 3 |  |  |
| Config Id | CF02 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having RESOURCE_TYPE announced variant resource containing link attribute set to ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the TARGET_RESOURCE_ANNC_ADDRESS } |  |  |

- +-----------------+--------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ANNC_ADDRESS and  | |
                     | From set to AE-ID  | |
                     |} | |

- +-----------------+--------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource representation  | |
                     |} | |
+———————-+—————–+——————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/RET/001_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/RET/001_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/RET/001_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/RET/001_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/RET/001_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/RET/001_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/RET/001_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/RET/001_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/RET/001_TS | 29 (timeSeries) |
| TP/oneM2M/CSE/ANNC/RET/001_TSI | 30 (timeSeriesInstance) |

###### TP/oneM2M/CSE/ANNC/RET/002

| TP Id | TP/oneM2M/CSE/ANNC/RET/002 |  |
| --- | --- | --- |
| Test objective | Host of <resourceAnnc> returns the original <resource> representation from the original resource hosting CSE in a RETRIEVE response when result content is set to "original-resource" |  |
| Reference | TS-0001 [1], clause 10.2.18.2 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the CSE having created RESOURCE_TYPE and the IUT having RESOURCE_TYPE announced variant resource containing link attribute set to ORIGINAL_RESOURCE_ADDRESS and the AE having privileges to perform RETRIEVE operation on the TARGET_RESOURCE_ANNC_ADDRESS and the IUT having received a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ANNC_ADDRESS and From set to AE-ID and Result Content set to 7 (original-resource) and the IUT having sent a a valid RETRIEVE Request to CSE containing To set to ORIGINAL_RESOURCE_ADDRESS and From set to AE-ID } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource representation  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/RET/002_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/RET/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/RET/002_CIN | 4 (contentInstance) |
| TP/oneM2M/CSE/ANNC/RET/002_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/RET/002_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/RET/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/RET/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/RET/002_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/RET/002_TS | 29 (timeSeries) |
| TP/oneM2M/CSE/ANNC/RET/002_TSI | 30 (timeSeriesInstance) |

##### 7.2.2.11.4	UPDATE Operation

###### TP/oneM2M/CSE/ANNC/UPD/001

| TP Id | TP/oneM2M/CSE/ANNC/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and its parent resource having been announced to the announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having created and announce d the PARENT_RESOURCE resource to the announcement target CSE and the IUT having created a RESOURCE_TYPE resource at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ADDRESS  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set REMOTE_CSE_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resourcecontaining  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/001_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/001_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/001_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/001_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/001_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/001_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/001_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/UPD/001_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/UPD/002

| TP Id | TP/oneM2M/CSE/ANNC/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT initiates de-announcement of the RESOURCE_TYPE resource when a CSE-ID is deleted from the announceTo optional attribute |  |
| Reference | TS-0001 [1], clause 10.2.18.5, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to NULL  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid DELETE Request containing  | |
                     | To set to TARGET_ANNC_RESOURCE_ADDRESS  | |
                     |} | |
+———————-+——————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/002_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/002_GRP | 4 (group) |
| TP/oneM2M/CSE/ANNC/UPD/002_LCP | 9 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/002_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/UPD/002_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/UPD/003

| TP Id | TP/oneM2M/CSE/ANNC/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT provides confirmation of resource de-announcement to the originator of the initiating request after a successful resource de-announcement procedure originator (for an UPDATE initiating resource de-announcement) |  |
| Reference | TS-0001 [1], clause 10.2.18.3, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content attribute containing RESOURCE_TYPE resourcecontaining announceTo attribute set to NULL and the IUT having sent a valid DELETE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid Response from ANNC_TARGET_CSE containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT updates RESOURCE_TYPE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resourcecontaining  | |
                     | announceTo attribute set to NULL  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/003_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/003_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/003_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/003_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/003_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/003_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/003_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/UPD/003_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/UPD/004

| TP Id | TP/oneM2M/CSE/ANNC/UPD/004 |  |
| --- | --- | --- |
| Test objective | HOST CSE of original resource returns a valid response where value in announcedAttributes is removed if such attribute is not marked OA |  |
| Reference | TS-0001 [1], clause 10.2.18.1 and clause 10.2.18.4, TS-0004 [2], clause 7.3.1.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to the CSE and the IUT having registered the AE and the IUT having a resource of type RESOURCE_TYPE at TARGET_RESOURCE_ADDRESS containing announceTo attribute set to NULL announcedAttribute attribute set to NULL parentId attribute set to PARENT_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS and the resource at PARENT_RESOURCE_ADDRESS containing announceTo attribute set to ANNOUNCED_PARENT_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing RESOURCE_TYPE resource  | |
                     | containing  | |
                     | announceTo set to ANNC_TARGET_CSE_ID and  | |
                     | announcedAttributes attribute set to pi  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | announceTo attribute set to ANNOUNCED_RESOURCE_ADDRESS and  | |
                     | announcedAttributes set to empty  | |
                     |} | |
+———————-+————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/004_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/004_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/004_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/004_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/004_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/004_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/UPD/004_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/UPD/005

| TP Id | TP/oneM2M/CSE/ANNC/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that the labels attribute is de-announced when such attribute is deleted from the original resource |  |
| Reference | TS-0001 [1], clause 10.2.18.4 and clause 9.6.26.3-1, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having created a container resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and valid labels attribute and the IUT having announced the container resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | labels attribute set to NULL   | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | The IUT sends a valid UPDATE Request containing  | |
                     | To set to CONTAINER_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | containerAnnc resource containing  | |
                     | labels attribute set to NULL  | |
                     |} | |
+———————-+—————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/ANNC/UPD/006

| TP Id | TP/oneM2M/CSE/ANNC/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and IUT having been already announced to announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having registered to the CSE and the IUT having been announced to the announcement target CSE and the IUT having created the RESOURCE_TYPE resource and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to CSE_BASE_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resourcecontaining  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/006_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/006_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/006_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/006_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/006_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/006_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/006_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/UPD/006_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/UPD/007

| TP Id | TP/oneM2M/CSE/ANNC/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT announces successfully the RESOURCE_TYPE resource when a new CSE-ID is provided in announcedTo optional attribute and IUT has registered and created a <remoteCSE> resource to the announcement target CSE |  |
| Reference | TS-0001 [1], clause 10.2.18.4, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created the RESOURCE_TYPE resource and the IUT having registered and created a CSE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announceTo attribute set to ANNC_TARGET_CSE_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | The IUT sends a valid CREATE Request containing  | |
                     | To set to REMOTE_ANNC_TARGET_CSE_ID and  | |
                     | Content containing  | |
                     | CSEBaseAnnc resourcecontaining  | |
                     | MA (mandatory) attributes  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/007_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/007_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/007_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/007_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/007_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/007_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/007_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/UPD/007_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/UPD/008

| TP Id | TP/oneM2M/CSE/ANNC/UPD/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT provides confirmation of the announcement of an attribute to the originator of the initiating request after a successful attribute announcement procedure |  |
| Reference | TS-0001 [1], clause 10.2.18.6, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announceTo attribute set to ANNC_TARGET_CSE_ID and announcedAttribute attribute set to NULL and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to OA_ATTRIBUTE and the IUT having sent a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resourcecontaining OA_ATTRIBUTE attribute set to VALUE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid Response from CSE containing  | |
                     | Response Status Code set to 2004 (UPDATED) containing  | |
                     | valid RESOURCE_TYPE announced variant resource representation  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT updates the RESOURCE_TYPE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announcedAttribute attribute set to OA_ATTRIBUTE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/008_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/008_GRP/MNM | 9 (group) | maxNrOfMembers |
| TP/oneM2M/CSE/ANNC/UPD/008_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/008_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/008_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/008_SCH/SE | 18 (schedule) | scheduleElement |
| TP/oneM2M/CSE/ANNC/UPD/008_TS/MBS | 29 (timeSeries) | maxByteSize |

###### TP/oneM2M/CSE/ANNC/UPD/009

| TP Id | TP/oneM2M/CSE/ANNC/UPD/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully RESOURCE_TYPE announced variant resource when a new OA attribute is provided to announcedAttribute attribute of the original resource |  |
| Reference | TS-0001 [1], clause 10.2.18.8, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE and announceTo attribute set to ANNC_TARGET_CSE_ID and announcedAttribute attribute set to NULL and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announcedAttribute attribute set to OA_ATTRIBUTE  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid UPDATE Request containing  | |
                     | To set to TARGET_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | OA_ATTRIBUTE attribute set to VALUE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/009_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/009_GRP/MNM | 9 (group) | maxNrOfMembers |
| TP/oneM2M/CSE/ANNC/UPD/009_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/009_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/009_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/009_SCH/SE | 18 (schedule) | scheduleElement |
| TP/oneM2M/CSE/ANNC/UPD/009_TS/MBS | 29 (timeSeries) | maxByteSize |

###### TP/oneM2M/CSE/ANNC/UPD/010

| TP Id | TP/oneM2M/CSE/ANNC/UPD/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the RESOURCE_TYPE announced variant resource when a MA attribute of its original resource is updated |  |
| Reference | TS-0001 [1], clause 10.2.18.10, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created a RESOURCE_TYPE resource containing expirationTime attribute set to VALUE_1 and announceTo attribute set to ANNC_TARGET_CSE_ID and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | expirationTime attribute set to VALUE_2  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid UPDATE Request containing  | |
                     | To set to TARGET_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | expirationTime attribute set to VALUE_2  | |
                     |} | |
+———————-+———————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/010_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/010_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/UPD/010_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/UPD/010_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/UPD/010_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/UPD/010_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/UPD/010_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/UPD/010_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/UPD/011

| TP Id | TP/oneM2M/CSE/ANNC/UPD/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the RESOURCE_TYPE announced variant resource when a optional attribute present in the announcedAttribute attribute of its original resource is updated |  |
| Reference | TS-0001 [1], clause 10.2.18.10, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created a RESOURCE_TYPE resource containing OA_ATTRIBUTE attribute set to VALUE_1 and announceTo attribute set to ANNC_TARGET_CSE_ID and announcedAttribute attribute set to OA_ATTRIBUTE and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | OA_ATTRIBUTE attribute set to VALUE_2  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid UPDATE Request containing  | |
                     | To set to TARGET_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | OA_ATTRIBUTE attribute set to VALUE_2  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/011_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/011_GRP/MNM | 9 (group) | maxNrOfMembers |
| TP/oneM2M/CSE/ANNC/UPD/011_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/011_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/011_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/011_SCH/SE | 18 (schedule) | scheduleElement |
| TP/oneM2M/CSE/ANNC/UPD/011_TS/MBS | 29 (timeSeries) | maxByteSize |

###### TP/oneM2M/CSE/ANNC/UPD/012

| TP Id | TP/oneM2M/CSE/ANNC/UPD/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT de-announces successfully an announced attribute when such attribute is deleted from announcedAttribute attribute |  |
| Reference | TS-0001 [1], clause 10.2.18.9, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ID and announcedAttribute attribute set to OA_ATTRIBUTE and valid OA_ATTRIBUTE attribute and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | announcedAttribute attribute set to NULL  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid UPDATE Request containing  | |
                     | To set to TARGET_ANNC_RESOURCE_ADDRESS and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | OA_ATTRIBUTE attribute set to NULL  | |
                     |} | |
+———————-+———————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE |
| --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/012_CNT/MBS | 3 (container) | maxByteSize |
| TP/oneM2M/CSE/ANNC/UPD/012_GRP/MT | 9 (group) | memberType |
| TP/oneM2M/CSE/ANNC/UPD/012_LCP/LOI | 10 (locationPolicy) | locationContainerID |
| TP/oneM2M/CSE/ANNC/UPD/012_MGO/DC | 13 (mgmtObj) | description |
| TP/oneM2M/CSE/ANNC/UPD/012_NOD/HCL | 14 (node) | hostedCSELink |
| TP/oneM2M/CSE/ANNC/UPD/012_SCH/SE | 18 (schedule) | scheduleElement |
| TP/oneM2M/CSE/ANNC/UPD/012_TS/MBS | 29 (timeSeries) | maxByteSize |

###### TP/oneM2M/CSE/ANNC/UPD/013

| TP Id | TP/oneM2M/CSE/ANNC/UPD/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT provides confirmation of the de-announcement of an attribute to the originator of the initiating request after a successful attribute de-announcement procedure |  |
| Reference | TS-0001 [1], clause 10.2.18.7, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ID and announcedAttribute attribute set to OA_ATTRIBUTE and valid OA_ATTRIBUTE attribute and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE resource containing announcedAttribute attribute set to NULL and The IUT having sent a valid UPDATE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS and Content containing RESOURCE_TYPE announced variant resource containing OA_ATTRIBUTE attribute set to NULL } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid Response from CSE containing  | |
                     | Response Status Code set to 2004 (UPDATED) containing  | |
                     | RESOURCE_TYPE announced variant resource containing  | |
                     | no OA_ATTRIBUTE attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT updates the RESOURCE_TYPE resource  | |
                     | and the IUT sends valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | RESOURCE_TYPE resource containing  | |
                     | no announcedAttribute attribute  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE | OA_ATTRIBUTE | PICS |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/ANNC/UPD/013_CNT/MBS | 3 (container) | maxByteSize | PICS_IN_PROFILE |
| TP/oneM2M/CSE/ANNC/UPD/013_GRP/MNM | 9 (group) | maxNrOfMembers |  |
| TP/oneM2M/CSE/ANNC/UPD/013_LCP/LOI | 10 (locationPolicy) | locationContainerID |  |
| TP/oneM2M/CSE/ANNC/UPD/013_MGO/DC | 13 (mgmtObj) | description |  |
| TP/oneM2M/CSE/ANNC/UPD/013_NOD/HCL | 14 (node) | hostedCSELink |  |
| TP/oneM2M/CSE/ANNC/UPD/013_SCH/SE | 18 (schedule) | scheduleElement |  |
| TP/oneM2M/CSE/ANNC/UPD/013_TS/MBS | 29 (timeSeries) | maxByteSize |  |

###### TP/oneM2M/CSE/ANNC/UPD/014

| TP Id | TP/oneM2M/CSE/ANNC/UPD/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT removes an announcement target CSE from the announceTo attribute if it does not return successful CREATE response for the announced resource (for an UPDATE initiating resource announcement) |  |
| Reference | TS-0001 [1], clause 10.2.18.4 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the CSE and the IUT having created the container resource and the AE having privileges to perform UPDATE operation on the resource CONTAINER_RESOURCE_ADDRESS and the IUT having received a valid UPDATE Request from AE containing To set to CONTAINER_RESOURCE_ADDRESS and From set to AE-ID and Content containing container resource containing announceTo attribute set to ANNC_TARGET_CSE_ID and the IUT having created a CSEBaseAnnc at CSE_BASE_ANNC_RESOURCE_ADDRESS and The IUT having sent a valid CREATE Request containing To set to CSE_BASE_ANNC_RESOURCE_ADDRESS and Content containin g containerAnnc resource containing MA (mandatory) attributes } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid Response from CSE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT updates the container resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)and  | |
                     | Content containing  | |
                     | container resourcecontaining  | |
                     | announceTo attribute empty  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————+————-+

##### 7.2.2.11.5	DELETE Operation

###### TP/oneM2M/CSE/ANNC/DEL/001

| TP Id | TP/oneM2M/CSE/ANNC/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT provides confirmation of resource de-announcement to the originator of the initiating request after a successful resource de-announcement procedure (for a DELETE initiating resource de-announcement) |  |
| Reference | TS-0001 [1], clause 10.2.18.3, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS and the IUT having received a valid DELETE Request from AE containing To set to TARGET_RESOURCE_ADDRESS and the IUT having sent a valid DELETE Request containing To set to TARGET_ANNC_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE |
                     | the IUT receives a valid Response from ANNC_TARGET_CSE  | |
                     | containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT deletes the RESOURCE_TYPE resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/DEL/001_ACP | 1 (accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/001_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/DEL/001_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/DEL/001_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/001_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/DEL/001_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/DEL/001_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/DEL/001_TS | 29 (timeSeries) |

###### TP/oneM2M/CSE/ANNC/DEL/002

| TP Id | TP/oneM2M/CSE/ANNC/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT deletes all announced resources when the original resource is deleted |  |
| Reference | TS-0001 [1], clause 9.6.26.1 and clause 10.2.18.5, TS-0004 [2], clause 7.3.3.10 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having registered to the remoteCSE and the IUT having created a RESOURCE_TYPE resource containing announceTo attribute set to ANNC_TARGET_CSE_ADDRESS and the IUT having announced the RESOURCE_TYPE resource to the announcement target CSE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => CSE |
                     | The IUT sends a valid DELETE Request containing  | |
                     | To set to TARGET_RESOURCE_ANNC_ADDRESS  | |
                     |} | |
+———————-+————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/ANNC/DEL/002_ACP | 1(accesControlPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/002_CNT | 3 (container) |
| TP/oneM2M/CSE/ANNC/DEL/002_GRP | 9 (group) |
| TP/oneM2M/CSE/ANNC/DEL/002_LCP | 10 (locationPolicy) |
| TP/oneM2M/CSE/ANNC/DEL/002_MGO | 13 (mgmtObj) |
| TP/oneM2M/CSE/ANNC/DEL/002_NOD | 14 (node) |
| TP/oneM2M/CSE/ANNC/DEL/002_SCH | 18 (schedule) |
| TP/oneM2M/CSE/ANNC/DEL/002_TS | 29 (timeSeries) |

#### 7.2.2.12	3GPP interworking (3GPP)

##### 7.2.2.12.1	BASIC Operation

###### TP/oneM2M/CSE/3GPP/TRIG/001

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an explicit 3GPP Device Trigger Request when it receives a valid <triggerRequest> from an A |  |
| Reference | TS-0026 [5], clause 7.5.1, step 3a |  |
| Parent Release | Release 3 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE1 and the AE1being hosted on a 3GPP UE and the IUT allowing to register an AE2 } |  |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid CREATE request from AE2 containing  | |
                     | To set to AE2_RESOURCE_ADDRESS and  | |
                     | From set to AE2_ID and  | |
                     | Resource Type set to 38 (triggerRequest) and  | |
                     | Content containing  | |
                     | triggerRequest resource containing  | |
                     | triggerPurpose set to TRIGGER_PURPOSE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT send s a Request to the SCEF containing  | |
                     | 3GPP Device Trigger Request message containing  | |
                     | A valid 3GPP Device Trigger Request containing  | |
                     | triggerPayload attribute  | |
                     |} | |
+———————-+———————————————————————————————————-+————-+

| TP Id | TRIGGER_PURPOSE |
| --- | --- |
| TP/oneM2M/CSE/3GPP/TRIG/001_CON | establishConnection |
| TP/oneM2M/CSE/3GPP/TRIG/001_REQ | registrationRequest |
| TP/oneM2M/CSE/3GPP/TRIG/001_EXE | executeCRUD |
| TP/oneM2M/CSE/3GPP/TRIG/001_ENR | enrolmentRequest |

###### TP/oneM2M/CSE/3GPP/TRIG/002

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an implicit 3GPP Device Trigger Request when it receives a valid <notification> from an AE |  |
| Reference | TS-0026 [5], clause 7.5.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE1 and the AE1being hosted on a 3GPP UE and the AE1 having subscribed to CONTAINER_RESOURCE_ADDRESS and the AE1 being not reachable and the IUT allowing to register an AE2 } |  |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives the UPDATE request from the AE2 containing  | |
                     | To set to CONTAINER_RESOURCE_ADDRESS containing  | |
                     | From set to AE2_ID and  | |
                     | Content containing  | |
                     | container resource containing  | |
                     | valid labels attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT send s a Request to the SCEF containing  | |
                     | 3GPP Device Trigger Request message containing  | |
                     | A valid 3GPP Device Trigger Requestcontaining  | |
                     | triggerPayload_ attribute set to NULL  | |
                     |} | |
+———————-+———————————————————————————————————+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/003

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the triggerStatus attribute of the <triggerRequest> to ERROR_NSE_NOT_FOUND when the SCEF does not respond to the Device Trigger Request prior to the timeout period |  |
| Reference | TS-0026 [5], clause 7.5.1, step 3 |  |
| Parent Release | Release 3 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE1 and the AE1being hosted on a 3GPP UE and the IUT allowing to register an AE2 and the AE2 having created a <triggerRequest> resource and the IUT having sent a 3GPP Device Trigger Request } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT does not receive a reponse from SCEF  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT |
                     | the IUT updates <triggerRequest> resource containing  | |
                     | triggerStatus attribute set to ERROR_NSE_NOT_FOUND  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/004

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the triggerStatus attribute of the <triggerRequest> to the proper value when the SCEF responds to the Device Trigger Request |  |
| Reference | TS-0026 [5], clause 7.5.1, step 3b |  |
| Parent Release | Release 3 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE1 and the AE1being hosted on a 3GPP UE and the IUT allowing to register an AE2 and AE2 having created a TRIGGER_REQUEST and the IUT having sent a POST 3GPP Device Trigger Request } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= SCEF |
                     | the IUT receives a valid Device Triggering response from SCEF | |
                     | containing  | |
                     | deliveryResult set to SCEF_RESULT  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT sets triggerStatus attribute of TRIGGER_REQUEST to | |
                     | TRIGGER_RESULT  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

| TP Id | SCEF_RESULT | TRIGGER_RESULT |
| --- | --- | --- |
| TP/oneM2M/CSE/3GPP/TRIG/004_1 | SUCCESS | TRIGGER-DELIVERED |
| TP/oneM2M/CSE/3GPP/TRIG/004_2 | UNKNOWN | TRIGGER-FAILED |
| TP/oneM2M/CSE/3GPP/TRIG/004_3 | FAILURE | TRIGGER-FAILED |
| TP/oneM2M/CSE/3GPP/TRIG/004_4 | TRIGGERED | TRIGGER-TRIGGERED |
| TP/oneM2M/CSE/3GPP/TRIG/004_5 | EXPIRED | TRIGGER-EXPIRED |
| TP/oneM2M/CSE/3GPP/TRIG/004_6 | UNCONFIRMED | TRIGGER-UNCONFIRMED |
| TP/oneM2M/CSE/3GPP/TRIG/004_7 | REPLACED | TRIGGER-FAILED |
| TP/oneM2M/CSE/3GPP/TRIG/004_8 | TERMINATE | TRIGGER-TERMINATED |

###### TP/oneM2M/CSE/3GPP/TRIG/005

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/005 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT returns a HTTP response when it receives a valid DeviceTriggeringDeliveryReportNotification Report message |  |  |
| Reference | TS-0026 [5], clause 7.5.1, step 6 and 7 |  |  |
| Parent Release | Release 3 |  |  |
| Config Id | CFG05 |  |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE1 and the AE1being hosted on a 3GPP UE and the IUT allowing to register an AE2 and AE2 having created a TRIGGER_REQUEST and the IUT having sent a POST 3GPP Device Trigger Request } |  |  |

- +--------+----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= SCEF |
                     | the IUT receives a valid DeviceTriggeringDeliveryReportNotification  | |
                     | request from SCEF containing  | |
                     | deliveryResult set to SCEF_RESULT  | |
                     |} | |

- +--------+----------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT sets triggerStatus attribute of TRIGGER_REQUEST to  | |
                     | TRIGGER_RESULT  | |
                     | and the IUT sends a valid HTTP response containing  | |
                     | Response Code set to 204 (NO_CONTENT)  | |
                     |} | |
+———————-+——–+—————————————————————————————————————-+————-+

| TP Id | SCEF_RESULT | TRIGGER_RESULT |
| --- | --- | --- |
| TP/oneM2M/CSE/3GPP/TRIG/005_1 | SUCCESS | TRIGGER-DELIVERED |
| TP/oneM2M/CSE/3GPP/TRIG/005_2 | UNKNOWN | TRIGGER-FAILED |
| TP/oneM2M/CSE/3GPP/TRIG/005_3 | FAILURE | TRIGGER-FAILED |
| TP/oneM2M/CSE/3GPP/TRIG/005_4 | TRIGGERED | TRIGGER-TRIGGERED |
| TP/oneM2M/CSE/3GPP/TRIG/005_5 | EXPIRED | TRIGGER-EXPIRED |
| TP/oneM2M/CSE/3GPP/TRIG/005_6 | UNCONFIRMED | TRIGGER-UNCONFIRMED |
| TP/oneM2M/CSE/3GPP/TRIG/005_7 | REPLACED | TRIGGER-FAILED |
| TP/oneM2M/CSE/3GPP/TRIG/005_8 | TERMINATE | TRIGGER-TERMINATED |

###### TP/oneM2M/CSE/3GPP/TRIG/006

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an explicit Device Trigger Replace Request when it receives a valid Device Trigger Replace Request from an AE |  |
| Reference | TS-0001 [1], clause 8.3.3.2.2 and clause 9.6.49, TS-0026 [5], clause 7.5.2 |  |
| Parent Release | Release 4 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE2 and the AE2being hosted on a 3GPP UE and the IUT allowing to register an AE1 and AE1 having created a TRIGGER_REQUEST and the IUT having a <triggerRequest> resource at TARGET_RESOURCE_ADDRESS and the IUT having sent a POST 3GPP Device Trigger Request containing triggerReference and the IUT having not received a Response for Device Trigger Request } |  |

- +-------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid UPDATE request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => SCEF |
                     | the IUT assigns a new triggerReference for the new trigger  | |
                     | message  | |
                     | and the IUT sends a HTTP PUT 3GPP Request to the SCEF  | |
                     | targeting the correct transactionID containing  | |
                     | 3GPP Device Trigger Request message containing  | |
                     | a valid 3GPP Device Replace Request with updated  | |
                     | triggerReference  | |
                     |} | |
+———————-+————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/007

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an explicit Device Trigger Recall Request when it receives a valid Device Trigger Recall Request from an AE |  |
| Reference | TS-0001 [1], clause 8.3.3.2 and clause 9.6.49, TS-0026 [5], clause 7.5.2 |  |
| Parent Release | Release 4 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE2 and the AE2being hosted on a 3GPP UE and the IUT allowing to register an AE1 and AE1 having created a TRIGGER_REQUEST and the IUT having a <triggerRequest> resource at TARGET_RESOURCE_ADDRESS and the IUT having sent a POST 3GPP Device Trigger Request and the IUT having not received a Response for Device Trigger Request } |  |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid DELETE request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT sends a HTTP DELETE 3GPP Request to the SCEF  | |
                     | targeting the correct transactionID with NULL payload  | |
                     |} | |
+———————-+———————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/008

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/008 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT sends an implicit (initiated by IN-CSE) Device Trigger Replace Request when it determines there is a need for a Replace based on internal policies |  |  |
| Reference | TS-0001 [1], clause 8.3.3.2.2 and clause 9.6.49, TS-0026 [5], clause 7.5.2 |  |  |
| Parent Release | Release 4 |  |  |
| Config Id | CF05 |  |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |  |
| Initial conditions | with { the IUT being in the "initial state" and IUT having created a TRIGGER_REQUEST for TARGET_RESOURCE and TARGET_RESOURCE (AE/MN-CSE/ADN-AE)being hosted on a 3GPP UE and the IUT having a <triggerRequest> resource and the IUT having sent a POST 3GPP Device Trigger Request for TARGET_RESOURCE containing triggerReference and the IUT having not received a Response for Device Trigger Request } |  |  |

- +-----+--------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT generates a UPDATE <triggerRequest> message after  | |
                     | determining a need for a Trigger Request Replace based on internal  | |
                     | policies  | |
                     |} | |

- +-----+--------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT assigns a new triggerReference for the new trigger  | |
                     | message  | |
                     | and the IUT sends a HTTP PUT 3GPP Request to the SCEF  | |
                     | targeting the correct transactionID containing  | |
                     | 3GPP Device Trigger Request message containing  | |
                     | a valid 3GPP Device Replace Request with updated  | |
                     | triggerReference  | |
                     |} | |
+———————-+—–+——————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/009

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends an implicit (initiated by IN-CSE) Device Trigger Recall Request when it determines there is a need for a Recall based on internal policies |  |
| Reference | TS-0001 [1], clause 8.3.3.2 and clause 9.6.49, TS-0026 [5], clause 7.5.2 |  |
| Parent Release | Release 4 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and IUT having created a TRIGGER_REQUEST for TARGET_RESOURCE and TARGET_RESOURCE (AE/MN-CSE/ADN-AE) being hosted on a 3GPP UE and the IUT having a <triggerRequest> resource and the IUT having sent a POST 3GPP Device Trigger Request for TARGET_RESOURCE containing triggerReference and the IUT having not received a Response for Device Trigger Request } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT generates a DELETE <triggerRequest> message after  | |
                     | determining a need for a Trigger Request Recall based on  | |
                     | internal policies  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT sends a HTTP DELETE 3GPP Request to the  | |
                     | SCEF targeting the correct transactionID with NULL payload  | |
                     |} | |
+———————-+——————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/010

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the triggerStatus attribute of the <triggerRequest> to ERROR_NSE_NOT_FOUND when the SCEF does not respond to the Device Trigger Recall/Replace Request prior to the timeout period |  |
| Reference | TS-0026 [5], clause 7.5.2, step 3 |  |
| Parent Release | Release 4 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and IUT having created a TRIGGER_REQUEST for TARGET_RESOURCE and TARGET_RESOURCE (AE/MN-CSE/ADN-AE) being hosted on a 3GPP UE and the IUT having a <triggerRequest> resource and the IUT having sent a 3GPP Device Trigger Replace/Recall Request } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT does not receive a response from SCEF for the 3GPP  | |
                     | Device Trigger Replace/Recall Request  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT |
                     | the IUT updates <triggerRequest> resource containing  | |
                     | triggerStatus attribute set to ERROR_NSE_NOT_FOUND  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/011

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the triggerStatus attribute of the <triggerRequest> to the proper value and generates an appropriate response to AE when the SCEF responds to the Device Trigger Replace Request |  |
| Reference | TS-0004 [2], clause 6.6.3 and clause 7.4.57.2.3, TS-0026 [5], clause 7.5.2 and clause 8.3.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE2 and the AE2being hosted on a 3GPP UE and the IUT allowing to register an AE1 and AE1 having initiated a TRIGGER_REQUEST and the IUT having a <triggerRequest> resource and the IUT having sent a PUT 3GPP Device Trigger Replace Request } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= SCEF |
                     | the IUT receives a valid Device Triggering response from SCEF  | |
                     | containing  | |
                     | SCEF_RESULT_RESPONSE_CODE  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sets triggerStatus attribute of a <triggerRequest> resource  | |
                     | to TRIGGER_RESULT  | |
                     | and the IUT sends a valid response to AE1  | |
                     | containing  | |
                     | Response Status Code set to TRIGGER_RESPONSE_CODE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————+————-+

| TP Id | SCEF_RESULT_RESPONSE_CODE | TRIGGER_RESULT | TRIGGER_RESPONSE_CODE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/3GPP/TRIG/011_1 | 200 OK( deliveryResult set to REPLACED) | TRIGGER-REPLACED | 2004 (UPDATED) |
| TP/oneM2M/CSE/3GPP/TRIG/011_2 | 400 Bad Request | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_3 | 401 Unauthorized | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_4 | 403 Forbidden | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_5 | 404 Not Found | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_6 | 411 Length Required | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_7 | 413 Payload Too Large | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_8 | 415 Unsupported Media Type | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_9 | 429 Too Many Requests | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_10 | 500 Internal Server Error | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/011_11 | 503 Service Unavailable | TRIGGER-FAILED | 5219 (UNABLE_TO_REPLACE_REQUEST) |

###### TP/oneM2M/CSE/3GPP/TRIG/012

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT deletes the <triggerRequest> resource and generates an appropriate response to AE when it receives a successful trigger recall response from SCEF |  |
| Reference | TS-0004 [2], clause 6.6.3 and clause 7.4.57.2.4, TS-0026 [5], clause 7.5.2 |  |
| Parent Release | Release 4 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE2 and the AE2being hosted on a 3GPP UE and the IUT allowing to register an AE1 and AE1 having created a TRIGGER_REQUEST and the IUT having a <triggerRequest> resource and the IUT having sent a POST 3GPP Device Trigger Request and the IUT having not received a Response for Device Trigger Request and the AE1 having sent a DELETE <triggerRequest> request to the IUT and the IUT having sent a DELETE 3GPP Device Trigger Recall Request } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= SCEF |
                     | the IUT receives a valid Device Triggering response from SCEF/ | |
                     | containing  | |
                     | Response Status Code set to 204 (NO CONTENT)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT deletes the <triggerRequest> resource  | |
                     | and the IUT sends a valid response to AE1  | |
                     | containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/3GPP/TRIG/013

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not delete the <triggerRequest> resource and generates an appropriate response to AE when it receives an unsuccessful trigger recall response from SCEF |  |
| Reference | TS-0004 [2], clause 6.6.3 and clause 7.4.57.2.4, TS-0026 [5], clause 7.5.2, 8.3.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE2 and the AE2being hosted on a 3GPP UE and the IUT allowing to register an AE1 and AE1 having created a TRIGGER_REQUEST and the IUT having a <triggerRequest> resource and the IUT having sent a POST 3GPP Device Trigger Request and the IUT having not received a Response for Device Trigger Request and the AE1 having sent a DELETE <triggerRequest> request to the IUT and the IUT having sent a DELETE 3GPP Device Trigger Recall Request } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= SCEF |
                     | the IUT receives a valid Device Triggering response from SCEF  | |
                     | containing  | |
                     | Response Status Code set to SCEF_ERROR_CODE  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid response to AE1  | |
                     | containing  | |
                     | Response Status Code set to TRIGGER RESPONSE_CODE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————+————-+

| TP Id | SCEF _ERROR_CODE | TRIGGER_RESPONSE_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/3GPP/TRIG/013_1 | 400 Bad Request | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_2 | 401 Unauthorized | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_3 | 403 Forbidden | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_4 | 404 Not Found | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_5 | 411 Length Required | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_6 | 413 Payload Too Large | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_7 | 415 Unsupported Media Type | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_8 | 429 Too Many Requests | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_9 | 500 Internal Server Error | 5220 (UNABLE_TO_RECALL_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/013_10 | 503 Service Unavailable | 5220 (UNABLE_TO_RECALL_REQUEST) |

###### TP/oneM2M/CSE/3GPP/TRIG/014

| TP Id | TP/oneM2M/CSE/3GPP/TRIG/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not accept the request for Trigger Replace and Recall operation if the original Device Trigger request has already been processed and responds with an error |  |
| Reference | TS-0001 [1], clause 9.6.49, TS-0004 [2], clause 6.6.3.6 and clause 7.4.57.2, TS-0026 [5], clause 7.5.2 and clause 8.3.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CFG05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT allowing to register an AE2 and the AE2being hosted on a 3GPP UE and the IUT allowing to register an AE1 and AE1 having created a TRIGGER_REQUEST and the IUT having a <triggerRequest> resource at TARGET_RESOURCE_ADDRESS containing triggerStatus not set to PROCESSING and the IUT having sent a POST 3GPP Device Trigger Request and the IUT having received a Response for Device Trigger Request and the IUT having processed the Device Trigger Request } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid OPERATION request from AE1  | |
                     | containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid response to AE1  | |
                     | containing  | |
                     | Response Status Code set to ERROR_RESPONSE_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION | ERROR_RESPONSE_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/3GPP/TRIG/014_1 | UPDATE | 5219 (UNABLE_TO_REPLACE_REQUEST) |
| TP/oneM2M/CSE/3GPP/TRIG/014_2 | DELETE | 5220 (UNABLE_TO_RECALL_REQUEST) |

##### 7.2.2.12.2	SCEF-based NIDD

###### TP/oneM2M/CSE/3GPP/NIDD/001

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends NIDD configuration request when it receives create request of <serviceSubscribedNode> resource associated with a UE hosting an ADN-AE with niddRequired attribute set to TRUE. |  |
| Reference | TS-0026 [5], clause 7.1.1.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_IN_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE request from AE  | |
                     | containing To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Typeset to 20(serviceSubscribedNode) and  | |
                     | Content containing valid serviceSubscribedNode resource  | |
                     | representation containing niddRequired attribute set to TRUE  | |
                     |and nodeID attribute set to M2M-Ext-ID of UE hosting an AND-AE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT sends a valid Request to the SCEF  | |
                     | containing NIDD configuration Request message containing  | |
                     | a valid NIDD configuration Request  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/002

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT issues NIDD configuration request when it receives update request of <serviceSubscribedNode> resource associated with a UE hosting an ADN-AE to set niddRequired attribute TRUE. |  |
| Reference | TS-0026 [5], clause 7.1.1.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <serviceSubscribedNode> resource at TARGET_RESOURCE_ADDRESS containing niddRequired attribute set to False and nodeID attribute set to M2M-Ext-ID of UE hosting an AND-AE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE request from AE  | |
                     | containing To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing valid serviceSubscribedNode resource  | |
                     | representation containing niddRequired attribute set to TRUE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT send s a valid Request to the SCEF  | |
                     | containing NIDD configuration Request message containing  | |
                     | A NIDD configuration Request  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/003

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT issues NIDD configuration delete request when it receives update request of <serviceSubscribedNode> resource associated with a UE hosting an ADN-AE to set niddRequired attribute False. |  |
| Reference | TS-0026 [5], clause 7.1.1.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_IN_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <serviceSubscribedNode> resource at TARGET_RESOURCE_ADDRESS containing niddRequired_ attribute set to True and nodeID attribute set to M2M-Ext-ID of UE hosting an AND-AE and the AE having privileges to perform UPDATE operation on the resource TARGET_RESOURCE_ADDRESS and IUT having established NIDD configuration for M2M-Ext-ID } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE request from AE  | |
                     | containing To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing valid serviceSubscribedNode resource  | |
                     | representation containing niddRequired attribute set to  | |
                     | FALSE  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT send s a valid Request to the SCEF  | |
                     | containing  | |
                     | NIDD configuration delete Request message containing  | |
                     | A NIDD configuration delete Request  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/004

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT issues NIDD configuration delete request when it receives delete request of <serviceSubscribedNode> resource associated with a UE hosting an ASN/MN-CSE or ADN-AE. |  |
| Reference | TS-0026 [5], clause 7.1.1.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_IN_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <serviceSubscribedNode> resource at TARGET_RESOURCE_ADDRESS containing niddRequired attribute set to True and nodeID attribute set to M2M-Ext-ID of UE hosting an AND-AE and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS IUT having established NIDD configuration for M2M-Ext-ID } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE request from AE  | |
                     | containing To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT send s a valid Request to the SCEF  | |
                     | containing  | |
                     | NIDD configuration delete Request message containing  | |
                     | A NIDD configuration delete Request  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/005

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT issues SCEF-based Mobile Terminated (MT) NIDD Downlink Data Transfer Request if the size of the oneM2M request primitive to be sent in the MT NIDD Request is less than or equal to the maximumPacketSize defined in the NIDD Configuration response. |  |
| Reference | TS-0026 [5], clause 7.1.1.2 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_IN_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having done successful NIDD Configuration Request for the targeted ADN-AE hosted on a UE and size of the oneM2M request primitive to be sent in the MT NIDD Request is less than or equal to the maximumPacketSize defined in the NIDD Configuration response. and the IUT having registered the IN-AE and the IUT having registered the AE1 hosted on AND-AE containing valid M2M-Ext-ID attribute and the IN-AE having created the container resource CONTAINER_RESOURCE_ADDRESS containing subscription child resource containing notificationURI attribute set to AE1 and notificationEventType attribute set to 3 and the IN-AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= IN-AE |
                     | the IUT receives a valid CREATE request from IN-AE  | |
                     | containing To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to IN-AE and  | |
                     | Content containing valid contentInstance resource  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT send s a valid MT NIDD Downlink Data Transfer Request to  | |
                     |the SCEF  | |
                     | containing   | |
                     |Non-IP data containing oneM2M request primitive containing Content containing valid Notification  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/006

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not issue SCEF-based Mobile Terminated (MT) NIDD Downlink Data Transfer Request if the size of the oneM2M request primitive to be sent in the MT NIDD Request is more than the maximumPacketSize defined in the NIDD Configuration response. |  |
| Reference | TS-0026 [5], clause 7.1.1.2 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_IN_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having done successful NIDD Configuration Request for the targeted ADN-AE hosted on a UE and size of the oneM2M request primitive to be sent in the MT NIDD Request is more than the maximumPacketSize defined in the NIDD Configuration response. and the IUT having registered the IN-AE and the IUT having registered the AE1 hosted on AND-AE containing valid M2M-Ext-ID attribute and the IN-AE having created the container resource CONTAINER_RESOURCE_ADDRESS containing subscription child resource containing notificationURI attribute set to AE1 and notificationEventType attribute set to 3 and the IN-AE having privileges to perform CREATE operation on the resource CONTAINER_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= IN-AE |
                     | the IUT receives a valid CREATE request from IN-AE  | |
                     | containing To set to CONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to IN-AE and  | |
                     | Content containing valid contentInstance resource  | |
                     | } | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   | |
                     | the IUT does not send a MT NIDD Downlink Data Transfer Request to the SCEF  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/007

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT issues SCEF-based Mobile Terminated (MT) NIDD Downlink Data Transfer Request if the size of the oneM2M response primitive to be sent in the MT NIDD Request is less than or equal to the maximumPacketSize defined in the NIDD Configuration response. |  |
| Reference | TS-0026 [5], clause 7.1.1.2 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_IN_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having done successful NIDD Configuration Request for the targeted ADN-AE hosted on a UE and size of the oneM2M response primitive to be sent in the MT NIDD Request is less than or equal to the maximumPacketSize defined in the NIDD Configuration response. } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE1 |
                     | the IUT receives a valid MO(Mobile Originated) request from AE1  | |
                     | containing Non-IP data containing valid oneM2M request containing To set to AE1 and  | |
                     | From set to AE1 and  | |
                     | Content containing valid AE resource  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT send s a valid MT NIDD Downlink Data Transfer  | |
                     | Request to the SCEF  | |
                     | containing   | |
                     |Non-IP data containing valid oneM2M response primitive  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/008

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/008 |  |
| --- | --- | --- |
| Test objective | Check that IUT responds with a 204 NO CONTENT acknowledging the notification when SCEF sends MT NIDD Downlink Data Delivery Status Notification. |  |
| Reference | TS-0026 [5], clause 7.1.1.2, step 7 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered AE1 hosted on a 3GPP UE and the IUT having sent a MT Downlink data transfer request } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= SCEF |
                     | the IUT receives a valid MT NIDD Downlink Data Delivery  | |
                     | Status Notificationfrom SCEF containing valid  | |
                     | NiddDownlinkDataDeliveryStatusNotification  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT sends a 204 NO CONTENT acknowledging the  | |
                     | notification  | |
                     |} | |
+———————-+———————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/3GPP/NIDD/009

| TP Id | TP/oneM2M/CSE/3GPP/NIDD/009 |  |
| --- | --- | --- |
| Test objective | Check that IUT responds with a 204 NO CONTENT acknowledging the notification when SCEF sends MO NIDD Uplink Data Notification. |  |
| Reference | TS-0026 [5], clause 7.1.1.2, step 7 |  |
| Parent Release | Release 3 |  |
| Config Id | CF05 |  |
| PICS Selection | PICS_IN_CSE, PICS_3GPP |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having register an AE1 hosted on a 3GPP UE } |  |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= SCEF |
                     | the IUT receives a valid MO NIDD Uplink Data  | |
                     | Notificationfrom SCEF  | |
                     | containing NiddUplinkDataNotification  | |
                     | containing Non_IP data containing a valid oneM2M  | |
                     | response primitive  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => SCEF |
                     | the IUT sends a 204 NO CONTENT acknowledging the  | |
                     | notification  | |
                     |} | |
+———————-+—————————————————————————————————————–+————-+

#### 7.2.2.13	FlexContainer (FLXC)

##### 7.2.2.13.1	CREATE Operation

###### TP/oneM2M/CSE/FLXC/CRE/001

| TP Id | TP/oneM2M/CSE/FLXC/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of the <flexContainer> resource when the resource representation does not comply with the schema definition. |  |
| Reference | TS-0001 [1], clause 3, 10.2.4.16, TS-0004 [2], clause 7.4.29.2.1 |  |
| Parent Release | Release 2 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource AE_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives an invalid CREATE request from AE containing  | |
                     | To set to AE_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Resource Type set to 28 (flexContainer) and  | |
                     | Content containing  | |
                     | an invalid resource representation  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the flexContainer resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/FLXC/CRE/002

| TP Id | TP/oneM2M/CSE/FLXC/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the CREATE request of <flexContainer> resource when the containerDefinition attribute does not include the schema value to validate the received resource representation |  |
| Reference | TS-0001 [1], clause 10.1.2 and 10.2.4.16, TS-0004 [2], clause 7.4.37.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE the AEhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | flexContainer resource containing  | |
                     | containerDefinition set to UNAVAILABLE_SCHEMA  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT does not create the flexContainer resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4125 (SPECIALIZATION_SCHEMA_NOT_FOUND)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/FLXC/CRE/003

| TP Id | TP/oneM2M/CSE/FLXC/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the CREATE request of <flexContainer> resource when value of contentSize attribute is greater than maxByteSize attribute |  |
| Reference | TS-0001 [1], clause 10.1.2 and 10.2.4.16, TS-0004 [2], clause 7.4.37.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE the AEhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | flexContainer resource containing  | |
                     | maxNrOfInstances set to non-zero value and  | |
                     | a valid maxByteSizeand  | |
                     | a valid CUSTOM_ATTRIBUTE_1, and CUSTOM_ATTRIBUTE_2 with total byte sizegreater than maxByteSize  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT does not create the flexContainer resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/FLXC/CRE/004

| TP Id | TP/oneM2M/CSE/FLXC/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully creates the <flexContainer> resource and its child <flexcontainerInstance> resource when ATTRIBUTE attribute of <flexContainer> resource is present with a non-zero value in the request |  |
| Reference | TS-0001 [1], clause 10.1.2 and 10.2.4.16, TS-0004 [2], clause 7.4.37.2.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the AEhaving privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | flexContainer resource containing  | |
                     | a valid ATTRIBUTE attribute set to non-zero value  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT creates the <flexContainer> resource  | |
                     | and the IUT creates the child <flexContainerInstance> resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/CRE/004/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/CRE/004/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/CRE/004/MIA | maxInstanceAge |

##### 7.2.2.13.2	UPDATE Operation

###### TP/oneM2M/CSE/FLXC/UPD/001

| TP Id | TP/oneM2M/CSE/FLXC/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT increments the field value of attribute stateTag in the <flexContainer> resource when a custom attribute of the resource is modified |  |
| Reference | TS-0001 [1], clause 9.6.35 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource FLEXCONTAINER_RESOURCE_ADDRESS containing stateTag attribute and the AE having privileges to perform UPDATE operation on the resource FLEXCONTAINER_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to FLEXCONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Contentcontaining  | |
                     | flexContainer resourcecontaining  | |
                     | ATTRIBUTE_NAME attribute set to VALUE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT increments the stateTag attribute of FLEXCONTAINER_RESOURCE_ADDRESS resource   | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/FLXC/UPD/002

| TP Id | TP/oneM2M/CSE/FLXC/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not increment the field value of attribute stateTag in the <flexContainer> resource when a common attribute of the resource is modified |  |
| Reference | TS-0001 [1], clause 9.6.35 |  |
| Config Id | CF01 |  |
| Parent Release | Release 2 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing a container resource FLEXCONTAINER_RESOURCE_ADDRESS containing stateTag attribute and the AE having privileges to perform UPDATE operation on the resource FLEXCONTAINER_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to FLEXCONTAINER_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | flexContainer resource containing  | |
                     | ATTRIBUTE_NAME attribute set to VALUE  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not increment the stateTag attribute of FLEXCONTAINER resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/FLXC/UPD/003

| TP Id | TP/oneM2M/CSE/FLXC/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update of the <flexContainer> resource when the resource representation does not comply with the schema definition. |  |
| Reference | TS-0001 [1], clause 10.2.4.18, TS-0004 [2], clause 7.4.29.2.3 |  |
| Parent Release | Release 2 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform UPDATE operation on the resource FLXC_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives an invalid UPDATE request from AE  | |
                     | containing  | |
                     | To set to FLXC_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | flexContainer resource containing  | |
                     | an invalid resource representation  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not update the flexContainer resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/FLXC/UPD/004

| TP Id | TP/oneM2M/CSE/DMR/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE request of <flexContainer> resource when the size value of the customAttributes exceeds the value specified in the maxByteSize attribute |  |
| Reference | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a valid maxByteSize attribute and the AEhaving privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | flexcontainer resource containing  | |
                     | a valid CUSTOM_ATTRIBUTE_1, and CUSTOM_ATTRIBUTE_2 with total byte size greater than maxByteSize  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/FLXC/UPD/005

| TP Id | TP/oneM2M/CSE/FLXC/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that IUT updates the <flexContainer> resource and creates its new child <flexContainerInstance> resource when request contains a custom attribute of the targeted <flexContainer> resource |  |
| Reference | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a valid ATTRIBUTE attribute and the AE having privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | flexcontainer resource containing  | |
                     | a valid CUSTOM_ATTRIBUTE attribute  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT updates the <flexContainer> resource  | |
                     | and the IUT creates the child <flexContainerInstance> resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————–+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/UPD/005/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/UPD/005/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/UPD/005/MIA | maxInstanceAge |

###### TP/oneM2M/CSE/FLXC/UPD/006

| TP Id | TP/oneM2M/CSE/FLXC/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that IUT updates the <flexContainer> resource and creates its new child <flexContainerInstance> resource when request contains no custom attribute of the targeted <flexContainer> resource |  |
| Reference | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a valid ATTRIBUTE attribute and the AEhaving privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT updates the <flexContainer> resource  | |
                     | and the IUT creates the child <flexContainerInstance> resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/UPD/006/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/UPD/006/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/UPD/006/MIA | maxInstanceAge |

###### TP/oneM2M/CSE/FLXC/UPD/007

| TP Id | TP/oneM2M/CSE/FLXC/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the currentNrOfInstances and currentByteSize attributes of the <flexContainer> resource when the UPDATE request creates, modifies or deletes the ATTRIBUTE attribute of the <flexContainer> resource |  |
| Reference | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing children <flexContainerInstance> resources and the AEhaving privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | flexContainer resource containing  | |
                     | a valid ATTRIBUTE attribute  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT updates the set of <flexContainerInstances> resources according to the modified <flexContainer> attributes  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | Content containing  | |
                     | flexContainer resource containing  | |
                     | currentNrOfInstances and currentByteSize attributes indicating modified values  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/UPD/007/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/UPD/007/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/UPD/007/MIA | maxInstanceAge |

###### TP/oneM2M/CSE/FLXC/UPD/008

| TP Id | TP/oneM2M/CSE/FLXC/UPD/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT deletes all child <flexContainerInstance> resources and the currentNrOfInstances, currentByteSize, maxNrOfInstances, maxByteSize and maxInstanceAge attributes of the <flexContainer> when UPDATE request updates ATTRIBUTE of the <flexContainer> resource with a value of zero |  |
| Reference | TS-0001 [1], clause 10.1.4 and 10.2.4.18, TS-0004 [2], clause 7.4.37.2.3 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT being a hosting CSE and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing children <flexContainerInstance> resources and the AEhaving privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Resource Type set to 28 (flexContainer)  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | <flexContainer> resource containing  | |
                     | ATTRIBUTE attribute set to ZERO  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT deletes all <flexContainerInstance> resources  | |
                     | and the IUT deletes currentNrOfInstances, currentByteSize, maxNrOfInstances, maxByteSize and maxInstanceAge attributes of <flexContainer>  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/FLXC/UPD/008/MNI | maxNrOfInstances |
| TP/oneM2M/CSE/FLXC/UPD/008/MBS | maxByteSize |
| TP/oneM2M/CSE/FLXC/UPD/008/MIA | maxInstanceAge |

##### 7.2.2.13.3	RETRIEVE Operation

###### TP/oneM2M/CSE/FLXC/RET/001

| TP Id | TP/oneM2M/CSE/FLXC/RET/001 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT performs the RETRIEVE request for the latest <flexContainerInstance> resource |  |  |
| Reference | TS-0001 [1], clause 10.1.3, 10.2.4.17, TS-0004 [2], clause 7.4.37.2.2 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 1 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 is less than creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AEhaving privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |

- +---------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | Resource Type set to 58 (flexContainerInstance) and  | |
                     | Toset to TARGET_RESOURCE_ADDRESS/la and  | |
                     | From set to AE_ID  | |
                     |} | |

- +---------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | flexContainerInstance resource containing  | |
                     | resourceName attribute set to FLEX_CONTAINER_INSTANCE_2  | |
                     |} | |
+———————-+—————————+—————————————————————————————————-+————-+

###### TP/oneM2M/CSE/FLXC/RET/002

| TP Id | TP/oneM2M/CSE/FLXC/RET/002 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT performs the RETRIEVE request for the oldest <flexContainerInstance> resource |  |  |
| Reference | TS-0001 [1], clause 10.1.3, 10.2.4.17, TS-0004 [2], clause 7.4.37.2.2 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 1 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 is less than creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AEhaving privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |

- +---------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | Resource Type set to 58 (flexContainerInstance) and  | |
                     | To set to TARGET_RESOURCE_ADDRESS/ol and  | |
                     | From set to AE_ID  | |
                     |} | |

- +---------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | flexContainerInstance resource containing  | |
                     | resourceName attribute set to FLEX_CONTAINER_INSTANCE_1  | |
                     |} | |
+———————-+—————————+—————————————————————————————————-+————-+

##### 7.2.2.13.4	DELETE Operation

###### TP/oneM2M/CSE/FLXC/DEL/001

| TP Id | TP/oneM2M/CSE/FLXC/DEL/001 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT performs the DELETE request for the latest <flexContainerInstance> resource |  |  |
| Reference | TS-0001 [1], clause 10.1.5, 10.2.4.19, TS-0004 [2], clause 7.4.37.2.4 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 1 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 is less than creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AEhaving privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |

- +-----------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | Resource Type set to 58 (flexContainerInstance)  | |
                     | To set to TARGET_RESOURCE_ADDRESS/la and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-----------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT deletes the FLEX_CONTAINER_INSTANCE_2 resource  | |
                     | the IUT updates the currentNrOfInstances, currentByteSize and contentSize attributes of the <flexContainer> resource | |
                     |and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+—————————–+—————————————————————————————————-+————-+

###### TP/oneM2M/CSE/FLXC/DEL/002

| TP Id | TP/oneM2M/CSE/FLXC/DEL/002 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT performs the DELETE request for the oldest <flexContainerInstance> resource |  |  |
| Reference | TS-0001 [1], clause 10.1.5, 10.2.4.19, TS-0004 [2], clause 7.4.37.2.4 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 1 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE containing and the IUT having a <flexContainer> resource at TARGET_RESOURCE_ADDRESS containing a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_1 a child <flexContainerInstance> resource as FLEX_CONTAINER_INSTANCE_2 and creationTime attribute of FLEX_CONTAINER_INSTANCE_1 is less than creationTime attribute of FLEX_CONTAINER_INSTANCE_2 and the AEhaving privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS } |  |  |

- +-----------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | Resource Type set to 58 (flexContainerInstance)  | |
                     | To set to TARGET_RESOURCE_ADDRESS/ol and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-----------------------------+----------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT deletes the FLEX_CONTAINER_INSTANCE_1 resource  | |
                     | the IUT updates the currentNrOfInstances, currentByteSize and contentSize attributes of the <flexContainer> resource | |
                     |and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+—————————–+—————————————————————————————————-+————-+

#### 7.2.2.14	Dynamic Authorization (DAU)

##### 7.2.1.14.1	NOTIFY Operation

###### TP/oneM2M/CSE/DAU/NOT/001

| TP Id | TP/oneM2M/CSE/DAU/NOT/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT starts the Dynamic Authorization for an incoming request and sends a Notify Request to DAS including all mandatory fields |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF06 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the <container> resource having a dynamicAuthorizationConsultationID attribute and corresponding <dynamicAuthorizationConsultation> not set to FALSE } |  |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid RETRIEVE request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID and  | |
                     | no Content  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |DAS <= IUT |
                     | the IUT sends a valid NOTIFY Request to DAS containing  | |
                     | To set to DAS_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Notification datacontaining  | |
                     | securityInfo containing  | |
                     | securityInfoType set to 1 (Dynamic Authorization Request) and  | |
                     | dasRequest containing  | |
                     | originator set to AE_ID and  | |
                     | targetedResourceType set to 3 (container) and  | |
                     | operation set to 2 (Retrieve)  | |
                     |} | |
+———————-+——————————————————————————————————————————-+————-+ Editor's Note: Config Id will be newly proposed to cover this test purpose.

###### TP/oneM2M/CSE/DAU/NOT/002

| TP Id | TP/oneM2M/CSE/DAU/NOT/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT starts the Dynamic Authorization for an incoming request and sends a Notify Request including all mandatory fields and one ore more optional fields |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF06 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the <container> resource has a dynamicAuthorizationConsultationID attribute and corresponding <dynamicAuthorizationConsultation> not set to FALSE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid RETRIEVE request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID and  | |
                     | no Content  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |DAS <= IUT |
                     | the IUT sends a valid NOTIFY Request to DAS containing  | |
                     | To set to DAS_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Notification datacontaining  | |
                     | securityInfo containing  | |
                     | securityInfoType set to 1 (Dynamic Authorization Request) and  | |
                     | dasRequest containing  | |
                     | originator set to AE_ID and  | |
                     | targetedResourceType set to 3 (container) and  | |
                     | operation set to 2 (Retrieve)  | |
                     | originatorIP set to AE_IP  | |
                     | originatorLocation set to location of originator of the received   | |
                     | request  | |
                     | originatorRole set to role of originator of the received request  | |
                     | requestTimestamp set to time of receipt of request  | |
                     | targetedResourceID set to identifier of the targeted resource  | |
                     | proposedPrivilegesLifetime set to time duration of requested   | |
                     | privileges  | |
                     | rolesFromACPs set to list of roles of requested resource  | |
                     | tokenIdDs set to list of token identifiers from the received request  | |
                     | authorSignIndicator set to value specified by the received request  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/DAU/NOT/003

| TP Id | TP/oneM2M/CSE/DAU/NOT/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not start the Dynamic Authorization for an incoming request and rejects that incoming request if the dynamicAuthorizationConsultationID attribute and corresponding <dynamicAuthorizationConsultation> resource can not be found |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the <container> resource does not have a dynamicAuthorizationConsultationID attribute nor corresponding <dynamicAuthorizationConsultation> } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid RETRIEVE request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID and  | |
                     | no Content  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE1 <= IUT |
                     | the IUT rejects the incoming request from the AE1  | |
                     | containing Response Status Code set to   | |
                     | "ORIGINATOR_HAS_NO_PRIVILEGE"  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DAU/NOT/004

| TP Id | TP/oneM2M/CSE/DAU/NOT/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not start the Dynamic Authorization for an incoming request and rejects that incoming request if the dynamicAuthorizationEnabled of a <dynamicAuthorizationConsultation> is set to FALSE |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the <container> resource has <dynamicAuthorizationConsultation> with dynamicAuthorizationEnabled set to FALSE } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid RETRIEVE request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1_ID and  | |
                     | no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE1 <= IUT |
                     | the IUT rejects the incoming request from the AE1  | |
                     | containing Response Status Code set to   | |
                     | "ORIGINATOR_HAS_NO_PRIVILEGE"  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/DAU/NOT/005

| TP Id | TP/oneM2M/CSE/DAU/NOT/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT correctly processes a Notify response received in response to a Notify request that started Dynamic Authorization |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF06 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the AE1 having sent a Retrieve request for the resource at TARGET_RESOURCE_ADDRESS and the AE1 having no privileges to perform RETRIEVE operation on the resource and the IUT having sent a NOTIFY Request to DAS to start Dynamic Authorization } |  |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |DAS => IUT |
                     | the IUT receives a valid NOTIFY Response containing  | |
                     | Notification datacontaining  | |
                     | securityInfo containing  | |
                     | securityInfoType set to 2 (Dynamic Authorization Response)  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE1 <= IUT |
                     | the IUT sends a valid Response to AE1 containing  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Content containing  | |
                     | <container> representation  | |
                     |} | |
+———————-+———————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/DAU/NOT/006

| TP Id | TP/oneM2M/CSE/DAU/NOT/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT correctly processes a Notify response including a dynamicACPInfo element received in response to a Notify request that started Dynamic Authorization |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF06 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the AE1 having sent a Retrieve request for the resource at TARGET_RESOURCE_ADDRESS and the AE1 having no privileges to perform RETRIEVE operation on the resource and the IUT having sent a NOTIFY Request to DAS to start Dynamic Authorization } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |DAS => IUT |
                     | the IUT receives a valid NOTIFY Response containing  | |
                     | Notification datacontaining  | |
                     | securityInfo containing  | |
                     | securityInfoType set to 2 (Dynamic Authorization Response) and  | |
                     | dynAuthDasResponse containing dynamicACPInfo  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE1 <= IUT |
                     | the IUT creates  | |
                     | <accesscontrolPolicy> child resource and  | |
                     | configures the privileges, expirationTime and selfPrivileges attributes  | |
                     | based on the data received in the dynamicACPInfo,  | |
                     | and the IUT sends a valid Response to AE1 containing  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Content containing  | |
                     | <container> representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/DAU/NOT/007

| TP Id | TP/oneM2M/CSE/DAU/NOT/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT correctly processes a Notify response including a tokens element received in response to a Notify request that started Dynamic Authorization |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF06 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the AE1 having sent a Retrieve request for the resource at TARGET_RESOURCE_ADDRESS and the AE1 having no privileges to perform RETRIEVE operation on the resource and the IUT having sent a NOTIFY Request to DAS to start Dynamic Authorization } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |DAS => IUT |
                     | the IUT receives a valid NOTIFY Response containing  | |
                     | Notification datacontaining  | |
                     | Tokens element  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE1 <= IUT |
                     | the IUT sends a valid Response to AE1 containing  | |
                     | ResponseStatusCode set to OK (2000) and  | |
                     | Content containing  | |
                     | <container> representation  | |
                     |} | |
+———————-+————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/DAU/NOT/008

| TP Id | TP/oneM2M/CSE/DAU/NOT/008 |  |
| --- | --- | --- |
| Test objective |  |  |
| Reference | TS-0001 [1], clause 10.1.6 and 10.2.10.6, TS-0004 [2], clause 7.2.2.1 and 7.5.1.2.10 |  |
| Config Id | CF06 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being registered and the IUT being a hosting CSE and the IUT having registered AE1 and the IUT having registered the DAS and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and the AE1 having sent a Retrieve request for the resource at TARGET_RESOURCE_ADDRESS and the AE1 having no privileges to perform RETRIEVE operation on the resource and the IUT having sent a NOTIFY Request to DAS to start Dynamic Authorization } |  |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |DAS => IUT |
                     | the IUT receives a valid NOTIFY Response NOT containing  | |
                     | Notification datacontaining  | |
                     | securityInfo containing  | |
                     | securityInfoType set to 2 (Dynamic Authorization Response)  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE1 <= IUT |
                     | the IUT rejects the incoming request from the AE1containing  | |
                     | Response Status Code set to   | |
                     | "ORIGINATOR_HAS_NO_PRIVILEGE"  | |
                     |} | |
+———————-+——————————————————————————————————————-+————-+

#### 7.2.2.15	Semantic (SEM)

##### 7.2.2.15.1	CREATE Operation

###### TP/oneM2M/CSE/SEM/CRE/001

| TP Id | TP/oneM2M/CSE/SEM/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticDescriptor> resource which descriptor attribute does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.1.2, and TS-0004 [2], clause 7.4.34.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a parent resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | descriptor attributeset to a value that does not conform to  | |
                     | the RDF/XML syntax  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the <semanticDescriptor> resource and  | |
                     | the IUT sends a valid Response containing  | |
                     | failure information with additional error information  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/CRE/002

| TP Id | TP/oneM2M/CSE/SEM/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a Response Status Code indicating a "ONTOLOGY_NOT_AVAILABLE" error when referenced ontology cannot be retrieved during a semantic validation process |  |
| Reference | TS-0004 [2], clause 7.4.34.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a local policy that allows semantic validation procedures } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | ontologyRef attribute set to a non-resolvable URI  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4118 (ONTOLOGY_NOT_AVAILABLE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/CRE/003

| TP Id | TP/oneM2M/CSE/SEM/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a Response Status Code indicating a "LINKED_SEMANTICS_NOT_AVAILABLE" error when local linked <semanticDescriptor> resources cannot be retrieved during a semantic validation process |  |
| Reference | TS-0004 [2], clause 7.4.34.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a local policy that allows semantic validation procedures } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | relatedSemantics attribute set to a non-resolvable URI  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4119  | |
                     | (LINKED_SEMANTICS_NOT_AVAILABLE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/CRE/004

| TP Id | TP/oneM2M/CSE/SEM/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retrieves the referenced ontology from the ontology hosting CSE when performing a semantic validation as a result of a creation of a stand-alone <semanticDescriptor> resource |  |
| Reference | TS-0034 [6], clauses 6.1.2 and 7.10.3, and TS-0004 [2], clause 7.4.34.2.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a local policy that allows semantic validation procedures and the ontology hosting CSE having an <ontology> resource at REFERENCED_ONTOLOGY_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | descriptor attribute and  | |
                     | ontologyRef attribute set to  | |
                     | REFERENCED_ONTOLOGY_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid RETRIEVE Request containing  | |
                     | To set to REFERENCED_ONTOLOGY_ADDRESS and  | |
                     | Content containing  | |
                     | <ontology> resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/CRE/005

| TP Id | TP/oneM2M/CSE/SEM/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a valid response when performing a semantic validation as a result of a creation of a stand-alone <semanticDescriptor> resource |  |
| Reference | TS-0034 [6], clauses 6.1.2 and 7.10.3, and TS-0004 [2], clause 7.4.34.2.1 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a local policy that allows semantic validation procedures and the ontology hosting CSE having an <ontology> resource at REFERENCED_ONTOLOGY_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | descriptor attribute and  | |
                     | ontologyRef attribute set to  | |
                     | REFERENCED_ONTOLOGY_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code indicating success of validation  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/CRE/006

| TP Id | TP/oneM2M/CSE/SEM/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupInstance> resource, indicating a "BAD_REQUEST" error, when the <semanticMashupJobProfile> resource referenced in its smjpID attribute does not exist |  |
| Reference | TS-0034 [6], clause 6.4.2, and TS-0004 [2], clause 7.4.50.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 41 (semanticMashupInstance) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticMashupInstance> resource representation containing  | |
                     | smjpID attributeset to a reference to a non-existent  | |
                     | <semanticMashupJobProfile> resource  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupInstance> resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/CRE/007

| TP Id | TP/oneM2M/CSE/SEM/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupInstance> resource, indicating a "BAD_REQUEST" error, when its smjpInputParameter attribute value does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.4.2, and TS-0004 [2], clause 7.4.50.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 41 (semanticMashupInstance) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticMashupInstance> resource representation containing  | |
                     | smjpID attribute set to a reference to the  | |
                     | <semanticMashupJobProfile> resource at  | |
                     | TARGET_RESOURCE_ADDRESS  | |
                     | smjpInputParameter set to a value that does not conform to  | |
                     | the RDF/XML syntax  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupInstance> resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/CRE/008

| TP Id | TP/oneM2M/CSE/SEM/CRE/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupInstance> resource, indicating a "BAD_REQUEST" error, when its smjpInputParameter attribute value does not meet the input parameter requirement specified by the inputDescriptor attribute of the corresponding <semanticMashupJobProfile> resource |  |
| Reference | TS-0034 [6], clause 6.4.2, and TS-0004 [2], clause 7.4.50.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE operation inputDescriptor attribute set to a valid value } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 41 (semanticMashupInstance) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticMashupInstance> resource representation containing  | |
                     | smjpID attribute set to a reference to the  | |
                     | <semanticMashupJobProfile> resource at  | |
                     | TARGET_RESOURCE_ADDRESS  | |
                     | smjpInputParameter set to a value of a type not allowed  | |
                     | according to the inputDescriptor attribute of the  | |
                     | <semanticMashupJobProfile> resource at  | |
                     | TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupInstance> resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/CRE/009

| TP Id | TP/oneM2M/CSE/SEM/CRE/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns a "MASHUP_MEMBER_NOT_FOUND" error when creating a <semanticMashupInstance> when a required mashup member resource cannot be identified |  |
| Reference | TS-0034 [6], clause 6.4.2, and TS-0004 [2], clause 7.4.50.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE operation memberFilter attribute set to a SPARQL query to discover an unidentified mashup member } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 41 (semanticMashupInstance) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticMashupInstance> resource representation containing  | |
                     | smjpID attributeset to a reference to the  | |
                     | <semanticMashupJobProfile> resource at  | |
                     | TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4121  | |
                     | (MASHUP_MEMBER_NOT_FOUND)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/CRE/010

| TP Id | TP/oneM2M/CSE/SEM/CRE/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupJobProfile> resource, indicating a "BAD_REQUEST" error, when its inputDescriptor attribute value does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.3.2, and TS-0004 [2], clause 7.4.49.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {  |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing inputDescriptor attribute set to a  | |
                     | value that does not conform to the RDF/XML syntax  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/CRE/011

| TP Id | TP/oneM2M/CSE/SEM/CRE/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupJobProfile> resource, indicating a "BAD_REQUEST" error, when its outputDescriptor attribute value does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.3.2, and TS-0004 [2], clause 7.4.49.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing outputDescriptor attributeset to  | |
                     | a value that does not conform to the RDF/XML syntax  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/CRE/012

| TP Id | TP/oneM2M/CSE/SEM/CRE/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupJobProfile> resource, indicating a "BAD_REQUEST" error, when its functionDescriptor attribute value does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.3.2, and TS-0004 [2], clause 7.4.49.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing functionDescriptor attributeset  | |
                     | to a value that does not conform to the RDF/XML syntax  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/CRE/013

| TP Id | TP/oneM2M/CSE/SEM/CRE/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupJobProfile> resource, indicating a "BAD_REQUEST" error, when its memberFilter attribute value does not conform to a valid SPARQL query request |  |
| Reference | TS-0034 [6], clause 6.3.2, and TS-0004 [2], clause 7.4.49.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE operation } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and Resource  | |
                     | Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing memberFilter attributeset to  | |
                     | a value that does not conform to a valid SPARQL query  | |
                     | request  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/CRE/014

| TP Id | TP/oneM2M/CSE/SEM/CRE/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates successfully the <semanticMashupInstance> resource |  |
| Reference | TS-0001 [1], clause 9.6.54, TS-0004 [2], clause 7.4.50.2.1, TS-0034 [6], clause 6.4.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE operation } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------+---------+

- |when {   |IUT <= AE|
                 | the IUT receives a valid CREATE Request from AE containing  | |
                 | To set to TARGET_RESOURCE_ADDRESS and  | |
                 | Resource Type set to 41 (semanticMashupInstance) and  | |
                 | From set to AE-ID and  | |
                 | Content containing <semanticMashupInstance> resource  | |
                 | representation containing  | |
                 | smjpID attribute set to a reference to the  | |
                 | <semanticMashupJobProfile> resource at  | |
                 | TARGET_RESOURCE_ADDRESS  | |
                 | smjpInputParameter set to a value that does conform to  | |
                 | the RDF/XML syntax  | |
                 |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------+---------+

- |then {   |IUT => AE|
                 | the IUT creates the semanticMashupInstance resource  | |
                 | and the IUT sends a valid Response containing  | |
                 | Response Status Code set to 2001 (CREATED)  | |
                 |} | |
+——————+———————————————————————————————————————————————-+———+

###### TP/oneM2M/CSE/SEM/CRE/015

| TP Id | TP/oneM2M/CSE/SEM/CRE/015 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT creates successfully the <semanticMashupInstance> resource with resultGenType set to WHEN_SMI_IS_CREATED |  |  |
| Reference | TS-0001 [1], clause 9.6.54, TS-0004 [2], clause 7.4.50.2.1, TS-0034 [6], clause 6.4.2 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 3 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform CREATE operation } |  |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------------------------------------------------------------------------------------------------------------------------------+---------+

- |when {   |IUT <= AE|
                 | the IUT receives a valid CREATE Request from AE containing  | |
                 | To set to TARGET_RESOURCE_ADDRESS and  | |
                 | Resource Type set to 41 (semanticMashupInstance) and  | |
                 | From set to AE-ID and  | |
                 | Content containing <semanticMashupInstance> resource  | |
                 | representation containing smjpID attributeset to a reference to  | |
                 | the <semanticMashupJobProfile> resource at  | |
                 | TARGET_RESOURCE_ADDRESS smjpInputParameter set to a  | |
                 | value that does conform to the RDF/XML syntaxand  | |
                 | resultGenType is set to WHEN_SMI_IS_CREATED  | |
                 |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+----------------------------------------------------------------------------------------------------------------------------------------------+---------+

- |then {  |IUT => AE|
                 | the IUT creates the semanticMashupInstance resource  | |
                 | and the IUT sends a valid Response containing  | |
                 | Response Status Code set to 2001 (CREATED)  | |
                 |} | |
+——————+———————————————————————————————————————————————————————————————————————————————————-+———————————————————————————————————————————————-+———+

###### TP/oneM2M/CSE/SEM/CRE/016

| TP Id | TP/oneM2M/CSE/SEM/CRE/016 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT creates successfully the <semanticMashupInstance> resource with resultGenType set to PERIODICALLY |  |  |
| Reference | TS-0001 [1], clause 9.6.54, TS-0004 [2], clause 7.4.50.2.1, TS-0034 [6], clause 6.4.2 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 3 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 41 (semanticMashupInstance) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupInstance> resource  | |
                     | representation containing smjpID attributeset to a reference to  | |
                     | the <semanticMashupJobProfile> resource at  | |
                     | TARGET_RESOURCE_ADDRESS smjpInputParameter set to  | |
                     | a value that does conform to the RDF/XML syntax and  | |
                     | resultGenType is set to PERIODICALLY with a period of t1  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT creates a new semanticMashupInstance resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+——————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/CRE/017

| TP Id | TP/oneM2M/CSE/SEM/CRE/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticDescriptor> resource in which descriptorRepresentation attribute is set to "IRI". |  |
| Reference | TS-0034 [6], clause 6.1.2, and TS-0004 [2], clause 7.4.34.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a parent resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | descriptorRepresentation attribute is set to "IRI"  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticDescriptor> resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————-+————-+

##### 7.2.2.15.2	UPDATE Operation

###### TP/oneM2M/CSE/SEM/UPD/001

| TP Id | TP/oneM2M/CSE/SEM/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <semanticDescriptor> resource indicating a "NOT_ACCEPTABLE" error when the request contains both semanticOpExec and descriptor attributes |  |
| Reference | TS-0004 [2], clause 7.4.34.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | descriptor attributeset to a valid value  | |
                     | semanticOpExec attribute set to a valid value  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not update the <semanticDescriptor> resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/002

| TP Id | TP/oneM2M/CSE/SEM/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <semanticDescriptor> resource indicating a "NOT_ACCEPTABLE" error when the request contains a semanticOpExec attribute which does not contain a valid SPARQL query request |  |
| Reference | TS-0004 [2], clause 7.4.34.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | no descriptor attributeand  | |
                     | semanticOpExec attributeset to a non-valid SPARQL query request  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not update the <semanticDescriptor> resource and  | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/003

| TP Id | TP/oneM2M/CSE/SEM/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <semanticDescriptor> resource when the request contains a descriptor attribute which does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0004 [2], clause 7.4.34.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | no semanticOpExec attributeand  | |
                     | descriptor attributeset to a value that does not conform to  | |
                     | the RDF/XML syntax  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not update the <semanticDescriptor> resource and  | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/004

| TP Id | TP/oneM2M/CSE/SEM/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retrieves the referenced ontology from the ontology hosting CSE when performing a semantic validation as a result of an update of a stand-alone <semanticDescriptor> resource |  |
| Reference | TS-0034 [6], clauses 6.1.4 and 7.10.3, and TS-0004 [2], clause 7.4.34.2.3 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a local policy that allows semantic validation procedures and the ontology hosting CSE having an <ontology> resource at REFERENCED_ONTOLOGY_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | descriptor attribute and  | |
                     | ontologyRef attribute set to  | |
                     | REFERENCED_ONTOLOGY_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid RETRIEVE Request containing  | |
                     | To set to REFERENCED_ONTOLOGY_ADDRESS and  | |
                     | Content containing  | |
                     | <ontology> resource representation  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/005

| TP Id | TP/oneM2M/CSE/SEM/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a valid response when performing a semantic validation as a result of an update of a stand-alone <semanticDescriptor> resource |  |
| Reference | TS-0034 [6], clauses 6.1.4 and 7.10.3, and TS-0004 [2], clause 7.4.34.2.3 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a local policy that allows semantic validation procedures and the ontology hosting CSE having an <ontology> resource at REFERENCED_ONTOLOGY_ADDRESS **}* |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | descriptor attribute and  | |
                     | ontologyRef attribute set to  | |
                     | REFERENCED_ONTOLOGY_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (UPDATED)and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource  | |
                     |} | |
+———————-+———————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/UPD/006

| TP Id | TP/oneM2M/CSE/SEM/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retrieves the linked <semanticDescriptor> resource from the hosting CSE when performing a semantic validation as a result of an update of the <semanticValidation> virtual resource |  |
| Reference | TS-0034 [6], clauses 6.9.4 and 7.10.3, and TS-0004 [2], clause 7.4.48.2.3 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <ontologyRepositoy> resource at TARGET_RESOUCE ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS and the IUThaving a <ontology> resource at REFERENCED_ONTOLOGY_ADDRESS and the IUT having a local policy that allows semantic validation procedures and the hosting CSE having a <semanticDescriptor> resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to the TARGET_RESOURCE_ADDRESS/smv and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | descriptor attribute and  | |
                     | ontologyRef attribute set to  | |
                     | REFERENCED_ONTOLOGY_ADDRESS and  | |
                     | relatedSemantics attributeset to  | |
                     | LINKED_SEMANTIC_DESCRIPTOR_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid RETRIEVE Request containing  | |
                     | To set to LINKED_SEMANTIC_DESCRIPTOR_ADDRESS and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/007

| TP Id | TP/oneM2M/CSE/SEM/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a valid response when performing a semantic validation a semantic validation as a result of an update of the <semanticValidation> virtual resource |  |
| Reference | TS-0034 [6], clauses 6.9.4 and 7.10.3, and TS-0004 [2], clause 7.4.48.2.3 |  |
| Parent Release | Release 3 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <ontologyRepositoy> resource at TARGET_RESOUCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a <ontology> resource at REFERENCED_ONTOLOGY_ADDRESS and the IUT having a local policy that allows semantic validation procedures and the hosting CSE having a <semanticDescriptor> resource } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to the TARGET_RESOURCE_ADDRESS/smv and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | descriptor attribute and  | |
                     | ontologyRef attribute set to  | |
                     | REFERENCED_ONTOLOGY_ADDRESS and  | |
                     | relatedSemantics attributeset to  | |
                     | LINKED_SEMANTIC_DESCRIPTOR_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid response containing  | |
                     | Response Status Code indicating success of validation  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/008

| TP Id | TP/oneM2M/CSE/SEM/UPD/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <semanticMashupJobProfile> resource, indicating a "NOT_ACCEPTABLE" error, when its inputDescriptor attribute value does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.3.4, and TS-0004 [2], clause 7.4.49.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform UPDATE operation } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing inputDescriptor attribute set to a  | |
                     | value that does not conform to the RDF/XML syntax  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource andg   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/009

| TP Id | TP/oneM2M/CSE/SEM/UPD/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <semanticMashupJobProfile> resource, indicating a "NOT_ACCEPTABLE" error, when its outputDescriptor attribute value does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.3.4, and TS-0004 [2], clause 7.4.49.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform UPDATE operation } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing outputDescriptor attribute set to  | |
                     | a value that does not conform to the RDF/XML syntax  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/UPD/010

| TP Id | TP/oneM2M/CSE/SEM/UPD/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <semanticMashupJobProfile> resource, indicating a "NOT_ACCEPTABLE" error, when its functionDescriptor attribute value does not conform to the RDF/XML syntax as defined in RDF 1.1 XML Syntax |  |
| Reference | TS-0034 [6], clause 6.3.4, and TS-0004 [2], clause 7.4.49.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform UPDATE operation } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing functionDescriptor attribute set  | |
                     | to a value that does not conform to the RDF/XML syntax  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource and  | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/011

| TP Id | TP/oneM2M/CSE/SEM/UPD/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of a <semanticMashupJobProfile> resource, indicating a "NOT_ACCEPTABLE" error, when its memberFilter attribute value does not conform to a valid SPARQL query request |  |
| Reference | TS-0034 [6], clause 6.3.4, and TS-0004 [2], clause 7.4.49.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform UPDATE operation } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 40 (semanticMashupJobProfile) and  | |
                     | From set to AE-ID and  | |
                     | Content containing <semanticMashupJobProfile> resource  | |
                     | representation containing memberFilter attribute set to  | |
                     | a value that does not conform to a valid SPARQL query  | |
                     | request  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not create the <semanticMashupJobProfile>  | |
                     | resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/012

| TP Id | TP/oneM2M/CSE/SEM/UPD/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates a new sematicMashupResult when the parameter smjpInputParameter is updated |  |
| Reference | TS-0034 [6], clause 6.3.4, and TS-0004 [2], clause 7.4.49.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupInstance> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform UPDATE operation } |  |

- +--------------------------------------------------------------------------------------------+-------------+

- |When {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 41 (semanticMashupInstance) and  | |
                     | From set to AE-ID and  | |
                     | Content containing smjpInputParameter attribute set  | |
                     | to a valid value  | |
                     |} | |

- +--------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT creates a new <sematicMashupResult>  | |
                     | resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+——————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/013

| TP Id | TP/oneM2M/CSE/SEM/UPD/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT changes the mashupMember attribute of sematicMashupInstance when the parameter memberStoreType is updated |  |
| Reference | TS-0034 [6], clause 6.4.4, and TS-0004 [2], clause 7.4.50.2.3, TS-0001 [1], clause 9.6.54 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupInstance> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform UPDATE operation } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |When {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 41 (semanticMashupInstance) and  | |
                     | From set to AE-ID and  | |
                     | Content containing memberStoreType attribute set  | |
                     | to a new valid value  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT updates mashupMember attribute of resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/UPD/014

| TP Id | TP/oneM2M/CSE/SEM/UPD/014 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <semanticDescriptor> resource in which descriptorRepresentation attribute is set to "IRI". |  |  |
| Reference | TS-0034 [6], clause 6.1.4, and TS-0004 [2], clause 7.4.34.2.3 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 3 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS } |  |  |

- +-------------------------------------------+-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource representation containing  | |
                     | descriptorRepresentation attribute is set to "IRI"  | |
                     |} | |

- +-------------------------------------------+-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not update the <semanticDescriptor> resource and   | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————-+———————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/UPD/015

| TP Id | TP/oneM2M/CSE/SEM/UPD/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a Response Status Code indicating a "SPARQL_UPDATE_ERROR" error to a update request of a <semanticDescriptor> resource in which semanticOpExec attribute contains a valid SPARQL update request and request cannot be executed by the Hosting CSE. |  |
| Reference | TS-0034 [6], clause 6.1.4, and TS-0004 [2], clause 7.4.34.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing semanticOpExec attribute set  | |
                     | to a SPARQL update request and  | |
                     | SPARQL update request cannot be executed by IUT  | |
                     |}  | |
                     | | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not update the <semanticDescriptor> resource and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5212 (SPARQL_UPDATE_ERROR)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————–+————-+

##### 7.2.2.15.3	RETRIEVE Operation

###### TP/oneM2M/CSE/SEM/RET/001

| TP Id | TP/oneM2M/CSE/SEM/RET/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a Response Status Code indicating a "NOT_ACCEPTABLE" error to a retrieve request of a <semanticFanOutPoint> resource when the request contains a semanticsFilter condition set to a non-valid SPARQL query request |  |
| Reference | TS-0004 [2], clause 7.4.35.2.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing semanticSupportIndicator attribute set to "True" and accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation and <semanticFanOutPoint> virtual resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/sfop and  | |
                     | From set to AE-ID and  | |
                     | Semantic Query Indicator set to "True" and  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter conditionset to a non-valid SPARQL query request  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/RET/002

| TP Id | TP/oneM2M/CSE/SEM/RET/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a <semanticMashupResult> resource as a child of a <semanticMashupInstance> when a retrieve operation is sent to its <mashup> virtual resource |  |
| Reference | TS-0004 [2], clause 7.4.51.2.2, and TS-0034 [6], clause 6.5.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupInstance> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation and <mashup> virtual resource } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/msp and  | |
                     | From set to AE-ID and  | |
                     | Content set to void  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUTcreates a child <semanticMashupResult> resource at  | |
                     | TARGET_RESOURCE_ADDRESS and  | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/RET/003

| TP Id | TP/oneM2M/CSE/SEM/RET/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a Response Status Code indicating a "NOT_ACCEPTABLE" error to a retrieve request of a <semanticFanOutPoint> resource when the request addresses a group resource with semanticSupportIndicator attribute set to "False" |  |
| Reference | TS-0004 [2], clause 7.4.35.2.2, and TS-0001 [1], clause 10.2.7.12 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <group> resource at TARGET_RESOURCE_ADDRESS containing semanticSupportIndicator attributeset to "False" and accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation and <semanticFanOutPoint> virtual resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS/sfop and  | |
                     | From set to AE-ID and  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter conditionset to a valid SPARQL query request  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 5207 (NOT_ACCEPTABLE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/RET/004

| TP Id | TP/oneM2M/CSE/SEM/RET/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a valid semantic discovery request for target resource other than <semanticFanOutPoint> |  |
| Reference | TS-0034 [6], clause 7.4, TS-0001 [1] clauses 8.1.2 and 10.2.14 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a AE resource containing <semanticDescriptor> and no restrictions from ACPs } |  |

- +--------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter conditionset to a valid SPARQL query request  | |
                     | not containing <Semantic Query Indicator> parameter  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Semantic resource discovery result containing  | |
                     | URI (of child resource with target semanticDescriptor)  | |
                     |} | |
+———————-+——————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/RET/005

| TP Id | TP/oneM2M/CSE/SEM/RET/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a valid semantic discovery request for group members |  |
| Reference | TS-0001 [1], clauses 8.1.2, 9.6.14a, 10.2.7.12 and 10.2.14, and TS-0034 [6], clauses 6.2.2 and 7.4 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having <semanticSupportIndicator> set to TRUE and the IUT having registered the AE and the IUT having a AE <group> resource containing <semanticDescriptor> and no restrictions from ACPs } |  |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS containing   | |
                     | <semanticFanOutPoint> and  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter conditionset to a valid SPARQL query request  | |
                     | not containing <Semantic Query Indicator> parameter  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Semantic resource discovery result containing  | |
                     | URI (of child resource with target semanticDescriptor)  | |
                     |} | |
+———————-+———————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/RET/006

| TP Id | TP/oneM2M/CSE/SEM/RET/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a valid semantic discovery request for group members hosted at other CSE and sends a discovery request to the remote CSE |  |
| Reference | TS-0034 [6], clauses 6.2.2 and 7.4 |  |
| Config Id | CF02 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <semanticDescriptor> having linked semantic resource at remote CSE and no restrictions from ACPs } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS containing   | |
                     | <semanticFanOutPoint> and  | |
                     | not containing <Semantic Query Indicator> parameter  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE2 |
                     | the IUT sends a valid RETRIEVE Request containing  | |
                     | To set to address of linked semantic resource at remote CSE  | |
                     |} | |
+———————-+————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/RET/007

| TP Id | TP/oneM2M/CSE/SEM/RET/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT accepts a valid semantic discovery response for group members hosted at other CSE and sends a discovery response to the originating AE |  |
| Reference | TS-0001 [1], clauses 8.1.2, 9.6.14a, 10.2.7.12 and 10.2.14, and TS-0034 [6], clauses 6.2.2 and 7.4 |  |
| Config Id | CF02 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticDescriptor> resource having linked semantic resource at remote CSE and no restrictions from ACPs and the IUT having receive d a valid RETRIEVE Request from AE containing To set to TARGET_RESOURCE_ADDRESS containing <semanticFanOutPoint> and Filter Criteria containing semanticsFilter conditionset to a valid SPARQL query request not containing <Semantic Query Indicator> parameter and the IUT having sent a valid RETRIEVE Request containing To set to address of linked semantic resource at remote CSE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= CSE2 |
                     | the IUT receives a valid Response from CSE2 containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | containing Semantic resource from remote CSE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Semantic resource discovery result containing  | |
                     | URI (of child resource with target semanticDescriptor)  | |
                     |} | |
+———————-+——————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/RET/008

| TP Id | TP/oneM2M/CSE/SEM/RET/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retrieves relatedSemantics in a valid semantic discovery request |  |
| Reference | TS-0034 [6], clauses 7.4, 7.4.3, and TS-0001 [1] clauses 8.1.2, 10.2.14 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a AE resource containing <semanticDescriptor> containing relatedSemantics attribute and no restrictions from ACPs } |  |

- +------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter conditionset to a valid SPARQL query request  | |
                     | not containing <Semantic Query Indicator> parameter  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Semantic resource discovery result containing  | |
                     | URI (of child resource with target semanticDescriptor) and  | |
                     | URI (of resources from relatedSemantics)  | |
                     |} | |
+———————-+——————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/RET/009

| TP Id | TP/oneM2M/CSE/SEM/RET/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retrieves content-related semantics in a valid semantic discovery request |  |
| Reference | TS-0034 [6], clauses 7.4, 7.6, and TS-0001 [1] clauses 8.1.2, 10.2.14 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a AE resource containing <semanticDescriptor> and <contentInstance> present and no restrictions from ACPs } |  |

- +----------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter conditionset to a valid SPARQL query request and  | |
                     | content-related semantics  | |
                     | not containing <Semantic Query Indicator> parameter  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Semantic resource discovery result containing  | |
                     | URI (of child resource with target semanticDescriptor)  | |
                     |} | |
+———————-+—————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/RET/010

| TP Id | TP/oneM2M/CSE/SEM/RET/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully the attributes of the <semanticMashupJobProfile> resource |  |
| Reference | TS-0004 [2], clause 7.4.49.2.2, TS-0034 [6], clause 6.3.3, TS-0001 [1], clause 9.6.53 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | attributes of resource representation  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/RET/011

| TP Id | TP/oneM2M/CSE/SEM/RET/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully the attributes of the <semanticMashupInstance> resource |  |
| Reference | TS-0004 [2], clause 7.4.50.2.2, TS-0034 [6], clause 6.4.3, TS-0001 [1], clause 9.6.54 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupInstance> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | attributes of resource representation  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/RET/012

| TP Id | TP/oneM2M/CSE/SEM/RET/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT returns successfully the attributes of the <semanticMashupResult> resource |  |
| Reference | TS-0004 [2], clause 7.4.52.2.2, TS-0034 [6], clause 6.6.2, TS-0001 [1], clause 9.6.56 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupResult> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | attributes of resource representation  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/RET/013

| TP Id | TP/oneM2M/CSE/SEM/RET/013 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT rejects a semantic query request with semanticsFilter condition set to a non-valid SPARQL query request |  |  |
| Reference | TS-0034 [6], clause 7.5, TS-0004 [1] clauses 7.3.3.19 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 3 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS containing <semanticDescriptor> resource as direct child or as a descendent with accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation } |  |  |

- +---------------------------------------------------------+-------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     |the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Semantic Query Indicator set to "True" and  | |
                     | Result Content shall be set to "semantic content"  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter condition set to a non-valid SPARQL query request  | |
                     |} | |

- +---------------------------------------------------------+-------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+———————————————————+————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/RET/014

| TP Id | TP/oneM2M/CSE/SEM/RET/014 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT accepts a valid semantic query request for target resource and returns a valid query result |  |  |
| Reference | TS-0034 [6], clause 7.5, TS-0004 [1] clauses 7.3.3.19 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 3 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at TARGET_RESOURCE_ADDRESS containing <semanticDescriptor> resource as direct child or as a descendent with accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation } |  |  |

- +---------------------------------------------------------------------+-------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     |the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Semantic Query Indicator set to "True" and  | |
                     | Result Content shall be set to "semantic content"  | |
                     | Filter Criteria containing  | |
                     | semanticsFilter condition set to a valid SPARQL query request  | |
                     |} | |

- +---------------------------------------------------------------------+-------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing string representing Semantic query result  | |
                     |} | |
+———————-+———————————————————————+————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/RET/015

| TP Id | TP/oneM2M/CSE/SEM/RET/015 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT returns successfully the attributes of the <semanticDescriptor> resource except the semanticOpEx attribute |  |  |
| Reference | TS-0004 [2], clause 7.4.34.2.2,, TS-0001 [1], clause 9.6.30 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 3 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticDescriptor> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform RETRIEVE operation } |  |  |

- +----------+------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +----------+------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000 (OK) and  | |
                     | Content containing  | |
                     | attributes of resource representation ensuring that the semanticOpExec attribute is never returned in the response.  | |
                     |} | |
+———————-+———-+————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SEM/CRE/016

| TP Id | TP/oneM2M/CSE/SEM/CRE/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a Response Status Code indicating a "ONTOLOGY_NOT_AVAILABLE" error when referenced ontology of the local linked <semanticDescriptor> resources cannot be retrieved during a semantic validation process |  |
| Reference | TS-0004 [2], clause 7.4.34.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE, PICS_SEMANTIC_VALIDATION |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS and the IUT having privileges to perform RETRIEVE operation on <semanticDescriptor> resource at SEMANTIC_DESCRIPTOR_RESOURCE_ADDRESS containing ontologyRef attribute set to an invalid URI } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | relatedSemantics attribute set to SEMANTIC_DESCRIPTOR_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4118 (ONTOLOGY_NOT_AVAILABLE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/CRE/017

| TP Id | TP/oneM2M/CSE/SEM/CRE/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates a Response Status Code indicating a "INVALID_SEMANTICS" error when any aspect of the semantic validation process fails (according to clause 7.10.3.7.10.4 in TS-0034[6]) |  |
| Reference | TS-0004 [2], clause 7.4.34.2.1, TS-0034 [6], clause 7.10.3, 7.10.4 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE, PICS_SEMANTIC_VALIDATION |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |iIUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 24 (semanticDescriptor) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticDescriptor> resource containing  | |
                     | validationEnable attribute set to "True" and  | |
                     | descriptor attribute set to invalid RDF triples  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4120  | |
                     | (INVALID_SEMANTICS)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————+————-+

##### 7.2.2.15.4	DELETE Operation

###### TP/oneM2M/CSE/SEM/DEL/001

| TP Id | TP/oneM2M/CSE/SEM/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT delete all <semanticMashupInstance> child resources when their parent <semanticMashupJobProfile> receives a delete request |  |
| Reference | TS-0034 [6], clause 6.3.5 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform DELETE operation and <semanticMashupInstance> resource and <semanticMashupInstance> resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticMashupJobProfile> resource representation  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2002 (DELETED) and  | |
                     | the IUT deletes the <semanticMashupJobProfile> resource and  | |
                     | the IUTdeletes thetwo <semanticMashupInstance> child resources  | |
                     |} | |
+———————-+————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SEM/DEL/002

| TP Id | TP/oneM2M/CSE/SEM/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT notifies all <semanticMashupInstance> resources referenced in the smiID attribute when the corresponding <semanticMashupJobProfile> receives a delete request |  |
| Reference | TS-0034 [6], clause 6.3.5 |  |
| Config Id | CF02 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupJobProfile> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform DELETE operation and smiID attribute set to a list of two <semanticMashupInstance> resources located in hosting CSE } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | <semanticMashupJobProfile> resource representation  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT notifies each referenced <semanticMashupJobProfile> resource  | |
                     | located in the hosting CSE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SEM/DEL/003

| TP Id | TP/oneM2M/CSE/SEM/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT deletes successfully <semanticMashupResult> resource |  |
| Reference | TS-0004 [2], clause 7.4.52.2.4, TS-0034 [6], clause 6.6.3, TS-0001 [1], clause 9.6.56 |  |
| Config Id | CF01 |  |
| Parent Release | Release 3 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <semanticMashupResult> resource at TARGET_RESOURCE_ADDRESS containing accessControlPolicyIDs attribute set to allow the AE privileges to perform DELETE operation } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_IDand  | |
                     | no content  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT deletes the TARGET_RESOURCE_ADDRESS resource and   | |
                     | the IUT sends a Response message containing  | |
                     | Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————–+————-+

#### 7.2.2.16	Initial Service Provisioning (ISP)

##### 7.2.2.16.1	BASIC Operation

TP/oneM2M/CSE/ISP/001

| TP Id |  | TP/oneM2M/CSE/ISP/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT accepts a valid M2M node subscription |  |
| Reference |  | TS-0001 [1], clauses 9.6.19, 9.6.20,11.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 3 |  |
| PICS Selection |  | PICS_IN_CSE, PICS_REMOTE_PROVISIONING |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having completed the enrolment function procedure with the AE and the IUT having completed the security provisioning procedure and the IUT having a <m2mserviceSubscriptionProfile > at PARENT_RESOURCE_ADDRESS } |  |  |

- +----------------------------------------+-------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 20_(_ serviceSubscribedNode ) and  | |
                     | Contentcontaining valid serviceSubscribedNode resource   | |
                     |representation  | |
                     |} | |

- +----------------------------------------+-------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED)  | |
                     |} | |
+———————-+—————————————-+————————————————-+————-+

TP/oneM2M/CSE/ISP/002

| TP Id |  | TP/oneM2M/CSE/ISP/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects a non-authorized M2M node subscription |  |
| Reference |  | TS-0001 [1], clauses 9.6.19, 9.6.20, 11.2 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 3 |  |
| PICS Selection |  | PICS_IN_CSE, PICS_REMOTE_PROVISIONING |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having completed the enrolment function procedure with the AE and the IUT having completed the security provisioning procedure and the IUT not containing a <m2m serviceSubscriptionProfile > } |  |  |

- +---------------------------------+----------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 20_(_ serviceSubscribedNode ) and  | |
                     | Content containing valid serviceSubscribedNode resource  | |
                     |} | |

- +---------------------------------+----------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)and  | |
                     | no Content  | |
                     |} | |
+———————-+———————————+—————————————————-+————-+

#### 7.2.2.17	Service Charging and Accounting (CSA)

##### 7.2.2.17.1	Event Based Charging (EBC)

TP/oneM2M/CSE/SCA/EBC/001

| TP Id | TP/oneM2M/CSE/SCA/EBC/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully generates the statistics collection record and send to target AE once the IUT receives an event collection trigger from AE when the eventType attribute is TIMERBASED in the stat collection configuration |  |
| Reference | TS-0001 [1], clause 10.2.11, clause 9.6.24, TS004 [2], clause 7.4.24 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE, PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource containing a child eventConfig resource containing eventType attribute set to TIMERBASED and eventID attribute having value EVENT_ID and the IUT having a statsCollect resource containing eventID attributeset to EVENT_ID and the AE having privileges to perform Create operation } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT is triggered to send statistic collection record  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Notify Request containing  | |
                     | Content (see note) containing  | |
                     | Statistic collection representation  | |
                     |} | |

| NOTE: Content value is not specified. |  |  |
| --- | --- | --- |

TP/oneM2M/CSE/SCA/EBC/002

| TP Id | TP/oneM2M/CSE/SCA/EBC/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully generates the statistics collection record and send to target AE once the IUT receives an event collection trigger from AE when the eventType attribute is TIMERBASED in the stat collection configuration |  |
| Reference | TS-0001 [1], clause 10.2.11, clause 9.6.24, TS004 [2], clause 7.4.24 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE, PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource containing a child eventConfig resource containing eventType attribute set to TIMERBASED and eventID attribute having value EVENT_ID and the IUT having a statsCollect resource containing eventID attributeset to EVENT_ID and the AE having privileges to perform Update operation } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT is triggered to send statistic collection record  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Notify Request containing  | |
                     | Content (see note) containing  | |
                     | statistic collection representation  | |
                     |} | |

| NOTE: Content value is not specified. |  |  |
| --- | --- | --- |

TP/oneM2M/CSE/SCA/EBC/003

| TP Id | TP/oneM2M/CSE/SCA/EBC/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully generates the statistics collection record and send to target AE once the IUT receives an event collection trigger from AE when the eventType attribute is STORAGEBASED in the stat collection configuration |  |
| Reference | TS-0001 [1], clause 10.2.11, clause 9.6.24, TS004 [2], clause 7.4.24 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE, PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource containing a child eventConfig resource containing eventType attribute set to STORAGEBASED and eventID attribute having value EVENT_ID and the IUT having a statsCollect resource containing eventID attributeset to EVENT_ID and the AE having privileges to perform Create operation } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT is triggered to send statistic collection record  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Notify Request containing   | |
                     | Content (see note) containing  | |
                     | statistic collection resource representation  | |
                     |} | |

| NOTE: Content value is not specified. |  |  |
| --- | --- | --- |

TP/oneM2M/CSE/SCA/EBC/004

| TP Id | TP/oneM2M/CSE/SCA/EBC/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully generates the statistics collection record and send to target AE once the IUT receives an event collection trigger from AE when the eventType attribute is DataOperation in the stat collection configuration |  |
| Reference | TS-0001 [1], clause 10.2.11, clause 9.6.24, TS004 [2], clause 7.4.24 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE, PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource containing a child eventConfig resource containing eventType attribute set to DataOperation and eventID attribute having value EVENT_ID and the IUT having a statsCollect resource containing eventID attributeset to EVENT_ID and the AE having privileges to perform Create operation } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT is triggered to send statistic collection record  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Notify Request containing   | |
                     | Content (see note) containing  | |
                     | statistic collection resource representation  | |
                     |} | |

| NOTE: Content value is not specified. |  |  |
| --- | --- | --- |

TP/oneM2M/CSE/SCA/EBC/005

| TP Id | TP/oneM2M/CSE/SCA/EBC/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT does not accept eventEnd time less than eventStart time |  |
| Reference | TS-0001 [1], clause 10.2.11.6, clause 9.6.24, TS004 [2], clause 7.4.24.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE, PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource and the AE having privileges to perform Create operation } |  |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE request containing  | |
                     | eventConfig resource containing  | |
                     | eventEnd time less than eventStart time  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————-+————-+

TP/oneM2M/CSE/SCA/EBC/006

| TP Id | TP/oneM2M/CSE/SCA/EBC/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT verifies dataSize attribute when eventType is set to "Storage based" |  |
| Reference | TS-0001 [1], clause 10.2.11.6, clause 9.6.24, TS004 [2], clause 7.4.24.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE,PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource and the AE having privileges to perform Create operation } |  |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE request containing  | |
                     | eventConfig resource containing  | |
                     | eventType set to STORAGEBASED and  | |
                     | dataSize attribute not set   | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————-+————-+

TP/oneM2M/CSE/SCA/EBC/007

| TP Id | TP/oneM2M/CSE/SCA/EBC/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT generates statistic collection records when multiple <statsCollect> resources are defined |  |
| Reference | TS-0001 [1], clause 10.2.11.10, clause 9.6.24, clause 9.6.25, TS004 [2], clause 7.4.24.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE, PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource containing a child eventConfig resource containing eventType attribute set to TIMERBASED and eventID attribute having value EVENT_ID and the IUT having a statsCollect resource containing eventID attributeset to EVENT_ID and collectingEntity attribute set to AE and the IUT having a statsCollect resource containing eventID attributeset to EVENT_ID and collectingEntity attribute set to AE } |  |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT is triggered to send statistic collection record  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Notify Request containing  | |
                     | Content (see note) containing  | |
                     | statistic collection representation1 and  | |
                     | statistic collection representation2  | |
                     |} | |

| NOTE: Content value is not specified. |  |  |
| --- | --- | --- |

TP/oneM2M/CSE/SCA/EBC/008

| TP Id | TP/oneM2M/CSE/SCA/EBC/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a CDR in an Accounting-Request message when sending to an external charging service |  |
| Reference | TS-0001 [1], clause 12.2.4.2, clause 9.6.24, clause 9.6.25, TS004 [2], clause 7.4.24.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE, PICS_SCA |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a statsConfig resource containing a child eventConfig resource containing eventType attribute set to TIMERBASED and eventID attribute having value EVENT_ID and the IUT having a statsCollect resource containing eventID attributeset to EVENT_ID and the charging messages are sent to an external Charging Server } |  |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT is triggered to send statistic collection record  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |AE <= IUT |
                     | the IUT sends a valid Notify Request containing  | |
                     | Content (see note) containing  | |
                     | statistic collection representation  | |
                     |} | |
+———————-+———————————————————————————————————————+————-+

#### 7.2.2.18	Service Subscription Management(SSM)

##### 7.2.2.18.1	Service Subscription User Profile(SSUP)

TP/oneM2M/CSE/SSM/SSUP/001

| TP Id | TP/oneM2M/CSE/SSM/SSUP/001 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT successfully performs operations on resource serviceSubscribedUserProfile for authorized M2M Service User |  |  |
| Reference | TS-0001 [1], clauses 9.6.68 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 4 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having m2mServiceSubscriptionProfile at TARGET_RESOURCE_ADDRESS containing a child resource and the AE having privileges to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUThaving an authorized M2M Service User AE-ID } |  |  |

- +--+---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized M2M Users AE-ID and  | |
                     |} | |

- +--+---------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status set to RESPONSE_STATUS_CODE and  | |
                     | Content containing  | |
                     | serviceSubscribedUserProfile resource representation (when required)  | |
                     |} | |
+———————-+–+—————————————————————————————————————+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/001_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSUP/001_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SSM/SSUP/001_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SSM/SSUP/002_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SSM/SSUP/002

| TP Id |  | TP/oneM2M/CSE/SSM/SSUP/002 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the operation on serviceSubscribedUserProfile resource when the attributes violate the limits specified in the parent resource |  |
| Reference |  | TS-0001 [1], clauses 9.6.68 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUThaving registered an AE and the IUT having serviceSubscribedUserProfile at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on TARGET_RESOURCE and the IUThaving an authorized M2M Service User AE-ID } |  |  |

- +---------------------------------------------------------------------------------------+--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized M2M Users AE-ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------+--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE)  | |
                     |} | |
+———————-+—————————————————————————————+——————————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/002_CRE | CREATE |
| TP/oneM2M/CSE/SSM/SSUP/002_UPD | UPDATE |

TP/oneM2M/CSE/SSM/SSUP/003

| TP Id |  | TP/oneM2M/CSE/SSM/SSUP/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT successfully restricts the usage of services by a M2M Service User |  |
| Reference |  | TS-0001 [1], clauses 9.6.68 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUThaving registered an AE and the IUT having m2mServiceSubscriptionProfile at TARGET_RESOURCE_ADDRESS containing a child resource containing a child resource the AE having privileges to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUThaving an authorized M2M Service User } |  |  |

- +-----------------------------------------+------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to an authorized M2M Service User AE-ID  | |
                     |} | |

- +-----------------------------------------+------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUSs_CODE and  | |
                     | Content containing  | |
                     | serviceSubscribedUserProfile resource representation  | |
                     |} | |
+———————-+—————————————–+————————————————————————+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/003_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSUP/003_UPD | UPDATE | 2004 (UPDATED) |

TP/oneM2M/CSE/SSM/SSUP/004

| TP Id |  | TP/oneM2M/CSE/SSM/SSUP/004 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects the OPERATION when the associated services of a M2M Service User reach limits provided in the serviceSubscribedUserProfile resource |  |
| Reference |  | TS-0001 [1], clauses 9.6.68 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUThaving registered an AE and the IUT having serviceSubscribedUserProfile at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on } |  |  |

- +----------------------+---------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to an authorized M2M Service User AE-ID  | |
                     |} | |

- +----------------------+---------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE)  | |
                     |} | |
+———————-+———————-+—————————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/004_CRE | CREATE |
| TP/oneM2M/CSE/SSM/SSUP/004_UPD | UPDATE |

TP/oneM2M/CSE/SSM/SSUP/005

| TP Id |  | TP/oneM2M/CSE/SSM/SSUP/005 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT rejects the request when the values of ATTRIBUTE in request exceed the actual resources associated with the Service user |  |
| Reference |  | TS-0001 [1], clauses 9.6.68 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUThaving registered an AE and the IUT having serviceSubscribedUserProfile at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE on TARGET_RESOURCE_ADDRESS } |  |  |

- +--------------------------------------------+----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to an authorized M2M Service User AE-ID  | |
                     |} | |

- +--------------------------------------------+----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4102 (CONTENTS_UNACCEPTABLE)  | |
                     |} | |
+———————-+——————————————–+—————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/005_MNA | maxNumAEs |
| TP/oneM2M/CSE/SSM/SSUP/005_MNN | maxNumNodes |
| TP/oneM2M/CSE/SSM/SSUP/005_MNC | maxNumContainers |
| TP/oneM2M/CSE/SSM/SSUP/005_MNI | maxNumInstancesPerContainer |

TP/oneM2M/CSE/SSM/SSUP/006

| TP Id |  | TP/oneM2M/CSE/SSM/SSUP/006 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that IUT accepts the request when the values of ATTRIBUTE in request do not exceed the actual resources associated with the Service user |  |
| Reference |  | TS-0001 [1], clauses 9.6.68 |  |
| Config Id |  | CF01 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS _CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUThaving registered an AE and the IUT having serviceSubscribedUserProfile at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE on TARGET_RESOURCE**_** ADDRESS } |  |  |

- +--------------------------------------+-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     |} | |

- +--------------------------------------+-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     |} | |
+———————-+————————————–+———————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/006_MNA | maxNumAEs |
| TP/oneM2M/CSE/SSM/SSUP/006_MNN | maxNumNodes |
| TP/oneM2M/CSE/SSM/SSUP/006_MNC | maxNumContainers |
| TP/oneM2M/CSE/SSM/SSUP/006_MNI | maxNumInstancesPerContainer |

TP/oneM2M/CSE/SSM/SSUP/007

| TP Id | TP/oneM2M/CSE/SSM/SSUP/007 |  |
| --- | --- | --- |
| Test objective | Check that IUT successfully accepts the request with the default access control policies when the accessControlPolicyIDs attribute of a resource serviceSubscribedUserProfile created by this M2M Service User is not configured |  |
| Reference | TS-0001 [1], clauses 9.6.68 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS _CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having m2mServiceSubscriptionProfile at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having an authorized M2M Service User AE-ID } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to an authorized M2M Service User AE-ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————-+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/007_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSUP/007_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SSM/SSUP/007_UPD | UPDATE | 2004 (UPDATED) |

TP/oneM2M/CSE/SSM/SSUP/008

| TP Id | TP/oneM2M/CSE/SSM/SSUP/008 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that IUT rejects OPERATION request if service subscription profile associated with the M2M service user/AE is not available |  |  |
| Reference | TS-0004 [2], clauses 7.3.2 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 4 |  |  |
| PICS Selection | PICS_IN_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT not having a <serviceSubscribedUserProfile_>_ associated with the M2M service user at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform operation on TARGET_RESOURCE_ADDRESS } |  |  |

- +---------------+----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from an AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | a valid content  | |
                     |} | |

- +---------------+----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41xx (UNAUTHORIZED_USER)  | |
                     |} | |
+———————-+—————+———————————————————————————————————————-+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSUP/008_RET | RETRIEVE |
| TP/oneM2M/CSE/SSM/SSUP/008_UPD | UPDATE |
| TP/oneM2M/CSE/SSM/SSUP/008_DEL | DELETE |

##### 7.2.2.18.2	Service Subscription Profile(SSP)

###### 7.2.2.18.2.1	Basic Operation

TP/oneM2M/CSE/SSM/SSP/001

| TP Id | TP/oneM2M/CSE/SSM/SSP/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs operations on resource m2mServiceSubscriptionProfile for authorized AE |  |
| Reference | TS-0001 [1], clauses 9.6.19 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+————————————————————————————————–+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSP/001_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/001_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SSM/SSP/001_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SSM/SSP/001_DEL | DELETE | 2002 (DELETED) |

TP/oneM2M/CSE/SSM/SSP/002

| TP Id | TP/oneM2M/CSE/SSM/SSP/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE not having service subscription tries to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS |  |
| Reference | TS-0004 [2], clause 7.3.2.1, clause 7.3.2.7 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT not having a m2mServiceSubscriptionProfile resource For the AE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4128  | |
                     |(SERVICE_SUBSCRIPTION_NOT_ESTABLISHED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSP/002_CRE | CREATE |
| TP/oneM2M/CSE/SSM/SSP/002_RET | RETRIEVE |
| TP/oneM2M/CSE/SSM/SSP/002_UPD | UPDATE |
| TP/oneM2M/CSE/SSM/SSP/002_DEL | DELETE |

TP/oneM2M/CSE/SSM/SSP/003

| TP Id | TP/oneM2M/CSE/SSM/SSP/003 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the AE having inactive service subscription tries to perform a request |  |  |
| Reference | TS-0004 [2], clause 7.3.2.1, clause 7.3.2.7 |  |  |
| Config Id | CF01 |  |  |
| Parent Release | Release 4 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having a m2mServiceSubscriptionProfile resource For the AE containing status attribute set to INACTIVE } |  |  |

- +----+--------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE To set to  | |
                     |TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +----+--------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4139  | |
                     |(SERVICE_SUBSCRIPTION_NOT_ACTIVE)  | |
                     |} | |
+———————-+—-+——————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSP/003_CRE | CREATE |
| TP/oneM2M/CSE/SSM/SSP/003_RET | RETRIEVE |
| TP/oneM2M/CSE/SSM/SSP/003_UPD | UPDATE |
| TP/oneM2M/CSE/SSM/SSP/003_DEL | DELETE |

TP/oneM2M/CSE/SSM/SSP/004

| TP Id | TP/oneM2M/CSE/SSM/SSP/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT processes the request when the AE having an active service subscription tries to perform a request |  |
| Reference | TS-0004 [2], clause 7.3.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a m2mServiceSubscriptionProfile resource For the AE containing status attribute set to ACTIVE } |  |

- +--------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request from AE  | |
                     | To set to TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK)  | |
                     |} | |
+———————-+——————————————————————————————————–+————-+

TP/oneM2M/CSE/SSM/SSP/005

| TP Id | TP/oneM2M/CSE/SSM/SSP/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the authorized AE exceeds the request rate limit specified in its service subscription profile |  |
| Reference | TS-0004 [2], clause 7.3.2.x |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a m2mServiceSubscriptionProfile resource For the AE containing maxRequestRate attribute set to MAX_REQUEST_RATE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request  | |
                     | at a rate higher than MAX_REQUEST_RATE  | |
                     | from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 413x  | |
                     |(SERVICE_SUBSCRIPTION_LIMITS_EXCEEDED)  | |
                     |} | |
+———————-+——————————————————————————————————————————–+————-+

| TP Id | OPERATION |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSP/005_CRE | CREATE |
| TP/oneM2M/CSE/SSM/SSP/005_RET | RETRIEVE |
| TP/oneM2M/CSE/SSM/SSP/005_UPD | UPDATE |
| TP/oneM2M/CSE/SSM/SSP/005_DEL | DELETE |

TP/oneM2M/CSE/SSM/SSP/006

| TP Id | TP/oneM2M/CSE/SSM/SSP/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT processes the request when the authorized AE sends requests within the request rate limit specified in its service subscription profile |  |
| Reference | TS-0004 [2], clause 7.3.2.x |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a m2mServiceSubscriptionProfile resource For the AE containing maxRequestRate attribute set to MAX_REQUEST_RATE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid RETRIEVE Request  | |
                     | at a rate less than MAX_REQUEST_RATE  | |
                     | from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2000(OK)  | |
                     |} | |
+———————-+———————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SSM/SSP/007

| TP Id | TP/oneM2M/CSE/SSM/SSP/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT responds with an error when the authorized AE tries to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS and it exceeds the limit for ATTRIBUTE_NAME in its m2mServiceSubscriptionProfile resource |  |
| Reference | TS-0004 [2], clause 7.3.3.x |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a m2mServiceSubscriptionProfile resource For the AE containing ATTRIBUTE_NAME attribute set to MAX_VALUE and the IUT having the current limit with CURRENT_VALUE corresponding to this ATTRIBUTE_NAME_ equal MAX_VALUE } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request  | |
                     | from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 413x (SERVICE_SUBSCRIPTION_LIMITS_EXCEEDED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE_NAME | OPERATION |
| --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSP/007_MAE | maxNumAEs | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MND | maxNumNodes | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MNB | maxNumBytes | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MNU | maxNumUsers | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MNC | maxNumContainers | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MIC_CRE | maxNumInstancesPerContainer | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MIC_UPD | maxNumInstancesPerContainer | UPDATE |
| TP/oneM2M/CSE/SSM/SSP/007_MNT | maxNumTimeSeries | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MMG_CRE | maxMembersPerGroup | CREATE |
| TP/oneM2M/CSE/SSM/SSP/007_MMG_UPD | maxMembersPerGroup | UPDATE |
| TP/oneM2M/CSE/SSM/SSP/007_MNF | maxNumFlexContainers | CREATE |

TP/oneM2M/CSE/SSM/SSP/008

| TP Id | TP/oneM2M/CSE/SSM/SSP/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT processes the request when the authorized AE having not exceeded the limit for ATTRIBUTE_NAME in its m2mServiceSubscriptionProfile resource tries to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS |  |
| Reference | TS-0004 [2], clause 7.3.3.x |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a m2mServiceSubscriptionProfile resource For the AE containing ATTRIBUTE_NAME_ attribute set to MAX_VALUE and the IUT having the current limit corresponding to this ATTRIBUTE_NAME with CURRENT_VALUE less than MAX_VALUE } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request  | |
                     | from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => CSE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE   | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSP/008_MAE | maxNumAEs | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MND | maxNumNodes | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MNB | maxNumBytes | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MNU | maxNumUsers | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MNC | maxNumContainers | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MIC_CRE | maxNumInstancesPerContainer | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MIC_UPD | maxNumInstancesPerContainer | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MNT | maxNumTimeSeries | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MMG_CRE | maxMembersPerGroup | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MMG_UPD | maxMembersPerGroup | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SSM/SSP/008_MNF | maxNumFlexContainers | CREATE | 2001 (CREATED) |

###### 7.2.2.18.2.2	UPDATE

TP/oneM2M/CSE/SSM/SSP/UPD/001

| TP Id | TP/oneM2M/CSE/SSM/SSP/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the value of the attributes activationTime , status , deactivationTime in <m2mserviceSubscriptionProfile> resource when activate attribute is set to TRUE in resource representation of UPDATE request |  |
| Reference | TS-0001 [1], clauses 9.6.19, TS-0004[2], clauses 7.4.19.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform UPDATE on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resource containing  | |
                     | activate_ attribute set to TRUE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT updates the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resourcecontaining  | |
                     | activate attribute set to TRUE  | |
                     | activation Time attribute set to Current Time  | |
                     | deactivationTime_ attribute set to Null  | |
                     | status attribute set to ACTIVE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/SSM/SSP/UPD/002

| TP Id | TP/oneM2M/CSE/SSM/SSP/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates the value of the attributes de_activationTime_ , status when activate attribute is set to FALSE in resource representation of UPDATE request |  |
| Reference | TS-0001 [1], clauses 9.6.19, TS-0004[2], clauses 7.4.19.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform UPDATE on the resource TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resource containing  | |
                     | activate attribute set to FALSE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT updates the TARGET_RESOURCE_ADDRESS resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resourcecontaining  | |
                     | activate attribute set to FALSE  | |
                     | deactivation Time attribute set to Current Time  | |
                     | status attribute set to INACTIVE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SSM/SSP/UPD/003

| TP Id | TP/oneM2M/CSE/SSM/SSP/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT overrides the value of the attribute ATTRIBUTE_NAME with the current limit when the value provided in the resource representation of the UPDATE request is less than the current limit. |  |
| Reference | TS-0001 [1], clauses 9.6.19, TS-0004[2], clauses 7.4.19.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a <m2mServiceSubscriptionProfile> resource at TARGET_RESOURCE_ADDRESS containing ATTRIBUTE_NAME attribute set to MAX_VALUE and the IUT having the current limit VALUE_1 corresponding to this ATTRIBUTE_NAME and the AE having privileges to perform UPDATE on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resource containing  | |
                     | ATTRIBUTE_NAME attribute set to value less than  | |
                     | VALUE_1.  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resource containing  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_1  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————–+————-+

| TP Id | ATTRIBUTE_NAME |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MAE | maxNumAEs |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MND | maxNumNodes |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MNB | maxNumBytes |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MNU | maxNumUsers |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MRR | maxRequestRate |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MNC | maxNumContainers |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MIC | maxNumInstancesPerContainer |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MNT | maxNumTimeSeries |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MIT | maxNumInstancesPerTimeSeries |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MMG | maxMembersPerGroup |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MNR | maxNotificationRate |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MNF | maxNumFlexContainers |
| TP/oneM2M/CSE/SSM/SSP/UPD/003_MIF | maxNumInstancesPerFlexContainer |

TP/oneM2M/CSE/SSM/SSP/UPD/004

| TP Id | TP/oneM2M/CSE/SSM/SSP/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates successfully the value of the attribute ATTRIBUTE_NAME of the TARGET_RESOURCE_ADDRESS resource, when the value provided in the resource representation of the UPDATE request is more than the current limit. |  |
| Reference | TS-0001 [1], clauses 9.6.19, TS-0004[2], clauses 7.4.19.2.3 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a m2mServiceSubscriptionProfile resource at TARGET_RESOURCE_ADDRESS containing ATTRIBUTE_NAME attribute set to MAX_VALUE and the IUT having the current limit with CURRENT_VALUE corresponding to this ATTRIBUTE_NAME and the AE having privileges to perform UPDATE on the resource TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resource containing  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_1 more than  | |
                     | CURRENT_VALUE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED) and  | |
                     | Content containing  | |
                     | m2mServiceSubscriptionProfile resource containing  | |
                     | ATTRIBUTE_NAME attribute set to VALUE_1  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | ATTRIBUTE_NAME |
| --- | --- |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MAE | maxNumAEs |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MND | maxNumNodes |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MNB | maxNumBytes |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MNU | maxNumUsers |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MRR | maxRequestRate |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MNC | maxNumContainers |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MIC | maxNumInstancesPerContainer |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MNT | maxNumTimeSeries |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MIT | maxNumInstancesPerTimeSeries |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MMG | maxMembersPerGroup |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MNR | maxNotificationRate |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MNF | maxNumFlexContainers |
| TP/oneM2M/CSE/SSM/SSP/UPD/004_MIF | maxNumInstancesPerFlexContainer |

##### 7.2.2.18.3	Service Subscribed App Rule(SSAR)

TP/oneM2M/CSE/SSM/SSAR/001

| TP Id | TP/oneM2M/CSE/SSM/SSAR/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs operations on resource seriviceSubscribedAppRule for authorized AE |  |
| Reference | TS-0001 [1], clauses 9.6.29, TS-0004, clauses 7.4.29 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+———————————————————————————————-+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSAR/001_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSAR/001_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SSM/SSAR/001_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SSM/SSAR/001_DEL | DELETE | 2002 (DELETED) |

##### 7.2.2.18.4	Service Subscribed Node(SSN)

###### 7.2.2.18.4.1	Basic Operation

TP/oneM2M/CSE/SSM/SSN/001

| TP Id | TP/oneM2M/CSE/SSM/SSN/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully performs operations on resource seriviceSubscribedNode for authorized AE |  |
| Reference | TS-0001 [1], clauses 9.6.20, TS-0004, clauses 7.4.20 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+——————————————————————————————-+————-+

| TP Id | OPERATION | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/SSM/SSN/001_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/SSM/SSN/001_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/SSM/SSN/001_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/SSM/SSN/001_DEL | DELETE | 2002 (DELETED) |

###### 7.2.2.18.4.2	CREATE Operation

TP/oneM2M/CSE/SSM/SSN/CRE/001

| TP Id | TP/oneM2M/CSE/SSM/SSN/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects CREATE request If the ruleLinks attribute is present in the resource representation, but the CSE-ID attribute is not present in the resource representation of <serviceSubscribedNode> resource |  |
| Reference | TS-0001 [1], clauses 9.6.20, TS-0004 [2], clause 7.4.20.2.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_IN_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE OPERATION on the resource TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to authorized AE-ID  | |
                     | Content containing  | |
                     | serviceSubscribedNode resource containing  | |
                     | ruleLinks attribute set to valid value and  | |
                     | CSE-ID attribute Not set  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD Request)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————+————-+

#### 7.2.2.19	Primitive Profile (PP)

##### 7.2.2.19.1	Basic Operation

###### TP/oneM2M/CSE/PP/001

| TP Id | TP/oneM2M/CSE/PP/001 |  |
| --- | --- | --- |
| Test objective | Check that IUT applies the referenced <primitiveProfile> over the request messages by adding the applicable parameters/attributes when the request message contains a valid Primitive Profile Identifier parameter and the referenced profile is accessible to the Hosting CSE |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having a <primitiveProfile> resource at PRIMITIVE_PROFILE_ADDRESS containing additions attribute having list of attributes to be added and AE having Retrieve privileges for the <primitiveProfile> resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | The IUT applies the profile by adding attributes defined in additions  | |
                     |attribute  | |
                     | and the IUT sends a valid Response to the AE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/PP/001_CRE | CREATE |
| TP/oneM2M/CSE/PP/001_RET | RETRIEVE |
| TP/oneM2M/CSE/PP/001_UPD | UPDATE |
| TP/oneM2M/CSE/PP/001_DEL | DELETE |

###### TP/oneM2M/CSE/PP/002

| TP Id | TP/oneM2M/CSE/PP/002 |  |
| --- | --- | --- |
| Test objective | Check that IUT applies the referenced < primitiveProfile_>_ over the request messages by deleting the applicable parameters/attributes when the request message contains a valid PrimitiveProfileIdentifier parameter and the referenced profile is accessible to the Hosting CSE |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ at PRIMITIVE_PROFILE_ADDRESS containing deletions attribute having list of attributes to be deleted and AE having Retrieve privileges for the <primitiveProfile> resource } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | The IUT applies the profile by deleting attributes defined in deletions  | |
                     | and the IUT sends a valid Response to the AE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/PP/002_CRE | CREATE |
| TP/oneM2M/CSE/PP/002_RET | RETRIEVE |
| TP/oneM2M/CSE/PP/002_UPD | UPDATE |
| TP/oneM2M/CSE/PP/002_DEL | DELETE |

###### TP/oneM2M/CSE/PP/003

| TP Id | TP/oneM2M/CSE/PP/003 |  |
| --- | --- | --- |
| Test objective | Check that IUT applies the referenced < primitiveProfile_>_ over the request messages by replacing the applicable parameters/attributes when the request message contains a valid PrimitiveProfileIdentifier parameter and the referenced profile is accessible to the Hosting CSE |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ at PRIMITIVE_PROFILE_ADDRESS containing additions attribute having list of attributes to be replaced and AE having Retrieve privileges for the <primitiveProfile> resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | the IUT receives a valid OPERATION Request from AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | The IUT applies the profile by replacing attributes defined in additions  | |
                     |attribute  | |
                     | and the IUT sends a valid Response to the AE  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/PP/003_CRE | CREATE |
| TP/oneM2M/CSE/PP/003_RET | RETRIEVE |
| TP/oneM2M/CSE/PP/003_UPD | UPDATE |
| TP/oneM2M/CSE/PP/003_DEL | DELETE |

###### TP/oneM2M/CSE/PP/004

| TP Id | TP/oneM2M/CSE/PP/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the request when the < primitiveProfile_>_ resource referenced by the PrimitiveProfileIdentifier parameter is not present |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the retrieve privilege for <primitiveProfile> resource to perform OPERATION_ on TARGET_RESOURCE_ADDRESS and the IUT not having <primitiveProfile> resource } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | The IUT receives a valid OPERATION Request from AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not apply the primitive profile  | |
                     | and the IUTsends a valid Response containing  | |
                     | Response Status Code set to 41XX  | |
                     |(PRIMITIVE_PROFILE_NOT_ACCESSIBLE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/PP/004_CRE | CREATE |
| TP/oneM2M/CSE/PP/004_RET | RETRIEVE |
| TP/oneM2M/CSE/PP/004_UPD | UPDATE |
| TP/oneM2M/CSE/PP/004_DEL | DELETE |

###### TP/oneM2M/CSE/PP/005

| TP Id | TP/oneM2M/CSE/PP/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the request when the originator does not have retrieve privileges for < primitiveProfile_>_ resource referenced by the PrimitiveProfileIdentifier parameter |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2] 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a < primitiveProfile_>_ at PRIMITIVE_PROFILE_ADDRESS and the AE not having the retrieve privilege for <primitiveProfile> resource to perform OPERATION_ on TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | The IUT receives a valid OPERATION Request from AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not apply the primitive profile  | |
                     | and the IUTsends a valid Response containing  | |
                     | Response Status Code set to 41XX  | |
                     |(PRIMITIVE_PROFILE_NOT_ACCESSIBLE)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/PP/005_CRE | CREATE |
| TP/oneM2M/CSE/PP/005_RET | RETRIEVE |
| TP/oneM2M/CSE/PP/005_UPD | UPDATE |
| TP/oneM2M/CSE/PP/005_DEL | DELETE |

###### TP/oneM2M/CSE/PP/006

| TP Id | TP/oneM2M/CSE/PP/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully processes the request without applying the profile when the attributes in the request do not match with the referenced < primitiveProfile_>_ resource attributes |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 6.6.3.5, 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a target resource at TARGET_RESOURCE_ADDRESS and the AE having the retrieve privilege for resource <primitiveProfile> to perform OPERATION_ on TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ resource at PRIMITIVE_PROFILE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | The IUT receives a valid OPERATION Request from AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | not matching with the <primitiveProfile> resource attributes  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT does not apply the primitive profile  | |
                     | and sends the request to TARGET_RESOURCE_ADDRESS  | |
                     | when the process is completed  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/PP/006_CRE | CREATE |
| TP/oneM2M/CSE/PP/006_RET | RETRIEVE |
| TP/oneM2M/CSE/PP/006_UPD | UPDATE |
| TP/oneM2M/CSE/PP/006_DEL | DELETE |

###### TP/oneM2M/CSE/PP/007

| TP Id | TP/oneM2M/CSE/PP/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of <primitiveProfile> resource request when the request parameters or resource attributes defined in the additions and deletions attributes of the referenced <primitiveProfile> resource are not valid |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 60? (primitiveProfile)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code s et to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/PP/008

| TP Id | TP/oneM2M/CSE/PP/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of <primitiveProfile> resource when the request parameters or resource attributes defined in the additions and deletions attributes of the referenced <primitiveProfile> resource are not valid |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on PRIMITIVE_PROFILE_ADDRESS and the IUT having a < primitiveProfile_>_ resource at PRIMITIVE_PROFILE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | The IUT receives a valid UPDATE Request from AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | To set to PRIMITIVE_PROFILE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT=> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/PP/009

| TP Id | TP/oneM2M/CSE/PP/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of a <primitiveProfile> resource when a mutual exclusivity check is failed on the names of the attributes or parameters specified in the additions and, deletions attributes |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | The IUT receives a valid CREATE Request from AEcontaining | |
                     | Resource Type set to 60? (primitiveProfile) and  | |
                     | Content containing  | |
                     | primitiveProfile resource containing  | |
                     | additions attribute and  | |
                     | deletions attribute  | |
                     | not being mutually exclusive  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/PP/010

| TP Id | TP/oneM2M/CSE/PP/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of <primitiveProfile> resource when a mutual exclusivity check is failed on the names of the attributes or parameters specified in the additions and deletions attributes |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ resource at PRIMITIVE_PROFILE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | The IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to PRIMITIVE_PROFILE_ADDRESS and  | |
                     | Content containing  | |
                     | additions attribute and  | |
                     | deletions attribute  | |
                     | not being mutually exclusive  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT=> AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/PP/011

| TP Id | TP/oneM2M/CSE/PP/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of a <primitiveProfile > resource when a request contains non permissible or mandatory parameters specified in the additions and deletions attributes |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | The IUTreceives a valid CREATE Request from AE containing  | |
                     | To set to PRIMITIVE_PROFILE_ADDRESS and  | |
                     | Content containing  | |
                     | primitiveProfile resource containing  | |
                     | additions attribute indicating ADD_PARAMETER and  | |
                     | deletions attribute indicating DEL_PARAMETER   | |
                     | } | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | The IUT sends a valid Response containing  | |
                     | Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/PP/012

| TP Id | TP/oneM2M/CSE/PP/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE operation on resource <primitiveProfile> when a request contains non permissible or mandatory parameters specified in the additions and deletions attributes |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ at PRIMITIVE_PROFILE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | The IUTreceives a valid UPDATE Request from AE containing  | |
                     | To set to PRIMITIVE_PROFILE_ADDRESS and  | |
                     | Content containing  | |
                     | primitiveProfile resource containing  | |
                     | additions attribute indicating ADD_PARAMETER and  | |
                     | deletions attribute indicating DEL_PARAMETER   | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     | REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/PP/013

| TP Id | TP/oneM2M/CSE/PP/013 |  |
| --- | --- | --- |
| Test objective | Check that IUT successfully applies the < primitiveProfile_>_ to the notification (Notify request primitive) in case of nonBlockingRequestAsynch request having valid attribute PrimitiveProfileIdentifier |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS0004 [2], clause 7.5.1.2.5 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a resource representation at TARGET_RESOURCE_ADDRESS and the IUT having received a valid nonBlockingRequestAsynch request containing a valid Primitive Profile Identifier and the IUT having a < primitiveProfile_>_ at PRIMITIVE_PROFILE_ADDRESS containing additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | The requested operation is completed  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | The IUT applies the < primitiveProfile_>_ to Notify request primitive by  | |
                     | adding ADD_PARAMETER  | |
                     | and deleting DEL_PARAMETER  | |
                     | and the IUT sends a valid NOTIFY request containing  | |
                     | content representation  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/PP/014

| TP Id | TP/oneM2M/CSE/PP/014 |  |
| --- | --- | --- |
| Test objective | Check that IUT successfully applies the < primitiveProfile_>_ to the response in case of nonBlockingRequestSynch request having valid Primitive Profile Identifier |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS0004 [2], clause 7.2.2.2 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having the <request> resource at REQUEST_RESOURCE_ADDRESS and the IUT having received a valid nonBlockingRequestSynch request containing a valid Primitive Profile Identifier and the IUT having a < primitiveProfile_>_ at PRIMITIVE_PROFILE_ADDRESS containing additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | The IUTreceives a valid RETRIEVE Request from AE containing  | |
                     | To set to REQUEST_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT |
                     | the IUT applies the referenced < primitiveProfile_>_ to the response by  | |
                     | adding ADD_PARAMETER  | |
                     | and deleting DEL_PARAMETER  | |
                     | and the IUT sends a valid Response containing  | |
                     | Content containing  | |
                     | Response Status Codeset to 2XXXand Response Primitive  | |
                     |representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/PP/015

| TP Id | TP/oneM2M/CSE/PP/015 |  |
| --- | --- | --- |
| Test objective | Check that IUT successfully applies the < primitiveProfile_>_ to the notification request generated for the <subscription> resource |  |
| Reference | TS-0001 [1], clause 9.6.8, clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF03 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a valid < subscription_>_ resource containing a primitiveProfileID attribute set to PRIMITIVE_PROFILE_ID and AE having subscribed to the < subscription_>_ resource and the IUT having a valid < primitiveProfile_>_ resource containing resourceID attribute set to PRIMITIVE_PROFILE_IDand additions attribute indicating ADD_PARAMETER and deletions attribute indicating DEL_PARAMETER } |  |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | The IUT generates a valid notification request for a the _<subscription>  | |
                     |resource containing primitiveProfileID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT |
                     | The IUT applies the < primitiveProfile_>_ resource referenced in the  | |
                     | primitiveProfileID attribute to the generated notification by  | |
                     | adding ADD_PARAMETER  | |
                     | and deleting DEL_PARAMETER  | |
                     | and the IUT sends a valid NOTIFY request containing  | |
                     | content representation  | |
                     |} | |
+———————-+——————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/PP/016

| TP Id | TP/oneM2M/CSE/PP/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT retargets the request to the hosting CSE without applying the < primitiveProfile_>_ when the To parameter contains the hosting CSE-ID which is known by the IUT |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered to a Hosting CSE (H-CSE) and the IUT having registered the AE and the Hosting CSE having create a RESOURCE on TARGET_RESOURCE_ADDRESS and the AE having privileges to perform OPERATION on the Hosting CSE } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | the IUT receives a valid OPERATION Request from the AE containing  | |
                     | a valid Primitive Profile Identifier and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => H-CSE |
                     | The IUT forwards the OPERATION Request to Hosting CSE without  | |
                     | applying the < primitiveProfile_>_ resource referenced in the  | |
                     | Primitive Profile Identifier  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/PP/016_CRE | CREATE |
| TP/oneM2M/CSE/PP/016_RET | RETRIEVE |
| TP/oneM2M/CSE/PP/016_UPD | UPDATE |
| TP/oneM2M/CSE/PP/016_DEL | DELETE |

###### TP/oneM2M/CSE/PP/017

| TP Id | TP/oneM2M/CSE/PP/017 |  |
| --- | --- | --- |
| Test objective | Check that IUT applies the referenced < primitiveProfile_>_ over the response messages by adding the applicable parameters/attributes when the originating AE specified a valid PrimitiveProfileIdentifier parameter in the corresponding request |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.12 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ resource containing resourceID attribute set to PRIMITIVE_PROFILE_IDand a valid additions attribute having list of attributes to be added and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUThaving received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT receives a valid response from a resource containing  | |
                     | To set to AE_ID and  | |
                     | Fromset to TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT applies the referenced primitive profile by adding attributes  | |
                     |defined in additions attribute  | |
                     | and the IUTsends a valid Response to the AE  | |
                     | containing valid RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/PP/017_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/PP/017_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/PP/017_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/PP/017_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/PP/018

| TP Id | TP/oneM2M/CSE/PP/018 |  |
| --- | --- | --- |
| Test objective | Check that IUT applies the referenced < primitiveProfile_>_ over the response messages by deleting the applicable parameters/attributes when the originating AE specified a valid PrimitiveProfileIdentifier parameter in the corresponding request |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ resource containing resourceID attribute set to PRIMITIVE_PROFILE_IDand deletions attribute having list of attributes to be deleted and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT receives a valid response from a resource containing  | |
                     | To set to AE_ID and  | |
                     | Fromset to TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT applies the referenced primitive profile by deleting attributes defined in deletions attribute  | |
                     | and the IUTsends a valid Response to the AE  | |
                     | containing valid RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/PP/018_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/PP/018_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/PP/018_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/PP/018_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/PP/019

| TP Id | TP/oneM2M/CSE/PP/019 |  |
| --- | --- | --- |
| Test objective | Check that IUT applies the referenced < primitiveProfile_>_ over the response messages by replacing the applicable parameters/attributes when the originating AE specified a valid PrimitiveProfileIdentifier parameter in the corresponding request |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ resource containing resourceID attribute set to PRIMITIVE_PROFILE_ID and additions attribute having list of attributes to be replaced and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT receives a valid response from a resource containing  | |
                     | To set to AE_ID and  | |
                     | Fromset to TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT applies the referenced primitive profile by replacing attributes defined in additions attribute  | |
                     | and the IUTsends a valid Response to the AE  | |
                     | containing valid RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | RESOURCE_TYPE | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/PP/019_CRE | CREATE | 2001 (CREATED) |
| TP/oneM2M/CSE/PP/019_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/PP/019_UPD | UPDATE | 2004 (UPDATED) |
| TP/oneM2M/CSE/PP/019_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/PP/020

| TP Id | TP/oneM2M/CSE/PP/020 |  |
| --- | --- | --- |
| Test objective | Check that IUT successfully applies the <primitiveProfile> to the representation of the requested resource returned in the Content parameter of the response and If the Content parameter of the response includes nested representations of child resource(s) and their descendant(s), the Hosting CSE shall not apply the profile to these child resource(s) or descendant(s) |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.12 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { The IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the IUT having a RESOURCE at TARGET_RESOURCE_ADDRESS and the IUT having a < primitiveProfile_>_ resource containing resourceID attribute set to PRIMITIVE_PROFILE_ID and modifications attribute having list of attributes to be modified and AE having the privilege to perform OPERATION on TARGET_RESOURCE_ADDRESS and the IUT having received a valid OPERATION Request from the AE containing a valid Primitive Profile Identifier and To set to TARGET_RESOURCE_ADDRESS and From set to AE_ID } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT |
                     | the IUT receives a valid response from a resource containing  | |
                     | To set to AE_ID and  | |
                     | Fromset to TARGET_RESOURCE_ADDRESS and  | |
                     | Result Content set to 4 (attributes and child resources) and  | |
                     | Content containing  | |
                     | Resource representation containing  | |
                     | nested representations of child resources and their descendants  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT applies the referenced primitive profile on the requested resource  | |
                     |representation and ignore the child resource(s) or descendant(s) of the  | |
                     |resource representation.  | |
                     | and the IUTsends a valid Response to the AE  | |
                     | containing valid RESPONSE_STATUS_CODE  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | RESOURCE_TYPE | RESPONSE_STATUS_CODE |
| --- | --- | --- |
| TP/oneM2M/CSE/PP/020_RET | RETRIEVE | 2000 (OK) |
| TP/oneM2M/CSE/PP/020_DEL | DELETE | 2002 (DELETED) |

###### TP/oneM2M/CSE/PP/021

| TP Id | TP/oneM2M/CSE/PP/021 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of <primitiveProfile> resource when the request parameters or resource attributes defined in the additions attribute have complex data type definition |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.2.1.2 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 60? (primitiveProfile)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | primitiveProfile resource containing   | |
                     | additions attribute indicating ADD_PARAMTER  | |
                     | having complex data type  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code s et to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/PP/022

| TP Id | TP/oneM2M/CSE/PP/022 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation of a <primitiveProfile> resource when any parameters specified in the additions attribute do not match the defined data types for the parameters |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | the IUTreceives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 60? (primitiveProfile) and  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing   | |
                     | primitiveProfile resource containing  | |
                     | additions attribute containing  | |
                     | an invalid ADD_PARAMETER attribute  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/PP/023

| TP Id | TP/oneM2M/CSE/PP/023 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update request of a <primitiveProfile> resource when any parameters specified in the additions attribute do not match the defined data types for the parameters |  |
| Reference | TS-0001 [1], clause 9.6.73 and clause 10.2.25, TS-0004 [2], clause 7.3.3.1 |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a hosting CSE and the IUT having registered an AE and the AE having the privilege to perform UPDATE operation on PRIMITIVE_PROFILE_ADDRESS and the IUT having a < primitiveProfile_>_ resource at PRIMITIVE_PROFILE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | the IUTreceives a valid UPDATE Request from AE containing  | |
                     | To set to PRIMITIVE_PROFILE_ADDRESSand  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | primitiveProfile resource containing   | |
                     | additions attribute containing  | |
                     | an invalid ADD_PARAMETER attribute  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41YY (PRIMITIVE_PROFILE_BAD  | |
                     |REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+————-+

#### 7.2.2.20	Transaction Management(TMG)

##### 7.2.2.20.1	CREATE Operation

###### TP/oneM2M/CSE/TMG/CRE/001

| TP Id | TP/oneM2M/CSE/TMG/CRE/001 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT sets value of transactionControl to INITIAL during the creation of <transactionMgmt> resource |  |  |
| Reference | TS-0004 [2], clause 7.4.60.2.1 Recv-6.4 (a) |  |  |
| Parent Release | Release 4 |  |  |
| Config Id | CF01 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE to perform CREATE operation } |  |  |

- +-+--------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 50 (transactionMgmt) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | transactionMgmt resource containing  | |
                     | requestPrimitives attribute set to  | |
                     | LIST_OF_REQUESTPRIMITIVES  | |
                     |} | |

- +-+--------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT creates the transactionMgmt resource  | |
                     | and the IUT sends Response Status Code set to 2001 (CREATED)  | |
                     | Content containing  | |
                     |transactionMgmt resource containing  | |
                     | transactionControl attribute set to  | |
                     | INITIAL  | |
                     |} | |
+———————-+-+——————————————————————————————————–+————-+

###### TP/oneM2M/CSE/TMG/CRE/002

| TP Id | TP/oneM2M/CSE/TMG/CRE/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects creation of <transactionMgmt> resource when From parameters contained in the requestPrimitives attribute of the received <transactionMgmt> resource is not AE1 |  |
| Reference | TS-0004 [2], clause 7.4.60.2.1 Recv-6.4 (a) |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE1 to perform CREATE operation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 50 (transactionMgmt) and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transactionMgmt resource containing  | |
                     | requestPrimitives attribute set to LIST_OF_REQUESTPRIMITIVES containing  | |
                     | From set to AE2  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the transactionMgmt resource and  | |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/TMG/CRE/003

| TP Id | TP/oneM2M/CSE/TMG/CRE/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets value of transactionControl to LOCK during the creation of <transaction> resource |  |
| Reference | TS-0004 [2], clause 7.4.61.2.1, Recv-6.4 |  |
| Parent Release | Release 4 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource PARENT_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation } |  |

- +-------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 51 (transaction) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | transaction resource containing  | |
                     | requestPrimitive attribute set to REQUESTPRIMITIVE  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT creates the transaction resource  | |
                     | and the IUT sends Response Status Code set to 2001 (CREATED)  | |
                     | Content containing  | |
                     |transaction resource containing  | |
                     | transactionControl attribute set to  | |
                     | LOCK  | |
                     |} | |
+———————-+————————————————————————————————-+————-+

###### TP/oneM2M/CSE/TMG/CRE/004

| TP Id | TP/oneM2M/CSE/TMG/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects creation of <transaction> resource when From parameters contained in the requestPrimitive attribute of the received <transaction> resource is not AE1 |  |
| Reference | TS-0004 [2], clause 7.4.60.2.1 Recv-6.4 (a) |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having registered the AE2 and the IUT having a resource PARENT_RESOURCE_ADDRESS allowing the AE1 to perform CREATE operation } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 51 (transaction) and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transaction resource containing  | |
                     | requestPrimitive attribute set to REQUESTPRIMITIVE  | |
                     |containing  | |
                     | From set to AE2  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT does not create the transaction resource   | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/TMG/CRE/005

| TP Id | TP/oneM2M/CSE/TMG/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates a <transaction> resource as child for each target resource contained in the requestPrimitives attribute when receiving a valid transactionMgmt create request |  |
| Reference | TS-0001 [2], clause 10.2.18.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource PARENT_RESOURCE_ADDRESS and the IUT having a resource TARGET_RESOURCE_ADDRESS the AE having privileges to perform CREATE operation at PARENT_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Contentcontaining  | |
                     | TransactionMgmt containing  | |
                     | requestPrimitives attribute containing  | |
                     | to set to TARGET_RESOURCE_ADDRESS  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | Valid transactionMgmt resource representation and IUT creates a  | |
                     |transaction resource as child of the target resource address  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————+————-+

##### 7.2.2.20.2	UPDATE Operation

###### TP/oneM2M/CSE/TMG/UPD/001

| TP Id | TP/oneM2M/CSE/TMG/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update of transactionControl attribute of transactionMgmt resource when Originator of update request is not the creator of transactionMgmt resource |  |
| Reference | TS-0001 [4], clause 7.4.60.2.3, Recv-6.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE2 and the IUT having registered the AE1 and the IUT having a transactionMgmt resource at TARGET_RESOURCE_ADDRESS containing creator attribute set to AE2 and the AE1 having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transactionMgmt resource containing  | |
                     | transactionControl attribute set to TRANSACTION_CONTROL_VALUE  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103  | |
                     |(ORIGINATOR_HAS _NO_PRIVILEGE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/TMG/UPD/002

| TP Id | TP/oneM2M/CSE/TMG/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of transactionControl attribute of transactionMgmt resource when transactionMode is "CSE_CONTROLLED" |  |
| Reference | TS-0004 [4], clause 7.4.60.2.3, Recv-6.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a transactionMgmt resource at TARGET_RESOURCE_ADDRESS containing creator attribute set to AE and transactionMode set to "CSE_CONTROLLED" } |  |

- +--------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE and  | |
                     | Content containing  | |
                     | transactionMgmt resource containing  | |
                     | transactionControl attribute set to  | |
                     |TRANSACTION_CONTROL_VALUE_  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000  | |
                     |(BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/TMG/UPD/003

| TP Id | TP/oneM2M/CSE/TMG/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of transactionMgmt resource when the transactionControl value in the request primitive does not transition to values specified in Table 10.2.18.1-1 of oneM2M TS-0001 |  |
| Reference | TS-0001 [4], Table 10.2.18.1-1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having a transactionMgmt resource at TARGET_RESOURCE_ADDRESS containing creator attribute set to AE1 and transactionMode set to "CREATOR_CONTROLLED" and transactionState set to TRANSACTION_STATE_VALUE } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transactionMgmt resource containing  | |
                     | transactionControl attribute set to  | |
                     | INVALID_CORRESPONDING_TRANSACTION_CONTROL_VALUE_  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4123  | |
                     |(ILLEGAL_TRANSACTION_STATE_TRANSITION_ATTEMPTED )  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————–+————-+

| TP Id | TRANSACTION_STATE_VALUE | INVALID_CORRESPONDING_TRANSACTION_CONTROL_VALUE |
| --- | --- | --- |
| TP/oneM2M/CSE/TMG/UPD/003_INI | INITIAL | EXECUTE, ABORT,COMMIT, INITIAL, ERROR |
| TP/oneM2M/CSE/TMG/UPD/003_LOC | LOCKED | COMMIT, INITIAL, LOCK, ERROR |
| TP/oneM2M/CSE/TMG/UPD/003_EXE | EXECUTED | INITIAL,LOCK, EXECUTE, ERROR |
| TP/oneM2M/CSE/TMG/UPD/003_ERR | ERROR | LOCK, EXECUTE, COMMIT, ERROR |
| TP/oneM2M/CSE/TMG/UPD/003_COM | COMMITTED | EXECUTE, ABORT,COMMIT, LOCK, ERROR |
| TP/oneM2M/CSE/TMG/UPD/003_ABT | ABORTED | EXECUTE, ABORT,COMMIT, LOCK, ERROR |

###### TP/oneM2M/CSE/TMG/UPD/004

| TP Id | TP/oneM2M/CSE/TMG/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully updates of transactionMgmt resource when transactionMode is "CREATOR_CONTROLLED" and the transactionControl value in the request primitive transitions to values specified in Table 10.2.18.1-1 of oneM2M TS-0001 and Originator of request matches creator attribute of transactionMgmt resource |  |
| Reference | TS-0001 [4], Table 10.2.18.1-1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having a transactionMgmt resource at TARGET_RESOURCE_ADDRESS containing creator attribute set to AE1 and transactionMode set to "CREATOR_CONTROLLED" and transactionState set to TRANSACTION_STATE_VALUE } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transactionMgmt resource containing  | |
                     | transactionControl attribute set to | |
                     | VALID_CORRESPONDING_TRANSACTION_CONTROL_VALUE_  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT updates the transactionMgmt resource  | |
                     | and the IUT sends a valid Response containing  | |
                     | Response Status Code set to 2004 (UPDATED)  | |
                     | Content containing  | |
                     | transactionMgmt resource containing  | |
                     | transactionControl attribute set to | |
                     |VALID_CORRESPONDING_TRANSACTION_CONTROL_VALUE  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————————————————————–+————-+

| TP Id | TRANSACTION_STATE_VALUE | INVALID_CORRESPONDING_TRANSACTION_CONTROL_VALUE |
| --- | --- | --- |
| TP/oneM2M/CSE/TMG/UPD/004_INI | INITIAL | LOCK |
| TP/oneM2M/CSE/TMG/UPD/004_LOC | LOCKED | EXECUTE, ABORT |
| TP/oneM2M/CSE/TMG/UPD/004_EXE | EXECUTED | COMMIT, ABORT |
| TP/oneM2M/CSE/TMG/UPD/004_ERR | ERROR | ABORT, INITIAL |
| TP/oneM2M/CSE/TMG/UPD/004_COM | COMMITTED | INITIAL |
| TP/oneM2M/CSE/TMG/UPD/004_ABT | ABORTED | INITIAL |

###### TP/oneM2M/CSE/TMG/UPD/005

| TP Id | TP/oneM2M/CSE/TMG/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the update of transactionControl attribute of transaction resource when Originator of update request is not the creator of transaction resource |  |
| Reference | TS-0001 [4], clause 7.4.61.2.3, Recv-6.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE2 and the IUT having registered the AE1 and the IUT having a transaction resource at TARGET_RESOURCE_ADDRESS containing creator attribute set to AE2 and the AE1 having privileges to perform UPDATE operation on the TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transaction resource containing  | |
                     | transactionControl attribute set to | |
                     |TRANSACTION_CONTROL_VALUE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS  | |
                     |_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/TMG/UPD/006

| TP Id | TP/oneM2M/CSE/TMG/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of transaction resource when the transactionControl value in the request primitive does not transition to values specified in Table 10.2.18.1-2 of oneM2M TS-0001 |  |
| Reference | TS-0001 [4], Table 10.2.18.1-2 |  |
| Parent Release | Release 4 |  |
| Config Id | CF02 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having a transaction resource at TARGET_RESOURCE_ADDRESS containing creator attribute set to AE1 and transactionState set to TRANSACTION_STATE_VALUE } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transaction resource containing  | |
                     | transactionControl attribute set to  | |
                     |TRANSACTION_CONTROL_VALUE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS  | |
                     |_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————-+————-+

| TP Id | TRANSACTION_STATE_VALUE | INVALID_CORRESPONDING_TRANSACTION_CONTROL_VALUE |
| --- | --- | --- |
| TP/oneM2M/CSE/TMG/UPD/006_LOC | LOCKED | COMMIT, LOCK, ERROR |
| TP/oneM2M/CSE/TMG/UPD/006_EXE | EXECUTED | INITIAL,LOCK, EXECUTE, ERROR |
| TP/oneM2M/CSE/TMG/UPD/006_ERR | ERROR | EXECUTE,COMMIT, ERROR |
| TP/oneM2M/CSE/TMG/UPD/006_COM | COMMITTED | EXECUTE, ABORT,COMMIT, ERROR |
| TP/oneM2M/CSE/TMG/UPD/006_ABT | ABORTED | EXECUTE, ABORT,COMMIT, ERROR |

###### TP/oneM2M/CSE/TMG/UPD/007

| TP Id | TP/oneM2M/CSE/TMG/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT successfully updates of transaction resource when the transactionControl value in the request primitive transitions to values specified in Table 10.2.18.1-2 of oneM2M TS-0001 and Originator of request is matches creator attribute of transaction resource |  |
| Reference | TS-0001 [4], Table 10.2.18.1-1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE1 and the IUT having a transaction resource at TARGET_RESOURCE_ADDRESS containing creator attribute set to AE1 and transactionState set to TRANSACTION_STATE_VALUE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE1 |
                     | the IUT receives a valid UPDATE Request from AE1 containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE1 and  | |
                     | Content containing  | |
                     | transaction resource containing  | |
                     | transactionControl attribute set to | |
                     |TRANSACTION_CONTROL_VALUE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE1 |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS  | |
                     |_NO_PRIVILEGE)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | TRANSACTION_STATE_VALUE | INVALID_CORRESPONDING_TRANSACTION_CONTROL_VALUE |
| --- | --- | --- |
| TP/oneM2M/CSE/TMG/UPD/007_LOC | LOCKED | EXECUTE, ABORT |
| TP/oneM2M/CSE/TMG/UPD/007_EXE | EXECUTED | COMMIT, ABORT |
| TP/oneM2M/CSE/TMG/UPD/007_ERR | ERROR | ABORT, LOCK |
| TP/oneM2M/CSE/TMG/UPD/007_COM | COMMITTED | LOCK |
| TP/oneM2M/CSE/TMG/UPD/007_ABT | ABORTED | LOCK |

##### 7.2.2.20.3	DELETE Operation

###### TP/oneM2M/CSE/TMG/DEL/001

| TP Id |  | TP/oneM2M/CSE/TMG/DEL/001 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT deletes RESOURCE_TYPE resource when transactionState is either "COMMITTED" or "ABORTED". |  |
| Reference |  | TS-0004 [2], clause 7.4.61.2.4 and 7.4.60.2.4 |  |
| Config Id |  | CF02 |  |
| Parent Release |  | Release 4 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having created a resource TARGET_RESOURCE_ADDRESS of type RESOURCE_TYPE containing transactionState set to "COMMITTED" or "ABORTED" and the AE having privileges to perform DELETE operation on the resource TARGET_RESOURCE_ADDRESS } |  |  |

- +--------------------------------------------------+-------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | no Content  | |
                     |} | |

- +--------------------------------------------------+-------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT deletes the RESOURCE_TYPE resource and sends a valid  | |
                     |Response containing Response Status Code set to 2002 (DELETED)  | |
                     |} | |
+———————-+————————————————–+————————————————————————————————-+————-+

| TP Id | RESOURCE_TYPE |
| --- | --- |
| TP/oneM2M/CSE/TMG/DEL/001_TRAM_COM | 50 (transactionMgmt) |
| TP/oneM2M/CSE/TMG/DEL/001_TRAM_ABT | 50 (transactionMgmt) |
| TP/oneM2M/CSE/TMG/DEL/001_TRAC_COM | 51 (transaction) |
| TP/oneM2M/CSE/TMG/DEL/001_TRAC_ABT | 51 (transaction) |

#### 7.2.2.21	Resource Schedule (SCH)

##### 7.2.2.21.1	BASIC Operation

TP/oneM2M/CSE/SCH/001

| TP Id | TP/oneM2M/CSE/SCH/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the CREATE request for the <schedule> resource, with the networkCoordinated attribute set, when the parent resource is of type <subscription> |  |
| Reference | TS-0001 [1], clause 9.6.9, TS-0004 [2], clause 7.4.9.2 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a Hosting CSE and the IUT having registered the AE and the IUT having a <subscription> resource at TARGET_RESOURCE_ADDRESS and AE having privileges to perform CREATE on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid CREATE request from AE containing  | |
                     | Resource Type set to schedule  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID  | |
                     | Content containing  | |
                     | schedule resource containing  | |
                     | networkCoordinated attribute  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response to AE containing  |IUT => AE |
                     | Response Status Code set to 4102  | |
                     |(CONTENTS_UNACCEPTABLE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/SCH/002

| TP Id | TP/oneM2M/CSE/SCH/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE request for the <schedule> resource, with the networkCoordinated attribute set, when the parent resource is of type <subscription> |  |
| Reference | TS-0001 [1], clause 9.6.9, TS-0004 [2], clause 7.4.9.2 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a Hosting CSE and the IUT having registered the AE and the IUT having a <subscription> resource containing a child resource <schedule> at TARGET_RESOURCE_ADDRESS and AE having privileges to perform UPDATE on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT <= AE |
                     | the IUT receives a valid UPDATE request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID  | |
                     | Content containing  | |
                     | schedule resource containing  | |
                     | networkCoordinated attribute  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid Response to AE containing  |IUT => AE  |
                     | Response Status Code set to 4102  | |
                     |(CONTENTS_UNACCEPTABLE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/SCH/003

| TP Id | TP/oneM2M/CSE/SCH/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT delivers notifications for the subscribed-to events of the <subscription> resource according to the allowed time window specified in the scheduleElement of the associated child <schedule> resource |  |
| Reference | TS-0001 [1], clause 9.6.9, TS-0004 [2], clause 7.4.9.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT being a Hosting CSE and the IUT having registered AE1 and AE2 and the IUT having a <container> resource at TARGET_RESOURCE_ADDRESS and AE1 having privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and AE2 having privileges to perform RETRIEVE operation on TARGET_RESOURCE_ADDRESS and AE2 having created a <subscription> resource for the <container> resource and the IUT having a child resource <schedule> for the <subscription> resource containing scheduleElement attribute set to SCHEDULE_TIME and AE1 having UPDATED the <container> resource at TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |IUT |
                     | the SCHEDULE_TIME is reached  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     | the IUT sends a valid NOTIFY Request to the AE containing  |IUT => AE2  |
                     | Content containing  | |
                     | notification message containing  | |
                     | a valid notificationEvent attribute  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————-+————-+

#### 7.2.2.22	Process Management(PMG)

##### 7.2.2.22.1	CREATE Operation

TP/oneM2M/CSE/PMG/CRE/001

| TP Id | TP/oneM2M/CSE/PMG/CRE/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT creates <processManagement> resource on the TARGET_RESOURCE_ADDRESS of type PARENT_RESOURCE_TYPE |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to XX (processManagement) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 2001 (CREATED) and  | |
                     | Content containing  | |
                     | processManagement resource containing   | |
                     | processStatus attribute set to value "Disabled" and | |
                     |currentState set to NULL  | |
                     |} | |
+———————-+—————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/CRE/002

| TP Id | TP/oneM2M/CSE/PMG/CRE/002 |  |  |
| --- | --- | --- | --- |
| Test objective | Check that the IUT rejects create of <state> resource when processStatus attribute of parent <processManagement> has value not equal to "Disabled" |  |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.1 |  |  |
| Parent Release | Release 4 |  |  |
| Config Id | CF01 |  |  |
| PICS Selection | PICS_CSE |  |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <processManagement> resource at TARGET_RESOURCE_ADDRESS having processStatus attribute set to value not equal to "Disabled" and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |  |

- +--------+-------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to XX (state) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource  | |
                     |} | |

- +--------+-------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+——–+——————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/CRE/003

| TP Id |  | TP/oneM2M/CSE/PMG/CRE/003 |  |
| --- | --- | --- | --- |
| Test objective |  | Check that the IUT rejects create of <state> resource when some referenced <state> resources in stateTransitions attribute don't exist |  |
| Reference |  | TS-0004 [2] clause 7.4.XX.2.1 |  |
| Parent Release |  | Release 4 |  |
| Config Id |  | CF01 |  |
| PICS Selection |  | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <processManagement> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |  |

- +------------------------------------------------------+-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                      | the IUT receives a valid CREATE Request from AE containing  | |
                      | To set to TARGET_RESOURCE_ADDRESS and  | |
                      | Resource Type set to XX (state) and  | |
                      | From set to AE-ID and  | |
                      | Content containing  | |
                      | state resource containing  | |
                      | stateTransitions attribute set to non-existing state resources  | |
                      |} | |

- +------------------------------------------------------+-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                      | the IUT sends valid Response to AE containing  | |
                      | Response Status Code set to 41xx  | |
                      |(INVALID_PROCESS_CONFIGURATION)  | |
                      |} | |
+———————–+——————————————————+——————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/CRE/004

| TP Id | TP/oneM2M/CSE/PMG/CRE/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <state> resource when for some referenced <state> resources in stateTransitions attributes Originator don't have retrieve privilege |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <processManagement> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to XX (state) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute set to state  | |
                     |resources on which Originator do not have retrieve privilege  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/CRE/005

| TP Id | TP/oneM2M/CSE/PMG/CRE/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <state> resource when some referenced <state> resources in stateTransitions attributes are not child resources of parent <processManagement> |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <processManagement> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to XX (state) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute set to some state resources not child  | |
                     |resources of TARGET_RESOURCE_ADDRESS resource  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 41xx | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/CRE/006

| TP Id | TP/oneM2M/CSE/PMG/CRE/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when resource referenced by ATTRIBUTE attribute doesn't exist |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | ATTRIBUTE set to not-existing resourceID.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+———————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/CRE/006_SRI | subjectResourceID |
| TP/oneM2M/CSE/PMG/CRE/006_ORI | objectResourceID |

TP/oneM2M/CSE/PMG/CRE/007

| TP Id | TP/oneM2M/CSE/PMG/CRE/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when resource referenced by subjectResourceID attribute is not accessible to it |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | subjectResourceID set to non-accessible resourceID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/CRE/009

| TP Id | TP/oneM2M/CSE/PMG/CRE/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when resource referenced by objectResourceID attribute is not accessible to it |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | objectResourceID set to non-accessible resourceID  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/CRE/010

| TP Id | TP/oneM2M/CSE/PMG/CRE/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when From parameter in actionPrimitive attribute is not same as Originator of create request |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | valid value set in actionPrimitive containing  | |
                     | From parameter not same as Originator of create request  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/CRE/011

| TP Id | TP/oneM2M/CSE/PMG/CRE/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when evalMode attribute has value either "off" or "once" and evalControlParam attribute is also set |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | evalMode set to "off" or "once" and  | |
                     | evalControlParam also set.  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/CRE/012

| TP Id | TP/oneM2M/CSE/PMG/CRE/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when subject element of evalCriteria attribute is not an attribute of resource type referenced by subjectResourceID attribute |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having child <container> resource at CHILD_ADDRESS and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | subjectResourceID set to CHILD_ADDRESS  | |
                     | valid value set in evalCriteria attribute containing  | |
                     | subject element not matching CHILD_ADDRESS attributes  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/CRE/013

| TP Id | TP/oneM2M/CSE/PMG/CRE/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when subjectResourceID attribute not set and subject element of evalCriteria attribute is not an attribute of the parent resource type |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | subjectResourceID not set  | |
                     | valid value set in evalCriteria attribute containing  | |
                     | subject element not matching | |
                     |TARGET_RESOURCE_ADDRESS attributes  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/CRE/014

| TP Id | TP/oneM2M/CSE/PMG/CRE/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when threshold element of evalCriteria is outside value space of data type of subject element |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE operation on the resource a TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | evalCriteria attribute set and  | |
                     | subject element set and  | |
                     | threshold element set outside value space of data type of  | |
                     |subject attribute  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/CRE/015

| TP Id | TP/oneM2M/CSE/PMG/CRE/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <action> resource when operator element of evalCriteria has invalid value |  |
| Reference | TS-0004 [2] clause 7.4.73.2.1, Table 6.3.4.2.86-1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the AE having privileges to perform CREATE on the resource at TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | Resource Type set to 63 (action) and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | evalCriteria attribute set and  | |
                     | operator set to invalid value  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/CRE/016

| TP Id | TP/oneM2M/CSE/PMG/CRE/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <dependency> resource when resource defined by referencedResourceID attribute doesn't exist |  |
| Reference | TS-0004 [2] clause 7.4.74.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <action> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Resource Type set to 64 (dependency) and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | referencedResourceID set to not-existing resourceID.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+———————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/CRE/017

| TP Id | TP/oneM2M/CSE/PMG/CRE/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <dependency> resource when Originator doesn't have retrieve privilege on resource defined by referencedResourceID attribute |  |
| Reference | TS-0004 [2] clause 7.4.74.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <action> resource at TARGET_RESOURCE_ADDRESS and the IUT having <container> resource at CHILD_ADDRESS not accessible to AE and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Resource Type set to 64 (dependency) and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | referencedResourceID set to CHILD_ADDRESS.  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/CRE/018

| TP Id | TP/oneM2M/CSE/PMG/CRE/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <dependency> resource when attribute referenced by subject element of evalCriteria attribute of request is not an attribute of resource type referenced by referencedResourceID attribute |  |
| Reference | TS-0004 [2] clause 7.4.74.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <action> resource at TARGET_RESOURCE_ADDRESS and the IUT having <container> resource at CHILD_ADDRESS accessible to AE and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Resource Type set to 64 (dependency) and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | referencedResourceID set to CHILD_ADDRESS and  | |
                     | valid value set in evalCriteria containing  | |
                     | subject element set not to one of the attributes of CHILD_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/CRE/019

| TP Id | TP/oneM2M/CSE/PMG/CRE/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <dependency> resource when value of threshold element of evalCriteria attribute is outside the value space of data type of subject element of evalCriteria |  |
| Reference | TS-0004 [2] clause 7.4.74.2.1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <action> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Resource Type set to 64 (dependency) and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | valid value set in evalCriteria containing  | |
                     | subject element set and  | |
                     | threshold element set to value outside value range of  | |
                     |referenced attribute data type of subject element  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/CRE/020

| TP Id | TP/oneM2M/CSE/PMG/CRE/020 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects create of <dependency> resource when value of operator element is invalid |  |
| Reference | TS-0004 [2] clause 7.4.74.2.1, Table 6.3.4.2.86 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <action> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform CREATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Resource Type set to 64 (dependency) and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | valid value set in evalCriteria containing  | |
                     | operator element set to invalid value  | |
                     |} | |

- +------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends valid Response to AE containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————+————-+

##### 7.2.2.22.2	DELETE Operation

TP/oneM2M/CSE/PMG/DEL/001

| TP Id | TP/oneM2M/CSE/PMG/DEL/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects delete of <processManagement> resource when the value of the processStatus attribute is not equal to "Disabled" |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.4 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS, processStatus set not equal to "Disabled" and the AE have privilege to perform DELETE operation on TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+——————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/DEL/002

| TP Id | TP/oneM2M/CSE/PMG/DEL/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects delete of <state> resource when the value of the stateActive attribute is equal to "true" |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.4 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <state> at TARGET_RESOURCE_ADDRESS, stateActive set to "true" and the AE have privilege to perform DELETE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+———————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/DEL/003

| TP Id | TP/oneM2M/CSE/PMG/DEL/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects delete of <state> resource when the value of the processStatus attribute of parent <processManagement> is not equal to "Disabled" |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.4 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at PARENT_RESOURCE_ADDRESS, processStatus set not equal to "Disabled" and the IUT having a resource <state> at TARGET _RESOURCE_ADDRESS and the AE have privilege to perform DELETE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————–+————-+

##### 7.2.2.22.3	UPDATE Operation

TP/oneM2M/CSE/PMG/UPD/001

| TP Id | TP/oneM2M/CSE/PMG/UPD/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of ATTRIBUTE attribute of <processManagement> resource when the processStatus attribute has value not equal to "Disabled" |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and and the IUT having a <processManagement> at TARGET_RESOURCE_ADDRESS having processStatus attribute set value not equal to "Disabled" and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | ATTRIBUTE attribute containing valid value.  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/001_AC | activateConditions |
| TP/oneM2M/CSE/PMG/UPD/001_EC | endConditions |
| TP/oneM2M/CSE/PMG/UPD/001_IS | InitialState |

TP/oneM2M/CSE/PMG/UPD/002

| TP Id | TP/oneM2M/CSE/PMG/UPD/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when originator does not have retrieve privilege on the resources or attributes referred by subject element of the evalCriteria of ATTRIBUTE attribute |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having < processManagement> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS and the IUT having a resource at SUBJECT_RESOURCE_ADDRESS and the AE not having privileges to perform RETRIEVE operation at SUBJECT_RESOURCE_ADDRESS. } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | ATTRIBUTE attribute containing  | |
                     | evalCriteria attribute containing  | |
                     | subject parameter set to SUBJECT_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code  | |
                     |set to 41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————–+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/002_AC | activateConditions |
| TP/oneM2M/CSE/PMG/UPD/002_EC | endConditions |

TP/oneM2M/CSE/PMG/UPD/003

| TP Id | TP/oneM2M/CSE/PMG/UPD/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the resources or attributes referred by subject attribute of evalCriteria parameters of ATTRIBUTE attribute do not exist |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <processManagement> resource at TARGET_RESOURCE_ADDRESS and the AE having privileges to perform UPDATE operation at TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | ATTRIBUTE attribute containing  | |
                     | evalCriteria attribute containing  | |
                     | subject parameter set to UNAVAILABLE_RESOURCE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     |41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/003_AC | activateConditions |
| TP/oneM2M/CSE/PMG/UPD/003_EC | endConditions |

TP/oneM2M/CSE/PMG/UPD/004

| TP Id | TP/oneM2M/CSE/PMG/UPD/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the evalCriteria of ATTRIBUTE has threshold element value not within the value space of the data type of the subject element |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <processManagement> resource at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | ATTRIBUTE attribute containing  | |
                     | evalCriteria element containing  | |
                     | threshold element set to invalid value  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     | 41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/004_AC | activateConditions |
| TP/oneM2M/CSE/PMG/UPD/004_EC | endConditions |

TP/oneM2M/CSE/PMG/UPD/005

| TP Id | TP/oneM2M/CSE/PMG/UPD/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the evalCriteria of ATTRIBUTE has invalid value in operator attribute |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3, Table 6.3.4.2.86-1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | ATTRIBUTE attribute containing  | |
                     | evalCriteria attribute having  | |
                     | operator parameter set to invalid value.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     |41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+———————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/005_AC | activateConditions |
| TP/oneM2M/CSE/PMG/UPD/005_EC | endConditions |

TP/oneM2M/CSE/PMG/UPD/006

| TP Id | TP/oneM2M/CSE/PMG/UPD/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of initialState attribute of <processManagement> resource when referred <state> resource doesn't exist |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | initialState set to UNAVAILABLE STATE RESOURCE.  | |
                     |.  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to | |
                     |41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+———————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/007

| TP Id | TP/oneM2M/CSE/PMG/UPD/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of initialState attribute of <processManagement> resource when Originator does not have retrieve privilege on referenced <state> resource |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having a <state> resource at STATE_RESOURCE_ADDRESS and the AE not having privileges to perform RETRIEVE operation at STATE_RESOURCE_ADDRESS. } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | initialState attribute set to STATE_RESOURCE_ADDRESS.  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to 41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/008

| TP Id | TP/oneM2M/CSE/PMG/UPD/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of initialState attribute of <processManagement> resource when the referred resource is not the child resource of <processManagement> |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS2 and the IUT having <state> resource at STATE_RESOURCE_ADDRESS as child resource of TARGET_RESOURCE_ADDRESS2. } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | initialState attribute set to STATE_RESOURCE_ADDRESS  | |
                     |.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to 41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/009

| TP Id | TP/oneM2M/CSE/PMG/UPD/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the value of the processControl attribute in the request is equal to "Enable" and the value of the processStatus attribute of the <processManagement> resource is equal to _PROCESS_STATUS VALUE |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having resource <processManagement> at TARGET_RESOURCE_ADDRESS having,processStatus set equal to "PROCESS_STATUS_VALUE and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | processControl attribute set to value "Enable".  | |
                     |.  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     | 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | PROCESS_STATUS_VALUE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/009_EN | Enabled |
| TP/oneM2M/CSE/PMG/UPD/009_AC | Activated |
| TP/oneM2M/CSE/PMG/UPD/009_PU | Paused |
| TP/oneM2M/CSE/PMG/UPD/009_CM | Completed |
| TP/oneM2M/CSE/PMG/UPD/009_AB | Aborted |

TP/oneM2M/CSE/PMG/UPD/010

| TP Id | TP/oneM2M/CSE/PMG/UPD/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the value of the processControl attribute in the request is equal to "Disable" and the value of the processStatus attribute of the <processManagement> resource is equal to "Disabled" |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having resource <processManagement> at TARGET_RESOURCE_ADDRESS, processStatus set to "Disabled" and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | processControl attribute set to value "Disable".  | |
                     |.  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     | 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/011

| TP Id | TP/oneM2M/CSE/PMG/UPD/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the value of the processControl attribute in the request is equal to "Pause" and the value of the processStatus attribute of the <processManagement> resource is equal to "_PROCESS_STATUS VALUE |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS, processStatus attribute set equal to _PROCESS_STATUS VALUE and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | processControl attribute set to value "Pause".  | |
                     |.  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     |4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————+————-+

| TP Id | PROCESS_STATUS_VALUE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/011_EN | Enabled |
| TP/oneM2M/CSE/PMG/UPD/011_DS | Disabled |
| TP/oneM2M/CSE/PMG/UPD/011_PU | Paused |
| TP/oneM2M/CSE/PMG/UPD/011_CM | Completed |
| TP/oneM2M/CSE/PMG/UPD/011_AB | Aborted |

TP/oneM2M/CSE/PMG/UPD/012

| TP Id | TP/oneM2M/CSE/PMG/UPD/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the value of the processControl attribute in the request is equal to "Reactivate" and the value of the processStatus attribute of the <processManagement> resource is equal to "_PROCESS_STATUS VALUE " |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource <processManagement> at TARGET_RESOURCE_ADDRESS, processStatus set equal to "_PROCESS_STATUS VALUE " and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | processControl attribute set to value "Reactivate".  | |
                     |.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     |4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————————————-+————-+

| TP Id | PROCESS_STATUS_VALUE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/012_EN | Enabled |
| TP/oneM2M/CSE/PMG/UPD/012_DS | Disabled |
| TP/oneM2M/CSE/PMG/UPD/012_AC | Activated |
| TP/oneM2M/CSE/PMG/UPD/012_CM | Completed |
| TP/oneM2M/CSE/PMG/UPD/012_AB | Aborted |

TP/oneM2M/CSE/PMG/UPD/013

| TP Id | TP/oneM2M/CSE/PMG/UPD/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the value of the processControl attribute in the request is equal to "Enable" and the value of the processStatus attribute of the <processManagement> resource is equal to "Disabled" when any of the referenced child resources is not available |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having resource <processManagement> at TARGET_RESOURCE_ADDRESS, processStatus attribute set to "Disabled" and the IUT having <state> resource as child resource of TARGET_RESOURCE_ADDRESS and referenced in initialState attribute and the resource referenced by stateAction attribute is not available and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | processControl attribute set to value "Enable".  | |
                     |.  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     | 41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/014

| TP Id | TP/oneM2M/CSE/PMG/UPD/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the value of the processControl attribute in the request is equal to "Enable" and the value of the processStatus attribute of the <processManagement> resource is equal to "Disabled" but Originator does not have required privileges on any of the referenced child resources |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having resource <processManagement> at TARGET_RESOURCE_ADDRESS containing processStatus attribute set to "Disabled" and initialState attribute set to child <state> resource at STATE_ADDRESS containing stateAction attribute set to child <action> resource at ACTION_ADDRESS and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and but AE not having privilege on STATE_ADDRESS and ACTION_ADDRESS. } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | processControl set to value "Enable".  | |
                     |.  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing Response Status Code set to  | |
                     |41xx (INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/015

| TP Id | TP/oneM2M/CSE/PMG/UPD/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <processManagement> resource when the value of the processControl attribute in the request is equal to "Enable" and the value of the processStatus attribute of the <processManagement> resource is equal to "Disabled" but any of referenced resources are not descendent resource of <processManagement> |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having <state> resource at STATE and the IUT having <action> resource at ACTION and the IUT having resource <processManagement> at TARGET_RESOURCE_ADDRESS containing processStatus set to "Disabled" and initialState attribute set to STATE and stateAction attribute set to ACTION the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS. } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | processManagement resource containing  | |
                     | processControl set to value "Enable".  | |
                     |.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to  | |
                     |41xx(INVALID_PROCESS_CONFIGURATION).  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/016

| TP Id | TP/oneM2M/CSE/PMG/UPD/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when the processStatus attribute of parent <processManagement> has value not equal to "Disabled" |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and and the IUT having a <processManagement> resource at PARENT_RESOURCE_ ADDRESS containing processStatus attribute set not equal to "Disabled" and and the IUT having a child <state> resource at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | ATTRIBUTE attribute containing valid value.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+————————————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/016_STAC | stateAction |
| TP/oneM2M/CSE/PMG/UPD/016_STTRS | stateTransitions |

TP/oneM2M/CSE/PMG/UPD/017

| TP Id | TP/oneM2M/CSE/PMG/UPD/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when the resource referenced by stateAction attribute does not exist |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateAction attribute value set to NON EXISTENT ACTION RESOURCE.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/018

| TP Id | TP/oneM2M/CSE/PMG/UPD/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when the Originator does not have retrieve privilege on resource referenced by stateAction attribute |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at PARENT_RESOURCE_ADDRESS and the IUT having a child <action> resource ACTION_ADDRESS Originator not having retrieve privilege of and the AE have privilege to perform UPDATE operation on PARENT_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateAction attribute value set to ACTION_ADDRESS.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/019

| TP Id | TP/oneM2M/CSE/PMG/UPD/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when the resource referenced by stateAction attribute is not a child resource |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at TARGET_RESOURCE_ADDRESS and the IUT having created RESOURCE_TYPE resource containing a child <action> resource at ACTION_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateAction attribute value set to ACTION_ADDRESS.  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/020

| TP Id | TP/oneM2M/CSE/PMG/UPD/020 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when the any of <state> resources referenced by stateTransitions attribute doesn't exist |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute set to NON EXISTENT STATE  | |
                     |RESOURCE.  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/021

| TP Id | TP/oneM2M/CSE/PMG/UPD/021 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when for any of <state> resources referenced by stateTransitions attribute Originator doesn't have retrieve privilege |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at STATE_ADDRESS1 and the IUT having a <state> resource at STATE_ADDRESS2 and the AE having privilege to perform UPDATE operation on STATE_ADDRESS1 AE not having privileges to perform RETRIEVE operation on STATE_ADDRESS2 } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to STATE_ADDRESS1and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute value set to STATE_ADDRESS2.  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/022

| TP Id | TP/oneM2M/CSE/PMG/UPD/022 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when any of <state> resources referenced by stateTransitions attribute is not child resource of parent <processManagement> |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <processManagement> resource at PARENT_ADDRESS having a child <state> resource at STATE_ADDRESS and the IUT having a <processManagement> resource at PARENT_ADDRESS2 having a child <state> resource at STATE_ADDRESS2 and the AE have privilege to perform UPDATE operation on STATE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to STATE_ADDRESS and  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute indicating  | |
                     | nextState element set to STATE_ADDRESS2  | |
                     | .  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/023

| TP Id | TP/oneM2M/CSE/PMG/UPD/023 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when any of resources and attributes referenced by subject element of evalCriteria of stateTransitions attribute doesn't exist |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute set having some  | |
                     | evalCriteria' subject element set to NON-EXISTING RESOURCES  | |
                     |AND ATTRIBUTES.  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/024

| TP Id | TP/oneM2M/CSE/PMG/UPD/024 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when Originator doesn't have retrieve privilege on any of resources and attributes referenced by subject element of evalCriteria of stateTransitions attribute |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the AE not having privileges to perform RETRIEVE operation at SUBJECT_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute containing  | |
                     | evalCriteria' subject element set to  | |
                     |SUBJECT_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/025

| TP Id | TP/oneM2M/CSE/PMG/UPD/025 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when threshold element of evalCriteria of stateTransitions attribute have value outside the value spaces of the data type of subject element |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute containing  | |
                     | evalCriteria' threshold element set to outside range spaces of the  | |
                     |data type of subject element.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/026

| TP Id | TP/oneM2M/CSE/PMG/UPD/026 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <state> resource when operator element of evalCriteria of stateTransitions attribute have invalid value |  |
| Reference | TS-0004 [2] clause 7.4.XX.2.3, Table 6.3.4.2.86-1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <state> resource at PARENT_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on PARENT_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | state resource containing  | |
                     | stateTransitions attribute containing  | |
                     | evalCriteria' operator element set to invalid value.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 41xx  | |
                     |(INVALID_PROCESS_CONFIGURATION)  | |
                     |} | |
+———————-+———————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/027

| TP Id | TP/oneM2M/CSE/PMG/UPD/027 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when resource referenced by ATTRIBUTE attribute doesn't exist |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at PARENT_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on PARENT_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | ATTRIBUTE attribute set to non-existing resourceID.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+———————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/027_SRI | subjectResourceID |
| TP/oneM2M/CSE/PMG/UPD/027_DEP | dependencies |
| TP/oneM2M/CSE/PMG/UPD/027_ORI | objectResourceID |

TP/oneM2M/CSE/PMG/UPD/028

| TP Id | TP/oneM2M/CSE/PMG/UPD/028 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when resource referenced by ATTRIBUTE attribute is not accessible by Originator |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at PARENT_RESOURCE_ADDRESS and the IUT having child <container> at CHILD_ADDRESS not accessible by AE and the AE have privilege to perform UPDATE operation on PARENT_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to PARENT_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | ATTRIBUTE attribute set to CHILD_ADDRESS.  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————-+————-+

| TP Id | ATTRIBUTE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/028_SRI | subjectResourceID |
| TP/oneM2M/CSE/PMG/UPD/028_DEP | Dependencies |
| TP/oneM2M/CSE/PMG/UPD/028_ORI | objectResourceID |

TP/oneM2M/CSE/PMG/UPD/029

| TP Id | TP/oneM2M/CSE/PMG/UPD/029 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when any of <dependency> resources referenced by dependencies attribute are not child resources of this <action> resource |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having another <action> at TARGET_RESOURCE_ADDRESS2 and and IUT having <dependency> resource at DEPENDENCY_RESOURCE_ADDRESS as child resource of TARGET_RESOURCE_ADDRESS2. } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | dependencies attribute set to  | |
                     |DEPENDENCY_RESOURCE_ADDRESS.  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/030

| TP Id | TP/oneM2M/CSE/PMG/UPD/030 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when From parameter contained in actionPrimitive attribute not equal to Originator |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | actionPrimitive attribute's From element not equal AE-ID.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/031

| TP Id | TP/oneM2M/CSE/PMG/UPD/031 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when request evalMode attribute has value EVAL_MODE_VALUE " and request evalControlParam attribute is also present |  |
| Reference | TS-0004 [2] clause 7.4.73is .2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS containing noevalControlParam attribute and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | evalMode set to EVAL_MODE_VALUE and  | |
                     | evalControlParam is present.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————-+————-+

| TP Id | EVAL_MODE_VALUE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/031_OFF | OFF |
| TP/oneM2M/CSE/PMG/UPD/031_ONCE | ONCE |

TP/oneM2M/CSE/PMG/UPD/032

| TP Id | TP/oneM2M/CSE/PMG/UPD/032 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when request evalMode attribute has value EVAL_MODE_VALUE ,request evalControlParam attribute not present but evalControlParam within <action> resource has value set |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS containing a valid evalControlParam attribute and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | evalMode set to EVAL_MODE_VALUE and  | |
                     | evalControlParam is not present  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————+————-+

| TP Id | EVAL_MODE_VALUE |
| --- | --- |
| TP/oneM2M/CSE/PMG/UPD/032_OFF | OFF |
| TP/oneM2M/CSE/PMG/UPD/032_ONCE | ONCE |

TP/oneM2M/CSE/PMG/UPD/033

| TP Id | TP/oneM2M/CSE/PMG/UPD/033 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when value of subject element of evalCriteria attribute of request is not one of the attribute of referenced resource of subjectResourceID attribute of request |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS, subjectResourceID not set and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and IUT having a resource at RESOURCE_ADDRESS not having attribute ATTRIBUTE } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | subjectResourceID set to RESOURCE_ADDRESS and  | |
                     | evalCriteria attribute containing  | |
                     | subject element set to ATTRIBUTE.  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/034

| TP Id | TP/oneM2M/CSE/PMG/UPD/034 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when value of subject element of evalCriteria attribute of request is not one of the attribute of referenced resource of subjectResourceID attribute of <action> |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and IUT having a resource at RESOURCE_ADDRESS not having attribute ATTRIBUTE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS containing subjectResourceID set to RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing   | |
                     | evalCriteria attribute containing  | |
                     | subject element set to ATTRIBUTE  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/035

| TP Id | TP/oneM2M/CSE/PMG/UPD/035 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when value of subject element of evalCriteria attribute of request is not one of the attribute of parent resource |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and AE not having attribute ATTRIBUTE and the AE having a child <action> at TARGET_RESOURCE_ADDRESS and havingsubjectResourceID not set and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | subjectResourceID not set and  | |
                     | valid value set in evalCriteria containing  | |
                     | subject element value set to ATTRIBUTE  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/PMG/UPD/036

| TP Id | TP/oneM2M/CSE/PMG/UPD/036 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when value of subject element of evalCriteria attribute of target <action> resource is not one of the attribute of resource referenced by request subjectResourceID attribute |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at RESOURCE_ADDRESS not having attribute ATTRIBUTE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS containing evalCriteria having subject element set to ATTRIBUTE and the AE having privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | subjectResourceID set to RESOURCE_ADDRESS  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/037

| TP Id | TP/oneM2M/CSE/PMG/UPD/037 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when value of threshold element of evalCriteria is not within value space of the data type of subject element of evalCriteria attribute |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | valid value set in evalCriteria containing  | |
                     | subject element set to ATTRIBUTE and  | |
                     | threshold element value outside value range of ATTRIBUTE data type.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/038

| TP Id | TP/oneM2M/CSE/PMG/UPD/038 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <action> resource when value of operator element of evalCriteria has invalid value |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 , Table 6.3.4.2.86-1 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <action> at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | action resource containing  | |
                     | valid value set in evalCriteria containing  | |
                     | operator element set to invalid value.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————-+————-+

TP/oneM2M/CSE/PMG/UPD/039

| TP Id | TP/oneM2M/CSE/PMG/UPD/039 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <dependency> resource when value present in subject element of evalCriteria of request is not an attribute of resource type referenced by referencedResourceID attribute |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a resource at RESOURCE_ADDRESS not having attribute ATTRIBUTE and the IUT having a <dependency> at TARGET_RESOURCE_ADDRESS, referencedResourceID set to RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | valid value set in evalCriteria containing  | |
                     | subject element set to ATTRIBUTE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/040

| TP Id | TP/oneM2M/CSE/PMG/UPD/040 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <dependency> resource when value present in threshold element of evalCriteria attribute of request is not within value space of data type of subject element |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <dependency> at TARGET_RESOURCE_ADDRESS having referencedResourceID attribute set and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | valid value set in evalCriteria containing  | |
                     | subject element value set to ATTRIBUTE and  | |
                     | threshold element value set to value outside value range of data type of ATTRIBUTE.  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/PMG/UPD/041

| TP Id | TP/oneM2M/CSE/PMG/UPD/041 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects update of <dependency> resource when value of operator element of evalCriteria attribute of request is invalid |  |
| Reference | TS-0004 [2] clause 7.4.73.2.3 , Table 6.3.4.2.86 |  |
| Parent Release | Release 4 |  |
| Config Id | CF01 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered the AE and the IUT having a <dependency> at TARGET_RESOURCE_ADDRESS and the AE have privilege to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | To set to TARGET_RESOURCE_ADDRESSand  | |
                     | From set to AE-ID and  | |
                     | Content containing  | |
                     | dependency resource containing  | |
                     | evalCriteria attribute set containing  | |
                     | operator element set to invalid value.  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {   |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     |Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————-+————-+

#### 7.2.2.23	Software Management(SM)

##### 7.2.2.23.1	BASIC Operation

TP/oneM2M/CSE/SM/001

| TP Id | TP/oneM2M/CSE/SM/001 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the creation request of <softwareCampaign> resource if target node referenced in softwareTargets attribute does not exist |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT not having the resource referenced in attribute softwareTargets and the AE having the privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 61? (softwareCampaign)  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTargets attribute set to  | |
                     | INVALID_RESOURCE_ADDRESS  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SM/002

| TP Id | TP/oneM2M/CSE/SM/002 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects creation of <softwareCampaign> resource when the AE does not have the privilege to perform operation on the resource referenced in softwareTargets attribute |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the AE not having the privileges to create [software] specialization child resource on the resource referenced in softwareTargets attribute and the AE having the privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 61? (softwareCampaign)  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTargets attribute set to  | |
                     | UNACCESSIBLE_RESOURCE_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SM/003

| TP Id | TP/oneM2M/CSE/SM/003 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects creation of <softwareCampaign> resource if the resource referenced in softwareTriggerCriteria attribute does not exist |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT not having the resource referenced in attribute softwareTriggerCriteria and the AE having the privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 61? (softwareCampaign)  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTriggerCriteria attribute containing  | |
                     | resources set to INVALID_RESOURCE_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SM/004

| TP Id | TP/oneM2M/CSE/SM/004 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects creation of <softwareCampaign> resource if the resource referenced in softwareTriggerCriteria attribute is not accessible by the AE |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the AE not having the privileges to perform CREATE operation on the resource referenced in softwareTriggerCriteria attribute and the AE having the privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 61? (softwareCampaign)  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTriggerCriteria attribute containing  | |
                     | resources set to UNACCESSIBLE_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SM/005

| TP Id | TP/oneM2M/CSE/SM/005 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects creation of <softwareCampaign> resource if it conflicts with any existing <softwareCampaign> resource in the hosting CSE |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT having a valid <softwareCampaign> resource containing softwareTargets attribute set to SOFTWARE_TARGETS and campaignEnabled set to TRUE and the AE having the privileges to perform CREATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid CREATE Request from AE containing  | |
                     | Resource Type set to 61? (softwareCampaign)  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and   | |
                     | Content containing  | |
                     | softwareCompaign resource representation containing  | |
                     | softwareTargets attribute set to SOFTWARE_TARGETS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4105 (CONFLICT)  | |
                     |} | |
+———————-+——————————————————————————————————————————————–+————-+

###### TP/oneM2M/CSE/SM/006

| TP Id | TP/oneM2M/CSE/SM/006 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects UPDATE of <softwareCampaign> resource when the campaignStatus attribute is set to CANCELLING |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignStatus attribute set to CANCELLING and the AE having the privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | Valid <softwareCampaign> resource representation} | |

- +-------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SM/007

| TP Id | TP/oneM2M/CSE/SM/007 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects UPDATE of <softwareCampaign> resource when the campaignEnabled attribute is set to TRUE and attributes mentioned in the request do not match with the attribute values of existing targeted <softwareCampaign> resource |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and the AE having the privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | <softwareCampaign> resource representation containing  | |
                     | MANDATORY_ATTR not matching with the existing <softwareCampaign> attribute values  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |

| Note: MANDATORY_ATTR can be softwareVersion, softwareName, softwareURL, softwareTargets, softwareTriggerCriteria, or softwareOperation. |  |  |
| --- | --- | --- |

TP/oneM2M/CSE/SM/008

| TP Id | TP/oneM2M/CSE/SM/008 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE request of <softwareCampaign> resource if target node referenced in softwareTargets attribute does not exist |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and the AE having the privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTargets attribute set to INVALID_RESOURCE_ADDRESS  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+—————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/SM/009

| TP Id | TP/oneM2M/CSE/SM/009 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects UPDATE of <softwareCampaign> resource when the AE does not have the privilege to perform operation on the resource referenced in softwareTargets attribute |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS and the AE having the privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the AE not having the privileges to perform CREATE operation on NODE_RESOURCE_ADDRESS } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTargets attribute set to NODE_RESOURCE_ADDRESS  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41XX (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SM/010

| TP Id | TP/oneM2M/CSE/SM/010 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects UPDATE of <softwareCampaign> resource if it conflicts with any existing <softwareCampaign> resource in the hosting CSE |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT having a <softwareCampaign> resource at SOFTWARE_RESOURCE_ADDRESS containing softwareTargets attribute set to SOFTWARE_TARGETS and campaignEnabled set to TRUE and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS and the AE having the privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the IUT having the privileges to perform CREATE operation on SOFTWARE_TARGETS } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE-ID and   | |
                     | Content containing  | |
                     | softwareCompaign resource representation containing  | |
                     | softwareTargets attribute set to SOFTWARE_TARGETS  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 41XX (SOFTWARE_CAMPAIGN_CONFLICT)  | |
                     |} | |
+———————-+——————————————————————————————————————————————+————-+

###### TP/oneM2M/CSE/SM/011

| TP Id | TP/oneM2M/CSE/SM/011 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects UPDATE of <softwareCampaign> resource if the resource referenced in softwareTriggerCriteria attribute does not exist |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to FALSE and the AE having the privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTriggerCriteria attribute containing  | |
                     | subjectResourceID set to INVALID_RESOURCE_ADDRESS  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4000 (BAD_REQUEST)  | |
                     |} | |
+———————-+————————————————————————————————————————————-+————-+

###### TP/oneM2M/CSE/SM/012

| TP Id | TP/oneM2M/CSE/SM/012 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects UPDATE of <softwareCampaign> resource if the resource referenced in softwareTriggerCriteria attribute is not accessible by the AE |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to FALSE and the IUT having subject resource at SUBJECT_RESOURCE_ADDRESS and the AE having the privileges to perform UPDATE operation on TARGET_RESOURCE_ADDRESS and the AE not having the privileges to perform CREATE operation on SUBJECT_RESOURCE_ADDRESS } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |IUT <= AE |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | softwareCampaign resource representation containing  | |
                     | softwareTriggerCriteria attribute containing  | |
                     | subjectResourceId set to SUBJECT_RESOURCE_ADDRESS  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4103 (ORIGINATOR_HAS_NO_PRIVILEGE)  | |
                     |} | |
+———————-+————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/SM/013

| TP Id | TP/oneM2M/CSE/SM/013 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends the create request for [software] specialization resource on targeted CSE when IUT successfully creates a <softwareCampaign> resource with campaignEnabled attribute set to TRUE |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUT having privileges to perform CREATE operation on NODE_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |  |
                     | the IUT receives a valid CREATE request from AE containing  |AE => IUT |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to AE_ID and  | |
                     | Content containing  | |
                     | <softwareCampaign> resource representation containing  | |
                     | campaignEnabled attribute set to TRUE  | |
                     | softwareTargets attribute set to NODE_RESOURCE_ADDRESS  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |AE <= IUT   |
                     | the IUT sends a valid response to AE containing  |IUT => CSE  |
                     | Response status code set to 2001 (CREATED)  | |
                     |and the IUT sends a valid CREATE request to CSE containing  | |
                     | To set to NODE_RESOURCE_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | [software] specialization resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————–+————-+
TP/oneM2M/CSE/SM/014

| TP Id | TP/oneM2M/CSE/SM/014 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sends a CREATE request for <subscription> resource to the [software] specialization resource when the IUT has successfully creates the [software] specialization resource |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and the IUT sends a valid CREATE request to CSE containing To set to NODE_RESOURCE_ADDRESS and From set to CSE_ID and Content containing [software] specialization resource representation } |  |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |  |
                     | the IUT receives a valid Response containing  |IUT <= CSE |
                     | Response Status Code set to 2001(CREATED)  | |
                     |} | |

- +----------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |  |
                     |and the IUT sends a valid CREATE request to CSE containing  |IUT => CSE |
                     | To set to SOFTWARE_SPECIALIZATION_ADDRESS and  | |
                     | From set to CSE_ID and  | |
                     | Content containing  | |
                     | <subscription> resource representation  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/SM/015

| TP Id | TP/oneM2M/CSE/SM/015 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the individualSoftwareStatuses attribute of <softwareCampaign> resource to FAILURE when it is not able to subscribe to the [software] specialization |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and the CSEhaving a [software] specialization at SOFTWARE_SPECIALIZATION_ADDRESS and the IUT having sent a valid CREATE request to CSE containing To set to SOFTWARE_SPECIALIZATION_ADDRESS and From set to CSE_ID and Content containing <subscription> resource representation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 4000(BAD_REQUEST)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT |
                     | the IUT updates the <softwareCampaign> resource  | |
                     | individualSoftwareStatuses set to FAILURE for the corresponding [software] specialization  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SM/016

| TP Id | TP/oneM2M/CSE/SM/016 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the individualSoftwareStatuses attribute of <softwareCampaign> resource to FAILURE when it is not able to create the [software] specialization |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and the IUT having sent a valid CREATE request to CSE containing To set to NODE_RESOURCE_ADDRESS and From set to CSE_ID Content containing [software] specialization resource representation } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 4000(BAD_REQUEST)  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT |
                     | the IUT updates the <softwareCampaign> resource  | |
                     | individualSoftwareStatuses set to FAILURE for the corresponding [software] specialization  | |
                     |} | |
+———————-+————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SM/017

| TP Id | TP/oneM2M/CSE/SM/017 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the individualSoftwareStatuses attribute of <softwareCampaign> resource to FAILURE when it is not able to update the corresponding [software] specialization |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and the CSEhaving a [software] specialization at NODE_RESOURCE_ADDRESS and the IUT having sent a valid UPDATE request to CSE containing To set to SOFTWARE_SPECIALIZATION_ADDRESS and From set to CSE_ID Content containing [software] specialization resource representation } |  |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE => IUT |
                     | the IUT receives a valid Response containing  | |
                     | Response Status Code set to 4000(BAD_REQUEST)  | |
                     |} | |

- +--------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT |
                     | the IUT updates the <softwareCampaign> resource  | |
                     | individualSoftwareStatuses set to FAILURE for the corresponding [software] specialization  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————–+————-+

TP/oneM2M/CSE/SM/018

| TP Id | TP/oneM2M/CSE/SM/018 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the UPDATE <softwareCampaign> request to set the campaignEnabled attribute of <softwareCampaign> resource to FALSE when its local policy is not to cancel the ongoing software management operations |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and campaignStatus attribute set to INITIATED and softwareOperationset to 1(INSTALL) and the IUT having the local policy to not cancel the ongoing software management operations and the CSEhaving a [software] specialization at SOFTWARE_SPECIALIZATION_ADDRESS containing install set to TRUE } |  |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | <softwareCampaign> resource representationcontaining  | |
                     | campaignEnabled set to FALSE  | |
                     |} | |

- +------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4XXX  | |
                     |} | |
+———————-+————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SM/019

| TP Id | TP/oneM2M/CSE/SM/019 |  |
| --- | --- | --- |
| Test objective | Check that the IUT tries to cancel the ongoing software management operation upon receiving an UPDATE <softwareCampaign> request with campaignEnabled attribute set to FALSE when local policy of the IUT is to try to cancel the ongoing software management operations |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and campaignStatus attribute set to INITIATED and softwareOperationset to 1(INSTALL) and the CSEhaving a [software] specialization at SOFTWARE_SPECIALIZATION_ADDRESS containing install set to TRUE } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |AE => IUT |
                     | the IUT receives a valid UPDATE Request from AE containing  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID  | |
                     | Content containing  | |
                     | <softwareCampaign> resource representationcontaining  | |
                     | campaignEnabled set to FALSE  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE  |
                     | the IUT sends a valid UPDATE response to AE containing  |  |
                     | Response Status Code set to 2004 (UPDATED) and  |  |
                     | Content containing  |  |
                     | <softwareCampaign> resource representation containing  |  |
                     | campaignStatus attribute set to CANCELLING  |IUT=> CSE |
                     | and the IUT sends a valid UPDATE Request to CSE containing  | |
                     | Toset to SOFTWARE_SPECIALIZATION_ADDRESSand  | |
                     | From set to CSE_ID  | |
                     | Content containing  | |
                     | [software] specialization resource representationcontaining  | |
                     | install set to FALSE  | |
                     |} | |
+———————-+—————————————————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SM/020

| TP Id | TP/oneM2M/CSE/SM/020 |  |
| --- | --- | --- |
| Test objective | Check that the IUT updates campaignStatus attribute of <softwareCampaign> resource based on the response to the update of targeted [software] specialization resource |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and campaignStatus attribute set to INITIATED and SOFTWATE_OPERATIONset to TRUE and the CSEhaving a [software] specialization at SOFTWARE_SPECIALIZATION_ADDRESS containing install set to TRUE and the IUT having updated the <softwareCampaign> resource containing campaignEnabled set to FALSE and the IUT having sent a valid UPDATE Request to CSE containing Toset to SOFTWARE_SPECIALIZATION_ADDRESS and From set to CSE_ID Content containing [software] specialization resource representationcontaining install set to FALSE } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |CSE => IUT |
                     | the IUT receives a valid UPDATE Response from CSE containing  | |
                     | Response Status Code set to RESPONSE_FROM_CSE  | |
                     |} | |

- +-----------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT updates the <softwareCampaign> resource  | |
                     | campaignStatus attribute set to CAMPAIGN_STATUS  | |
                     |} | |
+———————-+———————————————————————————————————————————————————–+————-+

| TP Id | RESPONSE_FROM_CSE | CAMPAIGN_STATUS |
| --- | --- | --- |
| TP/oneM2M/CSE/SM/020/UPD | 2004 | CANCELLED |
| TP/oneM2M/CSE/SM/020/NO_UPD | 4XXX | STATUS_NOT_CANCELLABLE |

TP/oneM2M/CSE/SM/021

| TP Id | TP/oneM2M/CSE/SM/021 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the individualSoftwareStatuses attribute of <softwareCampaign> resource to FAILURE upon the deletion of [software] specialization and its child <subscription > resource when the expiration time of [software] specialization is exceeded before the completion of software operation |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE having a <node> resource at NODE_RESOURCE_ADDRESS and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attribute set to TRUE and the CSEhaving a [software] specialization at NODE_RESOURCE_ADDRESS containing a child resource <subscription> and the expiration time has exceeded for the [software] specialization } |  |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {  |CSE => IUT |
                     | the IUT receives a valid NOTIFY response containing  | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to CSE  | |
                     | Content containing  | |
                     | notification indicating deletion of [software] specialization and its child <subscription> resource  | |
                     |} | |

- +---------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT |
                     | the IUT updates the <softwareCampaign> resource  | |
                     | individualSoftwareStatuses set to FAILURE for the corresponding [software] specialization  | |
                     | and aggregatedSoftwareStatus set to FAILURE  | |
                     |} | |
+———————-+———————————————————————————————————————————————————————————————————————————————————————————————————+————-+

TP/oneM2M/CSE/SM/022

| TP Id | TP/oneM2M/CSE/SM/022 |  |
| --- | --- | --- |
| Test objective | Check that the IUT sets the aggregatedSoftwareStatus attribute of <softwareCampaign> resource to FAILURE when the IUT receives the FAILURE notification from any one of the [software] specialization |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28 |  |
| Config Id | CF02 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the CSE_1 having a <node> resource at NODE_RESOURCE_ADDRESS and the CSE_2 having a <node> resource at NODE_RESOURCE_ADDRESS_2 and the IUT having a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing campaignEnabled attributeset to TRUE and the CSE_1having a [software] specialization at SOFTWARE_SPECIALIZATION_ADDRESS containing install attributeset to TRUE and a child resource <subscription> and the CSE_2having a [software] specialization at SOFTWARE_SPECIALIZATION_ADDRESS containing install attributeset to TRUE and a child resource <subscription> and the IUT having received a valid NOTIFY Response containing To set to TARGET_RESOURCE_ADDRESS and From set to CSE_1 and Content containing [software] specialization resource representation containing installStatus attribute set to SUCCESSFUL } |  |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |CSE_2 => IUT |
                     | the IUT receives a valid NOTIFY Response containing   | |
                     | To set to TARGET_RESOURCE_ADDRESS and  | |
                     | From set to CSE_2 and  | |
                     | Content containing  | |
                     | [software] specialization resource representation containing  | |
                     | installStatus attribute set to FAILURE  | |
                     |} | |

- +-------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT |
                     | the IUT updates the <softwareCampaign> resource  | |
                     | individualSoftwareStatus_1 attribute set to SUCCESSFUL  | |
                     | individualSoftwareStatus_2 attribute set to FAILURE  | |
                     | aggregatedSoftwareStatus attribute set to FAILURE  | |
                     | campaignStatus attribute set to FINISHED  | |
                     |} | |
+———————-+——————————————————————————————————————————————————————————————-+————-+

TP/oneM2M/CSE/SM/023

| TP Id | TP/oneM2M/CSE/SM/023 |  |
| --- | --- | --- |
| Test objective | Check that the IUT rejects the deletion request when campaignStatus attribute of the <softwareCampaign> resource is set to CAMPAIGN_STATUS |  |
| Reference | TS-0001 [1], clause 9.6.76, 10.2.28, TS-0004 [2], clause 6.6.3.5, 7.4.XX |  |
| Config Id | CF01 |  |
| Parent Release | Release 4 |  |
| PICS Selection | PICS_CSE |  |
| Initial conditions | with { the IUT being in the "initial state" and the IUT having registered an AE and the IUThaving a <softwareCampaign> resource at TARGET_RESOURCE_ADDRESS containing camapaignStatus attribute set to CAMPAIGN_STATUS and the AE having the privileges to perform DELETE operation on TARGET_RESOURCE_ADDRESS } |  |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |when {   |AE => IUT |
                     | the IUT receives a valid DELETE Request from AE containing  | |
                     | Toset to TARGET _RESOURCE_ADDRESS and  | |
                     | From set to AE_ID} | |

- +-----------------------------------------------------------------------------------------------------------------------------------+-------------+

- |then {  |IUT => AE |
                     | the IUT sends a valid Response containing  | |
                     | Response Status Code set to 4005 (OPERATION_NOT_ALLOWED)  | |
                     |} | |
+———————-+———————————————————————————————————————————–+————-+

| TP Id | CAMPAIGN_STATUS |
| --- | --- |
| TP/oneM2M/CSE/SM/0023/CANCEL | CANCELLING |
| TP/oneM2M/CSE/SM/0023/INITIATE | INITIATED |

Annex: A

Conformance Test Requirement

## A.1	MQTT Protocol Conformance Test Requirement

| Test Items |  | TS Chapter | Obligation | Condition for Optional Item | Checking point | Fail condition |
| --- | --- | --- | --- | --- | --- | --- |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |TS-0010, clause 5.1 |Mandatory | |"Retain" flag set as "0" |Fail if DUT sends with "Retain" flag=1 |
+————-+———-+———————+————–+————————————-+———————————————————————————————+————————————————-+ |Connection  |Connect |TS-0010, clause 6.3 |Mandatory | |Prefix A:: or C:: is added to the ID |Fail if Prefix A:: or C:: are not added to the ID| |Items | | | | | | |

- +          +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- | |TS-0010, clause 6.3 |Mandatory | |if Clean session (ex: Retain) flag is   |Fail when clean session is "True" when  |
            | | | | |set to "False" in CONNECT | DUT want to be connected |

- +          +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- | |TS-0010, clause 6.3 |Mandatory | |if "Will Flag" is not set in CONNECT |Fail when will flag is enabled |

- +----------+---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |Disconnect|TS-0010 , clause 6.3 |Mandatory | |if Clean session (ex: Retain) flag is   |Fail when clean session is "False" when  |
            | | | | |set to "True" in CONNECT | DUT want to be disconnected |
+————-+———-+———————+————–+————————————-+———————————————————————————————+————————————————-+ |Subscription Items |TS-0010, clause 5.1 |Mandatory | |if the message is set to Durable |Fail is the message is not Durable |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |TS-0010, clause 6.4.4|Conditional |(for Initial Registration)  |if DUT's Initial ID assigning   |Fail if DUT's ID assigning is other than this |
                       | | |when Originator doesn't knows it's ID|/oneM2M/reg_req/<originator>/<receiver> oneM2M/reg_resp/<originator>/<receiver> | |
+————-+———-+———————+————–+————————————-+———————————————————————————————+————————————————-+ |Publish Item |TS-0010,   |Conditional |when Originator knows   |if DUT's Initial ID assigning   |Fail if DUT's ID assigning is other than this | | |Figure 6.4.5-2 | |it's ID | oneM2M/req/<Originator>/<Receiver> | |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |TS-0010,   |Conditional |When Originator uses   |if DUT's Initial ID assigning oneM2M/req/+/<Receiver> |Fail if DUT's ID assigning is other than this |
                       |Figure 6.4.5-2 | |wild card | | |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |MQTT v3.1.1 |Conditional | |if the has proper Primitive contents  |Fail if any of primitive contents feature wrong |
                       | | | | ("op", "fr", "to", "ri", "pc", "ty") | |
+————-+———-+———————+————–+————————————-+———————————————————————————————+————————————————-+ |Reception Item |TS-0010,   |Conditional |when Originator knows   |if DUT's Initial ID assigning   |Fail if DUT's ID assigning is other than this | | |Figure 6.4.5-2 | |it's ID |oneM2M/resp/<Originator>/<Receiver> | |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |TS-0010,   |Conditional |When Originator uses   |if DUT's Initial ID assigning   |Fail if DUT's ID assigning is other than this |
                       |Figure 6.4.5-2 | |wild card |oneM2M/resp/<Originator>/+ | |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |TS-0010,   |Conditional | |server Delete received contents properly   |Fail if Receiver stores message |
                       |Figure 6.4.5-2 | | |not storing data | |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |TS-0010,   |Conditional | |If the receiver sends PubACK   |Fail if the receiver doesn't send PubACK |
                       |Figure 6.4.5-2 | | |when receives messages | |

- +---------------------+--------------+-------------------------------------+---------------------------------------------------------------------------------------------+-------------------------------------------------+

- |MQTT v3.1.1 |Mandatory | |if the has proper Primitive contents  |Fail if any of primitive contents feature wrong |
                       | | | | ("op", "fr", "to", "ri", "pc", "ty") | |
+————-+———-+———————+————–+————————————-+———————————————————————————————+————————————————-+

<PAGE BREAK>

Annex <B>(Informative):

## B.1	TP template

| TP Id |  |  |
| --- | --- | --- |
| Test objective |  |  |
| Reference |  |  |
| Config Id |  |  |
| PICS Selection |  |  |
| Initial conditions |  |  |

- +---------------+-------------+

- |when {   |IUT <= AE |
                     |} | |

- +---------------+-------------+

- |then {   |IUT => AE |
                     |} | |
+———————-+—————+————-+

# History

| Publication history |  |  |
| --- | --- | --- |
|  | := := := := | := := := := |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V4.0.0 | 2020-06-02 | This version of the document is based on TS-0018 V3.8.0 Incorporated agreed documents from TDE#44.1 TDE-2020-0023R01-Preparation_of_TS-0018_Rel-4 |
| V4.1.0 | 2020-07-21 | Incorporated agreed documents from TDE#45.2 TDE-2020-0068-TS-0018_New_TPs_for_service_charging_and_accounting_R4 TDE-2020-0069R01-TS-0018_New_TPs_for_service_subscription_user_profile_R4 |
| V4.2.0 | 2020-12-22 | Incorporated agreed documents from TDE#46, TDE#48 TDE-2020-0078R01-TS-0018_New_TPs_for_discovery_based_operations_R4 TDE-2020-0101R01-TS-0018_New_TPs_for_service_subscription_user_profile_R4 |
| V4.3.0 | 2021-04-08 | Incorporated agreed documents from TDE#49 TDE-2020-0100R04-TS-0018_New_TPs_for_primitiveProfile_R4 TDE-2020-0102R02-TS-0018_New_TPs_for_3GPP_interworking_R4 TDE-2020-0109R01-TS-0018_modify_Service_Subscription_User_Profile_cluase_R4 TDE-2021-0001-TS-0018_New_TPs_for_discovery_based_operations_R4 |
| V4.4.0 | 2021-06-30 | Incorporated agreed documents from TDE#49.2, TDE#49.4, TDE#50 TDE-2021-0002R02-TS-0018_New_TPs_for_cross_resource_notification_R4 TDE-2021-0010R02-TS-0018_New_TPs_for_semantics_based_operations_R4 TDE-2021-0014R02-TS-0018_Addition_of_transaction_management_test_case_for_common_action TDE-2021-0017R04-TS-0018_New_TPs_for_transaction_mgmt_R4 TDE-2021-0022R01-TS-0018_New_TPs_for_service_subscription_profile_R4 TDE-2021-0025R01-Charging_TPs TDE-2021-0026R02-TS-0018_New_TPs_for_schedule_functionality_R4 TDE-2021-0027R01-TS-0018_New_TPs_for_discovery_geoquery_R4 TDE-2021-0033R01-New_TestPurposes_Semantics_R4 TDE-2021-0035R01-New_TPs_for_Primitive_Profile |
| V4.5.0 | 2021-11-18 | Incorporated agreed documents from TDE#51 TDE-2021-0045R03-TS-0018_New_TPs_for_serviceSubscriptionProfile TDE-2021-0049R03-New_TPs_for_ProcessManagement TDE-2021-0050R02-New_TPs_for_FlexContainer_Resource |
| V4.6.0 | 2022-04-06 | Incorporated agreed documents from TDE#51.3, TDE#52, TDE#53 TDE-2021-0056R04-New_TPs_for_ProcessManagement-Update-Part-1 TDE-2021-0057R02-New_TPs_for_ProcessManagement-Update-Part-2 TDE-2021-0060R07-Software_Management_Resource_TPs TDE-2021-0061R01-New_TPs_for_Access_Control_Policy_Resource TDE-2021-0062R02-TS-0018_New_TPs_for_3GPP_Interworking_SCEF_NIDD TDE-2021-0067-SDT_Device_model_TPs TDE-2021-0076R04-New_TPs_for_SUB_NOTIF_notifContentType_notifEventType_R4 TDE-2021-0078R01-New_TP_for_Empty_memberID_R4 TDE-2022-0009-TS-0018_Changes_for_announcement_TPs_R4 |
| V4.6.1 | 2025-09-07 | This baseline is markdown converted baseline |