logo

| oneM2M Technical Specification |  |
| --- | --- |
| Document Number | TS-0025-V-3.2.1 |
| Document Name: | Product profiles |
| Date: | 2025-06-27 |
| Abstract: | The document provides definition to product profiles that comply with oneM2M standards. |
| Template Version: 08 September 2015 (Dot not modify) |  |

This Specification is provided for future development work within oneM2M only. The Partners accept no liability for any use of this Specification.

The present document has not been subject to any approval process by the oneM2M Partners Type 1. Published oneM2M specifications and reports for implementation should be obtained via the oneM2M Partners’ Publications Offices.

About oneM2M

The purpose and goal of oneM2M is to develop technical specifications which address the need for a common M2M Service Layer that can be readily embedded within various hardware and software, and relied upon to connect the myriad of devices in the field with M2M application servers worldwide.

More information about oneM2M may be found at: http//www.oneM2M.org

Copyright Notification

© 2015, oneM2M Partners Type 1 (ARIB, ATIS, CCSA, ETSI, TIA, TSDSI, TTA, TTC).

All rights reserved.

The copyright extends to reproduction in all media.

Notice of Disclaimer & Limitation of Liability

The information provided in this document is directed solely to professionals who have the appropriate degree of experience to understand and interpret its contents in accordance with generally accepted engineering or other professional standards and applicable regulations. No recommendation as to products or vendors is made or should be implied.

NO REPRESENTATION OR WARRANTY IS MADE THAT THE INFORMATION IS TECHNICALLY ACCURATE OR SUFFICIENT OR CONFORMS TO ANY STATUTE, GOVERNMENTAL RULE OR REGULATION, AND FURTHER, NO REPRESENTATION OR WARRANTY IS MADE OF MERCHANTABILITY OR FITNESS FOR ANY PARTICULAR PURPOSE OR AGAINST INFRINGEMENT OF INTELLECTUAL PROPERTY RIGHTS. NO oneM2M PARTNER TYPE 1 SHALL BE LIABLE, BEYOND THE AMOUNT OF ANY SUM RECEIVED IN PAYMENT BY THAT PARTNER FOR THIS DOCUMENT, WITH RESPECT TO ANY CLAIM, AND IN NO EVENT SHALL oneM2M BE LIABLE FOR LOST PROFITS OR OTHER INCIDENTAL OR CONSEQUENTIAL DAMAGES. oneM2M EXPRESSLY ADVISES ANY AND ALL USE OF OR RELIANCE UPON THIS INFORMATION PROVIDED IN THIS DOCUMENT IS AT THE RISK OF THE USER.

# Contents

1 Scope
2 References
    2.1 Normative references
    2.2 Informative references
3 Definitions, symbols and abbreviations
    3.1 Definitions
    3.2 Symbols
    3.3 Abbreviations
    3.4 Acronyms
4 Conventions
5 Product service profiles
    5.1 Introduction
    5.2 Constrained sensor as ADN
        5.2.1 Profile description
        5.2.2 Profile usage examples
        5.2.3 Fundamental feature sets
    5.3 Constrained actuator as ADN
        5.3.1 Profile description
        5.3.2 Profile usage examples
        5.3.3 Fundamental feature sets
        5.3.4 ADN1-requestReachable
        5.3.5 ADN1- pollingChannel
    5.4 ADN profile 3
        5.4.1 Profile description
        5.4.2 Profile usage examples
        5.4.3 Fundamental feature sets
        5.4.4 Extendable feature sets
    5.5 ADN Profile 4
        5.5.1 Profile description
        5.5.2 Profile usage examples
        5.5.3 Fundamental feature sets
    5.6 IN Profile
        5.6.1 Profile description
        5.6.2 Profile usage examples
        5.6.3 Fundamental feature sets
    5.7 Constrained actuator as ASN
        5.7.1 Profile description
        5.7.2 Profile usage examples
        5.7.3 Fundamental feature sets
        5.7.4 ASN-requestReachable
        5.7.5 ASN- pollingChannel
    5.8 Gateway as MN
        5.8.1 Profile description
        5.8.2 Profile usage examples
        5.8.3 Fundamental feature sets
    5.9 Field configurable oneM2M Entity Hosted on a 3GPP Device
        5.9.1 Profile description
        5.9.2 Profile usage examples
        5.9.3 Fundamental feature sets
History

# 1 Scope

The document specifies the detailed defined product profiles that can be used by manufactures and service providers where for each dedicated product, one of the defined product profile can be selected. The product profile would provide guidance to what features shall be implemented, what features should be implemented and what features may be implemented. The present document also describes the test purposes that need to go through if the product need to be certified.

# 2 References

## 2.1 Normative references

Void

## 2.2 Informative references

- [i.1] oneM2M Drafting Rules (http://member.onem2m.org/Static_pages/Others/Rules_Pages/oneM2M-Drafting-Rules-V1_0.doc)

- [i.2] TR-0032-Platform_features_catalogue

# 3 Definitions, symbols and abbreviations

## 3.1 Definitions

Void

## 3.2 Symbols

Void

## 3.3 Abbreviations

Void

## 3.4 Acronyms

Void

# 4 Conventions

The key words “Shall”, “Shall not”, “May”, “Need not”, “Should”, “Should not” in this document are to be interpreted as described in the oneM2M Drafting Rules [i.1]

# 5 Product service profiles

## 5.1 Introduction

The clause defines Product Profiles. Product may be Implementations of oneM2M specification. The Implementation is claimed to comply with one or multiple Product Profiles. Each profile is defined with fundamental feature set and extendable feature set. Fundamental feature set includes Features that shall be implemented if the Implementation under test needs to be certified against the profile. Extendable feature set includes features that are recommanded to be implemented for optimization purpose. The features in the extendable feature set may be tested based on the requirement. The features in the clause are defined in TR-0032[i.2]. Each profile is also composed of a profile description and a profile example. The profile description is the definition of profile and describes the suitable products that should comply to this profile. The profile example gives some real market product examples as well as the usage and scenarios of the product.

Each of the Profile contains the following Features as common Features.

Table 5.1-1: Fundamental feature sets for all Profiles

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| GEN | GE/GEN/00001 | At least one | Support one of the bindings. |
|  | GE/GEN/00002 | At least one | Support one of the serializations. |

## 5.2 Constrained sensor as ADN

### 5.2.1 Profile description

The profile defines constraint sensor that is powered by battery and intended to be used for years after deployment without any human interference for maintenance. Therefore, the function of constraint sensor need to be very limited.

A sensor is intended to be reporting sensed data to the CSE periodically or according to some other period. Everytime the sensor reports data, it sends one message to the CSE. In order to save battery, the sensor can be sleeping before and after the reporting of sensed data.

For the successful report of data, there is needed some context resource architecture like the AE resource, the container resource etc. The resource architecture can be pre-configured in order to minimize the complexity of the constraint sensor.

### 5.2.2 Profile usage examples

The profile applies to constraint sensors that report periodically sensed data. Some example of this kind of product include:

Electricity, water, gas meters that report consumed amount of electricity, water or gas to the CSE. The meters are deployed inside houses and are powered by battery.

Temperature, humidity sensors that are deployed in the open air for environment monitoring. The sensors can be deployed in places that are hardly reachable for people that the sensor’s need to be working without human inference for years powered with battery.

### 5.2.3 Fundamental feature sets

For constrained sensors, the features in the fundamental feature sets assume that the registration relationship between the ADN and the registrar CSE has been pre-configured before hand. The <container> resource has also been pre-configured and the resource id of the <container> resource was provisioned to the ADN as well. The ADN could utilize the resource id of the <container> resource to create <contentInstance> directly.

Table 5.2.3-1: Fundamental feature sets for constrained sensor as ADN

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| GEN | AE/GEN/00001 | At least one | Support one of the format of resource identification |
|  | AE/GEN/00002 | AE/GEN/00002/00001 | Support create request |
| DMR | AE/DMR/00002 | AE/DMR/00002/00001 | Support create contentInstance |

## 5.3 Constrained actuator as ADN

### 5.3.1 Profile description

The profile defines constraint actuator that is powered by battery and intended to be used fro years after deployment without any human interference for maintenance. Therefore, the function of constraint actuator need to be very limited.

An actuator is intended to be receiving control command from CSE via notification or other means. The actuator then actuates according to the control command.

The actuator need to receive control command. As a result, the actuator need to be request reachable to receive notification or be able to start a polling channel.

### 5.3.2 Profile usage examples

Swithes deployed along the street light to switch on or off the street light remotely.

### 5.3.3 Fundamental feature sets

Table 5.3.3-1: Fundamental feature sets for constrained sensor as ADN

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| GEN | AE/GEN/00001 | At least one | Resource identifiers |
| GEN | AE/GEN/00002 | AE/GEN/00002/00001 | Support Create request targeting one resource |
| REG | AE/REG/00002 | AE/REG/00002/00001 | Create <AE> with mandatory attributes[2] |
| REG | AE/REG/00002 | AE/REG/00002/00003 | Create <AE> with pointOfAccess |
| SUB | AE/SUB/00001 | AE/SUB/00001/00001 | Create <subscription> with mandatory attributes [2] |

### 5.3.4 ADN1-requestReachable

Table 5.3.4-1: Notification feature sets for constrained actuator as ADN

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| SUB | AE/SUB/00004 | AE/SUB/00004/00001 | Receiving a notification |

### 5.3.5 ADN1- pollingChannel

Table 5.3.5-1: Polling Channel feature sets for constrained actuator as ADN

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| PCH | GE/PCH/00001 | GE/PCH/00001/00001 GE/PCH/00001/00002 GE/PCH/00001/00003 | Polling Channel resource management |
| PCH | GE/PCH/00002 | GE/PCH/00002/00001 | Long Polling Procedure |

## 5.4 ADN profile 3

### 5.4.1 Profile description

This profile defines features for normal sensor devices or software components that desire to implement oneM2M sensing services. The sensing services such as monitoring temperature, detecting illumination, collection of location information etc are characterized with collecting and uploading measurement data into a destination e.g. a repository. The scope of normal sensor devices cover those that are powered by electricity power or by battery that can be easily changed when the battery is off and have rich resources compared to resource-constraint sensor devices.

The sensing service profile defines a set of features required to implement sensing functionalities including creation and update of <container>, creation of <contentInstance> etc.

### 5.4.2 Profile usage examples

This profile applies to normal sensors deployed in non critial environments have relative rich resources and are powered by electricity power or by battery that can be easily changed. This profile can also apply to software components that implements sensing services.

Taking smart socket as an example, when a manufacturer-A decides to design a smart socket with oneM2M sensing service, he can refer to the Sensing Service Profile to check the fundamental features that are required to implement. The fundamental features for Sensing Service Profile are defined in Section 7.4.3 Feature Set. When another manufacturer-B also designs a smart socket complying with the Sensing Service Profile, these two smart sockets produced from two different manufacturers are interoperable potentially to work together to implement complex tasks.

### 5.4.3 Fundamental feature sets

Container is used for storing the measurements of sensing device and whenever there is data measured from the sensing device, the data will be sent to the CSE for storage by sending a <contentInstance> create request from the originator where the encoded measurement is included within the payload of the <contentInstance> create request.

oneM2M sensing service profile consists of minimum features to implement Data Management for Container and ContentInstance functionality shown in Table 7.4.3-1.

Table 5.4.3-1: Fundamental feature set for ADN profile 3

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| GEN | AE/GEN/00001 | At least one | Support one of the format of resource identification |
|  | AE/GEN/00002 | AE/GEN/00002/00001 | Support Create request targeting one resource |
| REG | AE/REG/00002 | AE/REG/00002/00001 | Create <AE> with mandatory attributes |
| DMR | AE/DMR/00001 | AE/DMR/00001/00001 | Create <container> with no attribute set |
|  | AE/DMR/00002 | AE/DMR/00002/00001 | Create <contentInstance> with mandatory attributes |
| SUB | AE/SUB/00004 | AE/SUB/00004/00001 | Receiving a notification |

### 5.4.4 Extendable feature sets

## 5.5 ADN Profile 4

### 5.5.1 Profile description

oneM2M provides serveral logical entity concepts called ‘Node’ to explain oneM2M Architecture in TS-0001. And from the descriptions about the node types indicates some examples that particular node could match with pyshical oneM2M devices (e.g. ADN -> constrainted oneM2M Device). But for the oneM2M device developers, may require more detail information to design and implement physical devices.

This ADN Profile 1 defines set of common features which could be used for small originator device types of oneM2M services.

### 5.5.2 Profile usage examples

ADN Profile 1 could be the basic profile of oneM2M devices like sensor, actuator etc. And also this profile could be used independently when developers want to implement oneM2M device with special functions.

### 5.5.3 Fundamental feature sets

Table 5.5.3-1: Feature set for ADN Profile 4 GEN

| Feature Set | Feature | Remark |
| --- | --- | --- |
| AE/GEN/00001 | At least one | Support one of the format of resource identification |
| AE/GEN/00002 | AE/GEN/00002/00001 | Support Create request targeting one resource |
|  | AE/GEN/00002/00002 | Support Retrieve request targeting one resource |
|  | AE/GEN/00002/00003 | Support Update request targeting one resource |

Table 5.5.3-2: Feature set for ADN Profile 4 REG

| Feature Set | Feature | Remark |
| --- | --- | --- |
| AE/REG/00001 | AE/REG/00001/00001 | The Retrieval of <CSEBase> resource |
| AE/REG/00002 | AE/REG/00002/00001 | Create <AE> with mandatory attributes |
|  | AE/GEN/00003/00001 | Create <AE> with resourceNameattribute |
|  | AE/GEN/00003/00002 | Create <AE> with expirationTimeattribute |
|  | AE/DIS/00001/00017 | Create <AE> with labels |
|  | AE/REG/00002/00002 | Create <AE> with applicationName |
|  | AE/REG/00002/00003 | Create <AE> with pointOfAccess |
|  | AE/REG/00002/00004 | Create <AE> with nodeLink |
|  | AE/REG/00002/00005 | Create <AE> with contentSerialization |
|  | AE/GEN/00003/00002 | Update <AE> with expirationTime |
|  | AE/DIS/00001/00017 | Update <AE> with labels |
|  | AE/REG/00002/00007 | Update <AE> with applicationName |
|  | AE/REG/00002/00008 | Update <AE> with pointOfAccess |
|  | AE/REG/00002/00009 | Update <AE> with nodeLink |
|  | AE/GEN/00002/00005 | Update <AE> with requestReachability |
|  | AE/REG/00002/00010 | Update <AE> with contentSerialization |
|  | AE/REG/00002/00013 | Delete <AE> |
|  | AE/REG/00002/00012 | Retrieve <AE> |

Table 5.5.3-3: Feature set for ADN Profile 4 DMR

| Feature Set | Feature | Remark |
| --- | --- | --- |
| AE/DMR/00001 | AE/DMR/00001/00001 | Create <container> with no attribute set |
|  | AE/GEN/00003/00001 | Create <container> with resourceName |
|  | AE/GEN/00003/00002 | Create <container> with expirationTime |
|  | AE/DIS/00001/00017 | Create <container> with labels |
|  | AE/DMR/00001/00002 | Create <container> with maxNrOfInstances |
|  | AE/DMR/00001/00003 | Create <container> with maxByteSize |
|  | AE/DMR/00001/00004 | Create <container> with maxInstanceAge |
|  | AE/GEN/00003/00002 | Update <container> with expirationTime |
|  | AE/DIS/00001/00017 | Update <container> with labels |
|  | AE/DMR/00001/00006 | Update <container> with maxNrOfInstances |
|  | AE/DMR/00001/00007 | Update <container> with maxByteSize |
|  | AE/DMR/00001/00008 | Update <container> with maxInstanceAge |
|  | AE/DMR/00001/00011 | Delete <container> |
|  | AE/DMR/00001/00012 | Retrieve <container> |
| AE/DMR/00002 | AE/DMR/00002/00001 | Create <contentInstance> with mandatory attributes |
|  | AE/GEN/00003/00001 | Create <contentInstance> with resourceName |
|  | AE/GEN/00003/00002 | Create <contentInstance> with expirationTime |
|  | AE/DIS/00001/00017 | Create <contentInstance> with labels |
|  | AE/DMR/00002/00002 | Create <contentInstance> with contentInfo |
|  | AE/DMR/00002/00003 | Create <contentInstance> with contentRef |
|  | AE/DMR/00002/00004 | Retrieve <contentInstance> resource targeting the <contentInstance> resource |
|  | AE/DMR/00002/00005 | Retrieve oldest <contentInstance> resource targeting the <oldest> resource |
|  | AE/DMR/00002/00006 | Retrieve latest <contentInstance> resource targeting the <latest> resource |
|  | AE/DMR/00002/00007 | Delete <contentInstance> |
|  | AE/DMR/00002/00008 | Delete oldest <contentInstance> resource targeting the <oldest> resource |
|  | AE/DMR/00002/00009 | Delete latest <contentInstance> resource targeting the <latest> resource |

## 5.6 IN Profile

### 5.6.1 Profile description

oneM2M provides serveral logical entity concepts called ‘Node’ to explain oneM2M Architecture in TS-0001. And from the descriptions about the node types indicates some examples that particular node could match with pyshical oneM2M devices (e.g. ADN -> constrainted oneM2M Device). But for the oneM2M device developers, may require more detail information to design and implement physical devices.

This IN Profile 1 defines set of common features which could be used for server device type of oneM2M services.

### 5.6.2 Profile usage examples

IN Profile 1 could be the common profile of oneM2M devices like service server etc. And also this profile could be used independently when developers want to implement oneM2M device with special functions.

### 5.6.3 Fundamental feature sets

Table 5.6.3-1: Fundamental feature set for IN Profile GEN

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/GEN/00001 | CE/GEN/00001/00001 | Support Unstructured resource identifier |
|  | CE/GEN/00001/00002 | Support Structured resource identifier |
| CE/GEN/00002 | CE/GEN/00002/00001 | Support blocking request |
|  | CE/GEN/00002/00002 | Support ResultContent values of “attributes”, “hierarchical address”, “hierarchical address and attributes”, “nothing” |
|  | CE/GEN/00002/00016 | Support Result Content values of “modified attributes” |
|  | CE/GEN/00002/00004 | Support non-blocking synchronous request handling |
|  | CE/GEN/00002/00005 | Support non-blocking asynchronous request handling |
|  | CE/GEN/00002/00006 | Support for transit CSE forwarding of requests and responses |

Table 5.6.3-2: Fundamental feature set for IN Profile REG

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/REG/00001 | CE/REG/00001/00001 | Support <CSEBase> resource. |
|  | CE/REG/00001/00002 | Support the cseType attribute of <CSEBase> |
|  | CE/REG/00001/00003 | Support the nodeLink attribute of <CSEBase> |
| CE/REG/00002 | CE/REG/00002/00001 | Support <AE> resource |
|  | CE/REG/00002/00002 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘C’. |
|  | CE/REG/00002/00003 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘S’. |
|  | CE/REG/00002/00004 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where no AE-ID Stem is included. |
|  | CE/REG/00002/00007 | Support the applicationName attribute of <AE> resource |
|  | CE/REG/00002/00008 | Support the pointOfAccess attribute of <AE> resource |
|  | CE/REG/00002/00009 | AE registration with pre-assigned AE-ID |
| CE/REG/00004 | CE/REG/00004/00001 | Support <remoteCSE> resource with attributes multiplicity equals 1[1] |
|  | CE/REG/00004/00002 | Support the cseType attribute of <remoteCSE> resource |
|  | CE/REG/00004/00003 | Support the pointOfAccess attribute of <remoteCSE> resource |
|  | CE/REG/00004/00004 | Support the nodeLink attribute of <remoteCSE> resource |

Table 5.6.3-3: Fundamental feature set for IN Profile DMR

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/DMR/00001 | CE/DMR/00001/00001 | Support <container> resource with attributes multiplicity equals 1[1] |
|  | CE/DMR/00002/00001 | Support the creator attribute of <container> resource |
|  | CE/DMR/00001/00002 | Support the maxNrOfInstances attribute of <container> resource |
|  | CE/DMR/00001/00003 | Support the maxByteSize attribute of <container> resource |
|  | CE/DMR/00001/00004 | Support the maxInstanceAge attribute of <container> resource |
| CE/DMR/00002 | CE/DMR/00002/00001 | Support <contentInstance> resource with attributes multiplicity equals 1[1] |
|  | CE/DMR/00002/00001 | Support the creator attribute of <contentInstance> resource |
|  | CE/DMR/00002/00002 | Support the contentInfo attribute of <contentInstance> resource |
|  | CE/DMR/00002/00004 | Retrieve and Delete of latest <contentInstance> resource by <latest> virtual resource |
|  | CE/DMR/00002/00057 | Retrieve and Delete of oldest <contentInstance> resource by <oldest> virtual resource |
| CE/DMR/00003 | CE/DMR/00003/00001 | Support CRUD of <flexcontainer> resource |

Table 5.6.3-4: Fundamental feature set for IN Profile SUB

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/SUB/00001 | CE/SUB/00001/00001 | The Support of <subscription> resource with attributes multiplicity equals 1[1] |
|  | CE/SUB/00001/00001 | Support the creator attribute of <subscription> resource |
|  | CE/SUB/00001/00001 | Support the subscriberURI attribute of <subscription> resource |
| CE/SUB/00004 | CE/SUB/00004/00001 | Notify the address indicated in notificationURI with notification elements multiplicity equals 1[2] |
|  | CE/SUB/00004/00002 | Notify with notificationEvent/representation set in the notification |
|  | CE/SUB/00004/00003 | Notify with creator set in the notification |

Table 5.6.3-5: Fundamental feature set for IN Profile SEC

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/SEC/00001 | CE/SEC/00001/00001 | Support <accessControlPolicy> resource with attributes multiplicity equals 1[1] |
| CE/SEC/00002 | CE/SEC/00002/00001 | Support accessControlPolicyIDs attribute for <resource> that the Implementation supports. |
|  | CE/SEC/00002/00002 | Support access control decision and enforcement making on the Hosting CSE |

Table 5.6.3-6: Fundamental feature set for IN Profile GMG

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/GMG/00001 | CE/GMG/00001/00001 | Support the <group> resource with attributes multiplicity equals 1[1] Support <group> resource with MIXED memberType |
| CE/GMG/00002 | CE/GMG/00002/00001 | Support <group> resource with single specific resource typesSupport consistencyStrategy |
| CE/GMG/00003 | CE/GMG/00003/00001 | Fan out of request to all members |

Table 5.6.3-7: Fundamental feature set for IN Profile DIS

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/DIS/00001 | CE/DIS/00001/00001 | Support the Discovery request with filterUsage equals to “Discovery” |
|  | CE/DIS/00001/00002 | Support discovery with filterCriteria (createdBefore,createdAfter,modifiedSince,unmodifiedSince,stateTagSmaller,stateTagBigger,expireBefore,expireAfter, labels,resourceType,sizeAbove,sizeBelow,contentType, limit, attribute ) |
|  | CE/DIS/00001/00006 | Support discovery with filterCriteria (labels ) |

Table 5.6.3-8: Fundamental feature set for IN Profile PCH

| CE/PCH/00001 | CE/PCH/00001/00001 | Polling Channel resource management |
| --- | --- | --- |
| CE/PCH/00002 | CE/PCH/00002/00001 | Long Polling Procedure |

## 5.7 Constrained actuator as ASN

### 5.7.1 Profile description

The profile defines constraint actuator that is powered by battery and intended to be used for years after deployment without any human interference for maintenance. Therefore, the function of constraint actuator need to be very limited.

An actuator is intended to be receiving control command from CSE via notification or other means. The actuator then actuates according to the control command.

The actuator need to receive control command. As a result, the actuator need to be request reachable to receive notification or be able to start a polling channel.

The features described in this profile are features of CSE as the ASN exposes Mcc reference point outside. There are internal AEs of the ASN to synchronize the actual device state with the resource hosted by the ASN-CSE. However, in this profile, the ASN-AE is not sending information to the other CSE’s, it’s not necessary to include the oneM2M defined AE Features in this profile.

### 5.7.2 Profile usage examples

Swithes deployed along the street light to switch on or off the street light remotely.

### 5.7.3 Fundamental feature sets

The fundamental feature set assumes that the resource id of the <container> resource needed for create <contentInstance> is pre-configured at the ASN-CSE and provisioned to the AEs that tends to send request to the ASN-CSE.

Table 5.7.3-1: Fundamental feature sets for constrained sensor as ASN GEN

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/GEN/00001 | CE/GEN/00001/00001 | Unstructured resource id |
|  | CE/GEN/00001/00002 | Structured resource id |
| CE/GEN/00002 | CE/GEN/00002/00001 | Support blocking request |

Table 5.7.3-2: Fundamental feature sets for constrained sensor as ASN REG

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/REG/00005 | CE/REG/00005/00001 | The Creation with <remoteCSE> resource for CSE registration with mandatory attributes[2] |
|  | CE/REG/00005/00006 | Create the <remoteCSE> with pointOfAccess |

Table 5.7.3-3: Fundamental feature sets for constrained sensor as ASN DMR

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/DMR/00002 | CE/DMR/00002/00001 | Support <contentInstance> resource with attributes multiplicity equals 1[1] |

### 5.7.4 ASN-requestReachable

Table 5.7.4-1: Notification feature sets for constrained actuator as ASN

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| SUB | AE/SUB/00004 | AE/SUB/00004/00001 | Receiving a notification |

### 5.7.5 ASN- pollingChannel

Table 5.7.5-1: Polling Channel feature sets for constrained actuator as ASN

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| PCH | GE/PCH/00001 | GE/PCH/00001/00001 GE/PCH/00001/00002 GE/PCH/00001/00003 | Polling Channel resource management |
| PCH | GE/PCH/00002 | GE/PCH/00002/00001 | Long Polling Procedure |

## 5.8 Gateway as MN

### 5.8.1 Profile description

The profile defines gateways that may support multiple different area network technologies and connect devices from the corresponding area networks. The devices may be oneM2M defined ADNs, ASNs or NoDNs which are not defined by oneM2M. The gateway hosts resources to enforce common services that devices in the area network may utilize the enable local services without the dependency of the platform and the wide area network.

### 5.8.2 Profile usage examples

Home gateways deployed in the home for smart home:

Sensors, actuators and home appliances are connected with the home gateway. Applications can be connected through the platfrom to access the smart home service as well as applications can be directly connected with the gateway to do the same thing.

Head unit in connected vehicles:

Sensors and actuators in the car are connected with the head unit. The sensed data and control command are reported to delivered through the head unit. Applications may access the head unit and monitor the car state via the platform. Applications may also access the head unit once connected directly with the head unit via area network such as Wifi or Bluetooth.

### 5.8.3 Fundamental feature sets

Table 5.8.3-1: Fundamental feature sets for gateway as MN GEN

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/GEN/00001 | CE/GEN/00001/00001 | Unstructured resource id |
|  | CE/GEN/00001/00002 | Structured resource id |
| CE/GEN/00002 | CE/GEN/00002/00001 | Support blocking request |

Table 5.8.3-2: Fundamental feature sets for gateway as MN REG

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/REG/00001 | CE/REG/00001/00001 | Support <CSEBase> resource with attributes multiplicity equals 1[1]. |
|  | CE/REG/00001/00002 | Support the cseType attribute of <CSEBase> resource |
|  | CE/REG/00001/00003 | Support the nodeLink attribute of <CSEBase> resource. This is dependent on the feature set for <node> resource. |
| CE/REG/00002 | CE/REG/00002/00001 | Support <AE> resource with attributes multiplicity equals 1[1] |
|  | CE/REG/00002/00002 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘C’. |
|  | CE/REG/00002/00003 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘S’. |
|  | CE/REG/00002/00004 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where no AE-ID Stem is included. |
|  | CE/REG/00002/00005 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘S’ and where announcement to an IN-CSE is needed to get an AE-ID assigned |
|  | CE/DIS/00001/00006 | Support the labels attribute of <AE> resource |
|  | CE/REG/00002/00007 | Support the applicationName attribute of <AE> resource |
|  | CE/REG/00002/00008 | Support the pointOfAccess attribute of <AE> resource |
|  | CE/REG/00002/00009 | AE registration with pre-assigned AE-ID |
| CE/REG/00004 | CE/REG/00004/00001 | Support <remoteCSE> resource with attributes multiplicity equals 1[1] |
|  | CE/DIS/00001/00006 | Support the labels attribute of <remoteCSE> resource |
|  | CE/REG/00004/00002 | Support the cseType attribute of <remoteCSE> resource |
|  | CE/REG/00004/00003 | Support the pointOfAccess attribute of <remoteCSE> resource |
|  | CE/REG/00004/00004 | Support the nodeLink attribute of <remoteCSE> resource |
| CE/REG/00005 | CE/REG/00005/00001 | The Creation with <remoteCSE> resource for CSE registration with mandatory attributes[2] for Create and locally create <remoteCSE> that represents the Regstree CSE |
|  | CE/REG/00005/00001 | Create the <remoteCSE> with resourceName attribute |
|  | CE/REG/00005/00001 | Create the <remoteCSE> with expirationTime attribute |
|  | CE/REG/00005/00004 | Create the <remoteCSE> with labels attribute |
|  | CE/REG/00005/00002 | Create the <remoteCSE> with cseType attribute |
|  | CE/REG/00005/00003 | Create the <remoteCSE> with pointOfAccess attribute |
|  | CE/REG/00005/00005 | Create the <remoteCSE> with nodeLink attribute |
|  | CE/DIS/00001/00006 | Update the <remoteCSE> with labels |
|  | CE/REG/00005/00001 | Update the <remoteCSE> with expirationTime |
|  | CE/REG/00005/00006 | Update the <remoteCSE> with pointOfAccess attribute |
|  | CE/REG/00005/00007 | Update the <remoteCSE> with requestReachability |
|  | CE/REG/00005/00008 | Update the <remoteCSE> with nodeLink attribute |
|  | CE/REG/00005/00001 | Delete the <remoteCSE> resource on the Registrar, also delete the <remoteCSE> resource on the Registree |
|  | CE/REG/00005/00009 | Retrieve the <remoteCSE> resource |

Table 5. 8 .3- 3 : Fundamental feature sets for gateway as M N DMR

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/DMR/00001 | CE/DMR/00001/00001 | Support <container> resource with attributes multiplicity equals 1[1] |
|  | CE/DIS/00001/00006 | Support the labels attribute of <container> resource |
|  | CE/DMR/00001/00001 | Support the creator attribute of <container> resource |
|  | CE/DMR/00001/00002 | Support the maxNrOfInstances attribute of <container> resource |
|  | CE/DMR/00001/00003 | Support the maxByteSize attribute of <container> resource |
|  | CE/DMR/00001/00004 | Support the maxInstanceAge attribute of <container> resource |
| CE/DMR/00002 | CE/DMR/00002/00001 | Support <contentInstance> resource with attributes multiplicity equals 1[1] |
|  | CE/DIS/00001/00006 | Support the labels attribute of <contentInstance> resource |
|  | CE/DMR/00002/00001 | Support the creator attribute of <contentInstance> resource |
|  | CE/DMR/00002/00002 | Support the contentInfo attribute of <contentInstance> resource |
|  | CE/DMR/00002/00004 | Retrieve and Delete of latest <contentInstance> resource by <latest> virtual resource |
|  | CE/DMR/00002/00057 | Retrieve and Delete of oldest <contentInstance> resource by <oldest> virtual resource |

Table 5. 8 .3- 4 : Fundamental feature sets for gateway as M N SUB

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/SUB/00001 | CE/SUB/00001/00001 | The Support of <subscription> resource with attributes multiplicity equals 1[1] |
|  | CE/DIS/00001/00006 | Support the labels attribute of <subscription> resource. |
|  | CE/SUB/00001/00001 | Support the creator attribute of <subscription> resource |
|  | CE/SUB/00001/00001 | Support the subscriberURI attribute of <subscription> resource |
| CE/SUB/00004 | CE/SUB/00004/00001 | Notify the address indicated in notificationURI with notification elements multiplicity equals 1[2] |
|  | CE/SUB/00004/00002 | Notify with notificationEvent/representation set in the notification |
|  | CE/SUB/00004/00003 | Notify with creator set in the notification |

Table 5. 8 .3- 5 : Fundamental feature sets for gateway as M N SEC

| Feature Set | Feature | Remark |
| --- | --- | --- |
| CE/SEC/00001 | CE/SEC/00001/00001 | Support <accessControlPolicy> resource with attributes multiplicity equals 1[1] |
|  | CE/DIS/00001/00006 | Support the labels attribute of <accessControlPolicy> resource |
| CE/SEC/00002 | CE/SEC/00002/00001 | Support accessControlPolicyIDs attribute for <resource> that the Implementation supports. |
|  | CE/SEC/00002/00002 | Support access control decision and enforcement making on the Hosting CSE |

## 5.9 Field configurable oneM2M Entity Hosted on a 3GPP Device

### 5.9.1 Profile description

The profile defines the features required for a oneM2M entity hosted on a 3GPP device. This profile is intended to add required features to those specified in other profiles. For example, a temperature sensor hosted on an CIoT device would required the features described in this section plus the features described for a Constrained Sensor device.

### 5.9.2 Profile usage examples

The profile applies to CIoT devices that can receive trigger messages via a 3GPP core network. The trigger messages are used provision the device with enrolment credentials, register the device to a oneM2M CSE and re-establish a connection to the Registrar CSE.

### 5.9.3 Fundamental feature sets

Table 5.2.3-1: Fundamental feature sets for oneM2M entity hosted on a 3GPP Device

| Function | Feature Set | Feature | Remark |
| --- | --- | --- | --- |
| NSSE | GE/NSSE/00001 | GE/NSSE/00001/00001 | Support processing a trigger request for enrolment |
|  |  | GE/NSSE/00001/00002 | Support processing a trigger request for establishConnection |
|  |  | GE/NSSE/00001/00003 | Support processing a trigger request for registrationRequest |

Annex <A> (Normative): Mapping of feature to test purposes

Table A-1: Mapping of feature to Test Purposes for CSE

| Feature ID | Description | Mapped Test Purposes |
| --- | --- | --- |
| CE/GEN/00001/00001 | Support Unstructured resource identifier | TP/oneM2M/CSE/GEN/CRE/001_CSR TP/oneM2M/CSE/GEN/CRE/001_SPR TP/oneM2M/CSE/GEN/CRE/001_ABS TP/oneM2M/CSE/GEN/UPD/001_CSR TP/oneM2M/CSE/GEN/UPD/001_SPR TP/oneM2M/CSE/GEN/UPD/001_ABS TP/oneM2M/CSE/GEN/RET/001_CSR TP/oneM2M/CSE/GEN/RET/001_SPR TP/oneM2M/CSE/GEN/RET/001_ABS TP/oneM2M/CSE/GEN/DEL/001_CSR TP/oneM2M/CSE/GEN/DEL/001_SPR TP/oneM2M/CSE/GEN/DEL/001_ABS |
| CE/GEN/00001/00002 | Support Structured resource identifier | TP/oneM2M/CSE/GEN/CRE/002_CSR TP/oneM2M/CSE/GEN/CRE/002_SPR TP/oneM2M/CSE/GEN/CRE/002_ABS TP/oneM2M/CSE/GEN/UPD/002_CSR TP/oneM2M/CSE/GEN/UPD/002_SPR TP/oneM2M/CSE/GEN/UPD/002_ABS TP/oneM2M/CSE/GEN/RET/002_CSR TP/oneM2M/CSE/GEN/RET/002_SPR TP/oneM2M/CSE/GEN/RET/002_ABS TP/oneM2M/CSE/GEN/DEL/002_CSR TP/oneM2M/CSE/GEN/DEL/002_SPR TP/oneM2M/CSE/GEN/DEL/002_ABS TP/oneM2M/CSE/GEN/CRE/003_CSR TP/oneM2M/CSE/GEN/CRE/003_SPR TP/oneM2M/CSE/GEN/CRE/003_ABS TP/oneM2M/CSE/GEN/UPD/003_CSR TP/oneM2M/CSE/GEN/UPD/003_SPR TP/oneM2M/CSE/GEN/UPD/003_ABS TP/oneM2M/CSE/GEN/RET/003_CSR TP/oneM2M/CSE/GEN/RET/003_SPR TP/oneM2M/CSE/GEN/RET/003_ABS TP/oneM2M/CSE/GEN/DEL/003_CSR TP/oneM2M/CSE/GEN/DEL/003_SPR TP/oneM2M/CSE/GEN/DEL/003_ABS TP/oneM2M/CSE/GEN/CRE/002_CSR TP/oneM2M/CSE/GEN/CRE/004_SPR TP/oneM2M/CSE/GEN/CRE/004_ABS TP/oneM2M/CSE/GEN/UPD/004_CSR TP/oneM2M/CSE/GEN/UPD/004_SPR TP/oneM2M/CSE/GEN/UPD/004_ABS TP/oneM2M/CSE/GEN/RET/004_CSR TP/oneM2M/CSE/GEN/RET/004_SPR TP/oneM2M/CSE/GEN/RET/004_ABS TP/oneM2M/CSE/GEN/DEL/004_CSR TP/oneM2M/CSE/GEN/DEL/004_SPR TP/oneM2M/CSE/GEN/DEL/004_ABS |
| CE/GEN/00002/00001 | Support blocking request | TP/oneM2M/CSE/RT/BR/001_XXX |
| CE/GEN/00002/00002 | Support of request with result content | TP/oneM2M/CSE/DMR/CRE/015_XXX TP/oneM2M/CSE/DMR/CRE/016_XXX TP/oneM2M/CSE/DMR/CRE/017_XXX TP/oneM2M/CSE/DMR/CRE/018_XXX TP/oneM2M/CSE/DMR/CRE/019_XXX TP/oneM2M/CSE/DMR/CRE/020_XXX TP/oneM2M/CSE/DMR/RET/020_XXX TP/oneM2M/CSE/DMR/RET/021_XXX TP/oneM2M/CSE/DMR/RET/022_XXX TP/oneM2M/CSE/DMR/RET/023_XXX TP/oneM2M/CSE/DMR/RET/024_XXX TP/oneM2M/CSE/DMR/RET/025_XXX TP/oneM2M/CSE/DMR/UPD/019_XXX TP/oneM2M/CSE/DMR/UPD/020_XXX TP/oneM2M/CSE/DMR/UPD/021_XXX TP/oneM2M/CSE/DMR/UPD/022_XXX TP/oneM2M/CSE/DMR/DEL/011_XXX TP/oneM2M/CSE/DMR/DEL/012_XXX TP/oneM2M/CSE/DMR/DEL/013_XXX |
| CE/GEN/00002/00004 | Support non-blocking synchronous request handling | TP/oneM2M/CSE/NBR/SYN/001_XXX TP/oneM2M/CSE/NBR/SYN/002_XXX TP/oneM2M/CSE/NBR/SYN/003_XXX |
| CE/GEN/00002/00005 | Support non-blocking asynchronous request handling | TP/oneM2M/CSE/NBR/ASY/001_XXX TP/oneM2M/CSE/NBR/ASY/002_XXX TP/oneM2M/CSE/NBR/ASY/003_XXX TP/oneM2M/CSE/NBR/ASY/004_XXX TP/oneM2M/CSE/NBR/ASY/005_XXX |
| CE/GEN/00002/00006 | Support for transit CSE forwarding of requests and responses | TP/oneM2M/CSE/DMR/001_XXX TP/oneM2M/CSE/DMR/002_XXX |
| CE/REG/00001/00001 | Support <CSEBase> resource. | TP/oneM2M/CSE/REG/CRE/021 TP/oneM2M/CSE/REG/RET/001 TP/oneM2M/CSE/REG/RET/008 TP/oneM2M/CSE/REG/UPD/001 TP/oneM2M/CSE/REG/DEL/001 |
| CE/REG/00001/00002 | Support the cseType attribute of <CSEBase> | TP/oneM2M/CSE/REG/RET/002_CST TP/oneM2M/CSE/REG/RET/005 |
| CE/REG/00001/00003 | Support the nodeLink attribute of <CSEBase> | TP/oneM2M/CSE/REG/RET/002_NL |
| CE/REG/00002/00001 | Support <AE> resource | TP/oneM2M/CSE/REG/CRE/016 TP/oneM2M/CSE/REG/CRE/017_API TP/oneM2M/CSE/REG/CRE/017_RR TP/oneM2M/CSE/REG/RET/003 TP/oneM2M/CSE/REG/DEL/003 TP/oneM2M/CSE/DMR/RET/013_AE TP/oneM2M/CSE/DMR/UPD/016_AE/ET TP/oneM2M/CSE/DMR/UPD/016_AE/RR |
| CE/REG/00002/00002 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘C’. | TP/oneM2M/CSE/REG/CRE/004 TP/oneM2M/CSE/REG/CRE/011 TP/oneM2M/CSE/REG/CRE/022 TP/oneM2M/CSE/REG/CRE/023 |
| CE/REG/00002/00003 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘S’. | TP/oneM2M/CSE/REG/CRE/020 |
| CE/REG/00002/00004 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where no AE-ID Stem is included. | TP/oneM2M/CSE/REG/CRE/032 |
| CE/REG/00002/00005 | AE registration/de-registration without a pre-assigned AE-ID in the registration request and where the AE-ID Stem is configured with ‘S’ and where announcement to an IN-CSE is needed to get an AE-ID assigned | TP/oneM2M/CSE/REG/CRE/005 TP/oneM2M/CSE/REG/CRE/006 TP/oneM2M/CSE/REG/CRE/007 TP/oneM2M/CSE/REG/CRE/031 TP/oneM2M/CSE/REG/DEL/005 TP/oneM2M/CSE/REG/DEL/006 TP/oneM2M/CSE/REG/DEL/007 |
| CE/DIS/00001/00006 | Support discovery with filterCriteria (labels) | AE TP/oneM2M/CSE/REG/CRE/012_AE/LBL TP/oneM2M/CSE/REG/RET/004_LBL TP/oneM2M/CSE/DMR/UPD/015_AE/LBL |
|  |  | remoteCSE TP/oneM2M/CSE/REG/CRE/013_LBL TP/oneM2M/CSE/REG/CRE/028_LBL TP/oneM2M/CSE/REG/RET/007_LBL TP/oneM2M/CSE/REG/UPD/002_LBL |
|  |  | container TP/oneM2M/CSE/DMR/CRE/012_CNT/LBL TP/oneM2M/CSE/DMR/UPD/014_CNT/LBL TP/oneM2M/CSE/DMR/UPD/015_CNT/LBL |
|  |  | contentInstance TP/oneM2M/CSE/DMR/CRE/012_CIN/LBL |
|  |  | subscription TP/oneM2M/CSE/DMR/CRE/012_SUB/LBL TP/oneM2M/CSE/DMR/UPD/014_SUB/LBL TP/oneM2M/CSE/DMR/UPD/015_SUB/LBL |
|  |  | accessControlPolicy TP/oneM2M/CSE/DMR/CRE/012_ACP/LBL TP/oneM2M/CSE/DMR/UPD/014_ACP/LBL TP/oneM2M/CSE/DMR/UPD/015_ACP/LBL |
|  |  | group TP/oneM2M/CSE/DMR/UPD/015_GRP/LBL |
|  |  | remoteCSE TP/oneM2M/CSE/REG/CRE/026_LBL TP/oneM2M/CSE/REG/UPD/003_LBL TP/oneM2M/CSE/DMR/UPD/015_CSR/LBL\ |
| CE/REG/00002/00007 | Support the applicationName attribute of <AE> resource | TP/oneM2M/CSE/REG/CRE/012_AE/APN TP/oneM2M/CSE/REG/RET/004_APN TP/oneM2M/CSE/DMR/UPD/015_AE/APN |
| CE/REG/00002/00008 | Support the pointOfAccess attribute of <AE> resource | TP/oneM2M/CSE/REG/CRE/012_AE/POA TP/oneM2M/CSE/REG/RET/004_POA TP/oneM2M/CSE/DMR/UPD/015_AE/POA |
| CE/REG/00002/00009 | AE registration with pre-assigned AE-ID | IN-CSE TP/oneM2M/CSE/REG/CRE/001_CAE TP/oneM2M/CSE/REG/CRE/001_SAE TP/oneM2M/CSE/REG/CRE/002_CAE TP/oneM2M/CSE/REG/CRE/002_SAE TP/oneM2M/CSE/REG/CRE/009 TP/oneM2M/CSE/REG/CRE/029 |
|  |  | MN-CSE TP/oneM2M/CSE/REG/CRE/003 TP/oneM2M/CSE/REG/CRE/008 TP/oneM2M/CSE/REG/CRE/010 |
| CE/REG/00004/00001 | Support <remoteCSE> resource with attributes multiplicity equals 1[1] | TP/oneM2M/CSE/REG/CRE/018 TP/oneM2M/CSE/REG/CRE/019 TP/oneM2M/CSE/REG/CRE/025 TP/oneM2M/CSE/REG/RET/006 TP/oneM2M/CSE/REG/RET/010 TP/oneM2M/CSE/REG/DEL/002 TP/oneM2M/CSE/DMR/UPD/016_CSR/ET TP/oneM2M/CSE/DMR/UPD/016_CSR/RR |
| CE/REG/00004/00002 | Support the cseType attribute of <remoteCSE> resource | TP/oneM2M/CSE/REG/CRE/013_CST TP/oneM2M/CSE/REG/RET/007_CST TP/oneM2M/CSE/REG/CRE/027 (MN only) |
| CE/REG/00004/00003 | Support the pointOfAccess attribute of <remoteCSE> resource | TP/oneM2M/CSE/REG/CRE/013_POA TP/oneM2M/CSE/REG/CRE/028_POA TP/oneM2M/CSE/REG/RET/007_POA TP/oneM2M/CSE/REG/UPD/002_POA |
| CE/REG/00004/00004 | Support the nodeLink attribute of <remoteCSE> resource | TP/oneM2M/CSE/REG/CRE/013_NL TP/oneM2M/CSE/REG/CRE/028_NL TP/oneM2M/CSE/REG/RET/007_NL TP/oneM2M/CSE/REG/UPD/002_NL |
| CE/REG/00005/00001 | CSE Registration/De-registration | TP/oneM2M/CSE/REG/CRE/024 TP/oneM2M/CSE/DMR/RET/013_CSR TP/oneM2M/CSE/REG/UPD/003_ET TP/oneM2M/CSE/REG/DEL/004 |
| CE/REG/00005/00001 | Create the <remoteCSE> with resourceName attribute | TP/oneM2M/CSE/REG/CRE/026_RN |
| CE/REG/00005/00001 | Create the <remoteCSE> with expirationTime attribute | TP/oneM2M/CSE/REG/CRE/026_ET |
| CE/REG/00005/00002 | Create the <remoteCSE> with cseType attribute | TP/oneM2M/CSE/REG/CRE/026_CST |
| CE/REG/00005/00003 | Create the <remoteCSE> with pointOfAccess attribute | TP/oneM2M/CSE/REG/CRE/026_POA |
| CE/REG/00005/00005 | Create the <remoteCSE> with nodeLink attribute | TP/oneM2M/CSE/REG/CRE/026_NL |
| CE/REG/00005/00006 | Update the <remoteCSE> with pointOfAccess attribute | TP/oneM2M/CSE/REG/UPD/003_POA TP/oneM2M/CSE/DMR/UPD/015_CSR/POA |
| CE/REG/00005/00007 | Update the <remoteCSE> with requestReachability | TP/oneM2M/CSE/REG/UPD/003_RR |
| CE/REG/00005/00008 | Update the <remoteCSE> with nodeLink attribute | TP/oneM2M/CSE/REG/UPD/003_NL TP/oneM2M/CSE/DMR/UPD/015_CSR/NL |
| CE/REG/00005/00009 | Retrieve the <remoteCSE> resource | TP/oneM2M/CSE/REG/RET/009 |
| CE/DMR/00001/00001 | Support <container> resource with attributes multiplicity equals 1[1] | TP/oneM2M/CSE/DMR/CRE/001_CNT/CB TP/oneM2M/CSE/DMR/CRE/001_CNT/AE TP/oneM2M/CSE/DMR/CRE/001_CNT/CNT TP/oneM2M/CSE/DMR/CRE/002_CNT TP/oneM2M/CSE/DMR/CRE/003_CNT TP/oneM2M/CSE/DMR/CRE/007_CNT/CIN TP/oneM2M/CSE/DMR/CRE/008_CNT/CIN TP/oneM2M/CSE/DMR/CRE/009 TP/oneM2M/CSE/DMR/CRE/010 TP/oneM2M/CSE/DMR/CRE/011 TP/oneM2M/CSE/DMR/CRE/013_CNT_CIN TP/oneM2M/CSE/DMR/CRE/014_CNT_CIN TP/oneM2M/CSE/DMR/RET/001_CNT TP/oneM2M/CSE/DMR/RET/013_CNT TP/oneM2M/CSE/DMR/UPD/001_CNT/LBL TP/oneM2M/CSE/DMR/UPD/002_CNT/LBL TP/oneM2M/CSE/DMR/UPD/003_CNT/LBL TP/oneM2M/CSE/DMR/UPD/004_CNT/ET/MNI/LBL TP/oneM2M/CSE/DMR/UPD/005_CNT/EXC TP/oneM2M/CSE/DMR/UPD/007_CNT/CT TP/oneM2M/CSE/DMR/UPD/008_CNT/ET TP/oneM2M/CSE/DMR/UPD/009_CNT/LBL TP/oneM2M/CSE/DMR/UPD/010_CNT TP/oneM2M/CSE/DMR/UPD/016_CNT/ET TP/oneM2M/CSE/DMR/DEL/001_CNT TP/oneM2M/CSE/DMR/DEL/005 TP/oneM2M/CSE/DMR/DEL/006 |
| CE/DMR/00002/00001 | Support the creator attribute of <container> resource | TP/oneM2M/CSE/DMR/CRE/012_CNT/CR |
| CE/DMR/00001/00002 | Support the maxNrOfInstances attribute of <container> resource | TP/oneM2M/CSE/DMR/CRE/005_MNI TP/oneM2M/CSE/DMR/CRE/006_CNT/MNI TP/oneM2M/CSE/DMR/CRE/012_CNT/MNI TP/oneM2M/CSE/DMR/UPD/014_CNT/MNI TP/oneM2M/CSE/DMR/UPD/015_CNT/MNI |
| CE/DMR/00001/00003 | Support the maxByteSize attribute of <container> resource | TP/oneM2M/CSE/DMR/CRE/005_MBS TP/oneM2M/CSE/DMR/CRE/006_CNT/MBS TP/oneM2M/CSE/DMR/CRE/012_CNT/MBS TP/oneM2M/CSE/DMR/CRE/013_CNT/CIN TP/oneM2M/CSE/DMR/UPD/014_CNT/MBS TP/oneM2M/CSE/DMR/UPD/015_CNT/MBS |
| CE/DMR/00001/00004 | Support the maxInstanceAge attribute of <container> resource | TP/oneM2M/CSE/DMR/CRE/005_MIA TP/oneM2M/CSE/DMR/CRE/006_CNT/MIA TP/oneM2M/CSE/DMR/CRE/012_CNT/MIA TP/oneM2M/CSE/DMR/UPD/014_CNT/MIA TP/oneM2M/CSE/DMR/UPD/015_CNT/MIA |
| CE/DMR/00002/00001 | Support <contentInstance> resource with attributes multiplicity equals 1[1] | TP/oneM2M/CSE/DMR/CRE/001_CIN/CNT TP/oneM2M/CSE/DMR/RET/001_CIN TP/oneM2M/CSE/DMR/UPD/011_CIN/CNT TP/oneM2M/CSE/DMR/DEL/001_CIN |
| CE/DMR/00002/00001 | Support the creator attribute of <contentInstance> resource | TP/oneM2M/CSE/DMR/CRE/012_CIN/CR |
| CE/DMR/00002/00002 | Support the contentInfo attribute of <contentInstance> resource | TP/oneM2M/CSE/DMR/CRE/012_CIN/CNF |
| CE/DMR/00002/00004 | Retrieve and Delete of latest <contentInstance> resource by <latest> virtual resource | TP/oneM2M/CSE/DMR/RET/012 TP/oneM2M/CSE/DMR/UPD/012 TP/oneM2M/CSE/DMR/DEL/007 |
| CE/DMR/00002/00005 | Retrieve and Delete of oldest <contentInstance> resource by <oldest> virtual resource | TP/oneM2M/CSE/DMR/RET/010 TP/oneM2M/CSE/DMR/UPD/013 TP/oneM2M/CSE/DMR/DEL/009 |
| CE/DMR/00003/00001 | Support <flexcontainer> resource with attributes multiplicity equals 1 Support CRUD of <flexcontainer> resource | TP/oneM2M/CSE/DMR/CRE/002_FLXC |
| CE/SUB/00001/00001 | The Support of <subscription> resource with attributes multiplicity equals 1[1] | TP/oneM2M/CSE/DMR/CRE/001_SUB/CB TP/oneM2M/CSE/DMR/CRE/001_SUB/AE TP/oneM2M/CSE/DMR/CRE/001_SUB/CNT TP/oneM2M/CSE/DMR/CRE/001_SUB/ACP TP/oneM2M/CSE/DMR/CRE/001_SUB/CSR TP/oneM2M/CSE/DMR/CRE/001_SUB/GRP TP/oneM2M/CSE/DMR/CRE/002_SUB TP/oneM2M/CSE/DMR/CRE/003_SUB TP/oneM2M/CSE/DMR/RET/001_SUB TP/oneM2M/CSE/DMR/RET/013_SUB TP/oneM2M/CSE/DMR/UPD/001_SUB/LBL TP/oneM2M/CSE/DMR/UPD/002_SUB/LBL TP/oneM2M/CSE/DMR/UPD/003_SUB/LBL TP/oneM2M/CSE/DMR/UPD/004_SUB/ET/LBL/EXC TP/oneM2M/CSE/DMR/UPD/005_SUB/MNI TP/oneM2M/CSE/DMR/UPD/007_SUB/CT TP/oneM2M/CSE/DMR/UPD/008_SUB/ET TP/oneM2M/CSE/DMR/UPD/009_SUB/LBL TP/oneM2M/CSE/DMR/UPD/016_SUB/ET TP/oneM2M/CSE/DMR/UPD/016_SUB/NU TP/oneM2M/CSE/DMR/UPD/016_SUB/NCT TP/oneM2M/CSE/DMR/DEL/001_SUB |
| CE/SUB/00001/00001 | Support the creator attribute of <subscription> resource | TP/oneM2M/CSE/DMR/CRE/012_SUB/CR |
| CE/SUB/00004/00001 | Support the subscriberURI attribute of <subscription> resource | TP/oneM2M/CSE/DMR/CRE/012_SUB/SU |
| CE/SUB/00004/00001 | Notify the address indicated in notificationURI with notification elements multiplicity equals 1[2] | TP/oneM2M/CSE/SUB/CRE/001_SUB TP/oneM2M/CSE/SUB/CRE/001_CIN TP/oneM2M/CSE/SUB/CRE/003 TP/oneM2M/CSE/SUB/CRE/005 |
| CE/SUB/00004/00002 | Notify with notificationEvent/representation set in the notification | TP/oneM2M/CSE/SUB/CRE/005 TP/oneM2M/CSE/SUB/UPD/001 TP/oneM2M/CSE/SUB/UPD/002 TP/oneM2M/CSE/SUB/UPD/003 TP/oneM2M/CSE/SUB/UPD/004 TP/oneM2M/CSE/SUB/UPD/005 TP/oneM2M/CSE/SUB/UPD/006 TP/oneM2M/CSE/SUB/UPD/007 TP/oneM2M/CSE/SUB/UPD/008 TP/oneM2M/CSE/SUB/UPD/009 TP/oneM2M/CSE/SUB/DEL/001 TP/oneM2M/CSE/SUB/DEL/002 TP/oneM2M/CSE/SUB/DEL/003 TP/oneM2M/CSE/SUB/NTF/001 TP/oneM2M/CSE/SUB/NTF/002 TP/oneM2M/CSE/SUB/NTF/003 |
| CE/SUB/00004/00003 | Notify with creator set in the notification | TP/oneM2M/CSE/SUB/CRE/004 |
| CE/SEC/00001/00001 | Support <accessControlPolicy> resource with attributes multiplicity equals 1[1] | TP/oneM2M/CSE/DMR/CRE/001_ACP/CB TP/oneM2M/CSE/DMR/CRE/001_ACP/AE TP/oneM2M/CSE/DMR/CRE/001_ACP/AEA TP/oneM2M/CSE/DMR/CRE/002_ACP TP/oneM2M/CSE/DMR/CRE/003_ACP TP/oneM2M/CSE/DMR/RET/001_ACP TP/oneM2M/CSE/DMR/RET/013_ACP TP/oneM2M/CSE/DMR/UPD/001_ACP/LBL TP/oneM2M/CSE/DMR/UPD/002_ACP/LBL TP/oneM2M/CSE/DMR/UPD/003_ACP/LBL TP/oneM2M/CSE/DMR/UPD/004_ACP/PV/AT/LBL TP/oneM2M/CSE/DMR/UPD/005_ACP/EXC TP/oneM2M/CSE/DMR/UPD/007_ACP/CT TP/oneM2M/CSE/DMR/UPD/008_ACP/ET TP/oneM2M/CSE/DMR/UPD/009_ACP/LBL TP/oneM2M/CSE/DMR/UPD/016_ACP/ET TP/oneM2M/CSE/DMR/UPD/016_ACP/PV TP/oneM2M/CSE/DMR/UPD/016_ACP/PVS TP/oneM2M/CSE/DMR/DEL/001_ACP TP/oneM2M/CSE/SEC/ACP/CRE/001 TP/oneM2M/CSE/SEC/ACP/CRE/002 TP/oneM2M/CSE/SEC/ACP/CRE/003 TP/oneM2M/CSE/SEC/ACP/CRE/004 |
| CE/SEC/00002/00001 | Support accessControlPolicyIDs attribute for <resource> that the Implementation supports. | TP/oneM2M/CSE/DMR/CRE/012_SUB/ACPI TP/oneM2M/CSE/DMR/CRE/012_CNT/ACPI TP/oneM2M/CSE/DMR/UPD/014_CNT/ACPI TP/oneM2M/CSE/DMR/UPD/015_AE/ACPI TP/oneM2M/CSE/DMR/UPD/015_CNT/ACPI TP/oneM2M/CSE/DMR/UPD/015_SUB/ACPI TP/oneM2M/CSE/DMR/UPD/015_GRP/ACPI TP/oneM2M/CSE/SEC/ACP/UPD/001 TP/oneM2M/CSE/SEC/ACP/UPD/002 |
| CE/SEC/00002/00002 | Support access control decision and enforcement making on the Hosting CSE | TP/oneM2M/CSE/DMR/CRE/004_CNT TP/oneM2M/CSE/DMR/CRE/004_ACP TP/oneM2M/CSE/DMR/CRE/004_SUB TP/oneM2M/CSE/DMR/CRE/004_GRP TP/oneM2M/CSE/DMR/CRE/004_PCH TP/oneM2M/CSE/DMR/RET/003_ACPTP/oneM2M/CSE/DMR/RET/003_CNT TP/oneM2M/CSE/DMR/RET/003_SUB TP/oneM2M/CSE/DMR/RET/014_ACP TP/oneM2M/CSE/DMR/RET/014_AE TP/oneM2M/CSE/DMR/RET/014_CNT TP/oneM2M/CSE/DMR/RET/014_CSR TP/oneM2M/CSE/DMR/RET/014_SUB TP/oneM2M/CSE/DMR/RET/014_GRP TP/oneM2M/CSE/DMR/UPD/006_CNT/LBL TP/oneM2M/CSE/DMR/UPD/006_ACP/LBL TP/oneM2M/CSE/DMR/UPD/006_SUB/LBL TP/oneM2M/CSE/DMR/DEL/002_CNT TP/oneM2M/CSE/DMR/DEL/002_ACP TP/oneM2M/CSE/DMR/DEL/002_SUB TP/oneM2M/CSE/DMR/DEL/002_GRP TP/oneM2M/CSE/SUB/CRE/002 TP/oneM2M/CSE/SEC/ACP/001_CRE TP/oneM2M/CSE/SEC/ACP/001_UPD TP/oneM2M/CSE/SEC/ACP/001_RET TP/oneM2M/CSE/SEC/ACP/001_DEL TP/oneM2M/CSE/SEC/ACP/002_CRE TP/oneM2M/CSE/SEC/ACP/002_UPD TP/oneM2M/CSE/SEC/ACP/002_RET TP/oneM2M/CSE/SEC/ACP/002_DEL TP/oneM2M/CSE/SEC/ACP/003_CRE TP/oneM2M/CSE/SEC/ACP/003_UPD TP/oneM2M/CSE/SEC/ACP/003_RET TP/oneM2M/CSE/SEC/ACP/003_DEL TP/oneM2M/CSE/SEC/ACP/004_CRE TP/oneM2M/CSE/SEC/ACP/004_UPD TP/oneM2M/CSE/SEC/ACP/004_RET TP/oneM2M/CSE/SEC/ACP/004_DEL TP/oneM2M/CSE/SEC/ACP/005_CRE TP/oneM2M/CSE/SEC/ACP/005_UPD TP/oneM2M/CSE/SEC/ACP/005_RET TP/oneM2M/CSE/SEC/ACP/005_DEL TP/oneM2M/CSE/SEC/ACP/006_CRE TP/oneM2M/CSE/SEC/ACP/006_UPD TP/oneM2M/CSE/SEC/ACP/006_RET TP/oneM2M/CSE/SEC/ACP/006_DEL TP/oneM2M/CSE/SEC/ACP/007_CRE TP/oneM2M/CSE/SEC/ACP/007_UPD TP/oneM2M/CSE/SEC/ACP/007_RET TP/oneM2M/CSE/SEC/ACP/007_DEL TP/oneM2M/CSE/SEC/ACP/008_CRE TP/oneM2M/CSE/SEC/ACP/008_UPD TP/oneM2M/CSE/SEC/ACP/008_RET TP/oneM2M/CSE/SEC/ACP/008_DEL TP/oneM2M/CSE/SEC/ACP/009_CRE TP/oneM2M/CSE/SEC/ACP/009_UPD TP/oneM2M/CSE/SEC/ACP/009_RET TP/oneM2M/CSE/SEC/ACP/009_DEL TP/oneM2M/CSE/SEC/ACP/010_CRE TP/oneM2M/CSE/SEC/ACP/010_UPD TP/oneM2M/CSE/SEC/ACP/010_RET TP/oneM2M/CSE/SEC/ACP/010_DEL TP/oneM2M/CSE/SEC/ACP/011_CRE TP/oneM2M/CSE/SEC/ACP/011_UPD TP/oneM2M/CSE/SEC/ACP/011_RET TP/oneM2M/CSE/SEC/ACP/011_DEL TP/oneM2M/CSE/SEC/ACP/012_CRE TP/oneM2M/CSE/SEC/ACP/012_UPD TP/oneM2M/CSE/SEC/ACP/012_RET TP/oneM2M/CSE/SEC/ACP/012_DEL |
| CE/DIS/00001/00001 | Support the Discovery request with filterUsage equals to “Discovery” | TP/oneM2M/CSE/DIS/001 TP/oneM2M/CSE/DIS/004 TP/oneM2M/CSE/DIS/005 TP/oneM2M/CSE/DIS/006 |
| CE/DIS/00001/00002 | Support discovery with filterCriteria (createdBefore, createdAfter, modifiedSince, unmodifiedSince, stateTagSmaller, stateTagBigger, expireBefore, expireAfter, labels, resourceType, sizeAbove, sizeBelow, contentType, limit, attribute) | TP/oneM2M/CSE/DIS/002 TP/oneM2M/CSE/DIS/003 TP/oneM2M/CSE/DIS/007 TP/oneM2M/CSE/DIS/008 TP/oneM2M/CSE/DIS/009 |
| CE/DIS/00001/00003 | Support discovery with filterCriteria (level, offset) |  |
| CE/DIS/00001/00004 | Support discovery with content filter (contentFilterSyntax, contentFilterQuery of filterCriteria) |  |
| CE/DIS/00001/00005 | Support discovery with filterCriteria (filterOperation) |  |
| CE/GMG/00001/00001 | Support <group> resource with MIXED memberType | TP/oneM2M/CSE/DMR/CRE/001_GRP/CB TP/oneM2M/CSE/DMR/CRE/001_GRP/AE TP/oneM2M/CSE/DMR/CRE/002_GRP TP/oneM2M/CSE/DMR/CRE/003_GRP TP/oneM2M/CSE/DMR/RET/001_GRP TP/oneM2M/CSE/DMR/RET/013_GRP TP/oneM2M/CSE/DMR/UPD/001_GRP/LBL TP/oneM2M/CSE/DMR/UPD/002_GRP/LBL TP/oneM2M/CSE/DMR/UPD/003_GRP/LBL TP/oneM2M/CSE/DMR/UPD/004_GRP/ET/GN/LBL TP/oneM2M/CSE/DMR/UPD/005_GRP/EXC TP/oneM2M/CSE/DMR/UPD/006_GRP/LBL TP/oneM2M/CSE/DMR/UPD/007_GRP/CT TP/oneM2M/CSE/DMR/UPD/008_GRP/ET TP/oneM2M/CSE/DMR/UPD/009_GRP/LBL TP/oneM2M/CSE/DMR/UPD/016_GRP/ET TP/oneM2M/CSE/DMR/UPD/016_GRP/MNM TP/oneM2M/CSE/DMR/UPD/016_GRP/MID TP/oneM2M/CSE/DMR/DEL/001_GRP TP/oneM2M/CSE/DMR/DEL/002_GRP TP/oneM2M/CSE/GMG/CRE/001 TP/oneM2M/CSE/GMG/CRE/003 TP/oneM2M/CSE/GMG/UPD/002 TP/oneM2M/CSE/GMG/UPD/007 TP/oneM2M/CSE/GMG/UPD/008 TP/oneM2M/CSE/DMR/UPD/015_GRP/GN |
| CE/GMG/00002/00001 | Support <group> resource with single specific resource types | TP/oneM2M/CSE/GMG/CRE/002 TP/oneM2M/CSE/GMG/CRE/004 TP/oneM2M/CSE/GMG/CRE/005 TP/oneM2M/CSE/GMG/CRE/006 TP/oneM2M/CSE/GMG/CRE/007 TP/oneM2M/CSE/GMG/UPD/001 TP/oneM2M/CSE/GMG/UPD/003 TP/oneM2M/CSE/GMG/UPD/004 TP/oneM2M/CSE/GMG/UPD/005 TP/oneM2M/CSE/GMG/UPD/006 TP/oneM2M/CSE/GMG/UPD/009 TP/oneM2M/CSE/GMG/UPD/010 |
| CE/GMG/00003/00001 | Fan out of request to all members and aggregate member responses | TP/oneM2M/CSE/GMG/RET/001 TP/oneM2M/CSE/GMG/001_CRE TP/oneM2M/CSE/GMG/001_RET TP/oneM2M/CSE/GMG/001_UPD TP/oneM2M/CSE/GMG/001_DEL TP/oneM2M/CSE/GMG/002_CRE TP/oneM2M/CSE/GMG/002_RET TP/oneM2M/CSE/GMG/002_UPD TP/oneM2M/CSE/GMG/002_DEL TP/oneM2M/CSE/GMG/003_CRE TP/oneM2M/CSE/GMG/003_RET TP/oneM2M/CSE/GMG/003_UPD TP/oneM2M/CSE/GMG/003_DEL TP/oneM2M/CSE/GMG/004_CRE TP/oneM2M/CSE/GMG/004_RET TP/oneM2M/CSE/GMG/004_UPD TP/oneM2M/CSE/GMG/004_DEL TP/oneM2M/CSE/GMG/005_CRE TP/oneM2M/CSE/GMG/005_RET TP/oneM2M/CSE/GMG/005_UPD TP/oneM2M/CSE/GMG/005_DEL TP/oneM2M/CSE/GMG/006_CRE TP/oneM2M/CSE/GMG/006_RET TP/oneM2M/CSE/GMG/006_UPD TP/oneM2M/CSE/GMG/006_DEL |
| CE/GMG/00003/00002 | Create sub group resource in case multiple members belong to same member CSE |  |
| CE/GMG/00003/00003 | Create <subscription> resource via group without aggregating notifications |  |
| CE/GMG/00003/00004 | Create <subscription> resource via group and aggregate notifications |  |
| CE/GMG/00003/00005 | Support the membersAccessControlPolicyIDs attribute of <group> resource | TP/oneM2M/CSE/DMR/UPD/015_GRP/MACP |
| CE/PCH/00001/00001 | Polling Channel resource management | TP/oneM2M/CSE/DMR/CRE/001_PCH/AE TP/oneM2M/CSE/DMR/CRE/002_PCH TP/oneM2M/CSE/DMR/CRE/003_PCH TP/oneM2M/CSE/DMR/RET/001_PCH TP/oneM2M/CSE/DMR/UPD/001_PCH/LBL TP/oneM2M/CSE/DMR/UPD/002_PCH/LBL TP/oneM2M/CSE/DMR/UPD/003_PCH TP/oneM2M/CSE/DMR/UPD/007_PCH/CT TP/oneM2M/CSE/DMR/UPD/009_PCH/EXC TP/oneM2M/CSE/DMR/DEL/001_PCH |
| CE/PCH/00002/00001 | Long Polling Procedure | TP/oneM2M/CSE/PCH/001 TP/oneM2M/CSE/PCH/002 TP/oneM2M/CSE/PCH/003 TP/oneM2M/CSE/PCH/004 TP/oneM2M/CSE/PCH/005_XXX TP/oneM2M/CSE/PCH/006 |

Table A-2: Mapping of feature to Test Purposes for AE

| Feature ID | Description | Mapped Test Purposes |
| --- | --- | --- |
| AE/GEN/00001 | Support one of the format of resource identification | TP/oneM2M/AE/GEN/CRE/001_CSR TP/oneM2M/AE/GEN/CRE/001_SPR TP/oneM2M/AE/GEN/CRE/001_ABS TP/oneM2M/AE/GEN/CRE/002_CSR TP/oneM2M/AE/GEN/CRE/002_SPR TP/oneM2M/AE/GEN/CRE/002_ABS TP/oneM2M/AE/GEN/UPD/001_CSR TP/oneM2M/AE/GEN/UPD/001_SPR TP/oneM2M/AE/GEN/UPD/001_ABS TP/oneM2M/AE/GEN/UPD/002_CSR TP/oneM2M/AE/GEN/UPD/002_SPR TP/oneM2M/AE/GEN/UPD/002_ABS TP/oneM2M/AE/GEN/RET/001_CSR TP/oneM2M/AE/GEN/RET/001_SPR TP/oneM2M/AE/GEN/RET/001_ABS TP/oneM2M/AE/GEN/RET/002_CSR TP/oneM2M/AE/GEN/RET/002_SPR TP/oneM2M/AE/GEN/RET/002_ABS TP/oneM2M/AE/GEN/DEL/001_CSR TP/oneM2M/AE/GEN/DEL/001_SPR TP/oneM2M/AE/GEN/DEL/001_ABS TP/oneM2M/AE/GEN/DEL/002_CSR TP/oneM2M/AE/GEN/DEL/002_SPR TP/oneM2M/AE/GEN/DEL/002_ABS |
| AE/GEN/00002/00001 | Support create request | TP/oneM2M/AE/DMR/CRE/001 |
| AE/DMR/00002/00001 | Support create contentInstance | TP/oneM2M/AE/DMR/CRE/002 |
| AE/REG/00002/00001 | Create <AE> | TP/oneM2M/AE/REG//CRE/001 TP/oneM2M/AE/REG/CRE/002_RR |
| AE/GEN/00003/00001 | Create <AE> with resourceName | TP/oneM2M/AE/REG/CRE/002_RN |
| AE/GEN/00003/00002 | Create <AE> with expirationTime | TP/oneM2M/AE/REG/CRE/002_ET |
| AE/DIS/00001/00017 | Create <AE> with labels | TP/oneM2M/AE/REG/CRE/002_LBL |
| AE/REG/00002/00002 | Create <AE> with applicationName | TP/oneM2M/AE/REG/CRE/002_APN |
| AE/DMR/00001/00001 | Create <container> with no attribute set | TP/oneM2M/AE/DMR/CRE/001 |
| AE/GEN/00003/00001 | Create <container> with resourceName | TP/oneM2M/AE/DMR/CRE/004_RN |
| AE/GEN/00003/00002 | Create <container> with expirationTime | TP/oneM2M/AE/DMR/CRE/004_ET |
| AE/DIS/00001/00017 | Create <container> with labels | TP/oneM2M/AE/DMR/CRE/004_LBL |
| AE/DMR/00001/00002 | Create <container> with maxNrOfInstances | TP/oneM2M/AE/DMR/CRE/004_MNI |
| AE/DMR/00001/00003 | Create <container> with maxByteSize | TP/oneM2M/AE/DMR/CRE/004_MBS |
| AE/DMR/00001/00004 | Create <container> with maxInstanceAge | TP/oneM2M/AE/DMR/CRE/004_MIA |
| AE/GEN/00003/00001 | Create <contentInstance> with resourceName | TP/oneM2M/AE/DMR/CRE/003_RN |
| AE/GEN/00003/00002 | Create <contentInstance> with expirationTime | TP/oneM2M/AE/DMR/CRE/003_ET |
| AE/DIS/00001/00017 | Create <contentInstance> with labels | TP/oneM2M/AE/DMR/CRE/003_LBL |
| AE/DMR/00002/00002 | Create <contentInstance> with contentInfo | TP/oneM2M/AE/DMR/CRE/003_CNF |
| AE/REG/00002/00003 | Create <AE> with pointOfAccess | TP/oneM2M/AE/REG/CRE/002_POA |
| AE/SUB/00001/00001 | Create <subscription> with mandatory attributes [2] | TP/oneM2M/AE/SUB/CRE/001 |
| AE/SUB/00004/00001 | Receiving a notification |  |
| GE/PCH/00001/00001 GE/PCH/00001/00002 GE/PCH/00001/00003 | Polling Channel resource management |  |
| GE/PCH/00002/00001 | Long Polling Procedure |  |
| AE/GEN/00002/00002 | Support Retrieve request targeting one resource | TP/oneM2M/AE/DMR/RET/001_CB TP/oneM2M/AE/DMR/RET/001_AE TP/oneM2M/AE/DMR/RET/001_CNT |
| AE/GEN/00002/00003 | Support Update request targeting one resource | TP/oneM2M/AE/DMR/UPD/002_LBL\ |
| AE/GEN/00002/00004 | Support Delete request targeting one resource | TP/oneM2M/AE/DMR/DEL/003 |
| AE/REG/00001/00001 | The Retrieval of <CSEBase> resource | TP/oneM2M/AE/DMR/RET/001_CB |
| AE/REG/00002/00004 | Create <AE> with nodeLink | TP/oneM2M/AE/REG/CRE/002_NL |
| AE/REG/00002/00005 | Create <AE> with contentSerialization | TP/oneM2M/AE/REG/CRE/002_CSZ |
| AE/GEN/00003/00002 | Update <AE> with expirationTime | TP/oneM2M/AE/DMR/UPD/001_ET\ |
| AE/DIS/00001/00017 | Update <AE> with labels | TP/oneM2M/AE/DMR/UPD/001_LBL\ |
| AE/REG/00002/00007 | Update <AE> with applicationName | TP/oneM2M/AE/DMR/UPD/001_APN\ |
| AE/REG/00002/00008 | Update <AE> with pointOfAccess | TP/oneM2M/AE/DMR/UPD/001_POA\ |
| AE/REG/00002/00009 | Update <AE> with nodeLink | TP/oneM2M/AE/DMR/UPD/001_NL\ |
| AE/GEN/00002/00005 | Update <AE> with requestReachability | TP/oneM2M/AE/DMR/UPD/001_RR\ |
| AE/REG/00002/00010 | Update <AE> with contentSerialization | TP/oneM2M/AE/DMR/UPD/001_CSZ |
| AE/REG/00002/00013 | Delete <AE> | TP/oneM2M/AE/REG/DEL/001 |
| AE/REG/00002/00012 | Retrieve <AE> | TP/oneM2M/AE/DMR/RET/001_AE |
| AE/GEN/00003/00002 | Update <container> with expirationTime | TP/oneM2M/AE/DMR/UPD/002_ET |
| AE/DIS/00001/00017 | Update <container> with labels | TP/oneM2M/AE/DMR/UPD/002_LBL |
| AE/DMR/00001/00006 | Update <container> with maxNrOfInstances | TP/oneM2M/AE/DMR/UPD/002_MNI |
| AE/DMR/00001/00007 | Update <container> with maxByteSize | TP/oneM2M/AE/DMR/UPD/002_MBS |
| AE/DMR/00001/00008 | Update <container> with maxInstanceAge | TP/oneM2M/AE/DMR/UPD/002_MIA |
| AE/DMR/00001/00011 | Delete <container> | TP/oneM2M/AE/DMR/DEL/003 |
| AE/DMR/00001/00012 | Retrieve <container> | TP/oneM2M/AE/DMR/RET/004 |
| AE/DMR/00002/00001 | Create <contentInstance> with mandatory attributes | TP/oneM2M/AE/DMR/CRE/002\ |
| AE/DMR/00002/00003 | Create <contentInstance> with contentRef | TP/oneM2M/AE/DMR/CRE/003_CONR |
| AE/DMR/00002/00004 | Retrieve <contentInstance> resource targeting the <contentInstance> resource | TP/oneM2M/AE/DMR/RET/005 |
| AE/DMR/00002/00005 | Retrieve oldest <contentInstance> resource targeting the <oldest> resource | TP/oneM2M/AE/DMR/RET/002 |
| AE/DMR/00002/00006 | Retrieve latest <contentInstance> resource targeting the <latest> resource | TP/oneM2M/AE/DMR/RET/003\ |
| AE/DMR/00002/00007 | Delete <contentInstance> | TP/oneM2M/AE/DMR/DEL/004 |
| AE/DMR/00002/00008 | Delete oldest <contentInstance> resource targeting the <oldest> resource | TP/oneM2M/AE/DMR/DEL/001\ |
| AE/DMR/00002/00009 | Delete latest <contentInstance> resource targeting the <latest> resource | TP/oneM2M/AE/DMR/DEL/002\ |
| AE/REG/00002/00001 | Create <AE> with App-IDattribute | TP/oneM2M/AE/REG/CRE/002_API |
| VOID | Create <AE> with ontologyRefattribute | TP/oneM2M/AE/REG/CRE/002_OR |
| VOID | Update <AE> with ontologyRefattribute | TP/oneM2M/AE/DMR/UPD/001_OR |
| AE/SEC/00001/00007 | Create <container> with acpi | TP/oneM2M/AE/DMR/CRE/004_ACPI |
| VOID | Create <container> with ontologyRef | TP/oneM2M/AE/DMR/CRE/004_OR |
| VOID | Create <container> with locationID | TP/oneM2M/AE/DMR/CRE/004_LI |
| AE/SUB/00001/00002 | Create <subscription> with labels | TP/oneM2M/AE/SUB/CRE/002_LBL |
| AE/SUB/00001/00005 | Create <subscription> with subscriberURI | TP/oneM2M/AE/SUB/CRE/002_SU |
| AE/SEC/00001/00007 | Create <subscription> with acpi | TP/oneM2M/AE/SUB/CRE/002_ACPI |
| AE/SUB/00002/00001 | Create <subscription> with eventNotificationCriteria | TP/oneM2M/AE/SUB/CRE/002_ENC |
| AE/SUB/00002/00002 | Create <subscription> with expirationCounter | TP/oneM2M/AE/SUB/CRE/002_EXC |
| AE/SUB/00003/00001 | Create <subscription> with notificationForwardingURI | TP/oneM2M/AE/SUB/CRE/002_NFU |
| AE/SUB/00002/00009 | Create <subscription> with notificationContentType | TP/oneM2M/AE/SUB/CRE/002_NCT |
| AE/SUB/00002/00010 | Create <subscription> with notificationEventCat | TP/oneM2M/AE/SUB/CRE/002_NEC |
| VOID | Support NOTIFY Request | TP/oneM2M/AE/SUB/NTF/001 TP/oneM2M/AE/SUB/NTF/002 |

# History

This clause shall be the last one in the document and list the main phases (all additional information will be removed at the publication stage).

| Publication history |  |  |
| --- | --- | --- |
| V3.2.0 | 14 Dec 2023 | This baseline including agreed CR for markdown conversion: TDE-2023-0038R01-draft_updated_TS-0025_for_markdown_conversion |

| Draft history (to be removed on publication) |  |  |
| --- | --- | --- |
| V3.0.0 | 24 April 2019 | Initial skeleton (based on V2.2.0)& Incorporated contributions: TDE-2019-0033R01-TS-0025-Add_Profile_for_UE_hosted_sensor |
| V3.1.0 | 06 June 2019 | Incorporated contributions: TDE-2019-0088-TS-0025_Update_of_TP_mapping_R3 TDE-2019-0114-TS-0025_final_modifications_R3 TDE-2019-0115-TS-0025_Rel-3_Moving_ADN_TP_list_to_Annex |
| V3.2.1 | 27 Jun 2025 | This baseline is markdown converted baseline |